3GPP TSG-RAN WG4 Meeting #95-e	R4-2007237

# Electronic Meeting, 25th May - 5th Jun, 2020

Title: 	Simulation results on NR UE HST performance requirements for multi-path fading channel

Source: 	Huawei, HiSilicon

Agenda item:	6.17.2.1.4

Document for:	Discussion

Background

During RAN4#94e meeting, way forward [1] for NR Rel-16 UE demodulation was approved. The simulation assumption is derived as following Table 1-1.

Table 1-1 Simulation assumption for multi-path fading channel

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 2×2; 2×4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, S: 6D 4G 4U |  |
| MCS | MCS 4; MCS 13; MCS 17 based on 64QAM table |  |
| Propagation condition | TDL-C 300ns |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Rank | Rank = 1 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | 600Hz | 1200Hz |
| Testing metric | SNR @70% of maximum throughput |  |

In this contribution, we provide our updated simulation results for the UE demodulation requirements for NR Rel-16 HST multi-path fading channel.

Simulations

Table 2-1: Ideal Simulation results for multi-path fading channel for NR UE HST

| Case Number | Antenna configuration | CHBW/SCS | maximum Doppler shift(Hz) | SNR@70% Max TP |
| --- | --- | --- | --- | --- |
| 1 | 2x2 | 10MHz/15kHz | 600 | 6.70 |
| 2 | 2x4 | 10MHz/15kHz | 600 | 2.94 |
| 3 | 2x2 | 40MHz/30kHz | 1200 | 7.60 |
| 4 | 2x4 | 40MHz/30kHz | 1200 | 3.54 |

Figure 2-1: Ideal Simulation results for multi-path fading channel for NR UE HST

Conclusion

In this contribution, we provide our updated simulation results for multi-path fading channel.

Reference

R4-2005532, WF on UE demodulation for NR HST, RAN4#94e-bis, CMCC