3GPP TSG-RAN WG4 Meeting #95-e 	R4-2007306

Electronic Meeting, 25 May – 5 June, 2020

Source: 	Huawei

Title: 	UE IMT technology related parameters

Agenda Item:	11.1.1

Document for:	Approval

- Introduction

In the last plenary (RAN#87e) the Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz was approved [1]. IMT technology related parameters have been discussed in last RAN4#94e-Bis meeting. The WF on on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz was approved in [2].

This paper discusses UE IMT technology related parameters.

- Discussion

In last meeting there are parameters are identified as FFS. The two frequency ranges share the same open issues.

For 6.425-7.125 GHz,

  - Maximum output power: 23dBm, FFS on 20dBm

  - Power dynamic range:

    - Option 1: TS 38.101

    - Option 2: 56dB

  - Blocking:

    - Option 1: 38.101

    - Option 2: FFS

For 10.0-10.5 GHz

  - Maximum output power: 23dBm, FFS on 20dBm

  - Power dynamic range:

    - Option 1: TS 38.101

    - Option 2: 56dB

  - Blocking:

    - Option 1: 38.101

    - Option 2: FFS

Maximum output power:

For the purposes for this response the FR1 values can be quoted. It is proposed to refer to TS 38.101-1 clause 6.2 for mobile station. 23 dBm is adopted in the coexistence study.

TR 38.820 indicates that 23dBm is feasible at 10-10.5GHz hence this is a good assumption to make as a worst case for any co-existence analysis (although actual power may be lower due to implementation at the higher frequencies)

Power dynamic range (dB):

For 6.425-7.125 GHz,

For the purposes for this response the FR1 values can be quoted. It is proposed to refer to TS 38.101-1 clause 6.3.1 for mobile station.

For 10.0-10.5 GHz

For the UE there is a significant difference on the Tx minimum power between FR1 and FR2. FR1 is approx. -33dBm and FR2 approx. -13dBm (Power class 2,3,4). The increased PL due to frequency relaxes the minimum power requirement minimum power at 10-10.5GHz further study is needed to find the correct minimum power requirement (it will be between -13 and -33dBm for 100MHz CBW)

Blocking:

For 6.425-7.125 GHz,

For mobile station, it can refer to TS 38.101-1 clause 7.6.2

For 10.0-10.5 GHz

AS with the sensitivity it seems likely the UE will have FR1 like requirements so for IBB we can refer to TS 38.101-1 clause 7.6.2 however the wanted signal and the interferer levels will be adjusted based on the NF and the results of any blocking analysis.

- Conclusion

This paper looks at IMT parameters on the 6.425-7.125GHz and 10.0-10.5GHz frequency range.

Proposal: The following parameters are proposed for co-existence study and reply LS.

|  |  |  | IMT |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Parameter |  | Mobile station |  |  |
| 1 | Duplex Method |  | TDD |  |  |
| 2 | Channel bandwidth (MHz) |  | 100 MHz |  |  |
| 3 | Signal bandwidth (MHz) |  | 30 kHz: 98.28 MHz 60 kHz: 97.92 MHz |  |  |
| 4 | Transmitter characteristics |  |  |  |  |
| 4.1 | Power dynamic range (dB) |  | 6.425-7.125GHz: Reference to TS 38.101 10-10.5GHz: FFS |  |  |
| 4.2 | Spectral mask (dB) |  | Based on simulations |  |  |
| 4.3 | ACLR |  | Based on simulations |  |  |
| 4.4 | Spurious emissions |  | 30MHz ≤ f ≤ 1 GHz: -36dBm/100kHz 1 GHz ≤ f ≤ 26 GHz: -30dBm/1MHz |  |  |
| 4.5 | Maximum output power |  | 23dBm |  |  |
| 4.6 | Average output power |  | N/A |  |  |
| 5 |  | Receiver characteristics |  |  |  |
| 5.1 |  | Noise figure (dB) |  | 9 dB |  |
| 5.2 |  | Sensitivity (dBm) |  | N/A |  |
| 5.3 |  | Blocking response |  | 6.425-7.125GHz: Reference to TS 38.101 10-10.5GHz: FFS |  |
| 5.4 |  | ACS |  | Based on simulations |  |
| 5.5 |  | SINR operating range (dB) |  | TR 38.803 Annex F |  |

References

[1]	RP-200513, New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz	Ericsson, Huawei, HiSilicon

[2] R4-2005733, WF on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz, CATT

[3]	3GPP TR 38.820		NR;7 - 24 GHz frequency range