3GPP TSG-RAN WG4 Meeting #95-e 	R4-2007309

Electronic Meeting, 25 May – 5 June, 2020

Source: 	Huawei, HiSilicon

Title: 	Antenna characteristics on 6.425-7.125GHz and 10.0-10.5 GHz

Agenda Item:	11.1.2.

Document for:	Approval

- Introduction

In the last plenary (RAN#87e) the Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz was approved [1]. IMT technology related parameters have been discussed in last RAN4#94e-Bis meeting. The WF on BS antenna parameters for 6.425-7.125GHz and 10.0-10.5GHz was approved in [2].

This paper discusses the antenna characteristics on 6.425-7.125GHz and 10.0-10.5 GHz.

- Discussion

In last meeting there are parameters are identified as FFS, e.g, for 6.425-7.125 GHz,

- Element parameters

  - Horizontal/vertical element spacing

  - Horizontal/vertical 3 dB beamwidth

  - Element gain (GE,max)

- Antenna configuration, candidate options:

  - 8*8

  - 4*8

  - 16*8

- additional parameters

FR1 high gain antennas used previously range from:

8x8 element, 0.9λx0.6λ spacing -> 34.6λ2.	(LS to ITU on 3300-3400 MHz, RP-182783)

TABLE A2-1 of RP-182783

|  |  | Outdoor Suburban hotspot |  | Outdoor Urban hotspot | Indoor |
| --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |
| 1.1 | Antenna pattern | Refer to Recommendation ITU-R M.2101 |  |  |  |
| 1.2 | Element gain (dBi) | 8 | 8 |  | 8 |
| 1.3 | Horizontal/vertical 3 dB beamwidth of single element (degree) | 80º for H 65 for V | 80º for H 65 for V |  | 80º for H 65 for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V |  | 25 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º | Linear ±45º |  | Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) NOTE 2 | 8×8 elements | 8×8 elements |  | 8×8 elements |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.6 of wavelength for H, 0.9 of wavelength for V | 0.6 of wavelength for H, 0.9 of wavelength for V |  | 0.6 of wavelength for H, 0.9 of wavelength for V |
| 1.8 | Array Ohmic loss (dB) | 2 | 2 |  | 2 |
| 1.9 | Conducted power (before Ohmic loss) per antenna element (dBm/200 MHz) | 25/28/31 | 25/28/31 |  | 6/9/12 |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 120 | 120 |  | 120 |

FR2 high gain antennas

8x16 element, 0.5λx0.5λ spacing -> 32λ2.	(NR FR2 simulations)

Table 5.2.3.2.1-1 of TR 38.803: BS antenna modelling for Urban macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |
| (Mg, Ng, M, N, P) note | For 30GHz: (1, 1, 8, 16, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |

The maximum antenna aperture for both FR1 (higher frequencies) and FR2 has been 32 to 34λ2 the FR1 BS has greater gain in the vertical dimension (to maintain parity with existing passive antenna gain) and the FR2 has the greater gain in the horizontal dimension.

For 6-7GHz the physical size of the antenna is reduced compared to previous 2GHz studies but still not completely irrelevant, hence it is preferable to maintain the portrait aspect of the antenna. As there have been grating lobe issues related to the larger element spacing the total aperture is maintained but the separation of elements is reduced and hence the number of them is increased. Using 0.5λ element spacing avoids any grating lobes. We propose to adopt 0.5λ element spacing as below.

- Element parameters

  - Horizontal/vertical element spacing: 0.5λx0.5λ

  - Horizontal/vertical 3 dB beamwidth: 90deg x90deg

  - Element gain (GE,max): 3.5 dBi

- Antenna configuration: 16x8 (v x h)

It was also agreed to add following additional parameters to better characterize AAS antenna. According to the consideration on cell size in [3], we think 43 dBm/100 MHz is a typical value for co-existence study.

| Parameter | Wide area Sub-urban | Wide area Urban |
| --- | --- | --- |
| Horizontal coverage range (degrees) | +/- 60 | +/- 60 |
| Vertical coverage range (degrees) | [90 to 120] | [90 to 120] |
| BS output power, TRP (dBm) | 43 | 43 |

- Conclusion

This paper looks at antenna characteristics on the 6.425-7.125GHz and 10.0-10.5GHz frequency range. The following parameters are proposed for co-existence study.

| Parameter | Wide area Sub-urban | Wide area Urban |
| --- | --- | --- |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V | 0.5 of wavelength for H 0.5 of wavelength for V |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V | 90º for H 90º for V |
| Element gain (GE,max) | 3.5 | 3.5 |
| Antenna array configuration (Row × Column) | 16 × 8 elements | 16 × 8 elements |
| Horizontal coverage range (degrees) | +/- 60 | +/- 60 |
| Vertical coverage range (degrees) | [90 to 120] | [90 to 120] |
| BS output power, TRP (dBm) | 43 | 43 |

References

[1]	RP-200513, New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz	Ericsson, Huawei, HiSilicon

[2] R4-2005173, WF on BS Antenna parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz, Huawei

[3]	3GPP TR 38.820		NR;7 - 24 GHz frequency range