3GPP TSG-RAN WG4 Meeting #95-e	R4-2007397

# E-meeting, 25th May, 2020 – 5th June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 4899 | rev |  | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.104: Introduction of LTE based 5G terrestrial broadcast numerologies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast -Core |  |  |  |  |  | Date: |  |  | 2020-05-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of 2.5 kHz and 0.37 kHz numerologies agreed in RAN1 to be supported by LTE based 5G terrestrial broadcast.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extended applicability of requirements for LTE based terrestrial broadcast new numerologies; added tables with EVM window length for 2.5kHz and 0.37kHz numerologies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specifications do not support 2.5 kHz and 0.37 kHz numerologies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4, 6.5, Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

4.4	Applicability of requirements

For BS that is E-UTRA (single-RAT), E-UTRA with NB-IoT (in band and/or guard band) or standalone NB-IoT capable only, MBMS (including 15 kHz, 7.5 kHz,1.25 kHz, 2.5 kHz and 0.37 kHz subcarrier spacing), the requirements in the present document are applicable and additional conformance to TS 37.104 [15] is optional. For a BS additionally conforming to TS 37.104 [15], conformance to some of the RF requirements in the present document can be demonstrated through the corresponding requirements in TS 37.104 [15] as listed in Table 4.4-1.

<Next of change>

6	Transmitter characteristics

6.5	Transmitted signal quality

The requirements in clause 6.5 apply to the transmitter ON period.

6.5.2	Error Vector Magnitude

The Error Vector Magnitude is a measure of the difference between the ideal symbols and the measured symbols after the equalization. This difference is called the error vector. The equaliser parameters are estimated as defined in Annex E. The EVM result is defined as the square root of the ratio of the mean error vector power to the mean reference power expressed in percent.

For E-UTRA, for all bandwidths, the EVM measurement shall be performed for each E-UTRA carrier over all allocated resource blocks and downlink subframes within 10ms measurement periods for subframe TTI, and over all allocated resource blocks and downlink sTTIs within 10ms measurement periods for sTTI. The boundaries of the EVM measurement periods need not be aligned with radio frame boundaries. The EVM value is then calculated as the mean square root of the measured values. The EVM of each E-UTRA carrier for different modulation schemes on PDSCH or sPDSCH or PMCH shall be better than the limits in table 6.5.2-1:

Table 6.5.2-1: EVM requirements for E-UTRA carrier

| Modulation scheme for PDSCH or sPDSCH | Required EVM [%] |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |
| 1024QAM | 2.5 % |

For NB-IoT, for all bandwidths, the EVM measurement shall be performed for each NB-IoT carrier over all allocated resource and downlink subframes within 10ms measurement periods. The boundaries of the EVM measurement periods need not be aligned with radio frame boundaries. The EVM value is then calculated as the mean square root of the measured values. The EVM of each NB-IoT carrier on NB-PDSCH shall be better than the limits in Table 6.5.2-2:

Table 6.5.2-2: EVM requirements for NB-IoT carrier

| Modulation scheme for NB-PDSCH | Required EVM [%] |
| --- | --- |
| QPSK | 17.5 % |

<Next of change>

E.1	Reference point for measurement

The EVM shall be measured at the point after the FFT and a zero-forcing (ZF) equalizer in the receiver, as depicted in Figure E.1-1 below.

Figure E.1-1: Reference point for EVM measurement

E.2	Basic unit of measurement

For 15 kHz, 7.5 kHz,1.25 kHz, 2.5 kHz subcarrier spacing, t basic unit of EVM measurement is defined over one subframe (1ms) for subframe TTI and over one sTTI when supporting sTTI feature in the time domain and  subcarriers (180kHz) in the frequency domain:

For 0.37 kHz subcarrier spacing, the basic unit of EVM measurement is defined over one subframe (3ms) for subframe TTI in the time domain and  subcarriers (180kHz) in the frequency domain:

where

is the set of symbols with the considered modulation scheme being active within the subframe or within the sTTI,

is the set of subcarriers within the  subcarriers with the considered modulation scheme being active in symbol t,

is the ideal signal reconstructed by the measurement equipment in accordance with relevant Tx models,

is the modified signal under test defined in E.3.

Note:	Although the basic unit of measurement is one subframe or one sTTI, the equalizer is calculated over 10 subframe measurement periods to reduce the impact of noise in the reference symbols. The boundaries of the 10 subframe measurement periods need not be aligned with radio frame boundaries.

E.3	Modified signal under test

Implicit in the definition of EVM is an assumption that the receiver is able to compensate a number of transmitter impairments. The signal under test is equalised and decoded according to:

where

is the time domain samples of the signal under test.

is the sample timing difference between the FFT processing window in relation to nominal timing of the ideal signal. Note that two timing offsets are determined, the corresponding EVM is measured and the maximum used as described in E.7.

is the RF frequency offset.

is the phase response of the TX chain.

is the amplitude response of the TX chain.

E.4	Estimation of frequency offset

For 15 kHz, 7.5 kHz, 2.5 kHz, the observation period for determining the frequency offset  shall be 1 ms.

For 1.25kHz , the observation period for determining the frequency offset  shall be  [2/4] ms

For 0.37 kHz, the observation period for determining the frequency offset  shall be [12] ms.

E.5	Estimation of time offset

For 15kHz, 7.5 kHz, 1.25 kHz and 2.5 kHz, the observation period for determining the sample timing difference shall be 1 ms.

For 0.37 kHz, the observation period for determining the frequency offset  shall be 3 ms.

In the following   represents the middle sample of the EVM window of length  (defined in E.5.1)  or the last sample of the first window half if is even.

is estimated so that the EVM window of length  is centred on  the measured cyclic prefix of the considered OFDM symbol. To minimize the estimation error the timing shall be based on the primary synchronization signal and reference signals. To limit time distortion of any transmit filter the reference signals in the 1 outer RBs are not taken into account in the timing estimation

Two values for  are determined:

and

where  if  is odd and  if is even.

When the cyclic prefix length varies from symbol to symbol (e.g. time multiplexed MBMS and unicast) then   shall be further restricted to the subset of symbols with the considered modulation scheme being active and with the considered cyclic prefix length type.

E.5.1	Window length

Table E.5.1-1 and Table E.5.1-1a below specify EVM window length (W) for normal CP, the cyclic prefix length  is 160 for symbols 0 and 144 for symbols 1-6.

Table E.5.1-2, Table E.5.1-2a ,Table E.5.1-2b ,Table E.5.1-2c and Table E.5.1-2d specify the EVM window length (W) for extended CP for 15 kHz, 7.5 kHz,1.25 kHz, 2.5 kHz and 0.37 kHz sub-carrier spacing, the cyclic prefix length  is 512, 1024 6144, 3072 and 9094 respectively.

Table E.5.1-1: EVM window length for normal CP for E-UTRA

| Channel Bandwidth MHz | FFT size |  | Cyclic prefix length for symbols 0 in FFT samples | Cyclic prefix length for symbols 1-6 in FFT samples | EVM window length W | Ratio of W to total CP for symbols 1-6(Note 1) [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 1.4 | 128 |  | 10 | 9 | 5 | 55.6 |
| 3 | 256 |  | 20 | 18 | 12 | 66.7 |
| 5 | 512 |  | 40 | 36 | 32 | 88.9 |
| 10 | 1024 |  | 80 | 72 | 66 | 91.7 |
| 15 | 1536 |  | 120 | 108 | 102 | 94.4 |
| 20 | 2048 |  | 160 | 144 | 136 | 94.4 |
| Note 1: These percentages are informative and apply to symbols 1 through 6. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |  |  |

Table E.5.1-1a: EVM window length for normal CP for NB-IoT

| FFT size | Cyclic prefix length for symbols 0 in FFT samples | Cyclic prefix length for symbols 1-6 in FFT samples | EVM window length W | Ratio of W to total CP for symbols 1-6(Note 1) [%] |
| --- | --- | --- | --- | --- |
| 128 | 10 | 9 | 3 | 33.3 |
| Note 1: These percentages are informative and apply to symbols 1 through 6. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |

Table E.5.1-2 EVM window length for extended CP for 15 kHz sub-carrier spacing

| Channel Bandwidth [MHz] | FFT size | Cyclic prefix in FFT samples | EVM window length W | Ratio of W to total CP (Note 1) [%] |
| --- | --- | --- | --- | --- |
| 1.4 | 128 | 32 | 28 | 87.5 |
| 3 | 256 | 64 | 58 | 90.6 |
| 5 | 512 | 128 | 124 | 96.9 |
| 10 | 1024 | 256 | 250 | 97.7 |
| 15 | 1536 | 384 | 378 | 98.4 |
| 20 | 2048 | 512 | 504 | 98.4 |
| Note 1: These percentages are informative. |  |  |  |  |

Table E.5.1-2a EVM window length for extended CP for 7.5 kHz sub-carrier spacing

| Channel Bandwidth [MHz] | FFT size | Cyclic prefix in FFT samples | EVM window length W | Ratio of W to total CP (Note 1) [%] |
| --- | --- | --- | --- | --- |
| 1.4 | 256 | 64 | 56 | 87.5 |
| 3 | 512 | 128 | 116 | 90.6 |
| 5 | 1024 | 256 | 248 | 96.9 |
| 10 | 2048 | 512 | 500 | 97.7 |
| 15 | 3072 | 768 | 756 | 98.4 |
| 20 | 4096 | 1024 | 1008 | 98.4 |
| Note 1: These percentages are informative. |  |  |  |  |

Table E.5.1-2b EVM window length for extended CP for 1.25 kHz sub-carrier spacing

| Channel Bandwidth [MHz] | FFT size | Cyclic prefix in FFT samples | EVM window length W | Ratio of W to total CP (Note 1) [%] |
| --- | --- | --- | --- | --- |
| 1.4 | 1536 | 384 | 336 | 87.5 |
| 3 | 3072 | 768 | 696 | 90.6 |
| 5 | 6144 | 1536 | 1488 | 96.9 |
| 10 | 12288 | 3072 | 3000 | 97.7 |
| 15 | 18432 | 4608 | 4536 | 98.4 |
| 20 | 24576 | 6144 | 6048 | 98.4 |
| Note 1: These percentages are informative. |  |  |  |  |

Table E.5.1-2c EVM window length for extended CP for 2.5 kHz sub-carrier spacing

| Channel Bandwidth [MHz] | FFT size | Cyclic prefix in FFT samples | EVM window length W | Ratio of W to total CP (Note 1) [%] |
| --- | --- | --- | --- | --- |
| 1.4 | 768 | 192 | 168 | 87.5 |
| 3 | 1536 | 384 | 348 | 90.6 |
| 5 | 3072 | 768 | 746 | 96.9 |
| 10 | 6144 | 1536 | 1500 | 97.7 |
| 15 | 9216 | 2304 | 2268 | 98.4 |
| 20 | 12288 | 3072 | 3024 | 98.4 |
| Note 1: These percentages are informative. |  |  |  |  |

Table E.5.1-2d EVM window length for extended CP for 0.37 kHz sub-carrier spacing

| Channel Bandwidth [MHz] | FFT size | Cyclic prefix in FFT samples | EVM window length W | Ratio of W to total CP (Note 1) [%] |
| --- | --- | --- | --- | --- |
| 1.4 | 5120 | 570 | 500 | 87.5 |
| 3 | 10240 | 1138 | 1032 | 90.6 |
| 5 | 20480 | 2274 | 2204 | 96.9 |
| 10 | 40960 | 4548 | 4444 | 97.7 |
| 15 | 61400 | 6816 | 6708 | 98.4 |
| 20 | 81920 | 9094 | 8950 | 98.4 |
| Note 1: These percentages are informative. |  |  |  |  |

E.6	Estimation of TX chain amplitude and frequency response parameters

The equalizer coefficients and  are determined as follows:

1.	Calculate the complex ratios (amplitude and phase) of the post-FFT acquired signal  and the post-FFT Ideal signal , for each reference symbol, over 10 subframes. This process creates a set of complex ratios:

Where the post-FFT Ideal signal  is constructed by the measuring equipment according to the relevant TX specifications, using the following parameters: restricted content: i.e. nominal Reference Symbols and the Primary Synchronisation Channel, (all other modulation symbols are set to 0 V), nominal carrier frequency,  nominal amplitude and phase for each applicable subcarrier, nominal timing.

2.	Perform time averaging at each reference signal subcarrier of the complex ratios, the time-averaging length is 10 subframes. Prior to the averaging of the phases  an unwrap operation must be performed according to the following definition: The unwrap operation corrects the radian phase angles of  by adding multiples of 2*PI when absolute phase jumps between consecutive time instances ti are greater then or equal to the jump tolerance of PI radians. This process creates an average amplitude and phase for each reference signal subcarrier (i.e. every third subcarrier with the exception of the reference subcarrier spacing across the DC subcarrier).

Where N is the number of reference symbol time-domain locations ti from Z'(f,t) for each reference signal subcarrier .

3.	The equalizer coefficients for amplitude and phase  and  at the reference signal subcarriers  are obtained by computing the moving average in the frequency domain of the time-averaged reference signal subcarriers, i.e. every third subcarrier. The moving average window size is 19. For reference subcarriers at or near the edge of the channel the window size is reduced accordingly as per figure E.6-1.

4.	Perform linear interpolation from the equalizer coefficients  and  to compute coefficients ,  for each subcarrier.

Figure E.6-1: Reference subcarrier smoothing in the frequency domain

E.7	Averaged EVM

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of 10 downlink subframes:

For FDD the averaging in the time domain equals the 10 subframe duration of the 10 subframes measurement period from the equalizer estimation step.

For TDD the averaging in the time domain can be calculated from subframes of different frames and should have a minimum of 10 subframes averaging length. TDD special fields (DwPTS and GP) are not included in the averaging.

Where Ni is the number of resource blocks with the considered modulation scheme in subframe or sTTI i and Ndl is the number of allocated downlink subframes or sTTI in one frame.

The EVM requirements shall be tested against the maximum of  the RMS average at the window W extremities of the EVM measurements:

Thus   is calculated using in the expressions above and is calculated using  in the  calculation.

Thus we get:

The averaged EVM with the minimum averaging length of at least 10 subframes is then achieved by further averaging of the  results

,

<End of change>