3GPP TSG-RAN WG4 Meeting #95-e	R4-2007764

Online, 25th May– 06th June, 2020

Title: 	Discussion on measurement related requirements for NR V2X

Source: 	Huawei, HiSilicon

Agenda item:	6.4.5.3

Document for:	Discussion

Introduction

In this contribution, we provide our discussion on measurement related issues for NR V2X.

Discussion

L1 SL-RSRP Measurements

For L1 SL-RSRP measurements, it was agreed to define the same requirements for both PSSCH-RSRP and PSCCH-RSRP and the side condition was agreed as SNR>=0dB. For the absolute accuracy of L1 SL-RSRP measurement, two options are considered as follows:

- Option 1: ±4.5dB at SNR=0dB

- Option 2: ±5dB at SNR=0dB

Based on the simulation assumptions in [1], the simulation results of PSCCH-RSRP and PSSCH-RSRP measurement accuracy are provided in [2]. According to the simulation results, it can be observed that both PSCCH-RSRP absolute accuracy and PSSCH-RSRP absolute accuracy can be within +/-2dB at SINR ≥ 0dB in both AWGN and fading channel. In NR, the RF margin in FR1 is assumed as 2.5dB. We suggest to reuse the side condition on PSSCH-RSRP measurement for LTE V2X, i.e. SINR≥0dB. Then, the PSCCH-RSRP and PSSCH-RSRP absolute accuracy can be defined as +/-4.5dB under the condition of SINR≥0dB.

Proposal 1: It is suggested to use option 1 to define the L1 SL-RSRP absolute accuracy under the condition of PSCCH/PSSCH SINR≥0dB.

PSBCH-RSRP Measurements

For an S-SSB slot, 9 symbols will be used to transmit PSBCH and PSBCH-DMRS. Except the first symbol is used for AGC settling, it is assumed that 8 PSBCH/PSBCH-DMRS symbols are available for PSBCH-RSRP measurements. However, how many PSBCH/PSBCH-DMRS symbols will be used for PSBCH-RSRP measurements is up to UE implementation.

According to the simulation assumption in [3], the simulation results of PSBCH-RSRP measurement accuracy with using 3/5/8 PSBCH-DMRS symbols are provided in [4].  According to the simulation results, it can be observed that the measurement performance degradation due to PSBCH-DMRS symbols decreasing from 8 symbols to 3 symbols is within 1dB. When 3 symbols are used for PSBCH-RSRP measurement, the PSBCH-RSRP accuracy can be within 2 dB at the SINR level of -6dB and within 1.5dB at SINR level of -3dB. Considering 2.5dB RF margin in FR1, the existing intra-frequency SS-RSRP measurement accuracy requirements in FR1 can be reused for PSBCH-RSRP measurements.

Proposal 2: The existing intra-frequency SS-RSRP measurement accuracy requirements in FR1 can be reused for defining PSBCH-RSRP measurement accuracy requirements.

Conclusions

This contribution provides our considerations on L1 SL-RSRP measurement and PSBCH-RSRP measurement requirements for NR V2X sidelink. The followings are provided:

Proposal 1: It is suggested to use option 1 to define the L1 SL-RSRP absolute accuracy under the condition of PSCCH/PSSCH SINR≥0dB.

Proposal 2: The existing intra-frequency SS-RSRP measurement accuracy requirements in FR1 can be reused for defining PSBCH-RSRP measurement accuracy requirements.

Reference

- R4-2002232, “Simulation assumption of PSSCH-RSRP and PSCCH-RSRP measurement”, LG Electronics

- R4-2004317, “Simulation results of PSSCH-RSRP and PSCCH-RSRP measurements for NR V2X”, Huawei, HiSilicon

- R4-2005317, “Simulation assumption of PSBCH-RSRP measurement”, Huawei, HiSilicon

- R4-2007763, “Simulation results of PSBCH-RSRP measurement accuracy”, Huawei, HiSilicon