3GPP TSG-RAN WG4 Meeting #95-e 	R4-2007907

Online, 25 May – 05 Jun 2020

Source: 	Huawei

Title: 	TP to TS 38.174: Rx sensitivity

Agenda Item:	6.5.2.2.3

Document for:	Approval

# Introduction

The sub-clauses in the TS have been allocated to different companies to aid the drafting process, in the last meeting a TP to the TS was submitted for TX dynamic range was submitted by ourselves. Other TP’s covering other sub-classes were also submitted by the allocated authors.

Whilst in most cases the technical content of the TP’s was not questioned the vastly different approaches to referencing meant that the consistency between the TP;’s was low and would result in a inconstant overall specification.

Whilst some ground rules for referencing were agreed in R4-2002484, they have been interpreted differently. The agreements are listed below with numbers so they can be more easily pointed to

- Decision whether to reference will be taken case by case following the rules in this slide

- The examples in this slide are meant to illustrate the meaning, not to agree exact wording to be used in the specification.

- Referencing can be done only if requirement is the same, meaning that requirement values and principles are the same

  - Exact words do not need to be same. As a theoretical example, it can be said that ”BS type 2-O requirements in sub-clause x.x.x [ref X] apply for IAB-DU”

  - Referencing shall not be done if requirements are different, i.e. value or principle differs.

  - If referencing is used all node specific text and definitions must be clarified (for example: Where ”base station RF bandwidth” is replaced by ”IAB-DU RF bandwidth”)

  - Specific references must be made to versioned documents

- Referencing is not recommended if it results in a partial requirement

  - As a theoretical example, this means a case where specification would say ” BS type 2-O requirements in sub-clause x.x.x [ref X] apply for IAB-DU. In addition IAB-DU shall meet….”

- Referencing is not recommended if a complete specification sub-clause cannot be referenced

  - No referencing when it results in formulation ”BS type 2-O requirements in clause x.x.x [ref X] except [bad requirement] will apply for IAB-DU”

  - No referencing of individual tables or figures

- Referencing sub-clauses where requirements apply for frequencies which are not IAB frequencies is TBD

  - UE and BS specification shave many requirements which are band specific and result in tables containing many bands, its not clear if these sub-clauses should be referenced. This could be similar to the ”partial requirement” bullet above

Out approach has been to avoid referencing if the meaning of the referenced text could be unclear and has resulted in a clause with most of the text copied out in full and modified.

Others have taken a different view and referenced wherever possible.

The TP for the RX sensitivity has been re-written in an attempt to use referencing more to align with some of the other contributions (although it is hoped that other contributions will also be updated so a compromise between the extreme can be found.

This TP contribute the agreed parameters to the RX sensitivity requirement.

Currently it has been agreed that the IAB-DU will use the same requirements as the BS, this has been implemented and the class 1-C node has been removed as we do not have a 1-C IAB node.

It has been agreed that the FR2 IAB-MT will use the BS method for sensitivity, however the FRC’s for the IAB-MT are not yet agreed so the exact ranges of the declarations and the specification conditions are not known. The sub-clause is added but the requirement numbers are left blank at this stage.

No agreement has been made on FR1 IAB_MT sensitivity so this is left out for now.

Where the channel BW is referred to the term [IAB-DU] channel bandwidth or [IAB-MT] channel bandwidth has been used. The IAB node part is in square brackets as it is not yet agreed if we have separate definitions for IAB-DU and IAB-MT or just a general IAB band width definition.

# TP to TS 38.174 v0.0.1

--- Start of changes ---

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 38.104 (V16.3.0): “NR; Base Station (BS) radio transmission and reception”

--- Next change ---

## 7.2	Reference sensitivity level

### 7.2.1 IAB-DU reference sensitivity level

#### 7.2.1.1	General

The reference sensitivity power level PREFSENS is the minimum mean power received at the TAB connector for IAB-DU type 1-H at which a throughput requirement shall be met for a specified reference measurement channel.

#### 7.2.1.2	Minimum requirements for IAB-DU type 1-H

The wide area IAB-DU reference sensitivity level is specified the same as the wide area BS reference sensitivity level requirement for BS type 1-H in TS 38.104x[x], subclause 6.3.2.2, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

The medium range IAB-DU reference sensitivity level is specified the same as the medium range BS reference sensitivity level requirement for BS type 1-H in TS 38.104x[x], subclause 6.3.2.2, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

The local area IAB-DU reference sensitivity level is specified the same as the local area BS reference sensitivity level requirement for BS type 1-H in TS 38.104x[x], subclause 6.3.2.2, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

Referenced requirements applying to NB IoT are not applicable to the IAB-DU

### 7.2.2 IAB-MT reference sensitivity level

Detailed structure of the subclause is TBD.

--- Next change ---

## 10.2	OTA sensitivity

### 10.2.1 IAB-DU OTA sensitivity

#### 10.2.1.1	IAB-DU type 1-H and IAB-DU type 1-O

The OTA sensitivity requirement is a directional requirement based upon the declaration of one or more OTA sensitivity direction declarations (OSDD), related to a IAB-DU type 1-H and IAB-DU type 1-O receiver.

The IAB-DU reference sensitivity level is specified the same as the BS reference sensitivity level requirement for BS in TS 38.104x[x], subclause 10.2.1, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

#### 10.2.1.2	IAB-DU type 2-O

There is no OTA sensitivity requirement for FR2, the OTA sensitivity is the same as the OTA reference sensitivity in clause 10.3.

### 10.2.2 IAB-MT OTA sensitivity

#### 10.2.2.1	IAB-MT IAB-DU type 1-H and IAB-DU type 1-O

Detailed structure of the subclause is TBD.

#### 10.2.2.2	IAB-MT type 2-O

There is no OTA sensitivity requirement for FR2, the OTA sensitivity is the same as the OTA reference sensitivity in clause 10.3.

## 10.3	OTA reference sensitivity level

### 10.3.1	General

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level for a declared OTA REFSENS RoAoA. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received at the RIB at which a reference performance requirement shall be met for a specified reference measurement channel.

The OTA REFSENS requirement shall apply to each supported polarization, under the assumption of polarization match.

### 10.3.2 	IAB-DU OTA reference sensitivity level

#### 10.3.2.1	Minimum requirement for IAB-DU type 1-O

The wide area IAB-DU reference sensitivity level is specified the same as the wide area BS reference sensitivity level requirement for BS in TS 38.104x[x], subclause 10.3.2, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

The medium range IAB-DU reference sensitivity level is specified the same as the medium range BS reference sensitivity level requirement for BS in TS 38.104x[x], subclause 10.3.2, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

The local area IAB-DU reference sensitivity level is specified the same as the local area BS reference sensitivity level requirement for BS in TS 38.104x[x], subclause 10.3.2, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

#### 10.3.2.2	Minimum requirement for IAB-DU type 2-O

The wide area IAB-DU reference sensitivity level is specified the same as the wide area BS reference sensitivity level requirement for BS in TS 38.104x[x], subclause 10.3.3, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

The medium range IAB-DU reference sensitivity level is specified the same as the medium range BS reference sensitivity level requirement for BS in TS 38.104x[x], subclause 10.3.3, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

The local area IAB-DU reference sensitivity level is specified the same as the local area BS reference sensitivity level requirement for BS in TS 38.104x[x], subclause 10.3.3, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

### 10.3.3 	IAB-MT OTA reference sensitivity level

#### 10.3.3.1	Minimum requirement for IAB-MT type 1-O

Detailed structure of the subclause is TBD

#### 10.3.3.2	Minimum requirement for IAB-MT type 2-O

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in the corresponding table and annex A.1 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

EISREFSENS levels are derived from a single declared basis level EISREFSENS_50M, which is based on a reference measurement channel with 50 MHz [IAB-MT] channel bandwidth. EISREFSENS_50M itself is not a requirement and although it is based on a reference measurement channel with 50 MHz [IAB-MT] channel bandwidth it does not imply that IAB-MT has to support 50 MHz [IAB-MT] channel bandwidth.

For Wide Area IAB-MT, EISREFSENS_50M is an integer value in the range [-96 to -119] dBm. The specific value is declared by the vendor.

For Medium Range IAB-MT, EISREFSENS_50M is an integer value in the range [-91 to -114] dBm. The specific value is declared by the vendor.

For Local Area IAB-MT, EISREFSENS_50M is an integer value in the range [-86 to -109] dBm. The specific value is declared by the vendor.

Table 10.3.3.2-1: FR2 OTA reference sensitivity requirement

| [IAB-DU] channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full [IAB-MT] channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified above. |  |  |  |

Detailed structure of the subclause is TBD.

--- End of changes ---