3GPP TSG-RAN WG4 Meeting #95-e		                               R4-2008134

Electronic Meeting, 25 May – 6 June, 2020

Source: 	Skyworks Solutions, Inc.

Title: 	[NR_n3] 40MHz REFSENS Measurements

Agenda Item:	8.19.1	UE RF (38.101-1)	[NR_n3_BW]

Document for:	Approval

# Introduction

At RAN4 #94-e, WF [1] agreed on an n3 40MHz uplink configuration to study the corresponding MSD levels.  At RAN4 #94-e-Bis, measurements could not be completed on time prior to deadline in [2]. This document provides measured PA noise emission levels and estimated Maximum Sensitivity Degradation (MSD). Reference Sensitivity Tables are proposed.

# Experimental Measurements

## Power Amplifier Calibration and Test Waveforms

A band 3 power amplifier (PA) is calibrated to meet 1 dB MPR for Power Class 3 using a 100 RB QPSK DFT-s-OFDM SCS 15kHz with RBs allocated at the channel edge. Test conditions are set as follows:

- Image and Local Oscillator (LO) rejection is set to -28 dBc;

- Counter-IM3 (C-IM3) set to -60 dBc;

- Counter-IM5 (C-IM5) set to -70 dBc;

- Test waveform for 40MHz; and

- Tx noise in Rx band power level measurements are performed with QPSK DFT-s-OFDM SCS 15 kHz waveforms. Resource Block (RB) allocation is swept from 10 RBs to 216 RBs, for which the allocated RBs are located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth.

## MSD Measurement Results

The measured TX noise levels in RX band vs UL RB allocation (LCRB) are plotted for band 3 and 40MHz CBW in Figure 1. As LCRB increases, the Tx noise in Rx band reaches a minimum level across the LCRB range 50RB to 70RBs approximately. The Tx noise level in the Rx band for the agreed UL RB allocation of 50RBs in WF [1] is -40.4 dBm. Note that at this operating point, the measured PA ACLR is -27.4 dBc on the high side, i.e. approximately 3dB above the maximum rating.

Figure 1: Measured Tx noise level in Rx band 3 for DFT-S-OFDM QPSK SCS15kHz 40MHz CBW.

Observation 1: The measured 40MHz n3 Tx noise level in the Rx band reaches a minimum across of RB size allocation “LCRB” ranging approximately from 50 to 70 RBs.

Observation 2: At the agreed “LCRB” of 50 RBs (WF[1]), the measured 40MHz n3 Tx noise level in the Rx band is -40.4 dBm.

The Minimum Tx noise level of -40.9 dBm is reached for LCRB = 64RBs. Since this is only 0.5 dB lower than the level measured at the agreed UL configuration, we confirm that 50RB UL is a valid configuration for n3 40MHz Reference Sensitivity.

Observation 3: “LCRB” of 50 RBs (WF[1]) is a valid UL configuration for n3 40MHz Reference Sensitivity.

# MSD and Reference Sensitivity Estimation

We assume:

- 50 dB Tx to RX and Tx to Antenna duplexer rejection in Rx band, as a worst case assumption;

- 10 dB Primary antenna to diversity antenna isolation;

- 4 dB Post PA loss;

- 4 dB Antenna to LNA front-end losses;

- 3 dB Antenna diversity gain;

- -1 dB SNR; and

- Previously agreed rules to compute correlated and uncorrelated MRC contributions.

Figure 2 below shows that the maximum sensitivity degradation (MSD) due to 40MHz transmitter chain excess noise in the Rx n3 band is approximately 1.5 dB.

Figure 2: n3 MSD analysis for 40 MHz CBW.

We therefore propose to define n3 REFSENS as follows in Tables 1 and 2.

Proposal 1:

Table 1: Proposed n3 Two Antenna Port Reference Sensitivity.

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n3 | 15 | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 | -86.1 |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 | -86.2 |  |  |  |  |  |  |
|  | 60 |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 | -86.3 |  |  |  |  |  |  |

Table 2: Proposed n3 Uplink Configuration for Reference Sensitivity.

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n3 | 15 | 25 | 501 | 501 | 501 | 501 | 501 | 501 |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 241 | 241 | 241 | 241 | 241 |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 | 101 |  |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, we present PA noise emission levels in the n3 receiver band at 40MHz Channel Bandwidth. We propose the following Reference Sensitivity and UL configuration tables.

Proposal 1:

Table 1: Proposed n3 Two Antenna Port Reference Sensitivity.

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n3 | 15 | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 | -86.1 |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 | -86.2 |  |  |  |  |  |  |
|  | 60 |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 | -86.3 |  |  |  |  |  |  |

Table 2: Proposed n3 Uplink Configuration for Reference Sensitivity.

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n3 | 15 | 25 | 501 | 501 | 501 | 501 | 501 | 501 |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 241 | 241 | 241 | 241 | 241 |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 | 101 |  |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# References

[1]		R4-2002855, WF on UE RF requirements for adding channel BW to band n3, Ericsson, Skyworks Solutions Inc.

[2]		R4-2004819, n3 40MHz REFSENS Measurements, Skyworks Solutions Inc.