3GPP TSG-RAN WG4 Meeting #95 												R4-2008714

Electronic Meeting, 25 May – 5 Jun, 2020

Agenda item:			6.20.2

Source:	Moderator (ZTE Corporation)

Title:	Email discussion summary for [95e][#325] 2-step RACH BS demodulation requirements

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

# Topic #1: Timing offset (TO) related issues

Main technical topic overview. The structure can be done based on sub-agenda basis.

In RAN4#94bis-e, it was agreed that RAN4 Discuss and select typical TO values associated with the selected typical UE-gNB distances for performance evaluation and requirement definition, and furthermore, it is also open whether or not TO compensation is assumed when specifying BS performance requirements. So in this topic, TO compensation and TO values are the main focus.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006540 | Intel | Proposal #2: For 15 kHz and 30 kHz SCS define 2-step RACH performance requirements with 1.92 us TO. Proposal #3: For 60 kHz and 120 kHz SCS define 2-step RACH performance requirements with 0.5 us and 0.3 us TO correspondingly. |
| R4-2006604 | Nokia | Observation 1: PRACH BS demodulation requirements in 38.141 include timing offset from 0 to 0,9 us in steps of 0,1 us. Proposal 1: RAN4 not to specify BS Demod requirements for 2-step RACH for TO values larger than the ones for the 4-step RACH. Observation 2: The parameter msgA-RSRP-Threshold can be configured in order to limit the use of the 2-step RACH procedure to UEs with small time offset. Proposal 2: Specify time offset values that are smaller than a fraction of the cyclic prefix for the tested sub-carrier spacing, as describe in the table below: SCS; Time offset (us). 30 kHz; 0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9. 120 kHz; 0, 0.1, 0.2, 0.3, 0.4. |
| R4-2006662 | ZTE | Proposal 1: For large cells, specify performance requirements with two TO values: one is less then CP duration, the other is larger than CP duration. For small cells, specify performance requirements with only one TO value. Proposal 2: TO compensation is assumed when specifying BS demodulation performance requirements for 2-step RACH. |
| R4-2007238 | Huawei | Proposal 1: Define performance requirements for the cases that obvious performance difference should be observed between performing TO compensation and not. |
| R4-2007365 | Ericsson | For first transmission: FR1: [3800, 600] ns for WA, MR (possible downselect to one). No requirement for LA (or 130ns) FR2: [670, 180]ns for WA, MR (possible downselect to one). No requirement for LA (or 70ns) For retransmissions: T0=0 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description:

According to contributions submitted to this meeting, TO compensation plays little impact on the performance if TO is less than CP length, but some performance degradation may be expected in some cases, e.g., if TO is larger than CP length and TO compensation is not performed.

Open issues and candidate options before e-meeting:

Issue 1-1: Whether or not TO compensation is assumed when specifying BS performance requirements

- Proposals

- Option 1: Yes, for all TO values selected

- Option 2: Yes, only for TO values larger than CP length if such TO values are selected

- Option 3: No

- Option 4: Yes, only for cases with larger performance difference can be observed between performing TO compensation or not.

- Recommended WF

  - Option 1?

### Sub-topic 1-2

Sub-topic description:

The choice of 0us TO may impact the choice of BS class for 2-step RACH demodulation requirements, thus an explicit decision on whether include 0us TO value may be necessary.

Open issues and candidate options before e-meeting:

Issue 1-2: Whether or not to include 0 µs in the selected TO value set?

- Proposals

  - Option 1: Yes, include 0 µs TO

  - Option 2: No, exclude 0 µs TO

- Recommended WF

  - Option 2?

### Sub-topic 1-3

Sub-topic description:

In this subtopic, only non-zero TO values are discussed. And how the selected TO values are associated with BS type/CP etc.

Open issues and candidate options before e-meeting:

Issue 1-3: Whether or not to include a TO value larger than CP length?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

Issue 1-4: For TO values less than CP length, what is recommended value(s) for FR1 WA/MR BS ?

- Proposals

  - Option 1: 1.92 µs

  - Option 2: cycling of {0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9}µs as in clause 8.4.1.4.2 of 38.141

  - Option 3: {0.6, 3.8} µs

  - Option 4: cycling of {-X, …-0.1, 0, 0.1, 0.2, 0.3, …, Y}µs, FFS X and Y

- Recommended WF

Issue 1-5: For TO values less than CP length, what is recommended value(s) for FR2 WA/MR BS ?

- Proposals

  - Option 1: {0.3,0.5} µs

  - Option 2: subset of {0.1, 0.2, 0.3, 0.4} µs

  - Option 3: {0.18, 0.67} µs

  - Option 4: cycling of {-X, …-0.1, 0, 0.1, 0.2, 0.3, …, Y}µs, FFS X and Y

- Recommended WF

Issue 1-6: If BS demodulation requirements is agreed for LA BS, what is recommended for LA BS?

- Proposals

  - Option 1: 0.13 us for FR1, 0.07 for FR2

  - Option 2: Only FR2 with 0.07 to 0.1 us

- Recommended WF

Issue 1-7: Include BS declaration whether it supports 2-step RACH with T0 larger than some threshold value ?

- Proposals

  - Option 1: Yes, discuss further threshold

  - Option 2: No

- Recommended WF

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-1: For the MR and WA cells, T0 compensation should be assumed. For UEs close to the BS (in the LA scenario), T0 compensation should not be assumed, but it will not make much performance difference whether assumed or not anyhow. Issue 1-2: For the LA BS, an appropriate and small value should be considered if a requirement is made at all; it does not have to be zero though. Issue 1-3: In our view, the T0 should be based on a consideration of the likely BS-UE distance associated with the deployment scenario, regardless of the CP length. However if preferable, there could be a declaration of the BS-UE distance a BS supports, with options to declare support only of distances within the CP length and support of longer distances. Issue 1-4: As discussed for 1-3, we believe that the requirement should cover the expected UE-BS distance, not the CP length. For 15kHz SCS and WA, 3.8usec corresponds to the largest expected distance and is within the CP length. For the 30kHz SCS, in principle the same T0 of 3.8usec should be applied, but we could consider declaration of support for a smaller number that is within the CP length. For MR the distance based T0 is 0.6us and is within the CP length. So we support option 3 in general, but could discuss further whether to merge to one value, or include a declaration based on the maximum supported UE-BS distance being greater than the CP or not. Issue 1-5: We support option 3 in general for the same reasons as issue 1-4, but could further discuss the same principles as 1-4 (i.e. declaration of whether the BS supports a distance related to the CP length or a larger distance). Issue 1-6: We support option 1, but question whether a requirement is needed for an LA BS. Issue 1-7: We are open to discuss whether, instead of considering different BS class there is a declaration of supported maximum UE distance(s); e.g. max 20m, max 100m, max 500m 2020-05-27: Issue 1-3, 1-7: @Nokia: We understand that it is possible for the BS vendor to configure an RSRP threshold and not serve UEs whose distance corresponds to T0 larger than the CP with 2-step RACH. However, it would equally be possible to not set such a threshold (or rather, set it such that T0 is greater than the CP). Our understanding is that, with proper implementation the performance does not collapse in such a case. As written, we are open to define one or two thresholds such that the BS can in effect declare whether it supports only very close UEs, or only UEs whose T0 is within the CP, or UEs further away. Nokia (27-05-2020): We understand that nothing prevents a BS not to configure an RSRP threshold and let 2-step RACH to be used overall in the cell. However, as minimum requirements, it seems reasonable to assume that a BS will set it and have only 2-step RACH on the cell center. If this TO limit is set to a large value, that could lead to a configuration set that is only feasible in a lab environment, either because the BS won’t be able to process multiple PUSCH candidates from UE with wide range of TOs, or because the BS wont be able to schedule normal PUSCH on the same slot as MsgA PUSCH. In that case, the implementation could lead to over-engineering in order to pass the test, and not necessarily resulting in a more practical product. For this reason we stated on 1-3 and 1-7 the preference to no TO over CP, and only one set of requirements, which would result in simpler requirements. If the group decides to have a TO over CP we would like to reconsider our position on issue 1-7, and have the different thresholds as in your proposal. We are open to compromise in TO > CP length in Issue 1-3 and different thresholds in Issue 1-7 if the group decides for TO > CP length. |
| Nokia, Nokia Shanghai Bell | Issue 1-1: Whether or not TO compensation is assumed when specifying BS performance requirements Option 1 – TO compensation is assumed but implementation is vendor specific. Small cell implementations might forego TOC, while wide area implementations should not do so. Issue 1-2: Whether or not to include 0 µs in the selected TO value set? Option 2 – No, exclude 0 us TO. Issue 1-3: Whether or not to include a TO value larger than CP length? Option 2 No. The BS may configure threshold to limit the use of the 2-step RACH feature only by devices in the cell centre. Therefore, as we discuss minimum requirements for the BS, it is reasonable to assume minimum requirements excluding TO values larger than the CP length. Additionally, the existing PRACH requirements for 4-step RACH were staying far within the CP length; 2-step RACH should not be tested in a more adverse setting than 4-step RACH. Issue 1-4: For TO values less than CP length, what is recommended value(s) for FR1 WA/MR BS ? Option 2: In our view it makes sense to reuse the cycling of TO values from the existing PRACH requirements. Additionally, we believe MsgA requirements should not be stricter than the existing PRACH requirements, which include TO values up to 0,9 us. The figure bellow shows the TO cycling from 38.141: Issue 1-5: For TO values less than CP length, what is recommended value(s) for FR2 WA/MR BS ? Option 4, a modified version of Option 2, cycling of {0.1, 0.2, 0.3, 0.4} µs as in clause 8.4.1.4.2 of 38.141. Issue 1-6: If BS demodulation requirements is agreed for LA BS, what is recommended for LA BS? Option 2, in our opinion it would be enough to have TO requirements for LA in FR2, since the expected TO is not large enough to cause performance degradation in FR1 LA. Issue 1-7: Include BS declaration whether it supports 2-step RACH with T0 larger than some threshold value ? Option 2: No. The demodulation performance declines quickly after the TO exceeds the CP (e.g. assuming TOC not based on preamble), which would be not a practical deployment scenario. Therefore, we would rather have only one set of requirements with TO within the CP. 27-05-2020 If the decision in Issue 1-3 is to include TO values larger than the CP length, we would prefer Option 1 in this issue. |
| ZTE | Issue 1-1 TO compensation assumed or not Option 1. Though if TO is less than CP length, TO compensation plays little impact on the performance. It makes differences for a TO larger than CP. Issue 1-2 0us TO Option 1 Yes to include. It represents an achievable limit. Issue 1-3 TO value larger than CP Option 1, better to have one value larger than CP corresponding to a scenario where UE is not located close to gNB. Issue 1-4 TO values less than CP for FR1 WA/MR BS We are fine with one or two specific TO values (Option 1 and 3 are acceptable). Issue 1-5 TO values less than CP for FR2 WA/MR BS We are fine with one or two specific TO values((Option 1 and 3 are acceptable)) Issue 1-6 TO for LA BS If 0us is excluded, then we prefer Option 1. Issue 1-7 BS declaration No, similar view as Nokia. |
| Intel | Issue 1-1: Whether or not TO compensation is assumed when specifying BS performance requirements Agree with recommended WF. TO compensation should be assumed for requirements definition and TO values should be chosen that without TO compensation enough performance degradation will be observed. Issue 1-2: Whether or not to include 0 µs in the selected TO value set? Scenarios with 0 TO value are considered for normal PUSCH requirements. The main intention of 2-step RACH performance requirements is to verify that UE can handle TO due to non-synchronized UL Tx. Prefer option 2. Issue 1-3: Whether or not to include a TO value larger than CP length? We do not think that RAN4 should differentiate larger than CP or within CP length TO should be considered and define requirements only with one TO for each configuration. Our results show that there are within CP length values which are enough sensitive to wrong TO compensation except FR1 LA BS scenario. To move forward suggest focusing on Issue 1-4, 1-5 and 1-5. Issue 1-4: For TO values less than CP length, what is recommended value(s) for FR1 WA/MR BS ? Based on our results 0.6 us TO is not enough for requirements definition since BS with wrong TO compensation will pass the test anyway. We suggest mainly focusing on TO impact sensitivity to verify proper receive processing rather than typical ISD for each BS class. TO values which will be used for requirements definition will not restrict applicable cell sizes or BS-UE distances for 2-step RACH triggering. Also, we do not think that RAN4 should align 2-step RACH and 4-step RACH TO values since the test metric and requirement motivations are completely different: MsgA demodulation performance with TO and PRACH right detection with accuracy of TO estimation. Moreover, based on our results even 0.9 us TO is not enough for 15 and 30 kHz SCS to verify proper TO compensation. Based on our evaluation results 1.92 us TO leads to enough demodulation performance loss for 15 kHz and 30 kHz SCS. Less values should not be considered since BS with wrong TO compensation will pass the test. Prefer Option 1. Issue 1-5: For TO values less than CP length, what is recommended value(s) for FR2 WA/MR BS ? Based on our results prefer option 1. Issue 1-6: If BS demodulation requirements is agreed for LA BS, what is recommended for LA BS? Agree with Nokia’s comments. Option 1 is not enough to verify proper receive processing. Prefer option 2 but also open to consider other values for FR1 and large values for FR2. Probably to move forward can keep it open on this meeting and provide further evaluations. Issue 1-7: Include BS declaration whether it supports 2-step RACH with T0 larger than some threshold value ? Prefer option 2. Such declaration is not meaningful. It is up to BS implementation which BS-UE distances to support. |
| Huawei | Issue 1-1: Whether or not TO compensation is assumed when specifying BS performance requirements Option 4. TO compensation should be assumed when specifying BS performance requirements, but we should only consider to define performance requirements for the cases that a large performance difference can be observed between performing TO compensation or not. Issue 1-2: Whether or not to include 0 µs in the selected TO value set? We prefer Option 2, i.e. exclude 0 µs TO. Issue 1-3: Whether or not to include a TO value larger than CP length? We prefer Option 2, i.e. not to include a TO value larger than CP length. Issue 1-4: For TO values less than CP length, what is recommended value(s) for FR1 WA/MR BS? We prefer using the cycling model, i.e. cycling of {-X, …-0.1, 0, 0.1, 0.2, 0.3, …, Y}µs. the specific X and Y values can be decided as per further evaluations. The test setup should be near to the real scenario as much as possible, like did for 4-step RACH in both LTE and NR Rel-15, there are lots of UE with different distributed TO. Issue 1-5: For TO values less than CP length, what is recommended value(s) for FR2 WA/MR BS? Same as Issue 1-4, we prefer Option 4, i.e. cycling of {-X, …-0.1, 0, 0.1, 0.2, 0.3, …, Y}µs, FFS X and Y. Issue 1-6: If BS demodulation requirements is agreed for LA BS, what is recommended for LA BS? Need to discussion after Issue 2-1 is determined. Issue 1-7: Include BS declaration whether it supports 2-step RACH with T0 larger than some threshold value? We prefer Option 2, i.e. like did for 4-step, only one set of requirements need to be defined with cycling TO used during the test. |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: - TO Compensation is assumed when specifying BS demodulation requirements for 2-step RACH for WA and MR BS. For LA BS, FFS whether T0 compensation is assumed (examing impact of T0 compensation first) - Excluding 0us TO Candidate options: - For TO value(s) for WA/MR BS, first decide which of the following options applied: - Option 1: Specific TO values (non-cycling), or - Option 2: Cycling TO values configuration Recommendations for 2nd round: - Further discuss whether or not to include a TO larger than CP - Further discus TO value(s) for LA BS (if requirement is created) - Further discuss TO value(s) for WA/MR BS - Further discuss whether to create 1 or 2 thresholds for T0 with declaration of whether T0 above the threshold is supported (thresholds could relate to e.g. T0 so small that no need for T0 compensation, T0 smaller than CP) |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on BS demodulation requirements for 2-step RACH | ZTE |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: Requirements for LA BS

Main technical topic overview. The structure can be done based on sub-agenda basis.

In RAN4#94bis-e, one concern was raised about whether or not to exclude LA BS for BS demodulation performance requirements for 2-step RACH.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006540 | Intel | Proposal 1: Observation 1: |
| R4-2006604 | Nokia | Observation 5: Time offset in Local Area BS is expected to be in the range of 0.2 us, which is enough to cause performance degradation with 120 kHz SCS unless DM-RS or PRACH based TOC is implemented. Proposal 6: BS demodulation requirements are needed for Local Area BS with TO < 0.2 us. |
| R4-2006662 | ZTE | Proposal 3: If a zero TO value is introduced in the performance requirements, then local area BS should be included by limiting the BS type to the requirements with zero TO, otherwise, local area BS can be excluded. |
| R4-2007238 | Huawei | Proposal 2: Exclude Local Area BS for BS demodulation requirements for 2-step RACH |
| R4-2007365 | Ericsson | Proposal 7: Further discuss whether to create a single requirement for WA and MR. For LA, preferable do not create a requirement, or create a requirement with low T0. FR1: No requirement for LA (or 130ns) FR2: No requirement for LA (or 70ns) Proposal 8: Further discuss whether as an alternative to proposal 7 to create a requirement for BS declared to support UEs greater than a defined distance from the BS. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

The TO value selection is discussed in Issue 1-6 if requirements are agreed for LA BS.

Open issues and candidate options before e-meeting:

Issue 2-1: Whether or not to exclude Local Area BS for BS demodulation requirements for 2-step RACH

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Option 1?

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Issue 2-1: We support option 2 as we do not see the need for T0 compensation and an LA requirement. An alternative is to take option 1-7 above and declare maximum distance; then discuss whether a requirement is needed or not for the smallest distance threshold. |
| Nokia, Nokia Shanghai Bell | Issue 2-1: Whether or not to exclude Local Area BS for BS demodulation requirements for 2-step RACH Option 2: No, in our opinion RAN4 should not exclude LA BS for BS demodulation requirements. There were already results from companies showing performance impact of TO ranges within what is expected for LA BS for large sub-carrier spacing. |
| ZTE | Issue 2-1 Option 2. Though LA BS may have a small TO value, the BS demodulation requirements are still needed for LA BS |
| Intel | Issue 2-1: Whether or not to exclude Local Area BS for BS demodulation requirements for 2-step RACH Prefer Option 2 at this stage. Appropriate TO values should be further investigated. Same time we do not want to define requirements with small TO which is not sensitive to wrong TO compensation implementation. |
| Huawei | Issue 2-1: Whether or not to exclude Local Area BS for BS demodulation requirements for 2-step RACH As discussed in Issues 1-4 and 1-7, define the performance requirements with cycling TO values, after the X and Y value is decided by evaluations, RAN4 can further discussion if the performance requirements are applicable for a certain BS class. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: - Further discuss the corresponding TO value for LA BS (if requirement is created) |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Merged into the WF on Topic #1 |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: Simulation setup

Main technical topic overview. The structure can be done based on sub-agenda basis.

In this topic, simulation setup not related to TO selection is discussed for specifying BS demodulation performance requirements for 2-step RACH.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006540 | Intel | Proposal #1: Define 2-step RACH performance requirements with MCS 2, 2 PRB resource allocation and TDL-A30 ns channel model Proposal #4: Specify MsgA demodulation performance with 1% BLER metric. |
| R4-2006604 | Nokia | Observation 3: The demodulation of MsgA may result in more than one intermediate error state. If MsgA PUSCH is not received, the gNB may answer with a fallbackRAR, whereas if MsgA PRACH is not detected, the gNB will not be able to demodulate MsgA PUSCH. The performance in the case of the fallbackRAR can already be certified with the PRACH requirements. Proposal 3: Use as evaluation metric the SNR at which the joint MsgA PRACH and MsgA PUSCH missed detection rate is below 10%. Fallback to 4-step procedure is considered as an error. Observation 4: Typical use cases where 2-step RACH have traffic that comprises small RRC messages of 56 bits, e.g. RRCRequest, RRCReestablishmentRequest, and RRCResumeRequest with short I-RNTI, or 72 bits, e.g. RRCResumeRequest with Long I-RNTI. Proposal 4: Define requirements and tests for MCS index 2 and transport block sizes of 56 bits to 72 bits. Proposal 5: RAN4 to consider the parameters detailed in Table 1 as a basis for discussion for simulation alignment. Parameter; Value; Value. Scenario label; FR1; FR2. Packet payload; 56 bits; 56 bits. MCS index; 2; 2. Number of PRBs nrofPRBsperMsgAPO; 4; 4. subcarrier spacing; 30 kHz; 120 kHz. PUSCH Mapping type; DMRS 1+1 Type B; DMRS 1+1 Type B. Number of symbols; 6 OFDM; 6 OFDM. msgA-PUSCH-TimeDomainAllocation (Note 1); 7; 7. Waveform; CP-OFDM; CP-OFDM. UE antenna configuration; 1Tx; 1Tx. gNB antenna configuration; 2Rx; 2 Rx. Propagation channel & UE velocity; TDL-C 300ns, 3km/h; TDL-A 30ns, 3km/h. Time offset; 0, 0.1, ..., 0.9 us; 0, 0.1, 0.2, 0.3, 0.4 us. Preamble type; A2; A2. msgA-prach-ConfigurationIndex (Note 2); 96; 38. msgA-RO-FDM; 1; 1. nrofSlotsMsgAPUSCH; 1; 1. nrofMsgAPOperSlot; 1; 1. nrMsgAPO-FDM; 3; 3. nrofDMRS-Sequences; 1; 1. number CDM group (msgAPUSCHDMRSCDMgroup); 2; 2. ports per CDM group (msgAPUSCHNrOfPort ); 1; 1. msgAPUSCH-timeDomainOffset; 5; 5. guardBandMsgAPUSCH; 1; 1. NOTE 1: msgA-PUSCH-TimeDomainAllocation is chosen from Table 6.1.2.1.1-2 on 38.214 [9], which results in a length of 6 OFDM symbols. NOTE 2: msgA-prach-ConfigurationIndex is determined such that prachTDMoccasions = 3 from 38.211 clause 6.3.3.2 [10].. |
| R4-2006662 | ZTE | Proposal 4: Choose 2 PRBs for specifying BS demodulation performance requirements for 2-step RACH. Proposal 5: Set BLER equal to 0.1 as test metric for BS demodulation performance requirements for 2-step RACH. Proposal 6: RAN4 consider Table-2 as FRC for BS demodulation performance requirements for 2-step RACH |
| R4-2006661 | ZTE | Initial simulation results. |
| R4-2007238 | Huawei | Proposal 3: Consider both PUSCH mapping Type A and Type B for performance requirements definition with test applicability rule. |
| R4-2007365 | Ericsson | Proposal 1: Preamble format and power are left to vendor choice. Proposal 2: SNR for the requirement is defined as the SNR for the msgA part. Proposal 3: MCS1 for FR1, MCS3 for FR2 (assuming other proposals on number of PRBs and symbols) Proposal 4: 2 PRBs Proposal 5: DM-RS 1+1+1 Proposal 6: Channel model is TDLC300 for FR1 and TDLA30 for FR2. Proposal 9: No retransmissions. Proposal 10: PUSCH BLER is 1% |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1

Sub-topic description:

In this sub-topic, Tx transmitter parameters are discussed.

Open issues and candidate options before e-meeting:

Issue 3-1: Preamble format for BS requirements

- Proposals

  - Option 1: A2

  - Option 2: Not limited

  - Option 3: Other

- Recommended WF

  - Option 2

Issue 3-2: Should retransmission be considered for BS demodulation requirements?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Option 2

Issue 3-3: Power offset between preamble and PUSCH

- Proposals

  - Option 1: Not specified as part of the test; left for vendor to set

  - Option 2: Other

- Recommended WF

  - Option 1

Issue 3-4: MCS level

- Proposals

  - Option 1: MCS 2

  - Option 2: MCS 1 for FR1, MCS 3 for FR2

- Recommended WF

Issue 3-5: PRB number

- Proposals

  - Option 1: 2

  - Option 2: 4

- Recommended WF

Issue 3-6: Number of symbols

- Proposals

  - Option 1: 14 for FR1, 10 for FR2

  - Option 2: 7

- Recommended WF

Issue 3-7: Mapping type

- Proposals

  - Option 1: Type A for FR1, Type B for FR2

  - Option 2: both Type A and Type B for both FR1 and FR2

  - Option 3: both Type A and Type B for FR1, Type B for FR2

- Recommended WF

  - Option 1?

Issue 3-8: SCS

- Proposals

  - Option 1: 15k and 30k SCS for FR1, 60k and 120k SCS for FR2

  - Option 2: 15k for FR1, 120k for FR2

- Recommended WF

  - Option 1

Issue 3-9: DMRS configuration

- Proposals

  - Option 1: 1+1+1

  - Option 2: 1+1

- Recommended WF

  - Option 1

Issue 3-10: TB size

- Proposals

  - Option 1: from 56 to 72 bits

  - Option 2: 56 bits

- Recommended WF

### Sub-topic 3-2

Sub-topic description :

In this sub-topic, radio channel aspects are discussed.

Open issues and candidate options before e-meeting:

Issue 3-11: Antenna configuration

- Proposals

  - Option 1: 1T2R

  - Option 2: ?

- Recommended WF

  - Option 1

Issue 3-12: Fading channel

- Proposals

  - Option 1: 3km/h, TDLC300 for FR1, TDLA30 for FR2

  - Option 2: 3km/h, TDCL300 and TDLA30 for both FR1 and FR2

- Recommended WF

  - Option 1

### Sub-topic 3-3

Sub-topic description:

In this sub-topic, performance metric and test aspects are discussed.

Open issues and candidate options before e-meeting:

Issue 3-13: test metric

- Proposals

  - Option 1: 10% BLER on msgA

  - Option 2: 1% BLER on msgA

- Recommended WF

Issue 3-14: SNR reference

- Proposals

  - Option 1: SNR on PUSCH/msgA

  - Option 2: other

- Recommended WF

  - Option 1

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Issue 3-1: We support option 2. The intention with this requirement is not to test preambles, and there is no need to restrict the 2-step RACH support to certain preambles. The power offset between PUSCH and PRACH can be set by the vendor to ensure that during testing, the preamble can be detected. Issue 3-2: We support option 2, since the intention is to speed up the RACH access Issue 3-3: We support option 1. The SNR is referenced to PUSCH. The PUSCH to preamble power offset can be set by the vendor to ensure that the preamble is always detected. Issue 3-4: We support option 2 as explained in our paper; it relates to the number of PRB though. Issue 3-5: We support option 1 as previously analysis in RAN1 suggested 2 PRB is a good optimization. Issue 3-6: We propose option 1 to align to other PUSCH demodulation. Issue 3-7: We believe option 1 is sufficient. Issue 3-8: We support option 1, but an applicability rule should be used such that only 1 SCS is tested. Issue 3-9: We support option 1 as it is the default configuration and used for msg3. Issue 3-10: One TBS is preferable; 72 bits could be the more stringent of the two. Issue 3-11: Option 1, similar to other requirements. Issue 3-12: option 1. Note that TDLC-300 is not applied for FR2 today and TDLA30 is not used for FR1 PRACH. Issue 3-13: We prefer option 2, since 1% BLER is similar to the PRACH missed detection rate. Issue 3-14: We propose option 1; the SNR is on PUSCH and the preamble power can be set differently and appropriately to the actually used preamble. 2020-05-27: The number of PRBs, number of symbols and MCS should be decided together. In our RAN1 contribution R1-1905515 we observed that 4 PRBs provides only very marginal gain over 2 PRBs. Admittedly though this was considering only FR1, 30kHz SCS and the TDLa channel so we are open to further investigation. We should also be open to the possibility that the conclusion looks different for FR1 and FR2 (or even between different SCS) Nokia: Thank you for updating our comment. From our perspective we have a preference for the 4 PRBs case. However, we won’t object if the group decides for 2 PRBs. |
| Nokia, Nokia Shanghai Bell | Issue 3-1: Preamble format for BS requirements We are ok with both option 1 and 2. Issue 3-2: Should retransmission be considered for BS demodulation requirements? Option 2 No, retransmission cases should be accounted as errors in the BS Demodulation requirements. As discussed in our contribution, it is important to exclude “half received” MsgA, where the MsgA PRACH is received correctly but MsgA PUSCH is lost. In that case the BS answers with a fallbackRAR message and the UE sends a 4-step RACH Msg3 with the contents of MsgA. However, the fallback case is already certified with the existing PRACH requirements. Issue 3-3: Power offset between preamble and PUSCH Option 1: Not specified. Not applicable in case the preamble format is not defined. Issue 3-4: MCS level Option 1, that was chosen on our contribution in order to result in 4 PRBs, so this could be adjusted to fit the number of PRBs. Issue 3-5: PRB number Option 2, 4 PRBs. We prefer more PRBs since it would result in more frequency diversity and a better channel estimate. Issue 3-6: Number of symbols Option 3 (new), 6 OFDM symbols We proposed 6 OFDM symbols in our contribution since it would result in more 4 PRBs for the MCS 2. Depends also on how many DM-RS symbols are decided. Issue 3-7: Mapping type Option 3 (new) only for the mapping type declared to be supported in D.100. If both mapping type A and type B are declared to be supported, the tests shall be done for either type A or type B. Issue 3-8: SCS Option 3 (new) 15k and 30k SCS for FR1, 60k and 120k SCS for FR2 for the SCS declared to be supported. If 2 SCSs are declared to be supported in FR1 or FR2, the tests shall be done for only one SCS. Issue 3-9: DMRS configuration Depends on the configured number of OFDM symbols and mapping type. For the configuration on our proposal Option 2 1+1 but if more OFDM symbols is agreed, we could be better to choose 1+1+1. Issue 3-10: TB size We are ok with both options. 56 or 56 to 72 bits. Issue 3-11: Antenna configuration Option 1: 1T2R Issue 3-12: Fading channel Option 1, TDLC300 for FR1 and TDLA30 for FR2 Issue 3-13: test metric Option 1, 10% BLER on MsgA, where fallback to 4-step RACH is also considered as an error. Issue 3-14: SNR reference Option 1, SNR on MsgA-PUSCH |
| ZTE | Issue 3-1 Preamble format No strong view, both Option 1 and 2 is fine. Issue 3-2 Retransmission Option 2, no retransmission is assumed Issue 3-3 Power offset between preamble and PUSCH Option 1, since the requirements is specified for the correctly detected preamble Issue 3-4 MCS level Option 2. Issue 3-5 PRB number Option 1 (2 PRBs) Issue 3-6 # of symbols Option 1 Issue 3-7 Mapping type Option 2. Both type A and B Issue 3-8 SCS Option 1. Issue 3-9 DMRS conf. Option 1. Issue 3-10 TB size Option 1. Issue 3-11 Antenna conf. Option 1. Issue 3-12 Fading channel Option 2. Similar to normal PUSCH. Issue 3-13 test metric Option 1 10% BLER for MsgA. If 1% BLER is targeted, the SNR required may be much higher to that when preamble can be correctly detected. Issue 3-14 SNR reference Option 1. |
| Intel | Issue 3-1: Preamble format for BS requirements Ok with option 2 Issue 3-2: Should retransmission be considered for BS demodulation requirements? Prefer option 2. Issue 3-3: Power offset between preamble and PUSCH Prefer option 1. Issue 3-4: MCS level Prefer to discuss after resolving issues on PRB number and TO values. Otherwise chosen configuration might be not enough sensitive to wrong TO compensation implementation. Issue 3-5: PRB number Ok with both options but slightly prefer option 2. 4 PRB will lead to more accurate TO estimation and biggest impact of TO. Issue 3-6: Number of symbols It depends on MCS and PRB number. Prefer to keep it open until agree on issues 3-4 and 3-5. Issue 3-7: Mapping type Agree with new option provided by Nokia. Issue 3-8: SCS Prefer Option 1. Issue 3-9: DMRS configuration Both options are ok for us. Issue 3-10: TB size Ok with both options. Issue 3-11: Antenna configuration Ok with option 1. Issue 3-12: Fading channel Prefer Option 1. Issue 3-13: test metric Prefer option 2. Do not clearly understand ZTE comment since in our understanding there is no problems to detect PRACH preamble on high SNR value. Same time based on our results 1% BLER is much sensitive metric to wrong TO compensation. ZTE: Our comments means the other way round. PRACH preamble can be detected at low SNR values, so if we go for 1% BLER for PUSCH/MsgA, it requires a much higher SNR than that for preamble detection. Better to avoid big power jump from preamble to MsgA. Issue 3-14: SNR reference Prefer option 1. |
| Huawei | Issue 3-1: Preamble format for BS requirements We prefer Option 2, i.e. not limited for preamble format for BS requirements. Different manufacturers may use different configurations. Issue 3-2: Should retransmission be considered for BS demodulation requirements? We prefer Option 2, i.e. no retransmission. Issue 3-3: Power offset between preamble and PUSCH We prefer Option 1, i.e. not specified as part of the test and left for vendor to set to ensure correct preamble detection. Issue 3-4: MCS level We prefer Option 1, i.e. MCS 2. Issue 3-5: PRB number We prefer Option 1, i.e. 2PRBs. Issue 3-6: Number of symbols We prefer Option 1, i.e. 14 for FR1, 10 for FR2. Issue 3-7: Mapping type We propose to follow NR Rel-15 to consider both Type A and Type B with test applicability rule for FR1, i.e. BS can be tested either PUSCH mapping type A or B based on declaration. Type B for FR2 Issue 3-8: SCS We are OK to define all SCS with test applicability rule as defined in NR Rel-15. Issue 3-9: DMRS configuration We prefer Option 2, i.e. 1+1 that is used in NR Rel-15. Issue 3-10: TB size We prefer Option 1, i.e. from 56 to 72 bits. Issue 3-11: Antenna configuration We prefer Option 1, i.e. 1T2R. Issue 3-12: Fading channel We prefer Option 1, i.e. TDLC300 for FR1, TDLA30 for FR2. Issue 3-13: test metric We prefer Option 2, i.e. 1% BLER on msgA. Issue 3-14: SNR reference We prefer SNR on PUSCH of MsgA |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: - The following setup acceptable for specifying BS demodulation requirements for 2-step RACH: - Not limiting preamble format - No re-transmission assumed - Not specify power offset between preamble and PUSCH/MsgA in the specification and allow TE vendor to decide it during the test with always correct preamble decoding - 1T2R - TDLC300-100 for FR1, TDLA30-300 for FR2 - SNR on PUSCH/MsgA - SCS: 15k and 30k for FR1, 60k and 120k for FR2, but only one SCS can be tested - DMRS configuration 1+1+1 or 1+1 based on evaluation results. - TB size 56-72 bits, subject to changes with the selection of MCS, PRB number and symbol number, assuming the least padding bits - For mapping type, we can start with type A for FR1, and type B for FR2, but leave the other options open. - BLER requires further discussion. - Consider MCS level, PRB number and symbol number together, and start with MCS1, 2 PRBs and 14 symbols for FR1, MCS 3, 2 PRBs and 10 symbols for FR2, but leaving other options open. Candidate options: Recommendations for 2nd round: - Continue discussing BLER metric - Continue discussing PUSCH mapping type |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Merged to WF on topic #1 |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #4: Miscellaneous aspects

Main technical topic overview. The structure can be done based on sub-agenda basis.

In RAN4#94bis-e, one concern was raised about whether or not to exclude LA BS for BS demodulation performance requirements for 2-step RACH.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006604 | Nokia | Observation 6: Different terminologies are being used by RAN1 and RAN2 to distinguish the normal 4-step and the new 2-step RACH procedures. RAN1 adopts the terms Type-1 and Type-2 random access procedures. RAN2 adopts the terms 4-step RA type and 2-step RA type. Observation 7: RAN4 has decided as part of the 2-step RACH RRM discussion to adopt the RAN2 terms 4-step RA type and 2-step RA type. Proposal 7: RAN4 BS demodulation to adopt the terminology adopted in RAN4 RRM and by RAN2 and distinguish the random-access procedures by calling them “4-step RA type” and “2-step RA type”. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 4-1

Sub-topic description:

For the convenience of discussing and wording for specs to differentiate 4-step RACH and 2-step RACH, a commonly used terminology may be helpful, which is the aim of this sub-topic.

Open issues and candidate options before e-meeting:

Issue 4-1: In order to facilitate the discussion and probably for specs wording, an aligned terminology to differentiate 4-step RACH and 2-step RACH is helpful. Which of the following options is recommended for BS demodulation requirements?

- Proposals

  - Option 1: “4-step RA type” and “2-step RA type”, aligned with RAN2 and RRM

  - Option 2: “Type 1 RA procedure” and “Type-2 RA procedure”, aligned with RAN1

  - Option 3: other options not precluded

- Recommended WF

  - Option 1

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 4-1: Others: |
| Nokia, Nokia Shanghai Bell | Issue 4-1: In order to facilitate the discussion and probably for specs wording, an aligned terminology to differentiate 4-step RACH and 2-step RACH is helpful. Which of the following options is recommended for BS demodulation requirements? Option 1, this option was already aligned in the NR_2step_RACH_RRM discussion in the last RAN4 #94 Bis-e meeting. Additionally, this is also what RAN2 is adopting in e.g. 38.300, 38.311, 38.321. |
| ZTE | Option 1. Aligned with RRM session and RAN2. |
| Intel | Issue 4-1: In order to facilitate the discussion and probably for specs wording, an aligned terminology to differentiate 4-step RACH and 2-step RACH is helpful. Which of the following options is recommended for BS demodulation requirements? Prefer option 1. Also, from whole specification perspective it will be helpful to ask RAN1 to align their terminology too. |
| Huawei | We prefer Option 1, i.e. “4-step RA type” and “2-step RA type”, aligned with RAN2 and RRM |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: - Unanimous agreement on the terminology “4-step RA type” and “2-step RA type” aligned with RRM session and RAN2 Candidate options: Recommendations for 2nd round: - Capture the agreement in the WF |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Merge to the WF on topic #1 |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |