3GPP TSG-RAN WG4 Meeting #95-e 	R4-2008768

Online, 25 May - 05 Jun 2020

Source: 	Huawei

Title: 	TP to TR 38.809: Correction of IAB-DU and IAB-MT permutation in subclause 4.8

Agenda Item:	6.5.2.2.3

Document for:	Approval

# Introduction

During the discussion on the NR-IAB_General [309] subject area, for sub topic 1-2 it was agreed that flexibility to have different reference points should be allowed.

It was agreed a TP would be drafted to correct the statement in TR 38.809 clause 4.2 so that its clear the reference points of IAB-DU and IAB-MT do not need to be the same

This text proposal corrects the text in question.

# TP to TR 38.809 v0.2.0

--- Start of changes ---

## 4.2	RF Requirements reference points

The requirement reference points for BS is defined as below:

Table 4.2-1: BS requirement sets

| BS type / Requirement set | BS Description | Additional information |
| --- | --- | --- |
| 1-H | A BS operating at FR1 with a requirement set holding requirements defined at the TAB and OTA requirements defined at RIB. | The requirement set is like the one defined for Hybrid AAS BS. Following the approach used in 3GPP TS 37.105 |
| 1-O | A BS operating at FR1 with a requirement set consisting only OTA requirements defined at the RIB. | Following the approach developed in eAAS and documented in 3GPP TR 37.843. |
| 2-O | A BS operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB. | This requirement set is relevant for AAS BS and does not require access to RF connectors. |

The test and reference point of RF requirement for BS should be reused and thus the test spec will not be impacted.

The IAB-Du and IAB_MT may share the same hardware implementation, of so the reference points for the IAB-DU and the IAB-MT may be different.

Table 4.2-2: IAB requirement sets

| IAB type / Requirement set | IAB Description | Additional information |
| --- | --- | --- |
| 1-H | IAB-MT and IAB-DU operating at FR1 with a requirement set holding requirements defined at the respective TAB and OTA requirements defined at the respective RIB | The requirement set for both IAB-DU and IAB-MT. |
| 1-O | IAB-MT and IAB-DU operating at FR1 with a requirement set consisting only of OTA requirements defined at the respective RIB. | The requirement set for both IAB-DU and IAB-MT. |
| 2-O | IAB-MT and IAB-DU operating at FR2 with a requirement set consisting only of OTA requirements defined at the respective RIB. | The requirement set for both IAB-DU and IAB-MT. |

--- End of changes ---