3GPP TSG-RAN WG4 Meeting # 95-e 														R4-2008795 Electronic Meeting, 25 May – 5 June, 2020

Agenda Item:	6.5.2.2.2

Source: 	Ericsson

Title:	TP to TS 38.174: OTA ACS

Document for:	Approval

- Introduction

After RAN4#92bis the IAB TS is email approved. Section 2 provides text proposal on 10 OTA ACS for the IAB TS 38.174 (Integrated access and backhaul radio transmission and reception) based on WF [1]

# Text Proposal

Text changes compared to the BS specification original text is tracked to ease the company review.  A revision will be needed for the final approval TP.

During the 1st round discussion, there are comments received as below:

| R4-2007581 | Nokia: To our understanding the interferer offset cannot be directly re-used when the PRB number of interfering signal is changed. In 38.104 the channel spacing is optimized so that the frequency separation between wanted and interfering signal is the worst case and now with wider interfering signal wider spacing should be used. It seems change marks are not on top of current specification. |
| --- | --- |

--------------------------------------------------Start of TP------------------------------------------------------

## 10.5	OTA in-band selectivity and blocking

### 10.5.1	OTA adjacent channel selectivity

#### 10.5.1.1	General

OTA Adjacent channel selectivity (ACS) is a measure of the receiver’s ability to receive an OTA wanted signal at its assigned channel frequency in the presence of an OTA adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

#### 10.5.1.2	Minimum requirement for IAB-DU type 1-O

#### Minimum requirement is the same as specified for BS type 1-O in TS38.104[2], subclause 10.5.1.2.

#### 10.5.1.3	Minimum requirement for IAB-DU type 2-O

#### Minimum requirement is the same as specified for BS type 2-O in TS38.104[2], subclause 10.5.1.3.

#### 10.5.1.4	Minimum requirement for IAB-MT type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the -OTA REFSENS RoAoA.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR2, the OTA wanted and the interfering signal are specified in table 10.5.1.4-1 and table [ TBD] for ACS. The reference measurement channel for the OTA wanted signal is further specified in annex [ A.1]. The characteristics of the interfering signal is further specified in annex [ D].

The OTA ACS requirement is applicable outside the IAB-MT [ RF Bandwidth]. The OTA interfering signal offset is defined relative to the IAB-MT [ RF Bandwidth] edges.

For Wide Area IAB-MT, for RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA ACS requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table [ TBD]. The OTA interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

Table 10.5.1.4-1: OTA ACS requirement for Wide Area  and Local Area IAB MT

| IAB-MT channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 50, 100, 200, 400 | EISREFSENS + 6 dB (Note 3) | EISREFSENS_50M + 27.7 + ΔFR2_REFSENS (Note 1) EISREFSENS_50M + 26.7 + ΔFR2_REFSENS (Note 2) |
| NOTE 1: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz NOTE 2: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz NOTE 3: EISREFSENS is given in subclause [ 10.3.3] |  |  |

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-2005493, WF on IAB-MT In-band selectivity, Ericsson