3GPP TSG-RAN4 WG4 Meeting # 95-e	R4-2008842

# Electronic Meeting, 25 May - 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR: Test structure for NR PDSCH FR1 CA 4Rx performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-Perf |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Revised Rel-16 NR performance requirements enhancement WI RP-200472 is approved in RAN#87-e meeting. NR CA PDSCH normal demodulation requirements for NR CA are agreed to be defined for the following CA configs: - FDD CA with 15kHz SCS - TDD CA - 30kHz SCS + 30kHz SCS - 15kHz SCS + 30kHz SCS - TDD FDD CA - FDD 15kHz SCS + TDD 15kHz SCS - FDD 15kHz SCS + TDD 30kHz SCS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the test structure for the NR Rel-16 PDSCH normal demodulation requirements for 4Rx FR1 CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

<< Start of Changes >>

5.2	PDSCH demodulation requirements

The parameters specified in Table 5.2-1 are valid for all PDSCH tests unless otherwise stated.

<<Unchanged Sections Skipped>>

5.2A	PDSCH demodulation requirements for CA

The parameters specified in Table 5.2-1 for PDSCH single carrier tests are reused for PDSCH CA test unless otherwise stated.

5.2A.1	1RX requirements

(Void)

5.2A.2	2RX requirements

5.2A.3	4RX requirements

5.2A.3.1	Minimum requirements

For CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.3.1-6 based on the single carrier requirements for different bandwidth specified in Table 5.2A.3.1-3 ~ Table 5.2A.3.1-5, with the parameters in Table 5.2A.3.1-1 and the downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-cluase do not apply for UE single carrier test.

Table 5.2A.3.1-1: Test parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | FDD: 12 TDD: Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | As defined in Table 5.2A.3.1-2 |
| TDD UL-DL pattern |  |  | 15kHz SCS: FR1.15-1 30kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | TBD |

Table 5.2A.3.1-2: Test parameters for number of HARQ processes

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 8 |
|  | TDD PCell | 8 | 8 |
| FDD 15 kHz + TDD 15 kHz CA | FDD PCell | 4 | 4 |
|  | TDD PCell | 8 | 8 |
| TDD 15 kHz + TDD 30 kHz CA | 15kHz PCell | 8 | 12 (Note 1) |
|  | 30kHz PCell | 8 | TBD |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | 4 | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 8 | N/A |
| Note 1: FFS scheduling details |  |  |  |

Table 5.2A.3.1-3: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 10 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 15 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 20 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 25 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 30 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 40 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 50 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |

Table 5.2A.3.1-4 Single carrier performance for TDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 10 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 15 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 20 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 25 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 30 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 40 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 50 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |

Table 5.2A.3.1-5 Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 10 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 15 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 20 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 25 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 30 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 40 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 50 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 60 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 80 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 90 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 100 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |

Table 5.2A.3.1-6: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.3.1-3 |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.1-5 |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.1-3 and Table 5.2A.3.1-5 per CC |
| 4 | FDD 15 kHz + TDD 15 kHz | As defined in Table 5.2A.3.1-3 and Table 5.2A.3.1-4 per CC |
| 5 | TDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.1-4 and Table 5.2A.3.1-5 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.x. |  |  |

<< End of Changes >>