3GPP TR 38.820 V1.4.0 (2020-06)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

7 - 24 GHz frequency range

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	8

2	References	8

3	Definitions, symbols and abbreviations	12

3.1	Definitions	12

3.2	Symbols	12

3.3	Abbreviations	13

4	Regulatory landscape survey	15

4.1	General	15

4.2	ITU Region 1	18

4.2.1	Europe and CEPT	18

4.3	ITU Region 2	23

4.3.1	USA	23

4.4	WRC-19 outcomes	28

4.4.1	 New agenda items for WRC-23	28

4.4.2	 IMT for fixed wireless broadband in fixed services bands	28

5	Study on general aspects	30

5.1	General	30

5.2	Selection of frequency sub-ranges and example frequencies	30

5.3	Comparison of FR1 and FR2 characteristics	30

5.3.1	General	30

5.3.1.1	RAN1 specification impacts	30

5.3.1.2	RAN2 specification impacts	31

5.3.1.3	RAN4 specification impacts	31

5.3.1.3.1	General	31

5.3.1.3.2	BS RF	31

5.3.1.3.3	UE RF	31

5.3.1.3.4	RRM	32

5.3.2	Transmitter architecture	32

5.4	RF technology considerations in the 7 – 24 GHz range	34

5.4.1	PA trends	34

5.4.1.1	Saturated peak output power	35

5.4.1.2	Power added efficiency	37

5.4.2	PA power scaling and AAS dependencies	38

5.4.3	PA Output power, ACLR and PAE dependencies	39

5.5	Key parameters over the 7 – 24 GHz range	43

5.5.1	Noise figure	43

5.5.1.1	General aspects for noise figure	43

5.5.1.2	NF analysis for NR BS	43

5.5.1.3	NF analysis for NR UE	44

5.5.1.3.1	General	44

5.5.1.3.2	Frequency sub-range 1	45

5.5.1.3.3	Frequency sub-range 2	45

5.5.1.3.4	Frequency sub-range 3	46

5.5.2	Power amplifiers	47

5.5.3	Phase noise	47

5.5.3.1	 General	47

5.5.3.2	LO generation and distribution	48

5.5.3.3	Performance of reference source	48

5.5.3.4	Noise floor	49

5.5.4	Correlation of noise and emissions	50

5.6	 Deployment scenarios	50

5.6.1	Indoor hotspot	51

5.6.2	Dense urban	51

5.6.3	Urban macro	51

5.6.4	IAB	51

5.6.5	Summary	51

5.7	 NR system parameters analysis for 7 – 24 GHz range	53

5.7.1	General	53

5.7.2	SCS	54

5.7.2.1	General	54

5.7.2.2	SCS for SSB	54

5.7.2.3	SCS for shared / control channels	54

5.7.3	Channel bandwidth	54

5.7.4	Spectrum utilization	54

5.7.5	Channel raster	54

5.7.6	Sync raster	54

5.7.7	Channel spacing	55

5.7.8	Aggregated system bandwidth	55

5.7.8.1	 Single carrier operation	55

5.7.8.2	 Carrier aggregation	55

5.7.8.3	 Dual connectivity	56

6	NR UE	57

6.1	NR UE architecture	57

6.1.1	Mixed UE transceiver architecture	57

6.1.2	Frequency sub-range 1	57

6.1.3	Frequency sub-range 3	58

6.2	NR UE requirements classification	58

6.3	UE RF requirements	59

6.3.1	UE RF technology aspects	59

6.3.1.1	Antenna technology	59

6.3.1.1.1	Beamforming	59

6.3.1.1.2	MIMO	60

6.3.1.2	Propagation aspect for UE	61

6.3.1.2.1	Indoor propagation at 15 GHz	61

6.3.1.2.2	Outdoor propagation at 15 GHz	62

6.3.1.3	RF front end technology	65

6.3.1.3.1	Active technologies	65

6.3.1.3.2	Passive technologies	67

6.3.1.4	Testability considerations	68

7	NR BS	70

7.1	General	70

7.2	BS architecture and requirements classification	70

7.2.1	Reference architecture	70

7.2.2	Requirement sets	71

7.2.3	Antenna topologies	72

7.2.4	Array antenna model	73

7.3	BS classes	75

7.4	BS RF requirements	76

7.4.1	Transmitter requirements	76

7.4.1.1	Tx requirements overview	76

7.4.1.2	Output power accuracy	80

7.4.1.2.1	General	80

7.4.1.2.2	Conducted output power accuracy	81

7.4.1.2.3	TRP accuracy	81

7.4.1.2.4	EIRP accuracy	82

7.4.1.3	 Output power dynamics	82

7.4.1.3.1	 Conducted RE power control dynamic range	82

7.4.1.3.2	 OTA RE power control dynamic range	82

7.4.1.3.3	 Total power dynamic range	83

7.4.1.3.4	 OTA total power dynamic range	83

7.4.1.4	 Transmit ON/OFF power	83

7.4.1.5	Transmitted signal quality	84

7.4.1.5.1	Frequency error	84

7.4.1.5.2	EVM	84

7.4.1.5.3	TAE	84

7.4.1.6	ACLR	84

7.4.1.7	Operating Bands Unwanted emissions (OBUE)	85

7.4.1.7.1 	General	85

7.4.1.7.2 	OBUE limits in FR1 and FR2	86

7.4.1.7.3	 OBUE limits in 7 – 24 GHz	87

7.4.1.8	Spurious emissions	88

7.4.1.8.1	General	88

7.4.1.8.2	International regulation (ITU-R)	88

7.4.1.8.3	European regulation (CEPT)	88

7.4.1.8.4	Other regional regulation	90

7.4.1.8.5	Possible new limits in 7 – 24 GHz	90

7.4.1.9	TX IMD	90

7.4.1.10	Co-location requirements	91

7.4.1.10.1	General	91

7.4.1.10.2	Co-location emissions	91

7.4.1.11	Co-existence requirements	92

7.4.1.11.1	General	92

7.4.1.11.2	Background	92

7.4.1.11.3	Co-existence emissions	93

7.4.2	Receiver requirements	93

7.4.2.1	Rx requirements overview	93

7.4.2.2	Receiver sensitivity	96

7.4.2.2.1	Background	96

7.4.2.2.2	Conducted sensitivity	96

7.4.2.2.3	OTA sensitivity	97

7.4.2.2.4	OTA REFSENS	97

7.4.2.2.5	7 - 24 GHz	97

7.4.2.3	Dynamic range	98

7.4.2.3.1	 General	98

7.4.2.3.2	 Conducted dynamic range	98

7.4.2.3.3	 OTA dynamic range	98

7.4.2.4	In-band blocking	98

7.4.2.4.1	Background	98

7.4.2.4.2	7 - 24 GHz	99

7.4.2.5	Out-of-band blocking requirements	99

7.4.2.5.1	General	99

7.4.2.5.2	Background	99

7.4.2.5.3	Technical considerations for 7 to 24 GHz	101

7.4.2.5.4	Out-of-band co-location receiver blocking	101

7.4.2.6	Receiver spurious emissions	102

7.4.2.7	 In-channel selectivity	102

7.4.2.7.1 	In-channel selectivity for FR1 and FR2	102

7.4.2.7.2	 In-channel selectivity for 7 - 24 GHz	103

7.4.2.9	RX IMD	104

7.4.2.9.1 	RX IMD for FR1 and FR2	104

7.4.2.9.2 	RX IMD for 7 - 24 GHz	104

7.5	BS EMC requirements	105

7.5.1	General	105

7.5.2	Emission requirements	105

7.5.2.1	 Conducted requirements	105

7.5.2.2	 Radiated emission requirements	105

7.5.3	Immunity requirements	106

7.5.3.1	 Common EMC immunity requirements	106

7.5.3.2	 Radiated immunity	106

Annex A (informative): Example filter characteristics	107

A.1	20 GHz example frequency filtering	107

A.1.1	General	107

A.1.2	LTCC example filter	107

A.1.3	PCB integrated example filter	108

A.2	15 GHz example frequency filtering	110

A.3	10 GHz example frequency filtering	110

Annex B (informative): Frequency ranges of interest in 7 – 24 GHz	113

Annex C: Change history	115

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for the study item on 7.125 – 24.250 GHz frequency range, covering the regulatory framework study, general RF aspects, as well as BS- and UE-specific aspects.

During the NR study item in Rel-14, frequency range of 6 – 24 GHz was discussed as part of the RAN WG4 studies. This frequency range was included within the Rel-14 study item objectives and the operational requirements for Next Generation access technologies described in TR 38.913 [2].

As the frequencies below 7.125 GHz and above 24.25 GHz (i.e. up to 52.6 GHz) were already addressed in Rel-15 NR specifications, the discussion on the 7 - 24 GHz intermediate range was triggered at later stage.

NOTE 1:	During Rel-15 work on NR frequency bands, the FR1 frequency range was initially defined up to 6 GHz. At later stage of Rel-15, the FR1 was agreed at RAN#82 meeting to be extended up to 7.125 GHz in order to incorporate the NR-U work and potential licensed operation.

NOTE 2:	For simplicity purposes, whenever the term “7 – 24 GHz” will be used in this technical report, the frequency range of 7.125 – 24.250 GHz shall be considered.

This TR intends to provide information on the feasibility of NR operation in 7 – 24 GHz frequency range for set of potential deployment scenarios, considering BS RF and UE RF technology capabilities taking into account possible ranges for key requirements.

This TR does not intend to provide specific recommendations on NR system parameter values to be defined, nor specific RF or EMC requirement values to be defined for UE or BS operation in this frequency range, as those aspects are considered to be covered by the follow-up WI.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.913: “Study on Scenarios and Requirements for Next Generation Access Technologies”

[3]	3GPP TS 38.113: “NR; Base Station (BS) ElectroMagnetic Compatibility (EMC)”

[4]	3GPP TR 38.817-02: “NR; General aspects for Base Station (BS) Radio Frequency (RF) for NR”

[5]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception"

[6]	3GPP TS 38.141-2: "NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing"

[7]	Recommendation ITU-R SM.329-12: "Unwanted emissions in the spurious domain"

[8]	IEC 61000-4-3: 2006+AMD1:2007+AMD2:2010 CSV: "Electromagnetic compatibility (EMC) - Part 4-3: Testing and measurement techniques - Radiated, radio-frequency, electromagnetic field immunity test"

[9]	Hua Wang et al., "Power Amplifiers Performance Survey 2000-Present," 2008, Available: https://gems.ece.gatech.edu/PA_survey.html

[10]	3GPP TS 37.105: “Active Antenna System (AAS) Base Station (BS) transmission and reception”

[11]	Hua Wang, Fei Wang, Huy Thong Nguyen, Sensen Li, Tzu-Yuan Huang, Amr S. Ahmed, Michael Edward Duffy Smith, Naga Sasikanth Mannem, Jeongseok Lee, Edgar Garay, Sanghoon Lee, and David Munzer, "Power Amplifiers Performance Survey 2000-Present," 2020.02, https://gems.ece.gatech.edu/PA_survey.html

[12]	Code of Federal Regulations, Title 47: Telecommunication; Part 2—Frequency Allocations and Radio Treaty Matters; General Rules and Regulations, https://www.ecfr.gov/cgi-bin/text-idx?SID=983c3dd433919e69fce5a8bd4b565cfb&mc=true&node=pt47.1.2&rgn=div5

[13]	3GPP TS 38.211: “NR; Physical channels and modulation”

[14]	3GPP TS 38.213: “NR; Physical layer procedures for control”

[15]	3GPP TS 38.214: “NR; Physical layer procedures for data”

[16]	3GPP TS 38.141-1: “NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing”

[17]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[18]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[19]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

[20]	3GPP TS 38.133: “NR; Requirements for support of radio resource management”

[21]	R4-1907593: “LS on UE feature list for NR”, RAN4#91

[22]	3GPP TR 25.942: “Radio Frequency (RF) system scenarios”

[23]	The European table of frequency allocations and applications in the frequency range 8.3 kHz to 3000 GHz, ECO, https://www.ecodocdb.dk/document/593

[24]	3GPP TS 36.104: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”

[25]	3GPP TS 37.104: “NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception”

[26]	Kambiz Hadipour, Andrea Ghilioni1, Andrea Mazzanti1, Matteo Bassi1, Francesco Svelto, “A 40GHz to 67GHz Bandwidth 23dB Gain 5.8dB Maximum NF mm-Wave LNA in 28nm CMOS”, 2015 IEEE Radio Frequency Integrated Circuits Symposium

[27]	Domenico Pepe, Domenico Zito, “32 dB Gain 28 nm Bulk CMOS W-Band LNA”, IEEE Microwave and Wireless Components Letters, Vol. 25, No. 1, January 2015

[28]	Domenico Pepe1, Domenico Zito, “72 GHz CMOS LNA with Transformer-based Input Integrated Matching”, IEEE 2015

[29]	Hossein Noori, Miles Sanner, Naveen Yanduru, “A 0.8 dB NF, 4.6 dBm IIP3, 1.8 - 2.2 GHz, Low-Power LNA in 130 nm RF SOI CMOS Technology”, IEEE 2015

[30]	Joost Melai, Peter Magnée, Ivo Pouwel, Pieter Weijs, Ihor Brunets, Rob van Dalen, Anurag Vohra, Luuk Tiemeijer, Ralf Pijper, Hans Tuinhout, Nicole Wils, Nicolae Cazana, “QUBiC generation 9, a new BiCMOS process optimized for mmWave applications”, IEEE 2015

[31]	Cristina Andrei, Olof Bengtsson, Ralf Doerner, Serguei A. Chevtchenko, Wolfgang Heinrich, Matthias Rudolph, “Dynamic behaviour of a Low-Noise Amplifier GaN MMIC under input power overdrive”, Proceedings of the 45th European Microwave Conference

[32]	Recommendation ERC/REC 70-03 "Relating to the use of Short Range Devices (SRD)"

[33]	ECC, ECC Recommendation (02)/05 "Unwanted Emissions"

[34]	ERC Recommendation 74-01 "Unwanted emissions in the spurious domain" (amended 29 May 2019)

[35]		LMX2594, 15 GHz Wideband RF Synthesizer, Texas Instruments. http://www.ti.com/product/LMX2594

[36]		ADF41513 26.5 GHz, Integer N/Fractional-N, PLL Synthesizer, Analog Devices, https://www.analog.com/en/products/adf41513.html

[37] 		Staffan Ek et al., A 28-nm FD-SOI 115-fs Jitter PLL-Based LO System for 24-30-GHz Sliding-IF 5G Transceivers, IEEE Journal of Solid-State Circuits (Volume: 53, Issue: 7 , July 2018)

[38]	K. Raczkowski et. al. “A 9.2–12.7 GHz Wideband Fractional-N Subsampling PLL in 28 nm CMOS With 280 fs RMS Jitter”, IEEE Journal of Solid-State Circuits, vol. 50, no. 5, pp. 1203-1213, May 2015

[39]	Ximenes, G. Vlachogiannakis, R. Staszewski, “An Ultracompact 9.4–14.8-GHz Transformer-Based Fractional-N All-Digital PLL in 40-nm CMOS”, IEEE Transactions on Microwave Theory and Techniques, vol. 65, no. 11, pp. 4241-4254, Nov. 2017

[40]	N. Markulic et. al. “A Self-Calibrated 10Mb/s Phase Modulator with -37.4dB EVM Based on a 10.1-to-12.4GHz, -246.6dB-FOM, Fractional-N Subsampling PLL”, IEEE International Solid-State Circuits Conference 2016, pp. 176-177

[41]	R4-1700305, "[DRAFT] LS on Characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz"

[42]	Recommendation ITU-R SM.328-11 (2006), “Spectra and bandwidth of emissions”

[43]	Recommendation ITU-R M.1580-5 (02/2014), “Generic unwanted emission characteristics of base stations using the terrestrial radio interfaces of IMT-2000”

[44]	Recommendation ITU-R M.2070-1 (02/2017), “Generic unwanted emission characteristic of base stations using the terrestrial radio interfaces of IMT-Advanced”

[45]	ECC Recommendation (02)05, “Unwanted emissions” (Amended 30 March 2012)

[46]	ETSI TR 101 854: “Fixed Radio Systems; Point-to-point equipment; Derivation of receiver interference parameters useful for planning fixed service point-to-point systems operating different equipment classes and/or capacities”

[47]	P. Ökvist, H. Asplund, A. Simonsson, B. Halvarsson, J. Medbo and N. Seifi, "15 GHz propagation properties assessed with 5G radio access prototype," 2015 IEEE 26th Annual International Symposium on Personal, Indoor, and Mobile Radio Communications (PIMRC), Hong Kong, 2015, pp. 2220-2224.

[48]	K. Zhao et al., "Channel Characteristics and User Body Effects in an Outdoor Urban Scenario at 15 and 28 GHz," in IEEE Transactions on Antennas and Propagation, vol. 65, no. 12, pp. 6534-6548, Dec. 2017.

[49]	R4-1609122, NR RF parameters and template for WP5D, RAN4 #81

[50]	R4-168770, Way forward on UE and BS estimated NF for mm-waves and ITU-R related work, RAN4 #80bis

[51]	R4-1904127, 7-24GHz discuss implications of FR1 and FR2 architectures, RAN4 #90bis

[52]	R4-165001, UE reference architecture and other considerations at millimeter wave, RAN4 #80

[53]	R4-168076, NR UE system Noise Figure proposal, RAN4 #80bis

[54]	R4-1905846, [7-24GHz] Applicable FR1 UE Technologies, RAN4 #91

[55]	RP-170021, Reply LS to ITU-R WP5D/374 (Attachment 4.13) on Characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz, RAN #75

[56]	PTA(19)087, New Resolution IMT below 24GHz, ETNO

[57]	PTA(19)073, GSMA_Input to PTA on AI 10, GSMA

[58]	APT19-4, 	4th	/ Final African Preparatory Meeting for WRC-19 (APM19-4), report, 26-30.08.2019

[59]	World Radiocommunication Conference 2019 (WRC-19), Provisional Final Acts, ITU https://www.itu.int/dms_pub/itu-r/opb/act/R-ACT-WRC.13-2019-PDF-E.pdf

[60]		3GPP TS 25.104: “Base Station (BS) radio transmission and reception (FDD)”

[61]		3GPP TS 25.951: “FDD Base Station (BS) classification”

[62]		S. Iguchi et. al. “Variation-Tolerant Quick-Start-Up CMOS Crystal Oscillator With Chirp Injection and Negative Resistance Booster”, IEEE Journal of Solid-State Circuits, vol. 51, no. 2, pp. 496-508, Feb. 2016

[63]	Y. Rajavi et. al. “A 48-MHz Differential Crystal Oscillator With 168-fs Jitter in 28-nm CMOS”, IEEE Journal of Solid-State Circuits, vol. 52, no. 10, pp. 2735-2745, Oct. 2017

[64]	S. Igushi, T. Sakurai, M. Takamiya, “A Low-Power CMOS Crystal Oscillator Using a Stacked-Amplifier Architecture”, IEEE Journal of Solid-State Circuits, vol. 52, no. 11, pp. 3006-3017, Nov. 2017

[65]	RP-193226, Integrated access and backhaul for NR, WID, RAN#86

[66]	Recommendation ITU-R M.819-2: “International Mobile Telecommunications-2000 (Imt-2000) For Developing Countries”, 1997

[67]	W. L. Stutzman, "Estimating directivity and gain of antennas," in IEEE Antennas and Propagation Magazine, vol. 40, no. 4, pp. 7-11, Aug. 1998, doi: 10.1109/74.730532, URL: http://ieeexplore.ieee.org/stamp/stamp.jsp?tp=&arnumber=730532&isnumber=15753

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

BS type 1-C:	NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB

BS type 2-O:	NR base station operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

BS type xFR-C:	NR base station operating in the 7 – 24 GHz range with requirements set consisting only of conducted requirements defined at individual antenna connectors where xFR represents the frequency range, is band specific and could be 1, 2 or another number.

BS type xFR-H:	NR base station operating in the 7 – 24 GHz range with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB where xFR represents the frequency range, is band specific and could be 1, 2 or another number.

BS type xFR-O:	NR base station operating in the 7 - 24 GHz range with a requirement set consisting only of OTA requirements defined at the RIB where xFR represents the frequency range, is band specific and could be 1, 2 or another number.

FR1: frequency range defined between 410 MHz to 7125 MHz

FR2: frequency range defined between 24250 MHz to 52600 MHz

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

AA	Composite antenna array pattern

AE	Array element pattern

Am	Front-to-back ratio

BWChannel	BS channel bandwidth

BWSSB				Bandwidth of the SSB

Coff 	Isolation in OFF state

DE,max 	Array element peak directivity

dh	Horizontal elements separation in antenna array

dv	Vertical elements separation in antenna array

Fmax		Measure of the achievable power gain

Ft	Cut-off frequency

GE,max	Array element peak gain

IMF 	Industrial Margin

LE	Array element loss

M	Number of rows of the antenna array

N	Number of columns of the antenna array

Psat	Saturated output power

RON 	Losses in ON state

SLAv	Side-lobe suppression

xFR	Integer representing the BS frequency range

	Horizontal angle (defined between -180° and 180°)

3dB	Horizontal half power beam width in degrees

	Vertical angle of the signal direction (defined between -0° and 180°; 90° represents the direction perpendicular to the antenna array)

3dB	Vertical half power beam width in degrees

etilt	Electrical down-tilt angle

escan	Electrical scan angle

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AA	Antenna Array

AAS BS	Active Antenna System Base Station

AlN	Aluminium Nitride

BAW	Bulk Acoustic Wave

CA	Carrier Aggregation

CMOS	Complementary Metal Oxide Semiconductor

DFE	Digital Front End

CPE	Customer Premises Equipment

DC	Dual Connectivity

EESS 	Earth Exploration Satellite Service

FR	Frequency Range

FS 	Fixed Services

GaAs	Gallium Arsenide

GaN	Gallium Nitride

HBT	Heterojunction Bipolar Transistor

IAB	Integrated Access and Backhaul

IM	Implementation Margin

InP	Indium Phosphide

IPD 	Integrated Passive Device

LDMOS 	Laterally-Diffused Metal-Oxide Semiconductor

LTCC	Low Temperature Co-Fired Ceramic

METSAT	Meteorological Satellite

MIM	Metal Insulator Metal

MMSS	Maritime Mobile Satellite Service

MR	Medium Range (BS)

MSS	Mobile Satellite Services

NR		New Radio

OTA	Over The Air

PAE	Power Added Efficiency

pHEMT	Pseudomorphic High Electron Mobility Transistor

PT-RS 	Phase Tracking Reference Signal

PVT	Process-Voltage-Temperature

RAS	Radio Astronomy Service

RDN 	RF Distribution Network

RFFE	RF Front-end

RIB	Radiated Interface Boundary

RoAoA	Range of Angles of Arrival

SiGe	Silicon Germanium

SCS	Subcarrier Spacing

SOI	Silicon On Insulator

SRS 	Space Research Service

SSB	Synchronization Signal Block

SU	Spectrum Utilization

TAB	Transceiver Array Boundary

TP 	Transmit Point

TRXUA	Transceiver Unit Array

UWB	Ultra-wideband

WA	Wide Area (BS)

# 4	Regulatory landscape survey

## 4.1	General

For the summary of spectrum within the 7 – 24 GHz range which was identified by the interested operators, industry and regional regulatory bodies for potential use for IMT purposes, refer to annex B.

For the summary of WRC-19 conclusions related to the IMT in 7 – 24 GHz frequency range and applicable deployment scenarios, refer to clause 4.4.

The regulatory landscape survey for the frequency range from 7.125 to 24.25 GHz is provided in this clause. Tables 4.1-1 and 4.1-2 below summarize the radio regulation for regions 1, 2, and 3.

Table 4.1-1: Frequency bands in the range 7 000 to 10 000 MHz in radio regulation

| Frequency band (MHz) | Allocated to Mobile Service on a primary basis | Allocated to Fixed Service on a primary basis | Comments |
| --- | --- | --- | --- |
| 7 145 – 7 190 | Yes | Yes | EESS (passive) and SRS (passive), footnote 5.458. SRS (deep space) (Earth-to-space). EESS (passive) and SRS (passive), footnote 5.458. |
| 7 190 – 7 235 | Yes | Yes | EESS (passive) and SRS (passive) (Earth-to-space), footnote 5.458. Footnote 5.460A – EESS space stations shall not claim protection from existing and future stations in the fixed and mobile services. |
| 7 235 – 7 250 | Yes | Yes | EESS (passive) (Earth-to-space) and SRS (passive), footnote 5.458. Footnote 5.460A – EESS space stations shall not claim protection from existing and future stations in the fixed and mobile services. |
| 7 250 – 7 300 | Yes | Yes |  |
| 7 300 – 7 375 | Yes | Yes | Footnote 5.461AB MMSS shall not claim protection from, nor constrain the use and development of, stations in the fixed and mobile, except aeronautical mobile services. |
| 7 375 – 7 450 | Yes | Yes | Footnote 5.461AB MMSS shall not claim protection from, nor constrain the use and development of, stations in the fixed and mobile, except aeronautical mobile services. |
| 7 450 – 7 550 | Yes | Yes | METSAT (space-Earth) Footnote 5.461AB MMSS shall not claim protection from, nor constrain the use and development of, stations in the fixed and mobile, except aeronautical mobile services. |
| 7 550 – 7 750 | Yes | Yes | Footnote 5.461AB MMSS shall not claim protection from, nor constrain the use and development of, stations in the fixed and mobile, except aeronautical mobile services. |
| 7 750 – 7 900 | Yes | Yes | METSAT (Space-to-Earth) |
| 7 900 – 8 025 | Yes | Yes |  |
| 8 025 – 8 175 | Yes | Yes | EESS (space-to-Earth). Footnote 5.463 Aircraft stations are not permitted in 8 025-8 400 MHz. |
| 8 175 – 8 215 | Yes | Yes | EESS (space-to-Earth), METSAT (Earth-to-space) Footnote 5.463 Aircraft stations are not permitted in 8 025-8 400 MHz. |
| 8 215 – 8 400 | Yes | Yes | EESS (space-to-Earth) Footnote 5.463 Aircraft stations are not permitted in 8 025-8 400 MHz. |
| 8 400 – 8 500 | Yes | Yes | SRS (space-to-Earth), footnote 5.465 SRS is limited to deep space. |
| 8 500 – 8 550 | No | No | Radiolocation |
| 8 550 – 8 650 | No | No | EESS (active), Radiolocation, SRS (active) |
| 8 650 – 8 750 | No | No | Radiolocation |
| 8 750 – 8 850 | No | No | Radiolocation and Aeronautical Radionavigation |
| 8 850 – 9 000 | No | No | Radiolocation and Maritime Radionavigation |
| 9 000 – 9 200 | No | No | Radiolocation and Aeronautical Radionavigation |
| 9 200 – 9 300 | No | No | EESS (active), Radiolocation, Maritime Radionavigation, footnote 5.474 search and rescue transponders may be used. |
| 9 300 – 9 500 | No | No | EESS (active), Radiolocation, Radionavigation, SRS (active), footnote 5.474 search and rescue transponders may be used. |
| 9 500 – 9 800 | No | No | EESS (active), Radiolocation, Radionavigation, SRS (active) |
| 9 800 – 9 900 | No | No | Radiolocation, Fixed service secondary |
| 9 900 – 10 000 | No | No | EESS (active), Radiolocation. Fixed service secondary, footnote 5.478B EESS (active) shall not cause harmful interference to, nor claim protection from, stations in the fixed service. |

Table 4.1-2: Frequency bands in the range 10 to 24 GHz in radio regulation

| Frequency band (GHz) | Allocated to Mobile Service on a primary basis | Allocated to Fixed Service on a primary basis | Comments |
| --- | --- | --- | --- |
| 10 – 10.4 | Yes, excluding R2 | Yes, excluding R2 | EESS (active), Radiolocation, footnote 5.478B EESS (active) shall not cause harmful interference to, nor claim protection from, stations in the fixed service. Also used by weather radar on a secondary basis, footnote 5.479 |
| 10.4 – 10.45 | Yes, excluding R2 | Yes, excluding R2 | Radiolocation |
| 10.45 – 10.5 | No | No | Radiolocation, amateur and amateur-satellite secondary |
| 10.5 – 10.55 | Yes | Yes | Radiolocation secondary R1 |
| 10.55 – 10.6 | Yes | Yes | Radiolocation secondary |
| 10.6 – 10.68 | Yes | Yes | EESS (passive), RAS (footnote 5.149), SRS (passive), Radiolocation secondary. Footnote 5.482A EESS (passive) and mobile and fixed services, Res. 751 applies |
| 10.68 – 10.7 | No | No | EESS (passive), RAS, SRS (passive) Footnote 5.340 – all emissions prohibited |
| 10.7 – 10.95 | Yes | Yes | Footnote 5.441 FSS Appendix 30B band |
| 10.95 – 11.2 | Yes | Yes | Footnote 5.848B Res. 155 applies – unmanned aircraft systems using FSS |
| 11.2 – 11.45 | Yes | Yes | Footnote 5.441 FSS Appendix 30B |
| 11.45 – 11.7 | Yes | Yes | Footnote 5.484B Res. 155 applies – unmanned aircraft systems using FSS. |
| 11.7 – 12.5 | Yes, except 11.7-12.2 GHz in R2 | Yes, except 12.1-12.2 in R2 | BS and BSS Footnotes 4.587, 5.488, 5.490, and 5.493 denotes Appendix 30 and 30B planned bands |
| 12.5 – 12.75 | Yes, except R1 | Yes, except R1 | R3 BSS Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS. |
| 12.75 – 13.25 | Yes | Yes | SRS (deep space) (space-to-Earth) secondary Footnote 5.441 FSS Appendix 30B |
| 13.25 – 13.4 | No | No | EESS (active), ARNS, SRS (active) |
| 13.4 – 13.65 | No | No | EESS (active), Radiolocation, SRS, Standard frequency and time signal-satellite (Earth-to-space) |
| 13.65 – 13.75 | No | No | EESS (active), Radiolocation, SRS (active), standard frequency and time signal-satellite (Earth-to-space) secondary |
| 13.75 – 14 | No | No | Radiolocation, standard frequency and time signal-satellite (Earth-space) |
| 14 – 14.25 | No | No | Radionavigation Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS |
| 14.25 – 14.3 | No | No | Radionavigation Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS |
| 14.3 – 14.4 | Yes, except R2 | Yes, except R2 | Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS |
| 14.4 – 14.47 | Yes | Yes | Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS |
| 14.47 – 14.5 | Yes | Yes | Radio astronomy – Footnote 5.149 Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS |
| 14.5 – 14.75 | Yes | Yes | Res 163 and 164 restriction to FSS to protect aeronautical mobile |
| 14.75 – 14.8 | Yes | Yes | Res 163 and 164 restriction to FSS to protect aeronautical mobile |
| 14.8 – 15.35 | Yes | Yes | Footnote 5.339 also allocated to EESS and SRS (passive) secondary |
| 15.35 – 15.4 | No | No | EESS (passive), Radio Astronomy, SRS (passive) Footnote 5.340 – all emission prohibited |
| 15.4 – 15.43 | No | No | Radiolocation and radionavigation |
| 15.43 – 15.63 | No | No | Radiolocation and radionavigation |
| 15.63 – 15.7 | No | No | Radiolocation and ARNS |
| 15.7 – 16.6 | No | No | Radiolocation Footnote 5.513 – mobile and fixed primary, but subject to not claiming protection from or causing interference to services operating in accordance to the Table. |
| 16.6 – 17.1 | No | No | Radiolocation, SRS (deep space) (Earth-to-space) Footnote 5.513 – mobile and fixed primary, but subject to not claiming protection from or causing interference to services operating in accordance to the Table. |
| 17.1 – 17.2 | No | No | Radiolocation Footnote 5.513 – mobile and fixed primary, but subject to not claiming protection from or causing interference to services operating in accordance to the Table. |
| 17.2 – 17.3 | No | No | EESS (active), Radiolocation, SRS (active) Footnote 5.513 – mobile and fixed primary, but subject to not claiming protection from or causing interference to services operating in accordance to the Table. |
| 17.3 – 17.7 | No | No | Footnote 5.515 Appendix 30A Footnote 5.516B High density FSS applications (in Region 1) |
| 17.7 – 18.1 | Yes, excluding Region 2 (17.7-17.8) | Yes | Footnote 5.515 Appendix 30A (Region 2 17.7-17.8) |
| 18.1 – 18.4 | Yes | Yes | Footnote 5.516B High density applications in FSS (in Region 2) |
| 18.4 – 18.6 | Yes | Yes | Footnote 5.516B High density applications in FSS (in Region 2) |
| 18.6 – 18.8 | Yes | Yes | EESS (passive), SRS (passive) Footnote 5.516B High density applications in FSS (in Region 2) |
| 18.8 – 19.3 | Yes | Yes | 5.516B High density FSS applications (in Region 2) |
| 19.3 – 19.7 | Yes | Yes |  |
| 19.7 – 20.1 | No | No | 5.484B Res 155 applies – unmanned aircraft systems using FSS 5.516B High density FSS applications |
| 20.1 – 20.2 | No | No | 5.484B Res 155 applies – unmanned aircraft systems using FSS 5.516B High density FSS applications |
| 20.2 – 21.2 | No | No | Standard frequency and time signal-satellite (space-to-Earth) |
| 21.2 – 21.4 | Yes | Yes | EESS (passive) and SRS (passive) |
| 21.4 – 22 | Yes | Yes | BSS in Regions 1 and 3 Footnote 5.530A any station in the fixed or mobile services shall not produce a pfd in excess of -120.4 dB(W/m2 • MHz) at 3m above the ground for more than 20% of the time. (see Rec. ITU-R BO.1898) Footnote 5.530B Admins in R1 and R3 are encouraged no to deploy stations in the mobile service and are encouraged to limit the deployment of FS ptp links |
| 22 – 22.21 | Yes | Yes | Footnote 5.149 Radio astronomy |
| 22.21 – 22.5 | Yes | Yes | EESS (passive), Radio astronomy, SRS (passive), Footnote 5.149 Footnote 5.532 EESS and SRS (passive) shall not impose constraints on the FS and MS |
| 22.5 – 22.55 | Yes | Yes | Not applicable |
| 22.55 – 23.15 | Yes | Yes | SRS (Earth-to-space) Footnote 5.149 Radio astronomy Footnote 5.338A – Res 750 applies (passive emission limits) |
| 23.15 – 23.55 | Yes | Yes | Footnote 5.338A – Res 750 applies (passive emission limits) |
| 23.55 – 23.6 | Yes | Yes | WRC-19 Agenda item 1.13 determining IMT adjacent emissions limits for protection of passive services in the 23.6-24 GHz result would most likely be applicable to this band. |
| 23.6 – 24 | No | No | EESS (passive), Radio astronomy, SRS (passive) Footnote 5.340 all emissions prohibited – unwanted emissions from IMT is under review WRC-19 Agenda item 1.13 |

## 4.2	ITU Region 1

### 4.2.1	Europe and CEPT

General information for usages in CEPT can be found in the European table of frequency allocations and applications in the frequency range 8.3 kHz to 3000 GHz [23]. When it comes to Regulation about Short Range Devices additional information can be found in Recommendation ERC/REC 70-03 “Relating to the use of Short Range Devices (SRD)”(*) [32]. Where a corresponding EC decision exists it is marked with (+).  Unwanted emission limits can be found in ECC Recommendation (02)/05 “Unwanted Emissions” [33] and ERC/RECOMMENDATION 74-01 “Unwanted emissions in the spurious domain” [34].

Table 4.2.1-1: Frequency bands in the range 7 – 24 GHz

| Frequency range (MHz) | Allocations / applications | ECC decision / recommendation |
| --- | --- | --- |
| 7 145 – 7 190 | Fixed PMSE Radiodetermination applications UWB | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 7 190 – 7 235 | Fixed PMSE EESS (passive) Radiodetermination applications UWB | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 7 235 – 7 250 | Fixed PMSE EESS (passive) Radiodetermination applications UWB | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 7 250 – 7 450 | Fixed PMSE Land Mobile (military) (1) Radiodetermination applications UWB Satellite Systems (military) (1) MSS Earth stations | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 7 450 – 7 550 | Fixed PMSE Land Mobile (military) (1) Radiodetermination applications UWB Satellite Systems (military) (1) Weather satellites | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 7 550 – 7 750 | Fixed PMSE Land Mobile (military) (1) Radiodetermination applications UWB Satellite Systems (military) (1) | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 7 750 – 7 900 | Fixed PMSE Radiodetermination applications UWB Weather satellites | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 7 900 – 8 025 | Fixed PMSE Land Mobile (military) (1) Radiodetermination applications UWB Satellite Systems (military) (1) MSS Earth stations | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 8 025 – 8 215 | EESS (space-Earth) Fixed PMSE Land Mobile (military) (1) Land Mobile Radiodetermination applications UWB Satellite Systems (military) (1) | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 8 215 – 8 400 | EESS (space-Earth) Fixed PMSE Land Mobile (military) (1) Radiodetermination applications UWB Satellite Systems (military) (1) Radio Astronomy | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 8400 – 8500 | Fixed PMSE Radiodetermination applications UWB Space Research | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 8 500 – 8 550 | Aeronautical military systems (1) Aeronautical navigation Radiodetermination applications UWB Radiolocation (civil) Radiolocation (military) (1) | *+ ECC/DEC/(06)04+ |
| 8 550 – 8 650 | Active sensors (satellite) Aeronautical military systems (1) Aeronautical navigation Radiodetermination applications UWB Radiolocation (civil) Radiolocation (military) (1) | *+ ECC/DEC/(06)04+ |
| 8 650 – 9 000 | Aeronautical military systems (1) Aeronautical navigation Radiodetermination applications UWB Radiolocation (civil) Radiolocation (military) (1) | *+ ECC/DEC/(06)04+ |
| 9 000 – 9 200 | Aeronautical military systems (1) Aeronautical navigation Radiodetermination applications Radiolocation (civil) Radiolocation (military) (1) | *+ |
| 9 200 – 9 300 | Aeronautical military systems (1) Aeronautical navigation Radiodetermination applications Radiolocation (civil) Radiolocation (military) (1) Synthetic aperture radar | *+ |
| 9 300 – 9 500 | Aeronautical military systems (1) Aeronautical navigation Radiodetermination applications Radiolocation (civil) Radiolocation (military) (1) Satellite systems (military) (1) Weather radar | *+ |
| 9 500 – 9 800 | Active sensors (satellite) Aeronautical military systems (1) Aeronautical navigation Radiodetermination applications Radiolocation (civil) Radiolocation (military) (1) Satellite systems (military) (1) | *+ |
| 9 800 – 9 900 | Aeronautical military systems (1) Aeronautical navigation Radiodetermination applications Radiolocation (civil) Radiolocation (military) (1) Satellite systems (military) (1) | *+ |
| 9 900 – 10 000 | Aeronautical military systems (1) Aeronautical navigation Radiodetermination applications Radiolocation (civil) Radiolocation (military) (1) Satellite systems (military) (1) Synthetic aperture radar | *+ |
| 10 000 – 10 400 | Aeronautical military systems (1) Amateur FWA Fixed Land Mobile (military) (1) Maritime Mobile (military) (1) PMSE Radiodetermination applications Radiolocation (civil) Radiolocation (military) (1) Synthetic aperture radar | ERC/REC 12-05 ERC/REC 25-10 *+ |
| 10 400 – 10 450 | Aeronautical military systems (1) Amateur Land Mobile (military) (1) Maritime Mobile (military) (1) PMSE Radiodetermination applications Radiolocation (civil) Radiolocation (military) (1) | ERC/REC 25-10 *+ |
| 10 400 – 10 500 | Aeronautical military systems (1) Amateur Amateur-satellite Land Mobile (military) (1) Maritime Mobile (military) (1) PMSE Radiodetermination applications Radiolocation (civil) Radiolocation (military) (1) | ERC/REC 25-10 *+ |
| 10 500 – 10 600 | Fixed PMSE Radiodetermination applications | ERC/REC 12-05 ERC/REC 25-10 *+ |
| 10 600 – 10 680 | Fixed PMSE Passive sensors (satellite) Radio astronomy | ERC/REC 12-05 ERC/REC 25-10 ECC/DEC/(10)01 ECC/DEC/(10)01 |
| 10 680 – 10 700 | Passive Sensors Radio astronomy |  |
| 10 700 – 10 950 | FSS Fixed | AES ECC/DEC/(05)11 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 ESV ECC/DEC/(05)10 Earth Stations ERC/DEC/(00)08 HEST ECC/DEC/(06)03 LEST ECC/DEC/(06)02 NGSO ECC Decision (18)05 NGSO ECC/DEC/(17)04 ERC/REC 12-06 |
| 10 950 – 11 700 | FSS Fixed | AES ECC/DEC/(05)11 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 ESV ECC/DEC/(05)10 NGSO ECC Decision (18)05 NGSO ECC/DEC/(17)04 ERC/REC 12-06 |
| 11 700 – 12 500 | Broadcast satellite FSS | ERC/DEC/(00)08 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 HEST ECC/DEC/(06)03 LEST ECC/DEC/(06)02 NGSO ECC Decision (18)05 NGSO ECC/DEC/(17)04 |
| 12 500 – 12 750 | FSS | AES ECC/DEC/(05)11 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 ESV ECC/DEC/(05)10 Earth Stations ERC/DEC/(00)08 HEST ECC/DEC/(06)03 LEST ECC/DEC/(06)02 NGSO ECC Decision (18)05 NGSO ECC/DEC/(17)04 |
| 12 750 – 13 250 | FSS Fixed | ERC/REC 12-02 |
| 13 250 – 13 400 | Active sensors (satellite) Airborne Doppler navigation aids Maritime radar |  |
| 13 400 – 13 650 | Active sensors (satellite) Airborne Doppler navigation aids Maritime radar FSS Radiodetermination applications Radiolocation (military) (1) | *+ |
| 13 650 – 13 750 | Active sensors (satellite) Airborne Doppler navigation aids Maritime radar Radiodetermination applications Radiolocation (military) (1) | * |
| 13 750 – 14 000 | FSS Passive sensors (satellite) Maritime radar Radiodetermination applications Radiolocation (military) (1) | * |
| 14 000 – 14 250 | FSS | AES ECC/DEC/(05)11 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 ESV ECC/DEC/(05)10 Earth Stations ERC/DEC/(00)08 HEST ECC/DEC/(06)03 LEST ECC/DEC/(06)02 NGSO ECC Decision (18)05 NGSO ECC/DEC/(17)04 |
| 14 250 – 14 300 | FSS MSS Earth stations | AES ECC/DEC/(05)11 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 ESV ECC/DEC/(05)10 NGSO ECC/DEC/(17)04 VSAT ERC/REC 13-03 |
| 14 300 – 14 470 | FSS MSS Earth stations | AES ECC/DEC/(05)11 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 ESV ECC/DEC/(05)10 NGSO ECC/DEC/(17)04 VSAT ERC/REC 13-03 |
| 14 470 – 14 500 | FSS MSS Earth stations Radio astronomy | AES ECC/DEC/(05)11 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 ESV ECC/DEC/(05)10 NGSO ECC/DEC/(17)04 VSAT ERC/REC 13-03 |
| 14 500 – 14 750 | Aeronautical military systems (1) Fixed Land military systems related (1) Maritime military systems related (1) Radio astronomy | ERC/REC 12-07 |
| 14 750 – 14 800 | Aeronautical military systems (1) Land military systems related (1) Maritime military systems related (1) Radio astronomy |  |
| 14 800 – 15 350 | Aeronautical military systems (1) Fixed Land military systems related (1) Maritime military systems related (1) Radio astronomy | ERC/REC 12-07 |
| 15 350 – 15 400 | Passive sensors (satellite) Radio astronomy |  |
| 15 400 – 15 430 | Airborne Doppler navigation aids Radiolocation (civil) |  |
| 15 430 – 15 630 | Airborne Doppler navigation aids FSS Radiolocation (civil) |  |
| 15 630 – 15 700 | Airborne Doppler navigation aids Radiolocation (civil) |  |
| 15 700 – 17 100 | Radiolocation (military) (1) |  |
| 17 100 – 17 300 | Ground Based SAR Radiolocation (military) (1) | * |
| 17 300 – 17 700 | FSS Radiolocation (military) (1) | GSO ESOMP ECC/DEC/(13)01 NGSO ESOMP ECC/DEC/(15)04 Earth Stations ECC/DEC/(05)08 |
| 17 700 – 18 600 | Fixed FSS | ERC/DEC/(00)07 ERC/REC 12-03 Earth Stations ERC/DEC/(00)07 GSO ESOMP ECC/DEC/(13)01 NGSO ESOMP ECC/DEC/(15)04 |
| 18 600 – 18 800 | Fixed FSS Passive sensors (passive) | ERC/DEC/(00)07 ERC/REC 12-03 Earth Stations ERC/DEC/(00)07 GSO ESOMP ECC/DEC/(13)01 NGSO ESOMP ECC/DEC/(15)04 |
| 18 800 – 19 700 | Fixed FSS | ERC/DEC/(00)07 ERC/REC 12-03 Earth Stations ERC/DEC/(00)07 GSO ESOMP ECC/DEC/(13)01 NGSO ESOMP ECC/DEC/(15)04 |
| 19 700 – 20 200 | FSS MSS Earth stations | Earth Stations ERC/DEC/(00)07 GSO ESOMP ECC/DEC/(13)01 NGSO ESOMP ECC/DEC/(15)04 HEST ECC/DEC/(06)03 LEST ECC/DEC/(06)02 |
| 20 200 – 21 200 | MSS Earth stations Satellite systems (military) (1) |  |
| 21 200 – 21 400 | PMSE | ERC/REC 25-10 |
| 21 400 – 22 000 | Broadcasting (satellite) PMSE | ERC/REC 25-10 |
| 22 000 – 23 150 | Fixed PMSE Radio astronomy | T/R 13-02 ERC/REC 25-10 |
| 23 150 – 23 600 | Fixed PMSE | T/R 13-02 ERC/REC 25-10 |
| 23 600 – 24 000 | Passive sensors (satellite) Radio astronomy |  |
| NOTE: Refer to NATO NJFA Extract for information about harmonised military bands. |  |  |

## 4.3	ITU Region 2

### 4.3.1	USA

FCC rules part(s) and additional requirements are rules and regulations that pertain to specified frequency ranges and detailed description of the rules and regulations are listed in FCC code of federal regulations Title 47 Part 2 [12].

Table 4.3.1-1: USA spectrum allocation between 7 GHz and 24 GHz

| Frequency (GHz) | Allocations | Governing FCC Rule Part(s) |
| --- | --- | --- |
| 7.125 – 7.145 | FIXED | General Rules and Regulations (2.105- G116) RF Devices (15) |
| 7.145 – 7.19 | FIXED SPACE RESEARCH (deep space) (passive) | General Rules and Regulations (2.105- G116, US262, 5.548) RF Devices (15) |
| 7.19 – 7.235 | FIXED SPACE RESEARCH (Earth-to-space) | General Rules and Regulations (2.105 –G113, G134, US262, 5.458) RF Devices (15) |
| 7.235 – 7.25 | FIXED | General Rules and Regulations (2.105 – 5.458) RF Devices (15) |
| 7.25 – 7.3 | FIXED-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) Fixed | General Rules and Regulations (2.105 –G117) |
| 7.3 – 7.45 | FIXED FIXED-SATELLITE (space-to-Earth) Mobile-satellite (space-to-Earth) | General Rules and Regulations (2.105 – G117) |
| 7.45 – 7.55 | FIXED FISED-SATELLITE (space-to-Earth) METEROLOGICAL-SATELLITE (space-to-Earth) Mobile-satellite (space-to-Earth) | General Rules and Regulations (2.105 –G105, G117) |
| 7.55 – 7.75 | FIXED FIXED-SATELLITE (space-to-Earth) Mobile-satellite (space-to-Earth) | General Rules and Regulations (2.105 – G117) |
| 7.75 – 7.9 | FIXED METEROLOGICAL-SATELLITE (space-to-Earth) | General Rules and Regulations (2.105 – 5.461B) |
| 7.9 – 8.025 | FIXED-SATELLITE (Earth-to-space) MOBILE-SATELLITE (Earth-to-space) Fixed | General Rules and Regulations (2.105 – G117) |
| 8.025 – 8.175 | EARTH EXPLORATION-SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) Mobile-satellite (Earth-to-space) (no airborne transmissions) | General Rules and Regulations (2.105 –G117, US258) |
| 8.175 – 8.215 | EARTH EXPLORATION-SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) METEROLOGICAL-SATELLITE (Earth-to-space) Mobile-satellite (Earth-to-space) (no airborne transmissions) | General Rules and Regulations (2.105 – G104, G117, US258) |
| 8.215 – 8.4 | EARTH EXPLORATION-SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) Mobile-satellite (Earth-to-space) (no airborne transmissions) | General Rules and Regulations (2.105 – G117, US258) |
| 8.4 – 8.45 | FIXED SPACE RESEARCH (deep space) (space-to-Earth) | General Rules and Regulations (2.105) |
| 8.45 – 8.5 | FIXED SPACE RESEARCH (space-to-Earth) | General Rules and Regulations (2.105) |
| 8.5 – 8.55 | RADIOLOCATION | General Rules and Regulations (2.105 –G59) Private Land Mobile (90) |
| 8.55 – 8.65 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) | General Rules and Regulations (2.105 –G59) Private Land Mobile (90) |
| 8.65 – 9 | RADIOLOCATION | General Rules and Regulations (2.105 –US53) Aviation (87) Private Land Mobile (90) |
| 9 – 9.2 | AERONAUTICAL RADIONAVIGATION RADIOLOCATION | General Rules and Regulations (2.105 –G2, G19, 5.337, 5.473A) Aviation (87) Private Land Mobile (90) |
| 9.2 – 9.3 | MARITIME RADIONAVIGATION Radiolocation | General Rules and Regulations (2.105 – G59, US 110, 5.472, 5.474) Maritime (80) Private Land Mobile (90) |
| 9.3 – 9.5 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION RADIONAVIGATION SPACE RESEARCH (active) Meteorological aids | General Rules and Regulations (2.105 –G56, US475, US71, US67, US476A, 5.427, 5.474, 5.475A, 5.475B) Maritime (80) Aviation (87) Private Land Mobile (90) |
| 9.5 – 9.8 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) | General Rules and Regulations (2.105) Private Land Mobile (90) |
| 9.8 – 9.9 | RADIOLOCATION Earth exploration-satellite (active) Space research (active) | General Rules and Regulations (2.105) Private Land Mobile (90) |
| 9.1 – 10 | RADIOLOCATION | General Rules and Regulations (2.105, 5.479) Private Land Mobile (90) |
| 10 – 10.5 | RADIOLOCATION Amateur Amateur-satellite (10.45-10.5) | General Rules and Regulations (2.105, G32, NG50, US108, US128, 5.479) Private Land Mobile (90) Amateur Radio (97) |
| 10.5 – 10.55 | RADIOLOCATION | General Rules and Regulations (2.105, US59) Private Land Mobile (90) |
| 10.55 – 10.6 | FIXED | FIXED MICROWAVE (101) |
| 10.6 – 10.68 | EARTH EXPLORATION-SATELLITE (passive) FIXED SPACE RESEARCH (passive) | General Rules and Regulations (2.105, US120, US131, US482) FIXED MICROWAVE (101) |
| 10.68 – 10.7 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | General Rules and Regulations (2.105, US74, US131, US246) |
| 10.7 – 11.7 | FIXED FIXED-SATELITE (space-to-Earth) | General Rules and Regulations (2.105, US131, US211, NG52, 5.441) Satellite Communications (25) Fixed Microwave (101) |
| 11.7 – 12.2 | FIXED-SATELLITE (space-to-Earth) | General Rules and Regulations (2.105, NG55, NG143, 5.458, 5.488) Satellite Communications (25) |
| 12.2 – 12.7 | FIXED BROADCASTING-SATELLITE | General Rules and Regulations (2.105, 5.487A, 5.488, 5.490) Satellite Communications (25) Fixed Microwave (101) |
| 12.7 – 12.75 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE | General Rules and Regulations (2.105, NG118) TV Broadcast Auxiliary (74F) Cable TV Relay (78) Fixed Microwave (101) |
| 12.75 – 13.25 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE | General Rules and Regulations (2.105, NG52, NG53, NG57, NG118, US251, 5.441) Satellite Communications (25) TV Broadcast Auxiliary (74F) Cable TV Relay (78) Fixed Microwave (101) |
| 13.25 – 13.4 | EARTH EXPLORATION-SATELLITE (active) AERONAUTICAL RADIONAVIGATION SPACE RESEARCH (active) | General Rules and Regulations (2.105, 5.497, 5.498A) Aviation (87) |
| 13.4 – 13.75 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH Standard frequency and time signal-satellite (Earth-to-space) | General Rules and Regulations (2.105, G59, 5.501A, 5.501B) Private Land Mobile (90) |
| 13.75 – 14 | RADIOLOCATION FIXED-SATELLITE (Earth-to-space) Standard frequency and time signal-satellite (Earth-to-space) Space research | General Rules and Regulations (2.105, G59, US337, US356, US357) Satellite Communications (25) Private Land Mobile (90) |
| 14 – 14.2 | FIXED-SATELLITE (Earth-to-space) Mobile-satellite (Earth-to-space) Space research | General Rules and Regulations (2.105, NG55, US133) Satellite Communications (25) |
| 14.2 – 14.4 | FIXED-SATELLITE (Earth-to-space) Mobile-satellite (Earth-to-space) | General Rules and Regulations (2.105, NG55) Satellite Communications (25) |
| 14.4 – 14.47 | FIXED-SATELLITE (Earth-to-space) Mobile-satellite (Earth-to-space) Fixed Mobile | General Rules and Regulations (2.105, NG55) Satellite Communications (25) |
| 14.47 – 14.5 | FIXED-SATELLITE (Earth-to-space) Mobile-satellite (Earth-to-space) Fixed Mobile | General Rules and Regulations (2.105, NG55, US115, US133, US342) Satellite Communications (25) |
| 14.5 – 14.7145 | FIXED Mobile Space research | General Rules and Regulations (2.105) |
| 14.7.145 – 14.8 | FIXED Mobile Space research | General Rules and Regulations (2.105) |
| 14.8 – 15.1365 | MOBILE SPACE RESEARCH Fixed | General Rules and Regulations (2.105, US310) |
| 15.1365 – 15.35 | FIXED SPACE RESEARCH Mobile | General Rules and Regulations (2.105, US211, 5.339) |
| 15.35 – 15.4 | EARTH EXPLORATION-SATELLITE RADIO ASTRONOMY SPACE RESEARCH (passive) | General Rules and Regulations (2.105, US74, US246) |
| 15.4 – 15.43 | RADIOLOCATION AERONAUTICAL RADIONAVIGATION | General Rules and Regulations (2.105, US211, US260, US511E, 5.511E, 5.11F) Aviation (87) |
| 15.43 – 15.63 | RADIOLOCATION FIXED-SATELLITE (Earth-to-space) AERONAUTICAL RADIONAVIGATION | General Rules and Regulations (2.105,US211, US260, US359, US511E, 5.511C, 5.511E, 5.511F) Satellite Communications (25) Aviation (87) |
| 15.63 – 15.7 | RADIOLOCATION AERONAUTICAL RADIONAVIGATION | General Rules and Regulations (2.105, US211, US260, US511E, 5.511E, 5.511F) Aviation (87) |
| 15.7 – 16.6 | RADIOLOCATION | General Rules and Regulations (2.105, G59) Private Land Mobile (90) |
| 16.6 – 17.1 | RADIOLOCATION Space research (deep space) (Earth-to-space) | General Rules and Regulations (2.105, G59) Private Land Mobile (90) |
| 16.6 – 17.1 | RADIOLOCATION | General Rules and Regulations (2.105, G59) Private Land Mobile (90) |
| 17.1 – 17.2 | RADIOLOCATION | General Rules and Regulations (2.105, G59) Private Land Mobile (90) |
| 17.2 – 17.3 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) | General Rules and Regulations (2.105, G59) Private Land Mobile (90) |
| 17.3 – 17.7 | FIXED-SATELLITE (Earth-to-space) BROADCASTING-SATELLITE Radiolocation | General Rules and Regulations (2.105, G59, G117, US259, US271, US402, NG163) Satellite Communications (25) |
| 17.7 – 17.8 | FIXED FIXED-SATELLITE (Earth-to-space) | General Rules and Regulations (2.105, G117, US271, US334) Satellite Communications (25) TV Broadcast Auxiliary (74F) Cable TV Relay (78) Fixed Microwave (101) |
| 17.8 – 18.3 | FIXED FIXED-SATELLITE (space-to-Earth) | General Rules and Regulations (2.105, G117, US334, US519) Satellite Communications (25) TV Broadcast Auxiliary (74F) Cable TV Relay (78) Fixed Microwave (101) |
| 18.3 – 18.6 | FIXED-SATELLITE (space-to-Earth) | General Rules and Regulations (2.105, G117, US139, US334) Satellite Communications (25) |
| 18.6 – 18.8 | EARTH EXPLORATION-SATELLITE (passive) FIXED-SATELLITE (space-to-Earth) SPACE RESEARCH (passive) | General Rules and Regulations (2.105, G117, NG164 US139, US225, US254, US334) Satellite Communications (25) |
| 18.8 – 19.3 | FIXED-SATELLITE (space-to-Earth) | General Rules and Regulations (2.105, G117, NG165, US139, US334) Satellite Communications (25) |
| 19.3 – 19.7 | FIXED FIXED-SATELLITE (space-to-Earth) | General Rules and Regulations (2.105, G117, NG166, US139, US334) Satellite Communications (25) TV Broadcast Auxiliary (74F) Cable TV Relay (78) Fixed Microwave (101) |
| 19.7 – 20.2 | FIXED-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) | General Rules and Regulations (2.105, G117, US139, US334, 5.525, 5.526, 5.527, 5.528, 5.529) Satellite Communications (25) |
| 20.2 – 21.2 | FIXED-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) Standard frequency and time signal-satellite (space-to-Earth) | General Rules and Regulations (2.105, G117) |
| 21.2 – 21.4 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) | General Rules and Regulations (2.105, US532) Fixed Microwave (101) |
| 21.4 – 22 | FIXED MOBILE | General Rules and Regulations (2.105, US211) Fixed Microwave (101) |
| 22 – 22.21 | FIXED MOBILE (except aeronautical mobile) | General Rules and Regulations (2.105) Fixed Microwave (101) |
| 22.21 – 22.5 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE (except aeronautical mobile) RADIO ASTRONOMY SPACE RESEARCH (passive) | General Rules and Regulations (2.105, US342, US532) Fixed Microwave (101) |
| 22.5 – 22.55 | FIXED MOBILE | General Rules and Regulations (2.105, US211) Fixed Microwave (101) |
| 22.55 – 23.15 | FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (Earth-to-space) | General Rules and Regulations (2.105, US145, US278, US342, 5.532A) Satellite Communications (25) Fixed Microwave (101) |
| 23.15 – 23.55 | FIXED INTER-SATELLITE MOBILE | General Rules and Regulations (2.105, US145, US278) Satellite Communications (25) Fixed Microwave (101) |
| 23.55 – 23.6 | FIXED MOBILE | General Rules and Regulations (2.105) Fixed Microwave (101) |
| 23.6 – 24 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | General Rules and Regulations (2.105, US74, US246) |

## 4.4	WRC-19 outcomes

In this clause summary of the WRC-19 conclusions related to the IMT in 7 – 24 GHz range and applicable deployment scenarios is provided.

### 4.4.1		New agenda items for WRC-23

Based on the discussion on the new agenda items for WRC-23 conference, resolution COM6/1 (i.e. Agenda for the 2023 world radiocommunication conference) as in the WRC-19 Provisional Final Acts report [59] was agreed during WRC-19, including the following considerations:

1.2 to consider identification of the frequency bands 3 300-3 400 MHz, 3 600-3 800 MHz, 6 425-7 025 MHz, 7 025-7 125 MHz and 10.0-10.5 GHz for International Mobile Telecommunications (IMT), including possible additional allocations to the mobile service on a primary basis, in accordance with Resolution COM6/2 (WRC-19);

Based on the above identified frequency ranges for IMT, possible additional allocations to the mobile services on a primary basis are to be discussed for 7 – 24 GHz range during WRC-23 in the following frequency range: 10.0 – 10.5 GHz (Region 2).

Further referring to the resolution COM6/2 in [59] (i.e. Studies on frequency-related matters for the terrestrial component of International Mobile Telecommunications identification in the frequency bands), WRC-19 recognized to conduct and complete in time for WRC-23 the sharing and compatibility studies with a view to ensuring protection of serviced to which the frequency band is allocated on a primary basis, without imposing additional regulatory or technical constraints on those services, and also, as appropriate, on services in adjacent bands, for the above listed band 10.0 – 10.5 GHz.

2 to conduct and complete in time for WRC-23 the sharing and compatibility studies1, with a view to ensuring the protection of services to which the frequency band is allocated on a primary basis, without imposing additional regulatory or technical constraints on those services, and also, as appropriate, on services in adjacent bands, for the frequency bands: – 3 600-3 800 MHz and 3 300-3 400 MHz (Region 2); – 3 300-3 400 MHz (amend footnote in Region 1); – 7 025-7 125 MHz (globally); – 6 425-7 025 MHz (Region 1); – 10 000-10 500 MHz (Region 2),

### 4.4.2		IMT for fixed wireless broadband in fixed services bands

WRC-19 resolution COM6/1 in [59] concludes that WRC-23 shall consider and approve the report of the Director of the Radiocommunication Bureau, including the following:

Study the use of International Mobile Telecommunication system for fixed wireless broadband in the frequency bands allocated to the fixed services on primary basis, in accordance with Resolution COM6/18 (WRC-19);

In particular the above resolution includes reference to the resolution COM6/18 in [59] on study of IMT system for fixed wireless broadband in the frequency bands allocated to the fixed services on primary basis:

to conduct any necessary studies on the use of International Mobile Telecommunication systems for fixed wireless broadband in the frequency bands allocated to the fixed service on primary basis, COM6/18 in [59].

The above referred resolution COM6/18 [59] further recognizes (among others) that:

that the ITU-R Handbook on “Fixed Wireless Access” addressed the use of International Mobile Telecommunication (IMT) systems for Fixed Wireless Access, and Recommendation ITU-R M.819 contains specific requirements pertaining to fixed wireless access,

It shall be observed, that the above referred ITU-R Handbook and ITU-R recommendation M.819 [66] are Fixed Wireless Access (FWA) specific, while the resolution COM6/18 refers to the “fixed wireless broadband”. This terminology inconsistency may require further clarifications and studies on their applicability and deployment scenario assumptions. Such studies are considered to be out of scope of this study item.

The above text from COM6/18 in [59] is found to be applicable to the 7 – 24 GHz range where multiple fixed wireless broadband systems were deployed as fixed services on primary basis.

It is expected that results of the above studies will be reported to the WRC-23 conference.

# 5	Study on general aspects

## 5.1	General

This clause provides discussion on general aspects of the 7 – 24 GHz frequency range study item applicable to both UE and BS studies, unless otherwise stated.

## 5.2	Selection of frequency sub-ranges and example frequencies

The performance trends related to vital components and technologies to be used within the frequency range 7 – 24 GHz indicates that the frequency range needs to be split up into frequency sub-ranges, each with one example frequency selected. The intension with the example frequencies is that the technology evaluation is supposed to be performed per example frequency. The selected frequency sub-ranges and example frequencies are listed in table 5.2-1.

Table 5.2-1: Frequency sub-ranges and examples frequencies selection in 7 – 24 GHz range

| Frequency sub-range | Frequency (GHz) | Example frequency (GHz) |
| --- | --- | --- |
| 1 | 7.125 - fboundary, low | 10 |
| 2 | fboundary, low - fboundary, high | 15 |
| 3 | fboundary, high - 24.250 | 20 |
| Note 1: fboundary, low is within the frequency range 10 - 13 GHz. Note 2: fboundary, high is within the frequency range 16 - 18 GHz. |  |  |

The above frequency sub-ranges were selected only for purpose of RF technology analyses during this SI and shall not be confused with potential new NR frequency range(s) (as compared to the existing FR1 or FR2 defined for NR) and band plan which could be defined by RAN4 in harmonization with other services for NR in future.

In case outcomes of the RF requirement analyses will be found as sufficiently similar across adjacent example ranges, RAN4 may consider merging them towards conclusion on FR1/FR2 extension and/or new FR range introduction, in the follow-up WI.

## 5.3	Comparison of FR1 and FR2 characteristics

### 5.3.1	General

In a RAN4 context, there is a very substantial difference between the technology characteristics and system behaviors for FR1 and FR2 spectrum. Within RAN4, the understanding of what is meant with FR1 and FR2 differs between the base station RF specifications, the UE RF specifications and the RRM specifications. Furthermore, RAN1 and RAN2 have also used the terms FR1 and FR2 to refer to signaling concepts and types of UE and network behavior. Thus, terms “FR1” and “FR2” imply certain characteristics or types of behavior.

During related future WI when considering specific operating bands within 7 – 24 GHz range, other working groups besides RAN4 would need to confirm whether FR1 or FR2 behavior and requirements can be reused or whether something new is needed. RAN4 would need to consider the applicability of requirements, as well as requirement values. If the requirements correspond sufficiently to FR1 or FR2, extension of the FR may be preferred. Already now we have different RF requirements within the same FR, e.g. different ACLR in 28 GHz and 39 GHz frequencies. If there would be large deviations, a strategy for dealing with the deviations would be needed (e.g. creation of sub-FR, defining new FR, or similar) that could be applied consistently across the working groups to develop the necessary descriptions of requirements, functionality, signaling, etc.

#### 5.3.1.1	RAN1 specification impacts

The RAN1 specifications differentiate FR1 and FR2 in the following aspects:

- TS 38.211 [13]: Random access configuration depends on FR1/FR2. Transition times RXTX and TXRX depend on FR1/FR2. OffsetToPointA unit depends on FR1/FR2.

- TS 38.213 [14]: SSB pattern and cell search depend on FR1/FR2. Power control aspects for dual connectivity depend on FR1/FR2. BWP switching in RA procedure depends on FR1/FR2. Actions related to bwp-InactivityTimer in BWP switching depend on FR1/FR2. Type0-PDCCH monitoring behaviour depends on FR1/FR2.

- TS 38.214 [15]: Different behaviour w.r.t. overlapping reception of unicast PDSCH and SI-PDSCH. FR2 supports TRS in single slot. PT-RS support is different w.r.t. FR1/FR2. PDSCH and PUSCH processing capability 2 is only defined for FR1. FR1 supports almost contiguous allocation for UL CP-OFDM while FR2 does not.

An important question for 7 – 24 GHz range is whether all of the impacted RAN1 procedures are the same as for FR1 or FR2, or whether it is the case that some procedures are like one of the FRs and other procedures are more like the other FR, or different to either FR. If the latter is the case (i.e. not all procedures can be inherited from an FR), then a new FR seems needed, whereas if the latter is the case then from a RAN1 perspective the FR could be extended.

#### 5.3.1.2	RAN2 specification impacts

FR1 and FR2 differ in terms of requirements and functionality. This implies that if the total functionality in a band in the 7 – 24 GHz range does not correspond exactly to FR1 or to FR2 then the RRC signaling may need to differ. Furthermore, signaling of parameters may need to differ in case the parameter values do not correspond directly to FR1 or FR2 in all cases.

Based on input from RAN1 and RAN4, RAN2 distinguishes FR1 and FR2 both in the (interpretation of) UE capability signaling as well as in the (interpretation of) DL configuration messages. If the 7 – 24 GHz range would be introduced as e.g. a new FR3, RAN2 specifications would need to be updated. In some cases it may just be and textual amendment (“in FR1 and FR3”) but in other cases it will require new fields or entire branches in RRC signaling. If most properties of the new frequency range differ from FR1 and FR2 or if some are like in FR1 and others like in FR2, a new term may anyway be preferable. In case of the latter, RAN2 will have to discuss and incorporate many changes. Planning of work items should account for that.

However, if the new frequency range would inherit most of its properties from either FR1 or FR2, it would be preferably from signaling point of view to reuse those existing terms (and hence the signaling structure). This would minimize the changes to the RAN2 specifications and hence the work-load and possibly also the time to market.

#### 5.3.1.3	RAN4 specification impacts

##### 5.3.1.3.1	General

The FR categorization has some impact on general system parameters in RAN4. For example, the set of channel bandwidths and subcarrier spacings that can be applicable. The specific BWChannel and SCS are specified on a band specific basis but must be a subset of the BS/SCS defined for the applicable frequency range.

##### 5.3.1.3.2	BS RF

In the BS RF specifications (i.e. TS 38.104 [5], TS 38.141-1 [16], TS 38.141-2 [6]), apart from the frequencies of bands, the term frequency range encompasses aspects of requirement concept and design, requirement levels and test methodologies.

Requirement concept and design differs for the TDD OFF power requirement, where a co-location type requirement is applicable for all FR1 bands and a TRP/EIRP requirement for FR2 bands.

The concept for receiver requirements differs substantially. For all FR1 bands, RX requirements are based on base conducted values that are scaled with a virtual gain. For FR2 bands, the reference sensitivity level is declared within a specific range, and other interferer levels are relative to that value.

For all FR1 bands, co-location requirements are specified, whereas for FR2 bands, there are no co-location requirements. The set of performance requirements differ depending on whether the band is an FR1 or FR2 band.

##### 5.3.1.3.3	UE RF

In the UE RF specifications (TS 38.101-1 [17], TS 38.101-2 [18], TS 38.101-3 [19]), the major difference between FR1 and FR2 is that for FR1, requirements are defined and tested as conducted, whereas for FR2 OTA definitions and tests are applied. As a consequence of the OTA testing, in FR2 some additional requirements on spherical coverage and beam correspondence are included that are not applicable for FR1.

Apart from the OTA aspects, the requirement concept and applicability between FR1 and FR2 is in general more similar for the UE than for the BS. FR1 includes some additional blocking requirements compared to FR2. Handling of duty cycle and power back off for EMC & absorption regulatory compliance differs between the two ranges.

Requirement values differ between FR1 and FR2. Some requirements are band specific and others frequency range specific.

##### 5.3.1.3.4	RRM

The main difference in RRM specification TS 38.133 [20] between FR1 and FR2 UEs is that FR1 UEs are not assumed to perform RX beam sweeping, whereas RX beam sweeping is necessary for FR2 handover which leads to longer delays in requirements (larger multipliers for SMTC periodicity). In addition, the UE cannot simultaneously perform analogue beamforming towards the serving cell and RX beam sweeping so there are various scheduling restrictions specified in TS 38.133 [20] for FR2 operations to allow the UE to beam sweep.

Thus, there is a need to consider both whether beam sweeping is required, and how likely it is that the restriction is needed.

There are also differences in other aspects of RRM requirements such as UE measurement capabilities (e.g. minimum number of SSB that the UE shall be capable of measuring), however these are more a consequence of different parameterization of the physical layer (up to L = 64 TX beams are possible in FR2) rather than fundamentally different operations and requirements.

Hence, when considering a frequency band in the range 7 – 24 GHz the fundamental questions which need to be answered from an RRM perspective are:

- Whether RX beam sweeping is used, and if so, how does it differ from the existing RX beam sweeping operation assumed for FR2.

- At what frequency is it feasible to operate a UE without RX beam sweeping.

The answers to these questions would determine the extent to which existing FR1 or FR2 requirements could be reused in the 7 – 24 GHz range, and how the existing requirements could be modified to make them applicable to bands in 7 – 24 GHz range.

### 5.3.2	Transmitter architecture

It is generally assumed that FR1 BS adopt a BB beam forming architecture in addition to using digital adaptive pre-distortion to linearize the transmitter and that FR2 BS use analogue, digital or hybrid beam forming and does not use linearization. Whilst different architectures decisions may go together, they are not dependent on each other but are both dependent on a similar set of technical parameters.

Low frequencies lend themselves to both BB beam forming and digital linearization as they have the following characteristics:

- Antennas size and hence No of elements is limited by the physical size.

- Power amplifiers can be high power.

- Operating band widths are small (compared to FR2).

As such a separate transmitter chain from BB to antenna element is feasible.

For a base station the principle transmitter architecture is visualised in figure 5.3.2-1.

Figure 5.3.2-1: Reference architecture of base station TX

Based on the implementation the number of interfaces between the sub-systems may vary. The parameters in the reference architecture is described in table 5.3.2-1.

Table 5.3.2-1: Port parameters

| Parameters | Interface | Description |
| --- | --- | --- |
| NLP | BB | Number of supported logical ports defined in RAN1 specifications |
| NBB | internal | Number of I/Q data transmitter streams feed to DACs in the transceiver array |
| NRF | Transceiver array boundary | Number of RF connectors at the transceiver array boundary |
| NAE | internal | Number of RF transmission lines to antenna array |

The relations between the number of interfaces, can be used to discriminate between different beamforming approaches as described in table 5.3.2-2.

Table 5.3.2-2: Beamforming concepts

| Beamforming | Antenna Port Mapping | Transceiver Array | Radio Distribution Network |
| --- | --- | --- | --- |
| Analog | NLP=NBB | NBB<NRF | NRF<NAE |
| Digital | NLP<NBB | NBB=NRF | NRF<NAE |
| Hybrid | NLP<NBB | NBB<NRF | NRF<NAE |

Figure 5.3.2-2: Example of digital beam forming architecture

Because each PA has a path from the digital processing domain implementing digital pre-distortion is possible, the relatively small bandwidths of the operating bands mean that converters are feasible and also the high power of the PA means that the energy saved in linearizing the PA is greater than the energy used in implementing that linearization.

At high frequencies the individual PA power is much lower, in addition the antenna array size and the number of elements is no longer governed by the physical size of the array. Larger path losses at higher frequencies mean that higher gain antennas are more important. As such FR2 arrays tend to have a larger number of lower powered PA’s. In this case operating a separate path from digital processing domain to antennas becomes complex and hence hybrid beam forming architectures are assumed.

Figure 5.3.2-3: Example of hybrid beam forming architecture

As the beam forming is done in the RF it is not possible to individually digital pre-distort each PA in addition the wide operating and channel bandwidths mean that generating 3 to 5 times the bandwidth from the digital-to-analogue converters starts to become impractical and the relatively lower power saved per PA means the power saving compared to the power used in the lineariser is not attractive. Hence it is currently assumed digital linearization is assumed to be not used for FR2.

For both FR1 and FR2 however these are assumptions are not used for requirement setting, the antenna size, number of and power of the PA’s is assumed when investigating co-existence but the beam forming and linearization architectures are not important at this level.

For the 7 – 24 GHz region it is likely that implementation will change from a digital beam forming architecture to a hybrid beam forming or analogue beamforming architecture at some frequency in this range however this is not relevant to the requirement setting. As with FR1 and FR2 the size, number of and PA power rating assumptions are required to estimate typical OTA performance when studying co-existence, but the requirement do not need to make any assumption on how the beam forming is achieved or how or if the PA’s are linearized.

## 5.4	RF technology considerations in the 7 – 24 GHz range

### 5.4.1	PA trends

For 7 – 24 GHz frequency range, it is essential to perform in-depth analysis of power amplifiers based on different semi-conductor technologies. In addition, complex relation between output power, ACLR (linearity) and efficiency is an important aspect considering the performance and feasibility for example frequencies.

The PA technology and trends are based on professor Hua Wang [11] work where a large power amplifier survey consisting of more than 3200 data points with over 1200 data points for CMOS, SiGe PAs and over 1500 power amplifier data points has been collected for GaN, GaAs, InP. Wang’s database covers published results, both from the open literature, as well as commercial amplifiers from various vendors.

Based on the information from [11], the PA database information was summarized for all the considered RF technologies in figure 5.4.1-1. It can be observed that based on the available information, there is no data for the LDMOS technology for the 7 – 24 GHz range, while the InP technology data is available from 20 GHz upwards.

Figure 5.4.1-1: Saturated output power versus frequency (red box depicts 7 – 24 GHz range) [11]

In order to derive more accurate PA trends, all the figures below were plotted with PA operating frequencies much wider then just 7 – 24 GHz range. More detailed technology-specific plots (e.g. PAE vs. Psat, or Psat vs. frequency) can be found in the Excel sheet capturing all the PA survey data in [11].

Based on the analysis of the achievable Psat trends over the 7 – 24 GHz range, the saturated output power maximum trend values for 7.125 GHz and 24.25 GHz from [11] were listed in table 5.4.1-1, further extended with additional extrapolated values. It may be noted here that trend of SiGe BiCMOS PA in figure 5.4.1-1 is limited to 10 GHz frequency. If the trend line is extrapolated down to 7 GHz then Pout of 36 dBm is expected.

Table 5.4.1-1: Maximum trend values of the saturated output power

| RF technology | Estimated maximum trend value of the saturated output power @7.125 GHz (dBm) | Estimated maximum trend value of the saturated output power @24.25 GHz (dBm) |
| --- | --- | --- |
| CMOS | 30 | 26 |
| SiGe | 36 | 30 |
| GaN | 58 | 46 |
| GaAs | 45 (HBT) | 28 (pHEMT) |
| LDMOS | - | - |
| InP | - | 34 |

The following analysis covers the saturated peak output power and power added efficiency (PAE). It should be noted that for all presented characteristics, the results are based on peak power, non-linearized power amplifiers without considering the bandwidth impact to show the trends with respect to frequency for different technologies.

#### 5.4.1.1	Saturated peak output power

Based on material in [9] a more comprehensive analysis of achievable power amplifier performance has been investigated. In figure 5.4.1.1-1, a scatter diagram of saturated output power as a function of operating frequency for silicon technologies is shown. The technologies are grouped in three categories: LDMOS, CMOS and SiGe. The attainable output power at a given operating frequency is limited by the saturated electron velocity and the breakdown field strength in a given semiconductor material. This is captured in Johnsons’ figure of merit which states that the maximum output power will decrease with 20 dB/decade as the operating frequency is increased. The black line conveys the limitation due to Johnsons’ figure of merit.

Figure 5.4.1.1-1: Saturated output power versus frequency for LDMOS, CMOS and SiGe

Figure 5.4.1.1-2 presents the maximum output power as function of operating frequency for two compound semiconductor technologies: GaAs and GaN. Higher output power can be achieved but would necessitate excessive power combining associated with decreased operating efficiency.

Figure 5.4.1.1-2: Saturated output power versus frequency for GaAs and GaN (red box depicts 7 – 24 GHz range)

For PA operating within the 7 – 24 GHz frequency range the trends can be summarized as:

- Silicon technologies are limited to maximum 2 W of peak power.

- With GaN technology the peak output power can be increased to maximum 20 W peak power (at 24 GHz).

#### 5.4.1.2	Power added efficiency

Based on material in [9] a more comprehensive analysis of achievable power amplifier performance has been investigated for PAE. In figure 5.4.1.2-1, a scatter diagram of peak power added efficiency as function of operating frequency for power amplifiers made using silicon transistors (i.e. LDMOS, CMOS and SiGe).

Figure 5.4.1.2-1: Peak power added efficiency versus frequency for LDMOS, CMOS and SiGe (red box depicts 7 – 24 GHz range)

In figure 5.4.1.2-2 the same parameters are plotted for compound semiconductor transistors (i.e. GaAs and GaN). As expected PAE is mainly dependent on the operating frequency and not the transistor technology. The wide spread of data is mainly due to different power levels and different amplifier architectures (e.g. class-A, class-AB, Doherty etc.).

Figure 5.4.1.2-2: Peak power added efficiency versus frequency for GaAs and GaN (red box depicts 7 – 24 GHz range)

Grouping power amplifiers in different power categories gives a better view on the efficiency performance of different semiconductor technologies.

It should be noted that for all presented characteristics, the results are based on peak power, non-linearized power amplifiers without considering the bandwidth impact to show the trends with respect to frequency for different technologies. The presented characteristic neither consider practical implementation as the achievable average power (RMS) would being ~10 dB lower compared to saturated peak power to fulfil the needed modulation quality (EVM) or possibly necessary linearization range.

### 5.4.2	PA power scaling and AAS dependencies

The power capability  PC (the maximum level of signal power that can be generated) of a PA is fundamentally governed by the Johnsons’ figure of merit. Once the semiconductor technology has been chosen (CMOS, SiGe, GaN, etc.) for implementing a power amplifier, the power capability  PC is bounded by  PC∝1/f2 (-20 dB per decade).

Over the frequency range (7 – 24 GHz) considered, however, it is not enough to consider the power capability of single PA. Rather, it may be justified to assume some sort of multi-antenna arrangement or more specifically an antenna array, at least for the upper part of the frequency range. Figure 5.4.2-1 illustrates a conceptual unit area of an antenna array with N elements and will be used to support the discussion below. Here, the array is a uniform array (equidistant distance between elements) and without lack of generality one PA is associated with respective element.

One may rightfully say that the antenna element separation d is constant over frequency when measured in wavelengths λ, e.g. 0.5λ. Consequently, the antenna separation measured in meters will scale as 1/f where f is the target RF frequency for which the antenna array has been designed, and thus the number of elements per unit area N will be  ∝f2 (+20 dB per decade).

The DC power provided to drive one power amplifier is PDC and the DC power provided to the unit area equals:

PDC,UA=N PDC

Correspondingly, the RF power generated by one PA is PRF=PAE PDC where PAE is the efficiency of the PA, and the RF power generated by the unit area is PRF,UA=N PRF=N PAE PDC (the exact definition of PAE also includes the PA input signal power but this is left out here for simplicity).

Unit area of antenna array with N elements

PDC

PRF=PAE PDC

•   •   •

•   •   •

•   •   •

d

d

PDC,UA

PRF,UA

Figure 5.4.2-1: Relation between power, number of elements and frequency of operation

If PAE and ACLR had been constant over frequency, a constant DC power per unit area (irrespective of frequency) would obviously imply that the RF power per unit area PRF,UA would also be constant over frequency, as the power capability per PA scales -20 dB/decade while the number of antenna elements scales as +20 dB/decade.

For FR2 ACLR is 28 dB but it is expected that it should improve towards lower frequencies, given the substantially higher ACLR for FR1. The aim of this discussion is to present one such scaling mechanism.

A trend of -5 dB/decade (1/f) was identified for PAE based on empirical data at peak power. It is expected that this scaling also applies when operating in back-off. With a cap on DC power per unit area, such a trend would allow for more RF power per unit area with a preserved ACLR for lower frequencies, i.e. increasing by 5 dB when lowering the frequency by one decade. This exemplified in figure 5.4.2-2 (constant ACLR) for power levels referring to one PA.

Alternatively, one can assume a constant DC as well as constant RF power per unit area and let the Pas towards lower frequencies operate at a lower PAE, to instead allow a higher ACLR, see figure 5.4.2-2 (improved ACLR). Exploiting this to its full extent means that all the PAE improvement for lower frequencies is translated to an equivalent reduction of RF power per PA compared to the RF power per PA for a given ACLR. The basic analysis in [2], backed up by PA simulations, showed that ACLR improves by roughly 2 dB for every dB of RF power reduction for a given PA in the ACLR performance regime of interest.

Example: 	An array operating at 30 GHz provides ACLR of 28 dB. An array operating at 15 GHz with the same DC and RF power per unit area will have an ACLR of 28 + 2·5·log10(30/15) = 31 dB.

log PDC, log⁡( PRF)

)

log⁡(f)

PRF

PDC

log⁡(PAE)

ACLR=28dB

Improved ACLR
for lower frequencies

-20dB/dec

-20dB/dec

ACLR=30dB

ACLR=32dB

Figure 5.4.2-2: Illustration of PA power metrics vs frequency for constant ACLR vs improved ACLR (towards lower frequencies)

The analysis outlined above is based on one PA per antenna element, or at least the same sub-array configuration, over the frequency range considered. Although larger sub-array configurations may be used at lower frequencies the general principle still holds. For example, an array of elements based on 1x1 sub-arrays has a certain RF power PRF associated with one element and consequently one sub-array. The same array of elements but based on 8x1 sub-arrays will have an RF power of 8PRF per sub-array. The only difference lies in that the PA driving the sub-array must be capable of producing a higher level of RF power to drive the 4x1 sub-array compared to the 1x1 sub-array. And this is where the more costly by also more capable technologies come into the picture, e.g. GaN and GaAs, which may provide up to an order of magnitude higher power levels compared to Si-based technologies. So, as far as RF and DC power is concerned the scaling still applies. But using GaN and GaAs technologies also provide a somewhat higher PAE and it can be argued that this can be used to further increase the ACLR somewhat towards the lower end of the frequency range when the PAE benefit has been quantified.

### 5.4.3	PA Output power, ACLR and PAE dependencies

The current technological capability to achieve a certain average output power level at a prescribed ACLR level for power amplifiers operating in a weakly non-linear regime (e.g. in back-off), the relation between the 3rd order intercept point (IP3) and ACLR can be used. For an OFDM signal with a large number of sub-carriers (more than 10) the average output power as function of targeted ACLR can be calculated from:

Pout=IP3 – 12ACLR + 1.3 – 6

If the IP3 levels are not available they can be estimated from the 1dB gain compression point, and the average output power can then be calculated as:

Pout=9.6 +P1dB - 12ACLR + 1.3 - 6

In figure 5.4.3-1 the peak power added efficiency is plotted versus 1-dB output power compression point (P1dB) for amplifiers operating between 10 GHz to 30 GHz. Considering available data (see figure 5.4.1-1 and 5.4.1-2), a reasonable trade-off between linearity and efficiency is achieved around a P1 dB of 100 mW (with associated peak PAE of > 40%). This observation holds regardless of semiconductor technology. In figure 5.4.3-2 the effect on achievable ACLR of the efficiency – linearity trade-off is investigated. Three typical points from the maximum P1dB-PAE trajectory is chosen: P1dB = 10 dBm, 17 dBm and 20 dBm. For each point the ACLR is computed as function of average output power. The PAE numbers given in the figure are indicative of the average PAE resulting from the peak PAE for these example points (4%, 5% and 6%) under the assumption that the amplifiers operate in class-A.

Figure 5.4.3-1: Peak power added efficiency versus 1 dB gain compression point for power amplifiers using Silicon transistors (Left) and GaAs and GaN transistors (right). The data points show power amplifiers operating in the 10 GHz to 30 GHz frequency range

Figure 5.4.3-2: Average output power as function of ACLR with PAE as parameter

To verify the relations between output power vs. ACLR, we re-use to the results and approach derived earlier in the work for FR2 based on simulations of a GaN and a CMOS PA, both designed for operation around 30 GHz. Similar to the FR2 approach where the PA models at 45 GHz and 70 GHz FR2 proxy frequencies used the same nonlinear characteristics but with the output power scaled as -20 dB/decade while PAE scaled as -6 dB/decade, the same approach has been used for 7 – 24 GHz example frequencies. The results for relation between output power and ACLR as well as ACLR vs PAE for example frequencies of 10 GHz, 15 GHz and 20 GHz are presented in figure 5.4.3-3 and figure 5.4.3-4 respectively, but now with PAE scaling as -5 dB/decade as supported by the larger updated data set used in this work.

Figure 5.4.3-3: ACLR versus output power at 10, 15, and 20 GHz example frequencies, and 30 GHz based on 30 GHz PA model for reference

Figure 5.4.3-4: PAE versus output power at 10, 15, and 20 GHz example frequencies, and 30 GHz based on 30 GHz PA model for reference. Constant PAE for CMOS and GaN PA, respectively, also shown

As discussed in clause 5.4.2, assuming the DC power as well as the RF power associated with the Pas should be constant per antenna array unit area, the PAE scaling of -5 dB/decade can be exploited to a yield a -10 dB/decade ACLR scaling. Thus, from 30 GHz to 20, 15, 10 GHz, PAE should improve by 0.9, 1.5, 2.4 dB and consequently the ACLR can be improved by 1.8, 3.0, 4.8 dB.

### 5.4.4	Filtering aspects

In order to meet out of band emissions requirements, analogue filters are typically needed. Filters are thus an important block in the transceiver chain.

Due to limited size (area/volume) and level of integrations needed for AAS BS types for 7 – 24 GHz frequency range (i.e. BS type xFR-H or BS type xFR-O), filtering can pose a significant challenge. Traditional discrete filters are likely to be far too bulky to be fitted in limited size and difficult to embed into highly integrated systems (although may be suitable for BS type xFR-C implementations), and thus more advanced technologies are needed. It is important to understand the properties of filter technologies when considering the relationship between achievable output power, spectrum usage and out of band requirements.

There are certain fundamental parameter dependencies for filters which it is useful to consider, as follows:

- Insertion Losses decreases with increasing BW (for fixed fc).

- Insertion Losses increases with increasing fc (for fixed BW).

- Insertion Losses decreases with increasing Q.

- Insertion Losses increases with increasing N (number of poles).

To better understand the relationships between these parameters, some estimations of the properties of example filters, with example bandwidths and emissions requirements were considered.

- For the example frequency of 20 GHz, attenuation around 20 - 25 dB was considered.

- For 15 GHz example frequency the corresponding attenuation level was 30 dB.

It is important to note that the analysis presented here represents very rough estimates presented for understanding of the trends in filter performance and the specific numbers should not be relied upon for future standardization work. The exact attenuation level and filtering aspects can be further evaluated when future WI based on specific possible frequency bands and the adjacent systems are known.

In summary, the filter examples investigated imply the need to have reasonably large guard-band taking to account the tolerances and drift in frequency due to e.g. temperature etc.

The analysis is focused on LTCC filters as an example, but this is not the only feasible filter technology. There are other filter technologies which could be suitable for the 7 – 24 GHz frequency range such as ceramic wave-guide filters and air cavity filters which can be further investigated when in future band specific work items can be studied in detail.

Based on the example filter characteristics in annex B, filter parameters for some possible technologies have been collected for example frequencies in table 5.4.4-1.

Table 5.4.4-1: Filter parameters per example frequency

| Example frequency (GHz) | Filter technology | Guard (MHz) | Insertion loss (dB) | Suppression (dB) | Note |
| --- | --- | --- | --- | --- | --- |
| 10 | LTCC | 500 | 4 | 40 | A guard band of 0.2 GHz appears un-realistic for 10 GHz, for LTCC and PCB-integrated stripline filters regardless of passband bandwidth, due to the extreme requirements on frequency tolerance, and Q-value. |
| 15 | LTCC | 600 | 3 | 30 |  |
| 20 | PCB, LTCC | 1500 - 2000 | 4 | 20 - 25 |  |

The filter parameters can be used as input for decision for numerous RF parameters relevant for exclusion zones for receiver blocking, emission and other requirements. It is important to note that there are other filter technologies which should be also taken into account in future work.

## 5.5	Key parameters over the 7 – 24 GHz range

### 5.5.1	Noise figure

#### 5.5.1.1	General aspects for noise figure

Receiver Noise Figure (NF) is one of the essential metrics for determining receiver requirements. For determining typical NF value some fundamental principles should be considered:

- The NF is not given by the LNA alone, but also by bandwidth, linearity and dynamic range dependencies as there is a delicate balance which should be considered when future requirements are specified.

- A full RF receiver chain all the way up to radiating elements should be addressed as all parts in the chain would contribute to the overall receiver performance including switch (for TDD), routing and filter losses, etc.

- For some compact and highly integrated building practices with many transceivers and antennas, the power efficiency and heat dissipation in small area/volume is necessary needs to be considered.

  - As an example, in the context of NF, it might be possible to reduce the noise contribution from ADC by using more bits, but this would have significant implication in terms of power consumption and heat dissipation aspect as a single added bit to ADC would result in four times higher power consumption. In general the ADC power consumption is proportional to

BW2 · DRACD

where DRACD is the ADC dynamic range.

For UE, the typical NF values considering the usage of similar technologies as for BS, should be similar but slightly higher to allow more flexibility in the implementation and taking to account the size limitations.

Table 5.5.1.1-1: Typical noise figure for 7 – 24 GHz example frequencies

| Example frequency (GHz) | Typical NF values for NR BS (dB) | Typical NF values for NR UE (dB) |
| --- | --- | --- |
| 10 | 7 | 9 |
| 15 | 8 | 10 |
| 20 | 9 | 10 (Note) |
| NOTE: UE implementation margin can vary. NF value could be revised for higher frequencies. NF value shall not be revised to a value that is higher than the FR2 value. |  |  |

#### 5.5.1.2	NF analysis for NR BS

Considering recent state-of-the-art LNA-only noise figure publications in [22 – 27], figure 5.5.1.2-1 presents the LNA-only noise figure for different semi-conductor technologies over frequency based on published data.

Figure 5.5.1.2-1: LNA-only NF trend over frequency

The typical noise figure values presented in table 5.5.1.2-1 are based on assumption of pre-LNA noise figure of ~1.8 dB and per example frequency calculation of losses such as routing losses, etc.

ETSI has published an update of TR 101 854 [46] on fixed radio systems in April 2019, which contains a table on typical NF values and associated industrial margins (IMF) of the radio receivers. This table is extracted below in table 5.5.1.2-1. The IMF considers various performance variations of system elements over e.g. temperature extremes, voltage variations, or aging by capturing production spread of RF circuits.

Table 5.5.1.2-1: Typical Noise Figures (NF) and associated Industrial Margins (IMF)

| Frequency band (GHz) | Typical Noise Figure (NF) (dB) | Industrial margin (IMF) (dB) |
| --- | --- | --- |
| 1,3 - 3 | ~4 | +3 |
| 3 - 5 | ~5 | +3 |
| 6 - 15 | ~5 | +3 |
| 18 - 23 | ~6 | +3 |
| 26 - 28 | ~7 | +3 |
| 32 | ~7 | +3 |
| 38 - 42 | ~8 | +3 |
| 48 - 50 | ~9 | +3 |
| 52 - 55 | ~10 | +3 |
| 71 - 76 | ~13 | +4 |
| 81 - 86 | ~13 | +4 |

NOTE: 	The above ETSI originated Industrial Margins (IMF) shall not be confused with the Implementation Margins (IM) used in process of RAN4 conformance requirements derivation.

It can be seen from the table above that even though this TR is on fixed radio systems, the NF and IMF values of the radio receivers are similar to those that RAN4 has adopted for mobile radio systems (considering different margins for BS and UE). This would be explained by the fact that many underlying components would be the same in both fixed and mobile radio receiver. Since this TR has been officially published by ETSI, it is reasonable to use the data in the table above as references for the typical BS NF over the whole 7 – 24 GHz rang as shown in table 5.5.1.2-1.

#### 5.5.1.3	NF analysis for NR UE

##### 5.5.1.3.1	General

Receiver architecture (including antenna system) plays an important role in calculating system NF. At higher frequencies, Active Antenna Systems (AAS) with antenna arrays and beamforming networks become necessary to compensate for higher path loss and maintain similar link performance as lower frequencies [49]. Besides, at higher frequencies, AAS are integrated with the rest of the receive chain in a way that antenna connectors are no longer available.

It is well known that phase noise degrades at a rate of ~6 dB/octave with frequency. Since in a zero-IF architecture, RF LO will be at a higher frequency compared to a super-heterodyne architecture, it will have higher phase noise and will contribute more to overall system noise. Note that IF LO in a super-heterodyne architecture will have its own phase noise contribution. But degradation of phase noise with frequency combined with other issues related to generating RF LO for zero-IF architecture at higher frequencies, generally makes RF+IF down-conversion a more attractive choice at higher frequencies. On the other hand, a super-heterodyne architecture, even in the absence of any jammers in the image band, suffers from image band noise and requires either filtering before the first down conversion or image-reject downconverter designs.

Other considerations include availability and necessity of analog filters in RF and their placement in the RF chain.

##### 5.5.1.3.2	Frequency sub-range 1

Zero-IF or direct conversion is the reference architecture for all sub-7 GHz LTE and NR applications [50] and is likely to be used for Frequency sub-range 1 as well.

NF for a reference 6 GHz TDD system is presented in R4-1904127 [51]. Since the aim is to compare NF with a single-band mm-wave TDD system, no antenna switch module for band selection or sub-band duplexing is accounted for. Furthermore, it is assumed that ADC quantization noise contributes 0.4 dB to the overall system NF.

But more importantly, the frond-end architecture has a pre-LNA filter for rejecting out of band jammers and interferers. The presence of filter is important since in the absence of antenna arrays and beamforming, it is more likely than not that the UE picks up these types of jammer during field operation.

“IM” refers to implementation margin of 2.5 dB which is added to account for process and temperature variations.

Note that some passive technologies such as filters and matching networks used in FR1, may have degraded performance above 7 GHz [52]. Additionally, the above calculation assumes a single band UE, which is uncommon in sub-7 GHz. While further refinement can be at later stages, we propose keeping the same NF and using a NF of 9 dB for example frequency of 10 GHz. This value is lowered from the original proposal of 10 dB from [6] due to concerns that the implementation margin was too high.

Table 5.5.1.3.2-1: 6 GHz system noise figure calculation [51]

| 6 GHz current solution |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Ant. Loss | SWT | filter | LNA | Post LNA loss | Down conv. |
| Gain | -0.5 | -0.5 | -3 | 21 | -3 | na |
| NF | 0.5 | 0.5 | 3 | 0.8 | 3 | 13 |
| NF cumm | 5.8 | 5.3 | 4.8 | 1.8 | 16.0 | 13.0 |
| NF w ADC | 6.2 | 5.7 | 5.2 | 2.2 | 16.4 | 13.4 |
| NF w ADC+IM | 8.7 | 8.2 | 7.7 | 4.7 | 18.9 | 15.9 |

##### 5.5.1.3.3	Frequency sub-range 2

The choice of front-end architecture for frequency sub-range 2 is less certain than for frequency sub-ranges 1 or sub-range 3:

- As far as synthesizer phase noise concerned, frequency sub-range can end up using either zero-IF or super-heterodyne architectures.

- The performance of existing RF connectors is only cited up to 12 GHz [54] and there may in fact be a breakpoint for current RF connector technologies [55], forcing Fequency sub-range 2 to use OTA test methods.

- If link budget shows that antenna arrays are needed, then beamforming can provide some level of spatial filtering of jammers and interferers, requiring less or no filter rejection at all.

- Compared to frequency sub-range 3 or mm-wave, array elements and overall size will increase in this frequency sub-range. For example, while mm-wave can use a 2x8 array, frequency sub-range 2 may be limited to a 1x4 or smaller array, reducing the array gain and hence effectiveness of beamforming to reject jammers

- On the other hand, if omnidirectional antennas used instead, filtering becomes necessary.

In either case, the NF will likely be worse than system NF values proposed for frequency sub-range 1 and sub-range 3. Hence, for RAN4 standardization work, a system NF of 10 dB is proposed for frequency sub-range 2. This is reduced from the value in [6] on concerns that the implementation margin is too high.

##### 5.5.1.3.4	Frequency sub-range 3

In contrast to sub-7 GHz LTE and NR, at mm-wave frequencies, super-heterodyne architecture can outperform zero-IF architecture due to lower LO frequency and superior phase noise [50]. Frequency sub-range 3 is more likely to follow suit and use an RF+IF architecture.

In RF+IF architectures, even in the absence of interferers at image frequency, the thermal noise in the image band is down converted to IF and contributes to overall system noise figure. If LNA and down-conversion gain are the same in desired and image bands, thermal noise in the image band can degrade overall system NF by as much as 3dB (even though this is unlikely to happen in practice). Therefore, some mechanism, either in the form of pre-LNA filter or image reject mixer is necessary to protect the receiver.

While integrated transmission line or LC filters can be used to provide some rejection of out of band jammers and image band noise, placing then before the LNA will result in higher NF. Another option is placing the filter after LNA and before the downconverter. This architecture has better NF but cannot protect LNA from strong out of band jammers. However, with beamforming, it is unlikely to have a strong jammer in the direction of beam peak. So it may even be possible to use no filter at all and use image reject mixer to protect the receiver from image band noise. System NF calculations presented in the following two tables are from [51] and are more likely to be used in frequency sub-range 3.

Table 5.5.1.3.4-1: 30 GHz system noise figure calculations post-LNA filter [51]

|  | 30 GHz post LNA filter solution |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Ant. Loss | SWT | LNA | filter | Post LNA loss | Down conv. |
| Gain | -0.3 | -1.5 | 21.5 | -2.5 | 1.5 | na |
| NF | 0.3 | 1.5 | 3.2 | 2.5 | -1.5 | 13 |
| NF cumm | 5.3 | 5.0 | 3.5 | 14.0 | 11.5 | 13.0 |
| NF w ADC | 5.7 | 5.4 | 3.9 | 14.4 | 11.9 | 13.4 |
| NF w ADC+IM | 8.2 | 7.9 | 6.4 | 16.9 | 14.4 | 15.9 |

Table 5.5.1.3.4-2: 30 GHz system noise figure calculations with image reject mixer [51]

|  | 30 GHz Image reject solution |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Ant. Loss | SWT | filter | LNA | Post LNA loss | Down conv. |
| Gain | -0.3 | -1.5 | 0.0 | 21.5 | 1.5 | na |
| NF | 0.3 | 1.5 | 0.0 | 3.2 | -1.5 | 15.0 |
| NF cumm | 5.3 | 5.0 | 3.5 | 3.5 | 13.5 | 15.0 |
| NF w ADC | 5.7 | 5.4 | 3.9 | 3.9 | 13.9 | 15.4 |
| NF w ADC+IM | 8.2 | 7.9 | 6.4 | 6.4 | 16.4 | 17.9 |

While the losses will probably be slightly lower at 20 GHz, in accordance with approved NF values for FR2 [53], a NF of 10 dB is proposed for frequency sub-range 3.

### 5.5.2	Power amplifiers

In clauses 5.4.1.1, 5.4.1.2, 5.4.2 and 5.4.3, the PA trends and saturated output power, PAE trends, AAS power scaling principles and PA dependencies respectively are described. The analysis resulted in example ACLR values without linearization of ~33 dB, ~31 dB and ~30 dB for example frequencies of 10, 15 and 20 GHz respectively. The example ACLR values do not fully cater for the larger transmission bandwidths likely for spectrum within 7 – 24 GHz frequency range and do not include the effects of any possible linearization applied to the transmitter chain which would have the effect of improving ACLR.

For 10 GHz example frequency, where an AAS BS may possibly re-use the existing deployment grid due to propagation condition it would benefit from higher PA power and high PAE. To investigate the potential improvements achievable from linearization three different PA’s based on GaN technology from 3 different vendors (vendor A, vendor B and vendor C) were investigated without and with linearization. The measurements were performed at ~8 GHz for practical reasons indicating that the measured results will be slightly worse for ~10 GHz.

Due to possible size of spectrum at higher frequencies, the bandwidth of 200 MHz and 400 MHz corresponding to 2 or 4 carriers, each 100 MHz was assumed.

The measurement results of the important characteristics are summarized in table 5.5.2-1.

Table 5.5.2-1: Measurement results

|  | Power (dBm) 200 MHz / 400 MHz | PAE (%) 200 MHz / 400 MHz | ACLR (dB) Without linearization 200 MHz / 400 MHz | ACLR (dB) With linearization 200 MHz / 400 MHz |
| --- | --- | --- | --- | --- |
| PA vendor A | 35 | 11.7 / 11.5 | 36.2 / 35.5 | 42.1 / 39.8 |
| PA vendor B | 37 / 36 (1 dB back off to reach EVM) | 12.7 / 12.3 | 26.4/ 21.6 | 38.9 / 29.8 |
| PA vendor C | 33 | 16.8 / 17 | 32.7 / 23.2 | 42.8 / 24.3 |

The measured PAE for different PA’s are quite aligned with the PAE analysis in clause 5.4.1.2. Note that the PAE needs to be further improved in the AAS BS context given the need for fairly high power at these frequencies.

The result also shows the impact of bandwidth where the achievable ACLR is lower for 400 MHz bandwidth (which is quite a reasonable bandwidth at these frequencies), it is not clear if this is a limitation of the linearizer, the Tx chain or the PA.

Considering the need for a fairly high power and efficiency for these frequency ranges and the empirical analysis and measurement results, the achieved ACLR value for 10 GHz example frequency with one example linearization is around 33 to 35 dB, when the average performance of 200 MHz and 400 MHz channel bandwidth is considered.

### 5.5.3	Phase noise

#### 5.5.3.1		General

Sufficiently low phase noise at the measurement interface is required to avoid excessive EVM. Based on the available data [35 – 40], a descriptive BS phase noise profile is shown in figure 5.5.3.1-1. It shows performance for base stations operating in 7 – 24 GHz range based on current (i.e. as of 2019) component data. In the figure each colour matches to a single component.

It should be noted that throughout this section phase noise is referred to the observed level at a measurement interface. Noise levels internal to PLL and clock distribution network may differ from this and implementation freedom is not limited as described in clause 5.5.3.2. To account for different reference source performances a range was set from 100 Hz to 1 kHz where reference source can be dominant phase noise source, as described in clause 5.5.3.3.

For specific use cases where high data rates are targeted, the performance can be better as shown by the examples of component data. On the other hand, in cases where device size, power consumption and low cost are the highest priorities, the phase noise performance may be worse.

Figure 5.5.3.1-1: A representative phase noise profile for 7 – 24 GHz base stations

In figure 5.5.3.1-1 the phase noise profile is shown at 20 GHz example frequency. To adjust the model to a different frequency, the levels need to be scaled according to Leeson’s equation, i.e. halving the frequency reduces the noise power to one quarter of the original. When the profile is given at center frequency f0 and a given noise power P0, the correct noise power P at frequency f equals:

P=P020log10ff0

, where P and P0 describe the full single side-band noise profile for which the unit is dBc/Hz.

#### 5.5.3.2	LO generation and distribution

Array antenna transceivers may be based on different strategies in implementation of local oscillator (LO) signal generation and distribution. Put simply, there are two options:

1.	Centralized LO generation with a single PLL for all transceivers,

2.	Distributed LO generation with one PLL per transceiver.

These are two extreme cases and one could envision a combination of the two such that the transceivers are grouped together and where the transceivers within each group shares a common LO generation, i.e. semi-distributed LO generation.

#### 5.5.3.3	Performance of reference source

To highlight the problem of reference noise increase at lower reference frequencies due to an increased effective frequency multiplication factor, the investigation was focused on low power crystal oscillators at about 40 MHz. At the time of creation of this technical report, there were not as many publications on crystal oscillator circuits as on PLLs, but the few papers from recent years in IEEE Journal of Solid-State Circuits were used to represent state-of-the-art in CMOS low power crystal oscillators at about 40 MHz.

To span the design space, also the 491.25 MHz crystal oscillator is included in the table 5.5.3.3-1 summarizing the crystal oscillator performance.

Table 5.5.3.3-1: CMOS low power crystal oscillator performance examples [62, 63, 64]

|  | Frequency of exemplary reference sources (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 39.25 | 39.25 | 48 | 491.25 |
| Core power (uW) | 19 | 181 | 1500 | 840 |
| Process (nm) | 65 | 180 | 28 | 28 |
| PN @ 1kHz (dBc/Hz) | -139 | -147 | -114 | -107 |
| PN @ 10 kHz (dBc/Hz) | -153 | -161 | -148 | -130 |
| PN @100 kHz (dBc/Hz) | -153 | -172 | -156 | -140 |
| PN @ 1MHz (dBc/Hz) | -153 | -175 | -158 | -148 |
| 10GHz PN @ 1kHz (dBc/Hz) | -91 | -99 | -68 | -81 |
| 10GHz PN @ 10 kHz (dBc/Hz) | -105 | -113 | -102 | -104 |
| 10GHz PN @100 kHz (dBc/Hz) | -105 | -124 | -110 | -114 |
| 10GHz PN @ 1MHz (dBc/Hz) | -105 | -127 | -112 | -122 |
| 15GHz PN @ 1kHz (dBc/Hz) | -87 | -95 | -64 | -77 |
| 15GHz PN @ 10 kHz (dBc/Hz) | -101 | -109 | -98 | -100 |
| 15GHz PN @100 kHz (dBc/Hz) | -101 | -120 | -106 | -110 |
| 15GHz PN @ 1MHz (dBc/Hz) | -101 | -123 | -108 | -118 |
| 20GHz PN @ 1kHz (dBc/Hz) | -85 | -93 | -62 | -75 |
| 20GHz PN @ 10 kHz (dBc/Hz) | -99 | -107 | -96 | -98 |
| 20GHz PN @100 kHz (dBc/Hz) | -99 | -118 | -104 | -108 |
| 20GHz PN @ 1MHz (dBc/Hz) | -99 | -121 | -106 | -116 |
| FoM (dBc/Hz) | -248 | -246 | -220 | -227 |

To translate the phase noise into equivalent phase noise at a PLL output, an amount equal to 20log⁡(fPLL/fref) dB should be added.

Given the above we assume that the crystal oscillator can be made non-dominant above 1 kHz offset frequency, and that the PLL output phase noise will have a slope of -30 dB/decade below that frequency. This is achievable with reference frequencies ranging from 40 MHz to 491 MHz, and power consumptions below 1 mW, using CMOS technologies as different as 28 nm and 180 nm.

#### 5.5.3.4	Noise floor

When it comes to the noise floor it is difficult to find published works that include measurements in this region. Circuit simulations have therefore been performed to find realistic levels of the noise floor. The noise floor is typically set not by the oscillator of the PLL, but by buffers and LO distribution. For this reason, the phase noise of a chain of inverters acting as buffers was simulated. Three equal sized cascaded inverters were used for each of the two differential signal halves, where minimum length devices in a 22 nm technology were used. The input signal was 1 V peak differential at 20 GHz, and different width of devices were investigated, corresponding to different amounts of power consumption. The phase noise is shown in figure 5.5.3.4-1, where the lowest noise floor of close to -170 dBc/Hz was achieved with a 13.4 mW power consumption, the second-best curve was for 7.5 mW, the next was 4.2 mW, then 2.4 mW, and finally 1.3 mW. As can be seen -165 dBc/Hz can be reached at about 5 mW buffer power consumption. Taking attenuation in the LO distribution network into account, it is reasonable to add some margin to this level.

Figure 5.5.3.4-1: Phase noise of simulated differential LO buffers in 22 nm technology, ranging from 1.3 mW to 13.4 mW power consumption

### 5.5.4	Correlation of noise and emissions

With a centralized architecture, i.e. same input distributed to one or multiple transmit antenna connectors or antenna elements, the transmitted noise and emissions will be essentially the same, i.e. fully correlated. The primary downside of this solution is that the performance requirements will be high. When the composite output, the signal transmitted into air is formed by more distributed architecture, i.e. multiple PLLs, multiple PAs and possible non-identical input signal due to beamforming, each individual component of the composite output will be partially uncorrelated. This is beneficial from an EVM and emission requirements perspective as the noise and emissions performance is improved up to 10log(M) where M is the number of non-correlated signals.

This may be used to lower the EVM and emissions performance on the transmitters when they are specified per antenna connector.

## 5.6		Deployment scenarios

Referring to the previous NR discussions, three high level use cases were defined for NR: eMBB, URLLC and mMTC. Considering NR operation in 7 – 24 GHz range, the following was observed:

- eMBB: as 7 – 24 GHz range is considered to be mostly a capacity booster, the eMBB is a valid use case for 7 – 24 GHz range NR operation.

- URLLC: this use case includes such advanced services like e.g. factory automation, autonomous driving, and remote surgery, including both indoor and outdoor case. Therefore, applicability of the URLLC use cases in 7 – 24 GHz required case by case analysis, due to potential coverage limitation restriction.

- mMTC: this NR use case requires good penetration, including deep indoor and wide area coverage. Due to propagation characteristics of 7 – 24 GHz frequencies, the mMTC use case is not seen as well suited for 7 – 24 GHz due to potential coverage limitations.

Referring to the NR study item, range of deployment scenarios were considered and documents in TR 38.913 [2], including both FR1 and FR2 spectrum allocations. NR scenarios from TR 38.913 [2] were analysed for the 7 – 24 GHz applicability, considering the carrier frequency assumptions. Additionally, IAB scenario was included based on related Rel-16 work item.

For the 7 – 24 GHz deployment scenarios discussion it is valid to consider their potential dependencies from the existing incumbent users and spectrum environment in general. Therefore, for each of the potential deployment scenarios, it is essential to also consider information on the (regional) incumbent/neighbour systems information.

Below, brief description of the considered 7 -24 GHz deployment scenarios is captured. It should be noted, that none of the below listed deployment scenarios is considered to be 7 – 24 GHz range specific.

### 5.6.1	Indoor hotspot

Indoor hotspot scenario addresses an indoor case with relatively small coverage, high capacity and high user throughput driven by high density of low-mobility users. For the NR study, the carrier frequencies of 4 GHz, 30 GHz and 70 GHz were considered. Therefore, the whole 7 – 24 GHz range (with all three frequency sub-ranges being applicable) is seen as suitable for indoor hotspot scenario. Indoor hotspot scenario is expected to be addressed by the LA BS class.

### 5.6.2	Dense urban

Dense Urban scenario addresses outdoor and outdoor-to-indoor case in dense urban, interference-limited scenarios, based on macro grid coverage with continuous cellular layout, extended with randomly dropped micro sites, i.e. two-layer layout (i.e. macro + micro). This scenario is characterized by high user density of low and medium mobility, and high traffic loads. For the NR study, the carrier frequencies of 4 GHz and 30 GHz were considered.

This scenario is expected to be addressed by the WA and MR BS classes. This scenario is well suited for CA or DC application.

### 5.6.3	Urban macro

Urban macro scenario addresses an outdoor and indoor case with large cells and continuous coverage, serving users of low and medium mobility.

For the NR study, the carrier frequencies of 2 GHz, 4 GHz and 30 GHz were considered. Therefore, the whole 7 – 24 GHz range (with all three sub-ranges being applicable) is seen as suitable for urban macro scenario.

The difference to Dense urban scenario is that the Urban macro is a one layer macro layout.

Urban macro scenario is expected to be addressed by the WA BS class. It shall be noted, that WA BS class was defined also for FR2 operation.

### 5.6.4	IAB

A diverse range of deployment scenarios can be envisioned for IAB, including support for outdoor small cell deployments, or indoors. For the IAB performance evaluation, scenarios addressing outdoor and outdoor-to-indoor case in dense urban, or urban micro case were considered, based on macro grid IAB-donor coverage with continuous cellular layout, extended with randomly dropped micro sites as IAB-nodes, i.e. two-layer layout (i.e. macro + micro). The IAB WID in RP-193226 [65] also includes the multi-hop IAB network case, which can be seen as interesting feature to achieve range extensions even at higher frequency bands of the 7 – 24 GHz range.

From the propagation point of view, consideration of 7 – 24 GHz range brings more challenging scenario due to higher (compared to FR1) attenuation, with decreased cell range. On the other hand, provision of the LOS backhaul link from the rooftop donor-IAB node shall alleviate propagation challenge. Similar, for the multi-hop case of cascaded IAB-nodes, with the inter-IAB backhauling links being LOS, does not seem to be a bottleneck for the IAB deployments in 7 – 24 GHz range. Similar to Dense Urban scenario, the IAB scenario is characterized by high user density of low and medium mobility, and high traffic loads. For the IAB study, the carrier frequencies of 4 GHz and 30 GHz were considered for the IAB-donor and IAB-node, respectively. The micro layer of the IAB-nodes is considered as the capacity layer, where the IAB study assumed 400 MHz channel bandwidth for IAB-nodes.

### 5.6.5	Summary

Table 5.6.5-1 below summarizes the NR deployment scenarios for the 7 – 24 GHz frequency range.

Referring to clause 4.4.2 on the WRC-19 conclusions, one can find the information on the resolution COM6/18 for the IMT for fixed wireless broadband in fixed services bands. This resolution refers the Fixed Wireless Access (FWA) scenario, which can be considered as yet another deployment scenario applicable to 7 – 24 GHz frequency range.

Studies on relations and dependencies among the FWA, fixed wireless broadband, and/or IAB scenarios are considered to be out of scope of this study item, while all those cases are seen as applicable to 7 – 24 GHz frequency range.

Other NR deployment scenarios such as high speed train, highway scenario or urban grid for connected car were also considered during the 7 – 24 GHz study item, but no further analyses were provided for those.

Table 5.6.5-1: NR deployment scenarios for 7 – 24 GHz frequency range

| NR scenario | Carrier frequency (GHz) | Maximum aggregate system BW (DL+UL) (MHz) | BS antenna array size (Tx and Rx antenna elements) | Scenario layout | User characteristic | ISD (m) | Scenario description | BS class |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor hotspot | 7÷24 | ≤ 200÷1000 | ≤ 256 | Single layer, indoor only | 100% Indoor, 3 km/h, 10 users/TRxP | 20 | Indoor, small coverage, high capacity, high user throughput, high user density | LA |
| Dense urban | 7÷24 | ≤ 200÷1000 | ≤ 256 | Two layer, macro hex grid, random drop micro | Uniform/macro TRxP, 10 users/TRxP, Clustered/micro TRxP, 80% indoor (3 km/h), 20% outdoor (30 km/h) | 200 | Outdoor and outdoor-to-indoor, high user densities, high traffic loads, dense urban areas, continuous cellular layout, interference-limited | WA, WA+MR |
| Urban macro | 7÷24 | ≤ 200÷1000 | ≤ 256 | Single layer, hex grid, macro | 20% Outdoor in cars: 30km/h, 80% Indoor: 3km/h 10 users per TRxP | 500 | large cells and continuous coverage, interference-limited | WA |
| IAB (NOTE 3) | 7÷24 | IAB-donor: 100MHz (@4GHz) IAB-node: 400MHz (@30GHz) |  | two layer (macro IAB-donor + micro IAB-node(s)) for dense urban, single layer for urban micro | 80% indoor (3km/h), 20% outdoor (30km/h); DL/UL traffic = {4:1} (NOTE 4) | 200, 500 | Outdoor and outdoor-to-indoor, dense urban, or urban micro | WA+MR(s) |

NOTE 1:	Summary of all parameters is not intended for the system level simulation purposes, rather for the scenarios characteristics comparison.

NOTE 2:	Values of the aggregated system BW and BS antenna array sizes, as well as user characteristics were reused from the NR deployment scenarios in TR 38.913 [2].

NOTE 3:	Values of the aggregated system BW and BS antenna array sizes, as well as user characteristics were reused from the IAB system level evaluation assumptions. Values of the carrier frequencies of 4 GHz and 30 GHz are provided for reference, based on the IAB technical report.

NOTE 4:	The user characteristics in the IAB scenario are related to the end user (not the IAB node).

## 5.7		NR system parameters analysis for 7 – 24 GHz range

### 5.7.1	General

In this section, analysis for NR system parameters and background information on how to determine the system parameters selection is captured. This analysis and information can be used as guidance on whether FR1 and FR2 system parameters can be re-used or new system parameters are defined for frequency sub-ranges within the 7 – 24 GHz range.

### 5.7.2	SCS

#### 5.7.2.1	General

SCS selection is done within RAN4 according to RAN1’s candidate SCS design. From the RAN4 perspective, the SCS is dependent on the phase noise considerations and channel impairments, such as delay spread and Doppler spread. For the mmWave range, phase noise in general is worse at higher frequencies, therefore for the 7 – 24 GHz range, phase noise should be evaluated when selecting the SCS. The cyclic prefix must accommodate the channel delay spread in which can be a function of implementation and several frequency dependent factors, including path loss and scattering. The impact of antenna arrays, such as increased range and beam selectivity, may affect SCS selection. The SCS selection should consider the effects of Doppler on demodulation performance and tracking.

#### 5.7.2.2	SCS for SSB

The SCS options for the SSB are generally based on the SCS used for the band and the specified channel bandwidths in that band. For FR1, a trend is that 30 kHz SCS for the SSB tends to be used as the frequency of the bands increases. In contrast, the 120 and 240 kHz SCS options for the SSB are allowed for the FR2 bands.

The selection of the SCS of the SSB can occur once the SCS for the bands in the 7 – 24 GHz range and the channel bandwidths are determined.

#### 5.7.2.3	SCS for shared / control channels

The shared and control channels are scalable in SCS. However, the reference signal considerations for those channels are dependent on frequency range. For example, in FR2, a phase tracking reference signal (PT-RS) is mandatory. In addition, the channel bandwidths for bands in the 7 – 24 GHz range can restrict the allowed SCS.

### 5.7.3	Channel bandwidth

The maximum channel bandwidth is related to the RF considerations and sampling rate which is a function of FFT size and SCS selection. Additionally, depending on spectrum allocation restrictions and operators’ request, additional channel bandwidths may be considered.

From a UE perspective, a requirement to support a large number of channel bandwidths could increase the UE design complexity and conformance testing time.

### 5.7.4	Spectrum utilization

Regarding spectrum utilization (SU) perspective, it depends on the Tx emission mask defined for protecting the adjacent channel and Rx blocking requirements. Due to a lack of regulatory requirements defined for 7 – 24 GHz so far, the SU decision needs to be deferred to the future WI phase.

However, the filter length or implementation difficulty should be taken into account from both Tx side and Rx side. As the filter length will also have impacts on the EVM window length, if there are any changes on the SU tables for 7 – 24 GHz, then the EVM window requirement should be revisited correspondingly.

### 5.7.5	Channel raster

There are no refarming bands in 7 -24 GHz range, therefore the SCS-based channel raster could be adopted for better spectrum utilization, especially considering the intra-band contiguous CA scenario. Regarding the use of either 15 kHz, 30 kHz, 60 kHz or 120 kHz SCS-based raster for the 7 – 24 GHz range, it is dependent on the SCS selection.

### 5.7.6	Sync raster

The following basic principle for the sync raster design could be reused also for 7 – 24 GHz range:

sync raster ≤ BWconfig + channel raster – BWSSB

where BWconfig accounts for the bandwidth of the operating band, minimum bandwidth, and guardband, BWSSB is the bandwidth of the SSB.

Once the minimum BW, channel raster and SCS for SSB are determined, the sync raster for 7 – 24 GHz range can be determined.

### 5.7.7	Channel spacing

For the channel spacing between adjacent carriers, a formula is used to ensure nominal channel spacing between adjacent carriers is a multiple of channel raster. Therefore values of 0, ±(1/3) channel raster are considered on all possible cases. A similar spacing may be considered for the 7 – 24 GHz range.

### 5.7.8	Aggregated system bandwidth

Referring to the TR 38.913 [2] on the NR deployment scenarios, values of the aggregated system bandwidth were linked to the example carrier frequencies (i.e. 700 MHz, 4 GHz, 30 GHz, or 70 GHz):

- For the example carrier frequency of 4 GHz the aggregated system bandwidth was considered to be ≤ 200 MHz.

- For the example carrier frequency of 30 GHz the aggregated system bandwidth was considered to be ≤ 1 GHz.

Considering the above NR aggregated system bandwidth values as approximation of the corner cases for the 7 – 24 GHz range, conclusion for 7 – 24 GHz range was derived. The range of the the (upper bound of the) aggregated system bandwidth for 7 – 24 GHz range is considered to be within 200 – 1000 MHz range.

#### 5.7.8.1		Single carrier operation

Based on the comparison of NR operating bands and channel bandwidths with the widths of 7 – 24 GHz frequency ranges of interest, it was observed that the set of Rel-16 NR channel bandwidths (i.e. 5 MHz being the narrowest, and 400 MHz being the widest) shall be sufficient for potential NR applications in 7 – 24 GHz range. This is motivated by the fact that the largest width of the 7 – 24 GHz frequency range of interest are significantly narrower compared to the largest width of Rel-15 FR2 operating band. Therefore, if the set of NR channel bandwidths is considered as sufficient for Rel-15 FR2 operating bands, it is also considered as sufficient for the NR operating in 7 – 24 GHz range.

NOTE: 	The above does not preclude any additional NR channel bandwidth requests in future, e.g. based on the operators request motivated by the spectrum utilization optimisation.

#### 5.7.8.2		Carrier aggregation

As 7 – 24 GHz range is considered to be a capacity booster due to its propagation characteristics, a typical envisioned CA scenario is expected to use component carriers from the 7 – 24 GHz range as the Secondary Serving Cells (while the Primary Serving Cells are expected to be allocated in FR1 operating bands). Therefore, carrier aggregation deployment is seen as suitable and applicable to 7 – 24 GHz deployment scenarios, considering both contiguous and non-contiguous spectrum allocations.

Referring to the NR UE specification in TS 38.101-3 [19], the inter-band CA between FR1 and FR2 was also considered in Rel-15. Such NR inter-band CA case is including at least one component carrier from FR1 operating band, and at least one component carrier from FR2 operating band. Extending such CA arrangement with the 7 – 24 GHz range, it can be envisioned that the following inter-band CA combinations could be further considered:

- FR1 component carrier(s) plus 7 – 24 GHz component carrier(s), or

- 7 – 24 GHz component carrier(s) plus FR2 component carrier(s), or even

- FR1 component carrier(s) plus 7 – 24 GHz component carrier(s) plus FR2 component carrier(s).

NOTE: the above is valid irrespective of the future conclusions on potential FR1/FR2 range extension(s).

Referring to the UE RF feature list [21], UE capability 2-2 (i.e. simultaneous reception or transmission with same or different numerologies in CA) was defined. Considering potentially different set of the supported numerologies among the considered bands in any of the three options above, this UE RF capability will also be applicable for the inter-band CA with the potential 7 – 24 GHz range bands.

#### 5.7.8.3		Dual connectivity

Similar to the CA motivation being the capacity booster, the Dual Connectivity deployment is seen as suitable and applicable to 7 – 24 GHz deployment scenarios.

Referring to TS 38.101-3 [19], large amount of inter-band DC configurations between FR1 and FR2 was already defined in Rel-15, as listed below. It is expected, that 7 – 24 GHz range will further extend the list of inter-band DC configurations.

- Inter-band EN-DC (E-UTRA/NR Dual Connectivity) including FR1 and FR2, considering configurations up to six bands.

- Inter-band NR-DC (New Radio Dual Connectivity) between FR1 and FR2, considering two bands configuration.

NOTE: Referring to applicability of minimum requirements in TS 38.101-3 [19], NR UE that supports inter-band NR-DC between FR1 and FR2 configuration shall meet the requirements for corresponding CA configuration, unless otherwise specified.

Referring to the UE RF feature list [21], UE capability 2-4 (i.e. simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD)) would also be applicable for the inter-band EN-DC combinations including 7 – 24 GHz range carriers.

# 6	NR UE

## 6.1	NR UE architecture

### 6.1.1	Mixed UE transceiver architecture

Transceiver architecture and technology selection contribute to the overall system performance. Unlike the FR1 and FR2, the 7 – 24 GHz range presents unique challenges to the transceiver architecture that covers the entire frequency range. Two types of architectures are presented above. Homodyne architecture was transceiver architecture of choice for frequency sub-range 1 while heterodyne architecture was selected for frequency sub-range 3. The IF frequency of the heterodyne radio can be selected in 7 - 10 GHz range such IF section of the heterodyne radio in 10 – 24 GHz range uses similar circuits as homodyne 7 – 10 GHz radio as shown in figure 6.2-1. The 7 – 24 GHz range may requires both types to cover the entire frequency range. One possible way is to apply the homodyne architecture for 7 to 10 GHz range while using heterodyne architecture for 10 to 24 GHz range. Mixed transceiver architecture maybe more appropriate for this frequency range since as an integrated device, transceiver may contain both lower range 7 to 10 GHz homodyne architecture portion as well as the IF transceiver for a higher range. The transceiver would share much of the components for the RF/IF section. Additionally, it may be suitable to extend the 7 to 10 GHz RF transceiver to cover sub 6 GHz range as well. However, this may add complexity and cost to the UE transceiver.

Figure 6.1.1-1: 7 – 24 GHz UE transceiver architecture example

### 6.1.2	Frequency sub-range 1

The homodyne or direct conversion architecture is widely used in sub-6 GHz, although it has the drawback on the IQ-imbalance, DC offset and degradation of transmitted signal resulting in degradation of EVM, the major advantage of direct conversion architecture is that the image problem is avoided because of the zero IF, and the requirement of a single LO realization is fulfilled. Benefiting from the technology improvement on PLL design, real time I/Q imbalance calibration and DC eliminating technology, the direct conversion architecture becomes a mainstream architecture utilized by UE. For 10 GHz, direct conversion architecture is highly possible be used. For LO generating, one solution is to use transceiver-localized frequency multipliers (xLO) to generate the target LO frequency by the frequency divider. It may become the bottleneck for 10 GHz choosing direct conversion.

### 6.1.3	Frequency sub-range 3

For 20 GHz, considering the increased phase noise and difficulties on PLL design for localized implementation, the UE reference architecture with IF conversion is highly adopted for frequency sub-range 3. The heterodyne architecture has an advantage in the phase noise requirements compared to homodyne architecture, since the phase noise is logarithmically proportional to the LO frequency having two sources of LO reduce the phase noise of the system. However, the PLL implementation is also a problem for this frequency point if direct conversion architecture is adopted.

## 6.2	NR UE requirements classification

NR UE RF requirement type is studied based on the frequency sub-ranges specified in clause 5.2. The study starts from the feasibility of the current RF technology and testability, e.g. antenna design, PA trends, filter, IC process and RF connector for conduct test.

- For the sub-range 1 (i.e. 7.125 – fboundary, low; where fboundary, low is within the frequency range 10 - 13 GHz, as in table 5.2-1), conducted RF requirements (FR1-like) is expected to be defined considering the antenna size and PA performance shown in clause 5.4.1.

- For the sub-range 2 (i.e. fboundary, low - fboundary, high; where fboundary, low is within the frequency range 10 - 13 GHz and fboundary, high is within the frequency range 16 - 18 GHz, , as in table 5.2-1), PAs could have the considerable performance as for the frequency sub-range 1 with advanced IC process, and there is limited space for UE to place more antenna elements. Depending on the frequency boundaries of the frequency sub-range 2, more flexibility is envisioned:

  - If frequency sub-range 2 reaches to 16 GHz, the conducted RF requirements (FR1-like) can be considered,

  - If frequency sub-range 2 reaches up to 18 GHz, the radiated RF requirements (FR2-like) can be considered,

  - Some combination of both approaches is not precluded.

- For the sub-range 3 (i.e. fboundary, high – 24.25 GHz; where fboundary, high is within the frequency range 16 - 18 GHz, , as in table 5.2-1), the PA performance (i.e. Psat and PAE) has reduced much compared with Pas operating below 18 GHz. Beam forming which can provide high directional gain on EIRP could be considered. The other ramification for the sub-range 3 is whether conducted RF requirement can be applied for this range. Since it is provided with the acceptable RF connector parameter operating at 26 GHz, the testability issue can be resolved. For antenna integration, Antenna-in-Package (AiP) has become an attractive technology for mmWave designs. This antenna integration technology is part of a System-in-Package (SiP), where the antenna and other components are designed and manufactured in separate substrates that are then integrated into the full system’s package. The benefit of this integration type is that it enables separate optimization and substrate selection for each component in the system. As it is not possible to separate the antenna from the UE and consider relevant transitions for this level of integration, OTA conformance testing is required.

The NR UE requirements classification is summarized in figure 6.2-1.

Fig 6.2-1 Frequency sub-ranges and the RF requirement type

## 6.3	UE RF requirements

### 6.3.1	UE RF technology aspects

The RF requirements for NR UEs are broadly classified based on whether the UE is tested in the conducted domain (“FR1 like”) or OTA (“FR2 like”). With a mobile phone used as the typical UE example, some clear criteria may be found to assess the “FR1 like” or “FR2 like” sub-ranges within 7.125 – 24.25 GHz regions. Nevertheless, other types of UE (CPE) may also need to be considered and an exact frequency limit between “FR1 like” or “FR2 like” is harder to define.

From UE point of view, RF system design must balance several considerations such as antenna size and polarization, semiconductor NF and phase noise associated, gain and linearity.

#### 6.3.1.1	Antenna technology

##### 6.3.1.1.1	Beamforming

Figure below shows the equivalent antenna array size in free space conditions what has been used in earlier co-existence studies and referred among others in [2]. From this Figure we can see that the antenna module doubles in size going from 28 GHz to 20 GHz and further doubles at 14 GHz. Some handhelds are bigger today so it is hard to set an absolute limit for the size and new antenna implementations may also surface. At the same time, pathloss halves from 28 to 20 and further halves when moving from 20 to 14 GHz.

Figure 6.3.1.1.1-1: FR2 type antenna size (free space) vs. operating frequency

Another example that also takes the antenna substrate into account is shown below. Taking the microstrip patch array design as an example, the patch width and ground plane extension can be calculated based on a common Rogers substrate: the element dimensions and array length (assuming 4 elements with half-wavelength spacing) can be calculated, as shown in figure 6.3.1.1.1-2.

Figure 6.3.1.1.1-2: Patch array dimensions vs. frequency

Antenna size increases at lower frequencies it scales down at higher frequencies and around 20 GHz the AIP technology is getting feasible and similar RF architecture as used in FR2 could be used. However, the antenna size at the highest frequencies of 7 – 24 GHz range is still larger than antennas of Rel-15 28 GHz solutions.

It is important to note that for frequencies close to 24 GHz the antenna array dimensions are similar to the FR2 (24.25 – 52.6 GHz) array dimensions: especially if the UE is a multi-band design which co-locates 28 GHz and 39 GHz antenna arrays in the same volume.  Thus, in a handheld UE supporting both FR2 and 7 – 24 GHz, an antenna array for the upper frequencies in the 7 – 24 GHz range competes for physical space in a handset with an FR2 antenna array, and it is not likely that UE FR2 coverage performance can be traded off to support beamforming arrays in the 7 – 24 GHz frequency range. As the wavelength increases with decreasing frequency, the antenna array size becomes prohibitively large, with the array at 7 GHz approaching a significant percentage of the entire length of a handset. Thus, from the perspective of potential array size, a radiated requirement with an antenna beamforming assumption may be challenging in handset form factor.

Smaller antenna arrays, e.g. 2×1, or individual antenna element could be used. However, taking the complexity of beam management into account, the benefit from using a beam steering antenna system with such a small number of elements need to be understood.

UEs that have possibilities to deploy larger antennas like CPE or cars can possibly utilize technologies that are not feasible for handheld devices in this 7 – 24 GHz frequency range.

##### 6.3.1.1.2	MIMO

Using spatial multiplexing or MIMO antennas offers higher flexibility in terms of antenna design than antenna arrays, since it does not require specific antenna radiation pattern, polarization or inter-element distance. Such flexibility also eases the multi-band antenna design. However, the performance of spatial multiplexing is limited by the channel property, which requires that the channel can offer a high number of ranks. In order to the investigate the possibility of using spatial multiplexing/MIMO antenna system on the UE at a medium frequency in 7 – 24 GHz band, a measurement campaign, as described below was carried out at 15 GHz.

The measurement was carried out in an indoor studio hall, where multiple objects with different materials exist (see figure below). Two transmit points (TPs) are mounted on the internal wall of the hall, and the UE is moved along a closed route within the hall, as shown in figure 6.3.1.1.2-2. Two UEs with smart-phone form factor and a reference UE are measured in this campaign, where each of them has four antenna ports. The two UEs with smart-phone form factor are identical, and are placed vertically and horizontally, respectively. Each UE is equipped with 4 single polarized antenna elements which point to four different directions, as shown in figure 6.3.1.1.2-1. The UE with smart-phone form factor is placed both vertically and horizontally during the measurement. The reference UE has omni-coverage with dual-polarization on the horizontal plane. Each TP has two beams with dual polarizations for each of them.

Figure 6.3.1.1.2-1: The measurement environment in the hall space and the illustration of the UE antenna radiation patterns at 15 GHz

The measured channel rank is shown in figure 6.3.1.1.2-2. It can be observed that for such an environment, regardless of the UE antenna configuration, the rank of the channel is larger than 1 in most of the time, and in more than 60% of the time the rank of the channel reaches 3 or higher. This measurement result shows that the possibility that spatial multiplexing through UE pattern diversity is valid, at least for 15 GHz band. Optimization of antenna design on both UE and TP side may also further increase the rank of the channel.

Figure 6.3.1.1.2-2: The measurement route and the measured rank number distribution

#### 6.3.1.2	Propagation aspect for UE

##### 6.3.1.2.1	Indoor propagation at 15 GHz

In addition to the antenna configuration, the deployment scenario of mobile networks is limited by the propagation channel as well. From the same measurement as described above, we can also capture some insight on how to deploy a cellular network at 15 GHz. The RSRP at the reference UE antennas is also shown in figure 6.3.1.2.1-1. It can be observed that when the UE moves to the NLOS region, the RSRP drops about 30 – 40 dB.

Figure 6.3.1.2.1-1: The measured RSRP of the reference UE over the measurement route in the hall

The user body shadowing effect is also critical. Therefore, we have also measured the shadowing loss due to the user body blockage at 15 GHz. Measurements were performed in an anechoic chamber, where a real user was holding a UE in data mode portrait position. The measured far field radiation pattern of antenna 2 in the UE is shown in figure 6.3.1.2.1-2. A clear user body shadowing region can be observed, and a loss due to the user body blockage around 20 – 25 dB can be observed in this measurement.

This significant diffraction and shadowing losses due to the objects and user body will limit the deployment scenario of cellular networks at 15 GHz.

Figure 6.3.1.2.1-2: The measurement setup with a real user; the measured radiation pattern with and without the user

##### 6.3.1.2.2	Outdoor propagation at 15 GHz

For outdoor scenario, buildings can imply a much more dramatic blockage loss compared to the user body, and thus limit the coverage of the cell. An outdoor measurement at 15 GHz is carried out in Kista, Stockholm, Sweden, which is an urban environment with multiple office buildings. The outdoor coverage is measured with a test system which consists of two transmit points (TPs) and one mobile terminal. The mobile terminal antenna, installed on top of a van at a height of 2.9 m (see Fig. 6.3.1.2.2-1), has roughly omni-directional coverage in horizontal plane. The two TPs are installed on the walls of two office buildings as shown in Fig. 6.3.1.2.2-1, at a height of 8.5 m (TP1) and 12 m (TP2), with an inter-TP distance of 82 m. Each TP has an antenna with a maximum gain of 15 dBi, azimuth half power beam width (HPBW) of 90º, and elevation HPBW of 8.6º. The detail of the measurement and more results can be found in [47].

Figure 6.3.1.2.2-1: Photo of the test area, antennas for mobile terminal and TPs’ location [47]

Fig. 6.3.1.2.2-2 shows the received signal strength for each TP over streets and a scanned square. As indicated by the yellow arrows, the coverage within LOS is quite good where the signal strength is above -65 dBm. However, the coverage is limited in NLOS conditions where the signal strength quickly decreases towards the noise floor of the test system, i.e. -75 dBm.

Figure 6.3.1.2.2-2: Received signal strength and outdoor coverage area overview from TP1 (left) and from TP2 (right). Yellow arrows illustrate TPs’ positions and horizontal pointing directions [47]

The same scenario as described above has been modelled in order to carry out a 3D ray-tracing simulation including antenna designs in a smartphone form factor UE device at both 15 GHz and 28 GHz. The radiation pattern of the UE antennas used in the ray-tracing simulations comes from the measurements of real mockups. The ray-tracing simulation setup is shown in Fig. 6.3.1.2.2.-3. The ray-tracing simulation has been verified with measurements as reported in [48]. The UE antenna configuration at 15 GHz is the same as presented in as in Fig. 6.3.1.1.2-1, but antenna 3 is not used in here. The 28 GHz antenna designs are the same, but scaled up to higher frequency. The device is placed vertically without rotation during the simulation. The ray-tracing simulations can offer a better dynamic range than the measurements and therefore better can indicate feasible deployment scenarios.

Figure 6.3.1.2.3-3: The 3D model of the study area in ray-tracing simulations [48] and the schematic pictures of the proposed antennas in the phone prototype

UE antenna 1                                                        UE antenna 2                                                           UE antenna 4

(a)

UE antenna 1                                                        UE antenna 2                                                           UE antenna 4

(b)

Figure 6.3.1.2.4-4: The coverage overview with three different antenna design in a mobile device at (a) 15 GHz and (b) 28 GHz [48]

The simulated coverages with UE antenna 1, 2, 4 at both 15 GHz and 28 GHz are shown in Fig. 6.3.1.2.4-4, where, in this case, the base station antenna is simplified as an omni-directional antenna located at the same position as the TP1 above. Comparing 15 GHz to 28 GHz, it can be observed that the smaller free space path loss at 15 GHz (5 dB) gives a better coverage in the LOS and the LOS-NLOS transition regions.

In this ray-tracing simulation, there is neither roughness of the building surface nor random scatters included. Therefore, the number of multiple components in the channel is likely to be underestimated. Even so, it was observed that there is a difference between the three antenna ports in the UE, especially in the LOS-NLOS transition region. This means there is a large potential for utilizing spatial diversity gain in the UE to increase the cell coverage. In addition, for a rich scattering environment (which is likely to be the case in dense urban micro), the number of multiple components may still be high, which offers the possibility to use spatial multiplexing to increase the data rate.

#### 6.3.1.3	RF front end technology

Current RF front-end technology used for > 3.3 GHz TDD bands and Wi-Fi can be extended at least up to 12 GHz.

GaAs or Si based power amplifier technologies used in FR1 for NR and Wi-Fi can support up to 1W peak power at reasonable gain thus enabling reasonable peak currents. Still, some antenna gain via UL MIMO would be preferable as it would lower the total power and higher EIRP are achieved.

Technology used for LNA and switches are already suitable for both FR1 and FR2 with only more aggressive lithography used at higher frequencies, these can naturally support any approach within the 7.125 – 24.25 GHz range. Below 12 GHz it is feasible that a switch supporting branches for FR1 frequencies would support one branch up to 12 GHz.

##### 6.3.1.3.1	Active technologies

SOI CMOS is currently the work horse for the switch and LNA RFFE functions in FR1 for both cellular and Wi-Fi systems. Especially since it associates good noise figure/Gain and good switch losses/isolation on the same die for the main antenna Tx/Rx modules and also for the diversity and MIMO antennas Rx modules. It should be noted that SOI CMOS is also used for PA / SWT / LNA / phase shifter functions in FR2 with superior performance compared to bulk CMOS. So the coverage of the LNA and switch functions in the 7.125 – 24.25 GHz range is only a matter of picking the right node.

In order to illustrate the options that can be picked from, table 6.3.1.3.1-1 recapitulates the key figures of merit for LNA and switches versus nodes in SOI CMOS:

- Fmax in GHz provides a measure of the achievable power gain which is essential in LNA and PA designs. A ratio of 5 (and preferably > 10) between Fmax and the frequency of operation is desirable.

- RON * Coff in fs is key for switch performance as RON dictates the losses in ON state and Coff the isolation in off state, the lower the value, the better and the higher frequency of operation can be targeted.

- Transistor voltage is important to gage the output power capability for PAs and power handling for switches. In general, the voltage capability reduces with higher Fmax which is consistent with lower output power capability at higher frequencies of operation.

- Gain and Nfmin at 5 GHz is regularly used as our benchmark for LNA design, with typical values shown in the table not accounting for variation in PVT.

Table 6.3.1.3.1-1: Key figures of merit vs SOI CMOS node

| SOI CMOS data | Gate lithography (nm) |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | 180 | 180/130 | 130/65 | 45 |
| Fmax (GHz) | 150 | 230 | 250 | 320 |
| Voltage (Vdd) | 1.8 | 1.8 | 1.8 | 1.2 |
| RON * Coff (fs) | 150 | 120 | 90 | <80 |
| Gain@5 GHz (dB) | 25 | 28 | >30 | >30 |
| Nfmin@5 GHz (dB) | 0.36 | 0.3 | 0.25 | <0.25 |

NOTE: 	Nfmin above does not take to account the whole receiver chain including all possible losses and needed trade-offs.

SiGe BiCMOS technology has also been used for Wi-Fi RF front-end in 5 GHz band especially because it has enabled compact single band TDD PA / switch / LNA Tx/Rx modules. The Bipolar device has a better output power capability than CMOS due to higher voltage (>9 V) handling for Pas (>24 dBm) while the bulk CMOS device provides reasonable switches. LNAs can be implemented in either HBT or CMOS depending on the linearity and NF trade-off. LNA implemented in HBT has demonstrated NF similar to RF SOI technology. This technology has also been widely used as a cellular RFFE technology in FR1 and FR2. In FR2 it has capability to monolithically integrate TDD PA / switch / LNA / phase shifter / VGA in HBT with ‘light’ digital functions like control interface, memory, DFX, etc. in CMOS.

Similar to SOI CMOS, table 6.3.1.3.1-2 illustrates the key figure of merit for BiCMOS technologies for different nodes:

- Fmax * BVCEO provides a measure of power capability and gain for PA designs. It is a combination of the two first parameters of the SOI CMOS table.

- RON * Coff in fs is key for switch performance as RON dictates the losses in ON state and Coff the isolation in off state, the lower the value, the better and the higher frequency of operation can be targeted.

- Gain and Nfmin at 5 GHz is regularly used as our benchmark for LNA design, with typical values shown in the table not accounting for variation in PVT.

- State-of-the-art low noise transistors in SiGe HBT technology have nominal minimum NF values of 0.25 dB, 0.55 dB, 0.8 dB and 1.1 dB at 10 GHz, 20 GHz, 30 GHz and 40 GHz, respectively.

Table 6.3.1.3.1-2: Key figures of merit of SiGe HBT with indication of CMOS node

| BiCMOS Data | Gate lithography (nm) |  |  |  |
| --- | --- | --- | --- | --- |
| Generation | 1 | 2 | 3 | 4 |
| CMOS node | 350 | 350/250 | 180/130 | 130/65 |
| Fmax (GHz) | 30-90 | 110-200 | 200 | 320 |
| Voltage (Vcc) | - | 3.3/5 | 1.8/3.3/5 | 1.1/3.3/5 |
| RON * Coff (fs) | - | 250 | 200 | <200 |
| Gain@5GHz (dB) | 13 | 14 | 15 | 16 |
| Nfmin@5GHz (dB) | 0.9 | 0.6 | 0.4 | <0.25 |

NOTE: 	Nfmin above does not take to account the whole receiver chain including all possible losses and needed trade-offs.

Here again, multiple choices are available and for the 7 – 24 GHz range and 180/130 nm nodes provides a good compromise for PA / switch / LNA function till and including FR2 band. Above this frequency, if lower PA power is acceptable, SOI CMOS offers a better overall compromise.

When uncompromised performance in terms of output power, bandwidth and efficiency is needed, III-V GaAs pHEMT Pas, and at FR1 frequencies, GaAs HBTs are dominating in RF front-end modules. For low voltage (3 - 5 V) applications, GaAs HBT perform well up to >15 GHz and unless much higher voltage is available (12 V), GaAs HBT is on par with GaN which targets higher power applications like small cells or infrastructure.

One way to compare the different technologies is to look into achievable average output power performance for Wi-Fi OFDM at 6 GHz and extrapolate to 12 GHz. The linearity level we would design for is -30 dB EVM which is quite comparable to 30 dBc ACLR linearity level for NR with CP-OFDM:

- Bulk CMOS: 16 dBm (3.3 V) [@xx% peak PAE]

- SiGe BiCMOS: 24 dBm (3.3-5 V) @ [37.5% peak PAE]

- SOI CMOS: 20 dBm (3.3 V) [@xx% peak PAE]

- GaAs HBT: 26 dBm (3.3 – 5 V) [@xx% peak PAE]

- GaN HEMT: 33 dBm (12 V), similar to GaAs HBT at 5 V [@xx% peak PAE]

- For DFT-s-OFDM QPSK output power capability is about 2 dB higher

III-V based Pas have at 10% higher efficiency and >6 dB higher power capability, when compared to CMOS, which is essential for battery power especially for applications with significant post-PA losses such as FR1 UE supporting large number of bands and band combinations.

There is reported evidence that III-V based Pas can be designed inherently broadband whilst preserving good efficiency, linearity and output power which will thus address a wide range of perceivable band combinations without needing too many separate PA devices. ACLR could be better than -35 dB at +26 dBm. The channel bandwidth of III-V based PA can be beyond 500 MHz while the frequency coverage can be made larger than 40%.

SiGe HBT of BiCMOS has high Ft and Fmax compared to GaAs HBT. In FR1 band, PA in this technology has demonstrated performance comparable to that in GaAs HBT between 3 and 7 GHz. SiGe HBT Pas have high gain and wideband response because of high Ft/Fmax. Optimization in doping profile has enabled variants with higher breakdown voltage. In FR1 band SiGe HBT technology based PA have similar PAE as GaAs HBT for cellular and WiFi applications. At higher frequency, e.g.  12 GHz, no significant degradation is expected from active device because operating frequency is significantly lower than Ft/Fmax. However, some degradation is expected from back-end technology due to higher substrate losses at higher frequency. Minimum NF of LNA in this technology lies between 0.25 dB and 0.65 dB in 7 – 24 GHz range and a 2-stage LNA can produce small signal gain in excess of 25 dB. TDD switch in RF SOI is better than that in SiGe BiCMOS. At 12 GHz output power capability may be reduced by 1 dB but for technologies where power gain is reduced, the Power Added Efficiency (PAE) would suffer as it accounts for the power delivered at the input of the PA stages. If high power capability (>23 dBm) at 12 GHz is needed, GaAs and SiGe BiCMOS are the preferred choice to control the battery current to reasonable levels. At lower power levels, and when further integration on the die is needed both SOI CMOS and SiGe BiCMOS offer good performance/integration trade-off.

III-V pHEMT based LNAs can achieve a NF below 2 dB within the 7 – 24 GHz frequency range, while the gain of a two stage LNA can be above 20 dB.

##### 6.3.1.3.2	Passive technologies

LC filters using printed elements in a module or a passive substrate should also be feasible up to 12 GHz but would require special attention of the ground design. High Q filtering is limited but diplexing and harmonic rejection function should be feasible.

Technologies used in FR2 are obviously able to support the 7.125 – 24.25 GHz frequency range but at lower frequencies, integrating matching networks and phase shifters has a significant impact on size and using more advanced technologies, it may impact the cost negatively.

Although BAW filters are discussed above 10 GHz in literature, above 8 GHz maintaining high Qs will be difficult as thinner acoustic layer (AlN has a speed of ~10 km/s) or development of new material or modes would be needed. With current technologies the piezo + electrodes stack thickness should be smaller than 1um at 10 GHz. Thus, similar to the frequencies above 3.3 GHz in FR1, filter performance should be relaxed compared to those used in FDD bands to be essentially feasible with LC filter, while BAW can still be used to provide notches at some particular frequencies.

Miniature ceramic dielectric filters or MEM cavity-based filters can operate in the 7 – 24 GHz range but are usually too bulky for a smartphone.

Above 7 GHz, the use of discrete passive components like inductors and capacitors SMTs isn’t an option for RF matching and decoupling due to self-resonance issues. It can still be used for lower frequency like DC supply decoupling, but it needs to rely on integrated passives technologies like LTCC or IPD for critical RF passive functions.

Integrated Passive Device (IPD) LC designs are widely used in FR1 to integrate critical harmonic filters or PA matching. IPD works reasonably well up to 12 GHz provided capacitor Qs are improved, which is feasible by adapting the MIM (Metal Insulator Metal) capacitor dielectric thickness and/or material. Figure illustrates an IPD LC filter design at 6-7 GHz providing 40 dB rejection for cellular bands up to 5 GHz. It is also to be noted that transmission line designs can also be realized on IPD which allows designs above 12 GHz, furthermore some of these design techniques are also applicable on the active dies but with lower Q.

Figure 6.3.1.3.2-1: Simulation of an IPD filter operating at 7 GHz

As an example on the other filter technology simulated S21 of the LTCC filter is shown in figure below. The insertion loss is less than 2.5 dB for 2 GHz wide bandwidth and also the stop band attenuations are better than -20 dB at relative offset of ~5% (0.5 GHz) from the pass band edge. The level of stop-band attenuation is still far from duplex-filter performance, but it should be noted that the relative bandwidth of the designed filter is also several times wider than band-width of the current duplex filters, which is usually no more than 4% (100Meg/2.5GHz). The achieved attenuations are sufficient for providing protection for adjacent frequency bands and for protecting own receiver from interference from these bands. The results shown are based on the simulations and are not fully taking into account the manufacturing tolerances and ambient conditions.

9.75 / -32.8

13.25 / -41.8

12.75 / -21.2

10.25 / -2.2

12.25 / -2.3

Figure 6.3.1.3.2-2: Simulated S21 for the LTCC filter

- For the 7 – 24 GHz range, single-pole double-throw (SPDT) switches are expected to be used in many occasions of systems. Currently, the III-V based switch component can achieve reported insertion loss figures of around 1 dB at 30 GHz in FR2. Therefore, an even better performance can be obtained in lower frequencies. Insertion loss below 0.8 dB with isolation larger than 20 dB can be achieved by III-V based SPDT switch within the 7 – 24 GHz frequency range.

#### 6.3.1.4	Testability considerations

FR2 UE requirements carry with them an additional layer of complexity associated with beam management procedures both for the network and the UE, and additionally antenna array RF architecture for the UE. This complexity provides a motivation to determine whether conducted UE RF requirements over the entire frequency range of 7.125 to 24.25 GHz are feasible.

A reasonable starting point for this analysis is in the form of a connector and probe capability study, as shown in table 6.3.1.4-1.

Table 6.3.1.4-1: Antenna connector and probe capability

| Supplier | Connector dimensions (mm) | Frequency range (GHz) | Insertion loss (dB) | VSWR |
| --- | --- | --- | --- | --- |
| Supplier A | < 2x2 | 0 – 12 | < 2 | < 1.6 |
| Supplier B | < 3x3 | 0 – 11 | < 2 | < 1.5 |
| Supplier C | < 3x3 | 0 – 11 | < 2 | < 1.5 |
| Supplier D | < 2x2 | 0 – 15 (NOTE 1) | < 2 | < 1.6 |
| Supplier E | < 4x4 | 0 – 26.5 | < 0.5 | < 1.25 |
| Supplier F | < 2x2 | 0 – 11 | < 0.3 | < 1.4 |
| Supplier G | < 5x2 | 0 – 26.5 | < 0.4 | < 1.4 |
| NOTE: Operation up to 12 GHz per spec sheet; operation up to 15 GHz feasible based on experimental data. |  |  |  |  |

When evaluating the connector from Supplier D, it was possible to verify experimentally that the connector’s use up to 15 GHz (i.e. 3 GHz above the spec sheet) is feasible. Evaluations of connectors from Supplier E and Supplier G have revealed that the full range of the frequencies in this study item’s scope is supported; however, the connector dimensions pose a mechanical challenge to their integration into a handset form factor. In this aspect, an outlook on future capabilities in terms of connector dimensions is needed rather than a survey of currently available dimensions.

Market need tends to identify innovation trends, and it is feasible that a convergence of an actual band plan, deployment scenario, and real market need can promote the industry to further innovate the connector technology to enable integration into a handset form factor.

Radiated measurements are needed if beamforming is needed to meet the system performance requirements or conducted measurements are otherwise insufficient to verify the system performance.

For frequency larger than 20 GHz, the AIP technology has become a mainstream choice for RFIC and antenna capsule. It implements antennas with dies. It can improve the mmW signal integrity and help forming an array of antenna elements to get more directional gain. Since antennas cannot be removed from the package, testing has to be considered with OTA measurements. This is the reason NR FR2 only define the OTA RF requirements.

# 7	NR BS

## 7.1	General

For the purposes of this SI, the RF technology analyses of the NR BS operation in 7 – 24 GHz frequency range are limited to the single-band operation, only.

## 7.2	BS architecture and requirements classification

### 7.2.1	Reference architecture

For NR BS two different architectures have been defined. The architectures different with respect to defined requirement anchor points and requirement applicability. A base station can into three main components:

- Transceiver Unit Array (TRXUA)

- RF Distribution Network (RDN)

- Antenna Array (AA)

In figure 7.2.1-1, the architecture for requirement set category H is visualized, where requirements are defined at TAB and RIB.

Figure 7.2.1-1: BS architecture relevant for requirement set category H

In figure 7.2.1-2, the architecture for requirement set category O is visualized, where all requirements are defined as OTA requirements at RIB.

Figure 7.2.1-2: BS architecture relevant for requirement set category O

The Release 15 NR basestation specifications include 3 types of basestation:

- The conducted basestation (BS type 1-C) type refers to a non-AAS BS architecture. The basestation does not include an antenna. The antenna is built separately and is likely to correspond to a passive sector antenna for a macro basestation. For micro and indoor basestations the antenna may have a wider coverage angle. All RAN4 requirements for conducted basestations are specified at each individual antenna connector. BS type 1-C is applicable to FR1 only.

- The hybrid basestation type (BS type 1-H) is an AAS basestation that has connectors or other means for conducted testing of individual transceivers. An AAS basestation has an integrated antenna. BS type 1-H basestations comply with two far field OTA requirements as well as conducted requirements. Unlike BS type 1-C, some conducted requirements are specified as a sum across multiple connectors. BS type 1-H is applicable for FR1 only.

- The OTA basestation is an AAS basestation type (BS type 1-O for FR1, BS type 2-O for FR2) that has only a radiated interface. All requirements are specified OTA, as either directional, TRP or co-location type requirements. The OTA BS type is applicable for both release 15 frequency ranges.

### 7.2.2	Requirement sets

The NR RF core specification defines multiple requirement sets. The requirement set support is different for FR1 and FR2. In table 7.2.2-1, the requirement set defined in NR are listed. Together, the requirement set category and frequency range defines the NR BS type.

Table 7.2.2-1: Requirement set categories for NR BS

| Requirement set category | Applicability | Description |
| --- | --- | --- |
| C – conducted | Antenna connector(s) for non-AAS architecture or TAB connector(s) for AAS BS architecture; applicable to FR1 only | All requirements are defined at the RF connector. This is the traditional approach used e.g. for E-UTRA in TS 36.104 [24] or for MSR in TS 37.104 [25]. |
| H – hybrid | TAB connector(s) or RIB depending on requirement; applicable to FR1 only | All requirements defined for requirement category C is applicable. However, emission requirements are scaled to capture the whole AAS system and TX IMD is extended. In addition, two OTA requirements are defined for output power and sensitivity. This category is referred to as hybrid, since both conducted, and OTA requirements are included. This requirement set category was introduced in TS 37.105 [10] for AAS BS. |
| O – OTA, radiated | RIB; applicable to FR1 and FR2 | All requirements are defined as OTA requirements at the RIB. This requirement set category was introduced in TS 37.105 [10] for AAS BS. |

It is clear that xFR-O requirements will be needed for the 7 – 24 GHz frequency range. Type xFR-C BS (or non-AAS) will clearly be limited by lower antenna gain (compared to the beam forming architectures) and the larger path loss in this frequency range will result in smaller cell sizes than for FR1. However, at this stage, xFR-C and xFR-H requirements cannot be ruled out for BS operating within frequency sub-range 1 and sub-range 2.

Comparing the xFR-H and xFR-O AAS BS types, the principle differences from a testing perspective are that for 1-H, there is no need for out of band OTA testing facilities or for TRP testing. TRP emissions and conducted emissions limits could be set to be the same. In addition to this, most other TX requirements (i.e. signal quality and power dynamics) have the same value whether tested conducted or OTA.

For the receiver, for FR1 a method to relate conducted and radiated requirements has been established. For FR2, there is no such methodology as there are no conducted requirements. For the 7 – 24 GHz range, depending on the frequency and the expected receiver types, the FR1 approach may be applicable, or further work may be needed to develop a new approach or in the worst case, no mapping between radiated and conducted requirements may be possible. (In the latter case, it may of course be possible to derive conducted and radiated requirements without relating them). Depending on the applicable new band, if introducing both conducted and radiated requirements a future WI may need to take such factors into account.

Demodulation requirements for up to 2RX map directly between conducted and OTA. OTA testing of >2RX demodulation requirements is not feasible for FR1 or FR2 and will not be feasible in 7 – 24 GHz range. An assessment of the link budget in applicable test chambers for the frequency in question is needed to ensure OTA testability feasibility of demodulation requirements at sufficiently high SINR at the BS receiver. BS demodulation requirements that are not OTA tested may still be specified as conducted only. Within these constraints, demodulation requirements are possibly for any BS type in 7 – 24 GHz range. Therefore, discussion of further details of the BS demodulation testing in 7 – 24 GHz range is deferred to the related future WI.

### 7.2.3	Antenna topologies

The antenna array (AA) consists of M x N x 2 antenna elements (AE) placed is a certain lattice, arranged in M rows and N columns of cross-polarized elements. The signals from the AA are mapped in the RDN creating different antenna topologies, as presented on examples in figure 7.2.3-1. The RDN mapping is creating sub-arrays, where the radiating characteristics of a sub-array is different to single antenna elements.

Figure 7.2.3-1: Example RDN mappings

Depending on intended coverage scenarios different types of RDN mappings are foreseen for the frequency range 7 – 24 GHz. For BS type xFR-H and BS type xFR-O, the OTA RF characteristics defined for requirement set category H and requirement set category O is declared by the base station manufacturer in terms of full array capability as well as sub-array or element capability, see TS 38.141-2 [6], clause 4.6.

### 7.2.4	Array antenna model

In Table 7.2.4-1, the parameters used by the parameterized array antenna model are described. Based on AAS base station architecture in clause 7.2.1 (i.e. the BS type xFR-H and BS type xFR-O) and deployment scenarios envisioned for the 7 – 24 GHz range, different parameter sets are required to model an AAS base station.

Table 7.2.4-1: Parameters of the parameterized array antenna model

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Horizontal HPBW | 3dB | Degrees |
| Vertical HPBW | 3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Array element loss | LE | dB |
| Number of radiating elements rows and columns | (M, N) | Integer |
| Horizontal element separation | dh | m |
| Vertical element separation | dv | m |
| Electrical down-tilt angle | etilt | Degrees |
| Electrical scan angle | escan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 7.2.4-2.

Table 7.2.4-2: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max-+Aθ,φ | dBi |
| Element peak gain | GE,max=DE,max-LE , where the peak directivity DE,max is calculated from given values on 3dB, 3dB, dh and dv | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πn-1dvλcosθ+m-1dhλsinθsinφ wm,n=1MNexpj2πn-1dvλsinθetilt-m-1dhλcosθetiltsinφescan | dBi |

To conserve complexity the model is created so that the element is gain normalized, instead of the composite array pattern. As a consequence, parameters cannot be selected arbitrarily, since parameters are dependent on each other. The intension with the model is to model absolute gain patterns correctly without full pattern directivity normalization. To model absolute gain, parameters must be selected carefully, if not the model produces nonphysical and incorrect gain response.

When array antenna parameters are selected for the array antenna model it is preferable to consider physical aspects such as the gain/area relation and gain/beamwidth relations by checking following aspects in given order:

- The considered deployment scenario and coexistence situation will give the appropriate coverage range requirement for the horizontal and vertical domain.

NOTE 1: 	For analysis of the NR deployment scenarios considered for the 7 – 24 GHz range, refer to clause 5.6.

- From the coverage ranges and deployment scenario the required antenna gain can be determined, from which the array antenna geometry can be determined in terms of number of rows (M), the number of columns (N).

- From the coverage ranges the array antenna steering capability can be determined in terms element separations (dv, dh).

NOTE 2:	 The element separations dv and dh is the distance between radiating elements in the array antenna. The RDN can be used to create sub-arrays to optimize coverage. When sub-arrays are modelled, parameters can be selected to model the sub-array as a radiating element, in this case dv and dh is the distance between sub-arrays in the antenna array.

- From the given array lattice the element parameters can be considered with respect to the given area for a single element. The element peak gain (GE,max) and half power beamwidth product (3dB and 3dB) depend on each other and must be selected together to maintain accurate model gain response. The element loss (LE) needs to be included when the element peak gain is determined. Select parameter values for beamwidth based on the following two parameters checks:

  - Check the peak element directivity (DE,max) with the unit area available for a single element in the array lattice, as described in Eq. 7.2.4-1.

  - Check the peak element directivity (DE,max) with the half-power beam width product (3dB and 3dB), as described in Eq. 7.2.4-2.

- The model gain is guaranteed by an element peak directivity normalization directivity (DE,max) described in Eq. 7.2.4-3. The peak element gain GE,max is calculated based on Eq. 7.2.4-4.

The peak element directivity (assuming no losses for a given antenna aperture area) can be expressed as:

DE,max≤10log104πdhdvλ2		(Eq. 7.2.4-1)

Also, the peak element directivity for a given wide symmetrical beam can be approximated by:

DE,max≈10log10Kφ3dBθ3dB		(Eq. 7.2.4-2)

, where K is a factor that depends on the element properties. For single elements with symmetrical large beamwidths, K = 52525 is appropriate, while for sub-arrays with narrower beamwidth characteristics, K = 32400 is more appropriate. Depending on the element characteristics the relation between element peak gain and the half power beam width product is different as described in [65].

To be exact it is recommended to select element parameters, where the peak element gain is determined by calculating the directivity from a given geometry including beam widths. The element directivity can be calculated based on the pattern described by Table 7.2.4-1 in dBi as:

DE,max=10log104πAθ,φAlinθ,φmax-ππ0πAθ,φAlinθ,φsinθdθdφ		(Eq. 7.2.4-3)

, where Alin(,) is defined in linear scale as:

AAlinθ,φ=10Aθ,φ10		(Eq. 7.2.4-4)

## 7.3	BS classes

The BS to UE minimum coupling loss of each NR BS class as defined in TS 38.104 [5] is the same as that of UTRAN in TS 25.104 [60] or E-UTRA in TS 36.104 [24], while the BS to UE minimum distance of each NR BS class is calculated using the path-loss formula in TR 25.951 [61]:

L=38.25+20log10d

Which is derived from the free-space path-loss formula as in TR 25.942 [22]:

L=20log104πdfc

Where:

- d is the BS to UE distance in meters,

- c is the speed of light in m/s,

- f is the carrier frequency in Hz.

For FR1 studies the carrier frequency was assumed to be 2 GHz. For FR2 it was decided to use the same definition of BS classes using the same set of minimum distances and minimum coupling losses as for FR1.

The considered base station classes are Wide Area Base Stations, Medium Range Base Stations and Local Area Base Stations. The associated deployment scenarios (as discussed in clause 5.6) for each class are exactly the same for BS with and without connectors.

For BS type xFR-O, BS classes are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum distance along the ground equal to 5 m.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum distance along the ground equal to 2 m.

For BS type xFR-C and BS type xFR-H, BS classes are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum coupling loss equal to 70 dB.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum coupling loss equals to 53 dB.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum coupling loss equal to 45 dB.

Considering the 7 - 24 GHz frequency range is between FR1 and FR2 in the frequency domain, it is logical to use the same set of minimum distances and minimum coupling losses for the BS classes within the 7 - 24 GHz frequency range.

## 7.4	BS RF requirements

### 7.4.1	Transmitter requirements

#### 7.4.1.1	Tx requirements overview

Summary of the conducted and radiated Tx requirements specified in Rel-15 for the NR BS is presented in this clause. More detailed elaboration on the motivation on selected requirements is provided in dedicated clauses below.

All the findings captured for the conducted requirements in table 7.4.1.1-1 and related clauses below are considered to be applicable to the sub-range of the 7 – 24 GHz for which the conducted requirements will be found to be feasible during related WI.

While radiated requirements are considered to be applicable to the whole 7 – 24 GHz range, their definitions, values and levels may differ across the 7 – 24 GHz range.

Table 7.4.1.1-1: Overview of conducted Tx requirements for NR BS in 7 – 24 GHz range

| Tx requirement |  | Conclusions from SI | Items to be completed in related WI |
| --- | --- | --- | --- |
| Base station output power |  | 2.0≤Pconducted_ccuracy_7to24GHz For BS type xFR-H system, if the system is based on an BS type xFR-C system then the requirements will be the same as BS type xFR-C. If the BS type xFR-H system is derived from an xFC-O system then the conducted accuracy will need to be investigated in a different manner. Some indicative PA output power levels were discussed but BS output power depending on the intended deployment scenario was not investigated as no specific frequency band was specified. | Decide which requirement sets (BS type xFR-C, BS type xFR-H) are to be introduced. If agreed to be introduced, for BS type xFR-C systems in the 7 – 24 GHz range, the accuracy should be examined once the operating frequency bands are known. Decide if BS type xFR-H requirements are to be based on BS type xFR-C or BS type xFR-O (possibly depending on operating band). |
| Output power dynamics | RE power control dynamic range | If unwanted emission limits are not increased compared to FR1 limits, then the RE power control dynamic range requirements for QPSK and 16 QAM can be reused for the applicable 7 – 24 GHz sub-range. No requirement for 64QAM. | For sub-range(s) applicable to conducted requirements, if unwanted emission limits are increased compared to FR1 limits, then the RE power control dynamic range may require to be revisited. Decide if requirement for 256QAM is needed. |
|  | Total power dynamic range | Same requirement derivation methodology as for FR1, based on the set of supported channel bandwidths and SCS. | Derive the requirement for sub-range(s) applicable to conducted requirements. |
| Transmit ON/OFF power | Transmitter OFF power | There may be conducted requirement defined for applicable sub-range(s). For NR BS with conducted only requirements where the TX and Rx are separate it may be necessary to provide an additional conducted Tx OFF requirement (or have an appropriately lower level). | Potential conducted co-location OFF levels to be decided based on co-location scenarios and isolation. |
|  | Transmitter transient period |  | Transient period depends on system scenarios in WI. |
| Transmitted signal quality | Frequency error | Requirements can be reused from FR1. |  |
|  | Modulation quality | QPSK, 16QAM and 64QAM core requirements can be reused from FR1. | Define EVM window length and equalizer requirements. Define 256QAM requirements. |
|  | Time alignment error | Requirements for the MIMO transmission and for the inter-band CA (with or without MIMO) can be reused from FR1. | Derive intra-band CA TAE requirements, based on the set of the supported SCS. |
| Occupied bandwidth |  | Single carrier and CA requirements can be reused from FR1/FR2. |  |
| ACLR |  | ACIR and hence BS ACLR are frequency dependent. In addition the distribution of the ACIR budget between BS ACLR and UE ACS varies over frequency with the ACLR allocation increasing with frequency as PA linearity becomes harder to achieve. It is also important to consider the deployment scenarios which are identified for identified bands as these are very important parameter in the co-existence analysis. The 7 – 24 GHz region covers almost 3 octaves so there may be more than one ACIR (and hence BS ACLR) requirements over the 7 – 24 GHz range. | Co-existence simulations are carried out on specific bands (or ranges) as they are identified. The split between BS ACLR and UE ACS should also be decided on a frequency sub-range specific basis based on the relative difficulty in implementing each in the BS and UE respectively. |
| Operating band unwanted emissions |  | OBUE was discussed in SI but no conclusion around the OBUE frequency domain and possible mask shape was achieved. Higher ΔfOBUE, than in FR1 needs to be considered, probably with an approach similar to FR2. Existing mask shapes could also be a starting point. | To be completed in the WI when frequency bands are specified in 7 – 24 GHz range. Also consider the OBUE mask. |
| Transmitter spurious emissions | General transmitter spurious emissions | Regulation defines Category A and B limits, but Category B limits for AAS BS operating in 6 to 24.25 GHz are presently not defined ERC Recommendation 74-01 [34]. | A starting point for those limits could be the present limits above 24.25 GHz, but the work defining them will take place in ECC. |
|  |  | Protection of the BS receiver of own or different BS |  | For a 7 – 24 GHz system, the noise figure and hence the wanted signal power level for the receiver sensitivity requirement will be higher compared to those in FR1 and hence the co-location power level will be similarly higher. |  | Determine; appropriate antenna port isolation for specific band co-location scenarios, receiver noise figure and desensitization levels. |
|  |  | Additional spurious emissions requirements |  | Additional spurious emissions requirements in 7 – 24 GHz range depend on the local and regional regulations for specific bands in different regions. |  | For specific band check relevant regulation and co-existence in same geographical area deployment scenarios and requirements. |
|  | Co-location with other base stations | The port to port isolation needs to be further considered for the frequency range. | Determine appropriate antenna port isolation for specific operating band. |
| Transmitter intermodulation |  | The necessity of the TX IMD requirement can be analyzed by understanding the isolation, the aggressor Pout and the emissions requirements (notably ACLR), as frequency increases the ACLR requirement is smaller, achievable Pout is lower and isolation increases. As none of these parameters will be decided for the 7 – 24 GHz range in the SI the need for TX IMD cannot be accurately analyzed however it is clear as the frequency increases the needed for a TX IMD requirement reduces. | The necessity of the TX IMD requirement can be analyzed by understanding the isolation, the aggressor Pout and the emissions requirements (notably ACLR), for each specific operating band in 7 – 24 GHz range. |

Table 7.4.1.1-2: Overview of radiated Tx requirements for NR BS in 7 – 24 GHz range

| OTA Tx requirement |  | Conclusions from SI | Items to be completed in related WI |
| --- | --- | --- | --- |
| Radiated transmit power |  | 2.0≤TRPaccuracy7to24GHz≤ 3.0 TRP accuracy capability is dependent on operating frequency and transmitter architecture Some indicative PA output power levels are discussed but BS output power depending on the array size is not investigated as no specific frequency band is handled. | Accuracy capability to be investigated for each operating band. |
| OTA base station output power |  | 2.2≤EIRPaccuracy7to24GHz≤ 3.4 Some indicative PA output power levels are discussed but BS output power depending on the power capability is not investigated as no specific frequency band is handled. | EIRP accuracy analysis to be considered once the operating bands are known. |
| OTA output power dynamics | RE power control dynamic range | For “FR1-like” sub-range of the 7 – 24 GHz range, the requirement for QPSK and 16QAM is the same as conduced requirement for 7 – 24 GHz. For “FR2-like” sub-range of the 7 – 24 GHz range, there is no requirement defined. No requirement for 64QAM. | If unwanted emission limits are increased compared to FR1 limits, then the RE power control dynamic range may require to be revisited. Decide if requirement for 256QAM is needed. |
|  | Total power dynamic range | Same requirement derivation methodology as for FR1 and FR2, based on the set of supported channel bandwidths and SCS. | Derive the requirement based on the supported channel bandwidths and SCS. |
| OTA transmit ON/OFF power | Transmitter OFF power | The background assumptions for FR1 and FR2 are significantly different. It can be considered that: - There are no existing conducted requirements, so there is no need to maintain equivalence to anything - There will be OTA requirement sets - It will be difficult to implement FR1 like co-location requirements Co-location in the 7 – 24 GHz region has a number of implementation issues which may require a new method of measuring co-location emissions however if co-location and co-existence scenarios are separated as they are with FR2 the TX OFF requirements will be simpler to implement. | OTA requirements based on co-existence OFF levels to be decided based on system scenarios. If OTA co-location requirements are needed then a new method of co-location emissions measurement needs to be studied. |
|  | Transmitter transient period |  | Transient period dependent on system scenarios in WI. Depending on the operating band frequency, the FR1 approach or FR2 approach can be used. This would also depend on chosen SCS. |
| OTA transmitted signal quality | Frequency error | Requirements can be reused from FR1/FR2. |  |
|  | Modulation quality | QPSK, 16QAM, 64QAM core requirements can be reused from FR1/FR2. | Define EVM window length and equalizer requirements. Define 256QAM requirements. |
|  | Time alignment error | Requirements for the MIMO transmission and for the inter-band CA (with or without MIMO) can be reused from FR1/FR2. | Derive intra-band CA TAE requirements, based on the set of the supported SCS. |
| OTA occupied bandwidth |  | Single carrier and CA requirements can be reused from FR1/FR2. |  |
| OTA ACLR |  | ACIR and hence BS ACLR are frequency dependent. In addition the distribution of the ACIR budget between BS ACLR and UE ACS varies over frequency with the ACLR allocation increasing with frequency as PA linearity becomes harder to achieve. It is also important to consider the deployment scenarios which are identified for identified bands as these are very important parameter in the co-existence analysis The 7 – 24 GHz region covers almost 3 octaves so there may be more than 1 ACIR (and hence BS ACLR) requirements over the 7 – 24 GHz range. | Co-existence simulations are carried out on specific bands (or ranges) as they are identified. The split between BS ACLR and UE ACS should also be decided on a band specific basis based on the relative difficulty in implementing each in the BS and UE respectively. |
| OTA operating band unwanted emissions |  | No conclusion around the OBUE domain and possible mask shape. Higher ΔfOBUE than in FR1 needs to be considered, probably with an approach similar to FR2. Existing mask shapes could also be a starting point. | To be completed in the WI when specific frequency bands are specified. Also consider the OBUE mask. |
| OTA transmitter spurious emissions | General transmitter spurious emissions | Regulation defines Category A and B limits, but Category B limits for AAS BS operating in 6 to 24.25 GHz are presently not defined in ERC Recommendation 74-01 [34]. A starting point for those limits could be the present limits above 24.25 GHz, but the work defining them will take place in ECC. Feasibility of filters based on LTCC technology was discussed during WI where based on attenuation assumptions, insertion loss and needed guard has been investigated. | Requirement to be completed in the WI. |
|  | Protection of the BS receiver of own or different BS | For a 7 – 24 GHz system, the noise figure and hence the wanted signal power level for the receiver sensitivity requirement will be higher compared to those in FR1 and hence the co-location power level will be similarly higher. | Determine appropriate antenna port isolation for specific band. If a requirement is needed in the 7 – 24 GHz region then it will be a co-location requirement, co-location in the 7 – 24 GHz region has a number of implementation issues (see clause 7.4.1.10) which requires a new concept. |
|  | Additional spurious emissions requirements | Additional spurious emissions requirements in 7 – 24 GHz range depend on the local and regional regulations for specific bands in different regions. | For specific operating band check relevant regulation and co-existence in same geographical area deployment scenarios and requirements. |
|  | Co-location with other base stations | A new solution based on measurements of EUT based on field strength instead of using measurement antennas. | Determine appropriate antenna isolation for specific band. If a requirement is needed in the 7 – 24 GHz region then it will be a co-location requirement, co-location in the 7 – 24 GHz region has a number of implementation issues (see clause 7.4.1.10) which requires a new concept. |
| OTA transmitter intermodulation |  | The necessity of the TX IMD requirement can be analyzed by understanding the isolation, the aggressor Pout and the emissions requirements (notably ACLR), as frequency increases the ACLR requirement is smaller, achievable Pout is lower and isolation increases. As none of these parameters will be decided for the 7 – 24 GHz region will be agreed in the SI the need for TX IMD cannot be accurately analyzed however it is clear as the frequency increases the needed for a TX IMD requirement reduces. | The necessity of the TX IMD requirement to be analyzed by understanding the isolation, the aggressor Pout and the emissions requirements (notably ACLR). |

#### 7.4.1.2	Output power accuracy

##### 7.4.1.2.1	General

There are currently 3 types of output power accuracy requirement which applied depends on the BS type:

- Base station output power

Conducted requirement applicable to BS type 1-C and BS type 1-H normal and extreme conditions

- Radiated transmit power

Directional OTA requirement (EIRP) applicable to BS type 1-H normal conditions only and, BS type 1-O and BS type 2-O normal and extreme conditions)

- OTA base station output power

Total radiated power requirement (TRP) applicable to BS type 1-O and BS type 2-O normal conditions only

In all cases the requirement is a power accuracy requirement in dB which is applied to a declared power level.

The power accuracy requirements clearly have some interdependency, it has been agreed that the following relationship exists between the requirements

conducted poweraccuracy≤TRPaccuracy≤ EIRPaccuracy

Also the requirements which apply to both FR1 and FR2 BS the accuracy levels are different implying a frequency dependency, the correct accuracy value for the 7 – 24 GHz range is to be determined. The following relationships can also be stated:

EIRPaccuracy_FR1≤EIRPaccuracy_7to24GHz≤ EIRPaccuracy_FR2

TRPaccuracy_FR1≤TRPaccuracy_7to24GHz≤ TRPaccuracy_FR2

And by implication:

Pconducted_ccuracy_FR1≤Pconducted_ccuracy_7to24GHz

When setting the power accuracy requirements both the impact on the network performance and the practicality of implementation are considered. The goal of setting an optimum output power variation requirement therefore is to have a value which does not make implementation to restrictive but offers enough control that networks can be planned.

##### 7.4.1.2.2	Conducted output power accuracy

Conducted output power accuracy is applicable to FR1 BS with a conducted interface. For FR1 AAS BS there is the assumption that BB beam forming architectures are dominant and hence for both BS type 1-C and BS type 1–H each BB chain drives a single transmitter/PA path. In this case the power accuracy is dependent primarily on the detector accuracy and the variation after the detector.

For BS type xFR-C, the accuracy of these blocks should be examined once the operating frequency bands are known.

For BS type xFR-H the picture is not so clear as an BS type xFR-H may not be directly related to a BS type xFR-C in the same way BS type 1-H and BS type 1-C systems are related in FR1;

- If the BS type xFR-H is based on an equivalent BS type xFR-C then the requirements will be the same as BS type xFR-C .

- If the BS type xFR-H is derived from a BS type xFR-O system then the conducted accuracy will need to be investigated in a different manner.

However the BS type xFR-H figures are determined however we can state:

2.0≤Pconducted_ccuracy_7to24GHz

##### 7.4.1.2.3	TRP accuracy

TRP accuracy is the OTA equivalent of conducted power, however there are some additional error factors which must be taken into account relating to variation in loss of passive antenna elements, beam formers, distribution networks etc.

For FR1 the additional errors were small (< 0.5 dB) and hence the rms addition of the conducted error and the additional errors resulted in the same value as the conducted accuracy (±2.0 dB).

For FR2 there is no conducted output power requirement and hence the accuracy value was derived directly. For FR2 there ae 2 major considerations:

- The higher frequency means component variation is greater

- The predicted hybrid architecture makes power control for each PA more difficult to implement.

The FR2 TRP accuracy was agreed to be ±3.0 dB.

Hence for 7 – 24 GHz we can state:

2.0≤TRPaccuracy7to24GHz≤ 3.0

It can be noted that the higher FR2 value is reliant not only on the increased frequency but also the assumed architecture. In the 7 – 24 GHz range it is expected that the optimum architecture for implementation may change, as such there may be a step in the output power accuracy requirement associated with this. This can be further investigated when the operating bands are known.

##### 7.4.1.2.4	EIRP accuracy

The radiated EIRP accuracy was investigated using 3 techniques

- Impact of accuracy on network performance

- Existing non-AAS EIRP accuracy

- Estimated achievable accuracy by an AAS BS

Companies were free to contribute based on any or all the methods, however most chose to focus on the estimated achievable accuracy. Final agreement on the accuracy of was by considering all contributions (not just the error estimates) and agreeing an acceptable value by consensus.

For FR1 the agreed value was ±2.2 dB

For FR2 the agree value was ±3.4 dB

Hence we can state:

2.2≤EIRPaccuracy7to24GHz≤ 3.4

For the 7 – 24 GHz range EIRP accuracy analysis can be considered once the operating bands are known.

#### 7.4.1.3		Output power dynamics

##### 7.4.1.3.1		Conducted RE power control dynamic range

For the conducted RE power control dynamic range in 7 – 24 GHz range the following consideration shall be taken into account:

- For QPSK and 16QAM:

  - In case the conducted unwanted emission limits are not increased (compared to the existing FR1 limits) for 7 – 24 GHz sub-range, then the conducted RE power up requirement can be reused for the applicable 7 – 24 GHz sub-range. Otherwise, the conducted RE power control dynamic range requirement might require to be re-visited during the WI phase.

  - In case the conducted EVM requirements are kept unchanged within 7 – 24 GHz sub-range, the conducted RE power down range requirement can also be reused for the same 7 – 24 GHz sub-range. Otherwise, the conducted RE power control dynamic range requirement might require to be re-visited during the WI phase.

- For 64QAM: no requirement for the 7 – 24 GHz range is required.

- For 256QAM (if decided to be applicable to the 7 – 24 GHz sub-range): no requirement for the 7 – 24 GHz range is required.

##### 7.4.1.3.2		OTA RE power control dynamic range

For the OTA RE power control dynamic range in 7 – 24 GHz range the following consideration shall be taken into account:

- Verification of the OTA output power dynamics is not impacted by the spatial aspects around the DUT.

- For FR1-like sub-range of the 7 – 24 GHz range, the OTA RE power control dynamic range requirement for QPSK and 16QAM is to be the same as the conduced RE power control dynamic range requirement.

- For FR2-like sub-range of the 7 – 24 GHz range, there is no OTA RE power control dynamic range requirement defined.

##### 7.4.1.3.3		Total power dynamic range

The total power dynamic range is defined as the difference between the maximum (i.e. for full RB allocation) and the minimum (i.e. for single RB allocation) transmit power of an OFDM symbol for a specified reference condition.

Therefore, once the set of supported channel bandwidths and the set of SCS is agreed for 7 – 24 GHz sub-range supporting conducted requirements, the derivation of the total power dynamic range requirement will be straightforward.

##### 7.4.1.3.4		OTA total power dynamic range

Derivation of the OTA total power dynamic range in 7 – 24 GHz range is the same as for the conducted version of this requirement and is based on the PRB count for the supported channel bandwidth and SCS. For the frequency sub-range where both conducted and radiated requirements will apply, the conduced and OTA total power dynamic range requirements are the same.

#### 7.4.1.4		Transmit ON/OFF power

The TX OFF requirement is a TDD requirement used to protect receivers in the same band from the off level emissions from a transmitter. The requirement is in the assigned channel frequency.

For FR1 BS the level is set to protect co-sited BS, the conducted and the OTA levels are set accordingly, so that a co-sited BS suffers <1 dB degradation of the receiver sensitivity.

For the FR1 conducted the requirement this is:

Pint=-85dBm+30+10*log104.5=-108.5dBm

Psens=10*log1010-174+5+10*log104.5MHz10+10-108.510+1=-100.5dBm

The FR1 OTA requirement is specified as an equivalent output from the co-location reference antenna.

FR2 TX OFF requirements are set based on interference with BS is the same geographical area at a distance of 100 m.

When looking at any operating band the co-location OFF level is lower than the co-existence case, however for a BS which has both a transmitter and a receiver it is clear that the most vulnerable receiver is the BS’s own receiver, as such if the Tx is in the OFF state when performing the RX sensitivity test then conformance to TX OFF level is demonstrated by this.

FR1 conducted specification are written separately for TX and Rx and it is considered that there may be separate Tx and Rx connectors, as such it is possible to test the Rx sensitivity without the Tx, as such a separate TX OFF requirement is necessary. OTA systems cannot separate the Tx and Rx in the same way so the TX OFF requirement becomes redundant. For FR1 OTA requirements however as the goal was to provide requirements which were equivalent to the conducted requirements and as TX OFF measurement with the co-location methodology was possible the TX OFF requirement was maintained as a co-location requirement.

For FR2 as the interface is OTA and there is no equivalence to legacy conducted requirements it was sufficient to rely on the RX sensitivity test to show TX OFF co-location compliance.

The co-existence TX OFF level whilst being a lower level also has tighter timing requirements to account for the propagation time between the BS and the victim. So whilst the receiver sensitivity test guaranteed the OFF level assuming no propagation time it does not guarantee the OFF level is reached quickly enough. As such the co-existence OFF level is required in conjunction with the transient requirement to protect non-co-located BS receivers.

The TX OFF requirements are hence covered as follows:

Table 7.4.1.4-1: Transmit ON/OFF power requirements

|  | FR1 | FR2 |
| --- | --- | --- |
| co-location | Tx OFF and Transient | Rx sensitivity |
| co-existence |  | TX OFF and transient |

For FR1 the 2 requirements are merged with the toughest timing requirement and the toughest OFF level, this makes a simple requirement but is perhaps over specified in terms of performance.

For the 7 – 24 GHz range it can be considered that:

- There are no existing conducted requirements, so there is no need to maintain equivalence to anything

- There will be defiantly have OTA requirement sets

- It will be difficult to implement FR1 like co-location requirements

- They may have conducted requirement (in lower bands)

Co-location in the 7 – 24 GHz region has a number of implementation issues which may require a new method of measuring co-location emissions however if co-location and co-existence scenarios are separated as they are with FR2 the TX OFF requirements will be simpler to implement.

For 7 – 24 GHz systems with conducted only requirements where the TX and Rx are separate it may be necessary to provide an additional conducted Tx OFF requirement (or have an appropriately lower level).

For summary of the co-location requirements refer to clause 7.4.1.10.

#### 7.4.1.5	Transmitted signal quality

##### 7.4.1.5.1	Frequency error

Both conducted and radiated NR BS frequency error core requirements are the same for FR1 and FR2. Therefore, it is expected that same core requirements can be re-used for 7 – 24 GHz frequencies.

##### 7.4.1.5.2	EVM

Both conducted and radiated NR BS EVM core requirements for QPSK, 16QAM, 64QAM or 256QAM have been defined for FR1 and FR2 with the same core requirement values. Therefore, it is expected the same core requirement values can be re-used for 7-24 GHz frequencies.

Some aspects such as EVM window length differ between FR1 and FR2 and will need further consideration for 7 – 24 GHz range.

##### 7.4.1.5.3	TAE

TAE core requirements for MIMO and inter-band CA with or without MIMO are the same for FR1 and FR2 and therefore it is expected they can be re-used also for 7 – 24 GHz frequencies.

For intra-band CA TAE differs between FR1 and FR2 due to difference in supported SCS. Intra-band CA TAE can be derived once the set of supported SCS is known for 7 – 24 GHz frequencies.

#### 7.4.1.6	ACLR

BS ACLR requirements are derived, along with the UE ACS requirements from the downlink co-existence simulations. The simulations provide an ACIR value which is then shared between the BS ACLR and the UE ACS with the bias on the UE ACS being the less strict requirements so that UE complexity can be kept at a minimum, where:

ACLR=-10log1010-ACS10+10-ACLR10

FR1 systems currently all have the same ACLR/ACS values for all operating bands, FR2 has different requirements depending on the operating band as summarized below:

Table 7.4.1.6-1: DL ACIR, BS ACLR and UE ACS values

|  | Based on NR study on RF and co-existence aspects |  |  | TS 38.104 [5], TS 38.101-1 [17], TS 38.101-2 [18] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 30 GHz | 45 GHz | 70 GHz | 0.7 – 5 GHz | 24.25 – 33.4 GHz | 37 – 52.6 GHz |
| UE ACS (dB) | 22.5 | 21.5 | 20.5 | 33 | 23 | 22 |
| BS ACLR (dB) | 27.5 | 25.5 | 23.5 | 45 | 28 | 26 |
| ACIR (dB) | 21.3 | 20 | 18.7 | 32.7 (calculated) |  |  |
| ACS (%) | 75 | 71 | 66 | 94 |  |  |

For FR2 the co-existence simulations have been carried out based on AAS beam forming architectures, for FR1 the original values were derived for non-AAS 3 sectored sites with passive antennas and these results were then verified to ensure that AAS beam forming implementation did not negatively impact the network throughput.

From the existing requirements and the results of the co-existence simulations used to derive them it is clear that ACIR and hence BS ACLR are frequency dependent. In addition the distribution of the ACIR budget between BA ACLR and UE ACS varies over frequency with the ACLR allocation increasing with frequency as PA linearity becomes harder to achieve.

There are a number of key simulation parameters which vary over frequency which may influence the co-existence simulation:

- Path Loss (increases by approximately 20 log f)

- Due to higher Path Loss cells are smaller so average distance to UE is smaller

- BS Antenna gain is higher

- BS total output power drops with frequency

- UE’s antennas are directional

- UE NF is higher.

Some of these parameters change naturally with frequency but some may be step functions with technological breakpoints.

It is also important to consider the deployment scenarios which are identified for identified bands as these are very important parameter in the co-existence analysis.

As such it is important that co-existence simulations are carried out on specific bands (or ranges) as they are identified. The 7 – 24 GHz region covers almost 3 octaves so there may be more than 1 ACIR (and hence BS ACLR) requirements over the 7 – 24 GHz range.

The split between BS ACLR and UE ACS should also be decided on a band specific basis based on the relative difficulty in implementing each in the BS and UE respectively.

#### 7.4.1.7	Operating Bands Unwanted emissions (OBUE)

##### 7.4.1.7.1 	General

Unwanted emission limits for Base Stations (as well as UEs) depend heavily on regulation. Present NR BS limits are defined with an Operating Band Unwanted Emission requirement covering the operating band, including the Out-of-band domain and a small part of the spurious domain outside of the operating band.

##### 7.4.1.7.2 	OBUE limits in FR1 and FR2

The unwanted emission limits for NR BS follow the same principles as was used for LTE BS, and to some extent also UMTS BS. These are shown in figure 7.4.1.7.2-1. Instead of just defining a spectrum mask for the out-of-band domain close to the transmitter BS carrier, an Operating Band Unwanted Emissions (OBUE) mask is defined across the operating band, extending also outside the band. Close to the RF carrier, the OBUE mask has limits that drops in several step down to a “baseline” level that extends out to an offset ΔfOBUE outside the edges of the operating band.

The OBUE limits will in general cover both the out-of-band domain and parts of the spurious domain of unwanted emissions, as defined by the ITU-R [7]. The OBUE limits close to the transmitted carrier are in general within the Out-of-band domain. These limits are based on RAN4 considerations including implementation aspects and to some extent also regulation. The steps in the mask are mostly between “flat” limits, but “sloped” limits are also used in some cases.

Figure 7.4.1.7.2-1: Principle for OBUE limits used for NR BS

The “extended” part of the mask will to a large extent be within the spurious domain and must therefore align with regulation for spurious emissions. For Category A limits, there is no concept defined in regulation that supports having different limits in the OBUE range than in the rest of the spurious domain, so the limits are the same as the spurious emission limits defined outside the OBUE range.

For Category B limits, the recent version of European regulation [17] explicitly allows for a reduced reference bandwidth within a frequency range set relative to the operating band edges (instead of relative to the centre frequency of emission). This in 3GPP implemented through the OBUE, with the reduced measurement bandwidth translated into an adjusted emission limit. For operation above 24.25 GHz, the regulation in [17] defines an explicit OBUE mask with limits set in three steps, which has been implemented in NR BS specs for FR2.

Note that additional limits may apply regionally in the OBUE frequency range to protect other services.

In FR1, the OBUE limits for NR BS [5] are implemented in different frequency ranges as outlined in table 7.4.1.7.2-1. The offset ΔfOBUE for the OBUE limits is 10 or 40 MHz depending on BS type and the width of the downlink operating band. The higher number is used for BS defined with OTA or Hybrid limits in bands 100 MHz or wider, and for BS with conducted limits in bands 200 MHz or wider. The shape of the mask is very similar to the LTE mask. A tighter Option 2 mask for Category B is defined for some bands, based on specific European regulation.

In FR2, OBUE limits for NR BS [5] are defined for a limited number of bands up to 40 GHz as outlined in table 7.4.1.7.2-1. They all have an offset ΔfOBUE of 1500 MHz. The OBUE mask shape is based on the IMT characteristics submitted to the ITU-R during the initial work on NR [13]. While this is basically unchanged for Category B in the 3GPP specs, the Category B mask has an additional step defined and a limit based on the new regulation for operation above 24.25 GHz in [17].

While the limits follow the same principle, there are both similarities and differences:

-	Both in FR1 and FR2, there is a 3GPP specified NR specific mask defined in the out-of-band domain close to the transmitter carrier(s). This part of the mask is fixed in FR1 independent of bandwidth (out to 10 MHz), while it scales with the transmitted bandwidth (BWcontiguous) in FR2.

-	Both in FR1 and FR2, the OBUE limit outside the NR specific part mentioned above, are based on international regulation for spurious emissions (Category A and Category B) taken from [7] and [17].

-	In FR1, the offset ΔfOBUE outside the band edges is quite small (10 – 40 MHz), originally based on the 5 MHz channel bandwidth of UTRA and later also applied for LTE, plus some considerations of implementation for AAS BS (BS type 1-H and BS type 1-O).

-	In FR2, the offset ΔfOBUE outside the band edges is much larger (1500 MHz), considering the much higher maximum channel bandwidth of NR and the more difficult implementation aspects in mm-Wave frequency ranges.

Table 7.4.1.7.2-1: OBUE limits for NR in FR1 (0.41 to 7.125 GHz) [5]

| Frequency range for OBUE limits | Steps in OBUE mask |
| --- | --- |
| ΔfOBUE = 10 or 40 MHz: BS type; Operating band characteristics; ΔfOBUE (MHz). BS type 1-H BS type 1-O; FDL,high – FDL,low < 100 MHz; 10. 100 MHz FDL,high – FDL,low 900 MHz; 40. BS type 1-C; FDL,high – FDL,low 200 MHz; 10. 200 MHz < FDL,high – FDL,low 900 MHz; 40. | Category A, Category B (Option1): 5, 10 MHz, then extends out to ΔfOBUE from band edges |
|  | Category B (Option 2): 0.2, 1, 10 MHz, then extends out to ΔfOBUE from band edges |

Table 7.4.1.7.2-2: OBUE limits for NR in FR2 (24.25 to 52.6 GHz) [5]

| Range of OBUE limits | Steps in OBUE mask |
| --- | --- |
| ΔfOBUE = 1500 MHz: (for a limited number of bands up to 40 GHz) BS type; Operating band characteristics; ΔfOBUE (MHz). BS type 2-O; FDL,high – FDL,low 3250 MHz; 1500. | Category A: 0.1*BWcontiguous, extends out to ΔfOBUE from band edges |
|  | Category B: 0.1*BWcontiguous, 2*BWcontiguous, extends out to ΔfOBUE from band edges |

The main difference between OBUE in FR1 and FR2 is the much higher ΔfOBUE in FR2, which takes both the higher NR channel bandwidths and the implementation aspects at higher frequencies into account.

##### 7.4.1.7.3	 OBUE limits in 7 – 24 GHz

In the 7 – 24 GHz range, there will also be a need for higher ΔfOBUE values than in FR1. The specific ΔfOBUE value will vary for different example frequencies and depend mainly on width of the downlink operating band and the relation to the fundamental frequency.

The unwanted emissions mask in 7 – 24 GHz could use the same OBUE principle as used in FR1 and FR2. As both FR1 and FR2 emission mask are band-centric, the mask for 7 – 24 GHz can be also assumed as band-centric. Higher ΔfOBUE, than in FR1 needs to be considered with an approach similar to FR2 and should be further studied in relation to possible implementations at example frequencies.

The shape of the OBUE mask in the out-of-band domain (close to the carrier) will also need consideration. For the OBUE mask, the limits shall also follow the corresponding regulation for Category A or Category B. Since there is presently no explicit regulation of mobile services in the bands, the starting point could be the existing mask shapes used in FR1 and FR2. The following aspects should be taken into account:

- ACLR values

- Output power

- Frequency ranges

- Transmission bandwidth

#### 7.4.1.8	Spurious emissions

##### 7.4.1.8.1	General

In the Study on 7 – 24 GHz frequency range, spectrum regulation for specific frequency ranges is outside the scope of the study item (except the survey on existing regulation across regions). It is however of interest to look at the more generic regulation of unwanted emissions, which is a fundamental aspect for defining RF requirements.

##### 7.4.1.8.2	International regulation (ITU-R)

Spurious emission is regulated through the international Recommendation ITU-R SM.329 [7], which is used as a reference in 3GPP specifications. There is a difference in reference bandwidth over different frequency ranges of the spurious in Recommends 4.1 in Recommendation ITU-R SM.329-12 [7] but those are applicable regardless of operating frequency (also called as fundamental frequency). For Category B spurious emissions (from European regulation, see below), there is a difference in limits below and above 1 GHz.

There is a restriction in the frequency range of the spurious domain for practical purposes in Recommend 2.5 of Recommendation ITU-R SM.329-12 [7], where in particular the upper limit to use for measurements depends on the fundamental frequency, as shown in table 7.4.1.8.2-1 for the frequency ranges relevant for NR.

As seen in table 7.4.1.8.2-1, the recommended upper limit for measurements in the 7 – 13 GHz range is 26 GHz, while for 13 – 24 GHz, the upper limit is the 2nd harmonic.

Table 7.4.1.8.2-1: Frequency range for measurement of spurious emissions (from [7])

| Fundamental frequency range | Frequency range for measurements |  |
| --- | --- | --- |
|  | Lower limit | Upper limit |
| 300 MHz – 600 MHz | 30 MHz | 3 GHz |
| 600 MHz – 5.2 GHz |  | 5th harmonic |
| 5.2 GHz – 13 GHz |  | 26 GHz |
| 13 GHz – 150 GHz |  | 2nd harmonic |

There is also Recommendation ITU-R SM.328 [42], but it deals more with general principles and serves as a guidance for limits in the out-of-band domain, but has no specific limits documented. There are however very specific limits defined in Recommendation ITU-R M.1580 [43] for UTRA and ITU-R M.2070 [44] for E-UTRA, but these recommendations are fundamentally based on 3GPP limits. Similar recommendation are expected to be developed for IMT-2020, where limits for NR would be based on the submission of NR to ITU-R as a candidate IMT-2020 system.

In summary, there are presently no specific ITU-R limits documented for 7 – 24 GHz or above – the generic spurious emission limits in recommendations are homogeneous for operation above 1 GHz, except for the variation in the spurious frequency range for measurement, which depends on the fundamental frequency.

##### 7.4.1.8.3	European regulation (CEPT)

Europe applies the Category B emission limits documented in Recommendation ITU-R SM.329 [7]. These limits are based on the European ERC Recommendation 74-01 [34] and are more stringent than the general Category A limits. There is also a broader ECC recommendation (02)05 [45] on unwanted emissions, but it has no recommendation for specific limits.

There has been a recent revision of ERC Recommendation 74-01[34], making it not fully aligned with the corresponding Category B limits in ITU-R. There are new specific limits for “Base Stations using AAS and beamforming with integrated antennas” covering operation below 6 GHz and above 24.25 GHz. No new limits are defined in the 6 – 24.25 GHz range, motivated by lack of information on such BS operating in that range. It is stated in ECC Recommendation 74-01 [34] that these could be considered in a future revision as necessary. For non-AAS BS, the general Category B limits apply in Europe.

Next revision of ECC Recommendation 74-01 [34] is expected in 3 years’ time according to the normal procedure, which means that ECC may initiate a work item for an update around 2021-22. It is however possible to initiate work on such limits earlier if needed for mobile systems.

The Category B limits applicable to AAS BS below 6 GHz and above 24.25 GHz are shown in figure 7.4.1.8.3-1 and 7.4.1.8.3-2 respectively. It should be noted that the limits below 6 GHz are equivalent to the conducted limits in the same frequency range, while above 24.25 GHz, there are several changes made in the updated European regulation.

Figure 7.4.1.8.3-1: Category B spurious domain emission limits for BS using AAS and beamforming operating below 6 GHz (from [17])

Figure 7.4.1.8.3-2: Specific Category B mask for spurious domain emissions for Base Stations using AAS and beamforming operating above 24.25 GHz (from [17])

For the updated limits above 24.25 GHz in figure 7.4.1.8.3-2, the following differences in the updated ERC Recommendation 74-01 [34] compared to the legacy conducted limits should be noted:

1.	There is a relaxation of limits close to the transmitted carrier(s) in the spurious domain, but the “mask” defined is operating band centric instead of carrier centric.
NOTE: Such on operating band centric mask was allowed also previously (and implemented in 3GPP), but it was not explicitly described in the recommendation.

2.	The reference BW for the limits is 10 MHz, changed from the previous 1 MHz.

3.	The limits at large offsets from the operating band (more than Fe) is -20 dBm/10 MHz, which is equivalent (for broadband emissions) to the conducted limits of -30 dBm/1 MHz.

4.	The limits at smaller offsets from the operating band and inside the band allow in many cases for higher levels than the conducted limits.

NOTE:	There is in addition a “reference bandwidth mask” in ERC Recommendation 74-01 [34] defined for conducted limits which allows up to a 15 dB relaxation, but it is not expressed using the new 10 MHz reference bandwidth and the offsets are carrier centric.

##### 7.4.1.8.4	Other regional regulation

While there are additional countries outside of Europe that apply Category B limits, there is a general applicability of Category A limits, where the only variation with the operation frequency is the recommended frequency range for measurements, as shown in table 7.4.1.8.2-1.

As an example, Japan develops specific unwanted emission regulation for new bands, but has no generic regulation in frequency ranges where no bands are presently defined, such as 7 – 24 GHz. The same is true for FCC, which has specific emission limits for each band and type of service, but no generic unwanted emission regulation of emissions that would presently apply in 7 – 24 GHz for mobile service.

##### 7.4.1.8.5	Possible new limits in 7 – 24 GHz

As observed above, while limits in 7 – 24 GHz exist for Category A and for Category B non-AAS, an update of ERC Recommendation 74-01 [34] to add limits for AAS BS in 6 – 24.25 GHz will be needed for Category B for Europe.

Similar to the limits developed for above 24.25 GHz, the general Category B limit of -30 dBm could remain, but the reference bandwidth can possibly be changed, and the limit correspondingly scaled. Note that in the present ERC Recommendation 74-01 [34], such a changed bandwidth is allowed down to 18 GHz.

The new spurious emission limits in 7 – 24 GHz could in this way use a similar approach as for above 24.25 GHz, in particular in the upper part of 7 – 24 GHz. This means applying a larger reference bandwidth together with an explicit operating band centric mask. At lower frequencies closer to 6 GHz, an in-between solution giving a smoother transition to the limits in figure 7.4.1.8.3-1 may be implemented. Further studies will be needed for what reference bandwidth that are possible below 18 GHz, also from a victim point of view.

The frequency offsets needed for defining the separation between the spurious emissions and an operating band centric OBUE mask may be different from the ones above 24.25 GHz.

#### 7.4.1.9	TX IMD

TX IMD is specified under the assumption that the BS is co-located with another BS of the same class, output power and antenna configuration. The reverse interferer specified in the TX IMD requirement is therefore based on the BS output power (as interfering BS is assumed to be the same power) and the isolation between the 2 systems.

- For BS type 1-C the isolation between the victim and aggressor antenna ports is 30 dB.

- For BS type 1-H an intra system requirement is defined.

- For BS type 1-O co-location a co-location reference antenna is specified and placed physically at a distance of 10cm from AAS BS and the full BS power is fed into it as an interferer.

For FR2 the OTA scenario is the same however it can be shown that the isolation is such that the requirement becomes unnecessary.

Under these conditions the BS must meet all the emission requirements.

The necessity of the TX IMD requirement can be analyzed by understanding the isolation, the aggressor Pout and the emissions requirements (notably ACLR), as frequency increases the ACLR requirement is smaller, achievable Pout is lower and isolation increases. As none of these parameters will be decided for the 7 - 24 GHz region will be agreed in the SI the need for TX IMD cannot be accurately analyzed however it is clear as the frequency increases the needed for a TX IMD requirements reduces.

If a Tx IMD requirement is needed in the 7 – 24 GHz region then it will be a co-location requirement, co-location in the 7 – 24 GHz region has a number of implementation issues, which may require a new method of injecting the interfering signal. For summary of the co-location requirements refer to clause 7.4.1.10.

#### 7.4.1.10	Co-location requirements

##### 7.4.1.10.1	General

For FR1 BS requirements there are 4 different categories of requirements which are specified using the co-location reference antenna concept (see TS 38.141-2 [6]):

- Spurious emissions (i.e. Protection of the BS receiver of own or different BS, and co-location requirement)

- Transmitter ON/ OFF power level (see clause 7.4.1.4)

- Transmitter intermodulation (see clause 7.4.1.9)

- Out-of-band co-location blocking (see clause 7.4.2.5.4)

For BS type 2-O there are currently no co-location requirements specified. It has been shown that the increased isolation between co-located BS and the lower ACLR requirements mean transmitter intermodulation requirements are not needed. For FR2 BS, the transmitter ON/OFF power requirement is specified as a TRP requirement. Transmitter spurious emissions and out-of-band co-location blocking requirements have been not specified in release 15 of the TS 38.104 [5] as co-location between FR2 systems with the current band allocation is not seen as very probable.

In addition, it has been shown that due to the high isolation between FR1 and FR2 antennas that no co-location requirements are needed between FR1 and FR2 or vice versa.

For BS type 1-O the co-location scenarios are defined as having a co-location reference antenna placed at a distance (edge to edge) of 10 cm from the BS. At this distance the worst case isolation found between two passive 2 GHz BS antennas was 30 dB. The same coupling factor of 30 dB has been used to derive all the conducted co-location requirements for BS type 1-C and BS type 1-H.

The coupling assumption and the definition of the co-location reference antenna are based on the co-located system being the same frequency as the BS as this is assumed worst case. For OTA requirements when the co-located system is a different frequency the co-location reference antenna is one suitable for that frequency. This method assumes that the co-located system is in a band where a non-AAS BS and passive antenna are used.

##### 7.4.1.10.2	Co-location emissions

If there are no co-location requirement then we can assume that worst case the BS will be radiating CAT B emissions levels and co-location of two base station can’t be guaranteed without degradation of the victim base station sensitivity.

Using the noise figure from the technology overview in table 5.5.1-1 and assume the same degradation of victim sensitivity as for FR1 base station (1dB degradation) the emission level at the victim can be determined. A similar level of protection to the victim receiver as provided for FR1 the co-location noise in the victim must conservatively be less than -110dBm/100 kHz. If CAT B emissions are assumed then an isolation of at least 70dB will be required, this is greater than the isolation assumed for FR2 and as such it seems likely that co-location emissions requirements of some sort are required in the 7 – 24 GHz region.

Considering the BS type 1-O style co-location requirements they have three difficulties when considering their use at higher frequencies:

- The emissions power levels out of the co-location reference antenna required to secure the victim receiver sensitivity are very low as such difficult to measure.

- Define a relevant co-location scenario; e.g. distance between co-location reference antenna and base station.

- The antennas used for conformance testing (co-location test antennas) must be identified and a range of different antennas may be required.

Because of these difficulties it may be necessary to develop a general concept without dependencies to a legacy passive BS antenna.

For type BS type xFR-C and BS type xFR-H type of requirements the antenna-to-antenna isolation needs careful consideration.

For BS type 1-O the emissions power levels are set at the output of the co-location reference antenna and are very low and as such requires a very low noise figure test system to measure and requires that the test system noise is calibrated out from the final measurement.

For a higher frequency system, we predict that the noise figure and hence the wanted signal power level for the receiver sensitivity requirement will be higher and hence the co-location power level will be similarly higher, however the same high noise figure will impact the measurement equipment. The effect on the measurement equipment could be greater than that on the BS receiver as not only the higher LNA NF but also cable loss etc. have to be taken into account. As such the measurement of the power level is likely to become harder in the 7 - 24 GHz frequency region.

The availability of passive BS antennas in the 7 - 24 GHz frequency region is also likely to be a problem. Many frequency bands will rely on OTA only systems which do not have traditional passive antenna. As such co-location test antennas will have to be designed specifically for the purpose and will not represent a real scenario as is intended with the existing BS type 1-O co-location requirements.

Specifying and measuring co-location emissions levels in the 7 - 24 GHz region is more difficult than in BS type 1-O and may require an alternative approach.

#### 7.4.1.11	Co-existence requirements

##### 7.4.1.11.1	General

Co-existence emissions requirements are captured as additional requirements in the spurious emissions part of the specification. The requirements may be applied for the protection of system operating in frequency ranges other than the BS downlink operating band as an optional protection of such systems that are deployed in the same geographical area as the BS.

In release 15 there are co-existence emissions requirements specified for FR1 but not for FR2.

##### 7.4.1.11.2	Background

FR1 requirements are based on the analysis done for UTRA BS as documented in TR 25.942 [22].

The scenario considered is an un-coordinated cell layout with and ISD of 500 m, where the victim BS is 288 m (5003=288) from the aggressor.

Figure 7.4.1.3.2-1 Co-existence in same geographical area 	scenario

For FR1 the emissions level is calculated at 2GHz and the same value is used for all frequencies. Whilst it is the case that as frequency increases the path loss increases the ISD may also shrink (for the same reason) so this is a reasonable assumption.

FR2 release 16 has no co-existence emissions requirements, as with co-location emissions this is due to the low number of FR2 bands making co-existence unlikely. In addition the high path loss and the beam forming nature of FR2 systems make interference between BS more unlikely. No co-existence requirements have been created for FR2.

##### 7.4.1.11.3	Co-existence emissions

In the 7 – 24 GHz region it is expected the lower frequencies may behave more like FR1 and the higher frequencies may like FR2. Further study is needed to determine whether co-existence requirements are needed, depending on the specific frequency and band.

### 7.4.2	Receiver requirements

#### 7.4.2.1	Rx requirements overview

Summary of the conducted and radiated Rx requirements specified in Rel-15 for the NR BS is presented in this clause. More detailed elaboration on the motivation on selected requirements is provided in dedicated clauses below.

All the findings captured for the conducted requirements in table 7.4.2.1-1 and related clauses below are considered to be applicable to the sub-range of the 7 – 24 GHz for which the conducted requirements will be found to be feasible during related WI.

While radiated requirements are considered to be applicable to the whole 7 – 24 GHz range, their definitions, values and levels may differ across the 7 – 24 GHz range.

Table 7.4.2.1-1: Overview of conducted Rx requirements for NR BS in 7 – 24 GHz range

| Rx requirement |  | Conclusions from SI | Items to be completed in related WI |
| --- | --- | --- | --- |
| Reference sensitivity level |  | Based on BW of FRC and NF assumptions. Indicative noise figure values were concluded in the SI where for 10, 15 and 20 GHz example frequencies, the NF value of 7, 8 and 9 dB respectively. | CBW and reference measurement channels to be confirmed. Frequency specific NF assumptions to be confirmed. |
| Dynamic range |  | Requirement will have to be re-calculated to account for an updated NF, IM and the supported set of NRB and SCS. Required SNR for the wanted signal to be re-simulated. For the derivation of the requirement: reuse the 95% throughput threshold, reuse the 16QAM-based FRC (if possible). | Value(s) of the NF, IM and required SNR. Consider capturing this aspect by demodulation requirements as for FR2. |
| In-band selectivity and blocking | ACS | ACS requirements defined with the BS class specific values of the interfering signal mean power. | The ACS requirement to be decided based on co-existence simulations during the follow-up WI. |
|  | In-band blocking | As the 7 – 24 GHz specification has to deal with all the BS types and implementation architectures the in-band blocking level should be set based on the difference between the wanted signal to interferer signal power levels in the same way as the FR2 levels. The conducted requirement can then be extracted by using the same delta on the conducted REFSENS value. As there are no existing conducted requirements to maintain equivalence to and the 7 – 24 GHz range will primarily consider beam forming systems it is suitable to derive the OTA requirements 1st and then apply the same methodology to the conducted. | Over the 7 – 24 GHz range it is possible there are multiple in-band blocking deltas covering different bands |
| Out-of-band blocking | General out-of-band blocking | The interferer level is -15 dBm below 7.125 GHz. | Range above 7.125 GHz to be concluded in the WI. |
|  | Co-location | The necessity of the co-location blocking requirement can be analysed by understanding the isolation, the aggressor power, as frequency increases. As none of these parameters could be agreed in the SI the need for co-location blocking requirement cannot be accurately analysed. | Need to establish antenna port isolation for specific band in WI. |
| Receiver spurious emissions |  | For FDD systems, the receiver spurious emissions are not expected to increase the total spurious emission level. The receiver spurious emission limit is therefore lower than the transmitter spurious emission limit, so the total is approximately equal to the transmitter spurious emission level. For TDD systems, the receiver spurious emission requirements are the same as the transmitter spurious emission requirements. | In certain regions, receiver spurious emission limit will be specified during the WI based on applicable regional regulations. |
| Receiver intermodulation |  | For conducted RX IMD levels were reused from E-UTRA. The RX IMD requirement for 7 - 24 GHz must be consistent with the methodology used for sensitivity. The blocking interferer levels (and hence RX IMD interferer levels) are absolute power levels. | Requirement to be concluded in the WI. |
| In-channel selectivity |  | BS class specific requirements will have to be re-calculated to account for an updated NF and the supported set of NRB and SCS. Required SNR for the wanted signal to be re-simulated. For the derivation of the requirement: reuse the 95% throughput threshold, reuse the QPSK-based FRC. | BS class specific conducted requirements to be considered in the WI phase: Value(s) of the NF, required SNR for wanted signal. ICS for the interferer to be verified. |

Table 7.4.2.1-2: Overview of radiated Rx requirements for NR BS in 7 – 24 GHz range

| OTA Rx requirement |  | Conclusions from SI | Items to be completed in related WI |
| --- | --- | --- | --- |
| OTA sensitivity |  | The OTA sensitivity requirement is a minimum sensitivity requirement for FR1 and ensures correct operation of the receiver including the integral antenna. OTA sensitivity is a declared parameter, the minimum sensitivity over an associated range of angle of arrivals (RoAoA) are declared (the declaration also allows for the RoAoA to be redirected by non-real time means but this distinction is not needed for simple analysis). | Confirm if minimum sensitivity can be supplied by OTA REFSENS requirement. |
| OTA reference sensitivity level |  | The 7 – 24 GHz BS is expected to require beam forming and whilst there may be conducted requirements there is no legacy so there is no need to provide equivalence between OTA and conducted requirements. As such the FR2 type of OTA REFSENS requirement may be used even if conducted specifications are specified. Indicative noise figure values were concluded in the SI where for 10, 15 and 20 GHz example frequencies, the NF value of 7, 8 and 9 dB respectively. | The appropriate NF and ranges of expected antenna gain can be agreed when the exact operating bands are known. |
| OTA dynamic range |  | For FR1-like sub-range: reuse FR1 approach of deriving the OTA requirement. For FR2-like sub-range: further investigation will be required to decide if the OTA dynamic range requirement in the FR2-like sub-range of 7 – 24 GHz range can be skipped. Value(s) of the NF, IM and required SNR to be reused from the conducted requirement for 7 – 24 GHz. For the derivation of the requirement: reuse the 95% throughput threshold, reuse the 16QAM-based FRC (if possible). | Evaluations for the FR2-like sub-range. |
| OTA in-band selectivity and blocking | OTA ACS | Frequency range specific requirements defined for OTA ACS, with the BS class specific values of the interfering signal mean power. | The OTA ACS requirement to be decided based on co-existence simulations during the follow-up WI. |
|  | OTA in-band blocking | In-band requirements dependent on the sensitivity requirements, as the sensitivity is used as a metric of the receiver performance under interference conditions. The in-band blocking requirement for 7 – 24 GHz must therefore be consistent worth the methodology used for sensitivity. The delta value for the 7 – 24 GHz range would be expected to be between the FR1 and FR2 values (52.7 to 27 dB), however the precise values would have to be found by blocking simulation once the operating frequencies are known and co-existence simulation parameters have been defined. | Over the 7 – 24 GHz range it is possible there are multiple in-band blocking deltas covering different frequency ranges. To be concluded in WI when system scenarios, specific frequency bands are defined, and proper co-existence studies are performed. |
| OTA out-of-band blocking | General out-of-band blocking | The interferer level is 0.36 V/m below 7.125 GHz and 0.1 V/m above 24.125 GHz. | The interferer signal within the range 7.125 to 24.125 GHz is defined in the WI. |
|  | Co-location | As the frequency increases the need for an out-of-band co-location receiver blocking requirements reduces. If an out-of-band co-location receiver blocking requirement is needed in the 7 - 24 GHz region then it will be a co-location requirement. Co-location in the 7 - 24 GHz region has a number of implementation issues, which may require a new method of injecting the interfering signal. | Need to establish antenna port isolation for specific band in WI. A new concept of injecting the interferer signal is required. |
| OTA receiver spurious emissions |  | The RX IMD requirement for 7 - 24 GHz must be consistent with the methodology used for sensitivity. The RX IMD is closely related to the in-band blocking, for both FR1 and FR2 the RX IMD levels have been between 8 to 9 dB lower than the in-band blocking levels. It is likely that the same will be found in the 7 - 24 GHz frequency range once operating frequencies are known and co-existence simulation parameters have been defined. | In certain regions, receiver spurious emission limit will be specified during the WI based on applicable regional regulations. |
| OTA receiver intermodulation |  | The RX IMD is closely related to the in-band blocking, for both FR1 and FR2 the RX IMD levels have been between 8 to 9 dB lower than the in-band blocking levels. It is likely that the same will be found in the 7 - 24 GHz frequency range once operating bands are known and co-existence simulation parameters have been defined. | Requirement to be concluded in WI. |
| OTA in-channel selectivity |  | For FR1-like sub-range: reuse FR1 approach for deriving the BS class specific OTA requirement, based on offsetting the conducted requirement. For FR2-like sub-range: reuse FR2 approach for deriving wanted and interferer levels based on offsetting the declared sensitivity EISREFSENS_50M. For the derivation of the requirement: reuse the 95% throughput threshold, reuse the QPSK-based FRC. | BS class specific radiated requirements to be considered in the WI phase: Required ICS level for FR2-like interferer. |

#### 7.4.2.2	Receiver sensitivity

##### 7.4.2.2.1	Background

There are a number of different receiver sensitivity requirements for the different BS types and frequency ranges, these requirements fulfil 2 roles;

- Minimum sensitivity which guarantees a certain minimum power level at which the specified throughput can be met.

- Reference sensitivity which is a reference performance which is used as a basis for the wanted signal power level during the interference tests (blocking, RX IMD, etc.)

In the case of the BS type 1-C, and BS type 2-O these 2 are both achieved with a single requirement.

Figure 7.4.2.2.1-1: Summary of receiver sensitivity requirements

|  | BS type 1-C | BS type 1-H | BS type 1-O | BS type 2-O |
| --- | --- | --- | --- | --- |
| Minimum sensitivity | conducted REFSENS | OTA sensitivity | OTA sensitivity | OTA REFSENS (FR2) |
| Reference sensitivity | conducted REFSENS | conducted REFSENS | OTA REFSENS (FR1) | OTA REFSENS (FR2) |

Conducted REFSENS is based on the BS receiver having a noise figure of 5 dB (wide area):

PREFSENS=PkT+10*log10BW+IM+SNR+NF

OTA REFENS is represented as an EIS value and also assumes a maximum noise figure as well as an antenna gain:

EISREFSENS=PkT+10*log10BW+IM+SNR+NF+GANT

The difficulty with adding the antenna gain parameter is that antenna gain is a variable and it is not possible to attribute a minimum or fixed value. Even with non-AAS BS with passive antennas the antenna gain varies with deployment. For a passive antenna the beam shape is fixed and hence is directly related to the geographical area the antenna covers or the range of angle of arrival (RoAoA), the larger the RoAoA the lower the antenna gain and hence the higher the minimum EIS.

##### 7.4.2.2.2	Conducted sensitivity

The conducted sensitivity requirement is based on the agreed minimum noise figure for the operating band being considered as well as the CBW and reference channel (FRC) as well as the SNR and implementation margin as follows:

PREFSENSdBm=-174dBm+10*log10BWFRC+NF+IM+SNR

Where:

-	BWFRC is the transmission bandwidth of the FRC.

-	NF is BS noise figure, equal to 5 dB for Wide Area BS, 10 dB for Medium Range BS and 13 dB for Local Area BS.

-	IM is the implementation margin, equal to 2 dB.

-	SNR is the SNR value for which we reach 95% throughput. Each company provided simulation results, and average will be done for each BW.

Typical values for 7 – 24 GHz BS noise figure have been discussed in clause 5.5.1 and these will clearly affect the conducted reference sensitivity level, however the channel band widths in the 7 – 24 GHz range are not yet known as this may also influence the requirements and the specified FRC’s as such the reference sensitivity level do not need to be defined in the SI.

##### 7.4.2.2.3	OTA sensitivity

The OTA sensitivity requirement is a minimum sensitivity requirement and ensures correct operation of the receiver including the integral antenna. OTA sensitivity is a declared parameter, the minimum sensitivity over an associated range of angle of arrivals (RoAoA) are declared (the declaration also allows for the RoAoA to be redirected by non-real time means but this distinction is not needed for simple analysis).

##### 7.4.2.2.4	OTA REFSENS

OTA REFSENS is the OTA reference sensitivity requirement which provided equivalence to the existing conducted requirements as both a reference for the interference measurements but also providing some equivalence to the minimum noise figure aspect of the conducted requirement.

As antenna gain is a variable a fixed OTA sensitivity value is not possible, however as antenna gain varies with coverage it is possible to provide a minimum expected gain (and therefore a minimum sensitivity) over a declared coverage area. Hence the declared RoAoA is used to calculate a minimum antenna gain and hence a reference sensitivity. The declared RoAoA corresponds to the 3 dB contour of the antenna gain and is called the OTA REFSENS RoAoA.

FR2 has no conducted requirements and no need for equivalence with legacy requirements, as such the sensitivity requirements are OTA only. However the issue still exists that different products may have different maximum antenna gains. In the case of FR2 however beam forming is necessary and hence the coverage area (or RoAoA) is not directly related to the antenna gain as a narrow beam can be steered over the RoAoA. This was not the case in FR1 where the AAS may not have beam steering capability.

The link between the RoAoA and sensitivity is hence not existed in FR2, the OTA sensitivity is hence a purely declared variable within a range of expected antenna gains.

##### 7.4.2.2.5	7 - 24 GHz

At this stage we must consider all the BS types for the 7 – 24 GHz range, however it is clear that OTA BS will be an important implementation. OTA REFSENS will hence be necessary.

OTA REFSENS is slightly different for FR1 and FR2 BS, for FR1 the antenna gain is directly related to the RoAoA as it offers equivalence with the conducted requirement and may need to be applied to non-beam forming architectures. For FR2 it is assumed that the system is beam forming and hence there is no link between the antenna gain and the RoAoA, also there is no equivalence to a conducted requirement.

The 7 - 24 GHz BS is expected to require beam forming and whilst there may be conducted requirements there is no legacy so there is no need to provide equivalence between OTA and conducted requirements. As such the FR2 type of OTA REFSENS requirement may be used even if conducted specifications are specified. The appropriate NF and ranges of expected antenna gain can be agreed when the exact operating bands are known.

Conducted requirements for BS type xFR-C and BS type xFR-H are based on expected noise figure and hence if needed can easily be derived using the same noise figure values as the OTA REFSENS requirement.

#### 7.4.2.3	Dynamic range

##### 7.4.2.3.1		General

The following is considered to be applicable to both the conducted and radiated dynamic range requirements:

- For the values of the implementation margin, it cannot be precluded that different IM values could be selected for the sub-ranges of 7 – 24 GHz range, leading to different values of the requirements across the frequency.

- The throughput threshold to derive the required SNR for the wanted signal derivation (i.e. 95% of the FRC) can be reused for 7 – 24 GHz range.

- The 16QAM-based FRCs (for the NRB and SCS that will be supported in the 7 – 24 GHz range) can be reused for 7 – 24 GHz range.

##### 7.4.2.3.2		Conducted dynamic range

The conducted dynamic range requirement for 7 – 24 GHz sub-range will have to be re-calculated:

- For the wanted and interfering signal levels will have to be re-calculated to account for an updated NF values and the supported set of NRB and SCS, as well as the implementation margin (IM).

- For the wanted signal: required SNR will be subject to the simulation campaigns.

##### 7.4.2.3.3		OTA dynamic range

For the OTA dynamic range requirement the following was concluded:

- For the FR1-like sub-range of the 7 – 24 GHz range: the same approach of deriving the OTA requirement from the conducted dynamic range requirement can be reused, i.e. conducted requirement plus ΔOTAREFSENS offset.

- For the FR2-like sub-range of the 7 – 24 GHz range: as there was no such requirement defined for FR2, further investigation will be required to decide if the OTA dynamic range requirement in the FR2-like sub-range of 7 – 24 GHz range can be skipped. Based on the estimated dynamic range value(s), the FR2 BS demodulation requirements could be also considered to assess for the required SNR values (and their relation to the noise floor levels), to further evaluate the need for the OTA dynamic range requirement in FR2-like sub-range of 7 – 24 GHz range.

#### 7.4.2.4	In-band blocking

##### 7.4.2.4.1	Background

In-band blocking levels are identified by using co-existence simulations to identify the CDF of in-band interference levels. For conducted system analysis only the interferer was considered, however for OTA systems this method is not suitable as the level varies depending on the architecture of the victim

- An RF beam forming system applies the beam forming to both the wanted signal and the interferer. As such if the interferer is in the same direction as the wanted signal it is much larger, however the probability of this occurring is low. If the interferer is in a different direction then it is attenuated by the beam forming.

- A BB beam forming system only applies the beam forming in the BB, therefore the RF front end is exposed to only the element gain. The element gain is much lower, however the beam is wider and hence the probability of the gain being applied is greater.

In addition it was found that the direction of the worst case blockers is not stable and hence testing the worst case interferer level in the reference direction may result in a level which is not consistent with the findings of the simulation. For compliance it is preferable to have the interferer and the wanted signal coming from the same direction.

For the OTA requirements however it was found that the ratio between the wanted signal and the in-band interferer was less dependent on the architecture so by setting the blocking level as an offset from the OTA REFSENS offered an architecturally independent method for setting the blocking level.

The FR1 and FR2 in-band clocking specs are hence:

Figure 7.4.2.4.1-1: Wide area in-band blocking wanted signal and interferer levels

|  | BS type 1-C | BS type 1-H | BS type 1-O | BS type 2-O |
| --- | --- | --- | --- | --- |
| Wanted signal (dBm) | -101.7 + 6 | -101.7 + 6 | -101.7 - ΔOTAREFSENS + 6 | EISREFSENS_50M + ΔFR2_REFSENS + 6 |
| Interferer level (dBm) | -43 | -43 | -43 - ΔOTAREFSENS | EISREFSENS_50M + 33 + ΔFR2_REFSENS |
| Delta (dB) | 52.7 | 52.7 | 52.7 | 27 |

It can be seen all the FR1 levels are consistent and the FR2 levels is lower.

##### 7.4.2.4.2	7 - 24 GHz

In-band requirements dependent on the sensitivity requirements, as the sensitivity is used as a metric of the receiver performance under interference conditions. The in-band blocking requirement for 7 - 24 GHz must therefore be consistent worth the methodology used for sensitivity. The proposed metrics for sensitivity are identified in clause 7.4.2.2.

As the 7 - 24 GHz specification has to deal with all the BS types and implementation architectures the in-band blocking level should be set based on the wanted signal to interferer level in the same way as the FR2 levels. The conducted requirement can then be extracted by using the same delta on the conducted REFSENS value. As there are no existing conducted requirements to maintain equivalence to and the 7 – 24 GHz range will primarily consider beam forming systems it is suitable to derive the OTA requirements first and then apply the same methodology to the conducted.

The delta value for the 7 - 24 GHz range would be expected to be between the FR1 and FR2 values (52.7 to 27 dB), however the precise values would have to be found by blocking simulation once the operating frequencies are known and co-existence simulation parameters have been defined.

Over the 7 - 24 GHz range it is possible there are multiple in-band blocking deltas covering different frequency ranges.

#### 7.4.2.5	Out-of-band blocking requirements

##### 7.4.2.5.1	General

The out-of-band blocking requirement relies on the sensitivity as reference. The base station is illuminated with a wanted signal and an interferer signal at the same time maintaining specified link quality. Hence, the FRC used for conformance testing must support the carrier bandwidth to be supported for the frequency range 7 - 24 GHz.

##### 7.4.2.5.2	Background

There are NR BS type specific concepts for how the out-of-band blocking requirement is defined with respect to the wanted signal power used as reference for the requirement, see table 7.4.2.5.2-1.

Table 7.4.2.5.2-1: Wanted signal characteristics

|  | Wanted signal conducted power (dBm) | Wanted signal EIS (dBm) |
| --- | --- | --- |
| BS type 1-C, BS type 1-H | PREFSENS + 6 dB | NA |
| BS type 1-O | NA | EISminSENS + 6 dB |
| BS type 2-O | NA | EISREFSENS_50M + 6 dB |

For BS type 1-C and BS type 1-H, reference sensitivity (PREFSENS) is used as reference for out-of-band receiver blocking. For BS type 1-O, the minimum sensitivity (EISminSENS) relating to the full array antenna sensitivity is used as reference while for BS type 2-O, the reference sensitivity (EISREFSENS) relating to the antenna element/sub-array sensitivity is used as reference. In all cases these parameters represent the minimum sensitivity requirements, see clause 7.4.2.2.1.

The conducted requirement defined for BS type 1-C and BS type 1-H is only defined for FR1. It is based on the analysis done for UTRA BS and the derivation of the -15 dBm conducted interferer level is based on interference analysis showing that worst case interference is from other BS at approximately the same frequency.

For simplicity, it is treated as the worst case and applied to the whole test frequency range up to 12.75 GHz which is shown below in table 7.4.2.5.2-2.

The OTA requirement is defined for both FR1 and FR2 (which are concluded below in table 7.4.2.5.2-2) and it can be found that a discontinuity point of the FR2 requirement occurs at 12.75 GHz.

Table 7.4.2.5.2-2: Interferer signal characteristics

|  | Frequency range of interferer signal (MHz) | Interferer level at TAB/antenna connector (dBm) | Interferer RMS field-strength at RIB (V/m) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| BS type 1-C, BS type 1-H | 1 to FUL,low – ΔfOOB | -15 | NA | CW |
|  | FUL,high + ΔfOOB to 12750 | -15 |  |  |
| BS type 1-O | 30 to FUL,low – ΔfOOB | NA | 0.36 |  |
|  | FUL,high + ΔfOOB to 12750 |  | 0.36 |  |
| BS type 2-O | 30 to 12750 |  | 0.36 |  |
|  | 12750 to FUL,low – ΔfOOB |  | 0.10 |  |
|  | FUL,high + ΔfOOB to flimit (Note) |  | 0.10 |  |
| Note: flimit is the 2nd harmonic of the upper frequency edge of the operating band. |  |  |  |  |

The interferer signal level 0.36 V/m from 30 MHz to 12750 MHz is based on the calculation of -15 dBm conducted interferer signal level at frequency 2000 MHz and 17 dBi antenna gain and a distance between aggressor and victim of 30 m.

The EIRP at the aggressor is then calculated as:

EIRP(30m) = Prx – Gant + FSPL(30m) = -15dBm – 17dBi + 68dB(frequency=2 GHz, FSPL 30m)  = 36 dBm

, where the aggressor EIRP level in linear scale is then converted to an RMS field strength level at the victim base station as:

E=30*1036-301030=0.36V/m

For FR2 requirements the blocking signal is calculated as 0.1 V/m and applied to the whole frequency range as shown below in figure 7.4.2.5.2-1. Note when FR2 requirements were derived no systems between 7 – 24 GHz were considered.

The aggressor out of band BS is considered to have a total power of 29 dBm, an antenna gain of 26 dBi hence an EIRP of 55 dBm. As the aggressor main beam is pointed at UE’s at the ground, an aggressor sidelobe suppression level of 13 dB to get the aggressor signal pointing towards the victim is considered.

With the interferer at a distance of 200 m this gives an interfering field strength of:

E=30*1055-13-3010200=0.1V/m

In figure 7.4.2.5.2-1, the interferer signal level is plotted for FR1 and FR2.

Figure 7.4.2.5.2-1: OOB blocking interferer level

##### 7.4.2.5.3	Technical considerations for 7 - 24 GHz

When developing out-of-band blocking requirements for base stations operating within the 7 - 24 GHz frequency range, RAN4 needs to consider requirements defined for FR1 and FR2. In table 7.4.2.5.3-1, the wanted signal characteristics is captured.

Table 7.4.2.5.3-1: Wanted signal characteristics for base stations operating within 7 - 24 GHz

|  | Wanted signal power (dBm) | Wanted signal (dBm) |
| --- | --- | --- |
| BS type xFR-C, BS type xFR-H | PREFSENS + X dB | NA |
| BS type xFR-O | NA | EIS + Y dB |
| NOTE: Where PREFSENS, EIS, X and Y values will be defined in the WI phase. |  |  |

In figure 7.4.2.5.3-1, the interferer level for frequencies outside the range 7125 MHz to 24250 MHz is plotted. The interferer level between 7125 MHz to 24250 MHz will be defined in the WI phase.

Figure 7.4.2.5.3-1: Baseline for interferer signal level

##### 7.4.2.5.4	Out-of-band co-location receiver blocking

Out-of-band co-location receiver blocking requirement is defined as co-location requirement. Summary of the co-location requirements is captured in clause 7.4.1.10.

For FR1 BS, requirements which are specified using assumptions on antenna port-to-port isolation between two co-located base stations is referred to as co-location requirements. For the receiver co-location requirement for out-of-band blocking is specified for BS type 1-C, BS type 1-H and BS type 1-O. For BS type 2-O there are currently no co-location requirements specified.

In addition, it has been shown that due to the high isolation between FR1 and FR2 antennas that no co-location requirements are needed between FR1 and FR2 BS or vice versa.

For BS type 1-O the co-location scenarios are defined as having a co-location reference antenna placed at a distance (edge to edge) of 10 cm from the BS. At this distance the worst-case isolation found between two passive 2 GHz BS antennas was 30 dB. The same coupling factor of 30 dB has been used to derive all the conducted co-location requirements for BS type 1-C and BS type 1-H.

The coupling assumption and the definition of the co-location reference antenna are based on the co-located system being the same frequency as the BS as this is assumed worst case. For OTA requirements when the co-located system is a different frequency the co-location reference antenna is one suitable for that frequency. This method assumes that the co-located system is in a band where a non-AAS BS and passive antenna are used.

For BS operating within 7 - 24 GHz, the necessity of the out-of-band colocation receiver blocking requirement can be analyzed by understanding the isolation, the aggressor Pout. As none of these parameters will be decided for the 7 - 24 GHz region will be agreed in the SI the need for out-of-band co-location receiver blocking cannot be accurately analyzed. However, it is clear as the frequency increases the need for an out-of-band co-location receiver blocking requirements reduces.

If an out-of-band co-location receiver blocking requirement is needed in the 7 - 24 GHz region then it will be a co-location requirement, co-location in the 7 - 24 GHz region has a number of implementation issues, which may require a new method of injecting the interfering signal.

#### 7.4.2.6	Receiver spurious emissions

For conducted systems, receiver spurious emissions are specified separately for FDD and TDD systems.

For conducted FDD systems, as the transmitter and the receiver are both on at the same time and can be specified on separate connectors, the receiver spurious emissions are not expected to increase the total spurious emission level. The receiver spurious emission limit is therefore lower than the transmitter spurious emission limit, so the total is approximately equal to the transmitter spurious emission level.

For OTA FDD systems, the requirement is over the air and hence the transmitter and receiver cannot be separated, so there are no receiver spurious emission requirements needed.

For OTA TDD systems in applicable regions, the receiver spurious emission requirements are the same as the Category B transmitter spurious emission requirements as discussed in clause 7.4.1.8.

In certain regions, receiver spurious emission limit will be specified during the WI based on applicable regional regulations.

#### 7.4.2.7		In-channel selectivity

##### 7.4.2.7.1 	In-channel selectivity for FR1 and FR2

In-channel selectivity (ICS) is a measure of the receiver’s ability to receive a wanted signal at its assigned resource block locations in the presence of another in-channel wanted signal received at a much larger power spectral density.

The ICS requirement applies to FR1 (conducted and radiated requirements) and FR2 (radiated requirement).

The conducted requirement is derived similar to E-UTRA requirement which assumes that the UL signal is defined for 2 users, one being the “wanted” signal and the other one being the “interfering” signal at elevated power. Following equations are used for interfering signal and wanted signal correspondingly.

Interfering signal power level = -174dBm/Hz + 10*log10(BW) + NF + ICS

Wanted signal power level = -174dBm/Hz + 10*log10(BW) + NF + SNR + IM + 3 dB

The type of interfering signal is DFT-s-OFDM NR signal and BW is corresponding to number of RBs and SCS.

25 dBc ICS is used based on 16 dB interference-over-thermal (IOT) and 9 dB SNR considering the modulation and link level simulation.

For this conducted requirement, the modulation scheme for interfering signal is assumed as 16QAM and modulation scheme for wanted signal is assumed as QPSK.

For the OTA requirement, the OTA ICS for FR1 is based on conducted requirement and an offset of ΔminSENS is used for transferring the conducted requirement to OTA requirement. However, for FR2 requirement there is no conducted requirement to be referred to. Hence the OTA ICS for FR2 is derived using the same equation as for conducted requirement, with additional offset for the gain G:

Interfering signal power level = -174dBm/Hz + 10*log10(BW) + NF + ICS - G,

Wanted signal power level = -174dBm/Hz + 10*log10(BW) + NF + SNR + IM + 3dB - G;

14 dBc ICS is used based on the simulation result of IOT with 9 dB SNR and the worst case is applied.

For radiated requirement, the modulation scheme for interfering signal is assumed as 16QAM and modulation scheme for wanted signal is assumed as QPSK. This is the same as the conducted requirement.

##### 7.4.2.7.2	 In-channel selectivity for 7 - 24 GHz

For the FR1-like frequency range, the conducted requirements should not be precluded. Hence, both conducted as well as radiated requirements need to be considered in the WI phase.

The conducted in-channel selectivity for 7 – 24 GHz will have to be re-calculated with the following considerations:

- The wanted and interfering signal levels of the ICS requirement will have to be re-calculated to account for an updated NF values and the supported set of bandwidths. For the wanted signal, the required SNR will be subject to the simulation campaigns.

- Similar to the FR1 requirements, BS class specific requirements for conducted ICS need to be considered in the WI phase.

- Selection of FRC to be used for the ICS requirement will depend on the set of supported channel bandwidths and SCS in 7 – 24 GHz. QPSK can be reused from FR1.

The throughput threshold (i.e. 95%) to derive the required SNR for the wanted signal can be reused for 7 – 24 GHz range.

For the OTA in-channel selectivity the following was concluded:

- For the OTA ICS, reuse the QPSK for the wanted signal (subject to the supported channel bandwidths and SCS in 7 – 24 GHz).

- For the FR1-like sub-range of the 7 – 24 GHz range:

  - BS class specific requirements for OTA ICS need to be considered in the WI phase.

  - The same approach of deriving the OTA requirement from the conducted one can be reused, i.e. conducted requirement plus ΔminSENS offset for both the wanted and interfering signals.

- For the FR2-like sub-range of the 7 – 24 GHz range:

  - The interfere signal and the wanted signal will be based on specific FRC, SCS and also the OTA reference sensitivity requirements.

#### 7.4.2.8	RX ACS

The following values of BS ACS and UE ACLR were defined for FR1 and FR2 frequency ranges:

Table 7.4.2.8-1: Overview of Rx ACS requirements for NR

|  |  | FR1 | FR2 |  |
| --- | --- | --- | --- | --- |
|  |  | 2 GHz | 24 to 33 GHz | 37 to 52 GHz |
| ACS | dB | 46 | 24 | 23 |
| ACLR | dB | 31 | 17 | 16 |
| ACIR | dB | 30.86 | 16.21 | 15.21 |

To get accurate estimations on the ACIR for the range 7 – 24 GHz the co-existence simulations would need to be carried out. It will be done when the bands are allocated. Based on the values in table 7.4.2.8-1 it can be estimated that the BS ACS value for 7 – 24 GHz range operation would be in the range 24 to 46 dB. The split between UE ACLR and BS ACS should also be decided on a frequency range specific basis based on the relative difficulty in implementing each in the BS and UE respectively.

Hence we can conclude that the ACS value for the range 7 – 24 GHz will be in the range 24 dB to 46 dB. The exact requirement need to be decided based on co-existence simulations during the follow-up WI.

#### 7.4.2.9	RX IMD

##### 7.4.2.9.1 	RX IMD for FR1 and FR2

The blocking requirements capture the selectivity of the receiver, whilst the receiver intermodulation given the two interferers condition would capture the linearity of the receiver.

For FR1 both blocking and RX IMD levels were reused from E-UTRA, for FR2 the in-band blocking requirements were investigated with blocking simulations. For RX IMD more limited simulations were performed however the interferer RX IMD interferer levels were a very similar and 8 – 10 dB lower than the in-band blocking levels (under the same assumptions). A level of 8 dB below the in-band blocking levels was agreed for FR2.

For FR1 the conducted blocking interferer levels (and hence RX IMD interferer levels) are absolute power levels, for OTA these absolute power levels are offset by the difference between conducted reference sensitivity and OTA REFSENS (i.e. ΔOTAREFSENS).

For FR2 the blocking interferer levels (and hence RX IMD interferer levels) are offset from the declared sensitivity value EISREFSENS_50M.

In addition to the level the interferer signal type and BW is also important. For FR1 a CW and a modulated interferer are used with the modulated signals BW being 5 or 20 MHz DFT-s-OFDM NR signal dependent on the wanted signal BW. For FR2 a CW and modulated signal are also used with the modulated signal BW being 50 MHz DFT-s-OFDM NR signal.

##### 7.4.2.9.2 	RX IMD for 7 - 24 GHz

The RX IMD is closely related to the in-band blocking, for both FR1 and FR2 the RX IMD levels have been between 8 to 9 dB lower than the in-band blocking levels. It is likely that the same will be found in the 7 - 24 GHz frequency range once operating frequencies are known and co-existence simulation parameters have been defined.

As with the blocking the in-band interference requirements are dependent on the sensitivity requirements, as the sensitivity is used as a metric of the receiver performance under interference conditions. The RX IMD requirement for 7 - 24 GHz must therefore be consistent with the methodology used for sensitivity. The proposed metrics for sensitivity are identified in clause 7.4.2.2.

The RX IMD interferer levels are offset from the blocking levels.

The interferer signal definition are the same for FR1 and FR2 with the exception of the modulated signal BW, this will depend on the BW’s defined for the operating band and may vary in the 7 - 24 GHz range.

## 7.5	BS EMC requirements

### 7.5.1	General

The EMC requirements for the NR BS and ancillary equipment are defined in TS 38.113 [3], including emission requirements and immunity requirements. Background on derivation of selected EMC requirements for the AAS BS architecture was captured in the NR technical report TR 38.817-02 [4].

### 7.5.2	Emission requirements

#### 7.5.2.1		Conducted requirements

Irrespective of the BS architecture (i.e. non-AAS or AAS architecture) considered for the NR operation in 7 – 24 GHz frequency range, the following EMC emission requirements can be directly reused from the existing NR BS specification TS 38.113 [3]:

- Conducted emission DC power input/output port

- Conducted emissions, AC mains power input/output port

- Conducted emissions, telecommunication port

- Harmonic Current emissions (AC mains input port)

- Voltage fluctuations and flicker (AC mains input port)

The only EMC emissions requirement which has dependency on the BS architecture is the radiated emission requirement.

#### 7.5.2.2		Radiated emission requirements

Based on the discussion on EMC radiated emission requirements for the NR BS, the following findings are also applicable to the NR BS operation in the 7 – 24 GHz frequency range:

- For AAS BS architecture not equipped with connectors the NR BS EMC approach for BS type 1-O and BS type 2-O shall be reused:

  - EMC radiated emissions cannot be distinguished from the RF radiated spurious emissions in the OTA test setup, therefore it becomes a single emission requirement. This requirement is performed by measuring TRP emissions at the enclosure of the DUT with the transmitters switched ON and not terminated, radiating the output power subject to the antenna array gain.

  - The measurement is performed at the rated output power.

  - The core requirement shall be captured in the BS RF specification TS 38.104 [5] under the OTA transmitter spurious emission requirement, subject to radiated conformance test specification TS 38.141-2 [6].

  - Emission limits shall be based on the ITU-R SM.329 [7] recommendation.

- For non-AAS BS architecture equipped with antenna connectors the NR BS EMC approach for BS type 1-C shall be reused.

NOTE: 	The above summary can be further updated once the decision on the BS architecture will be taken for the 7 – 24 GHz operation (e.g. in case the non-AAS architecture will be excluded).

### 7.5.3	Immunity requirements

#### 7.5.3.1		Common EMC immunity requirements

Irrespective of the BS architecture (i.e. non-AAS or AAS architecture) considered for the NR operation in 7 – 24 GHz frequency range, the following EMC immunity requirements can be directly reused from the existing NR BS specification TS 38.113 [3]:

- Electrostatic discharge

- Fast transients common mode

- RF common mode (0.15 MHz - 80 MHz)

- Voltage dips and interruptions

- Surges, common and differential mode

The only EMC immunity requirement which has dependency on the NR BS architecture is the radiated immunity requirement.

#### 7.5.3.2		Radiated immunity

As of Rel-15 timeframe, the IEC 61000-4-3 [8] specification defines the radiated immunity requirement up to 6 GHz. This frequency limit would also apply to the EMC radiated immunity testing of the NR BS operating in the 7 – 24 GHz frequency range, until any future updates to the IEC specification are released.

Based on the discussion on EMC radiated immunity requirements for the NR BS with AAS BS architecture (i.e. no antenna connectors available), the findings on solutions for the RF Rx chain protection from higher radiated power levels in the OTA test chamber are considered to be also applicable to the NR operation in the 7 – 24 GHz frequency range. The following alternative solutions are applicable to NR BS for the purposes of the RF chain protection during the EMC radiated immunity testing:

- Spatial exclusion zone, as described in TS 38.113 [3], clause 9.2,

- Extended frequency exclusion, as described in TS 38.113 [3], clause 4.4.2.

Annex A (informative):
Example filter characteristics

# A.1	20 GHz example frequency filtering

## A.1.1	General

The filter characteristics and needed guard for state-of-the art LTCC technology and PCB integrated filter providing Q value around 200 and 120 respectively at 20 GHz is further investigated in this annex. It should be noted that LTCC is more difficult to integrate than PCB integrated filters.

Manufacturing tolerances and temperature drift will have a significant impact on performance and must be accounted for by addition of a sufficient design margin. It is assumed that individual tuning of filters, which is done in many applications, is not feasible for array antennas operating at 20 GHz due to the large amount, small size, and compact integration into an array without test ports. A frequency tolerance around 250 MHz is assumed (i.e. the frequency at which a filter provides the desired suppression can differ from a nominal value by maximum 250 MHz), which represents what is achievable for state-of-the-art LTCC filters. PCB integrated filters will generally need to allow for a worse frequency tolerance than LTCC filters. It is for simplicity assumed that the frequency tolerance appears as a plain frequency shift of the filter characteristic by +/-250 MHz for LTCC filters in the worst case. This corresponds to a case in which all resonators have the same frequency error. In general, though errors will be uncorrelated to a certain degree and the tolerance may not be quite as bad as this worst case.

Although the use of transmission zeros in general tends improve steepness of a filter, the use of zeros has not been assumed in this example since it increases the sensitivity to frequency errors and can even lead to worse performance if the frequency tolerance is large compared to the size of the guard band.

## A.1.2	LTCC example filter

An equivalent model based on RLC-resonators was used to estimate the performance of an LTCC filter. The filter is assumed to be tuned under the following conditions:

- Q = 200

- 250 MHz margin to frequency shift

- 6-poles, Chebyshev type

- Center frequency: 20 GHz

- Bandwidth: 2 GHz

- Return loss: 15 dB

- Attenuation: 25 dB

In figure A.1.1-1, the resulting filter performance is plotted. The blue trace represents S21, the red trace represents S11, and the dotted gray lines represent suggested requirement levels.

Figure A.1.2-1: Estimated characteristics of a bandpass filter based on LTCC for example frequency of 20 GHz

Considering a tolerance/drift-related frequency shift of +/-250 MHz for LTCC filters, approximate estimates of the guard and insertion loss that would need to be allowed for if the filter would be implemented and the assumed amount of suppression would be required are:

- Guard band: ~1.1 GHz

- Insertion loss: ~3.5 dB (at edges, with worst case frequency error)

## A.1.3	PCB integrated example filter

For PCB integrated filters due to lower achievable Q-values compared to LTCC technology and worse frequency tolerance, the guard is expected to be larger. Typical characteristics of such filters can be found by looking at the following design example:

- 5-pole, coupled line, strip-line filter

- Permittivity: 3.4

- Dielectric thickness: 500 μm (ground to ground)

- Unloaded Q: 120 (using low loss microwave laminate/prepreg)

Figure A.1.3-1: Layout of a strip line filter in PCB technology

The filter is assumed to be tuned for 16 dB return loss in the nominal case. Margins are added to make room for variations in the manufacturing processes of the PCB according to the following assumptions:

- Tolerance on permittivity: 1.5 % (min/max with uniform distribution).

- Tolerance on line width: 20 μm (min/max with uniform distribution).

- Tolerance on thickness of dielectric: 7 % (min/max with uniform distribution).

The resulting filter performance is shown in figure A.1.2-2 for the nominal case (without variations). Dashed pink lines represent suggested requirement levels for a PCB-based filter. Note that for this type of requirement it is necessary to relax the required attentuation to 20 dB, compared to 25 dB for LTCC.

Figure A.1.3-2: Estimated S21 (left) and S11 (right) of a bandpass filter based in PCB-technology

With the above assumptions about variations, a Monte-Carlo sweep with 250 iterations was conducted. Result are shown in figure A.1.3-3, in which the dashed white trace corresponds to the nominal design and the thin blue traces correspond to 250 randomly selected filters.

Figure A.1.3-3: Estimated S21 representing transmission (left) and S11 representing return loss (right) of a bandpass filter based on PCB-technology, taking fabrication tolerances into account

From this design example, estimates of key parameters for a PCB implementation are as follows:

- About 4 dB insertion loss at the band edge.

- About 500 MHz frequency variation.

- About 1.5-2 GHz guard band to get 20-25 dB suppression respectively.

- Size: 25 mm2. This can be difficult to fit if there is one filter per antenna element.

- Return loss is relatively poor for many of the filters.

Several aspects were not considered in this analysis:

- Temperature drift will lead to additional shift of the pass-band, mainly through a change in the dielectric constant.

- Some of the randomly picked filters will violate the suggested requirement limits. The impact of this violation on the sum of unwanted emissions from all branches (and filters) will depend on the exact distribution of the set of filters used in a certain radio. For simplicity it was assumed that there is no correlation between different filters in a certain radio. This is an over-simplification in many cases.

- The use of transmission zeroes will improve steepness but may on the other hand increase tolerance sensitivity.

- Comparing to previously reported results for filters at 26.5 GHz these results are similar. When comparing one should note that the ratio between guard band and pass-band for the 26.5 GHz results is smaller in the 20 GHz case, and that more than 20 dB is targeted at 20 GHz.

In summary, approximate estimates of key parameters are:

- Guard band: ~1.5 - 2 GHz for 20 - 25 dB suppression

- Insertion loss: ~4.0 dB (at edges, with worst case frequency error)

Given the analysis for 20 GHz example frequency, the filter performance is quite similar to what was observed during NR Rel-15 for mm-wave filter analysis.

# A.2	15 GHz example frequency filtering

For 15 GHz example frequency considering the assumed attenuation requirements of 30 dB, two example 6 pole LTCC filters with bandwidth of 1 GHz and 1.5 GHz respectively were analyzed, assuming a Q-value of 190, and a tolerance/drift related frequency shift of 1 % (150 MHz). Two transmission zeros were used in an attempt to minimize the guard band. One should be aware that transmission zeroes tend to increase the sensitivity to manufacturing errors and may require more margin for tolerance/drift related frequency shift than what was assumed here. Requiring a worst-case insertion loss of 3 dB (at the band edge for a worst-case batch translation of the pass-band) the required guard band is around 0.6 GHz in both cases.  The filter response of the two example filters is presented in figure A.2-1.

Figure A.2-1: Estimated characteristics of a bandpass filter based on LTCC for example frequency of 15 GHz, 1.0 GHz bandwidth (left) and 1.5 GHz bandwidth (right)

As shown in figure A.2-1, the key characteristics for the two example filters are as following:

- Guard is ~0.6 GHz, and

- Insertion losses are ~3 dB.

# A.3	10 GHz example frequency filtering

For 10 GHz example frequency considering the assumed attenuation requirements of 40 dB, several example filters were investigated with Q-value of 150, 300 and 600, without considering any particular technology. An attempt was first made to meet very sharp slopes to handle a 0.2 GHz guard band, and achieve 2 dB insertion loss, and tested filter with 8 poles and 2 transmission zeroes, with bandwidths of 0.6 GHz and 1 GHz. Figure A.3-1 shows the result of this investigation. Different traces in the plots correspond to different Q-values. For 0.6 GHz bandwidth it would require a Q-value of 600, or actually 1000 to get the insertion loss down to 2 dB, and a frequency tolerance better than 60 MHz. For 1 GHz bandwidth the result is similar, except that the mid band insertion loss is somewhat lower, while the required tolerances are even tighter. From this investigation it appears that it is not feasible to have 0.2 GHz guard at 10 GHz with any established technology that is suitable in array systems.

Figure A.3-1: Principle investigation of what it would take to achieve a 0.2 GHz guard band, indicating that it is not feasible, neither for 0.6 GHz bandwidth (left) nor for 1 GHz bandwidth (right)

As a next step the guard band was increased to 0.5 GHz, and look at 6 pole filter with 2 transmission zero with bandwidth of 1 GHz. This gives the filter response shown in figure A.3-2. It is found that a frequency tolerance better than 1 % (100 MHz) is required, and that the worst-case insertion loss is close to 5 dB for a Q-value of 150 (the lowest blue trace). This is roughly what could potentially be achieved with an LTCC filter in best case. Note that the insertion loss does not meet the goal. To get the worst-case insertion loss down to desired 2 dB one would have to find an implementation that provides a Q-value better than approximately 400, which is very challenging in array systems.

Figure A.3-2: Estimated characteristics of a LTCC filter with 1 GHz bandwidth and 0.5 GHz guard band, for example frequency of 10 GHz

As a final step, filters with bandwidth increased to 2 GHz, still assuming 0.5 GHz and attenuation of 40 dB, were investigated, one 6-pole filter and one 8-pole filter. 2 transmission zeroes were used in both cases. The filter characteristics are presented in figure A.3-3. The 6-pole filter requires a frequency tolerance of 50 MHz, and a Q-value around 300 to get 2 dB insertion loss in worst case, while the 8-pole filter requires a frequency tolerance of 1% (100 MHz), and the same Q-value. According to these estimations, a LTCC filter could potentially achieve roughly the performance for the 8-pole filter, for the case with a Q of 150, and would give up to 4 dB insertion loss in worst case (at edge of the band for the worst sample/batch of filters).

Figure A.3-3: Estimated characteristics of a 2 GHz bandpass filter based on LTCC for example frequency of 10 GHz, 6-poles /zero (left) and 8-pole /2 zero (right)

The above predictions would have to be verified in experiments, considering the uncertainty regarding impact of manufacturing tolerances, temperature drift, and insertion loss.

One should also note that the degradation in return loss that appears due to tolerance errors was ignored, especially for uncorrelated errors. This effect can be significant but is difficult to model in a general and simple way. Additional margins in terms of guard band should be considered to handle this.

To summarize, the following was observed:

- A guard band of 0.2 GHz appears un-realistic for 10 GHz, for LTCC and PCB-integrated stripline filters regardless of passband bandwidth, due to the extreme requirements on frequency tolerance, and Q-value.

- A guard band of 0.5 GHz is potentially feasible with a LTCC filter, preferably with a relatively large bandwidth, around 1-2 GHz, in order to keep insertion loss down below 4 dB close to the band edges.

Manufacturing tolerances and temperature drift will have a significant impact on performance and must be accounted for by addition of sufficient design margin.

In summary, the filter examples investigated, imply the need to have sufficient guard-band taking to account the tolerances and drift in frequency due to e.g. temperature etc.

Annex B (informative):
Frequency ranges of interest in 7 – 24 GHz

In order to facilitate adequate and timely availability of the 7 - 24 GHz spectrum for IMT services with appropriate regulatory provisions, the RAN-level discussion was initiated capturing number of requests from operators. Three operators expressed their interest by providing frequency ranges of interest, as captured in table B-1.

Additionally, MNO’s provided their harmonized views in form of European Telecommunications Network Operators (ETNO) and GSM Association (GSMA) contributions as inputs to the CEPT ECC PT A meetings discussion on the potential additional agenda items for the WRC-2023 conference.

Africa Telecommunication Union (ATU) provided Africa’s common proposal with the list of frequency ranges of interest for studies on frequency-related matters for IMT identification including possible additional allocations to the mobile services on a primary basis in portions of the 7 – 24 GHz frequency range for the future development of IMT-2020 and beyond.

For future reference, all the above mentioned requests within 7 – 24 GHz range were listed in table B-1 and depicted on figure B-1 below.

Table B-1: Frequency ranges of interest in 7.125 – 24.250 GHz

| Source |  | Frequency range of interest (GHz) | Region, applicability |
| --- | --- | --- | --- |
| 1 | Orange | 7.125 – 8.5 21.4 – 22.0 | Region 1/ Europe (Note 1) |
| 2 | Etisalat | 10.7 – 11.7 | Region 1/ MENA (Note 1) |
| 3 | Dish Network | 12.2 – 12.7 | Region 2/ USA (Note 1) |
| 4 | ETNO [56] | 7.125 – 8.5 14.3 – 15.35 | Region 1/ Europe (Note 2, 4) |
| 5 | GSMA [57] | 7.125 – 8.5 10.7 – 11.7 14.3 / 14.5 – 15.35 | Global (Note 3, 4) |
| 6 | ATU [58] | 7.125 – 8.5 8.5 – 10.0 10.0 – 10.5 14.3 / 14.8 – 15.35 15.35 – 15.63 15.63 – 17.3 | Africa |
| NOTE 1: Interest indicated by individual company during RAN Drafts discussion, before creation of this study item. NOTE 2: Those frequency ranges offer large amounts of contiguous spectrum with primary Mobile allocation on a global basis (except 14.3 -14.4 GHz in Region 2). NOTE 3: As usage of bands varies across countries and regions, it is likely to be necessary to consider frequency bands/ranges from within which different portions may be used in different countries/regions according to their particular situations and needs. NOTE 4: Those proposals for the IMT allocation and introduction of new agenda item for WRC-23 were not agreed by CEPT during WRC-19 preparatory meetings. |  |  |  |

Figure B-1: Summary of the frequencies of interest in 7.125 – 24.25 GHz

The above initial proposals for IMT in 7 – 24 GHz and for introduction of new agenda items for WRC-23 conference were not agreed during WRC-19.

For the summary of WRC-19 conclusions for IMT in 7 – 24 GHz range, refer to clause 4.4.

Annex C:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-02 | RAN4#90 | R4-1902510 | - | - | - | First version of the draft TR | 0.0.0 |
| 2019-04 | RAN4#90 bis | R4-1903328 | - | - | - | TR number assigned | 0.0.1 |
| 2019-05 | RAN4#91 | R4-190xxxx | - | - | - | TR skeleton update | 0.0.2 |
| 2019-08 | RAN4#92 | R4-1909279 | - | - | - | Implementation of the agreed TPs on top of version 0.0.2: - R4-1906172 TP to TR 38.820: Addition of NR terminology in clause 3 - R4-1907022 TP to TR 38.820: single-band operation of NR BS - R4-1907024 TP to TR 38.820: BS EMC immunity requirements for 7-24 GHz - R4-1907792 TP to TR 38.820: BS EMC emission requirements for 7-24 GHz - R4-1907795 TP to TR 38.820: Addition of BS requirement sets in subclause 8.2 - R4-1907796 TP to TR 38.820: Addition of PA trends in subclause 6.3.1 - R4-1907798 TP to TR 38.820: Addition of example frequencies in subclause 6.2 - R4-1907799 TP to TR38.820 on Scope - R4-1907802 TP to TR 38.820: Inclusion of NF Background Information for BS requirements | 0.1.0 |
| 2019-09 | RAN4#92 | R4-1909280 | - | - | - | Implementation of the agreed TPs on top of version 0.1.0: - R4-1909290 TP to TR 38.820: aggregated system bandwidth for 7 – 24 GHz - R4-1910499 TP to 38.820 on the regulatory overview of 7-24 GHz for ITU Regions 1 and 2 - R4-1910501 TP to TR38.820: system parameters - R4-1910503 TP to TR 38.820: UE RF technology considerations in the 7-24 GHz range - R4-1910504 TP to TR 38.820: Updates on the PA trends - R4-1910505 TP to TR 38.820: Addition of BS PA trends conclusion in subclause 5.4.1 - R4-1910506 TP to TR 38.820: PA power scaling and AAS dependencies in subclause 5.4.2 - R4-1910523 TP to TR 38.820, capture transmitter architecture discussion. - R4-1910524 TP to TR 38.820: Adding reference architecture for base station requirements sets in subclause 7.2 - R4-1910526 TP to TR 38.820: placeholder for the BS RF requirements summary - R4-1910527 TP to TR38.820 - co-location requirements - R4-1910528 TP to TR38.820 - co-existence emissions requirements - R4-1910529 TP to TR 7-24GHz OOB requirement - R4-1910599 TP to TR 38.820: Frequency range characteristics - R4-1910601 TP to TR 38.820: Description of BS architectures and types | 0.2.0 |
| 2019-10 | RAN4#92bis | R4-1912208 | - | - | - | Implementation of the agreed TPs on top of version 0.2.0: - R4-1912209 TP to TR 38.820: cleanup - R4-1913004 TP to TR 38.820: Editorial update of draft TR 38.820 - R4-1911843 TP to TR 38.820: Improvement of example frequency range in subclause 5.2 - R4-1913005 TP to TR 38.820: system parameters for 7-24 GHz study - R4-1913006 TP to TR38.820: corrections of deployment scenarios - R4-1913007 TP to TR 38.820: Signal quality considerations for 7-24 GHz base station operation - R4-1913008 TP to TR 38.820: PA indicative ACLR values in subclause 5.5.2 - R4-1913034 TP to TR 38.820: Technical background for BS filter technologies for 7 to 24 GHz in subclause 5.4.5 - R4-1913011 TP to TR 38.820 updated on UE front end technology - R4-1913012 TP to TR 38.820: Addition of UE noise figure and some other clarifications in subclauses 5.5.1 & 6.3.3.1 - R4-1913032 TP to TR 38.820: Update of BS noise figure over the 7-24 GHz range - R4-1913013 TP to TR 38.820: Improvement of transmitter architecture in subclause 5.3.2 - R4-1912212 TP to TR 38.820: on Tx requirements applicability - R4-1912218 TP to TR 38.820: occupied bandwidth - R4-1913014 TP to TR 38.820 OBUE requirement for 7-24GHz - R4-1913015 TP to TR 38.820 - BS output power accuracy - R4-1913016 TP to TR 38.820 - BS TX IMD - R4-1913017 TP to TR 38.820 - BS TX OFF levels - R4-1913018 TP to TR 38.820: Improvement of technical background for co-location requirements in subclause 7.4.1.2 - R4-1913020 TP to TR 38.820: output power dynamics - R4-1913022 TP to 38.820 on Spurious emission regulation for BS in 7-24 GHz - R4-1911690 TP to TR 38.820 - BS in-band blocking - R4-1912213 TP to TR 38.820: on Rx requirements applicability - R4-1913023 TP to TR 38.820 - BS ALCR - R4-1913024 TP to TR 38.820 - BS receiver sensitivity - R4-1913026 TP to TR 38.820: NR BS dynamic range | 0.3.0 |
| 2019-12 | RAN4#93 | R4-1912208 | - | - | - | Implementation of the agreed TPs on top of version 0.3.0: - R4-1914266 TP to TR 38.820: Improvements on power amplifier section - R4-1916103 TP to TR 38.820: cleanup - R4-1916104 TP to TR 38.820 updated on UE RF Requirement - R4-1916106 TP for TR 38.820: UE RF testability - R4-1916107 TP to TR 38.820 on Transceiver technology for NR UE 7-24 - R4-1916108 TP for TR 38.820: scope of NR UE RF - R4-1916109 TP to TR 38.820: Update of technical background for BS out-of-band blocking in subclause 7.4.2.5 - R4-1916110 [7to24] TP to TS 38.820 BS Rx spurious emissions - R4-1916111 TP to TR 38.820 on ICS 7-24GHz - R4-1916112 TP to TR 38.820: NR BS in-channel selectivity (ICS) for 7-24 GHz range - R4-1916113 TP to TR 38.820: NR UE System Noise Figure - R4-1916114 TP to TR 38.820: clarification on the IM - R4-1916115 TP to TR 38.820: IAB scenario for 7-24GHz - R4-1916116 [7to24] TP to TR 38.820 BS terminology clean up - R4-1916117 TP to TR 38.820: Update of applicability of BS requirement sets for 7-24 GHz frequency range - R4-1916118 TP to TR 38.820: Conclusions on signal quality - R4-1916120 [7to24] TP to TR 38.820 BS TX summary table update - R4-1916121 [7to24] TP to TR 38.820 BS Rx conducted sensitivity - R4-1916122 TP to TR 38.820: Update of BS requirement sets for 7-24 GHz frequency range - R4-1916123 [7to24] TP to TR 38.820 BS RX summary table update - R4-1916158 TP to TR 38.820: Active RFFE technology for UE | 0.4.0 |
| 2019-12 | RAN#86 | RP-192655 | - | - | - | TR submitted for Information to RAN#86. | 1.0.0 |
| 2020-03 | RAN4#94e | R4-2002866 | - | - | - | Implementation of the agreed TPs on top of version 1.0.0: - R4-2002865 TP to TR 38.820: cleanup - R4-2002867 TP to TR 38.820: summary on the frequency bands of interest within 7-24 GHz range - R4-2002868 TP to TR 38.820: WRC-19 conclusions - R4-2002869 TP to TR 38.820: Addition of technical background for BS classes in subclause 7.3 - R4-2002871 TP to TR 38.820: Update of BS receiver requirements for 7-24 GHz frequency range - R4-2002872 TP to TR 38.820: Addition of technical background for co-location OOB receiver blocking in subclause 7.4 | 1.1.0 |
| 2020-04 | RAN4#94-e-Bis | R4-2004013 | - | - | - | Version correction and editorial corrections to the content based on R4-2002866. | 1.2.0 |
| 2020-05 | RAN4#94-e-Bis | R4-2005740 | - | - | - | Implementation of the agreed TPs on top of version 1.2.0: - R4-2004196 TP to TR 38.820: Updates to the list of frequency ranges of interest - R4-2004950 TP to TR 38.820: BS classes for 7-24 GHz frequency range - R4-2005178 TP to TR 38.820: Improvement of technical background for phase noise characteristics in subclause 5.5.3 and Annex B - R4-2005179 TP to TR 38.820: Updates to the PA survey - R4-2005180 TP to TR 38.820: ACS - R4-2005181 TP to TR 38.820: Rx IMD | 1.3.0 |
| 2020-06 | RAN4#95-e | R4-2009161 | - | - | - | Implementation of the agreed TPs on top of version 1.3.0: - R4-2008140 TP to TR 38.820: clarification on WRC-19 resolution for IMT for fixed wireless broadband in fixed services bands - R4-2008916 TP to TR 38.820: deployment scenarios cleanup - R4-2008917 TP to TR 38.820: Addition of antenna parameter selection guideline in subclause 7.2.3 - R4-2008918 TP to TR 38.820: Summary Tables for Transmitter Requirements - R4-2008919 TP to TR 38.820: Summary Tables for Receiver Requirements | 1.4.0 |