3GPP TSG-RAN WG4 Meeting # 95-e	R4-2006096

# Electronic Meeting, 25 May – 5 June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | 0118 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.141-1: Corrections on generation of test configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1) The symbol “Foffset” is not defined but is used in the note in clause 4.7.1, the wordings of which is unclear. 2) For NRTC3 generation, the undefined symbol “FOffset“ is used in clause 4.7.5.1 for sub-blocks generation. 3) For NRTC4 generation, Maximum number of supported carriers per operating band (D.17) is used for carrier placement in each supported operating band (2nd bullet in clause 4.7.6.1), but Maximum number of supported carriers in multi-band operation (D.18) is used to calculate the sum of the maximum number of supported carriers of each supported operating band (last bullet in clause 4.7.6.1). 4) For NRTC4 generation, Total number of supported carriers for the declared band combinations (D.28) which may apply for single-band connector(s) according to Operating band combination support (D.27) is used to compare to the calculated sum of the maximum number of supported carriers of each supported operating band (last bullet in clause 4.7.6.1). 5) Some declared parameters are ambiguous, or even not relevant, which leads to confusion when declaring values or referring to those parameters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Remove the undefined symbol “Foffset” and clarify the wordings in the note in clause 4.7.1. 2) For NRTC3 generation, use the correct symbols “Foffset_high“ and “Foffset_low“ in clause 4.7.5.1 for sub-blocks generation. 3) For NRTC4 generation, clarify Maximum number of supported carriers in multi-band operation (D.18) is declared per supported operating band and use it for carrier placement in each supported operating band (2nd bullet in clause 4.7.6.1). 4) For NRTC4 generation, use Total maximum number of supported carriers (D.19) to compare to the calculated sum of the maximum number of supported carriers of each supported operating band (last bullet in clause 4.7.6.1). 5) Clarify naming and description of some parameters. Propose to remove 2 parameters which seems not needed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors remain and would lead to different interpretations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6, 4.7.1, 4.7.5.1, 4.7.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Resubmission of endorsed Draft CR R4-2005603 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

## 4.6	Manufacturer declarations

The following BS declarations listed in table 4.6-1, when applicable to the BS under test, are required to be provided by the manufacturer for the conducted requirements testing of the BS type 1-C and BS type 1-H.

For the BS type 1-H declarations required for the radiated requirements testing, refer to TS 38.141-2 [3].

Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.1 | BS requirements set | Declaration of one of the NR base station requirement's set as defined for BS type 1-C, or BS type 1-H. | x | x |
| D.2 | BS class | BS class of the BS, declared as Wide Area BS, Medium Range BS, or Local Area BS. | x | x |
| D.3 | Operating bands and frequency ranges | List of NR operating band(s) supported by single-band connector(s) and/or multi-band connector(s) of the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in. Declarations shall be made per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.4 | Spurious emission category | Declare the BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. | x | x |
| D.5 | Additional operating band unwanted emissions | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.6.4.5.6 apply. (Note 3). | x | x |
| D.6 | Co-existence with other systems | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another band are deployed. | x | x |
| D.7 | Co-location with other base stations | The manufacturer shall declare whether the BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another band. | x | x |
| D.8 | Single band connector or multi-band connector | Declaration of the single band or multi-band capability of single band connector(s) or multi-band connector(s), declared for every connector. | x | x |
| D.9 | Contiguous or non-contiguous spectrum operation support | Ability to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier. Declared per single band connector or multi-band connector, per operating band. | x | x |
| D.10 | void | void | x | x |
| D.11 | Maximum Base Station RF Bandwidth | Maximum Base Station RF Bandwidth in the operating band for single-band operation. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. (Note 2) | x | x |
| D.12 | Maximum Base Station RF Bandwidth for multi-band operation | Maximum Base Station RF Bandwidth for multi-band operation. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.13 | Total RF bandwidth (BWtot) | Total RF bandwidth BWtot of transmitter and receiver, declared per the band combinations (D.27). | x | x |
| D.14 | NR supported channel bandwidths and SCS | NR supported SCS and channel bandwidths per supported SCS. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.15 | CA only operation | Declaration of CA-only operation (with equal power spectral density among carriers) but not multiple carriers, declared per operating band per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.16 | Single or multiple carrier | Capable of operating with a single carrier (only) or multiple carriers. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.17 | Maximum number of supported carriers per operating band in single band operation | Maximum number of supported carriers per supported operation band in single band operation. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. (Note 2) | x | x |
| D.18 | Maximum number of supported carriers per operating band in multi-band operation | Maximum number of supported carriers per supported operation band in multi-band operation. (Note 2) | x | x |
| D.19 | Total maximum number of supported carriers in multi-band operation | Maximum number of supported carriers for all supported operating bands in multi-band operation. Declared for all connectors (D.18). | x | x |
| D.20 | Other band combination multi-band restrictions | Declare any other limitations under simultaneous operation in the declared band combinations (D.35) for each multi-band connector which have any impact on the test configuration generation. Declared for every multi-band connector. | x | x |
| D.21 | Rated carrier output power (Prated,c,AC, or Prated,c,TABC) | Conducted rated carrier output power, per single band connector or multi-band connector. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. (Note 1, 2) | x | x |
| D.22 | Rated total output power (Prated,t,AC, or Prated,t,TABC) | Conducted total rated output power. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. For multi-band connectors declared for each supported operating band in each supported band combination. (Note 1, 2) | x | x |
| D.23 | Rated multi-band total output power, Prated,MB,TABC | Conducted multi-band rated total output power. Declared per supported operating band combinations, per multi-band connector. (Note 1) | x | x |
| D.24 | Ncells | Number corresponding to the minimum number of cells that can be transmitted by a BS in a particular operating band with transmission on all TAB connectors supporting the operating band. |  | x |
| D.25 | Maximum supported power difference between carriers | Maximum supported power difference between carriers. Declared per supported operating band, per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.26 | Maximum supported power difference between carriers is different operating bands | Supported power difference between any two carriers in any two different supported operating bands. Declared per supported operating band combination, per multi-band connector. | x | x |
| D.27 | Operating band combination support | List of operating bands combinations supported by single-band connector(s) and/or multi-band connector(s) of the BS. Declared per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.28 | void | void | x | x |
| D.29 | Intra-system interfering signal declaration list | List of single band connector(s) or multi-band connector(s) for which an intra-system interfering signal level is required to be declared. Declaration is required if the intra-system interfering signal level is larger than the co-location interfering signal level. |  | x |
| D.30 | Intra-system interfering signal level | The interfering signal level in dBm. Declared per supported operating band, per TAB connector for BS type 1-H covered by D.29. |  | x |
| D.31 | TAE groups | Set of declared TAB connector beam forming groups on which the TAE requirements apply. All TAB connectors belong to at least one TAB connector beam forming group (even if it's a TAB connector beam forming group consisting of one connector). The smallest possible number of TAB connector beam forming groups need to be declared such that there is no TAB connector not contained in at least one of the declared TAB connector beam forming groups. Declared per supported operating band. |  | x |
| D.32 | Equivalent connectors | List of antenna connectors of BS type 1-C, or TAB connector of BS type 1-H, which have been declared equivalent. Equivalent connectors imply that the antenna connector of BS type 1-C, or TAB connector of BS type 1-H, are expected to behave in the same way when presented with identical signals under the same operating conditions. All declarations made for the antenna connector of BS type 1-C, or TAB connector of BS type 1-H are identical and the transmitter unit and/or receiver unit driving the antenna connector of BS type 1-C or TAB connector of BS type 1-H are of identical design. | x | x |
| D.33 | TAB connector RX min cell group | Declared as a group of TAB connectors to which RX requirements are applied. This declaration corresponds to group of TAB connectors which are responsible for receiving a cell when the BS type 1-H setting corresponding to the declared minimum number of cells (Ncells) with transmission on all TAB connectors supporting an operating band. |  | x |
| D.34 | TAB connector TX min cell group | Declared group of TAB connectors to which TX requirements are applied. This declaration corresponds to group of TAB connectors which are responsible for transmitting a cell when the BS type 1-H setting corresponding to the declared minimum number of cells (Ncells) with transmission on all TAB connectors supporting an operating band. |  | x |
| D.35 | Connecting network loss range for BS testing with ancillary RF amplifiers | Declaration of the range of connecting network losses (in dB) for BS type 1-C testing with ancillary Tx RF amplifier only, or with Rx RF amplifier only, or with combined Tx/Rx RF amplifiers. (Note 4) | x |  |
| D.36 | Relation between supported maximum RF bandwidth, number of carriers and Rated total output power | If the rated total output power and total number of supported carriers are not simultaneously supported, the manufacturer shall declare the following additional parameters: - The reduced number of supported carriers at the rated total output power; - The reduced total output power at the maximum number of supported carriers. | x | x |
| D.37 | TAB connectors used for performance requirement testing | To reduce test complexity, declaration of a representative (sub)set of TAB connectors to be used for performance requirement test purposes. At least one TAB connector mapped to each demodulation branch is declared. |  | x |
| D.38 | Inter-band CA | Band combinations declared to support inter-band CA (per CA capable multi-band connector(s), as in D.15). Declared for every multi-band connector which support CA. | x | x |
| D.39 | Intra-band contiguous CA | Bands declared to support intra-band contiguous CA (per CA capable single band connector(s) or multi-band connector(s), as in D.15). Declared per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.40 | Intra-band non-contiguous CA | Bands declared to support intra-band non-contiguous CA (per CA capable single band connector(s) or multi-band connector(s), as in D.15). Declared per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| D.41 | NB-IoT operation | Manufacturer shall declare the support of NB-IoT operation in NR in-band and the number of supported NB-IoT carriers in total and for each supported band, frequency range and channel bandwidth. | x |  |
| D.42 | NB-IoT sub-carrier spacing | If the BS supports NB-IoT operation in NR in-band, manufacturer shall declare if it supports 15 kHz sub-carrier spacing, 3.75 kHz sub-carrier spacing, or both for NPUSCH. | x |  |
| D.43 | NB-IoT power dynamic range | If the BS supports NB-IoT operation in NR in-band, manufacturer shall declare the maximum power dynamic range it could support with a minimum of +6dB or +3dB as specified in clause 6.3.4 of TS 38.104 [2] (Note 5). | x |  |
| D.100 | PUSCH mapping type | Declaration of the supported PUSCH mapping type as specified in TS 38.211 [17], i.e., type A, type B or both. | x | x |
| D.101 | PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s), i.e., pos0, pos1 or both. |  |  |
| D.102 | PUCCH format | Declaration of the supported PUCCH format(s) as specified in TS 38.211 [17], i.e., format 0, format 1, format 2, format 3, format 4. | x | x |
| D.103 | PRACH format and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: 0, A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both. | x | x |
| D.104 | Additional DM-RS for PUCCH format 3 | Declaration of the supported additional DM-RS for PUCCH format 3: without additional DM-RS, with additional DM-RS or both. | x | x |
| D.105 | Additional DM-RS for PUCCH format 4 | Declaration of the supported additional DM-RS for PUCCH format 4: without additional DM-RS, with additional DM-RS or both. | x | x |
| D.106 | PUCCH multi-slot | Declaration of multi-slot PUCCH support. | x | x |
| D.107 | UL CA | For the highest supported SCS, declaration of the carrier combination with the largest aggregated bandwidth. If there is more than one combination, the carrier combination with the largest number of carriers shall be declared. | x | x |
| NOTE 1: If a BS is capable of 256QAM DL operation then two rated output power declarations may be made. One declaration is applicable when configured for 256QAM transmissions and the other declaration is applicable when not configured for 256QAM transmissions. NOTE 2: Parameters for contiguous or non-contiguous spectrum operation in the operating band are assumed to be the same unless they are separately declared. When separately declared, they shall still use the same declaration identifier. NOTE 3: If BS is declared to support Band n20 (D.3), the manufacturer shall declare if the BS may operate in geographical areas allocated to broadcasting (DTT). Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 4: This manufacturer declaration is optional. NOTE 5: This manufacturer may declare two values, one with a minimum of +6dB and the other with a minimum of +3dB. |  |  |  |  |

<Next change>

### 4.7.1	General

The test configurations shall be constructed using the methods defined below, subject to the parameters declared by the manufacturer for the supported RF configurations as listed in clause 4.6. The test configurations to use for conformance testing are defined for each supported RF configuration in clauses 4.8.3 and 4.8.4.

The applicable test models for generation of the carrier transmit test signal are defined in clause 4.9.

NOTE:	If required, carriers are shifted to align with the channel raster.

<Next change>

#### 4.7.5.1	NRTC3 generation

NRTC3 is constructed on a per band basis using the following method:

-	The Base Station RF Bandwidth shall be the maximum Base Station RF Bandwidth supported for non-contiguous spectrum operation (D.11). The Base Station RF Bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum supported Base Station RF Bandwidth (D.11).

-	Select the carrier to be tested according to 4.7.2. Place it adjacent to the upper Base Station RF Bandwidth edge and another carrier (as described in 4.7.2) adjacent to the lower Base Station RF Bandwidth edge.

-	If NB-IoT operation in NR in-band is supported, place the power boosted NB-IoT RB at the lower outermost RB eligible for NB-IoT operation in NR in-band according to clause 5.7.3 of TS 36.104 [22] and the definition in clause 3.1. If more than one NB-IoT carrier is supported with NB-IoT operation in NR in-band, place the power boosted NB-IoT RB at the upper outermost RB eligible for NB-IoT operation in NR in-band.

-	For single-band operation receiver tests, if the remaining gap is at least 15 MHz (or 60 MHz if channel bandwidth of the carrier to be tested is 20 MHz) plus two times the channel BW used in the previous step and the BS supports at least 4 carriers, place a carrier of this BW adjacent to each already placed carrier for each sub-block. The nominal channel spacing defined in TS 38.104 [2], clause 5.4.1 shall apply.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified Foffset_high and Foffset_low for the carriers adjacent to the sub-block gap.

<Next change>

#### 4.7.6.1	NRTC4 generation

NRTC4 is based on re-using the previously specified test configurations (NRTC1, NRTC2 and NRTC3) applicable per band involved in multi-band operation. It is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth in multi-band operation (D.12).

-	The number of carriers of each supported operating band shall be the declared maximum number of supported carriers per operating band in multi-band operation (D.18). Carriers shall be selected according to 4.7.2 and shall first be placed at the outermost edges of the declared maximum Radio Bandwidth. Additional carriers shall next be placed at the Base Station RF Bandwidths edges, if possible.

-	The allocated Base Station RF Bandwidth of the outermost bands shall be located at the outermost edges of the declared maximum Radio Bandwidth.

-	Each concerned band shall be considered as an independent band and the carrier placement in each band shall be according to NRTC1, where the declared parameters for multi-band operation shall apply. The mirror image of the single-band test configuration shall be used in each alternate band(s) and in the highest band being.

-	If only three carriers are supported, two carriers shall be placed in one band according to the relevant test configuration while the remaining carrier shall be placed at the edge of the maximum Radio Bandwidth in the other band.

-	If the sum of the maximum Base Station RF Bandwidths of each supported operating bands is larger than the declared Total RF Bandwidth BWtot (D.13) of transmitter and receiver for the declared band combinations of the BS, repeat the steps above for test configurations where the Base Station RF Bandwidth of one of the operating band shall be reduced so that the Total RF Bandwidth of transmitter and receiver is not exceeded and vice versa.

-	If the sum of the maximum number of supported carriers per operating band in multi-band operation (D.18) is larger than the declared total maximum number of supported carriers in multi-band operation (D.19), repeat the steps above for test configurations where in each test configuration the number of carriers of one of the operating band shall be reduced so that the total number of supported carriers is not exceeded and vice versa.

<End of change>