3GPP TSG-RAN WG4 Meeting #95-e 	R4-2008342

Electronic Meeting, 25th May. 2020 – 5th June. 2020

Source: 	Nokia, Verizon

Title: 	TP for 37.716-11-11 to introduce DC_5_n5

Agenda Item: 			8.3.2 [DC_R16_1BLTE_1BNR_2DL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.716-11-11 to introduce DC_5_n5 as defined in the WID [1].

# 2	Text Proposal

*********************** Start of the TP ***************************************************

### 6.4.x	DC_5_n5

#### 6.4.x.1	Channel bandwidths per operating band for CA

Table 6.4.x.1-1: Band combinations for intra-band non-contiguous EN-DC

| EN-DC Band Uplink Combination | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5_n5 | 5 | n5 | Yes |

Table 6.4.x.1-2: Supported channel bandwidths per DC configuration

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | Channel bandwidths for LTE carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for LTE carrier (MHz) |  |  |
| DC_5A_n5A | DC_5A_n5A1 | 5, 10 | 5, 10, 15 |  | 20 | 0 |
| NOTE 1: Only single switched UL is supported |  |  |  |  |  |  |

#### 6.4.x.2	Configuration for DC

Table 6.4.x.2-1:  Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A_n5A | DC_5A_n5A2 | 5A | n5A |
| NOTE 2: Only single switched UL is supported |  |  |  |

#### 6.4.x.3	Maximum output power for DC

Table 6.4.x.3-1: Maximum output power for  Intra-band non-contiguous EN-DC

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_5A_n5A2 |  |  | 23 | +2/-3 |
| NOTE 2: Only single switched UL is supported |  |  |  |  |

#### 6.4.x.4	Spurious emission band UE co-existence for DC

Note that only Single Tx Switched UL mode is supported for this combination, no spurious emission band UE co-existence requirement is needed

#### 6.4.x.5	MSD analysis for DC

Note that only Single Tx Switched UL mode is supported for this combination, no further MSD is required besides that given in section 6.4.x.6.

#### 6.4.x.6	∆RIBNC values

The delta R IBNC values for intra-band non-contiguous EN-DC DC_5_n5 are shown in Table 6.4.x.6-1. The value is derived from values specified for E-UTRA CA_5A-5A as given in Table 7.3.1A-3 of TS 36.101.

Table 6.4.x.6-1: Intra-band non-contiguous EN-DC with one uplink configuration for reference sensitivity (NR carrier is higher than the E-UTRA carrier)

| DC configuration | Aggregated channel bandwidth |  | Wgap / (MHz) | UL NR allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | NR | E-UTRA |  |  |  |  |
| DC_5A_n5A | 5 MHz | 5 MHz | NOTE 10 | 121 | 5.3 | FDD |
|  | 10 MHz | 5 MHz |  |  | 4.4 |  |
|  | 15 MHz | 5 MHz |  |  | 6.1 |  |
|  | 5 MHz | 10 MHz |  |  | 5.9 |  |
|  | 10 MHz | 10 MHz |  |  | 4.6 |  |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The table only applies when the center frequency of NR carrier is higher than the E-UTRA carrier, and the ΔRIBNC applies to the E-UTRA DL carrier only. NOTE 4: All combinations of channel bandwidths defined in Table 5.3B.1.3-1. NOTE 5: 5 refers to the UL resource blocks shall be located at RBstart=25. NOTE 6: 6 refers to the UL resource blocks shall be located at RBstart=35. NOTE 7: 7 refers to the UL resource blocks shall be located at RBstart=50. NOTE 8: 8 refers to the UL resource blocks shall be located at RBstart=60. NOTE 9: 9 refers to the UL resource blocks shall be located at RBstart=75. NOTE 10: All applicable sub-block gap sizes. |  |  |  |  |  |  |

#### 6.4.x.7	MPR, AMPR requirements

Note that only Single Tx Switched UL mode is supported for this combination, no MPR or A-MPR requirement is needed

*********************** End of the TP ***************************************************

# 3	References

RP-192742 - Revised WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band + 1 NR band)