3GPP TSG-RAN WG4 Meeting # 95e	 												R4-2008490

Electronic Meeting, 25 May – 5 June, 2020

Agenda item:			4.9

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [95e][201] NR_NewRAT_RRM_Core

Document for:	Information

Introduction

This contribution provides the summary of Rel-15 NR RRM core maintenance in Agenda 4.9. It will be used to capture the comments in the 1st round and 2nd round. The tentative agreements will be provided based on the proposals and comments.

Topic #1: UE measurement capability

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2007713 | Huawei, Hisilicon | Discussion on reporting criteria for EN-DC Proposal 1: The equation in TS 38.133 for EN-DC NR reporting criteria shall be modified as follows to reflect the total number including NR inter-RAT configured by PCell. Ecat,EN-DC,NR=10+10+9×n is the total number of NR reporting criteria configured by PSCell and E-UTRA PCell applicable for UE configured with EN-DC according to Table 9.1.4.2-1, and is the number of configured NR serving frequencies, including PSCell and SCells carrier frequencies |
| R4-2007710 | Huawei, Hisilicon | CR on NR reporting criteria for EN-DC (CR) Based on the endored CR R4-2005429, the total number NR reporting criteria is refer to TS 38.133. However, the NR inter-RAT configured by PCell is missing in the equation in TS 38.133. Add the NR inter-RAT in the Ecat,EN-DC,NR |
| R4-2007709 | Huawei, Hisilicon | CR on NR reporting criteria for EN-DC (CR, Cat A) |
| R4-2007638 | ZTE | CR to 36.133 on NR reporting criteria (CR) Resubmission of endorsed Draft CR R4-2005429 For EN-DC, reporting criteria has not been specified when E-UTRA SCell carrier frequencies are configured. For NE-DC, reporting criteria has not been finalized and reporting criteria has not been specified when E-UTRA SCell carrier frequencies are configured. The structure of the reporting criteria requirements should be aligned with agreed WF R4-2005266 - Specified reporting criteria requirements for EN-DC when E-UTRA SCell carrier frequencies are configured. - Specified reporting criteria requirements for NE-DC when E-UTRA SCell carrier frequencies are configured. - Structure of the reporting criteria requirements are re-organzied. - Change the property of Table 8.2.2-1 so it can be on the same page with the title. - Editorial changes |
| R4-2007639 | ZTE | CR to 36.133 on NR reporting criteria (CR Cat A) |
| R4-2007961 | Ericsson | NR reporting criteria (CR) The draft CR endorsed in RAN4#94-e-Bis has some issues. It was not fully based on 38.133 either, which is not aligned with the dsicussion and agreement. Reporting criteria for EN-DC and NE-DC are further clarified. Square brackets are removed too. |
| R4-2007962 | Ericsson | NR reporting criteria (CR Cat A) |
| R4-2006880 | Mediatek Inc., Huawei, Hisilicon, Apple, Intel | CR on TS38.133 for modification on number of cells and number of SSB to be measured for FR2 intra-frequency measurement (CR) For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PCI, Current spec mandates UE to monitor 6 cells and 24 SSBs for each intra-frequency layer in FR2: Finalize the UE measurement capability of the cell number and SSB number: FR2. Intra-Frequency. #cells; #SSBs. 6 only for one CC in each band (for Pcell or PSCell , or if PCell or PSCell is not on FR2 then it is for any Scell within the band); 24 only for one CC and [1 to 4] for each of the other CCs and all of these beams are from Scell.. However, the agreement in RAN4 #87 meeting is “UE is required to measure 6 cells and 24 SSBs on only 1 serving carrier in a FR2 band”: Clarify that UE is only required to measure 6 cells and 24 SSBs on 1 serving carrier in a FR2 band. |
| R4-2006881 | Mediatek Inc., Huawei, Hisilicon, Apple, Intel | CR on TS38.133 for modification on number of cells and number of SSB to be measured for FR2 intra-frequency measurement (CR Cat A) |

Open issues summary

Sub-topic 1-1

Based on the endorsed CR R4-2005429, the total number NR reporting criteria is refer to TS 38.133. However, the NR inter-RAT configured by PCell is missing in the equation in TS 38.133

Issue 1-1: Update EN-DC NR reporting criteria to reflect the total number including NR inter-RAT configured by PCell

- Proposals

  - The equation in TS 38.133 for EN-DC NR reporting criteria shall be modified as follows to reflect the total number including NR inter-RAT configured by PCell.

    - E_(cat,EN-DC,NR)=10+10+9×n is the total number of NR reporting criteria configured by PSCell and E-UTRA PCell applicable for UE configured with EN-DC according to Table 9.1.4.2-1, and   is the number of configured NR serving frequencies, including PSCell and SCells carrier frequencies,

- Recommended WF

  - TBA

Sub-topic 1-2

Please provide the comments on CRs directly in Section 1.3.2.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub topic 1-1: Ok, but for the CR needs to be clarified that these are E-UTRA-NR inter-RAT specified in 36.133, otherwise one may confuse with NR-E-UTRA inter-RAT which are in 38.133 but not NR carriers and there is no HO to E-UTRA so these are not applicable for EN-DC (also obvious from notes in 38.133). …. Others: |
| Huawei | Sub-topic 1-1: To Ericsson comments, we can revise the CR to clarify these are E-UTRA-NR inter-RAT specified in 36.133, as the way inter-RAT E-UTRA reporting criteria configured by PCell in NE-DC is defined in 36.133. |
| ZTE | The reporting criteria for inter-RAT NR measurements configured by E-UTRA PCell has been included in Ecat,EN-DC,E-UTRA. Ecat,EN-DC,E-UTRA is the total number of E-UTRA reporting criteria configured by PCell, except NR PSCell and NR SCells carrier frequencies: If it is added in Ecat,EN-DC,NR=10+9×n , the inter-RAT NR measurements reporting criteria on will be calculated twice. If it is necessary, changing the wording for Ecat,EN-DC,NR , or add a note to say that reporting criteria for inter-RAT NR measurement configured by E-UTRA PCell is not included in Ecat,EN-DC,NR |

CRs/TPs comments collection

The following CRs will be discussed in this section.

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2007710 | Huawei, Hisilicon | CR on NR reporting criteria for EN-DC (CR) |
| R4-2007709 | Huawei, Hisilicon | CR on NR reporting criteria for EN-DC (CR, Cat A) |
| R4-2007638 | ZTE | CR to 36.133 on NR reporting criteria (CR) |
| R4-2007639 | ZTE | CR to 36.133 on NR reporting criteria (CR Cat A) |
| R4-2007961 | Ericsson | NR reporting criteria (CR) |
| R4-2007962 | Ericsson | NR reporting criteria (CR Cat A) |
| R4-2006880 | Mediatek Inc., Huawei, Hisilicon, Apple, Intel | CR on TS38.133 for modification on number of cells and number of SSB to be measured for FR2 intra-frequency measurement (CR) |
| R4-2006881 | Mediatek Inc., Huawei, Hisilicon, Apple, Intel | CR on TS38.133 for modification on number of cells and number of SSB to be measured for FR2 intra-frequency measurement (CR Cat A) |

Please provide comments in the table below.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007710 R4-2007709 (Cat A) | Ericsson : Ok, but needs to be clarified that these are E-UTRA-NR inter-RAT specified in 36.133, otherwise one may confuse with NR-E-UTRA inter-RAT which are in 38.133 but not NR carriers and there is no HO to E-UTRA so these are not applicable for EN-DC (also obvious from notes in 38.133). |
|  | Huawei: We can revise the CR to clarify these are E-UTRA-NR inter-RAT specified in 36.133, as the way inter-RAT E-UTRA reporting criteria configured by PCell in NE-DC is defined in 36.133. |
|  | ZTE : If it is necessary, changing the wording for Ecat,EN-DC,NR , or adding a note to say that reporting criteria for inter-RAT NR measurement configured by E-UTRA PCell is not included in Ecat,EN-DC,NR |
|  | Nokia: More discussion is needed among companies on the similar topic (R4-2007961, R4-2007638, R4-2007710), We had discussion paper and CR in last meeting as well and views from there are still valid. |
| R4-2007638 R4-2007639 (Cat A) | Resubmission of endorsed Draft CR R4-2005429 |
|  | Ericsson : Need to discuss whether the additional changes in R4-2007961 are OK as well. |
|  | Company B |
| R4-2007961 R4-2007962 (Cat A) | ZTE: - Adding ‘, where for UE configured with PCell and NR PSCell:’ and , ‘where for UE configured with PSCell and NR PCell:’ is not necessary. The context already indicates this. - The following changes are not fine since in 38.133 description are added for both NR and E-UTRA. These changes don’t help improve clarity of the requirements. Ecat,EN-DC,NR is defined in section 9.1.4.2 in TS 38.133 [50], and Ecat,NE-DC,NR is defined in section 9.1.4.2 in TS 38.133 [50], and Ecat,NE-DC,E-UTRA,inter-RAT is defined in section 9.1.4.2 in TS 38.133 [50], - It is not necessary to change ‘n’ to ‘k’. We see no point to do so. - We are okay to all the other changes. - In addition, changes as highlighted follows are necessary by taking the changes in the CR. The UE, which is capable of supporting and configured with EN-DC operation with PCell and NR PSCell and one or more NR carrier frequencies in total shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1 in the current section and table 9.1.4.2-1 in TS 38.133 [50]. The UE, which is capable of supporting and configured with NE-DC operation with PSCell and NR PCell and one or more NR carrier frequencies in total shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1 in the current section and table 9.1.4.2-1 in TS 38.133 [50]. |
|  | Ericsson : We have the following comments to ZTE’s feedback, which we copy and provide our further comments in-line. - Adding ‘, where for UE configured with PCell and NR PSCell:’ and , ‘where for UE configured with PSCell and NR PCell:’ is not necessary. The context already indicates this. [Ericsson] it is helpful for clarity in the counting, to avoid mentioning these cells in the reporting criteria description - The following changes are not fine since in 38.133 description are added for both NR and E-UTRA. These changes don’t help improve clarity of the requirements. Ecat,EN-DC,NR is defined in section 9.1.4.2 in TS 38.133 [50], and [Ericsson] we do not agree to duplicate the text, we only need to refer to it. Furthermore, there are already mistakes in ZTEs in the attempt to duplicate the text.Ecat,NE-DC,NR is defined in section 9.1.4.2 in TS 38.133 [50], and [Ericsson] same comment as above. Ecat,NE-DC,E-UTRA,inter-RAT is defined in section 9.1.4.2 in TS 38.133 [50], [Ericsson] same comment as above. - It is not necessary to change ‘n’ to ‘k’. We see no point to do so. [Ericsson] more convenient, to use different letter for NR and LTE reporting criteria. - We are okay to all the other changes. - In addition, changes as highlighted follows are necessary by taking the changes in the CR. The UE, which is capable of supporting and configured with EN-DC operation with PCell and NR PSCell and one or more NR carrier frequencies in total shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1 in the current section and table 9.1.4.2-1 in TS 38.133 [50]. The UE, which is capable of supporting and configured with NE-DC operation with PSCell and NR PCell and one or more NR carrier frequencies in total shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1 in the current section and table 9.1.4.2-1 in TS 38.133 [50]. [Ericsson] do not understand the comment, what is the issue? |
|  | Nokia: More discussion is needed among companies on the similar topic (R4-2007961, R4-2007638, R4-2007710), We had discussion paper and CR in last meeting as well and views from there are still valid. |
| R4-2006880 R4-2006881 (Cat A) | Ericsson : OK |
|  | MTK: These are also the resubmission of endorsed CR R4-2005411 |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 | 4 companies participates in the discussion. 3 companies are OK to add the clarification that the related reporting criteria are for E-UTRA-NR inter-RAT measurement. 1 company suggested to have further discussion together with ZTE CRs. The issue is related to ZTE CR. From moderator perspective, it is suggested to close the topic given that ZTE CR were endorsed in last meeting. Tentative agreements: Revise the CR to clarify these are E-UTRA inter-RAT NR measurement and capture the other concrete comments if any. Candidate options: Recommendations for 2nd round: Revise R4-2007710 to capture the comment from companies. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2007710 | Revised |
| R4-2007709 | Return to. Cat A CR to R4-2007710. |
| R4-2007638 | Return to. Ericsson commented to see if the additional changes are acceptable. Nokia proposed to have further discussion. If no further consensus was reached on the additional changes, it is proposed to formally agree on this CR. The additional changes will be discussed in future meeting based on companies’ contributions. |
| R4-2007639 | Return to. Cat A CR to R4-2007638 |
| R4-2007961 | Return to. If the additional changes are accepted, then the status of CR will be marked as “Merged” |
| R4-2007962 | Withdrawn. Cat A CR to R4-2007961. |
| R4-2006880 | Agreed. |
| R4-2006881 | Agreed. |

Discussion on 2nd round (if applicable)

In the second round the CRs R4-2007710, R4-2007638/9, and R4-2007961 will be further discussed in an email thread. Please ZTE trigger the email discussion.

[Comments and responses will be captured by moderator here]

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2007710 |  |
| R4-2007709 |  |
| R4-2007638 |  |
| R4-2007639 |  |
| R4-2007961 |  |

Topic #2: RRM measurement and measuremnet gap

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006878 | Mediatek Inc., Huawei, Hisilicon, Apple | CR on TS38.133 for modification of the layer 3 and layer 1 measurement sharing factor when both SSB and RSSI symbol to be measured are considered (CR) Revise the conditions for Klayer1_measurement =1, - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped by with any of the SSB symbols and the RSSI symbols, indicated by SSB-ToMeasure and 1 symbol before each consecutive SSB symbols and RSSI symbols indicated by SSB-ToMeasure and 1 symbol after each consecutive SSB symbols and RSSI symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure and SS-RSSI-Measurement isare configured, where SSB symbols are indicated by SSB-ToMeasure and RSSI symbols are indicated by SS-RSSI-Measurement; |
| R4-2006879 | Mediatek Inc., Huawei, Hisilicon, Apple | CR on TS38.133 for modification of the layer 3 and layer 1 measurement sharing factor when both SSB and RSSI symbol to be measured are considered (CR, Cat A) |
| R4-2006185 | Apple | CR on CSSF correction for R15 TS38.133 (CR) 1. The CSSF outside MG in current spec does not consider the following cases: 1. In EN-DC with FR1 +FR2 CA (FR1 PSCell) only one SCell is configured only one NR Scell is configured 2. in FR1 +FR2 CA (FR1 PCell) only one SCell is configured only one NR Scell is configured 3. in FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell) no SCell is configured 4. in NE-DC with FR1 +FR2 CA (FR1 PCell) only one SCell is configured only one NR Scell is configured Add the note in the CSSF table for the above cases |
| R4-2006186 | Apple | CR on CSSF correction for R16 TS38.133 (CR Cat A) |
| R4-2007757 | Huawei, Hisilicon | Correction on gap pattern applicability in TS 36.133 R15 (CR) [R4-2005412] was endorsed at RAN4#94bis-e. The common understanding is that for UE supporting ENDC, NEDC and LTE standalone, all gap patterns #0~11 can be configured for measurement of NR carrier only, and gap pattern#0, 1, 2, 3, 4, 6, 7, 8, 10 can be configured for measurement of E-UTRA carrier and inter-RAT NR carrier. However in existing specification, only NE-DC operation had explicit description. This would lead misunderstanding that the above restriction in gap applicability table specified in Table 8.1.2.1-1 can only for apply for NEDC. Clarify that the appliability restriction of gap pattern #0, 1, 2, 3, 4, 6, 7, 8, 10 apply for ENDC, NEDC and LTE standalone operation. |
| R4-2007758 | Huawei, Hisilicon | Correction on gap pattern applicability in TS 36.133 R16 (CR Cat A) |
| R4-2007805 | Huawei, HiSilicon, MediaTek, Ericsson, Nokia, Nokia Shanghai Bell | CR on FR2 measurement requirements outside gaps R15 (CR) DratfCR endorsed in R4-2005267 in RAN4#94-e-bis UE Rx beam scheduling will be quite complex to account for all combinations of SMTC periods and offsets in different CCs. To reduce the implementation complexity, the applicability for FR2 intra-frequency measurement requirements should be defined. Define applicability for FR2 intra-freqeuncy measurement requirements. |
| R4-2007806 | Huawei, HiSilicon, MediaTek, Ericsson, Nokia, Nokia Shanghai Bell | CR on FR2 measurement requirements outside gaps R16 (CR Cat A) |
| R4-2006602 | Nokia, Nokia Shanghai Bell | Correction of CFRA RSRP threshold (CR) CR implementing endorsed draftCR R4-2003395. Parameter for RSRP CFRA threshold was changed on clause 6.3.2 of 38.331 after version 15.1.0. Replacing cfra-csirs-DedicatedRACH-Threshold by rsrp-ThresholdCSI-RS. |
| R4-2006603 | Nokia, Nokia Shanghai Bell | Correction of CFRA RSRP threshold (CR Cat A) |
| R4-2007807 | Huawei, HiSilicon | CR to remove RSTD requirements for NE-DC in 36.133 R15 (CR) DratfCR endorsed in R4-2004334 in RAN4#94-e-bis In section 8.19.4 of 36.133, intra-frequency RSTD measurement requirements are specified for NE-DC. However, in NE-DC LPP message can only be transmitted from NR PCell, so LTE PSCell cannot configure RSTD measurement. Therefore, the corresponding requirements should be removed from 36.133. During RAN4#94-e, there were different views about whether the RSTD measuremnet configured by LPP via NR PCell on LTE serving frequency can be considered as LTE intra-frequency. In our view, this measurement should be same as other RRM measurement configured by NR PCell on the LTE serving frequency, which are considered as inter-RAT measurement but the LTE intra-frequency requirements apply. This is already captrued in section 9.4.1 in 38.133. Remove intra-frequency RSTD measurement requirements for NE-DC from 36.133. Note that a companion CR R4-20xxxx to 38.133 is submitted to update inter-RAT RSTD measurement requirements in 38.133. |
| R4-2007808 | Huawei, HiSilicon | CR to remove RSTD requirements for NE-DC in 36.133 R16 (CR Cat A) |
| R4-2007809 | Huawei, HiSilicon | CR on inter-RAT RSTD requirements for NE-DC in 38.133 R15 (CR) DratfCR endorsed in R4-2005268 in RAN4#94-e-bis In section 8.19.4 of 36.133, intra-frequency RSTD measurement requirements are specified for NE-DC. However, in NE-DC LPP message can only be transmitted from NR PCell, so LTE PSCell cannot configure RSTD measurement. Therefore, the corresponding requirements should be removed from 36.133. In 38.133 it is specified that for NE-DC the inter-RAT RSTD measurement configured by NR PCell on LTE serving frequency apply the measurement requirements in section 8.19 in 36.133. Since there is no LTE intra-freqeuncy RSTD measurement in NE-DC, it should refer to the LTE SA intra-freqeuncy RSTD measurement requirements. |
| R4-2007810 | Huawei, HiSilicon | CR on inter-RAT RSTD requirements for NE-DC in 38.133 R16 (CR Cat A) |

Open issues summary

Sub-topic 2-1

Please provide the comments on CRs directly in Section 2.3.1.

Companies views’ collection for 1st round

CRs/TPs comments collection

The following CRs will be discussed in this section.

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006185 | Apple | CR on CSSF correction for R15 TS38.133 (CR) |
| R4-2006186 | Apple | CR on CSSF correction for R16 TS38.133 (CR Cat A) |
| R4-2007757 | Huawei, Hisilicon | Correction on gap pattern applicability in TS 36.133 R15 (CR) [R4-2005412] was endorsed at RAN4#94bis-e. |
| R4-2007758 | Huawei, Hisilicon | Correction on gap pattern applicability in TS 36.133 R16 (CR Cat A) |
| R4-2007805 | Huawei, HiSilicon, MediaTek, Ericsson, Nokia, Nokia Shanghai Bell | CR on FR2 measurement requirements outside gaps R15 (CR) DratfCR endorsed in R4-2005267 in RAN4#94-e-bis |
| R4-2007806 | Huawei, HiSilicon, MediaTek, Ericsson, Nokia, Nokia Shanghai Bell | CR on FR2 measurement requirements outside gaps R16 (CR Cat A) |
| R4-2006602 | Nokia, Nokia Shanghai Bell | Correction of CFRA RSRP threshold (CR) CR implementing endorsed draftCR R4-2003395. |
| R4-2006603 | Nokia, Nokia Shanghai Bell | Correction of CFRA RSRP threshold (CR Cat A) |
| R4-2007807 | Huawei, HiSilicon | CR to remove RSTD requirements for NE-DC in 36.133 R15 (CR) DratfCR endorsed in R4-2004334 in RAN4#94-e-bis |
| R4-2007808 | Huawei, HiSilicon | CR to remove RSTD requirements for NE-DC in 36.133 R16 (CR Cat A) |
| R4-2007809 | Huawei, HiSilicon | CR on inter-RAT RSTD requirements for NE-DC in 38.133 R15 (CR) DratfCR endorsed in R4-2005268 in RAN4#94-e-bis |
| R4-2007810 | Huawei, HiSilicon | CR on inter-RAT RSTD requirements for NE-DC in 38.133 R16 (CR Cat A) |
| R4-2006878 | Mediatek Inc., Huawei, Hisilicon, Apple | CR on TS38.133 for modification of the layer 3 and layer 1 measurement sharing factor when both SSB and RSSI symbol to be measured are considered (CR, Cat A) Draft CR endorsed in R4-2005419 in RAN4#94-e-bis |
| R4-2006879 | Mediatek Inc., Huawei, Hisilicon, Apple | CR on TS38.133 for modification of the layer 3 and layer 1 measurement sharing factor when both SSB and RSSI symbol to be measured are considered (CR, Cat A) Draft CR endorsed in R4-2005419 in RAN4#94-e-bis |

Please provide comments in the table below.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2006185 R4-2006186 (Cat A) | Ericsson : Idea is OK, but I don’t understand the 2nd part of the sentence : “… and neighbour cell measurement is required on this Scell.” I think if a measurement object is configured, UE is supposed to measure it? If only one Scell is configured with an FR1 PSCell (using table 9.1.5.1.1-1 as an example) the UE will always be required to measure that Scell with CSSF=1 |
|  | Apple: To answer Ericsson question, because in FR2 UE is only required to measurement neighbor cell on one CC and measurement serving cell 2 SSBs on other CCs, so here since only one FR2 Scell is assumed, we think the most possible case is that this Scell is configured with intra-freq MO in that column, and CSSF is always as 1. If Ericsson thinks it’s better to only capture as “CSSFoutside_gap,i =1 if only one SCell is configured” to be more generic, we are also fine. |
|  | MTK: OK |
|  | NEC: In general OK with change. We prefer Apple’s modified text. |
|  | Ericsson: Thanks to Apple for clarification. Since the column heading already captures that this is the requirement for cells where Ncell measurement is required (“CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required”) that is where the confusion arose and we prefer Apple’s proposal to capture this as “CSSFoutside_gap,i =1 if only one SCell is configured”. We fully agree that this change is needed. |
| R4-2007757 R4-2007758 (Cat A) | [R4-2005412] was endorsed at RAN4#94bis-e |
|  | Company A |
|  | Company B |
| R4-2007805 R4-2007806 (Cat A) | DratfCR endorsed in R4-2005267 in RAN4#94-e-bis |
|  | Company A |
|  | Company B |
| R4-2006602 R4-2006603 (Cat A) | CR implementing endorsed draftCR R4-2003395. |
|  | Company A |
|  | Company B |
| R4-2007807 R4-2007808 (Cat A) | DratfCR endorsed in R4-2004334 in RAN4#94-e-bis |
|  | Company A |
|  | Company B |
| R4-2007809 R4-2007810 (Cat A) | DratfCR endorsed in R4-2005268 in RAN4#94-e-bis |
|  | Company A |
|  | Company B |
| R4-2006878 R4-2006879 (Cat A) | Draft CR endorsed in R4-2005268 in RAN4#94-e-bis |
|  | Company A |
|  | Company B |

Summary for 1st round

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2006185 | Revised. Capture Ericsson’ comment “CSSFoutside_gap,i =1 if only one SCell is configured” |
| R4-2006186 | Return to. Cat A CR to R4-2006185. |
| R4-2007757 | Agreed. |
| R4-2007758 | Agreed. |
| R4-2007805 | Agreed. |
| R4-2007806 | Agreed. |
| R4-2006602 | Agreed. |
| R4-2006603 | Agreed. |
| R4-2007807 | Agreed. |
| R4-2007808 | Agreed. |
| R4-2007809 | Agreed. |
| R4-2007810 | Agreed. |
| R4-2006878 | Agreed. |
| R4-2006879 | Agreed. |

Discussion on 2nd round (if applicable)

In the second round, the revised CR of R4-2006185 will be provided for review. Please Apple trigger the email discussion.

[Comments and responses will be collected by moderator here]

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2006185 |  |
| R4-2006186 |  |

Topic #3: Connected state mobility

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006005 | ZTE Corporation | Discussion on RRC procedure delay in RRC release with redirection Proposal 1: TRRC_procedure_delay = 20 ms. Proposal 2: Specify TRRC_procedure_delay = 20 ms in core requirements and test cases. Observation 1: For redirection to NR there is no test case. Proposal 3: Specify TRRC_procedure_delay = 110 ms in clause 6.3.2.4 in 36.133 since there is no test case for redirection to NR. Agree CR [4]. |
| R4-2006002 | ZTE Corporation | [CR] RRC release with redirection 38.133 R16 (CR) As agreed in WF R4-2002206, references to 38.331 in test cases shall be removed. What’s more, we propose to modify the delay from 110 ms, which is too long in our view, to 20 ms. Remove references to 38.331 as agreed in WF R4-2002206 and modify the value of the delay from 100 ms to 20 ms. Specify the delay in both core requirements and test cases. |
| R4-2006003 | ZTE Corporation | [CR] RRC release with redirection 36.133 R15 (CR) TRRC_procedure_delay is unspecified in redirection to NR. Specify the value of TRRC_procedure_delay = 110 ms. |
| R4-2006004 | ZTE Corporation | [CR] RRC release with redirection 36.133 R16 Cat A |
| R4-2007981 | Ericsson | Correction to RRC release with redirection requirements in 36.133 Rel-15 (CR) The definition of the parameter, TPRACH, in RRC release with redirection to NR requirement is aligned with that used in the core NR specification (clause 6.2.3.2.1: RRC connection release with redirection to NR) in TS 38.133. A typo is also corrected. |
| R4-2007982 | Ericsson | Correction to RRC release with redirection requirements in 36.133 Rel-16 (CR Cat A) |
| R4-2006006 | ZTE Corporation | Discussion on RRC re-establishment requirement Observation 1: The UE is not aware of whether the network contains UE context before sending RRCReestablishmentRequest. Thus, the UE has to fulfill the delay requirement defined in clause 6.2.1.2.1 in TS 38.133 always. Observation 2: Having a line saying “There is no requirement if the target cell does not contain the UE context” in the specification gives impression to readers that under some cases, the UE is certain that the network doesn’t have UE context, which can be misleading. Proposal 1: The UE shall meet the delay requirement always since it can’t be sure whether the network has UE context or not. Proposal 2: Agree on the CRs [3][4] (Cat F and Cat A) to remove the statement “There is no requirement if the target cell does not contain the UE context” in the specification. |
| R4-2006007 | ZTE Corporation | CR on RRC re-establishment requirements R15 (CR) Remove the statement “There is no requirement if the target cell does not contain the UE context”. |
| R4-2006008 | ZTE Corporation | CR on RRC re-establishment requirements R16 (CR Cat A) |

Open issues summary

Sub-topic 3-1

As to the value of TRRC_procedure_delay, 110  ms inherited from LTE specification seems unreasonably long, compared to other RRC procedures. It is proposed to change the value of to be aligned with 38.331 other RRC procedure delay.

Issue 3-1: RRC procedure delay in RRC release with redirection

- Proposals (ZTE, R4-2006005, R4-2006002, R4-2006003, R4-2006004)

  - For 38.133, specify TRRC_procedure_delay = 20 ms in core requirements and test cases.

  - For 36.133, specify TRRC_procedure_delay = 110 ms in clause 6.3.2.4 in 36.133 since there is no test case for redirection to NR.

- Recommended WF

  - TBA

Sub-topic 3-2

The definition of the parameter, TPRACH, in RRC release with redirection to NR requirement is aligned with that used in the core NR specification (clause 6.2.3.2.1: RRC connection release with redirection to NR) in TS 38.133.

Issue 3-2: The definition of the parameter, TPRACH, in RRC release with redirection to NR requirement

- Proposal (Ericsson, R4-2007981, R4-2007982)

  - The change on TRACH is proposed as below:

- Recommended WF

  - TBA

Sub-topic 3-3

In TS 38.133 [1], the delay requirement of RRC re-establishment procedure is defined. It’s stated that there is no requirement if the target cell does not contain the UE context. In the paper R4-2006006, it was proposed that this statement is not necessary since the UE is not aware whether the target cell contains UE context or not before sending the RRCReestablishmentRequest message.

Issue 3-3: UE context for RRC re-establishment requirement

- Proposal (ZTE, R4-2006006, R4-2006007, R4-2006008)

  - Proposal 1: The UE shall meet the delay requirement always since it can’t be sure whether the network has UE context or not.

  - Proposal 2: Remove the statement “There is no requirement if the target cell does not contain the UE context” in the specification.

- Recommended WF

  - TBA

Sub-topic 3-4

Please provide the additional comments on the CRs in Section 3.3.2.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub topic 3-1: ZTE proposals and CR are OK for Ericsson Sub topic 3-2: Sub topic 3-3 : ZTE proposals and CR are OK for Ericsson …. Others: |
| Qualcomm | Sub topic 3-1: For 38.133, 20ms does not seem to be aligned with 38.331. From 38.331, we have: The UE shall: - delay the following actions defined in this sub-clause 60 ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier; So 60ms seems more reasonable. |
| MTK | Sub topic 3-1: We don’t agree to update spec. in current stage. We don’t see the technical analysis from ZTE’s paper on how to deduce this value. In ZTE’s paper, it only compare with some typical RRC procedure time. Actually, this is depends on UE’s processing. Based on chair’s guideline, we suggest don’t discuss this issue any more and keep the spec. unchanged. Sub topic 3-2: We agree to update this definition to align with Handover. Sub topc 3-3: This is the 3rd time discussing this issue. We don’t believe this is a critical issue in R15. Based on chair’s guideline, we suggest don’t discuss this issue any more and keep the spec. unchanged. |
| ZTE | Sub topic 3-1: - There are two proposals under this sub-topic: - For 38.133, specify TRRC_procedure_delay = 20 ms in core requirements and test cases. - For 36.133, specify TRRC_procedure_delay = 110 ms in clause 6.3.2.4 in 36.133 since there is no test case for redirection to NR. For the first bullet, considering feedback from other companies, we’re willing to discuss the value. Now we propose 20 ms, Qualcomm prefers 60 ms and there might still be options from other companies. We’re open to discuss this. 60 ms seems also fine for us. For the second bullet, this is just to update 36.133 since there is no test case for redirection to NR. Can companies check if this is agreeable? Sub topic 3-3 : In our view we think this is critical because it gives people a false impression that the UE is aware of if gNB has UE context. This is our concern. Can @MediaTek agree that removing the statement won’t affect UE implementation? If yes then for the clarity of the spec we prefer to remove it. Again, we’re not trying to tighten or losen any requirements since we believe the UE has to meet the requirements anyways even with this statement in the spec. |
| Huawei | Sub topic 3-1: We share the similar views as MTK. We don’t see the need to improve the RRC processing delay for RRC release redirection requirement at current stage. From our understanding, the issue is resulted from the wrong reference to TS 38.133 in the test cases. And there is also no need to specify the delay value in the core requirements. It shall be specified in the test cases (i.e. TSI-NR) |
| Mediatek | Sub topic 3-1: We don’t suggest to discuss this RRC redirection enhancement in R15 and R16. We’re open to discuss this issue in R17. Sub topic 3-3: To ZTE, we suggest to keep current sentence to align with legacy LTE spec. Otherwise, it may give the developer a misunderstanding that some procedures are different in NR now. |
| NEC | Sub topic 3-2: Agree with the proposal |
| ZTE | Sub topic 3-1: - There are two proposals under this sub-topic (perhaps we should not group them together): - For 38.133, specify TRRC_procedure_delay = 20 ms in core requirements and test cases. - For 36.133, specify TRRC_procedure_delay = 110 ms in clause 6.3.2.4 in 36.133 since there is no test case for redirection to NR. For the first bullet, considering feedback from other companies, we’re willing to discuss the value either in R16 maintenance or R17. We understand that time is very limited to conclude during this meeting so we’re open to either discuss a new value in R17 or update the value through maintenance for R16. For the second bullet, since there is no test case for redirection to NR, the value needs to be specified in core requirements. Sub topic 3-3: Well we still believe that this statement is unnecessary. However if other companies have strong view not to remove this statement due to non-technical reasons we’re willing to compromise. We can use some guidance from the moderator / chairman or collect companies view during second round. |

CRs/TPs comments collection

The following CRs will be discussed in this section.

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006002 | ZTE Corporation | [CR] RRC release with redirection 38.133 R16 (CR) |
| R4-2006003 | ZTE Corporation | [CR] RRC release with redirection 36.133 R15 (CR) |
| R4-2006004 | ZTE Corporation | [CR] RRC release with redirection 36.133 R16 Cat A |
| R4-2007981 | Ericsson | Correction to RRC release with redirection requirements in 36.133 Rel-15 (CR) |
| R4-2007982 | Ericsson | Correction to RRC release with redirection requirements in 36.133 Rel-16 (CR Cat A) |
| R4-2006007 | ZTE Corporation | CR on RRC re-establishment requirements R15 (CR) |
| R4-2006008 | ZTE Corporation | CR on RRC re-establishment requirements R16 (CR Cat A) |

Please provide comments in the table below.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2006002 | MTK: We don’t agree on this CR as mentioned in the open issue discussion |
|  | ZTE: We can discuss the exact value. Now we propose 20 ms and QC proposes 60 ms. We can capture the outcome of this discussion using this CR. Right now 60 ms seems also fine to us. |
|  | Huawei: Wo don’t agree on changing the value as mentioned in our comments to 3-1. It should also be noted that we have also provided a Cat A CR R4-2007716 to the thread [202], in which the reference to TS 38.133 is removed without changing the value. |
| R4-2006003 R4-2006004 (Cat A) | MTK: We don’t agree on this CR. If there is no test case, do not need to specify the value |
|  | ZTE: We think it’s still necessary to specify the value. There’s requirement on the overall delay Tconnection_release_redirect_NR which is expressed as Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH If a part of this delay is not specified then the requirement for the overall delay is also useless. 110 ms is used in 36.133 already and our intention here is not to tighten any requirement. |
|  | Huawei: We can’t agree with the CR. |
|  | ZTE: Can companies clarify on the technical concerns? This CR doesn’t change the value of 110 ms, the intention is to specify that TRRC_procedure_delay = 110 ms since it’s not specified in the test case. If we don’t specify it in core requirements then there is a hole in the spec. |
| R4-2007981 R4-2007982 (Cat A) | MTK: We agree on this CR |
|  | NEC: Agreeable to us |
| R4-2006007 R4-2006008 (Cat A) | MTK: We don’t agree on this CR as mentioned in the open issue discussion |
|  | ZTE: Thanks for the comments. Can @MediaTek agree that removing the statement won’t affect UE implementation? If yes then for the clarity of the spec we prefer to remove it. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #3-1 | 5 companies participated in the discussion. 2 companies prefer to no change. 3 companies proposed the changes, where 1 company proposed the additional option. In last meeting, Mr. Chair’s guidance is “Aim to conclude in RAN4#94ebis”. Tentative agreements: No. Candidate options: - Option 1 (ZTE, Ericsson): - For 38.133, specify TRRC_procedure_delay = 20 ms in core requirements and test cases. - For 36.133, specify TRRC_procedure_delay = 110 ms in clause 6.3.2.4 in 36.133 since there is no test case for redirection to NR. - Option 2 (Qualcomm, ZTE): - For 38.133, specify TRRC_procedure_delay = 60 ms in core requirements and test cases. - Option 3 (Mediatek, Huawei, HiSilicon): No change. Recommendations for 2nd round: Given the guidance of Chair in last meeting, the target is to close the topic as soon as possible. From moderator perspective, the following is suggested as a compromise - Keep Rel-15 and Rel-16 specifications unchanged and make changes in Rel-17 specifications - For 38.133, specify TRRC_procedure_delay = 60 ms in test cases. - For 36.133, specify TRRC_procedure_delay = 110 ms in clause 6.3.2.4 in 36.133 since there is no test case for redirection to NR. - The change will be captured in Rel-17 when Rel-17 specification is available. Companies are encouraged to provide the compromised solution. If no consensus is reached in this meeting, it is suggested to close this topic in Rel-16. There are two sub-bullets. These two sub-bullets are not related to each other. In the 2nd discussion companies are welcome to comment both two sub-bullets below Issue 3-1 separately. - Issue 3-1-1: For 38.133, specify TRRC_procedure_delay = 20 ms in core requirements and test cases. - Issue 3-1-2: For 36.133, specify TRRC_procedure_delay = 110 ms in clause 6.3.2.4 in 36.133 since there is no test case for redirection to NR. |
| Sub-topic #3-2 | 2 companies supported the proposal. Tentative agreements: Ericsson proposal is agreeable. Candidate options: Recommendations for 2nd round: |
| Sub-topic #3-3 | 3 companies participated in discussion. 1 company objected the change. Except for the proponent, 1 company supported the change. In last meeting, Chair’s guidance is to “aim to conclude in RAN4#94ebis”. Tentative agreements: No Candidate options: Either remove the sentence or keep the specification unchanged. Recommendations for 2nd round: No possible compromise was foreseen. Given the guidance from Chair and considering this proposal for Rel-15 was discussed long time, it is suggested to close the topic, if the proponent cannot find a way forward to convince the company who is against the removal in this meeting. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2006002 | Return to. |
| R4-2006003 | Return to. |
| R4-2006004 | Return to. Cat A CR to R4-2006003. |
| R4-2007981 | Agreed. |
| R4-2007982 | Agreed. |
| R4-2006007 | Return to. |
| R4-2006008 | Return to. Cat A CR to R4-2006007. |

Discussion on 2nd round (if applicable)

In the second round, two email threads are suggested.

- Email thread for CR R4-2006002 and R4-2006003, which will be triggered and led by ZTE.

- Email thread for CR R4-2006007 and R4-2006008, which will be triggered and led by ZTE.

If no consensus were reached, it is suggested to close those two topics and no further discussion in future meetings.

Please notice that

There are two sub-bullets. These two sub-bullets are not related to each other. In the 2nd discussion companies are welcome to comment both two sub-bullets below Issue 3-1 separately.

  - Issue 3-1-1: For 38.133, specify TRRC_procedure_delay = 20 ms in core requirements and test cases.

  - Issue 3-1-2: For 36.133, specify TRRC_procedure_delay = 110 ms in clause 6.3.2.4 in 36.133 since there is no test case for redirection to NR.

[Comments and responses are collected by moderator here]

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2006002 |  |
| R4-2006003 |  |
| R4-2006004 |  |
| R4-2006007 |  |
| R4-2006008 |  |

Topic #4: Timing

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2007712 | Huawei, Hisilicon | CR on UE transmit timing (CR) Resubmission of Endorsed Draft CR R4-2004284 The one shot timing adjustment shall be removed. |
| R4-2007711 | Huawei, Hisilicon | CR on UE transmit timing (CR Cat A) |

Companies views’ collection for 1st round

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007712 R4-2007711 (Cat A) | Resubmission of Endorsed Draft CR R4-2004284 |
|  | Company A |
|  | Company B |

Summary for 1st round

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2007712 | Agreed. |
| R4-2007711 | Agreed. |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #5: Signaling characteristics

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2007811 | Huawei, HiSilicon | Discussion on SCell activation requirements Proposal: For FR2 SCell activation where there is active serving cell in the band, the requirements apply provided that the SSB pattern is same for the active serving cell and the SCell being activated. |
| R4-2007659 | Huawei, Hisilicon | CR on LTE SCell activation and deactivation delay (CR) - The starting/ending point of interuption window on the NR serving cell caused by LTE SCell activation/deactivation is clarified. - Missing section number is added. |
| R4-2007660 | Huawei, Hisilicon | CR on LTE SCell activation and deactivation delay_r16 (CR Cat A) |
| R4-2007661 | Huawei, Hisilicon | CR on SCell activation and deactivation delay (CR) - Brackets on requirements are removed. - Missing agreements is captured. - Agreements reached in R4-2005426 is generalized to cover interruption window on LTE/NR serving cell in the same/different CG with Scell being activated. |
| R4-2007662 | Huawei, Hisilicon | CR on SCell activation and deactivation delay_r16 (CR Cat A) |
| R4-2007783 | Ericsson | CR 38.133 (8.3.2) Corrections to SCell Activation delay requirements At earlier meetings, corrections to SCell activation timelines by introducing usage of TFirstSSB or TFirstSSB_MAX have been made. However, corresponding corrections are missing for the definition of interruption windows. Introducing the following corrections: - Modifying activation timeline for activation of first known SCell in FR2 when SP CSI-RS is used, by merging the two cases into a single one and where the value of Tuncertainty_MAC is specified to 0 for one of the cases - Modifying the expression for the interruption window to now define the starting point of the interruption, which is related to whichever of TFirstSSB, TFirstSSB_MAX, and TFineTiming that applies for the activation delay requirement. Referring to clause 8.2 for the length of the interruption, which depends on whether the impacted cell is intra- or inter-band to the SCell being activated. |
| R4-2007784 | Ericsson | CR 38.133 (8.3.2) Corrections to SCell Activation delay requirements |
| R4-2007812 | Huawei, HiSilicon | CR on SCell activation requirements R15 (CR) DratfCR endorsed in R4-2005834 in RAN4#94-e-bis Summary of changes: - Add condition for FR2 SCell activation where there is active serving cell in the band that the SSB pattern is same for the active serving cell and the SCell being activated. - Update the definition of TFirstSSB and TFirstSSB_MAX such that it includes the complete SSB burst. - Add a statement that UE is not expected to meet the requirements for RRM measurement and in case of FR2 L1 measurement requirements during cell detection time for unknown SCell activation. |
| R4-2007813 | Huawei, HiSilicon | CR on SCell activation requirements R16 (CR Cat A) |
| R4-2007280 | Qualcomm Incorporated | CR to T parameters in 8.3.2 of 38.133 Current version 38.133 spec ambiguosly defines TFirstSSB and TFirstSSB_MAX in terms of whether or not SSB duration is accommodated and in terms of which SSB within SSB burst is indicated. The parameters should be similarly defined as TFineTiming such that SSB duration is included and the SSB corresponds to the activated TCI state. Clarified the definitions of TFirstSSB, TFirstSSB_MAX, and TFineTiming to explicitly include SSB duration and TCI state. |
| R4-2006847 | MediaTek inc. | CR for SCell activation delay in FR2 Resubmission of endorsed Draft CR R4-2005270 in RAN4#94-e-Bis. It is proposed to capture the agreement in RAN4#93: Agreement Fine time tracking will not depend on the SP CSI-RS activation used for CSI reporting Currently, when semi-persistant CSI-RS is configured for CSI report, Tuncertainty_MAC will include the waiting time of MAC-CE for SP CSI-RS. However, performing fine timing tracking (TFineTiming ) is not necessarily to wait for the SP CSI-RS. Remove the waiting time of MAC-CE for SP CSI-RS from Tuncertainty_MAC, and make fine timing tracking not depeding on the SP CSI-RS. |
| R4-2006848 | MediaTek inc. | CR for SCell activation delay in FR2 (CR, Cat A) |
| R4-2007706 | Huawei, Hisilicon | CR on CSI-RS based RLM requirement (CR) The minimum BW of CSI-RS is 24 PRBs, so there is no need to state the condition that the CSI-RS resource is configured over 24 PRBs. Remove the condition “over the bandwidth ≥ 24 PRBs” |
| R4-2007705 | Huawei, Hisilicon | CR on CSI-RS based RLM requirement (CR Cat A) |
| R4-2007963 | Ericsson | Clarification on RLM (CR) RAN4 received LS (R1-2002992) from RAN1 requesting to remove the term “candidate” in RLM requirements in 38.133, since in unlicensed operation the term “candidate” has a specific meaning which is in conflict with the current wording in TS 38.133. The term “candidate” is removed. |
| R4-2007964 | Ericsson | Clarification on RLM (CR Cat A) |
| R4-2007663 | Huawei, Hisilicon | CR on Psharingfactor (CR) Following changes are made: - For SSB based RLM/BFD/CBD/L1-RSRP, the first condition of Psharingfactor=1 is removed; - For SSB based RLM/BFD/CBD/L1-RSRP, the second condition of Psharingfactor=1 is changed. RSSI is considered. - Same rules apply to CSI-RS based RLM/BFD/CBD/L1-RSRP. - Editorial corrections. |
| R4-2007664 | Huawei, Hisilicon | CR on Psharingfactor_r16 (CR Cat A) |
| R4-2006174 | Qualcomm Incorporated | Corrections to R15 MAC-CE based TCI state switching requirements Observation 1. Guaranteeing PDCCH performance on the new TCI state is independent of receive on the old TCI state. Observation 2. In case target TCI state is not in the active TCI state list for PDSCH (TOk = 1), there will only be at most ~2ms (TSSB-proc) when PDCCH performance cannot be guaranteed if NW schedules MAC-CE based TCI state switch exactly 3ms before an SSB. Observation 3. Many R15 gNB and R15 UE products already deployed with commercial quality are compliant with RAN1 specification rather than RAN4. They cannot be recalled or modified with OTA changes. Hence, they will remain non-compliant with RAN4 if the specification is not modified. Proposal 1. Modify MAC-CE based TCI state switch delay requirements as: “The UE shall be able to receive on the old TCI state until slot n+ THARQ +3 ms +TOk*(Tfirst-SSB).” |
| R4-2006177 | Qualcomm Incorporated, MediaTek | CR for correction to MAC-CE based TCI State switch timeline (Clause 8.10.3) (CR) MAC-CE based TCI state switch timelines in RAN4 are in conflict with RAN1 agreements. This CR is based on endorsed draftCR from RAN4#94-e-Bis (R4-2005430). Additional corrections in RAN4#95-e are marked separately. UE to switch to new beam 3ms after HARQ. Demodulation performance guranteed after reception of SSB |
| R4-2006178 | Qualcomm Incorporated, MediaTek | CR for correction to MAC-CE based TCI State switch timeline (Clause 8.10.3) (CR Cat A) |
| R4-2006209 | Apple | CR on Active TCI State Switching requirements - Rel15 (CR) - Changes from R4-2005430. [Endorsed in RAN4#94e-Bis] - For MAC CE based TCI state switch for both known and unknown case : The UE shall be able to receive PDCCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ . The UE is not required to receive with old or target TCI state from slot n+ THARQ + 3Nslotsubframe,µ+1 until the end of the switching delay. - For RRC based switch, PDCCH/PDSCH/CSI-RS or transmit PUCCH/PUSCH from slot n + THARQ + 1 until the end of switching period. |
| R4-2006210 | Apple | CR on Active TCI State Switching requirements - Rel16 (CR Cat A) |
| R4-2006465 | MediaTek inc. | CR on TCI state switch (CR) The following columns were agreed in RAN4 #94-e meeting but not implemented in the TCI state switch requirements - The basic UE timing requirements needs to be aligned with the RAN1 requirements. - If the target TCI state is unknown, the UE shall execute L1-RSRP measurement with CSI-RS which shall be set with Repition ON - There is a mismatch between RAN1 spec and RAN4 spec on this TCI change delay. The CR capturing the following new update TCI state switch requirements - PUSCH QCL relation is based on spatial relation and will be defined in active spatial relation switching delay. - Scheduling restriction shall be also applied to CSI-RS for CQI. - Scheduling restriction for transmission shall be defined in active spatial relation switching based on the configuration of spatial relation. |
| R4-2006466 | MediaTek inc. | CR on TCI state switch (CR) |
| R4-2007780 | Ericsson | CR 38.133 (8.10.5) Corrections to RRC-based TCI state change (CR) Introducing the following correction: - Changing “slot n+ TRRC_processing +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length” to “slot n+ (TRRC_processing +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length” - Changing “slot n+ TRRC_processing +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc) / NR slot length” to “slot n+ (TRRC_processing +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc)) / NR slot length” - Correcting some punctuation. |
| R4-2007781 | Ericsson | CR 38.133 (8.10.5) Corrections to RRC-based TCI state change (CR Cat A) |
| R4-2006891 | ZTE Corporation | [CR] TCI state switch delay 38.133 R15 In TCI state switching delay requirement, TOuk depends on whether the UE has already conducted SSB based measurement for Rx beam refinement. In this case, when the TCI state switching involves QCL-TypeD and other types, TOuk should equal to 0. To make this clear, we propose to clarify that “TOuk = 1 when TCI state switching involves other QCL types only”. |
| R4-2006892 | ZTE Corporation | [CR] TCI state switch delay 38.133 R16 Cat A |
| R4-2007751 | Huawei, Hisilicon | Correction on TCI state switching R15 (CR) [R4-2004287] was endorsed at RAN4# 94bis-e. Since it is difficult for UE to implement the existing one-shot timing adjustment requirements, the one-shot timing adjustment requirement is removed in RAN4#94e. The content related with one-shot timing adjustment is removed accordingly. |
| R4-2007752 | Huawei, Hisilicon | Correction on TCI state switching R16 (CR Cat A) |
| R4-2006189 | Apple | On issues of R15 BWP switching delay requirement Propose 1: Revise the UE behaviour in current RRC based BWP switching requirement as: The UE is not required to transmit UL signals or receive DL signals until the first DL or UL slot occurs right after a time duration of (TRRCprocessingDelay+TBWPswitchDelayRRC -THARQ) which is right after UE transmitting HARQ feedback for associated RRC reconfiguration signalling involving active BWP switching or parameter change of its active BWP. where, THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]. Proposal 2: RAN4 to define the duration within which UE is not required to transmit or receive signals for UE specific CBW change in R16. |
| R4-2006190 | Apple | CR on BWP switching delay requirement for R15 (CR) The current BWP switching requirements have some issues, as discussed in R4-2006189 Revise the RRC based BWP switching to avoid potential issues on UE behavior. |
| R4-2006191 | Apple | CR on BWP switching delay requirement for R16 (CR Cat A) |
| R4-2007707 | Huawei, Hisilicon | CR on interruption due to Acitve BWP switch (CR Cat A) Resubmission of endorsed Draft CR R4-2004277 The interruption length shall be defined based on the SCS of victim CC as other interruption requirement (i.e. SCell activation). Therefore, the note in Table 8.2.1.2.7-1 and Table 8.2.2.2.5-1 shall be removed, because the note refers to the SCS of aggressor CC. Remove the note in table Table 8.2.1.2.7-1 and Table 8.2.2.2.5-1. |
| R4-2007708 | Huawei, Hisilicon | CR on interruption due to Acitve BWP switch (CR) |

Open issues summary

Sub-topic 5-1

Issue 5-1: The assumptions on SSB pattern for FR2 intra-band CA

- Proposals (Huawei, HiSilicon, R4-2007811)

  - For FR2 SCell activation where there is active serving cell in the band, the requirements apply provided that the SSB pattern is same for the active serving cell and the SCell being activated.

- Recommended WF

  - TBA

Sub-topic 5-2

In RAN4#94-e meeting, it (R4-2002052) is proposed to correct the R15 MAC-CE based active TCI state switching requirements in order to align it with the specifications in TS 38.214.

Issue 5-2-1: Remove the extra term TOk*(Tfirst-SSB) for receiving on the old state

- Option 1 (Qualcomm R4-2006174, R4-2006177)

- Option 2 (Apple R4-2006209, R4-2006210)

- Recommended WF

  - TBA

Issue 5-2-2:UE Tx and RX restriction during RRC based TCI state switch

- Option 1 (Apple R4-2006209, R4-2006210):

  - the proposed changes are as follows

Sub-topic 5-3

In R15 BWP switching requirement, the clarification on the duration where UE is not required to perform Rx and Tx has some potential issues and uncertainties. Extending to R16 UE specific CBW switching, we also observed the similar risk.

Issue 5-3: UE behaviour for transmission or reception for BWP switching requirements

- Proposals (Apple R4-2006189, R4-2006190, R4-2006191)

  - Propose 1: Revise the UE behaviour in current RRC based BWP switching requirement as:

The UE is not required to transmit UL signals or receive DL signals until the first DL or UL slot occurs right after a time duration of (TRRCprocessingDelay+TBWPswitchDelayRRC -THARQ) which is right after UE transmitting HARQ feedback for associated RRC reconfiguration signalling involving active BWP switching or parameter change of its active BWP.

where,

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

  - Proposal 2: RAN4 to define the duration within which UE is not required to transmit or receive signals for UE specific CBW change in R16.

- Recommended WF

  - TBA

Sub-topic 5-4

Please provide the additional comments on the CRs in Section 5.3.2.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub topic 5-1: We do not agree with the need for this restriction. When scheduled on multiple carriers simultaneously, those have to be transmitted to the UE using the same spatial transmission filter. This does however not mean that SSBs on different CCs need to be transmitted simultaneously using a common beam. Different SSB indexes for the same spatial transmission filter can still be used on different CCs. Sub topic 53: UE would not know it is a BWP change before processing the RRC message. The RRC processing will be finished after HARQ ACK has been sent. Hence this change seems unnecessary. …. Others: |
| Apple | Sub topic 5-1: Propose to slightly revise the proposal as: For FR2 SCell activation where there is active serving cell(s) in the band, the requirements apply provided that the SSB pattern is same for the active serving cell(s) and the SCell being activated Since the active serving cell might be more than 1, and we agree that the serving CCs share the same ssb-PositionsInBurst in FR2 intra-band. Sub topic 5-2: Issue 5-2-1: Remove the extra term TOk*(Tfirst-SSB) for receiving on the old state Prefer option 2. In order to align RAN1 and RAN4 spec on until when the UE is supposed to receive with the old TCI state, the changes are proposed. We also provide clarification that - The UE is not required to receive with old or target TCI state from slot n+ THARQ + 3Nslotsubframe,µ+1 until the end of the switching delay. Issue 5-2-2:UE Tx and RX restriction during RRC based TCI state switch Currently during RRC based TCI state switch UE is not required to transmit or receive during entire switching delay. The switching delay starts from the slot PDSCH is received. The UE needs to transmit ACK/NACK at a minimum for the received PDSCH. The restriction should start after THARQ rather than the slot when PDSCH is received. --Update 05/26 6AM UTC-- To Qualcomm : We prefer to add the additional clarification, otherwise UE behavior is ambiguous during the time. For scenarios where TOk = 0, the additional time in the delay is not there anyway, the delay is same as RAN1 requirement. Sub-topic 5-3: We proposed it since we believe the HARQ procedure shall still be there for RRC based BWP switching and UE specific CBW change case. To Ericsson: the HARQ feedback is nothing to do with the RRC message content, it’s a legacy ACK/NACK to the PDSCH carrying this RRC signaling. In the current requirement, UE is not required to transmit anything after it received PDSCH of RRC signaling, so that means UE may not feedback HARQ to network for this PDSCH and it’s not a correct behavior. [Ericsson response 2020-05-27 11AM UTC]: Right, but the UE would not know that the received PDSCH transport block carries a RRC reconfiguration message for BWP switching until having parsed the RRC message itself. So UE would anyways transmit until the RRC message has been parsed and the UE finds that it carries a BWP switch |
| Qualcomm | Subtopic 5-2: we support option 1. We realize that option 2 is also saying the same but the last sentence seems redundant for scenarios when TO_k = 0 anyways. We think option 1 has sufficient wording. |
| MTK | Sub topic 5-1: We agree on this proposal. As mentioned in Huawei’s paper, if the SSB pattern is different, UE still cannot leverage the info from known FR2 cell. Sub topic 5-2-1: As mentioned in QC’s paper, the reason on updating RAN4 spec is the gNB have been deployed in the field that are compliant with RAN1. If RAN4 spec keeps unchanged, it means UE shall maintain two ways to both supporting the real field and RAN4 test case. Sub topic 5-2-2: We’re fine with this proposal. Sub topic 5-3: We’re fine with proposal 1. Proposal 2 is also fine to us, but it is better to discuss in Rel-16 RRM enhancement WI. |
| Huawei | Sub topic 5-1: We think the condition is needed for UE to meet the following requirements. If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirstSSB+ 5ms provided: - The UE is provided with SMTC for the target SCell, and - The SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3.. To Ericsson comments: For SCell activation, UE only performs fine time tracking based on one SSB of the to-be-activated SCell, and if the two cells are using different SSB patterns, UE would not know which one to use. There is no time in the above requirement for TCI indication. Moreover, we agree that different SSB indexes for the same spatial transmission filter can be used on different CCs, e.g. SSB1 on cell 1 can be transmitted with same Tx beam as SSB2 on cell2, but this information is not known to the UE because SSB cannot be configured with a TCI state. So even a TCI state to SSB2 is provided for cell2, UE still does not know which Rx beam it should use for receiving. [Ericsson response 2020-05-27 11AM UTC]: Separate time is not included for TCI state configuration, but if TCI state configuration is provided in the same MAC PDU as the activation command, this may not be needed. The UE knows how to map a TCI state to a SSB index. Hence it could be a split requirement depending on whether TCI state is provided or not. Response to Ericsson 2020-05-27 15PM UTC: We understand even the TCI state is provided, e.g. an SSB on the to-be-activated SCell with index #m, is pointed by this TCI state, UE may not know which Rx beam it should use to receive SSB#m because UE does not know in which Tx beam the SSB is transmitted. Even SSB#m on the to-be-activated SCell and SSB#n on the active serving cell are transmitted in the same Tx beam, this is not known to the UE, unless they are on the same OFDM symbol, and that’s why we propose to add the condition of same SSB pattern for the requirements. To Apple, we think the proposed change is reasonable. Sub topic 5-2-1: option 1 and option 2 is similar. Either one is ok. Sub topic 5-2-2: We disagree with option 1. This change is also proposed in BWP switching (issue 5-3), we have the same view that no need to change the current requirements. Sub topic 5-3: We do not agree with the changes. From our understanding, the indicated value of k1 could be up to 16 slot. In this case, does it mean UE shall be able to receive and transmit for the entire BWP switching delay (16ms)? Also, THARQ is not considered in the other RRC reconfiguration cases, where there is ambiguity between network and UE when the new configuration becomes effective. Apple response: the longest HARQ time is a candidate range of dl-DataToUL-ACK in RRC signaling, but in this particular case we don’t believe network will indicate UE to use such long HARQ feedback delay for this BWP switching. On physical layer HARQ is the most fundamental mechanism for network PDSCH transmission regardless of the RRC/MAC message purpose, we think delay-wise HARQ delay can be merged into the specified delay equation, but UE behavior-wise we need to make sure this HARQ feedback will not be interrupted or dropped, this is like that we guaranteed HARQ not interrupted in SCell activation. |
| NEC | Sub topic 5-1: We agree that there is an issue when ssb-PositionInBurst are not overlapping. Additional condition may be required. Sub topic 5-2-1: We prefer option 1. Sub topic 5-2-2 and 5-3-3: If the main motivation for this change is that UE may not transmit HARQ after receiving RRC message, we can clarify in the current spec that UE is allowed to transmit HARQ during total switching delay time. As Ericsson pointed, UE may not know the contents of RRC message and purpose of it till it completes RRC processing. So UE shall be still able to transmit HARQ with current behaviour. However, we can clarify if companies find that it is not clear. |
| Nokia | Issue 5-1: This needs more discussion. Question for clarification: Isn’t the assumption is that the UE use same Rx beam as used with the other serving cell in same the band? Issue 5-2-1: In general we see the reasoning behind the correction. However, it will leave a potential gap in the scheduling. In general we will see a gap of up to an SSB period and on average half an SSB period. Before agreeing to the change we would like to have some clarification related to the condition: TOk = 1 if target TCI state is not in the active TCI state list for PDSCH, 0 otherwise. Use of active TCI state is not always necessary e.g. if refined beams are not in use. If active TCI state is not used this condition will lead to scheduling the scheduling gaps mentioned and scheduling inefficiency. Timing beam management among all devices in the field to specific time occasions (just prior to the SSB) complicates BM on network side significantly. Our question is whether the UE would, when SSB based beams are used, have accurate enough time/frequency tracking on the target SSB to ensure DL scheduling prior to receiving SSB on the target beam? Issue 5-2-2: Issue 5-3: The proposal is fine. The wording in CR need to rework to make it clear. |

CRs/TPs comments collection

The following CRs will be discussed in this section.

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2007659 | Huawei, Hisilicon | CR on LTE SCell activation and deactivation delay (CR) |
| R4-2007660 | Huawei, Hisilicon | CR on LTE SCell activation and deactivation delay_r16 (CR Cat A) |
| R4-2007661 | Huawei, Hisilicon | CR on SCell activation and deactivation delay (CR) |
| R4-2007662 | Huawei, Hisilicon | CR on SCell activation and deactivation delay_r16 (CR Cat A) |
| R4-2007783 | Ericsson | CR 38.133 (8.3.2) Corrections to SCell Activation delay requirements |
| R4-2007784 | Ericsson | CR 38.133 (8.3.2) Corrections to SCell Activation delay requirements |
| R4-2007812 | Huawei, HiSilicon | CR on SCell activation requirements R15 (CR) DratfCR endorsed in R4-2005834 in RAN4#94-e-bis |
| R4-2007813 | Huawei, HiSilicon | CR on SCell activation requirements R16 (CR Cat A) |
| R4-2007280 | Qualcomm Incorporated | CR to T parameters in 8.3.2 of 38.133 |
| R4-2006847 | MediaTek inc. | CR for SCell activation delay in FR2 Resubmission of endorsed Draft CR R4-2005270 in RAN4#94-e-Bis. It is proposed to capture the agreement in RAN4#93: |
| R4-2006848 | MediaTek inc. | CR for SCell activation delay in FR2 (CR, Cat A) |
| R4-2007706 | Huawei, Hisilicon | CR on CSI-RS based RLM requirement (CR) |
| R4-2007705 | Huawei, Hisilicon | CR on CSI-RS based RLM requirement (CR Cat A) |
| R4-2007963 | Ericsson | Clarification on RLM (CR) |
| R4-2007964 | Ericsson | Clarification on RLM (CR Cat A) |
| R4-2007663 | Huawei, Hisilicon | CR on Psharingfactor (CR) |
| R4-2007664 | Huawei, Hisilicon | CR on Psharingfactor_r16 (CR Cat A) |
| R4-2006177 | Qualcomm Incorporated, MediaTek | CR for correction to MAC-CE based TCI State switch timeline (Clause 8.10.3) (CR) |
| R4-2006178 | Qualcomm Incorporated, MediaTek | CR for correction to MAC-CE based TCI State switch timeline (Clause 8.10.3) (CR Cat A) |
| R4-2006209 | Apple | CR on Active TCI State Switching requirements - Rel15 (CR) |
| R4-2006210 | Apple | CR on Active TCI State Switching requirements - Rel16 (CR Cat A) |
| R4-2006465 | MediaTek inc. | CR on TCI state switch (CR) |
| R4-2006466 | MediaTek inc. | CR on TCI state switch (CR) |
| R4-2007780 | Ericsson | CR 38.133 (8.10.5) Corrections to RRC-based TCI state change (CR) |
| R4-2007781 | Ericsson | CR 38.133 (8.10.5) Corrections to RRC-based TCI state change (CR Cat A) |
| R4-2006891 | ZTE Corporation | [CR] TCI state switch delay 38.133 R15 |
| R4-2006892 | ZTE Corporation | [CR] TCI state switch delay 38.133 R16 Cat A |
| R4-2007751 | Huawei, Hisilicon | Correction on TCI state switching R15 (CR) [R4-2004287] was endorsed at RAN4# 94bis-e. |
| R4-2007752 | Huawei, Hisilicon | Correction on TCI state switching R16 (CR Cat A) |
| R4-2006190 | Apple | CR on BWP switching delay requirement for R15 (CR) |
| R4-2006191 | Apple | CR on BWP switching delay requirement for R16 (CR Cat A) |
| R4-2007707 | Huawei, Hisilicon | CR on interruption due to Acitve BWP switch (CR Cat A) Resubmission of endorsed Draft CR R4-2004277 |
| R4-2007708 | Huawei, Hisilicon | CR on interruption due to Acitve BWP switch (CR) |

Please provide comments in the table below.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007659 R4-2007660 (Cat A) | Ericsson WI should not be “NR_newRAT-Perf since this is core. |
|  | Apple: Not sure if this revision is necessary. We think the interruption window range could be defined based on the aggressor cell, but the interruption length already considered the async case for EN-DC on victim cell. Apple2: To Huawei, From LTE, the interruption range is defined only on the time-line of aggressor cell rather than victim CCs. And we don’t see the strong necessity to have such new definition for NR. |
|  | Huawei: We agree that the interruption length has already considered the async case, but in our understanding, the interruption is on the victim cell, so the interruption window should be defined based on the timing of the victim cell. |
|  | Nokia: These exceptions should be moved to the corresponding section which captures the rules (clause 7.32.2.5 and NR PSCell and NR SCell interruptions specified in clause 8.2.1.4.2 of TS 38.133) |
| R4-2007661 R4-2007662 (Cat A) | Ericsson: We have also provided a CR (R4-2007783) based on the already endorsed CR and the agreements in the late GTW session and we will further update it as needed based on comments received in this meeting. |
|  | Apple: Similar comment as to 7659. We think the current interruption window range is based on the aggressor cell timeline, while the interruption length requirement already considered the impact to the victim cell in other CG or RAT. |
|  | Qualcomm: m1 is the index of the first slot of interrupted serving cell or the last subframe of interrupted E-UTRA serving cell in the different cell group with SCell being activated which overlaps with slot n. This should be first; not last. ---- new comments on 5/26 ----- We have following comments: - “3 ms + max(Tuncertainty_MAC +TFineTiming + 25ms, Tuncertainty_SP), where Tuncertainty_MAC=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time.” When Tuncertainty_SP is larger than the other factor (Tuncertainty_MAC +TFineTiming + 25ms), there seems to be a certain amount of duplicated time period because TCSI_reporting also includes uncertainty in acquiring the first available downlink CSI reference resource - TFirstSSB and TFirstSSB_MAX Why should it be SSB burst irrespective of side condition on to-be-activated SCell? It seems that the update with SSB burst relaxes the current requirements to some extent. Though it was endorsed in the previous meeting, could you explain why this should always include all SSBs in the SSB burst rather than a particular SSB? - For THARQNR slot length in interruption window In case aggressor and victim cells have different numerologies, which numerology should be assumed for “NR slot length”? - For E-UTRA interruption requirement, shouldn’t that appear in TS 36.133, not in TS38.133? |
|  | MTK: One suggested further correction: m1 is the index of the first slot of interrupted serving cell or the lastfirst subframe of interrupted E-UTRA serving cell in the different cell group with SCell being activated which overlaps with slot n. In high-level, we prefer to use some general description other than the detail equation to specify this async cases to make the spec more readable, e.g., the timing reference is still based on NR, and interruption is allowed for LTE subframes fully or partially overlapped with those NR slots which may have interruption. |
|  | Huawei: To Ericsson, yes, we can work on 7783 to capture the outcomes of the discussions on interruption window for 38.133. To Apple, similar as for 7659, we think the interruption is on the victim cell, so the interruption window should be defined based on the timing of the victim cell. To Qualcomm and MTK, yes, this is a typo and should be corrected. To MTK, we are also fine to use some general description as you mentioned, and we can provide some wording suggestion to Ericson when revising 7783. ---- new comments in response to Qualcomm ----- - This change was brought by MTK and endorsed in last meeting in R4-2005270. We can explain with our understanding here, but MTK can provide more comments if needed. Tuncertainty_SP is the time between the SCell activation command and the SP-CSI-RS activation command, while TCSI_reporting is the time between the end of Tactivation_time and the first available SP-CSI-RS resource, so they are different. Maybe the confusion comes from the current description of Tuncertainty_SP, and I guess MTK can update their CR R4-2006847. We also added comments to R4-2006847. - We have provided our views for 7280. Basically, the problem is that we cannot define the particular SSB in the contexts where TFirstSSB and TFirstSSB_MAX are used. When TFirstSSB and TFirstSSB_MAX are used for FR1, there is no TCI indication assumed in the activation process, so we cannot include "activated TCI state" in the definition. When TFirstSSB and TFirstSSB_MAX are used for FR2, they are needed only for the unknown case, which means there is no TCI indication to the UE before the first SSB, so again we cannot include "activated TCI state" in the definition. - We understand it should be the numerology of the aggressor. Then it causes the problem on the description of the end point, because Ninterruption is defined based on victim cell numerology. We will provide wording suggestion in the second round. - So far the interruption window is defined together with activation delay, and that’s why we put it in 38.133 (NR SCell activation delay is only defined in 38.133), but we have no strong view. We would like to hear views from Ericsson, because it needs to be coordinated between 7659 and 7783. |
|  | Nokia: requirements related to the interruptions on PCell, and MCG SCell in EN-DC are captured in 36.133. The new changes also relate to E-UTRAN interruptions – should they be captured in 36.133 instead? |
| R4-2007783 R4-2007784 (Cat A) | Huawei: We have submitted CR 7661 on interruption window, where the interruption window for serving cells in a different CG (in case of EN-DC, NE-DC and NR-DC) are further clarified. We can work on 7783 if the new changes in 7661 can be captured. |
|  | Company B |
|  | Qualcomm: For THARQNR slot length in interruption window, in case aggressor and victim cells have different numerologies, which numerology should be assumed for “NR slot length”? [Ericsson response 2020-05-27 11AM UTC]: We can clarify that the timeline for interruptions is set by the aggressor, i.e., NR slot length would be with respect to the aggressor. |
| R4-2007812 R4-2007813 (Cat A) | DratfCR endorsed in R4-2005834 in RAN4#94-e-bis |
|  | Huawei: we suggest return to this CR in the first round. If we can reach agreement on sub-topic 5-1, we can revise this CR to capture it, otherwise we can agree the CR as it is in the second round. |
|  | Company B |
| R4-2007280 | MTK: TFirstSSB_MAX is related to both FR1 and FR2 for AGC retuning. Then there is a chicken-and-egg problem. In this stage, in FR2, the network has no knowledge which TCI state to be indicated. The TCI state info. shall be indicated later after L1-RSRP reporting. But here, the CR assumes that TCI-state is already indicated. For FR1, why do we need to configure TCI state info.? If no TCI state is configured, how to handle this for UE in FR1? For this TCI issue, we suggest to use Huawei’s version in R4-2007661. |
|  | Huawei: We have draftCR endorsed in R4-2005834 in RAN4#94-e-bis to address the same issue. The changes are similar, but one difference is that in our CR the duration of the complete SSB burst is included in the definition. When TFirstSSB and TFirstSSB_MAX are used for FR1, there is no TCI indication assumed in the activation process, so we cannot include "activated TCI state" in the definition. When TFirstSSB and TFirstSSB_MAX are used for FR2, they are needed only for the unknown case, which means there is no TCI indication to the UE before the first SSB, so again we cannot include "activated TCI state" in the definition. Basically, we have same comment as MTK above. |
|  | Qualcomm: It’s okay to have further discussion under R4-2007661. We added comments to R4-2007661. |
| R4-2006847 R4-2006848 (Cat A) | endorsed Draft CR R4-2005270 in RAN4#94-e-Bis. |
|  | Huawei: the description of Tuncertainty_SP is incorrect, it should be the time period between reception of the activation command for semi-persistent CSI-RS for CQI reporting relative to … |
|  | Company B |
| R4-2007706 R4-2007705 (Cat A) | Ericsson : |
|  | MTK: No. The BWP BW can still be less than 24 PRBs, and UE is only expected to perform RLM within active BWP. Therefore the condition of BW is still needed. |
|  | Huawei: we will further check MTK comments, and CR can be postponed. |
| R4-2007963 R4-2007664 (Cat A) | Qualcomm: Not needed in R15. Confusion for the word candidate is only applicable to R16. |
|  | MTK: Ok. This is to avoid the confusion between R15 terminology and R16 terminology. Note that a CR R4-2007263 from Nokia was submitted to NR-U agenda (6.1.5.10). If this CR is agreed here. Then R4-2007263 in thread [205] can be ‘not pursued’ |
|  | Huawei: We share the similar view as MTK. The terminology shall be aligned between R15 and R16 though the issues is raised in R16 NR-U. We have also provided 2 CR (R15 and R16) R4-2007698 and R4-2007699 in thread [205]. |
|  | Nokia: OK, we have the same CR R4-2007263 in NR-U AI. |
| R4-2006177 R4-2006178 (Cat A) | Apple: This CR addresses similar issue as in R4-2006209. In the change, the term “/ NR slot length” is no longer needed as the units are already in slots. We propose to merge with R4-2006209 as it covers the additional clarification on UE behavior from slot n+ THARQ + 3Nslotsubframe,µ+1 until the end of the switching delay. |
|  | Qualcomm: we are ok to have one CR but please see our comments regarding Apple’s CR. |
| R4-2006209 R4-2006210 (Cat A) | Ericsson: ”Consequence if not approved” section is incorrect (copy-paste from a different CR) |
|  | Huawei: Pending on issue 5-2-2. We have concern on the changes on the RRC based switching. No need to change the current requirements. |
| R4-2006465 R4-2006466 (Cat A) | Company Aendorsed Draft CR R4-2005430 in RAN4#94-e-Bis |
|  | Company Nokia: the additional change in 8.10.5 which was not in the endorsed R4-2005430 need to be discussed. |
|  | Company B |
| R4-2007780 R4-2007781 (Cat A) | MTK: ok |
|  | NEC: OK |
|  | Ericsson: Resubmission of draft CR R4-2004417 which was endorsed at RAN4#94-e-Bis |
| R4-2006891 R4-2006892 (Cat A) | MTK: We don’t agree on this change. For QCL-TypeD, the reason on TOuk=0 is it can leverage the SSB to track the timing when it’s a SSB based L1-RSRP meas, but TOuk=1 when it’s a CSI-RS based L1-RSRP measurement |
|  | ZTE: To MediaTek: I totally agree with your point. The change is intended to address to a different issue. Suppose the TCI switch involves Type D and Type A, then according to the definition now it’s unclear what value TOuk is since the switch falls into both categories. By adding “only”, any switch can only fall into one category, not two and thus, it’s clear for the reader to determine the value of TOuk. |
|  | MTK: We’re fine with ZTE’s clarification, but we think ZTE needs to update the cover sheet in the CR. The change reason in the CR is misunderstanding. |
|  | ZTE: Thank MediaTek for the discussion. We will revise the cover sheet and state more clearly the intended reason for the proposed change. |
| R4-2007751 R4-2007752 (Cat A) | Huawei: DraftCR R4-2004287 endorsed at RAN4# 94bis-e. |
|  | Company B |
| R4-2006190 R4-2006191 (Cat A) | Ericsson: Unnecessary to include HARQ delay since UE should not wait for HARQ transmission to start BWP switching. This is similar to HO where UE starts HO after receiving the RRC message |
|  | To Ericsson: We have different views on this. Here, the point is UE still needs to do the HARQ transmission. In HO, the interruption time starts from the end of Trrc-processing, and the HARQ transmission can be done within Trrc-processing (as defined in HO requirement that “The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.”), and we didn’t preclude UE to transmit HARQ for PDSCH of HO command. However, in current RRC based BWP switching, if without this revision, that means UE will not transmit HARQ to network for the PDSCH carrying RRC signaling of BWP switching. |
|  | MTK: We are ok with Apple’s proposal to permit the HARQ feedback in RRC-based BWP switch, but the wording here needs to be update. |
|  | Huawei: Pending on issue 5-3. |
|  | Nokia: proposal is fine. Wording need rework to make it more clear and more readable. |
| R4-2007707 R4-2007708 (Cat A) | Resubmission of endorsed Draft CR R4-2004277 |
|  | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#5-1 | 6 companies participated in the discussion. 3 companies supported the proposal. 1 company disagreed. 2 companies would like to have more discussion. Tentative agreements: No Candidate options: - Option 1: Agree on the proposed change below (with changes from Apple) - For FR2 SCell activation where there is active serving cell(s) in the band, the requirements apply provided that the SSB pattern is same for the active serving cell(s) and the SCell being activated - Option 2: Consider having the split requirements with the same SSB pattern and without the same SSB pattern Recommendations for 2nd round: Based on the recommended options, further discussion is needed. |
| Sub-topic#5-2-1 | 6 companies participated in the discussion. 4 companies supported Option 1. 2 companies supported Option 2 1 company had concern on this topic. Tentative agreements: No Candidate options: - Option 1 (Qualcomm, Mediatek, Huawei, HiSilicon, NEC) - Option 2 (Apple, Huawei, HiSilicon) Recommendations for 2nd round: Based on the recommended options, further discussion is needed. The response to Nokia comment and questions are needed. |
| Sub-topic#5-2-2 | 4 companies participated in the discussion. 2 companies supported the proposal. 1 company seems OK but proposed additional clarifications. 1 company disagreed with the change. Tentative agreements: No Candidate options: - Option 1 (Apple, Mediatek): - the proposed changes are as follows - Option 1a (NEC): we can clarify in the current spec that UE is allowed to transmit HARQ during total switching delay time. - Option 2 (Huawei, HiSilicon): no change Recommendations for 2nd round: Based on the recommended options, further discussion is needed. |
| Sub-topic#5-3 | 6 companies participated in the discussion. 3 companies supported the proposal. 3 company disagreed with the change. Tentative agreements: No Candidate options: - Option 1 (Apple, Nokia): Agree on proposal 1 and proposal 2. - CR needs modification (Nokia). - Option 1a (Mediatek): Agree on proposal 1. Agree on proposal 2 for Rel-16. - Option 2 (Ericsson, Huawei, NEC): No change needed. Recommendations for 2nd round: Based on the recommended options, further discussion is needed. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on maintenance topics for signaling characteristics for issue 5-1~5-3 | Apple |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2007659 | Revised. Capture the comments from Ericsson and Nokia. To agree CR, try to convince Apple. |
| R4-2007660 | Return to. Cat A CR to R4-2007659. |
| R4-2007661 | Merged into revised version of R4-2007783 Capture the comments from Qualcomm, Mediatek, Huawei, and Nokia. To agree CR, try to convince Apple. |
| R4-2007662 | Withdrawn. |
| R4-2007783 | Revised. Try to capture the new changes in R4-2007661 and comments from Qualcomm. |
| R4-2007784 | Return to. Cat A CR to R4-2007783 |
| R4-2007812 | Return to To capture the tentative agreement for sub-topic 5-1 if agreed. If no agreement for sub-topic 5-1 was reached, then this endorsed CR can be formally agreed. |
| R4-2007813 | Return to. Cat A CR to R4-2007812. |
| R4-2007280 | Merged into revised version of R4-2007783. |
| R4-2006847 | Revised. Have further discussion and try to capture comment from Huawei if agreed. |
| R4-2006848 | Return to. Cat A CR to R4-2006847. |
| R4-2007706 | Postponed. |
| R4-2007705 | Withdrawn. |
| R4-2007963 | Return to. This one is related to CR R4-2007263 from Nokia and CR R4-2007698 and R4-2007699 from Huawei under NR-U RRM agenda. So R4-2007963 is expected to discuss with those Tdocs. Ericsson should provide the responses to companies’ comments on how to treat the CRs. |
| R4-2007964 | Return to. Cat A CR to R4-2007963 |
| R4-2007663 | Return to. Formal CR of R4-2005835 (endosed in RAN4 #94-ebis) This document is missing in the first round discussion. We have to go directly to 2nd round to collect comments. |
| R4-2007664 | Return to. Cat A CR to R4-2007663. |
| R4-2006177 | Merged into the revised CR R4-2006209. |
| R4-2006178 | Withdrawn. |
| R4-2006209 | Revised. Try to capture the comments from Qualcomm, Ericsson. To agree CR, need convince Huawei. |
| R4-2006210 | Return to. Cat A CR to R4-2006209. |
| R4-2006465 | Return to More discussion on the additional change compared to the endorsed one. |
| R4-2006466 | Return to. Cat A CR to R4-2006465. |
| R4-2007780 | Agreed. |
| R4-2007781 | Agreed |
| R4-2006891 | Revised Change on the cover page for the change reason. |
| R4-2006892 | Return to. Cat A CR to R4-2006891. |
| R4-2007751 | Agreed. |
| R4-2007752 | Agreed. Cat A CR to R4-2007751. |
| R4-2006190 | Return to Pending on the discussion for issue 5-3 |
| R4-2006191 | Return to. Cat A CR to R4-2006190. |
| R4-2007707 | Agreed. |
| R4-2007708 | Agreed. Cat A CR to R4-2007707 |

Discussion on 2nd round (if applicable)

In the second round the following email threads are suggested to organize the discussions:

- Email thread for Issue 5-1, 5-2-1, 5-2-2, 5-3, including new WF, R4-2007812, R4-2007813, R4-2006209, R4-2006210, R4-2006190, R4-2006191, which will be triggered and led by Apple.

- Email thread for R4-2007659 and R4-2007660, which will be triggered and led by Huawei

- Email thread for R4-2007783 and R4-2007784, which will be triggered and led by Ericsson

- Email thread for R4-2006847 and R4-2006848, which will be triggered and led by Mediatek

- Email thread for R4-2007963 and R4-2007964, which will be triggered and led by Ericsson

- Email thread for R4-2007663 and R4-2007664, which will be triggered and led by Huawei

- Email thread for R4-2006465 and R4-2006466, which will be triggered and led by Mediatek

- Email thread for R4-2006891 and R4-2006892, which will be triggered and led by ZTE

[Comments and responses will be collected by moderator here]

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XX | WF on maintenance topics for signaling characteristics |
| R4-2007812 |  |
| R4-2007813 | Cat A CR to R4-2007812 |
| R4-2006209 |  |
| R4-2006210 | Cat A CR to R4-2006209 |
| R4-2006190 |  |
| R4-2006191 | Cat A CR to R4-2006190 |
| R4-2007659 |  |
| R4-2007660 | Cat A CR to R4-2007659 |
| R4-2007783 |  |
| R4-2007784 | Cat A CR to R4-2007783 |
| R4-2006847 |  |
| R4-2006848 | Cat A CR to R4-2006847 |
| R4-2007963 |  |
| R4-2007964 | Cat A CR to R4-2007963 |
| R4-2007663 |  |
| R4-2007664 | Cat A CR to R4-2007663 |
| R4-2006465 |  |
| R4-2006466 | Cat A CR to R4-2006465 |
| R4-2006891 |  |
| R4-2006892 | Cat A CR to R4-2006891 |

Topic #6: Beam management

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006187 | Apple | CR on SMTC2 configuration in SSB based CBD for R15 (CR) - The SMTC2 configuration is missing in SSB based CBD requirement. - The exception condition for longer evaluation delay is missing in SSB based CBD requirement. - Other correction on the wrong section IDs - Add the SMTC2 configuration and exception condition into SSB based CBD requirement, which is same as in BFD requirement. |
| R4-2006188 | Apple | CR on SMTC2 configuration in SSB based CBD for R16 (CR Cat A) |
| R4-2006852 | MediaTek inc., Huawei, HiSilicon | CR on SMTC period for beam management requirements - On 8.1.2.2, 8.1.3.2, 8.5.2.2, 8.5.3.2, 8.5.5.2, 8.5.6.2, 9.5.4.1, 9.5.4.2, - Add clarification on TSMTCperiod for multiple FR2 CCs. - On 8.5.5.2 and 8.5.6.2 - Add clarification on smtc1 and smtc2 for TSMTCperiod in candidate beam detection. - Add the SMTC2 configuration and exception condition into CBD requirement, which is same as in BFD requirement. - On 9.5.3.1, 9.5.3.2, 9.5.3.3 - correction on the wrong section IDs |
| R4-2006853 | MediaTek inc., Huawei, HiSilicon | CR on SMTC period for beam management requirements (CR Cat A) |
| R4-2006849 | MediaTek inc. | Semi-persistent or aperiodic SSB based L1-RSRP reporting on PUSCH in FR2 Observation 1: For a semi-persistent or aperiodic SSB based L1-RSRP report on PUSCH in FR2, the measurement time is insufficient, if the scheduling restriction only allowed after report triggering. Thus, the UE will not meet the corresponding accuracy requirement. Observation 2: If scheduling restriction is allowed after the SSB is configured for L1-RSRP measurement, no matter the reporting type is periodic, semi-persistent or aperiodic, UE can have sufficient measurement time for SSB based L1-RSRP in FR2. Proposal 1: Down-select one of the following options, - Option 1: L1-RSRP accuracy requirement doesn’t apply for SSB based L1-RSRP in FR2, when the reporting is aperiodic or semi-persistent on PUSCH. - Option 2: Scheduling restriction should apply once the SSB is configured for L1-RSRP measurement, no matter the reporting type is periodic, semi-persistent or aperiodic. Proposal 2: if no consensus, there is no requirement for SSB based L1-RSRP in FR2, when the reporting is aperiodic or semi-persistent on PUSCH. |
| R4-2006850 | MediaTek inc. | CR for Semi-persistent or aperiodic SSB based L1-RSRP reporting on PUSCH in FR2 For a semi-persistent or aperiodic SSB based L1-RSRP report on PUSCH in FR2, the offset between the CSI report request and PUSCH specified in TS 38.214 is shorter than the measurement period specified in TS 38.133. In other words, measurement time is insufficient. On 9.5.3.2 and 9.5.3.3 Add “This requirement does not apply for SSB based L1-RSRP measurements contained in a L1-RSRP report send on PUSCH in FR2.” |
| R4-2006851 | MediaTek inc. | CR for Semi-persistent or aperiodic SSB based L1-RSRP reporting on PUSCH in FR2 |
| R4-2007814 | Huawei, HiSilicon | Discussion on SSB based L1-RSRP measurement Observation 1: SSB can only be configured as periodic resource for L1-RSRP measurement. Observation 2: UE performs periodic L1-RSRP measurement on SSB regardless of the reporting type. Proposal: scheduling restriction should be allowed on SSBs configured for L1-RSRP measurement. |
| R4-2007815 | Huawei, HiSilicon | CR on SSB based L1-RSRP measurement R15 Clarify the wording in L1-RSRP scheduling restriction with regard to the “symbols to be measured”, for different resource type. |
| R4-2007816 | Huawei, HiSilicon | CR on SSB based L1-RSRP measurement R16 |
| R4-2006854 | MediaTek inc. | CR for CSI-RS based L1-RSRP measurement period (CR) Resubmission of endorsed Draft CR R4-2005231 in RAN4#94-e-Bis. N factor is intended to capture UE behavior related to QCL-Type D, i.e. RX beam behavior, but in the current requirement of N factor, QCL-Type D is missing for the case of CSI-RS repetition 'ON'. CSI-RS repetition 'ON' is provided to facilitate UE Rx beam refinement within a relative small angular area, instead of global beam searching, so the QCL-Type D is needed for the determination of rough beam. Add side condition that QCL-Type D should be provided in FR2 for CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON. |
| R4-2006855 | MediaTek inc. | CR for CSI-RS based L1-RSRP measurement period (CR Cat A) |
| R4-2007492 | Qualcomm | Applicability of QCL (CR) The “Applicability of QCL” section in 15.8.0 defines the concept of TCI chain where multiple reference signals are assumed to be QCLed with each other if they are located in the same TCI chain. In Rel-15, one reference signal can be QCLed to two other reference signals. But, the “Applicability of QCL” section does not clarify whether there can be single or multiple QCL types per TCI chain. Add the following sentence to this section “It is assumed there is single QCL type per TCI chain” |
| R4-2007493 | Qualcomm | Applicability of QCL (CR) |

Open issues summary

Sub-topic 6-1

L1-RSRP reporting is one kind of CSI report, and it shall follow the same procedure as CSI report. Regarding the CSI report on PUSCH, the offset between the CSI report request and PUSCH has been specified in TS 38.214. However, for SSB based L1-RSRP reporting in FR2, the measurement period specified in TS 38.133 is far longer than the offset specified in TS 38.214. As a result, the SSB based L1-RSRP measurement reporting on PUSCH in FR2 cannot be completed within the configured offset.

Issue 6-1: Semi-persistent or aperiodic SSB based L1-RSRP reporting on PUSCH in FR2

- Proposals (Mediatek, R4-2006849, R4-2006850, R4-2006851; Huawei, R4-2007814, R4-2007815, R4-2007816)

  - Proposal 1: Down-select one of the following options,

    - Option 1 (Mediatek): L1-RSRP accuracy requirement doesn’t apply for SSB based L1-RSRP in FR2, when the reporting is aperiodic or semi-persistent on PUSCH.

    - Option 2 (Mediatek, Huawei, HiSilicon): Scheduling restriction should apply once the SSB is configured for L1-RSRP measurement, no matter the reporting type is periodic, semi-persistent or aperiodic.

  - Proposal 2: if no consensus, there is no requirement for SSB based L1-RSRP in FR2, when the reporting is aperiodic or semi-persistent on PUSCH.

- Recommended WF

  - TBA

Sub-topic 6-2

In RAN4#94-e-bis the issue of scheduling restriction for SSB based L1-RSRP was raised in R4-2003611 and discussed in email 101 R4-2005382. However there was no conclusion.

Issue 6-2: Scheduling restriction for SSB based L1-RSRP

- Proposals (Huawei, HiSilicon, R4-2007814, R4-2007815, R4-2007816)

  - Scheduling restriction should be allowed on SSBs configured for L1-RSRP measurement.

- Recommended WF

  - TBA

Sub-topic 6-3

Please provide the additional comments on the CRs in Section 6.3.2.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub topic 6-1: : It is true that with beamsweep SSB based L1-RSRP measurement requires at least 8 x SMTC period and it exceed the aperiodic CSI reporting delay specified in RAN1. On the other hand, it is network's responsibility to schedule SSB based L1-RSRP reporting so that UE can measure SSB within the measurement period. So we tend to support option 2, scheduling restriction for semi-persistent/aperiodic SSB based L1-RSRP reporting in FR2. Sub topic 6-2: : It is true that with beamsweep SSB based L1-RSRP measurement requires at least 8 x SMTC period and it exceed the aperiodic CSI reporting delay specified in RAN1. On the other hand, it is network's responsibility to schedule SSB based L1-RSRP reporting so that UE can measure SSB within the measurement period. So we tend to support option 2, scheduling restriction for semi-persistent/aperiodic SSB based L1-RSRP reporting in FR2. …. Others: |
| Apple | Sub topic 6-1: Issue 6-1 and issue 6-2 shall be merged. We support option 2. When SSB is configured for L1-RSRP, even though the reporting is aperiodic, the SSB shall be monitored all the time, and therefore scheduling restriction shall be applied all time as long as SSB is configured. Sub topic 6-2: …. Others: |
| Qualcomm | Subtopic 6-1: we support option 2. Suptopic 6-2: Huawei’s proposal is fine. |
| MTK | Sub topic 6-1: Our 1st preference is the Option 2 in the Proposal 1. Because in the delay requirement, N=8 is assumed for SSB, which means RX beam sweeping is expected and the scheduling restriction shall be expected when performing SSB measurement. Thus, Option 2 is the most logical way to have the consistency in the spec. If option 2 is not agreeable, no requirement shall apply for this scenario, because the conflict between TS 38.133 and TS 38.214. Sub topic 6-2: Support “Scheduling restriction should be allowed on SSBs configured for L1-RSRP measurement”, as discussed in Issue 6-1. |
| NTT DOCOMO, INC. | Sub topic 6-1: We understand this issue, and prefer option 2. If accuracy is not ensured, semi-persistent and aperiodic reporting would be meaningless feature, and as Ericsson mentioned, network can consider how to schedule SSB in FR2. |
| Huawei | First we agree with Apple that 6-1 and 6-2 should be merged, as the proposal in 6-2 is same as option 2 in 6-1. For 6-1, we support option 2. We do not think option 1 is a good way to go. UE still has to report at the indicated slot, most likely with inaccurate results, and this is just a waste of resource for both UE and network, and there could be even adverse impact to the performance, e.g. if the report is used by the network for scheduling. Also, this option basically disables semi-persistent and aperiodic report for SSB measurement, which is also a restriction from NW side. |
| NEC | Subtopic 6-1: We support option2; Subtopic 6-2: We agree with proposal. We also agree with Apple and Huawei that 6-1 and 6-2 can be merged. |

CRs/TPs comments collection

The following CRs will be discussed in this section.

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006187 | Apple | CR on SMTC2 configuration in SSB based CBD for R15 (CR) |
| R4-2006188 | Apple | CR on SMTC2 configuration in SSB based CBD for R16 (CR Cat A) |
| R4-2006852 | MediaTek inc., Huawei, HiSilicon | CR on SMTC period for beam management requirements |
| R4-2006853 | MediaTek inc., Huawei, HiSilicon | CR on SMTC period for beam management requirements (CR Cat A) |
| R4-2006850 | MediaTek inc. | CR for Semi-persistent or aperiodic SSB based L1-RSRP reporting on PUSCH in FR2 |
| R4-2006851 | MediaTek inc. | CR for Semi-persistent or aperiodic SSB based L1-RSRP reporting on PUSCH in FR2 |
| R4-2007815 | Huawei, HiSilicon | CR on SSB based L1-RSRP measurement R15 |
| R4-2007816 | Huawei, HiSilicon | CR on SSB based L1-RSRP measurement R16 |
| R4-2006854 | MediaTek inc. | CR for CSI-RS based L1-RSRP measurement period (CR) Resubmission of endorsed Draft CR R4-2005231 in RAN4#94-e-Bis. |
| R4-2006855 | MediaTek inc. | CR for CSI-RS based L1-RSRP measurement period (CR Cat A) |
| R4-2007492 | Qualcomm | Applicability of QCL (CR) |
| R4-2007493 | Qualcomm | Applicability of QCL (CR) |

Please provide the comments in the table below.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2006187 R4-2006188 (Cat A) | Ericsson : The correction of 9.5.3.1/2/3 is wrong. Intention is to refer to the CSI reporting (not CS-RS resource configuration). We should keep to refer to 5.2.1.4 / 6.1.2.1 in TS38.214. Close to CR 6852. Suggest to merge to 6852. |
|  | Apple: similar change as to 6852, can discuss which one could be used |
|  | MTK: overlapped with 6852/6853. Suggest to merge |
|  | Nokia: do we need the wording 'higher layer parameter'? 'smallest' to 'shortest'? Why 'CCs'? Isn't it among Q1 set? Why the note as it addresses CSI-RS and CBD? |
| R4-2006852 R4-2006853 (Cat A) | Ericsson : The correction of 9.5.3.1/2/3 is wrong. Intention is to refer to the CSI reporting (not CS-RS resource configuration). We should keep to refer to 5.2.1.4 / 6.1.2.1 in TS38.214. |
|  | Apple: similar change as to 6187, can discuss which one could be used |
|  | MTK: overlapped with 6187/6188. Suggest to merge |
|  | Nokia: Because it is same band it seems ok. But same band as cell on which RLM is evaluated? Also indend the 'where' as it applies to FR2 rules. |
| R4-2006850 R4-2006851 (Cat A) | Ericsson: We prefer considering the scheduling restriction for SP/AP SSB based L1-RSRP reporting. See comments for issue 6-1 |
|  | MTK: pending on the conclusion of Issue 6-1 |
| R4-2007815 R4-2007816 (Cat A) | Ericsson : Support the approach of this CR (consider scheduling restriction for SP/AP SSB based L1-RSRP reporting) |
|  | MTK: pending on the conclusion of Issue 6-1 |
|  | Nokia: Could it be clarified what does 'when the resource is activated' (SP) and 'reporting is triggered' (AP) cover? |
| R4-2006854 R4-2006855 (Cat A) | Resubmission of endorsed Draft CR R4-2005231 in RAN4#94-e-Bis. |
|  | Company A |
|  | Company B |
| R4-2007492 R4-2007493 (Cat A) | Ericsson : CR number is missing from cover sheet, |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#6-1 | 7 companies participated in the discussion. Almost all the companies are OK with Option 2 in proposal 1. Sub-topic #6-1 is merged with sub-topic #6-2. Tentative agreements: Scheduling restriction should apply once the SSB is configured for L1-RSRP measurement, no matter the reporting type is periodic, semi-persistent or aperiodic. Candidate options: Recommendations for 2nd round: |
| Sub-topic#6-2 | Merged with sub-topic #6-2. Tentative agreements: Candidate options: Recommendations for 2nd round: |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2006187 | Merged into R4-2006852. |
| R4-2006188 | Withdrawn. Cat A CR to R4-2006187. |
| R4-2006852 | Revised Capture comments from Ericsson and Nokia. Response to Nokia question is needed. |
| R4-2006853 | Return to. Cat A CR to R4-2006852. |
| R4-2006850 | Merged into R4-2007815. Capture the agreement |
| R4-2006851 | Withdrawn. Cat A CR to R4-2006850. |
| R4-2007815 | Return to The tentative technique agreement seems be reached. This CR is aligned with the tentative agreement. Response to Nokia comments before formally agreed. |
| R4-2007816 | Return to. Cat A CR to R4-2007815. |
| R4-2006854 | Agreed. |
| R4-2006855 | Agreed. |
| R4-2007492 | Revised. Add the CR number in cover page. |
| R4-2007493 | Return to. |

Discussion on 2nd round (if applicable)

In the second round the following email threads are needed:

- Email thread for R4-2006852 and R4-2006853, which will be triggered and led by Mediatek

- Email thread for R4-2007815 and R4-2007816 which will be triggered and led by Huawei

- Email thread for R4-2007492 and R4-2007493 which will be triggered and led by Qualcomm

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2006852 | Revised |
| R4-2006853 | Return to. Cat A CR to R4-2006852. |
| R4-2007815 | Return to. |
| R4-2007816 | Return to. Cat A CR to R4-2007815. |
| R4-2007492 | Revised. |
| R4-2007493 | Return to. |