3GPP TSG-RAN WG4 Meeting #95-e	R4-20xxxxx

# Electronic Meeting, 25 May – 5 June, 2020

Agenda item:			6.4.5.3

Source:	Moderator (Mediatek Inc.)

Title:	Email discussion summary for [95e][210]5G_V2X_NRSL_RRM _2

Document for:	Information

Introduction

This email discussion is to address the open issues in NR V2X measurement.

- Discuss the necessary on whether update current L1 SL-RSRP measurements requirement

- Discuss the issues related to L1 SL-RSRP measurement accuracy

- Discuss the issues related to PSBCH-RSRP measurement accuracy

Topic #1: Measurement

  - Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006469 | MediaTek inc. | Proposal 1: Capture the general description on re-evaluation procedure as part of resource (re-)selection requirement as following sentence: After resource (re-)selection procedure, the UE shall re-evaluate the reserved resources by L1 SL-RSRP before transmission of SCI with reservation. Proposal 2: The absolute accuracy of L1 SL-RSRP can be ±4.5dB in SNR=0dB. |
| R4-2006471 | MediaTek inc. | Observation 1: Based on the simulation, the absolute accuracy of PSBCH RSRP can be achieved at SNR=-6dB before adding RF impairment margin: - AWGN: 1.1dB - TDL-C 100ns 300Hz: 1.8dB - TDL-C 100ns 150Hz: 2.2dB - TDL-C 30ns 1400Hz: 1.7dB Observation 2: Based on the simulation, the relative accuracy of PSBCH RSRP can be achieved at SNR=-3dB: - AWGN: 0.6dB - TDL-C 100ns 300Hz: 0.8dB - TDL-C 100ns 150Hz: 0.9dB - TDL-C 30ns 1400Hz: 0.8dB Proposal 1: Define absolute accuracy of ±4.5dB with side condition of SNR = -6dB for V2X PSBCH-RSRP measurement. Proposal 2: Define relative accuracy of ±2.0dB with side condition of SNR = -3dB for V2X PSBCH-RSRP measurement. |
| R4-2006671 | LG Electronics Inc. | Proposal 1: Specify re-evaluation for resource reselection with general description in RRM core requirement. Proposal 2: Define ±4.5dB as absolute accuracy of L1 SL-RSRP measurement. |
| R4-2006674 | LG Electronics Inc. | Observation 1: Absolute measurement accuracy of NR V2X PSBCH-RSRP for SCS of 15kHz, 30kHz and 60kHz meets NR Uu SS-RSRP measurement accuracy of ±4.5dB at SNR ≥ -6dB. Proposal 1: Define ±4.5dB as NR V2X PSBCH-RSRP measurement accuracy for SCS of 15kHZ, 30kHz and 60kHz. Proposal 2: Define SNR ≥ -6dB as side condition for NR V2X PSBCH-RSRP measurement accuracy. |
| R4-2006711 | Qualcomm | Proposal 1: NR SL-RSRP measurement accuracy requirement is not tighter than LTE SL-RSRP measurement accuracy requirement. Proposal 2: SL-RSRP measurement requirement is defined as Table 2-3, requirement is set based on PSCCH simulation results and proposal 1. This requirement applies to both PSCCH and PSSCH DMRS measurement. Accuracy; Conditions. Normal condition; Extreme condition; Ês/Iot Note 4; Io Note 1 range. NR V2X operating band groups Note 3; Minimum Io; Maximum Io. dB; dB; dB; dBm/15kHz Note 2; dBm/BWChannel; dBm/BWChannel. 5; 9.5; 0; [TDD_A]; [-121]; N/A; -70. [TDD_G]; [-118]; N/A; -70. 8.5; 11.5; 0; [TDD_A ,TDD_G]; N/A; -70; -50. NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections [B.4.2] and [B.4.3]. NOTE 3: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 4: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS or PSSCH-DMRS, depending on which DMRS the requirement applies to.. Proposal 3: PSBCH-RSRP measurement requirement is defined as Table 2-5. Accuracy; Conditions. Normal condition; Extreme condition; Ês/Iot Note 4; Io Note 1 range. NR V2X operating band groups Note 3; Minimum Io; Maximum Io. dB; dB; dB; dBm/15kHz Note 2; dBm/BWChannel; dBm/BWChannel. 4.5; 9; -3 dB; [TDD_A]; [-121]; N/A; -70. [TDD_G]; [-118]; N/A. 8; 11; -3 dB; [TDD_A ,TDD_G]; N/A; -70; -50. NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: NR V2X operating band groups are as defined in Section 3.5 for the corresponding E-UTRA operating bands. NOTE 4: Ês/Iot for a SyncRef UE is the minimum of the Ês/Iot of PSSS/PSBCH and the Ês/Iot of SSSS. |
| R4-2007763 | HUAWEI | NA |
| R4-2007764 | HUAWEI | Proposal 1: It is suggested to use option 1 to define the L1 SL-RSRP absolute accuracy under the condition of PSCCH/PSSCH SINR≥0dB. Proposal 2: The existing intra-frequency SS-RSRP measurement accuracy requirements in FR1 can be reused for defining PSBCH-RSRP measurement accuracy requirements. |

  - Open issues summary

Sub-topic 1: Update the core requirement of SL L1-RSRP measurements

RAN4 to discuss whether to update SL L1-RSRP measurements requirement for re-evalation.

Issue 1-1: Whether to capture general re-evaluation behavior in RAN4 spec.

- Proposals

  - Option 1: Yes. (Mediatek, LG)

  - Such as After resource (re-)selection procedure, the UE shall re-evaluate the reserved resources by L1 SL-RSRP before transmission of SCI with reservation.

- Recommended WF

  - Need further discussion.

Sub-topic 2: L1 SL-RSRP Measurement Accuracy

RAN4 to discuss the measurement accuracy of L1 SL-RSRP.

Issue 2-1: Absolute accuracy of L1 SL-RSRP measurement

- Proposals

  - Option 1: ±4.5dB (LG, Huawei, Mediatek)

  - Option 2: ±5dB (Qualcomm)

- Recommended WF

  - Need further discussion.

Sub-topic 3: PSBCH-RSRP measurement accuracy

RAN4 to discuss the measurement accuracy of PSBCH-RSRP.

Issue 3-1: Absolute accuracy of PSBCH-RSRP measurement

- Proposals

  - Option 1: Define absolute accuracy of ±4.5dB with SNR = -6dB. (Mediatek, LG, Huawei)

  - Option 2: Define absolute accuracy of ±4.5dB with SNR = -3dB. (QC)

- Recommended WF

  - Need further discussion.

Issue 3-2: Relative accuracy of PSBCH-RSRP measurement

- Proposals

  - Option 1: Define relative accuracy of ±2.0dB with SNR = -3dB. (Mediatek, Huawei)

- Recommended WF

  - Define PSBCH-RSRP relative accuracy of ±2.0dB with SNR = -3dB.

  - Companies views’ collection for 1st round

    - Open issues

| Company | Comments |
| --- | --- |
| Mediatek | Sub topic 1-1: This section L1 SL-RSRP measurements is only defined the core requirement for L1 SL-RSRP measurements and not define any related procedure to how to use this measurements. Our purpose is to guarantee no mismatch between RAN1 and RAN4 spec. It just say that UE can use L1 SL-RSRP to re-evaluation the resources. Sub topic 2-1: Based on the simulation results in #94-e meeting from each company, we suggest to use option 1. It’s reasonable to have better performance in new generation system than legacy one. Sub topic 3-1: Based on our simulation results, we suggest to use option1. We disagree to define NR SL requirement even worse than LTE SL. Sub topic 3-2: Option 1. |
| LG Electronics | Sub topic 1-1: We suggest to specify with general description without any procedure as follows. ## 12.5.1 Introduction This section contains the measurement requirements related to resource reselection, resource re-evaluation and resource pre-emption of the UE capable of V2X sidelink communication. ## 12.5.2 SL-RSRP measurements The UE physical layer shall be capable of performing the L1 SL-RSRP measurements on the carrier operating V2X sidelink communication for determining the subset of resources to be excluded in PSSCH resource selection in sidelink transmission mode 2 and for resource re-evaluating the reserved resources. Sub topic 2-1: Option 1 Sub topic 3-1: Option 1 Sub topic 3-2: Option 1 |
| QC | Subtopic 1-1 As we commented in previous meeting, RAN4 measurement requirement only specifies the related procedure that is testable, not captures all the measurement procedures defined in RAN1/2. We can have further discussion on whether a test is required and how to agree on test procedure, but before a measurement related procedure is captured in RAN4 core requirement, we at least have to verify that whether this is testable. We do not support adding a requirement when it is not testable from our perspective, as we already comment previous why this requirement may not be testable. Subtopic 2-1 Our concern for having a requirement tighter than LTE is that the number of RSRE is fewer in NR measurement than in LTE measurement, how NR can achieve better precision than LTE. It is true that new technology can improve performance, but that is conditioned on a change in physical layer design that can benefit performance. In this case, it is the opposite: in NR SL measurement procedure, number of RSRE is less than LTE SL measurement. Unless companies can provide valid argument for how NR can achieve better accuracy than LTE with “less” RSRE available to measure, we still consider option 2 as a reasonable option with enough RF + implementation margin. Subtopic 3-1 Our results show that side condition of -3dB provides sufficient margin. However, we can compromise to support -6dB side condition as less than 1% BLER is observed in our simulation. |
| LG | Subtopic 2-1 To QC, the number of REs for PSCCH-DMRS in NR is 30REs for 10RB based on agreed simulation assumption. And, the number of REs for PSSCH-DRR in LTE is 36REs for 3RBs based on agreed simulation assumption (R4-168695). The difference is 6REs. However, from simulation results in R4-2003428(NR-V2X) and R4-1609888(LTE-V2X), we can see PSCCH-RSRP(1.79dB) in NR is better than PSSCH-RSRP(2.48dB) in LTE at AWGN & SNR=0dB. So, we think option 1 is acceptable. |
| Huawei | Sub topic 2-1: We support option 1. The number of DMRS REs in NR is comparable with the number of DMRS REs in LTE. Based on the simulation results, it can be observed that the measurement performance in NR is even better than that in LTE due to larger measurement BW. Sub topic 3-1: We support option 1. In RAN4#94 e-meeting, RAN4 agreed that the side condition of S-RSRP measurements for initiation/cease SLSS transmissions is defined as -6dB. Sub topic 3-2: We agree with the recommended WF. |
| Mediatek | Subtopic 1-1 To QC, we don’t depict the re-evaluation procedure. This is a requirement for L1 SL-RSRP. We can give an example that the procedure was defined in core requirement but it cannot be tested. In SRS carrier switching, we say it shall extend the measurement when collision with SRS carrier switching. Could QC further clarify how to test this? From our understanding, it cannot be tested but we defined the procedure. Could QC give an example which measurement procedure didn’t capture in R4 RRM except this re-evaluation? Subtopic 2-1 To QC, another reason in LTE to have a ‘loose’ requirement is LTE sidelink needs to support high speed moving(500km/h). The accuracy defined in LTE refers on tdoc R4-1610638. If we preclude the high speed moving result in the paper, it could find that the RSRP accuracy will be also better. In NR, we don’t need to support such high speed now. At the same time, QC suggested to not consider any high Doppler case in the simulation to NR L1 SL-RSRP requirement in #94e meeting. This is another reason the simulation performance in NR is better than LTE. |

    - CRs/TPs comments collection

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2006470 | QC: need to agree on subtopic 1 first |
| R4-2006685 | QC: square bracket in lo and band group can't be removed as REFESNESE is not finalized. Also pending discussion on subtopic 2 |
| R4-2007765 | QC: square bracket in lo and band group can't be removed as REFESNESE is not finalized. Also pending discussion on subtopic 3 |

  - Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1 | Tentative agreements: Candidate options: Option 1: Capture the general description on re-evaluation in RAN4 spec.(LG, Mediatek) Option 2: Not needed to capture the description(Qualcomm) Recommendations for 2nd round: Since this is the last meeting in R16 Core part, moderator suggests to conclude this issue in 2nd round. |
| Issue 2-1 | Tentative agreements: Candidate options: Option 1: ±4.5dB (LG, Huawei, Mediatek) Option 2: ±5dB (Qualcomm) Recommendations for 2nd round: Since this is the last meeting in R16 Core part, moderator suggests to conclude this issue in 2nd round. |
| Issue 3-1 | Tentative agreements: Define PSBCH absolute accuracy of ±4.5dB with SNR = -6dB Candidate options: Recommendations for 2nd round: |
| Issue 3-2 | Tentative agreements: Define PSBCH relative accuracy of ±2.0dB with SNR = -3dB Candidate options: Recommendations for 2nd round: |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on NR V2X RRM requirements Note: This is the same WF with V2X part 1 | LG Electronics, Mediatek |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2006470 | To be revised |
| R4-2006685 | To be revised |
| R4-2007765 | To be revised |

  - Discussion on 2nd round (if applicable)

Issue 1-1: Whether to capture general re-evaluation behavior in RAN4 spec.

- Proposals

  - Option 1: Yes. (Mediatek, LG)

## 12.5.1 Introduction This section contains the measurement requirements related to resource reselection, resource re-evaluation and resource pre-emption of the UE capable of V2X sidelink communication. ## 12.5.2 SL-RSRP measurements The UE physical layer shall be capable of performing the L1 SL-RSRP measurements on the carrier operating V2X sidelink communication for determining the subset of resources to be excluded in PSSCH resource selection in sidelink transmission mode 2 and for resource re-evaluating the reserved resources.

  - Option 2: No. (Qualcomm)

| Company | Comments |
| --- | --- |

Issue 2-1: Absolute accuracy of L1 SL-RSRP measurement

- Proposals

  - Option 1: ±4.5dB (LG, Huawei, Mediatek)

  - Option 2: ±5dB (Qualcomm)

| Company | Comments |
| --- | --- |

  - Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |