3GPP TSG-RAN4 Meeting #95-e	R4-2008588

# Electronic meeting, 25th May – 5th June, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0636 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on interruption requirements for NR V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  |  | Date: |  | 2019-3-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Resubmission of endorsed draft CR R4-2005312. And additional change on sub-section 12.7.1 is updated. Square bracket in the spec should be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The interruption length is specified based on async case. - Changes are endorsed in draft CR R4-2005312. - Remove the square bracket in the section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interruption requirements are confusing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 12.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.533 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

12.7	Interruption

12.7.1	Interruptions to WAN due to V2X Sidelink Communication

This sub-clause contains the requirements related to the interruptions on the PCell/serving cell due to V2X sidelink communication.

A UE capable of V2X sidelink communication may indicate its interest (initiation or termination) in V2X sidelink communication to the connected gNodeB using IE SidelinkUEInformationNR in TS 38.331[2].

The UE is allowed an interruption of up to the duration shown in table 12.7.1-1 on the PCell/serving cell during the RRC reconfiguration procedure that includes the V2X sidelink communication configuration message SL-ConfigDedicatedNR in TS 38.331[2] (setup and release). This interruption is for both uplink and downlink of the PCell/serving cell.

Table 12.7.1-1: Interruption length at V2X RRC reconfiguration

|  | NR Slot length (ms) | Interruption length (number of slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 5 |  |
| 3 | 0.125 | 9 |  |

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change

This sub-clause contains the requirements related to the interruptions on the V2X sidelink communication due to synchronization source change.

For only NR V2X sidelink capable UE, UE is allowed to drop V2X SL transmission or reception for up to 1ms when synchronization source is changed:

- From GNSS

  - to syncRef UE that is synchronized to GNSS directly/in-directly

  - to syncRef UE that has the lowest priority

- From syncRef UE that is synchronized to GNSS directly/in-directly

  - to GNSS

  - to syncRef UE that has the lowest priority

- From syncRef UE that has the lowest priority

  - to GNSS

  - to syncRef UE that is synchronized to GNSS directly/in-directly

  - to syncRef UE that has the lowest priority

For NR V2X UE supporting gNB/eNB as synchronization reference source, UE is allowed to drop V2X SL transmission or reception for up to 1ms when synchronization source is changed:

- From GNSS

  - to syncRef UE that is synchronized to GNSS directly/in-directly

  - to gNB/eNB

  - to syncRef UE that is synchronized to gNB/eNB directly

  - to syncRef UE that is synchronized to gNB/eNB in-directly

  - to syncRef UE that has the lowest priority

- From syncRef UE that is synchronized to GNSS directly/in-directly

  - to GNSS

  - to gNB/eNB

  - to syncRef UE that is synchronized to gNB/eNB directly

  - to syncRef UE that is synchronized to gNB/eNB in-directly

  - to syncRef UE that has the lowest priority

- From gNB or eNB

  - to GNSS

  - to syncRef UE that is synchronized to GNSS directly/in-directly

  - to eNB or gNB

  - to syncRef UE that is synchronized to gNB or eNB directly

  - to syncRef UE that is synchronized to gNB or eNB in-directly

  - to syncRef UE that has the lowest priority

- From syncRef UE that is synchronized to gNB/eNB directly

  - to GNSS

  - to syncRef UE that is synchronized to GNSS directly/in-directly

  - to gNB/eNB

  - to syncRef UE that is synchronized to gNB/eNB directly

  - to syncRef UE that is synchronized to gNB/eNB in-directly

  - to syncRef UE that has the lowest priority

- From syncRef UE that is synchronized to gNB/eNB in-directly

  - to GNSS

  - to syncRef UE that is synchronized to GNSS directly/in-directly

  - to gNB/eNB

  - to syncRef UE that is synchronized to gNB/eNB directly

  - to syncRef UE that is synchronized to gNB/eNB in-directly

  - to syncRef UE that has the lowest priority

- From syncRef UE that has the lowest priority

  - to GNSS

  - to syncRef UE that is synchronized to GNSS directly

  - to syncRef UE that is synchronized to GNSS in-directly

  - to gNB/eNB

  - to syncRef UE that is synchronized to gNB/eNB directly

  - to syncRef UE that is synchronized to gNB/eNB in-directly

  - to syncRef UE that has the lowest priority

UE is allowed to interruption any V2X sidelink signals including PSSCH, PSCCH, PSBCH, PSFCH and SLSS signals.

<< End of Change #1>>