3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2008593

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 681 | Rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR of NR V2X measurement accuracy requirements(SL-RSSI and L1 SL-RSRP) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2020-05-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce V2X measurement accuracy requirements for SL-RSSI and L1 SL-RSRP |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resubmission of endorsed Draft CR R4-2005315 Additional changes from R4-2005315 are, - Remove title of 10.4.4 and 10.4.4.1, and Es/Iot in Table 10.4.4.1-1 - PSCCH-RSRP[/PSSCH-RSRP] PSCCH-RSRP and/or PSSCH-RSRP - In Table 10.4.4.1-1, - SCSPSCCH SCS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Miss NR V2X measurement accuracy requirements requirements for SL-RSSI and L1 SL-RSRP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.3, 10.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1

10.4.3.1	Absolute SL-RSSI Accuracy

The intra-frequency SL-RSSI requirements are specified in Table 10.4.3.1-1. The requirements apply for measurement period of 1slot and for any configured measurement bandwidth larger than 10 RBs, provided that:

-	All symbols duing each RSSI measurement duration are available for RSSI sampling within the same measurement interval.

Table 10.4.3.1-1: Intra-frequency SL-RSSI absolute accuracy

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |
|  |  | NR V2X operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB |  | dBm/SCSSL |  |  | dBm/BWChannel |
|  |  |  | SCSSL = 15kHz | SCSSL = 30kHz | SCSSL = 60kHz |  |
| 2.5 | 5.5 | [NR_TDD_FR1_A] | [-121] | [-118] | [-115] | -50 |
|  |  | [NR_TDD_FR1_I] | [-117] | [-114] | [-111] | -50 |
| 4.5 | 7.5 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1

10.4.4.1	Absolute L1 SL-RSRP Accuracy

The requirements for absolute accuracy of L1 SL-RSRP in this clause apply to a UE performing PSCCH-RSRP and/or PSSCH-RSRP measurements on the same frequency as used by operating V2X sidelink communication.

The accuracy requirements in Table 10.4.4.1-1 are valid under the following conditions:

-	Demodulation reference signals for PSCCH and/or PSSCH are transmitted from one port.

-	Conditions defined in clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	PSCCH-RSRP|dBm and/or PSSCH-RSRP|dBm according to Annex B.4.4 for a corresponding Band are fulfilled.

Table 10.4.4.1-1: Intra-frequency L1 SL-RSRP absolute accuracy for UE capable of V2X sidelink communication

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR V2X operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm/SCS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCS = 15kHz | SCS = 30kHz | SCS = 60kHz |  |  |
| TBD | TBD | 0 dB | [NR_TDD_FR1_A] | [-121] | [-118] | [-115] | N/A | -70 |
|  |  |  | [NR_TDD_FR1_I] | [-117] | [-114] | [-111] | N/A | -70 |
| TBD | TBD | 0 dB | [NR_TDD_FR1_A] [NR_TDD_FR1_I] | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS and/or PSSCH-DMRS. |  |  |  |  |  |  |  |  |

< END OF CHANGE #1 >