TSG-RAN Working Group 4 (Radio) meeting #95-E	R4-2008773

Electronic Meeting, May 25th – June 5th 2020

Source:	Ericsson

Title:	WF on IAB-MT maximum output power and scaling factor for emission

Agenda item:	6.5.2.2.2

Document for:	Approval

- Introduction

At RAN4#95-E  meeting a way-forward on  IAB-MT maximum output power and scaling factor for emission

was created based on the discussion [1].

- Way-Forward

In R4-1907825, it is agreed to use the TRP of 38 dBm for the IAB node Tx power and with 64 antenna elements. As the conducted power is per TAB and rated power could  be scaled with number of the declared TXU_active, the rated output  power limit can be 33 dBm with scaleing factor of 8 . The rated output power limit will be 47 dBm and this is not simulated and thus is not preferred.

Issue 1-1: maximum output power per TAB connector for Local area IAB-MT 1-H

- Proposals

  - Option 2: 24dBm

  - Option 1:38dBm

Issue 1-2-1: scaling factor for rated outptut power limit IAB-MT 1-H

- Proposals

  - Option 2: N = min(NTXU,active , 8)

NTXU,active depends on the actual number of active transmitter units of IAB-MT

Issue 1-2-2: scaling factor for emission requirement of IAB-MT 1-O

- Proposals

  - Option 2: N = 8

Issue 1-3: minimum number of TRX for IAB-MT 1-O

- Proposals

  - Option 3: not defined

- References

[1]	R4-2008883, Email discussion summary for [95e][308] NR_IAB_RF_Part_1, ZTE