3GPP TSG RAN WG4 Meeting #95-e	R4-2007407

E-Meeting, 25th May- 5th June, 2020

Agenda Item:	6.5.2.1.3

Source: 	ZTE Corporation

Title:	TP to TS 38.174: IAB ICS requirement (section 7.8 and 10.9)

Document for:	Approval

- Introduction

After RAN4#94-e-Bis the IAB TS[1] is email approved. Section 2 provides text proposal on 7.8 and 10.9 IAB RX ICS requirement for the IAB TS 38.174 (Integrated access and backhaul radio transmission and reception).

- Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

7.8	In-channel selectivity

Detailed structure of the subclause is TBD.

7.8.1 General

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations TAB connector for IAB-DU type 1-H in the presence of an interfering signal received at a larger power spectral density. In this condition a throughput requirement shall be met for a specified reference measurement channel. The interfering signal shall be an NR signal which is time aligned with the wanted signal.

7.8.2 Minimum requirement for IAB-DU type 1-H

The wide area IAB-DU receiver in-channel selectivity requirement is specified the same as the wide area receiver in-channel selectivity requirement for BS type 1-H in TS 38.104[x], subclause 7.8.2, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

The medium range IAB-DU receiver in-channel selectivity requirement is specified the same as the medium range BS  receiver in-channel selectivity requirement for BS type 1-H in TS 38.104[x], subclause 7.8.2, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

The local area IAB-DU receiver in-channel selectivity requirement is specified the same as the local area BS  receiver in-channel selectivity requirement for BS type 1-H in TS 38.104[x], subclause 7.8.2, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

Referenced requirements applying to NB-IoT are not applicable to the IAB-DU

--------------------------------------------------Next of TP------------------------------------------------------

10.9	OTA in-channel selectivity

10.9.1 General

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of an interfering signal received at a larger power spectral density. In this condition a throughput requirement shall be met for a specified reference measurement channel. The interfering signal shall be an NR signal as specified in annex [A.1] and shall be time aligned with the wanted signal

10.9.2 Minimum requirement for IAB-DU type 1-O

The wide area IAB-DU receiver in-channel selectivity requirement is specified the same as the wide area receiver in-channel selectivity requirement for BS type 1-O in TS 38.104[x], subclause 10.9.2, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

The medium range IAB-DU receiver in-channel selectivity requirement is specified the same as the medium range BS  receiver in-channel selectivity requirement for BS type 1-O in TS 38.104[x], subclause 10.9.2, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

The local area IAB-DU receiver in-channel selectivity requirement is specified the same as the local area BS  receiver in-channel selectivity requirement for BS type 1-O in TS 38.104[x], subclause 10.9.2, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

10.9.3 Minimum requirement for IAB-DU type 2-O

The wide area IAB-DU receiver in-channel selectivity requirement is specified the same as the wide area receiver in-channel selectivity requirement for BS type 2-O in TS 38.104[x], subclause 10.9.3, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

The medium range IAB-DU receiver in-channel selectivity requirement is specified the same as the medium range BS  receiver in-channel selectivity requirement for BS type 2-O in TS 38.104[x], subclause 10.9.3, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

The local area IAB-DU receiver in-channel selectivity requirement is specified the same as the local area BS  receiver in-channel selectivity requirement for BS type 2-O in TS 38.104[x], subclause 10.9.3, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-2004801, “Updated IAB TS”, Qualcommn Inc.