3GPP TSG-RAN WG4 Meeting # 95-e 														R4-2008796 Electronic Meeting, 25 May – 5 June, 2020

Agenda Item:	6.5.2.2.3

Source: 	Ericsson

Title:	TP to TS 38.174: OTA RX spurious

Document for:	Approval

- Introduction

Section 2 proivde some reasoning on the text proposal and Section 3 provides text proposal on chapter 10 for Rx spurious for the IAB TS 38.174 (Integrated access and backhaul radio transmission and reception).

# Disucssion

The text proposal is based on the WF [1] and WF [2]. WF [4] agrees the free permutation of different IAB-MT class and IAB-DU class. BS spurious requirement does not differentiate class, so this agreement does not introduce additional complexity to specifiy IAB-MT RX spurious requirement. However, whether the free permutation of the different Type is not decided and thus there is uncertainty on this aspect. There is a possibility that different type could be mixed in one IAB node, For example, IAB-MT of type 1-O and IAB-DU of type 1-H. As the spurious requirement is different for different BS type, so the question is how the IAB-MT spurious should be specified. In our companion paper [4], we propose the TX spurious should follow the IAB-DU of the declared class and type irrespective the IAB-MT class and type. This is to guarantee the protection of the BS coexisting service. We believe it is the same for RX spurious requirement and IAB-MT spurious should follow the spurious requirement of the IAB-DU within the same node. Then perhaps one thing needs to mention is how to relate the IAB-MT type 1-O requirement to the IAB-DU type 1-H. Here the only thing to change is the total emission limit which should be refer to the fixed scaling factor as the same as type 1-H. This is the case when IAB-MT type 1-O is mixed with IAB-DU type 1-H or vice versa, which should be sepecified separately and cannot be refered.

The suggested text proposal below captured the above proposals.

During the 1st round, there are comments for this TP and lists below:

| R4-2007582 | This TP already applies the variable emission scaling for IAB-MT type 1-O so there is some dependency to other agreements in this thread. |
| --- | --- |

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[3]	ERC Recommendation 74-01, "Unwanted emissions in the spurious domain".

## 10.7	OTA receiver spurious emissions

### 10.7.1 General

The OTA RX spurious emission is the power of the emissions radiated from the antenna array from a receiver unit.

The metric used to capture OTA receiver spurious emissions for IAB-MT and IAB-DU for IAB type 1-O and IAB type 2-O is total radiated power (TRP), with the requirement defined at the RIB.

### 10.7.2 IAB-DU OTA receiver spurious emissions

#### 10.7.2.1 Minimum requirement for IAB-DU type 1-O

Minimum requirement is the same as specified for BS type 1-O in TS 38.104[2], subclause 10.7.2.

#### 10.7.2.2 Minimum requirement for IAB-DU type 2-O

Minimum requirement is the same as specified for BS type 2-O in TS 38.104[2], subclause 10.7.3.

### 10.7.3	IAB-MT OTA receiver spurious emissions

#### 10.7.3.1 Minimum requirement for IAB-MT type 1-O

#### 10.7.3.2 Minimum requirement for IAB-MT type 2-O

The OTA RX spurious emissions requirement shall apply during the transmitter OFF period only.

For the Wide Area IAB-MT type 2-O, the power of any RX spurious emission shall not exceed the limits in table 10.7.3.2-1.

10.7.3.2-1: Radiated Rx spurious emission limits for IAB-MT type 2-O

| Spurious frequency range (Note 4) | Limit (Note 5) | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz 1 GHz | -36 dBm | 100 kHz | Note 1 |
| 1 GHz 18 GHz | -30 dBm | 1 MHz | Note 1 |
| 18 GHz Fstep,1 | -20 dBm | 10 MHz | Note 2 |
| Fstep,1 Fstep,2 | -15 dBm | 10 MHz | Note 2 |
| Fstep,2 Fstep,3 | -10 dBm | 10 MHz | Note 2 |
| Fstep,4 Fstep,5 | -10 dBm | 10 MHz | Note 2 |
| Fstep,5 Fstep,6 | -15 dBm | 10 MHz | Note 2 |
| Fstep,6 2nd harmonic of the upper frequency edge of the DL operating band | -20 dBm | 10 MHz | Note 2, Note 3 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [2], s4.1. NOTE 2: Limit and bandwidth as in ERC Recommendation 74-01 [3], Annex 2. NOTE 3: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 4: The step frequencies Fstep,X are defined in table 10.7.3.2-2. NOTE 5: Additional limits may apply regionally. |  |  |  |

Table 10.7.3.2-2: Step frequencies for defining the radiated Rx spurious emission limits for IAB-MT type 2-O

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) | Fstep,4 (GHz) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n260 | 25 | 34 | 35.5 | 41.5 | 43 | 52 |
| n261 | 18 | 25.5 | 26.0 | 29.85 | 30.35 | 38.35 |

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-2002484, WF on TS referencing rules, Nokia

- R4-2002483, WF on IAB RX RF requirement, Samsung