3GPP TSG-RAN WG4 #95-e	R4-2008803

Electronic Meeting, May 25th – June 5th, 2020

Agenda item:	6.7.3

Source:	MediaTek Inc.

Title:	Simulation assumption for PDCCH-WUS test

Document for:	Discussion

- Introduction

In this contribution, we present simulation assumptions for joint test for PDCCH-WUS in DRX OFF and PDCCH in DRX ON. Our simulation assumptions are based on the existing Rel-15 PDCCH demodulation test number 5 defined in section 5.3.3.1.1 [1].

- Simulation Assumptions

Table 1: General parameters

| Parameter | Value |
| --- | --- |
| Bandwidth | 10MHz |
| Channel model | TDLA30-10 |
| Subcarrier spacing | 15kHz |
| Number of BS antennas | 1Tx |
| Number of UE antennas | 2Rx, 4Rx |
| DM-RS channel estimation | Realistic |
| DRX period | 80ms (Other options are not precluded) |

Table 2: PDCCH and PDCCH-WUS parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | PDCCH | PDCCH-WUS |
| DCI format | 1_0 | 2_6 |
| DCI length (excluding 24bits CRC) | 39 bits | 12 bits, 36 bits (Other options are not precluded) |
| Aggregation level | 16 | 8, 16 (Other options are not precluded) |
| CORESET symbol | 2 | 2 |
| CORESET bandwidth | 48RB | 48RB |
| Mapping type | Interleaved | Interleaved |
| REG bundle size | 6 | 6 |
| Interleaver size | 2 | 2 |

- Performance Metrics

The following performance metrics are to be provided:

- BLERPDCCH: BLER of PDCCH for the case that only PDCCH transmission

- BLERPDCCH-WUS: BLER of PDCCH-WUS for the case that only PDCCH-WUS transmission

- BLERPDCCH-JOINT: BLER of PDCCH for the case that joint transmission of PDCCH-WUS and PDCCH (UE does not wake up when missing PDCCH-WUS in DRX-OFF period)

- Reference

- TS38.101-4-f40