3GPP TSG-RAN WG4 Meeting # 95-e											            R4-2008945

Electronic Meeting, 25 May – 5 June, 2020

Agenda item:			6.6.1, 6.6.2

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [95e][115] LTE_NR_DC_CA_enh_RF

Document for:	Information

# Introduction

This email discussion concerns three topics

- Handling NR-DC configurations (if applicable: one tdoc allocated but not available)

- Specification of requirements for LTE-NR DC, NR-NR DC and NR CA within FR1

  - CRs endorsed at RAN4-e-bis

  - Specification of DC_12-n71

  - Consideration of a “blind” scheme for inter-band TDD EN-DC PC2 (related to AI 8.15)

- Cell- and UE-specific P-Max for FR2

List of candidate target of email discussion for 1st round and 2nd round (item 1)

- 1st round: TBD (depends on availability/treatment of the tdoc not available before the meeting)

List of candidate target of email discussion for 1st round and 2nd round (item 2)

- 1st round: agree CRs endorsed at RAN4#94-e-bis

- 2nd round: decision on introduction of UE time mask and proposed capability for DC_12-n71

List of candidate target of email discussion for 1st round and 2nd round (item 3)

- 2nd round: WF (with one option) on introduction of P-Max, and type (cell- and/or UE-specific) of P-Max limitation if introduced

- 2nd round: decisions on proposed CRs and Reply LS to RAN2

# Topic #1: Handling NR-DC configurations

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006655 | T-Mobile USA | Title: Reconsideration of mandatory UL NR-CA and NR-DC Not available |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1: TBA

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

### Sub-topic 1-2

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2: TBA

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: Specification of requirements for LTE-NR DC, NR-NR DC and NR CA within FR1

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006451 | MediaTek Inc. | CR endorsed at RAN4#94-e-bis (R4-2005086) |
| R4-2006997 | ZTE Corporation, Ericsson | CR endorsed at RAN4#94-e-bis (R4-2005650) |
| R4-2007799 | Ericsson | CR endorsed at RAN4#94-e-bis (R4-2005648) |
| R4-2008083 | Huawei, HiSilicon | Title: Discussion on RF requirements about DC_12_n71 (for Approval) Proposal 1: To specify the switching time mask requirements as above when only single switched UL is supported. Proposal 2: To specify new capability “only single switched UL” as Annex. Proposal 3: It’s proposed not to reflect restriction due to form factor in the spec for DC_12_n71. Proposal 4: Since only single switched UL is supported by UE for this band combination, the transmitter requirements specified in TS 38.101-1 and TS 36.101 are applicable to DC_12_n71 respectively. |
| R4-2008084 | Huawei, HiSilicon | Title: CR for 38.101-3 to specify the RF requirements for DC_12_n71 (For Approval) Reason for change: To specify the RF requirements for DC_12_n71 which supporting “only single switched UL”. |
| R4-2007048 | Ericsson | Title: The "blind" scheme for FDD-TDD EN-DC PC2 adopted for inter-band TDD-TDD EN-DC PC2 (For Discussion) Revised version available (the revision subsequently available as R4-2008460) Conclusions: why not adopt the “blind” scheme as a baseline also for inter-band TDD EN-DC PC2? Related to AI 8.15.2 |

## Open issues summary

### Sub-topic 2-1 Specification of DC_12-n71

Sub-topic description: specification of DC_12-n71 that may require single-UL transmission due to the proximity of the two UL bands.

See also discussion on DC_12_n71 Single Uplink Operation and UE types in R4-2008135 (AI 8.3 EN-DC of 1 LTE band and 1 NR band)

This is why we requested SUO only and limitation to some UE types and we have a contribution this meeting on the subject (R4-2008135 DC_12_n71 Single Uplink Operation in the basket agenda)

Open issues and candidate options before e-meeting:

Issue 2-1-1: introduction of new capability “only single switched UL”

- Proposals

  - Option 1: Needed (and associated with the TDM-pattern capability)

  - Option 2: Not needed (e.g. reuse existing capability for single-UL transmission)

  - Option 3: Other (e.g. single UL not allowed)

- Recommended WF

  - TBA

Issue 2-1-2: introduce time mask for UEs indicating IE [only supporting single switched UL] as proposed in R4-208084

- Proposals

  - Option 1: Introduce the UE time mask as proposed in R4-2008084

  - Option 2: Modify the time mask proposed in R4-2008084 (revise the technical content of the CR)

  - Option 3: Do not introduce a specific UE time mask for DC_12-n71

- Recommended WF

  - TBA

Issue 2-1-3: consider ‘restriction due to form factor’ in the specification

- Proposals

  - Option 1: Consider

  - Option 2: Do not consider

- Recommended WF

  - TBA

### Sub-topic 2-2 Blind scheme for inter-band TDD EN-DC PC2

Sub-topic description: adopt the “blind” scheme proposed for FDD-TDD EN-DC PC2 as a baseline also for inter-band TDD EN-DC PC2?

Open issues and candidate options before e-meeting:

Issue 2-2: consideration of the blind scheme for inter-band TDD EN-DC PC2

- Proposals (only for discussion)

  - Option 1: Consider

  - Option 2: Do not consider

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Intel | Sub topic 2-1: Issue 2-1-1: introduction of new capability “only single switched UL” Option 2 (not needed) Issue 2-1-2: introduce time mask for UEs indicating IE [only supporting single switched UL] as proposed in R4-208084 Option 3 (Don’t introduce a specific UE time mask for DC_12-n71) Issue 2-1-3: consider ‘restriction due to form factor’ in the specification Option 2 Sub topic 2-2: Issue 2-2: consideration of the blind scheme for inter-band TDD EN-DC PC2 Option 2 (at least don’t consider it for Rel-16) |
| OPPO | Option 2, not considered. Similar discussion happens in FDD/TDD EN-DC HPUE, and not agreeable to companies. We do not see the benefit of opening the new discussion to introduce this not agreeable new mechanisms in the closed Rel-16 WI. And the SAR solution in TDD/TDD EN-DC HPUE actually is more flexible than the solution in FDD/TDD HPUE. And no tight coordination is needed, since UE report the SA duty cycle capability based on the TDD LTE NW UL/DL broadcast information in SIB which is a static configuration unlike FDD LTE in FDD/TDD HPUE solution. In our view, current solution is simple enough and no more changes is needed. |
| ZTE | Issue 2-1-1: Option 2. It is not clear for “ single switched UL" , it seems can be seen as one of the special mode SUO according to 8135. Currently, single-UL transmission is already defined in the spec. Therefore in our view, existing capability for single-UL transmission can be reused. Issue 2-1-2: Option 3. Issue 2-2: In our view, for both PC2 TDD-TDD ENDC and PC2 TDD-FDD ENDC are share the similar apporach. Also whether to adopt the PC2 TDD-FDD ENDC 'blind' scheme is still under discussion. We propose to discuss it together with PC2 TDD-FDD ENDC in thread [131]. |
| Skyworks | Sub topic 2-1: We have a contribution R4-2008135 for which we also propose that DC_12_n71 be only supported in Single UL mode of operation. So we support proposal 1 of R4-2008083. Issue 2-1-1: introduction of new capability “only single switched UL” Option 2 (not needed). Current signalling capability can be re-used to indicate UE requests single uplink operation for DC_12_n71. New signalling message is not needed. Only single UL switched operation for DC_12_n71 could be specified as a new footnote in Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands), as proposed in CR R4-2008084. Issue 2-1-3: consider ‘restriction due to form factor’ in the specification P3: We disagree to proposal 3 of R4-2008083: Restriction to form factor must remain. P4: We disagree. Even in the case of Single UL only operation, n71 Rx and n12Rx performance might be impacted depending on RFFE partitioning. For example, if triplexer is used, it is difficult to assume no impact on Delta T/R, or same isolation as that of n71 duplexer/n12 duplexer standalone operation. |
| CMCC | Issue 2-2: consideration of the blind scheme for inter-band TDD EN-DC PC2 Option 2: Do not consider for TDD EN-DC PC2. The blind scheme should to be confirmed whether it is necessary to introduce for PC2 EN-DC FDD+TDD. It is not recommended that the blind scheme be discussed under this AI. |
| Qualcomm | Issue 2-1-1: Introducing a new capability for “only single switched UL” is a new concept that needs broader discussion. There was long discussion a year ago about SUO where the conclusion was that dual uplink should be the default except where some RF restrictions could make it difficult. In those cases, then the spec allowed SUO. And in fact, even for this device, the network might still schedule dual uplink but the performance is unspecified. Here, the concept is different in that only single uplink is supported, not that it is allowed. Issue 2-1-3 It is unclear what is meant by “consider restriction” or “do not consider restriction”. R4-2008083 seems to propose that the specification does not mention anything at all about potential form-factor restrictions but instead that the UE that is not capable simply does not report the new NS. We don’t think the specification should ignore the form factor restriction for most devices that cannot support two low-band antennas. Moreover, the idea of new NS and of the UE signaling an NS is not agreed either. Issue 2-2 Blind scheme is not receiving much support for FDD-TDD, so it doesn’t make sense to introduce it here now for TDD-TDD. It would be better to wait for the conclusion in the FDD-TDD discussion. |
| Samsung | Sub-topic 2-1: Issue 2-1-1: introduction of new capability “only single switched UL” Support Option 2, existing capability for single-UL transmission can be reused. Issue 2-1-3: consider ‘restriction due to form factor’ in the specification Restriction to form factor should remain Sub-topic 2-2: Issue 2-2: consideration of the blind scheme for inter-band TDD EN-DC PC2 Support Option 2 “Blind” scheme is still being discussed on the topic of PC2 FDD-TDD EN-DC, the benefits and impacts are not concluded yet. There is not much support to blind scheme in the open WI now, it is not appropriate to open the same discussion to a closed WI. We suggest to wait for the conclusion in FDD-TDD WI. |
| Huawei | Sub topic 2-1: Issue 2-1-1: introduction of new capability “only single switched UL” o Option 1: Needed (and associated with the TDM-pattern capability) We have the same view as Qualcomm. “Only single switched UL” is a new concept which is totally different from single-UL transmission. The existing capability for single-UL transmission can’t be used for “Only single switched UL”, or it will cause confusion from NW perspective and NBC issue since RAN4 give it new meaning. Issue 2-1-2: introduce time mask for UEs indicating IE [only supporting single switched UL] as proposed in R4-208084 o Option 1: Introduce the UE time mask as proposed in R4-2008084 The time mask for intra-band ENDC is introduced into the spec. ENDC combos which support “Only single switched UL” should follow the same requirements. Issue 2-1-3: consider ‘restriction due to form factor’ in the specification Option 2: Do not consider. There is no need to restrict the specific implementation. If UE don’t support this combination, they don’t have to report it. Firstly, the “form factor restriction” is ambiguous. We need to use accurate description in the spec. As we said, note 1 in table 7.3.2 from 38.101-1 can be considered as reference. Secondly, if ENDC combos are optional, we don’t have to restrict the specific implementation in the spec. Sub topic 2-2: Issue 2-2: consideration of the blind scheme for inter-band TDD EN-DC PC2 Option 2. Do not consider blind scheme for TDD EN-DC PC2 and the proposal does not get much support for FDD-TDD PC2 EN-DC. We don't think it is an attractive solution for EN-DC HPUE even from network perspective. |
| Ericsson | Sub-topic 2.1 Issue 2-1-1: Option 1, needed if SUO is the only option Issue 2-1-3: not clear how form factor would be covered in the specification (compare 31 dBm PC1 for Public Safety) Sub-topic 2.2 Issue 2-2: there are a lot of statements that the blind scheme does not provide any benefits for FDD-TDD EN-DC PC2. That a 4-6 dB increase of the SCG TDD UL power for PLTE = 22 dBm and common 25-40% max UL duty cycle without required CG coordination and reuse of standard EN-DC UE power control is not seen as beneficial is beyond our understanding. The blind scheme would work equally well as a baseline for TDD EN-DC PC2, it was put under this agenda item as an “enhancement” for discussion since it was not considered during the TDD-TDD HPUE work item. The network would be aware of the minimum UE EN-DC output power given the U/D patterns on the two CGs, no CG coordination required. It appears beneficial. Just an idea. There are concerns about configuring PLTE < 23 dBm that is standard for FDD-TDD PC3. This is not always necessary for the TDD-TDD blind scheme if the UL duty cycles are small. However, in practice it would be set for LTE PC3 to make sure that the NR TDD is not dropped when the UE is in fallback (PC3) also for the duty-cycle scheme. |
| Skyworks | 2-1-1: SUO only is not a new concept it is already used for intra-band EN-DC for n66,n12,n5… It does not need new signaling since SUO signaling already exists, and there is nothing that the networks needs to know other than scheduling with SUO TDM pattern which is not different than for the other SUO only cases. Note that DC_12_n71 has MSD issues due to IMD3 so TDM pattern is needed whatever. 2-1-3: On form factor: even with SUO, it is essential for this combination that two good LB antennas are available. Even if some companies claim that a triplexdr is feasible it won’t be able to meet the same performance than each duplexer which are the two critical LB cases. Also Band 29 protection would not be granted. Since the proponent of this combination has already agreed that these limitaions are real and acceptable for the type of device they want to deploy we do not understand why this should even be discussed. |
| CHTTL | Sub topic 2-1: Issue 2-1-1: introduction of new capability “only single switched UL” Option 2 (not needed) Current signalling can already allow UE to indicate not supporting simultaneous UL transmissions; it is not clear why additional signalling is needed. And this DC_12_n71 is proposed to support single switched UL only, so basically the network will assume all UE support DC_12_n71 with single switched UL only, not sure what is going to differentiate by this new capability. |
| Qualcomm | Issue 2-1-1: Sorry, our previous comment was not clear. We do not support option 1. We do not think that this new capability is needed without first having a broader discussion. As we tried to comment, there was a long discussion about this previously with the conclusion that dual uplink should be the predominant UE capability. This new SUO capability seems to go against that. |
| Apple | Issue 2-1-1: Option 2, i.e. no additional signaling needed. This has already be specified for several other combinaitons Issue 2-1-2: Option 3. We have used SUO only for multiple other combinations. There is no need for a specific time mask for 12+71 Issue 2-1-3: Option 1: Yes, restriction of the form factor is required. We do not see the feasibility of this band combination in a formfactor of a Smartphone or even a tablet, since two good antennas for 600MHz will be a problem and the feasibility of such a triplexer is not given for small form factor mass production |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| TS 38.101-1 CR 311 (R4-2006451) | Company A |
|  | Company B |
| TS 38.101-1 CR 336 (R4-2006997) | Company A |
|  | Company B |
|  | Huawei: 1. It seems that we need only one official cat-B CR for one spec in one WI. 6997 and 7799 can be merged. 2. 5.5B is the second level title instead of third level title. |
| TS 38.101-1 CR 362 (R4-2007799) | Company A |
|  | Ericsson: for Huawei, there are square bracket for one scheme for dynamic power sharing that might not be kept in NR-DC 38.213 (pending decision in RAN1) CRs could be merged if desirable. |
|  | Huawei: 1. It seems that we need only one official CR for one spec in one WI. 6997 and 7799 can be merged. 2. There are stilll some backets in the CR. |
| TS 38.101-3 CR 372 (R4-2008084) | Intel: No need to introduce a specific UE time mask for DC_12-n71. ZTE: Same view with intel. |
|  | Ericsson: acceptable to include a time mask for adjacent UL bands. |
|  | CHTTL: would like to clarify that is this CR supposed to cover all the DC_12_n71 requirements? - Since the delta T, and the Maximum output power for DC are missing in this CR. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Tentative agreements: a. (issue 2-1-1), for DC_12-n71, keep the existing SUO capability (and the association with the TDM patterns) b. (issue 2-1-3) consider a form-factor restriction (can only be informative, e.g. a note) Candidate options: Recommendations for 2nd round: discuss the form-factor restriction (topic). Agreement on the time mask not possible. |
| Sub-topic#2-2 | Tentative agreements: none Candidate options: Recommendations for 2nd round: no further discussion under this AI. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on single UL for DC-12-n71 | Huawei |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| TS 38.101-1 CR 311 (R4-2006451) | “Agreeable” |
| TS 38.101-1 CR 336 (R4-2006997) | “Agreeable” Moderator comment: a. it was proposed to merge with 7799 into a feature CR. While indeed a relevant comment, the moderator proposes not to create any revisions. There is no overlap between the two CRs, all relevant clauses covered by the two CRs b. the MCC can (and will) correct the formatting error (title of 5.5B) when implementing the CR |
| TS 38.101-1 CR 362 (R4-2007799) | “Agreeable” Moderator comment: a. it was proposed to merge with 6997 into a feature CR. While indeed a relevant comment, the moderator proposes not to create any revisions. There is no overlap between the two CRs, all relevant clauses covered by the two CRs |
| TS 38.101-3 CR 372 (R4-2008084) | “Not pursued” Moderator comment: no consensus, several companies against, but some support. |

## Discussion on 2nd round (if applicable)

Sub-topic #2-1 discussed further in the 2nd round, with focus on issues 2-1-2 and 2-1-3. It is assumed that it is agreed not to introduce a new capability for SUO. Further agreements to be captured in the WF in R4-2008459.

| Company | Comments |
| --- | --- |
| Skyworks | Sub-topic#2-1: About R4-2008459, we believe WF should focus on a list of agreements rather than a list of FFS. Here are comments for slide 4: - We are ok to keep " It’s mandatory to use only single switched uplink for DC_12_n71 in Rel-16." - About “FFS whether to introduce a new NS for only single switched uplink operation for ENDC band combinations". We don’t see the need for a new NS. Release-16 SUL only combinations have been agreed using the legacy Release 15 signaling. We propose to delete this bullet, - About "FFS FFS whether to restrict the specific implementation for DC_12_n71 in the specification" is a bit vague. As discussed in round 1, we propose to replace with previously discussed wording “DC_12_n71 is applicable only to non-form-factor constrained device use cases (e.g. customer premise or vehicle-mounted devices)”. - About “FFS whether a general solution can be considered for only single switched UL for EN-DC band combinations, which can also be used for existing band combinations, e.g. DC_3_n3, DC_7_n7, DC_48A_n48A, DC_66A_n66A. We propose to replace with “In coexistence studies, companies are encouraged to study the impact on MSD, band protection and potentially regulatory requirements even in the case when SUL is mandatory.” For example, MSD has been defined for DC_(n)12AA and DC_(n)5AA at previous meeting even though SUL only operation is supported. We suggest to perhaps document the list of EN-DC combinations for which only SUL operation is supported in an Annex. In which case - - DC_3_n3 should be removed from the list since SUL only is restricted to Release 15 devices, - DC_(n)12AA and DC_(n)5AA should be added to the list as it has been agreed at previous meeting that SUL only operation is supported on these combinations. …. Others: |
| CHTTL | 1. For the first bullet, if DC_12_n71 cannot be completed in this meeting, then it will be Rel.17. 2. We suggest to remove the third bullet "FFS whether a general solution ......" completely, and focus on the DC_12_n71 in this WF. Even the new NS for this DC_12_n71 is not agreeable, as we still don’t understand why a new NS is needed, it is premature to expand the case to other cases. Also it is not clear what needs to be further study for the combination you listed, they are already completed in the spec. |
| Apple | Sub-topic#2-1-1 Concerning the WF R4-2008459 we have the following comments: - We think that the first sentence “It’s mandatory to use only single switched uplink for DC_12_n71 in Rel-16.“ is really essential as it is now and should not be softened as proposed in the email thread, since RAN4 doesn’t do work to specify dual simultaneous UL, therefore UEs cannot use dual UL. - Then the WF lists a lot of FFS items, some of them have already been discussed and discarded in the first round, so it would be better to remove the FFS items and concentrate on the agreements - The sentence above this section says “It is assumed that it is agreed not to introduce a new capability for SUO.”. However, the initial version of the WF R4-2008459 lists this as FFS. It should be corrected to have this as an agreement, not FFS. A capability signaling as proposed doesn’t make sense, the UE can anyway signal it supports single UL. The problem here is that it needs to be clarified in the spec that the spec and the UE don’t support dual UL. This is already stated in the first sentence “It’s mandatory to use only single switched uplink for DC_12_n71 in Rel-16.” of the WF and needs to be added into the spec later on. If RAN4 would want to allow also dual simultaneous UL, there would a lot of additional work be needed like specification of A-MPR, MPR, MSD etc. Therefore currently the only way is to restrict this combination to single UL. - The last bullet discusses a general solution and then lists a lot of other band combinations. A general solution for what problem? We should discuss completing the issues for this combination.. This bullet can be removed. Conclusion: - Add the conclusion there is no need for a new signaling - Remove all the FFS bullets Sub-topic#2-1-3 - We need to agree a form factor restriction. Currently there is no way to implement this band combination in form factor restricted devices of tablet or smaller size, as there are no data on wideband triplexers including insertion losses, size and availability from multiple filter vendors, nor seems it possible to have two or more 600MHz antennas in such a form factor - We need to exclude this combination and requirements from being specified for such UEs. - It would be good to add a bullet on form factor restriction to the WF, although ti is not SUO related but 12+71 related. |
| Huawei | For DC_12_n71, if it can’t be completed in Rel-16, it has to be defer to Rel-17. Sub-topic#2-1-1 - We raise an issue about whether the existing NS IE “singleUL-Transmission” can be reused for “only single switched uplink” for ENDC combinations. And the conclusion “UE is mandated to support operation in dual and triple uplink mode for EN-DC configuration in NR FR1” are shown in Annex I from 38.101-3 in Rel-15. For the existing IE “singleUL-Transmission”, dual uplink should be the default. In those cases, then the spec allowed SUO. And in fact, even for this device, the network might still schedule dual uplink but the performance is unspecified. However, for “only single switched uplink”, the single uplink should be the default. And the network can’t schedule dual uplink for DC_12_n71. It’s totally different meaning and understanding. I also wonder whether RAN1 or RAN2 agree to reuse the existing IE “singleUL-Transmission” for “only single switched uplink”. - Right now, more and more ENDC combinations are expected to use this feature, such as DC_(n)12AA and DC_(n)5AA. We can’t clarify the only SUO in RAN4 spec by adding a note. It makes the spec unstable and confusion. From network perspective, it doesn’t make sense that UE doesn’t report only SUO capability instead of using exceptional manner. Sub-topic#2-1-3 For a form factor restriction, we don’t know what the specific implementation is. Thus, further discussion and analysis is needed. |
| Intel | On WF (R4-2008459) - We are okay to use only single switched uplink for DC_12_n71 in Rel-16 if there is consensus. If so, then other similar DC combinations, i.e., DC_5_n5, DC_7_n7, DC_12_n12, and DC_66_n66, also need to be considered as only single switched UL, and dual UL operation is no longer available for those bands. For the signaling, RAN4 would need further discussion because the existing signaling is allowing single UL exception with default of dual UL. |
| Dish | On WF -8459: -Feasibility of the triplexer should be one of the bullets |
| ZTE | - For bullet 1. ‘It’s mandatory to use only single switched uplink for DC_12_n71 in Rel-16.’ if single UL is mandatory, does it mean this configuration cannot be operated as ENDC operation? if it is yes, it seems these two bands should be removed from the table since these two bands cannot suitable for ENDC. - For the bullet ‘FFS whether to restrict the specific implementation for DC_12_n71 in the specification.’ what’s is the specific implementation mean? How to refelct it in the specification? In our understanding, specification is transparency for implentation. - For the other bullet, we share similar with with Apple, CHTTL, i.e it is no need to introduce new NS and FFS should be removed. Also this WF should be only focus on the DC_12_n71, it is premature to expand to other cases. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| WF R4-2008459 | “Agreeable” |

# Topic #3: Cell- and UE-specific P-Max for FR2

Main technical topic overview: feasibility and need for cell-specific and/or UE-specific P-Max limitations. RAN4 has received an LS from RAN2 regarding support in RAN4 specifications of UE-specific P-Max information elements/fields in the Rel-16 version of 38.331.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006566 | Intel Corporation | Title: Pmax in FR2 (For Discussion) Observation 1: Limiting peak EIRP by introducing P-max looks possible but needs careful design Pumax lower bound. Existing TRP inequality can be automatically met. Observation 2: Limiting maximum TRP by introducing P-max but not modifying Pumax inequality seem not possible. The impact on Pumax needs also to be considered and complexity seems significant. |
| R4-2006585 | Nokia, Nokia Shanghai Bell | Title: On p-Max for FR2 (for Approval) Proposal 1: It is proposed to introduce p-Max in FR2 for interference suppression purpose for operators to coordinate in unsynchronized network operation. Proposal 2: p-Max in FR2 is based on TRP metric. Proposal 3: The range of p-Max for FR2 is (-20..43) Proposal 4: Pcompensation shall be based on TRP metric that is compensated by p-Max. |
| R4-2006586 | Nokia, Nokia Shanghai Bell | Title: introduction of p-Max for FR2 (CR to 38.101-2) Reason for change: It is not possible to limit the UE output power in network deployment, when interfrence coordination is needed among operators for unsynchronized network operation. |
| R4-2006587 | Nokia, Nokia Shanghai Bell | Title: draft Reply LS on power control for NR-DC Draft Reply LS to R4-2003363/R2-2000294 |
| R4-2006828 | MediaTek Inc. | Title: Views on P-max for FR2 in Rel-16 (for Approval) Proposal 1: P-max shall be determined by the peak EIRP. Observation 1: When P-max is close to and less than PPowerclass, it is seemingly an absolute power control requirement but with much less tolerance than absolute power tolerance as defined in TS 38.101-2 [4] Clause 6.3.4.2. Observation 2: If FR2 absolute power tolerance as defined in TS 38.101-2 Clause 6.3.4.2 is allowed for P-max, the P-max requirement would become less meaningful. Proposal 2: RAN4 do not introduce P-max requirement in Rel-16 UE specifications. |
| R4-2007917 | Ericsson, Sony | Title: Introduction of cell-specific and user-specific P-Max for FR2 (CR to 38.101-2) Reason for change: For FR2 it is not possible to limit the UE output power by cell-specific indication if needed in deployments. The radiated power (EIRP) in the direction of maximum cannot be controlled but the TRP is not larger than the total conducted power that can be limited. It is also possible to configure a UE-specific P-Max as given by the parameter P-NR-FR2 in the PhysicalCellGroupConfiguration, this can be done also without DC operation (an MCG is always configured). The parameter P-UE-FR2 is only relevant when there are more than one cell group configured (NR-DC in FR2). |
| R4-2008053 | Qualcomm Incorporated | Title: p-max feasibility for Fr2 (for Approval) Observation 1: Antenna radiation pattern consists of multiple lobes and nulls where antenna gain varies. Observation 2: UE does not have knowledge of the current antenna radiation pattern in field operation Observation 3: UE cannot know what is the maximum EIRP at a given transmission occasion. Proposal 1: P-max is not applied to EIRP in UE Proposal 2: If p-max is added to the rel-16 UE requirements, the p-max limitation is applied to the TRP and lower limit of the PUMAX. Proposal 3: Motivation for p-max limitation shall be clarified before agreements on UE implementation |
| R4-2008265 | vivo | Title: Further discussion on the necessity of p-UE-FR2 (for Approval) Observation 1: UE antenna gain would be greatly impacted by the blockage of hand or other objects. Observation 2: It is difficult for UE to do precise estimation of the type and angle of blocking object, not to say accurate estimation of the impacted antenna gain which would be even more complicated. Observation 3: Both EIRP and TRP are facing similar feasibility problems, though TRP may be somewhat easier. Observation 4: In all, it is believed that neither EIRP nor TRP is technically feasible to be accurately predicted by UE in the actual field, which would make p-Max not that meaningful. Proposal: Not to introduce Pmax for FR2 in RAN4 Rel-16 spec unless the feasibility of fairly accurate UE estimation of TRP/EIRP in different blockage scenarios can be proved. |
| R4-2008266 | vivo | Title: [Draft] Reply LS on power control for NR-DC Draft Reply LS to R4-2003363/R2-2000294 |
| R4-2008272 | NTT DOCOMO | Title: Introduction of P-max in FR2 [Draft] (for Approval) Proposal 1: Take Option C + Option 3: - Introduce P-max in both TRP and EIRP metric - The step size of P-max values is set roughly so that UE can implement both TRP and EIRP limitation. The exact value of step size is FFS. Proposal 2: Introduce FR2 P-max in both RRC reconf and SIB. Proposal 3: How to handle legacy UEs that do not understand P-max should be studied. - Possible options: - NSA case (RRC connected mode) - Alt 1: Introduce UE capability of informing whether UE support FR2 P-max. - UE capability is transferred through LTE before FR2 is configured. - Legacy UE: NW do not configure FR2 - New UE: NW configure FR2 and new UE connect under P-max limitation - SA case (Idle mode) - Alt 1: Define NS values (NR-NS-PmaxValue) associating with P-max. - Define NS_20xP with additional P-max in TS 38.101-2. NW using P-max signals NS_20xP - Legacy UE: Cell barred since legacy UE do not understand NS_20xP. - New UE: Connect under P-max limitation - Alt 2: Use additional P-max in existing NS - NOTE: Alt 2 can work only if all legacy UE can understand additional P-max in NR-NS-PmaxValue. |

## Open issues summary

### Sub-topic 3-1 Feasibility of P-Max limitation if introduced

Sub-topic description: need for and feasibility of P-Max limitations. Deployment scenarios motivating P-Max limitation(s), the feasibility of EIRP and/or TRP restrictions from a UE implementation and minimum requirement standpoints are addressed.

Open issues and candidate options before e-meeting:

Issue 3-1-1: Need for P-Max limitation

- Proposals

  - Option 1: P-Max needed as motivated in R4-2006586 and R4-2008272

  - Option 2: P-Max not needed and/or not meaningful

- Recommended WF

  - TBA

Issue 3-1-2: Feasibility of EIRP and TRP upper bounds if P-Max needed

- Proposals

  - Option 1: EIRP limitation impossible (from a UE implementation standpoint)

  - Option 2: TRP restriction impossible (from a UE implementation standpoint)

- Recommended WF

  - TBA

Issue 3-1-3: Cell-specific and UE-specific P-Max if introduced

- Proposals

  - Option 1: Cell-specific and UE-specific

  - Option 2: Cell-specific only

  - Option 3: UE-specific only

- Recommended WF

  - TBA

### Sub-topic 3-2 Handling of legacy UEs if P-Max limitation introduced

Sub-topic description: how to handle legacy UEs not recognizing P-Max indications (if at all).

Open issues and candidate options before e-meeting:

Issue 3-2: Handling of legacy UEs

- Proposals

  - Option 1: Handling as proposed in R4-2008272 (Proposal 3)

  - Option 2: No need to consider legacy UEs

- Recommended WF

  - TBA

### Sub-topic 3-3 Reply LS to RAN2

Sub-topic description: contents of a Reply LS to RAN2 (and when to send)

Open issues and candidate options before e-meeting:

Issue 3-3: Reply LS to RAN2

- Proposals

  - Option 1: Send LS as proposed in R4-2006587 (introduce P-Max in Rel-16)

  - Option 2: Send LS as proposed in R4-8266 (do not introduce P-Max in Rel-16)

  - Option 3: Revise one of the proposed draft Reply LS (state preferred)

  - Option 4: Send a Reply LS at a future meeting

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Intel: | Sub topic 3-1: Issue 3-1-1: Need for P-Max limitation Option 2: P-Max not needed and/or not meaningful in Rel-16 time frame Issue 3-1-2: Feasibility of EIRP and TRP upper bounds if P-Max needed If only limit upper bounds (not lower bounds limitation), both EIRP and TRP are possible. Issue 3-1-3: Cell-specific and UE-specific P-Max if introduced Option 2: Cell-specific Sub topic 3-2: Issue 3-2: Handling of legacy UEs Option 2: No need to consider legacy UEs Sub topic 3-3: Issue 3-3: Reply LS to RAN2 Option 2: Send LS as proposed in R4-8266 (do not introduce P-Max in Rel-16) |
| OPPO | Issue 3-1-1: Need for P-Max limitation Option 2. We see the motivation of introducing Pmax for FR2, but as analyzed by papers this meeting, the feasibility in technical and also in production line needs to be solved first otherwise it is not meaningful to discuss whether it is needed or not. Issue 3-1-2: Feasibility of EIRP and TRP upper bounds if P-Max needed If we only talk about lowering UE power below certain power level and regardless how lower the power is below the Pmax limit then the TRP is easier than peak EIRP, however, this has large impact on UE performance. For example, in order to always below the power level, then UE needs to apply the worst antenna sphere efficiency which actually will be far from necessary considering the peak EIRP in most cases is the real factor causes impact to other system. And UE in global peak EIRP antenna pattern usually is not the max TRP pattern. So in total, we do not think it is proper to only talk about which one is possible regardless of impact to UE performance. Issue 3-1-3: Cell-specific and UE-specific P-Max if introduced Option 2 if introduced. |
| MediaTek | Issue 3-1-1: Need for P-Max limitation We need to understand what is the real concern on the network side. What kind of interference are we referring to? How do we test this requirement if P-Max limitation would be introduced? Issue 3-1-2: Feasibility of EIRP and TRP upper bounds if P-Max needed If only upper bounds are concerned, both EIRP and TRP look to be feasible. However, without the lower bound limit for Pumax, this requirement would not be so meaningful. UE can choose to limit its P-max to a very low value which may cause UE to fail connecting to the network due to insufficient output power. Issue 3-1-3: Cell-specific and UE-specific P-Max if introduced Cell specific if P-max would be introduced. Issue 3-2: Reply LS to RAN2 Option 2: Send LS as proposed in R4-8266 (do not introduce P-Max in Rel-16) |
| Qualcomm: | Issue 3-1-1: 6586 is a CR. 8272 is not very clear what is the exact limit in hospitals so how do we avoid the situation that UE’s are forced to implement expensive limitation method and it is still not enabling those deployments? Also intra-frequency inter-cell interference is really a TRP issue os that dfoe snot justify EIRP. We would like to work on this but we would really like to understand what is the actual need. So for now we say option 2, No. Issue 3-1-2: Option 1, EIRP is impossible. TRP maybe do-able. Issue 3-1-3: For us, it does not matter if the value is cell or UE specific, p-max mechanism is the same in UE. Issue 3-2: We did not understand how would the NS really work in this case. How do the legacy UE’s implement new NS? Issue 3-2: (wy two issue 3-2’s?) Option 4 for now. We do not have enough information to do a decision. |
| Nokia | Issue 3-1-1: Option 1. This will bring a deployment flexibility for unsynchronized network operation. Issue 3-1-2: EIRP control may be difficult, though it is not impossible. TRP should be more feasible than EIRP. Issue 3-1-3: The main purpose is to introduce cell specific p-Max, but it can be easily extended to UE specific as already specified in FR1. So, we support option 3. Issue 3-2: There is no need of special handling of legacy UEs. The purpose of introducing p-Max from Rel-16 is to provide Network capability to control the aggregated interference. Network can still control Rel-15 UE by redirecting to other frequency bands. Issue 3-3: Option 1. |
| Samsung | Sub topic 3-1: Issue 3-1-1: Need for P-Max limitation Option 2: P-Max not needed and/or not meaningful at least in Rel-16 time frame. Further discussion is needed about the motivation and demands of introducing P-Max to FR2 in RAN4. Issue 3-1-2: Feasibility of EIRP and TRP upper bounds if P-Max needed TRP is more feasible than EIRP, however, it should not be determined based on feasibility but depending on the detailed motivation and demands of network side. Issue 3-1-3: Cell-specific and UE-specific P-Max if introduced Option 2: Cell-specific if introduced Sub topic 3-2: Issue 3-2: Handling of legacy UEs Option 2: No need to consider legacy UEs Sub topic 3-3: Issue 3-3: Reply LS to RAN2 Option 2: Send LS as proposed in R4-8266 (do not introduce P-Max in Rel-16) |
| Huawei | Issue 3-1-1: Need for P-Max limitation Option 2. TRP as Pmax can not reach the target from avoiding interference. EIRP as Pmax, if only peak direction, meaningless. Issue 3-1-2: Feasibility of EIRP and TRP upper bounds if P-Max needed For option 1, it is impossible from implementation perspective if the whole sphere is required on each configured point. Meanwhile, the tolerance is not easy to define. For option 2, if UE restrict TRP, EIRP is not ensured. Then network performance is highly impacted. So, both options are not realistic. Issue 3-1-3: Cell-specific and UE-specific P-Max if introduced Disagree to introduce Pmax, regardless of UE specific or cell specific. Issue 3-2: Handling of legacy UEs Disagree to introduce Pmax in Rel-16 Issue 3-2: Reply LS to RAN2 Option 2 or Option 4. If we cannot reach agreement on introducing it in Rel-16 in RAN4, and LS should be sent to RAN2 request on remove the CR on applying FR2 Pmax. |
| Ericsson | Issue 3-1-1: Option 1, motivated as stated in 6586 and 8272. Issue 3-1-2: Option 1 (TRP limit is possible and would also reduce the EIRP) Issue 3-1-3: Option 1, both a cell-specific and UE-specific TRP limitation Issue 3-2: Option 2 (TBD) Issue 3-3: Option 4 most likely. |
| vivo | Issue 3-1-1: Need for P-Max limitation Option 2. No. It is still not clear what the scenario that is actually needed. Issue 3-1-2: Feasibility of EIRP and TRP upper bounds if P-Max needed We think both EIRP and TRP have feasibility problem. TRP might be easier, but considering the nature of interference, TRP might be less meaningful. Issue 3-3: Reply LS to RAN2 Option 2 or Option 4. (Do not introduce P-Max in Rel-16) |
| NTT DOCOMO, INC. | Issue 3-1-1: Need for P-Max limitation - Option 1: P-Max needed as motivated in R4-2006586 and R4-2008272 Issue 3-1-2: Feasibility of EIRP and TRP upper bounds if P-Max needed We understood that we need to discuss how to introduce P-max in Pcmax equation: at least there is an issue that it is difficult to keep lower limit of EIRP while TRP is limited. Our alternative is lower limit of EIRP and higher limit of EIRP is decreased by the delta between max TRP – P-max. For example, assuming PC3 UE and P-max is set as 20dBm, then the delta between max TRP-P-max=23dBm-20dBm=3dB. Therefore lower limit of EIRP is set as 22.4dBm -3dB=19.4dBm(if there is no MPR) and upper limit of EIRP is set as 43dBm-3dB=40dBm. Issue 3-1-3: Cell-specific and UE-specific P-Max if introduced - Option 1: Cell-specific and UE-specific We prefer option 1 as same with FR1. Issue 3-2: Handling of legacy UEs - Option 1: Handling as proposed in R4-2008272 (Proposal 3) The intention is how legacy UE is not allowed to transmit under NW which need P-max limitation. >Intel, OPPO, MediaTek We understood that we need to discuss how to introduce P-max in Pcmax equation: at least there is an issue that it is difficult to keep lower limit of EIRP while TRP is limited. How about your view on out alternative described above in issue 3-1-2? >Qualcomm Issue 3-1-1 and 3-1-2: We agree that inter-cell interference is a TRP issue. P-max limitation for TRP or EIRP is depending on use case. For feasibility of implementation, is there possibility to implement P-max for EIRP considering the value range of P-max is limited and the step size of P-max is set as rough granularity. Issue 3-2 The intention of using NS is that legacy UE not supporting P-max is not allowed to transmit under NW which need P-max limitation. We expect that legacy UEs don’t understand new NS and stop its transmission. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| TS 38.101-2 CR 159 (R4-2006586) | OPPO: Need to wait for the conclusion. |
|  | Intel: 1) we need to decide whether or not we need P_max. 2) If introduced, TRP or EIRP? 3) If TRP, the current CR is not acceptable since with P-max in TRP and no modifications in Pumax correspondingly, it is not possible to meeting existing Pumax inequality with reduced TRP limit. |
|  | MediaTek: Agree with Intel, the Pumax lower limit cannot be kept the same. It is better not to rush for a quick decision if we do not have a clear picture on how P-max can be introduced in FR2. |
|  | NTT DOCOMO, INC.: We would like to discuss our alternative described above in issue 3-1-2 in out comments. |
| TS 38.101-2 CR 192 (R4-2007917) | OPPO: Need to wait for the conclusion. |
|  | Intel: 1) we need to decide whether or not we need P_max. 2) If introduced, TRP or EIRP? 3) If TRP, the current CR is not acceptable since with P-max in TRP and no modifications in Pumax correspondingly, it is not possible to meeting existing Pumax inequality with reduced TRP limit. |
|  | MediaTek: “The lower bound on the PUMAX,f,c does not apply if the UE is configured with a PEMAX,f,,c such that PEMAX,f,c + 10 ≤ TRPmax or a PNR such that PNR + 10 ≤ TRPmax.” would allow UE to set its P-max to a very low value to simply pass the compliance test which however may cause UE to fail connecting to the network due to insufficient output power. |
|  | NTT DOCOMO, INC.: We would like to discuss our alternative described above in issue 3-1-2 in out comments. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Tentative agreements: a. (Issue 3-1-1): introduce a P-Max restriction (an operator has motivated a need for use cases, only vendors claim there is no need) b. (Issue 3-1-2): in terms of TRP (no company considers this “impossible” technically) c. (Issue 3-1-3): leave this for now, RAN2 is only asking about UE-specific (but from a RAN4 perspective no difference between a cell-specific and UE-specific limitation) Candidate options: a. (Issue 3-1-1): do not introduce a P-Max if not possible (no Reply LS to RAN2 at this meeting, discussions can continue at the next meeting and vendors explain further why it is not needed or not useful) Recommendations for 2nd round: discuss a possible limitation in terms of TRP (topic and WF) |
| Sub-topic#3-2 | Tentative agreements: none Candidate options: Recommendations for 2nd round: no further discussion at this meeting, handling legacy UEs is pending decision on sub-topic #3-1 |
| Sub-topic#3-3 | Tentative agreements: postpone the Reply LS to RAN2 to the next meeting Candidate options: Recommendations for 2nd round: no action needed at this meeting. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on a P-Max limitation for FR2 in terms of TRP How to implement the TRP in the Pcmax sub-clause and the applicable limits. The “need” for the restriction and its introduction can be TBD. | Qualcomm |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| TS 38.101-2 CR 159 (R4-2006586) | “Not pursued” |
| TS 38.101-2 CR 192 (R4-2007917) | “Not pursued” |

## Discussion on 2nd round (if applicable)

Sub-topic #3-1 discussed further in the 2nd round. Agreements to be captured in the WF R4-2008458.

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| vivo | Sub topic 3-1: We keep our altitude that P-max is not needed in Rel-16. The feasibility is also questionable, even for TRP. |
| Intel | Subtopic 3-1: We don’t agree to moderator’s conclusion to consider P-max in TRP now. Since majority companies showed the concerns on both TRP and EIRP. Limiting TRP is not a dominate preference from companies. From interference control/coexistence perspective, EIRP matters. The FR2 ACLR requirements are derived based coexistence study based analog beam forming, i.e. directional beam pattern with EIRP metric. As we commented in the 1st round and also in our paper R4-2006566, EIRP at peak direction (boresight) can be controlled with less calibration effort, and it is also aligned with coexistence study. Limiting TRP has significant implication on EIRP handling which is not well studied by companies. So both EIRP and TRP need further study. |
| Samsung | Sub topic 3-1: We share the same view as vivo and Intel. Based on majority companies’ view in first round discussion, it is difficult to get tentative agreement to introduce P-Max as moderator concluded. For issue 3-1-2, TRP seems more feasible than peak EIRP with precondition that if P-max is needed. On the other hand, the impact of limiting TRP is higher than limiting EIRP. Further study is needed and it is not needed to introduce in Rel-16. |
| Qualcomm | One company justifies this by a need but does not reveal what is the criteria, i.e. what limit satisfies the original requirement? Same company inisist on EIRP based limit and Japanese regulation has antenna gain limit (implies EIRP need) but vendors say only TRP is feasible for implementation needs. Vivo above says will not agree to p-max in rel-16. Seems difficult to agree anything. |
| OPPO | Same as vivo, intel and Samsung. |
| Nokia | We support moderator’s summary. |
| NTT DOCOMO, INC. | In our view, at least we would like to continue to discuss it since there are several topics found in this meeting that we need to discuss such as how to introduce in Pcmax equation and how to handle legacy UEs. Although we discussed P-max at the end of Rel-15, but we have not been discussing in Rel-16 time frame. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| WF R4-2008458 | Withdrawn |