3GPP TSG-RAN WG4 Meeting #95-e	R4-2009043

Electronic Meeting, 25 May - 5 June, 2020

Agenda item:			6.18.1

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [95e][323] NR_perf_enh_Demod_UE

Document for:	Information

Introduction

This email thread discusses the NR Rel-16 UE performance requirements in agenda 6.18.1.

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round: Invite companies to review the recommended WF in section 1~5, and provide comments (if any) in section 1.3, 2.3, 3.3, 4.3, 5.3 and 6.3.

- 2nd round:

  - Focus on the WFs and the draft CRs.

  - For the WFs, check if the tentative agreements in the 1st round summary are agreeable, and try to make down-selection on candidate options. 5 sub-threads on WFs:

  - [95e][323] NR_perf_enh_Demod_UE - draft WF R4-2008837 (Release independent aspect, led by Huawei)

  - [95e][323] NR_perf_enh_Demod_UE - draft WF R4-2008838 (CA normal PDSCH, led by Intel)

  - [95e][323] NR_perf_enh_Demod_UE - draft WF R4-2008846 (PMI reporting, led by Ericsson and Samsung)

  - [95e][323] NR_perf_enh_Demod_UE - draft WF R4-2008848 (Power imbalance, led by NTT DOCOMO)

  - [95e][323] NR_perf_enh_Demod_UE - draft WF R4-2008849 (CA CQI reporting , led by China Telecom)

  - 2 sub-threads on draft CRs:

  - [95e][323] NR_perf_enh_Demod_UE - draft CR R4-2008842 (FR1 4Rx CA normal, led by Huawei)

  - [95e][323] NR_perf_enh_Demod_UE - draft CR R4-2008843 (PMI reporting, led by Ericsson)

    - Discuss the 3 draft CRs (R4-2008843/4/5) on PMI reporting in the same sub-thread

Topic #1: General issue for UE requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006036 | China Telecom | Updated CR work split for NR performance requirement enhancement WI |
| R4-2007220 | Huawei, HiSilicon | Proposal 1: PMI reporting test for Rel-15 type II codebook can be release independent from Release 15. Proposal 2: - Rel-15 UE: there is a mature mechanism to ensure that only UEs compliant with related conformance requirements can indicate supporting the respective capabilities - Rel-16 UE: if needed, UE capability and test applicability rule can be introduced to indicate support a feature or not, but no additional features/capabilities shall be introduced to inform gNB that UE can fulfil the respective requirements. |

Open issues summary

Sub-topic 1-1: Updated CR work split

Issue 1-1: Updated CR work split

- Proposal (China Telecom, R4-2006036)

  - Based on the approved CR work split in R4-1915864, the CR responsibilities for UE CA CQI and power imbalance requirements have been added in the updated version in R4-2006036.

- Recommended WF

  - Approve the updated CR work split in R4-2006036.

Sub-topic 1-2: Release independent issue

Issue 1-2-1: Release independent issue for type II PMI

- Agreement in RAN4 #94e-bis (R4-2005545, WF)

  - PMI reporting requirements for Rel-15 type II codebook

    - Option 1: Release independent from Rel-15

    - Option 2: Not release independent from Rel-15

  - CA CQI reporting requirements

    - Delay the discussion after RAN4 decides the specific test scopes for CA CQI reporting requirements

- Proposals

  - PMI reporting requirements for Rel-15 type II codebook

    - Option 1: Release independent from Rel-15 (Huawei)

- Recommended WF

  - Taking into account companies’ views in the recent two meetings, can we agree with option 1?

Issue 1-2-2: Requirements applicability rule / additional capabilities

- Agreement in RAN4 #94e-bis (R4-2005545, WF)

  - For all topics under NR performance requirement enhancement WI

    - Whether requirements applicability needs to be defined

      - Option 1: In general, Rel-15 UEs are already expected to support the respective features but the performance may not be guaranteed. In case the requirements are defined from Rel-15 it is important to ensure a mechanism that only UEs compliant with the newly defined conformance requirements can indicate the respective capabilities. In case the requirements are defined from Rel-16, additional features/capabilities shall be introduced to inform gNB that UE can fulfill the respective requirements.

      - Option 2: Define test applicability rule if needed, no additional features/capabilities needs to be defined

- Proposals

  - Option 1 (Huawei)

    - Rel-15 UE: there is a mature mechanism to ensure that only UEs compliant with related conformance requirements can indicate supporting the respective capabilities

    - Rel-16 UE: if needed, UE capability and test applicability rule can be introduced to indicate support a feature or not, but no additional features/capabilities shall be introduced to inform gNB that UE can fulfil the respective requirements.

- Moderator’s observation

  - This is a general issue related to all the features which are introduced in Rel-15 but the requirements are defined in Rel-16.

- Recommended WF

  - Encourage companies to provide:

    - Views on the above options

    - Whether to discuss this issue in each individual WI, or alternatively, discuss together as a common issue in UE feature list thread.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| China Telecom | Issue 1-2-1: Release independent issue for type II PMI Support to agree option 1. Issue 1-2-2: Requirements applicability rule / additional capabilities For us, the motivation to introduce additional capabilities is not very clear. In addition, we think this is a common issue related to all the features which are introduced in Rel-15 but the requirements are defined in Rel-16, and can be discussed in UE feature list thread. |
| CMCC | Issue 1-1: Updated CR work split Agree to approve the CR work split Issue 1-2-1: Release independent issue for type II PMI Support option 1 Issue 1-2-2: Requirements applicability rule / additional capabilities In our view, this is a common issue that requirements are defined in Rel-16 but features are introduced in Rel-15. The first bullet about Rel-15 UE in option 1 is OK. The 2nd bullet about Rel-16 UE needs further clarification on what kind of UE capabilities can be introduced, since in our view, all the capabilities should be already introduced in Rel-15. We prefer to discuss in UE feature list thread, since we observe there are also other features have the same issue. |
| Qualcomm | Issue 1-2-2: Regarding Option 1 for Rel-15 UE, the tests in this WI are not defined in RAN5 or GCF yet. By the time, those tests get defined in RAN5 and GCF, there will be plenty of Rel-15 UEs in the market that may not pass these performance requirements, even if they support the feature. It may also be possible that all Rel-15 UEs supporting these features will pass the requirements. But, it is not clear how existing mechanism will ensure that the tests defined in this WI will be passed by all Rel-15 UEs supporting these features. Regarding Rel-16 UEs, it is not clear what kind of capabilities are needed since all the features in this WI already have UE capabilities defined in Rel-15 if those capabilities were needed. |
| Ericsson | Issue 1-2-2: This issue is also discussed in HST performance part, i.e. HST single tap scenario and multi-path fading scenario. Since LTE release independent requirements depends on RF capability or UE category, i.e., 4Rx/8Rx or CA/DC, NB-IoT/eMTC, it is clear the applicability. One exception is HST test case, but it is applicable when UE supports capability signaling. On the other hand, several features discussed in Rel-16 NR performance enhancement do not require capability signaling or the required capability has already been supported from Rel-15, such as 32Tx PMI or HST single tap/multi-path fading. We think these cases should be optional for Rel-15 UE, because some UEs already in the market support the feature e.g., 32 Tx ports but may not pass the tests defined in Rel-16 spec. Moreveor, no additional features/capabilities shall be introduced to inform gNB that UE can fulfil the respective requirements. |
| Intel | Issue 1-2-1/1-2-2: Our concern is that some Rel-15 UEs on the market may signal support of certain features which were not covered by Rel-15 requirements. For such UEs we cannot guarantee proper processing in case one of these (not tested) features is activated. It may lead to degradation of system performance due to incorrect processing. If we can ensure that such scenario is not practical and UE always signal support of features, which were verified, then requirements for Rel-15 features can be defined in realize independent manner. But, if this is practical case and UE can signal support of features, which were not verified, then it is better to define such requirements applicable from Rel-16. Also, we can introduce Rel-16 RAN4 feature and corresponding capability signalling to indicate that UE supports requirements for Rel-15 features. |
| docomo | Issue 1-2-2: Requirements applicability rule / additional capabilities The requirements in this WI are introduced in Rel.16 NR. However, all of the features in this WI are defined in Rel.15 NR. In this situation, we have no clear motivation to introduce the additional features/capabilities. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Topic#1: General issue | - Issue 1-1: Updated CR work split Tentative agreement: - Approve the updated CR work split in R4-2006036 (CMCC) - Issue 1-2-1: Release independent issue for type II PMI reporting requirements - Option 1: Release independent from Rel-15 (Huawei, CTC, CMCC) - Option 2: If there is a mechanism to ensure that UE always signal support of features, which were verified, then requirements for Rel-15 features can be defined in realize independent manner. Otherwise, it is better to have Rel-16 requirements. (Intel) Recommendations for 2nd round: Further discuss the candidate options above. - Issue 1-2-2: Requirements applicability rule / additional capabilities - Additional capabilities - For Rel-15 UE: - Tentative agreement: Not needed (Huawei, CMCC, CTC, E///, QC, DCM) - For Rel-16 UE: - Option 1: Not needed (CMCC, CTC, E///, QC, DCM) - CMCC, CTC: Can discuss together as a common issue in UE feature list thread - Option 2: Introduce Rel-16 RAN4 feature and corresponding capability signalling to indicate that UE supports requirements for Rel-15 features. (Intel) - Whether the existing mechanism will ensure that the tests defined in this WI will be passed by all Rel-15 UEs supporting these features - Option 1: It is not clear (QC, Intel) - Option 2: Not for all UEs, since some UEs already in the market support the feature (E///) Recommendations for 2nd round: Further discuss the candidate options above. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Way forward on release independent aspect for UE demodulation and CSI reporting requirements | Huawei, HiSilicon |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round

R4-2008837	Way forward on release independent aspect for UE demodulation and CSI reporting requirements

Type: other		For: Approval
					Source: Huawei

Abstract:

Discussion:

Moderator’s note: The WF is discussed in sub-thread [95e][323] NR_perf_enh_Demod_UE - draft WF R4-2008837 (led by Huawei).

- Issue 1-2-1: Release independent issue for type II PMI reporting requirements

  - Option 1: Release independent from Rel-15 (Huawei, CTC, CMCC)

  - Option 2: If there is a mechanism to ensure that UE always signal support of features, which were verified, then requirements for Rel-15 features can be defined in realize independent manner. Otherwise, it is better to have Rel-16 requirements.  (Intel)

[Huawei]: As per the first round discussion, companies’ main concern is that the requirements are defined in Release 16, but some UEs already available in the market that support the features may or may not pass these performance requirements, it is the truth that we cannot change, also RAN4 did not define performance requirements for some other Rel-15 features that UE may support, it is up to vendor to take the risk to implement feature but guarantee to pass the future requirement. Also this is not new for NR, LTE has the same issues. But to clearly address the concern from company, we are not sure if the following wording is acceptable?

  - PMI reporting requirements for Rel-15 type II codebook

      - Release independent from Rel-15 and is optional for Rel-15 UE

[CTC] We have one clarification question, here does the “optional” mean optional for a Rel-15 UE (i.e., up to UE declaration), or optional for a Rel-15 UE declaring the support the corresponding feature?

[Ericsson] our intention of ‘optional’ is Rel-15 UE capable of type-II PMI declare to satisfy the requirements introduced in Rel-16 spec.

[Intel] Does it mean that, on top of optional capability signalling introduced by RAN1, we will have UE declaration to decide whether to verify UE for this scenario or not? We suggest to consider it as candidate option, to keep this issue open for this meeting and to make conclusion in the next meeting.

[Apple] The purpose of option for Rel-15 UE is not clear. Would we have 2 capability - one for support of Type II and another for capability to meet the requirements?

[Huawei] Our intention is testing the requirements of Type II codebook defined in Rel-16 specification is optional for Rel-15 UE, we should not mandate all Rel-15 UEs to pass the related requirements. Actually it is same for all release independent requirements and same as did in LTE for those requirements specified as release independent in TS 36.307. I understand the declaration to support and test the requirements here is used for the real testing to allow UE to decide whether to choose run this test case, it is reflected as a test applicability rule that is captured in RAN5 specification as did for LTE in TS 36.521-2

[Huawei2] If company has concern on addition of the wording “is optional for Rel-15 UE”, I removed from uploaded v1 version, we can just follow the traditional approach on how to handle release independent requirements as did in LTE by default.

@Intel, Considering this has been discussed for several meetings, and all other agreed UE performance requirements discussed in this WI have been agreed to be release independent from Rel-15, it is better that we can conclude the left one in this meeting.

[Intel]: Based on our understanding, Type II PMI is only one optional Rel-15 feature in this WI. Therefore, we would like to keep this topic open for this meeting, taking into account that this issue does not block progress on discussion of requirements definition. We can also add note that “conclusion will be reached in the RAN4# 96-e meeting”. I hope our proposal is fine for everyone.

[Huawei]: We are fine to keep it open for this meeting.  I reverted back to the original two options and added the following note you proposed

- Issue 1-2-2: Requirements applicability rule / additional capabilities

  - Additional capabilities

    - For Rel-16 UE:

      - Option 1: Not needed (CMCC, CTC, E///, QC, DCM)

      - Option 2: Introduce Rel-16 RAN4 feature and corresponding capability signalling to indicate that UE supports requirements for Rel-15 features. (Intel)

[Huawei]: Based on the first round discussion, only one company think that it is necessary to introduce Rel-16 UE features/capability signaling, with the further clarification of “optional for Rel-15 UE”, is it acceptable for company:

No additional features/capabilities needs to be defined in RAN4 Rel-16 feature list to indicate that UE supports requirements for Rel-15 features

[CTC] Agree with the proposal. This is also discussed in UE feature list thread as a common issue.

Recommendation:		Agreeable

Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2008837 | agreeable |

Topic #2: UE	CA PDSCH requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006037 | China Telecom | TDD-FDD CA and TDD-TDD CA with different SCSs Proposal 1: For performance requirement definition: - For CA with different SCSs, define requirements for both 15kHz Pcell and 30kHz Pcell. - For FDD + TDD CA with 15 kHz SCS, define requirements for both FDD 15 kHz Pcell and TDD 15 kHz Pcell, or alternatively, only for TDD 15 kHz Pcell. Proposal 2: Select option 2 for the test applicability, i.e., - Option 2: If Pcell in both carriers are supported, configure TDD cell as Pcell in TDD-FDD CA, configure 15 kHz SCS cell as Pcell in TDD 15+30kHz SCS CA. (scenarios with larger number of HARQ processes) Proposal 3: For HARQ process for 30kHz SCell in TDD 15 kHz + TDD 30 kHz CA, - With 12 HARQ processes, both options on the scheduling details are ok. - Considering that the K3 values are different for the two options, discuss whether to define the K3 values in TS 38.101-4 for CA PDSCH demodulation requirements. Proposal 4: For HARQ process number for 15kHz SCell in TDD 15 kHz + TDD 30 kHz CA, both option are ok, and option 1 is slightly preferred. Test applicability Proposal 5: Reuse the LTE approach for CA capability categorization, i.e., define different capabilities for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands. Proposal 6: Test all the supported CA capabilities, including intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands. Proposal 7: Selection of CA configuration(s) and CBW combination: For FR1, for each supported CA duplex mode and each supported CA capability, - Step 1: Select the CA configuration(s) satisfying the following conditions: - For each CC, single carrier performance requirement is specified for any one of the supported SCS(s). - For each CC, the supported maximum modulation order is not lower than 16 QAM. - For each CC, the supported maximum number of MIMO layers is not lower than 2. - For each band, the supported max data rate (calculated according to 4.1.2 of TS 38.306) is not lower than the date rate corresponding to using 2-layer and MCS 13 on the largest (aggregated) channel bandwidth on the band. - Step 2: Select any one of the CA configuration(s) with the largest aggregated CA bandwidth among the selected the CA configuration(s) based on step 1. For FR2, for each supported CA duplex mode and each supported CA capability, - Step 1: Select the CA configuration(s) satisfying the following conditions: - For each CC, single carrier performance requirement is specified for any one of the supported SCS(s) - For each CC, the supported maximum modulation order is not lower than 16 QAM - For each CC, the supported maximum number of MIMO layers is not lower than 2 - For each band, the supported max data rate (calculated according to 4.1.2 of TS 38.306) is not lower than the date rate corresponding to using 2-layer and MCS 10 on the largest (aggregated) channel bandwidth on the band. - Step 2: Calculate the largest aggregated CA bandwidth for the selected the CA configuration(s) based on step 1, denoted as CBWlargest. - Step 3: Calculate the maximum aggregated channel bandwidth that can be testable in the test system, denoted as CBWtestable. - Step 4: - If CBWlargest <= CBWtestable, select any one of the CA configuration(s) with the largest aggregated CA bandwidth among the selected the CA configuration(s) based on step 1. - If CBWlargest > CBWtestable, select any one of the CA configuration(s) with the aggregated channel bandwidth no smaller than CBWtestable among the selected the CA configuration(s) based on step 1. Requirement values and CRs Proposal 8: Decide the requirement values in this meeting, and agree the CRs in the next meeting. |
| R4-2006530 | Intel Corporation | Proposal 1: Define TDD-FDD CA and TDD-TDD CA with different SCSs for all PCell configurations (i.e. both FDD Pcell and TDD Pcell; both 15kHz Pcell and 30kHz Pcell) and test UE for any one PCell configuration. Proposal 2: Consider the following HARQ process configuration for TDD-TDD CA with different SCSs: - PCell TDD 15kHz + SCell TDD 30kHz: PCell – 8, SCell – 12 (same RTT for all HARQ processes) - PCell TDD 30kHz + SCell TDD 15kHz: PCell – 8, SCell – 8 Proposal 3: Align categorizing of CA capabilities for NR Normal CA requirements with RF specifications. Use references to sections with CA configurations descriptions in RF specifications (for example, 5.2A and 5.5A) for definition of CA capabilities to avoid regular maintenance of TS 38.101-4. Proposal 4: Consider the following CA capabilities for NR Normal CA testing: Intra-band contiguous CA, Intra-band non-contiguous CA and Inter-band CA with the largest number of bands Proposal 5: Use the following approach for selection of CA configuration for NR FR1 Normal CA testing: - Step 1: Select CA configurations with maximum number of CCs, on which UE capability field supportedSubCarrierSpacingDL is equal to SCSreq, among all supported CA configurations - Step 2: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq, among all the selected CA configurations from Step 1 - Step 3: Select any one of CA configurations, which contain CBW combination with the largest data rate not exceeding DataRatereq, among all the selected CA configurations from Step 2. Proposal 6: Use the following approach for selection of CA configuration for NR FR2 Normal CA testing: - Step 1: Select CA configurations, which contain CBW combinations with SNRTEmax higher or equal to SNRreq, among all supported CA configurations - Step 2: Select CA configurations with maximum number of CCs, on which UE capability field supportedSubCarrierSpacingDL is equal to SCSreq, among all the selected CA configurations from Step 1 - Step 3: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq, among all the selected CA configurations from Step 2 - Step 4: Select any one of CA configurations, which contain CBW combination with the largest data rate not exceeding DataRatereq and aggregated bandwidth with SNRTEmax higher or equal to SNRreq, among all the selected CA configurations from Step 3. |
| R4-2006531 | Intel Corporation | Summary of Normal CA simulation results (FR2) |
| R4-2006628 | Qualcomm Incorporated | Simulation Results for NR CA PDSCH Demodulation Performance Tests |
| R4-2006629 | Qualcomm Incorporated | Observation 1: Initial transmission and retransmission should happen on the same type of slot. Otherwise, it will degrade the HARQ performance. Proposal 1: In case of TDD 15kHz + TDD 30kHz CA with TDD 15kHz as PCell, different RTTs (10 or 20 slots) are used for different HARQ processes, and initial transmission and retransmission are scheduled on the same type of TDD slot. Proposal 2: In case of TDD 15kHz + TDD 30kHz CA with TDD 30kHz as PCell, use 8 HARQ processes. Proposal 3: If PCell in both carriers are supported, configure FDD cell as PCell in TDD-FDD CA, configure 30 kHz SCS cell as PCell in TDD 15kHz+30kHz SCS CA. |
| R4-2006808 | CMCC | Observation 1: There is no UE capability to indicate support of TDD PCell or FDD PCell Observation 2: There is UE capability to indicate the support of SCS for each DL in a CA band combination. Proposal 1: It is proposed that: - For FDD 15 kHz + TDD 30 kHz: Configure TDD 30KHz as PCell - For FDD 15KHz + TDD 15KHz: Configure TDD 30KHz as PCell - For TDD 15KHz + TDD 30KHz: Configure TDD 15KHz as PCell Proposal 2: It is proposed to define different capabilities for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands. |
| R4-2007139 | NTT DOCOMO, INC. | Proposal 1: The following options should be supported for Pcell configuration for TDD-FDD CA and TDD CA with different SCSs Pcell configuration for performance requirements • (Option 3) Decide after conclusion on “Pcell configuration for the test” will be reached Pcell configuration for the test – (Option 5) If Pcell in both carriers are supported, both FDD and TDD cell should be tested as Pcell for TDD-FDD CA and configure 30 kHz SCS cell as Pcell in TDD 15+30kHz SCS CA Proposal 2: Use the following approach on CA test applicability Categorizing of CA capabilities - Define different capabilities for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands. Test of different CA capabilities - Test all the supported CA capabilities, including intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands. |
| R4-2007221 | Huawei, HiSilicon | Observation 1: From the Figure 2.1.1-1 shown above, 6 HARQ processes are feasible for SCell TDD 15kHz SCS. Observation 2: From the Figure 8 from R4-2000359 shown above, 6 HARQ processes for SCell TDD 15kHz are analysed. Observation 3: From Figure 2-2 from R4-2001419 shown above, 6 HARQ processes for SCell TDD 15kHz are analyzed. Proposal 1: No scheduling restriction should be imposed on the initial transmission and retransmission on the type of TDD slots. Proposal 2: Not use UL symbols in special slot for HARQ-ACK feedback in PUCCH. Proposal 3: Adopt 6 HARQ processes for SCell TDD 15kHz for TDD 30kHz + 15kHz CA with TDD 30kHz PCell. Proposal 4: Use the following number of HARQ process and K1 values for CA with different duplex mode or mixed numerology: Table 2.1-2: Number of HARQ process and K1 value for CA with different duplex mode or mixed numerology Scenario; PCell; Number of HARQ process for PCell; K1 for PCell; SCell; Number of HARQ process for SCell; K1 for SCell. FDD 15kHz SCS + TDD 30kHz SCS; FDD 15kHz; 4; 2; TDD 30kHz; 8; {2}. TDD 30kHz; 8; {8,7,6,5,5,4,3,2}; FDD 15kHz; 8; {7,6,4,11,9,7,6,4}. FDD 15kHz SCS + TDD 15kHz SCS; FDD 15kHz; 4; {2}; TDD 15kHz; 4; {2}. TDD 15kHz; 8; {4,3,2,6,5}; FDD 15kHz; 8; {4,3,2,6}. TDD 15kHz SCS + TDD 30kHz SCS; TDD 15kHz; 8; {4,3,2,6}; TDD 30kHz; 12; {4,4,3,3,2,2,6,6}. TDD 30kHz; 8; {8,7,6,5,5,4,3,2}; TDD 15kHz; 6; {7,5,4,11}. |
| R4-2007222 | Huawei, HiSilicon | Observation 1: The performance requirements can be defined agnostic to the specific PCell or SCell. Observation 2: No essential difference for Option 1 and Option 2 for CA capability categorizing if specification reference method is used and CA capability definition is not combined with test of different CA capabilities. Proposal 1: Adopt Option 4, i.e. If PCell in both carriers are supported, configure FDD 15kHz cell as PCell in FDD 15kHz + TDD 15kHz CA, configure 30kHz SCS cell as PCell in both FDD 15kHz + TDD 30kHz CA and TDD 15kHz + TDD 30kHz CA. Proposal 2: Choose Option 1, i.e. test intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with the largest number of bands for test of different CA capabilities. Proposal 3: Adopt the following test applicability rule for selection of CA configurations and CBW combination for test: - For intra-band contiguous CA and intra-band non-contiguous CA - Select any one of the supported CA configurations with the largest aggregated CA bandwidth combination for certain selected CA duplex mode - If more than one CA configurations with the same largest aggregated CA bandwidth combination, select the CA configurations with the largest number of CCs - For inter-band CA - Select any one of the supported CA configurations with the largest number of bands aggregated |
| R4-2007223 | Huawei, HiSilicon | draftCR for NR FR1 PDSCH CA normal demodulation requirements with 4Rx. |

Open issues summary

Sub-topic 2-1: Pcell configuration

Issue 2-1: Pcell configuration for TDD-FDD CA and TDD-TDD CA with different SCSs

- Agreement in RAN4 #94e-bis (R4-2005546, WF)

  - Pcell configuration for performance requirements

    - Option 1: Reuse single carrier performance for CA, and no matter which cell is Pcell for the requirements.

    - Option 2:

      - For CA with different SCSs, define requirements for both 15kHz Pcell and 30kHz Pcell.

      - For FDD + TDD CA with 15 kHz SCS, define requirements for both FDD 15 kHz Pcell and TDD 15 kHz Pcell

    - Option 3: Decide after conclusion on “Pcell configuration for the test” will be reached

  - Pcell configuration for the test

    - Option 1: The test coverage can be considered fulfilled if UE passes one of scenario with one of the CC as PCell as per the real testing request

    - Option 2: If Pcell in both carriers are supported, configure TDD cell as Pcell in TDD-FDD CA, configure 15 kHz SCS cell as Pcell in TDD 15+30kHz SCS CA. (scenarios with larger number of HARQ processes)

    - Option 3: If Pcell in both carriers are supported, configure FDD cell as Pcell in TDD-FDD CA, configure 30 kHz SCS cell as Pcell in TDD 15+30kHz SCS CA. (scenarios with less number of HARQ processes)

    - Option 4: If PCell in both carriers are supported, configure FDD 15kHz cell as PCell in FDD 15kHz + TDD 15kHz CA, configure 30kHz SCS cell as PCell in both FDD 15kHz + TDD 30kHz CA and TDD 15kHz + TDD 30kHz CA

    - Option 5: If Pcell in both carriers are supported, both FDD and TDD cell should be tested as Pcell for TDD-FDD CA and configure 30 kHz SCS cell as Pcell in TDD 15+30kHz SCS CA

    - Note: Companies are encouraged to check if there are UE capability signalling which allows to check

Issue 2-1-1: Pcell configuration for performance requirements

- Summary of understanding on Rel-15 UE capability signalling

  - View #1 (China Telecom, Huawei)

    - For CA with different SCSs, different capabilities are defined for Pcell on larger SCS (i.e., diffNumerologyWithinPUCCH- GroupLargerSCS) and Pcell on smaller SCS (i.e., diffNumerologyWithinPUCCH-GroupSmallerSCS), where Pcell is the cell carrying PUCCH.

    - For FDD + TDD CA with 15 kHz SCS, there is no UE capability defined for TDD Pcell and FDD Pcell.

  - View #2 (CMCC)

    - There is no UE capability to indicate support of TDD PCell or FDD PCell.

    - There is UE capability to indicate the support of SCS for each DL in a CA band combination, i.e. supportedSubCarrierSpacingDL.

  - View #3 (Intel)

    - There is no UE capability on support of TDD PCell or FDD PCell and 15 kHz PCell or 30 kHz PCell.

- Proposals

  - Option 1: Reuse single carrier performance for CA, and no matter which cell is Pcell for the requirements.

  - Option 2 (CTC, Intel)

    - For CA with different SCSs, define requirements for both 15kHz Pcell and 30kHz Pcell.

    - For FDD + TDD CA with 15 kHz SCS, define requirements for both FDD 15 kHz Pcell and TDD 15 kHz Pcell

  - Option 3: Decide after conclusion on “Pcell configuration for the test” will be reached (DCM)

- Recommended WF

  - It looks that the 3 options are not conflicting with each other. Given that the same single carrier performance is reused, and the Pcell for testing is discussed separately, can we agree with option 2 as baseline?

Issue 2-1-2: Pcell configuration for the test

- Proposals

  - Option 1: The test coverage can be considered fulfilled if UE passes one of scenario with one of the CC as PCell as per the real testing request (Intel)

  - Option 2: If Pcell in both carriers are supported, configure TDD cell as Pcell in TDD-FDD CA, configure 15 kHz SCS cell as Pcell in TDD 15+30kHz SCS CA. (scenarios with larger number of HARQ processes). (CTC, CMCC)

  - Option 3: If Pcell in both carriers are supported, configure FDD cell as Pcell in TDD-FDD CA, configure 30 kHz SCS cell as Pcell in TDD 15+30kHz SCS CA. (scenarios with less number of HARQ processes) (QC)

  - Option 4: If PCell in both carriers are supported, configure FDD 15kHz cell as PCell in FDD 15kHz + TDD 15kHz CA, configure 30kHz SCS cell as PCell in both FDD 15kHz + TDD 30kHz CA and TDD 15kHz + TDD 30kHz CA (HW)

  - Option 5: If Pcell in both carriers are supported, both FDD and TDD cell should be tested as Pcell for TDD-FDD CA and configure 30 kHz SCS cell as Pcell in TDD 15+30kHz SCS CA (DCM)

- Moderator’s observations

  - The main difference due to different Pcell configurations is the HARQ process number.  So the question is: if the test is conducted for one of the Pcell configurations, can we guarantee the demodulation performance for the other Pcell configuration?

  - Not sure if it is feasible to select the Pcell based on the real deployment scenarios, since the deployment scenario can be different for different operators. Even for one operator, the Pcell configuration can be different in different cities and scenarios, and can also change with the time.

- Recommended WF

  - Given 5 different options are proposed, encourage companies to re-consider their positions: in addition to the favourite option, are there any other options also acceptable?

  - Aim to make decision in this meeting.

Sub-topic 2-2: HARQ process number

Issue 2-2: HARQ process number for TDD-FDD CA and TDD-TDD CA with different SCSs

- Agreement in RAN4 #94e-bis (R4-2005546, WF)

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 8 |
|  | TDD PCell | 8 | 8 |
| FDD 15 kHz + TDD 15 kHz CA | FDD PCell | 4 | 4 |
|  | TDD PCell | 8 | 8 |
| TDD 15 kHz + TDD 30 kHz CA | 15kHz PCell | 8 | 12 (Note 1) |
|  | 30kHz PCell | 8 | Option 1: 6 Option 2: 8 |
| Note 1: FFS scheduling details: - Option 1: different RTTs (10 or 20 slots) are used for different HARQ processes, and initial transmission and retransmission are scheduled on the same type of TDD slot. - Option 2: initial transmission and retransmission can be scheduled on different types of TDD slot |  |  |  |

Issue 2-2-1: HARQ process number for 30kHz SCell in TDD 15 kHz + TDD 30 kHz CA

- Proposals

  - Down-selection of the options

    - Option 1: 12, different RTTs (10 or 20 slots) are used for different HARQ processes, and initial transmission and retransmission are scheduled on the same type of TDD slot. (CTC, QC)

      - QC: Initial transmission and retransmission should happen on the same type of slot. Otherwise, it will degrade the HARQ performance.

    - Option 2: 12, initial transmission and retransmission can be scheduled on different types of TDD slot (CTC, Intel, HW)

      - Huawei, Intel: Based on our simulations, there is no performance difference in case initial transmission and retransmission are scheduled in the same or different type of slots

  - Additional issue: Is it necessary to differentiate the two options in TS 38.101-4?

    - China Telecom: Considering that the K3 values are different for the two options, discuss whether to define the K3 values (DL NACK to DL re-tx grant) in TS 38.101-4 for CA PDSCH demodulation requirements.

- Recommended WF

  - Down-selection of the options

    - Allow more time for simulation, and make decision in the next meeting

    - In the next meeting, if no simulation results show there is performance impact by scheduling the initial transmission and retransmission in different types of slots, then option 2 will be selected.

  - Encourage feedback on whether it is necessary to differentiate the two options in TS 38.101-4.

Issue 2-2-2: HARQ process number for 15kHz SCell in TDD 15 kHz + TDD 30 kHz CA

- Proposals

  - Option 1: 6 (CTC, HW)

  - Option 2: 8 (CTC, Intel, QC)

    - CTC: 8 HARQ process is slightly preferred, since the same HARQ process number for 15 kHz SCell is used as when it is configured as Pcell.

    - QC: Initial transmission and retransmission should happen on the same type of slot. Otherwise, it will degrade the HARQ performance.

- Recommended WF

  - Can we go with option 2?

Issue 2-2-3: K1 values

- Proposals

  - Option 1 (Huawei)

| K1 |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 2 | {2} |
|  | TDD PCell | {8,7,6,5,5,4,3,2} | {7,6,4,11,9,7,6,4} |
| FDD 15 kHz + TDD 15 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | {4,3,2,6,5} | {4,3,2,6} |
| TDD 15 kHz + TDD 30 kHz CA | 15kHz PCell | {4,3,2,6} | {4,4,3,3,2,2,6,6} |
|  | 30kHz PCell | {8,7,6,5,5,4,3,2} | {7,5,4,11} |

- Recommended WF

  - Encourage feedback on the above option 1

Sub-topic 2-3: Simulation results and performance requirements

Issue 2-3-1: Performance requirements for FR1

- Summary (submitted in RAN4 #94e-bis)

  - R4-2004554 Summary of Normal CA simulation results (FR1 15 kHz FDD and TDD)

  - R4-2004555 Summary of Normal CA simulation results (FR1 30 kHz TDD)

- Observations

  - 5 companies provided alignment and impairment simulation results for all the cases.

  - 1 company (QC) updated the simulation results in this meeting

  - Based on the latest simulation results from companies, both alignment and impairment simulation results are well aligned, with the span less than 2dB.

- Recommended WF

  - Agree the proposed requirements in the updated simulation result summary in the draft folder:

    - Draft_R4-200xxxx - Summary of Normal CA simulation results (NR FR1 15 kHz FDD and TDD).xlsx

    - Draft_R4-200xxxx - Summary of Normal CA simulation results (NR FR1 30 kHz TDD).xlsx

Issue 2-3-2: Performance requirements for FR2

- Summary

  - R4-2006531 Summary of Normal CA simulation results (FR2) (to be uploaded)

- Observations

  - Alignment simulation results: 5 companies provided results, which are well aligned.

  - Impairment simulation results: 2 companies (Intel, CTC) provided results in the last meeting, and 1 company (QC) provided results in this meeting.

- Recommended WF

  - If Huawei and Ericsson will provide impairment simulation results in this meeting, discuss the requirement values in this meeting;

  - If not, discuss and decide the requirement values in the next meeting.

Sub-topic 2-4: CA capabilities, Selection of CA configuration(s) and CBW combination

Issue 2-4: CA capabilities, Selection of CA configuration(s) and CBW combination

- Moderators’ observations

  - Based on the tdocs submitted to RAN4 #95e, due to the short time for tdoc preparation, companies’ positions and arguments for the test applicability rule on different CA capabilities, selection of CA configuration(s) and CBW combination are generally the same as that in RAN4 #94e-bis.

- Recommended WF

  - For the test applicability rule on different CA capabilities, selection of CA configuration(s) and CBW combination

    - Further discuss and make decision in the next meeting.

Sub-topic 2-5: Plan for CRs

Issue 2-5: Plan for CRs

- Agreed CR work split for CA normal PDSCH

|  |  |  | CR Responsibility |
| --- | --- | --- | --- |
| CA normal demodulation for NR CA, EN-DC, NE-DC, NR-DC | FR1 | Applicability | Intel |
|  |  | 2Rx requirements | CMCC |
|  |  | 4Rx requirements | Huawei |
|  |  | FRC | Intel |
|  | FR2 | Applicability | Intel |
|  |  | 2Rx requirements | Qualcomm |
|  |  | FRC | Intel |

  - In this meeting, Huawei provided draft CR for NR FR1 requirements with 4Rx.

- Recommended WF

  - Endorse the draft CR for NR FR1 requirements with 4Rx in this meeting

    - Encourage companies to provide comments for this draft CR in section 2.3.2.

  - Endorse all others CRs on requirements and FRCs in RAN4 #96e (Aug) meeting

    - For CMCC and QC’s CRs, align the structure with Huawei’s draft CR

  - Agree all the CRs for CA normal PDSCH together (including CRs requirements, FRCs and test applicability rules) in RAN4 #97e (Oct-Nov) meeting

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| China Telecom | Issue 2-1: Pcell configuration for TDD-FDD CA and TDD-TDD CA with different SCSs Issue 2-1-1: Pcell configuration for performance requirements Support to agree option 2. Issue 2-1-2: Pcell configuration for the test As presented in our contribution, we believe option 2 is the feasible way to go. The prerequisite to discuss this issue is that UE declares to support Pcell configurations in both carriers. So the UE should be able to support the PCell scenarios resulting in larger number of HARQ processes. Note that if one UE declares to only support the PCell configuration corresponding to the larger number of HARQ processes, the UE should conduct test for scenarios resulting in larger number of HARQ processes. Our comments to the other options are as below: - Option 1 is not very clear to us, and it looks like that UE can decide the PCell configuration to be used in the test. - Option 3 results in testing scenarios with less number of HARQ processes, so the PDSCH demodulation performance cannot be guaranteed if larger HARQ process happens in the real network. - Option 4 is proposed considering the real deployment scenarios, but the deployment scenario can be different for different operators. Even for one operator, the Pcell configuration can be different in different cities and scenarios, and can also change with the time (e.g., the number of sites and coverage of different carriers can change with time). - Option 5 looks good from the test coverage point of view. At the same time, if the only difference between different Pcell configurations is the HARQ process, it might not be essential to test both FDD and TDD Pcell. Issue 2-2: HARQ process number for TDD-FDD CA and TDD-TDD CA with different SCSs Issue 2-2-1: HARQ process number for 30kHz SCell in TDD 15 kHz + TDD 30 kHz CA For the two options, Ok with either option 1 or option 2 or the recommended WF. Regarding whether it is necessary to differentiate the two options in TS 38.101-4, our view is if there is no performance difference, we might not need to defined K3 in the spec, consideirng that it was not defined in Rel-15. Issue 2-2-2: HARQ process number for 15kHz SCell in TDD 15 kHz + TDD 30 kHz CA Ok with the recommended WF. Issue 2-2-3: K1 values In option 1, the K1 values are provided based on Pcell’s SCS in scenarios with mixed SCSs. But in our understanding, K1 values are based on each cell’s own SCS. Issue 2-3: Simulation results and performance requirements Issue 2-3-1: Performance requirements for FR1 Ok with the recommended WF. Issue 2-3-2: Performance requirements for FR2 Ok with the recommended WF. Issue 2-4: CA capabilities, Selection of CA configuration(s) and CBW combination Ok with the recommended WF. Issue 2-5: Plan for CRs Ok with the recommended WF. |
| CMCC | Issue 2-1: Pcell configuration for TDD-FDD CA and TDD-TDD CA with different SCSsIssue 2-1-1: Pcell configuration for performance requirements OK with the recommended WF.Issue 2-1-2: Pcell configuration for the test Agree with Moderator’s observation that it is difficult to select the PCell based on deployment scenario, since the PCell configuration can change in real network. That is why to support option 2 to consider the scenario with larger number of HARQ processes. By considering the worst case, we think the demodulation performance for the other PCell configuration can be guaranteed. Issue 2-2-2: HARQ process number for 15kHz SCell in TDD 15 kHz + TDD 30 kHz CA Support option 2. |
| Qualcomm | Issue 2-1: Pcell configuration for TDD-FDD CA and TDD-TDD CA with different SCSs Issue 2-1-1: Pcell configuration for performance requirements As the only difference between choosing different PCell is number of HARQ processes and many companies have already established based on simulations that number of HARQ processes doesn’t change the performance much, our preference is to define the requirements agnostic to which carrier is PCell, i.e., Option 1. With Option 2, we will end up unnecessarily defining two sets of similar looking requirements. Issue 2-1-2: Pcell configuration for the test We are ok with Option 1 or Option 3 since number of HARQ processes doesn’t change the performance much. So, we don’t need to test all possible HARQ combinations. Issue 2-2: HARQ process number for TDD-FDD CA and TDD-TDD CA with different SCSs Issue 2-2-1: HARQ process number for 30kHz SCell in TDD 15 kHz + TDD 30 kHz CA Prefer Option 1 because in general, having retransmission in different type of slot will degrade the HARQ performance. But we are ok to evaluate the loss in this case. Issue 2-2-2: HARQ process number for 15kHz SCell in TDD 15 kHz + TDD 30 kHz CA Prefer Option 2. Issue 2-2-3: K1 values Ok with recommended WF Issue 2-3: Simulation results and performance requirements Issue 2-3-1: Performance requirements for FR1 Ok with recommended WF Issue 2-3-2: Performance requirements for FR2 Ok with recommended WF Issue 2-4: CA capabilities, Selection of CA configuration(s) and CBW combination Ok with recommended WF Issue 2-5: Plan for CRs Ok with recommended WF |
| Huawei, HiSilicon | Issue 2-1: Pcell configuration for TDD-FDD CA and TDD-TDD CA with different SCSs Issue 2-1-1: Pcell configuration for performance requirements Actually we did not fully understand Option 2. Does it mean that RAN4 defines requirements by reusing single carrier requirements, but just additionally configure the PCell for different SCS and then set test applicability rule for real testing as per agreement of Issue 2-1-2? Considering the UE capability for support of PCell on larger SCS (i.e., diffNumerologyWithinPUCCH- GroupLargerSCS) and PCell on smaller SCS (i.e., diffNumerologyWithinPUCCH-GroupSmallerSCS), maybe both PCell with larger and smaller SCS need to be configured, which one is used for test is based on UE capability. So first item of “For CA with different SCSs, define requirements for both 15kHz Pcell and 30kHz Pcell” is agreeable. There is no UE capability for FDD + TDD CA with 15 kHz SCS, it is better to define the requirements, i.e. the PCell configuration, as per the agreement of Issue 2-1-2. Issue 2-1-2: Pcell configuration for the test We already agreed that no performance difference for which cell acted as PCell, except the number of HARQ processes, no other essential difference, the performance requirements for maximum number HARQ process of 16 is verified in single carrier performance, we do not think it is necessary to focus on verifying the number of HARQ process again in PCell, so if companies have concern on Option 4, we think that Option 1 is agreeable, i.e. UE can choose any one cell as PCell in the real testing. Issue 2-2: HARQ process number for TDD-FDD CA and TDD-TDD CA with different SCSs Issue 2-2-1: HARQ process number for 30kHz SCell in TDD 15 kHz + TDD 30 kHz CA Prefer Option 2 and also ok with the recommended WF. There is no performance difference as per our evaluations and no such constrain from the core specification point of view, we don’t think that it is necessary to set such limitation for testing. Issue 2-2-2: HARQ process number for 15kHz SCell in TDD 15 kHz + TDD 30 kHz CA If we go option 2 for Issue 2-2-1, Option 1 is best choice for this issue. @Intel, Qualcomm, as mentioned in our contribution R4-2007221, as per R4-2000359 from Intel and R4-2001419 from Qualcomm, 6 HARQ processes is analysed, we wonder the reason to propose 8 HARQ processes. Issue 2-2-3: K1 values Based on our understanding about the core specification, the K1 values should be based on the PCell’s SCS. Please double check the core specification TS 38.213 section 9.2.3. Issue 2-3: Simulation results and performance requirements Issue 2-3-1: Performance requirements for FR1 Ok with the recommended WF Issue 2-3-2: Performance requirements for FR2 We submitted our impairment results in the draft folder. Issue 2-4: CA capabilities, Selection of CA configuration(s) and CBW combination Ok with the recommended WF Issue 2-5: Plan for CRs Ok with the recommended WF |
| Ericsson | Issue 2-3: Simulation results and performance requirements Issue 2-3-1: Performance requirements for FR1 OK with the recommended WF. Issue 2-3-2: Performance requirements for FR2 Ericsson provided impairment results for FR2. ‘draft_R4-2006531 - Summary CA (NR FR2 120 kHz TDD)_QC_Huawei_Ericsson.xlsx’ We are ok with the recommend WF. One comment for FR2 simulation summary. It looks Intel’s impairment results for 400MHz is typo because it is same as alignment results. Please double check. |
| Intel | Issue 2-1: Pcell configuration for TDD-FDD CA and TDD-TDD CA with different SCSs Issue 2-1-1: Pcell configuration for performance requirements We think that Option 2 is acceptable in case we go with Option 1 for Issue 2-1-2. Al least, we have common view that “There is no UE capability to indicate support of TDD PCell or FDD PCell.”. Therefore, if we will use any of Options from 2 to 5 from 2-1-2 then only one scenario with TDD 15 kHz PCell or FDD 15 kHz PCell will be always tested. Issue 2-1-2: Pcell configuration for the test We think that Option 1 is rather compromise solution because it does not have restrictions on tested scenarios and any of scenarios listed in Options from 2 to 5 can be tested. Issue 2-2: HARQ process number for TDD-FDD CA and TDD-TDD CA with different SCSs Issue 2-2-1: HARQ process number for 30kHz SCell in TDD 15 kHz + TDD 30 kHz CA Taking into account that details of HARQ index mapping to DL slot are not defined in the TS 38.101-4, we can omit differentiation of these two options in case same performance will be confirmed by all companies. Issue 2-2-3: K1 values We can take Option 1 as baseline and confirm it in the next RAN4 meeting if no technical issue will be observed. |
| docomo | Issue 2-1: Pcell configuration for TDD-FDD CA and TDD-TDD CA with different SCSs Issue 2-1-1: Pcell configuration for performance requirements Ok with the recommended WF. Issue 2-1-2: Pcell configuration for the test At the last meeting, we proposed Opt. 5 since different companies have different prioritization on PCell configuration while larger number of HARQ process should be configured and tested in the sense of guarantee of UE PDSCH performance. However, to make a better progree, we can compromise to further down-selection from Option 2-4 and our preference is Option 4. Based on the QC's comments above, QC seems have no strong preference on this issue. If so, at least, we may quickly agree to configure TDD cell as Pcell for FDD 15 kHz + TDD 30 kHz CA. Issue 2-4: CA capabilities, Selection of CA configuration(s) and CBW combination Ok with the recommended WF. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007223, Huawei, HiSilicon | China Telecom: we have uploaded an revision with our comments in the draft inbox: Revised R4-2007223 draftCR NR CA FR1 4Rx PDSCH v1-CTC.docx |
|  | Qualcomm: In our opinion, Table 5.2A-1 and 5.2A-2 are same as corresponding tables for single carrier requirements. In that case, we should refer to the tables in Section 5.2 instead of copying it over again. Also, Table 5.2A-3 should have PCell as TBD for mixed SCS and mixed Duplex cases since we are still discussing that issue. @Huawei: Our preference is not to duplicate as much as possible. If there are any differences, we can just mention those changes in test parameter tables. For SDR tests, we did the same to avoid duplicating the work. But, we are open to discussion and would like to know views from other companies. |
|  | Huawei: We updated the draftCR as per comments from CTC. Some other parts also needs to be updated as per further agreements. To make the CR easier to read, I removed the comments from CTC that we accepted. @Qualcomm, for test parameters table, considering they are parallel section for Section 5.2 and Section 5.2A, so we listed all necessary test parameters even most of them are same as existing single carrier performance, we have no strong views to refer to the tables in section 5.2, but we need to list those different parameters; as CTC suggested, we can list the PCell configuration for test in the test applicability rule part. |
|  | Intel: As one of options, we can also consider alignment of sections structures with PDSCH section for single carrier 5.2A PDSCH demodulation requirements for CA … 5.2A.3 4RX requirements 5.2A.3.1 FDD 5.2A.3.1.1 Minimum requirements 5.2A.3.2 TDD 5.2A.3.2.1 Minimum requirements 5.2A.3.3 FDD TDD 5.2A.3.1 Minimum requirements |

Note: To save time on typing the comments one by one, companies can also directly revise the draft CR and upload the revision in the draft inbox.

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Topic #2: CA normal PDSCH | - Issue 2-1-1: Pcell configuration for performance requirements - Option 1: Reuse single carrier performance for CA, and no matter which cell is Pcell for the requirements. (QC) - Option 2 (CTC, CMCC, DCM) - For CA with different SCSs, define requirements for both 15kHz Pcell and 30kHz Pcell. - For FDD + TDD CA with 15 kHz SCS, define requirements for both FDD 15 kHz Pcell and TDD 15 kHz Pcell - Option 3: Decide after conclusion on “Pcell configuration for the test” will be reached (DCM) - Option 4 (HW, Intel) - For CA with different SCSs, define requirements for both 15kHz Pcell and 30kHz Pcell, and set test applicability rule for real testing as per agreement of Issue 2-1-2 - For FDD + TDD CA with 15 kHz SCS, define requirements for either FDD 15 kHz Pcell or TDD 15 kHz Pcell as per the agreement of Issue 2-1-2. Recommendations for 2nd round: Decide after conclusion on “Pcell configuration for the test” will be reached - Issue 2-1-2: Pcell configuration for the test - Option 1: The test coverage can be considered fulfilled if UE passes one of scenario with one of the CC as PCell as per the real testing request (Intel, QC, HW) - Option 2: If Pcell in both carriers are supported, configure TDD cell as Pcell in TDD-FDD CA, configure 15 kHz SCS cell as Pcell in TDD 15+30kHz SCS CA. (scenarios with larger number of HARQ processes). (CTC, CMCC) - Option 3: If Pcell in both carriers are supported, configure FDD cell as Pcell in TDD-FDD CA, configure 30 kHz SCS cell as Pcell in TDD 15+30kHz SCS CA. (scenarios with less number of HARQ processes) (QC) - Option 4: If PCell in both carriers are supported, configure FDD 15kHz cell as PCell in FDD 15kHz + TDD 15kHz CA, configure 30kHz SCS cell as PCell in both FDD 15kHz + TDD 30kHz CA and TDD 15kHz + TDD 30kHz CA (HW, DCM) - Option 5: If Pcell in both carriers are supported, both FDD and TDD cell should be tested as Pcell for TDD-FDD CA and configure 30 kHz SCS cell as Pcell in TDD 15+30kHz SCS CA (DCM) Recommendations for 2nd round: The proponents for option 3 and option 5 have provided another acceptable option, so suggest to further discuss option 1, 2 and 4 in the 2nd round. - For option 1, clarify what does it mean by saying “as per the real testing request”, and how to capture it in the spec. - For option 2, clarify what is the issue to use option 2 for UE declaring the support of Pcell configurations in both carriers. - Try to make further down-selection in this meeting. - Issue 2-2-1: HARQ process number for 30kHz SCell in TDD 15 kHz + TDD 30 kHz CA - Down-selection of the options - Option 1: 12, different RTTs (10 or 20 slots) are used for different HARQ processes, and initial transmission and retransmission are scheduled on the same type of TDD slot. (CTC, QC) - Option 2: 12, initial transmission and retransmission can be scheduled on different types of TDD slot (CTC, Intel, HW) - Is it necessary to differentiate the two options in TS 38.101-4? - Option 1: if there is no performance difference, we might not need to define K3 in the spec, considering that it was not defined in Rel-15. (CTC, Intel) Tentative agreement: - Allow more time for simulation, and make decision in the next meeting - In the next meeting, if no simulation results show there is performance impact by scheduling the initial transmission and retransmission in different types of slots, then option 2 will be selected, or alternatively, no need to differentiate the two options in TS 38.101-4. - Issue 2-2-2: HARQ process number for 15kHz SCell in TDD 15 kHz + TDD 30 kHz CA - Option 1: 6 (HW) - Option 2: 8 (CTC, Intel, QC, CMCC) Recommendations for 2nd round: Further discuss and try to make decision in this meeting - Issue 2-2-3: K1 values - Option 1 (Huawei): K1 values are provided based on Pcell’s SCS in scenarios with mixed SCSs. K1; CCs with the same duplex mode & SCS with Pcell; CCs with different duplex mode / SCS with Pcell. FDD 15 kHz + TDD 30 kHz CA; FDD PCell; 2; {2}. TDD PCell; {8,7,6,5,5,4,3,2}; {7,6,4,11,9,7,6,4}. FDD 15 kHz + TDD 15 kHz CA; FDD PCell; {2}; {2}. TDD PCell; {4,3,2,6,5}; {4,3,2,6}. TDD 15 kHz + TDD 30 kHz CA; 15kHz PCell; {4,3,2,6}; {4,4,3,3,2,2,6,6}. 30kHz PCell; {8,7,6,5,5,4,3,2}; {7,5,4,11}. - Option 2: K1 values are based on each cell’s own SCS (CTC) Tentative agreement: - Allow more time to check the RAN1 spec. Keep the two options and make decision in the next meeting. - Issue 2-3-1: Performance requirements for FR1 Tentative agreement: - Agree the proposed requirements in the latest simulation result summary (CTC, QC, HW, E///) - Issue 2-3-2: Performance requirements for FR2 - Huawei and Ericsson have provided impairment simulation results in the 1st round. Now 5 companies provided both alignment and impairment simulation results for all the cases. - One issue raised by E///: for FR2 simulation summary, it looks Intel’s impairment results for 400MHz is typo because it is same as alignment results. Recommendations for 2nd round: - Invite Intel to check the issue raised by E///. If this is a typo and can be corrected, companies to check if the proposed requirements in the latest simulation result summary can be agreeable. - Issue 2-4: CA capabilities, Selection of CA configuration(s) and CBW combination Tentative agreement: - For the test applicability rule on different CA capabilities, selection of CA configuration(s) and CBW combination (CTC, QC, HW, DCM) - Further discuss and make decision in the next meeting. - Issue 2-5: Plan for CRs Tentative agreement: - Endorse the draft CR for NR FR1 requirements with 4Rx in this meeting - Encourage companies to provide comments for this draft CR in section 2.3.2. - Endorse all others CRs on requirements and FRCs in RAN4 #96e (Aug) meeting - For CMCC and QC’s CRs, align the structure with Huawei’s draft CR - Agree all the CRs for CA normal PDSCH together (including CRs requirements, FRCs and test applicability rules) in RAN4 #97e (Oct-Nov) meeting |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Way forward on PDSCH CA normal demodulation requirements | Intel Corporation |
| #2 | Simulation assumptions for NR normal CA UE performance requirements | Intel Corporation |
| #3 | Summary of Normal CA simulation results (FR1 15 kHz FDD and TDD) | Intel Corporation |
| #4 | Summary of Normal CA simulation results (FR1 30 kHz TDD) | Intel Corporation |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2007223, Huawei, HiSilicon, | to be revised |

Discussion on 2nd round

Way forward

R4-2008838	Way forward on PDSCH CA normal demodulation requirements

Type: other		For: Approval
					Source: Intel

Abstract:

Discussion:

GTW Session on May.29th

- Issue 2-1-2: Pcell configuration for CA normal PDSCH test

  - For option 1, clarify what does it mean by saying “as per the real testing request”, and how to capture it in the spec.

  - For option 2, clarify what is the issue to use option 2 for UE declaring the support of Pcell configurations in both carriers.

Action points for 2nd round:

- Further check the capability signalling

- For option 1, how to decide the selected test cases?

Email discussion in the 2nd round:

Moderator’s note: The WF, simulation assumptions and simulation result summary are discussed in sub-thread [95e][323] NR_perf_enh_Demod_UE - draft WF R4-2008838 (led by Intel).

- Issue 2-1-2: Pcell configuration for the test

  - Option 1: The test coverage can be considered fulfilled if UE passes one of scenario with one of the CC as PCell as per the real testing request (Intel, QC, HW)

  - Option 2: If Pcell in both carriers are supported, configure TDD cell as Pcell in TDD-FDD CA, configure 15 kHz SCS cell as Pcell in TDD 15+30kHz SCS CA. (scenarios with larger number of HARQ processes). (CTC, CMCC)

  - Option 3: If Pcell in both carriers are supported, configure FDD cell as Pcell in TDD-FDD CA, configure 30 kHz SCS cell as Pcell in TDD 15+30kHz SCS CA. (scenarios with less number of HARQ processes) (QC)

  - Option 4: If PCell in both carriers are supported, configure FDD 15kHz cell as PCell in FDD 15kHz + TDD 15kHz CA, configure 30kHz SCS cell as PCell in both FDD 15kHz + TDD 30kHz CA and TDD 15kHz + TDD 30kHz CA (HW, DCM)

  - Option 5: If Pcell in both carriers are supported, both FDD and TDD cell should be tested as Pcell for TDD-FDD CA and configure 30 kHz SCS cell as Pcell in TDD 15+30kHz SCS CA (DCM)

[Intel]: Many options are available after 1st round discussion. We would like to suggest the following potential option for further discussion:

  - Proposal: The test coverage can be considered fulfilled if UE passes any of supported scenario with one of the CC as PCell

  - Such option allows to avoid scenarios prioritization in RAN4 and leave it up to groups/companies involved in the final testing (TE vendors, operators etc.)

[Huawei] We are ok for the proposal.

[CTC] Firstly, for UE capability, we share the view with Huawei, i.e.,

- For CA with different SCSs, different capabilities are defined for Pcell on larger SCS and Pcell on smaller SCS;

- For FDD + TDD CA with 15 kHz SCS, there is no UE capability defined for TDD Pcell and FDD Pcell.

We still cannot understand what is issue with the following option 2 (at least for scenarios where different capabilities are defined for different Pcell configurations).

  - Option 2: If Pcell in both carriers are supported, configure TDD cell as Pcell in TDD-FDD CA, configure 15 kHz SCS cell as Pcell in TDD 15+30kHz SCS CA. (scenarios with larger number of HARQ processes). (CTC, CMCC)

For example, for TDD 30kHz + FDD 15kHz CA:

- UE #1 supports only TDD 30kHz Pcell configuration, then TDD 30kHz will be configured as Pcell in the test.

- UE #2 supports both TDD 30kHz Pcell and FDD 15kHz Pcell, then probably the UE will select FDD 15kHz as Pcell in the test.

We would also hope to make progress on this issue, since it has been discussed for several meetings. But more detailed technical discussion would be appreciated, or maybe one compromised proposal can be considered?

- For scenarios with different capabilities defined for different Pcell configurations, use option 2.

- For scenarios with no different capabilities defined for different Pcell configurations, use option 1.

[Intel]: We would like to express our view of capability for Pcell on larger SCS and Pcell on smaller SCS. Based on 38.306 we have the following description of diffNumerologyWithinPUCCH-GroupLargerSCS

Indicates whether UE supports different numerology across carriers within a PUCCH group and a same numerology between DL and UL per carrier for data/control channel at a given time in NR CA, EN-DC/NE-DC and NR-DC. In case of NR CA and EN-DC/NE-DC with one NR PUCCH group, the UE supports different numerologies across NR carriers within the same NR PUCCH group up to two different numerologies within the same NR PUCCH group for data and control channel at a given time. [Comment]: Based on our understanding, it indicates whether UE supports different numerologies for scenarios with one NR PUCCH group. In case of NR CA with two NR PUCCH groups, the UE supports different numerologies across NR carriers up to two different numerologies within the same NR PUCCH group, wherein NR PUCCH is sent on the carrier with larger SCS for data and control channel at a given time. [Comment]: Based on our understanding, it indicates whether UE supports different numerologies and NR PUCCH transmission on carrier with larger SCS for scenarios with two NR PUCCH group.

Therefore, our understanding is that diffNumerologyWithinPUCCH-GroupLargerSCS and diffNumerologyWithinPUCCH-GroupSmallerSCS indicate that UE supports PUCCH transmission on CC with larger or smaller SCS only for scenario with two NR PUCCH group.

For scenario with one NR PUCCH, these fields just indicate that UE support different numerologies.

We were thinking that Option 1 will allow to avoid further discussion on scenarios prioritization. At current stage, we have four different options. If we continue discussion on scenarios prioritization then it is not clear how we can reach consensus on this topic due to rather diverse views.

Therefore, our suggestion was to consider option for which all considered scenarios have equal priority from RAN4 specification point of view.

[DCM]: If we go with Option 1, we are not sure what happens in real testing. We think Opt. 1 means that TE vendors needs to choose which one of scenarios should be implemented. In the case, how TE vendors know which is typical one in the siuation that different operators have different prioritization for each sceanrios ? In the sense, we do not understand whether it is really feasible compromise solution for TE vendors and also operators.

So, let me repeat CTC's question. Is there any technical issue/concern with option 2?

As in the 1st round summary, our preference was Option 4 since we would like to set Pcell configuration with typical scenarios that we believe. However, we understand that from UE requirement's perspective, Option 2, which assume the scenarios with larger number of HARQ processes, is more feasible than Option 4. So now we support Option 2.

[CTC2] To Intel, I just checked the capability, it is due to the different versions of 38.306. In f80 version, it says:

In case of NR CA and EN-DC/NE-DC with one NR PUCCH group and in case of NR CA with two NR PUCCH groups, it also indicates whether the UE supports different numerologies across NR carriers within the same NR PUCCH group up to two different numerologies within the same NR PUCCH group, wherein NR PUCCH is sent on the carrier with larger SCS for data and control channel at a given time.

[Intel] @DCM: Our concern to use Option 2 is that only scenarios with certain PCell configuration will be always tested which has rather negative impact on test coverage. Same time, we have a lot of LTE tests with applicability rule, that it is sufficient for UE to pass requirements for any one of supported scenarios, and there was no issue raised for such applicability rule.

@CTC: Thank you very much for clarification. Now, it is clear for us that for CA with different SCSs, different capabilities are defined for Pcell on larger SCS and Pcell on smaller SCS. With this clarification, we are fine to use your compromised solution:

  - For scenarios with different capabilities defined for different Pcell configurations, use option 2.

  - For scenarios with no different capabilities defined for different Pcell configurations, use option 1.

[Intel]: After long discussion, we would like to check if the following proposal can be acceptable for everyone

- Pcell configuration for the test - The test coverage can be considered fulfilled if UE passes any one of scenario with one of the CC as PCell for FDD 15 kHz + TDD 15 kHz - If Pcell in both carriers are supported for FDD 15 kHz + TDD 30 kHz, configure TDD cell as Pcell - If Pcell in both carriers are supported for TDD 15 kHz + TDD 30 kHz, configure 15 kHz SCS cell as Pcell

[CTC]: With this common understanding on the Rel-15 UE capability signalling, we are ok with your proposal.

[Intel]: I’ve uploaded the updated version of WF based on discussion. This WF contains two versions of slides on issues “Pcell configuration for performance requirements” and “Pcell configuration for the test”. I would like to check if version on slide slide 3 is acceptable for everyone. If not then slide 2 will be used in the final WF version.

[QC]: On slide 3, is it possible to have TDD 30kHz as PCell in last bullet because TDD 30kHz PCell will be more typical scenario rather than TDD 15kHz in our view. That is why we picked TDD 30kHz for most of the single carrier test cases.

[Intel]: I don’t have strong view on this topic. I think we need to check if CTC and CMCC is fine with you proposal.

Otherwise, taking into account limited time, probably we can keep two options for this bullet

- Pcell configuration for the test - The test coverage can be considered fulfilled if UE passes any one of scenario with one of the CC as PCell for FDD 15 kHz + TDD 15 kHz - If Pcell in both carriers are supported for FDD 15 kHz + TDD 30 kHz, configure TDD cell as Pcell - If Pcell in both carriers are supported for TDD 15 kHz + TDD 30 kHz, configure - Option 1: 15 kHz SCS cell as Pcell - Option 2: 30 kHz SCS cell as Pcell

[CTC]: Agree with Dmitry, taking into account limited time, for TDD 15 kHz + TDD 30 kHz, probably we can keep two options for now.

[QC]: We are ok to keep two options for now.

[Intel]: Please check the v3 in the e-mail folder.

[Huawei]: It is feasible for UE to pass any one scenario with one of the CC as PCell for FDD 15 kHz + TDD 15kHz, why it is not feasible for UE to pass any one scenario with one of the CC as PCell for other two CA combinations of FDD 15 kHz + TDD 30 kHz and TDD 15 kHz + TDD 30 kHz?

We agree with Qualcomm to choose TDD 30kHz as PCell in CA TDD 15kHz + TDD 30kHz

@ DCM: “If we go with Option 1, we are not sure what happens in real testing. We think Opt. 1 means that TE vendors needs to choose which one of scenarios should be implemented. In the case, how TE vendors know which is typical one in the siuation that different operators have different prioritization for each sceanrios ? In the sense, we do not understand whether it is really feasible compromise solution for TE vendors and also operators.”  My understanding is that the TE vendors needs to implement both scenarios, UE vendors can choose any one of scenario for testing considering the same performance for both scenarios.

[Intel]: I think this proposal, which is captured in the latest WF version, is compromised solution among all companies, originally proposed by CTC:

- For scenarios with different capabilities defined for different Pcell configurations, use option 2 (i.e. selection depending on capability).

- For scenarios with no different capabilities defined for different Pcell configurations, use option 1 (i.e. testing on any one scenario).

Please let me know if you are not OK with this proposal and I come back slide 2 from v2.

[Huawei]: Thanks for your clarification. I just want to reach common understanding that it is feasible:

The test coverage can be considered fulfilled if UE passes any one of scenario with one of the CC as PCell for one CA combinations with different duplex mode or different numerology. Because as per the previous discussions, as if companies have different views on this and then some contradiction for the compromised solution.

Based on such common understanding, the current v3 is fine for us.

[Intel]: We think that sentence which you capture below is feasible, at least for us. Same time, as far as I understand, operators want to prioritize certain scenarios for certain conditions. Therefore, we will get this option for Pcell configuration to take into account views from different companies.

- Issue 2-2-2: HARQ process number for 15kHz SCell in TDD 15 kHz + TDD 30 kHz CA

  - Option 1: 6 (HW)

  - Option 2: 8 (CTC, Intel, QC, CMCC)

[Intel]: We would like to check if proponents of Option 1 can compromise and Option 2 can be used?

[Huawei] As we commented in the 1st round discussion, Option 2 is based on the assumption that initial transmission and retransmission are scheduled on the same type of slots, if we have conclusion on Issue 2-2-1 based on companies’ simulation results that no performance impact by scheduling the initial and retransmission in different types of slots, Option 1 is feasible and has the same assumption with the number of HARQ process for TDD 30kHz SCell.

[CTC]: Same view as in the 1st round. 8 HARQ process is slightly preferred, since the same HARQ process number for 15 kHz SCell is used as when it is configured as Pcell. Option 2 is also acceptable.

[Intel]: We share the similar view as CTC that currently we consider 8 process or PCell HARQ configuration even if 6 is also feasible. Same time, we agreed to use 8 to have consistent configuration with single carrier requirements. Probably it is fine to keep it open for this meeting and let’s try to make conclusion in the next RAN4 meeting if consensus cannot be reached.

- Issue 2-5: Plan for CRs

  - Endorse the draft CR for NR FR1 requirements with 4Rx in this meeting

    - Encourage companies to provide comments for this draft CR in section 2.3.2.

  - Endorse all others CRs on requirements and FRCs in RAN4 #96e (Aug) meeting

    - For CMCC and QC’s CRs, align the structure with Huawei’s draft CR

  - Agree all the CRs for CA normal PDSCH together (including CRs requirements, FRCs and test applicability rules) in RAN4 #97e (Oct-Nov) meeting

[CTC]: Could Intel add the plan for CRs in the WF, which would remind companies the time to bring CRs?

[Intel]: We will add it in the new WF version.

- FR2 simulation summary

[CTC]: For FR2 simulation summary, E/// commented in the 1st round that it looks Intel’s impairment results for 400MHz is typo because it is same as alignment results. Could you double check?

[Intel]: Thanks to E/// for finding the issue with our results. New version is uploaded into e-mail thread folder.

Recommendation:		Agreeable

R4-2008839	Simulation assumptions for NR normal CA UE performance requirements

Type: other		For: Approval
					Source: Intel

Abstract:

Discussion:

Recommendation:		Agreeable

R4-2008840	Summary of Normal CA simulation results (FR1 15 kHz FDD and TDD)

Type: other		For: Information
					Source: Intel

Abstract:

Recommendation:

Recommendation:		Noted

R4-2008841	Summary of Normal CA simulation results (FR1 30 kHz TDD)

Type: other		For: Information
					Source: Intel

Abstract:

Discussion:

Recommendation:		Noted

Draft CRs

R4-2008842	draftCR for NR FR1 PDSCH CA normal demodulation requirements with 4Rx

Type: draftCR		For: Endorsement
					38.101-4 v15.5.0
					Source: Huawei, HiSilicon

Discussion:

Moderator’s note: The draft CR is discussed in sub-thread [95e][323] NR_perf_enh_Demod_UE - draft CR R4-2008842 (led by Huawei).

[Huawei]:

@China Telecomm: I removed the PCell configuration so that they can be captured in the test applicability part if agreeable for companies.

@Qualcomm: I checked the common test parameters definition for SDR for CA in section 5.5A.1, it created new table for the common test parameters and not refer to the common test parameters for single carrier in section 5.2. also like we replied, they belong to different parallel sections.

[QC] We meant that for SDR tests for single carrier and CA, we used single table. Similarly, we should use single table for section 5.2 and 5.2A since all the parameters are same. We don’t understand why we need to duplicate the same table again. Any differences can be captured in test specific parameters, if needed.

[Huawei]: I compared the common test parameters, currently I did not find any different parameters need to be specified, but we can add it later if any test parameters specific to PDSCH CA. as per your comments, I removed the common test parameters table in section 5.2A and refer to section 5.2.  CR v3 is produced by capturing your comments, please double check, and any further comments, please let me know.

@Intel: for the specification structure, currently one table for FDD with 15kHz SCS, TDD with 15kHz SCS and TDD with 30kHz SCS that include the performance requirements for all defined channel bandwidth, if we follow the LTE structure like you proposed, there are many duplicated contents considering we already agree to define the performance requirement per CC and is agnostic to specific PCell type, it is not needed to split it to FDD CA, TDD CA and FDD TDD CA.

[Intel]: Thank you for providing the view on our suggestion for section structure. We are fine to keep structure captured in your draft CR. We just wanted to check companies view.

Another comment from 1st round discussion, we were thinking that information about “Propagation condition” and “Correlation matrix and antenna configuration” can be captured only in Table 5.2A.3.1-1 instead of duplication of this information in multiple tables with minimum requirements. Please let us know what do you think about such procedure

[Huawei]For your another comments, I replied in the CR as inline note.

I prefer to keep them in the minimum requirements table to align with the structure of other test cases in the specification, and it also leave flexibility to add other possible cases with different propagation condition and/or correlation matrix and antenna configuration in the future.

I am not sure if you have very strong preference to move them into Table 5.2A.3.1-1?

[Intel]: I don’t have strong preference and just had this suggestion to make tables with minimum requirements more compact. We are fine to keep it as it is.

[QC] We agree with Huawei comment to align minimum requirements table with other tests in the specification.

[QC]: One minor comment: “The number of slots between PDSCH and corresponding HARQ-ACK information” will be different depending on number of HARQ processes used. For example, TDD 15kHz could have 4 or 8 HARQ processes depending on the scenario. So, we will have to define different k1 for same TDD pattern based on number of HARQ processes. We are ok to leave it as TBD in this meeting until those k1 values are defined in Annex for each scenario. Also, there is “Note 1” in table for number of HARQ processes which is not specified in the draft CR. That note needs to be specified. I assume all open issues such as number of HARQ processes for TDD 15kHz in TDD15kHz+30kHz scenario will be replaced by TBD in the final version since this is a draft CR?

[Huawei]:

1: K1 values: if we follow the approach defined in NR Rel-15, definition in Annex will be needed for the specific scheduling information, as you said the K1 value will be different for same TDD pattern but with different number of HARQ processes, maybe they can be captured in one table for one SCS, let’s try the detailed definition for K1 in next meeting, for this meeting, I just leave the specific table FFS, but I am ok to put TBD.

2: Information for Note 1 is FFS

3: TBD replaced the option for number of HARQ process for TDD 15kHz CC in CA TDD 15kHz + 30kHz.

[QC]: Thank you for updated CR. This version is ok for us.

Recommendation:		Endorsed

Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2008838, WF | Agreeable |
| R4-2008839, Simulation assumptions | Agreeable |
| R4-2008840, FR1 15kHz simulation result summary | Noted |
| R4-2008841, FR1 30kHz simulation result summary | Noted |
| R4-2008842, draft CR for FR1 4Rx | Endorsed |

Topic #3: UE	PMI reporting requirements with larger number of Tx ports

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006038 | China Telecom | For type I PMI: - Proposal 1: Concerning whether to introduce subband PMI requirements for 16 Tx ports, encourage companies to consider compromised proposals so as to make decision in this meeting. - Proposal 2: For 32 Tx wideband, set gamma (gain) values as 5.0 and 8.0 for 2Rx and 4Rx respectively. For type II PMI: - Proposal 3: Use SU-MIMO test setup. - Proposal 4: Use 16Tx ports with (N1,N2) = (4,2), (O1, O2) = (4,4) to reduce the test complexity. Proposal 5: For beam steering model, ok with either option 2 or option 3. |
| R4-2006318 | Samsung | - Proposal 1: Define the PMI requirement of Rel-15 type II codebook construction as - phaseAlphabeSize: Npsk= 4 - subbandAmpltitude: false - PMI-FormatIndicator: Wideband |
| R4-2006615 | Qualcomm Incorporated | Simulation results for Type I PMI Proposal 1: Define subband Type -I PMI reporting requirements for 16 Tx ports. Proposal 2: Use SU-MIMO test setup for defining Type II PMI reporting tests. Proposal 3: Use subband PMI reporting for defining Type II PMI reporting tests. Proposal 4: Define Type II PMI reporting requirements with N_PSK = 8 and subbandAmplitude = true Proposal 5: Define Type II PMI reporting requirements for only 16Tx ports. Proposal 6: Define Type II PMI reporting requirements for XP High MIMO correlation. - Proposal 7: Discuss extension of beam steering approach to more than 2 clusters under eMIMO WI and use the 2 cluster beam steering approach from 36.101 for defining Type II PMI reporting requirements under NR performance enhancement WI. |
| R4-2007201 | Huawei, HiSilicon | Simulation results |
| R4-2007202 | Huawei, HiSilicon | Proposal 1: Prefer to use SU-MIMO for test setup Proposal 2: Prefer only introduce 16 Tx ports requirements for Type II codebook Proposal 3: Use (N1, N2) = (4, 2) and (O1, O2) = (4, 4) for 16 Tx ports Proposal 4: Prefer to use equation listed in the last slide of the Way forward as beam steering model Proposal 5: 4 for Npsk Proposal 6: False for SubbandAmplitude Proposal 7: Wideband for PMI-FormatIndicator |
| R4-2007203 | Huawei, HiSilicon | Proposal 1: Not to define Subband PMI requirements for 16Tx ports and covers 16 Tx port requirements with Wideband |
| R4-2007927 | Ericsson | Simulation results. Observation 1: PMI reporting throughput curves do not differ between wideband and Subband PMI reporting. Proposal 1: From a test coverage point of view, we think introducing subband PMI for 16Tx ports can be agreeable. |
| R4-2007928 | Ericsson | Summary of simulation results of NR UE CSI PMI with 16, and 32Tx antennas |
| R4-2007934 | Ericsson | Proposal 1: Use Equation 1 as beam steering model for Type II codebook performance requirements. Observation 1: Gain metric Follow Type II PMI over Random Type II PMI does not verify that L number of beams are transmitted for MU-MIMO support. Observation 2: there is marginal gain when comparing SP Type I with Type II codebook with the current SU-MIMO based test setup. Observation 3: Gain test metric γ by following Type II PMI over SP Type I PMI does see marginal gain for few channel models and channel correlations. Proposal 2: Design Type II tests to ensure UE CSI reporting with substantially better performance than Type I reporting for MU-MIMO, in line with the big performance benefits shown in RAN1 evaluations. Proposal 3: If RAN4 agree to use multi-user scheduling for type-II PMI reporting test, RAN4 study further how to derive precoder based on the type-II PMI feedback from UE under test. Proposal 4: Test parameters for Type II codebook may need to be tuned to properly suit MU-MIMO based test setup proposed. |
| R4-2007935 | Ericsson | Simulation results for Rel-15 Type II codebook |

Open issues summary

Sub-topic 3-1: Type I PMI test

Issue 3-1-1: Whether to introduce subband PMI test for type I single-panel codebook

- Agreement in RAN4 #94e-bis (R4-2005549, WF)

  - PMI requirements for 16 Tx ports (decision in RAN4#95-e)

    - Option 1: Introduce subband PMI requirements for 16 Tx ports as baseline

    - Option 2: Not introduce subband PMI requirements for 16Tx ports and covers 16Tx port requirements with wideband PMI

- Proposals

  - Option 1: Introduce subband PMI requirements for 16 Tx ports as baseline (Qualcomm, Ericsson)

  - Option 2: Not introduce subband PMI requirements for 16Tx ports and covers 16Tx port requirements with wideband PMI (Huawei)

- Recommended WF

  - Since it has been agreed to make decision in RAN4 #95-e, encourage companies to consider if one of the following compromised solutions is acceptable, and any other possible compromised solution are welcome:

    - Solution A: Define subband PMI requirement for type I 16Tx, and wideband PMI requirement for type II

      - Use TDLC300-5 channel for type I 16Tx subband

    - Solution B: Define wideband PMI requirement for type I 16Tx, and subband PMI requirement for type II

      - Use TDLA30-5 channel for type I 16Tx wideband

Issue 3-1-2: Gamma (gain) values for Type I PMI test

- Previous Agreements

  - Agreements in RAN4 #92bis (R4-1912834, WF)

    - Test metric: Relative throughput ratio between following PMI and random PMI at SNR point corresponding to 90% TP with follow PMI

  - Agreement in RAN4 #94e-bis (R4-2005549, WF)

    - Set gamma (gain) values based on simulation results in RAN4#95-e

- Summary of simulation results

  - R4-2007928, Summary of simulation results of NR UE CSI with 16, and 32Tx antennas

- Proposals on Gamma (gain) values

  - For 32 Tx wideband:

    - Option 1: 5.0 for 2Rx, 8.0 for 4Rx (CTC)

  - For 16 Tx wideband/subband:

    - FFS

- Recommended WF

  - For 32 Tx wideband:

    - Encourage companies to provide feedback on the above option 1.

  - For 16 Tx wideband/subband:

    - Discuss after issue 3-1-1 is addressed

Sub-topic 3-2: Type II PMI test setup

Issue 3-2-1: Test setup for type II

- Agreement in RAN4 #94e-bis (R4-2005549, WF)

  - Test setup:

    - Option 1: Use SU-MIMO test setup as baseline scenario

    - Option 2: MU-MIMO based test setup

- Proposals

  - Option 1: Only use SU-MIMO test setup (CTC, Qualcomm, Huawei)

    - CTC: 1) UE processing will not be different if we change the test setup from SU-MIMO to MU-MIMO. 2) Practical MU-MIMO scenario is hard to be reflected in the test. 3) We will need to re-design many test parameters if MU-MIMO is agreed.

    - QC: RAN4 is discussing to define minimum requirements for UE and UE implementation is unaware of whether it is SU-MIMO setup or MU-MIMO setup.

    - Huawei: There is a similar scenario in the WI of LTE eFD-MIMO that MU-MIMO setup had been well discussed and finally not happened to the requirements.

  - Option 2: MU-MIMO based test setup (Ericsson, Verizon)

    - Design Type II tests to ensure UE CSI reporting with substantially better performance than Type I reporting for MU-MIMO, in line with the big performance benefits shown in RAN1 evaluations.

    - If RAN4 agree to use multi-user scheduling for type-II PMI reporting test, RAN4 study further how to derive precoder based on the type-II PMI feedback from UE under test.

    - Test parameters for Type II codebook may need to be tuned to properly suit MU-MIMO based test setup proposed.

    - Observations from our simulation results

      - Type II codebook provides larger benefits for MU-MIMO based transmission scheme

- Recommended WF

  - Taking into account the discussions in the recent two meetings, can we go with option 1?

Sub-topic 3-3: SU-MIMO Type II PMI test parameters

Issue 3-3-0: Summary of companies’ simulation results

- Summary of companies’ simulation results for Type II 16T2R PMI under TDLA30-5 (for information)

| Duplex Mode | MIMO Correlation | NPSK | subbandAmplitude | PMI-FormatIndicator | SNR point @90%TP (dB) / TP ratio |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Samsung | Qualcomm | Ericsson |
| FDD | XP Medium | 4 |  |  | 15.9 |  |  |
|  | XP Medium | 8 |  |  | 15.5 |  |  |
|  | XP Medium |  | False |  | 15.9 |  |  |
|  | XP Medium |  | True |  | 15.8 |  |  |
|  | XP Medium |  |  | Wideband | 16.8/3.997 |  |  |
|  | XP Medium |  |  | Subband | 15.9/1051.5 |  |  |
|  | XP High | 4 | False |  |  |  | 9.82/3.81 |
| TDD | XP Medium | 8 | True | Subband |  | 11.13/3.19 |  |
|  | XP High | 4 | True | Subband |  | 13.57/3.04 |  |
|  | XP High | 4 | False | Subband |  | 13.80/2.99 |  |
|  | XP High | 8 | True | Subband |  | 10.04/5.88 |  |
|  | XP High | 8 | False | Subband |  | 10.29/5.49 |  |

Issue 3-3-1: Type II codebook construction

- Agreement in RAN4 #94e-bis (R4-2005549, WF)

  - Codebook construction

    - Option 1: 16Tx ports (N1,N2) = (4,2), (O1, O2) = (4,4)

    - Option 2: 32Tx ports (N1,N2) = (4,4), (O1, O2) = (4,4)

- Proposals

  - Option 1: 16Tx ports (N1,N2) = (4,2), (O1, O2) = (4,4) (CTC, Qualcomm, Huawei)

    - CTC: Ok with option 1 to reduce the test complexity.

    - QC: 32Tx ports provide too large throughput ratios compared to 16Tx ports..

    - Huawei: 16 Tx ports is more typical than 32 Tx ports in practice, for it is supported by more UE and it was chosen to be defined requirements for LTE MIMO in Rel-14.

- Recommended WF

  - Select option 1

Issue 3-3-2: Npsk  (phaseAlphabetSize) for type II codebook construction

- Agreement in RAN4 #94e-bis (R4-2005549, WF)

  - Npsk (phaseAlphabetSize)

    - Option 1: 4

    - Option 2: 8

- Proposals

  - Option 1: 4 (Samsung, Huawei)

    - Samsung: The Type II performance have limited differences with ‘NPSK=4’ and ‘NPSK=8’

    - Huawei: QPSK is more typical and will be used much usual than 8PSK.

  - Option 2: 8 (Qualcomm)

    - QC: Based on simulation results, N_PSK = 8 and subbandAmplitude = true provide the best throughput ratios.

- Moderator’s observation:

  - This issue has been discussed for 3 meetings.

  - 2 companies (Samsung, Qualcomm) provided simulation results from the last meeting, with different simulation observations and proposals; and no more simulation results provided in this meeting.

- Recommended WF

  - Can we make decision based on majority companies’ view after the 1st round discussion?

Issue 3-3-3: subbandAmplitude for type II codebook construction

- Agreement in RAN4 #94e-bis (R4-2005549, WF)

  - SubbandAmplitude

    - Option 1: False

    - Option 2: True

- Proposals

  - Option 1: False (Samsung, Huawei)

    - Samsung: The gain is marginal if ‘SubbandAmplitude=True’ under TDL-A XP Medium correlation channel.

    - Huawei: Much more number of candidate codebooks and parameters are introduced for Random PMI when True is configured, which lead to bigger ratio than that of configuring False. Thus, we do not think configuring True for SubbandAmplitude will improve the performance.

  - Option 2: True (Qualcomm)

    - QC: Based on simulation results, N_PSK = 8 and subbandAmplitude = true provide the best throughput ratios.

- Recommended WF

  - Similar situation as Npsk , can we make decision based on majority companies’ view after the 1st round discussion?

Issue 3-3-4: PMI-FormatIndicator for type II codebook

- Agreement in RAN4 #94e-bis (R4-2005549, WF)

  - PMI-FormatIndicator

    - Option 1: Wideband

    - Option 2: Subband

- Proposals

  - Option 1: Wideband (Samsung, Huawei)

    - Samsung: The corresponding TP ratio among following PMI and random PMI under sub-band PMI set-up is extremely high, which not feasible to introduce proper performance requirements.

    - Huawei: Configuring subband may result in bad performance for the throughput of Random PMI as the codebooks and parameters are significantly increased.

  - Option 2: Subband (Qualcomm)

    - QC: It makes more sense to have Subband PMI reporting for Type II codebook so that this codebook can be used to its full potential.

- Recommended WF

  - This can be discussed together with issue 3-1-1 on whether to test subband PMI for Type I codebook.

  - Regarding the issue of extremely high TP ratio for subband PMI raised by Samsung, companies are encouraged to check if this is a common issue for different simulators. If yes, potential solutions to address this issue can be discussed later, for example, use wideband random PMI as baseline.

Issue 3-3-5: MIMO correlation for type II codebook

- Agreement in RAN4 #94e-bis (R4-2005549, WF)

  - MIMO correlation

    - Option 1: XP High

    - Option 2: XP Medium

- Proposals

  - Option 1: XP High (Qualcomm, Huawei)

    - QC: We have defined other PMI reporting tests with XP High correlation and based on results, XP High provides better performance than XP Medium correlation.

    - Huawei: 1) XP High can achieve better performance gain between follow PMI and random PMI. 2) We have defined test cases with XP high before (in Type I codebook), and it is comparable for defining the same MIMO correlation in Type II codebook.

- Recommended WF

  - Can we agree to use option 1 as baseline?

Issue 3-3-6: Beam steering model for Type II Codebook

- Agreement in RAN4 #94e-bis (R4-2005549, WF)

  - Beam steering model

    - Option 1: Reusing beam steering approach with dual-cluster beams as specified in B.2.3B.4A of TS 36.101

    - Option 2: Use Equation 1 as beam steering model for Type II codebook performance requirements.

    - Option 3: Use option 1 if L = 2, and use option 2 if L > 2

Equation 1 (for information) y=11+i=2Lpi2HmDθk,1,θk,2(m)+i= 2Lpi21+i=2Lpi2HiDθk,1,θk,2(i)Wx+n And the steering matrix is further expressed as following: where - Hm, Hi are independent channels for the first beam and the consecutive i beams with the Nr x Nt channel matrix per subcarrier. - pi is the relative power difference from the first beam. - , Dθk,1,θk,2(i)are the steering matrix for first beam and consecutive i number of beams - is the steering matrix in first dimension with same polarization, - is the steering matrix in second dimension with same polarization, - is the number of antenna elements in first dimension with same polarization, - is the number of antenna elements in second dimension with same polarization,

- Proposals

  - Option 1: Reusing beam steering approach with dual-cluster beams as specified in B.2.3B.4A of TS 36.101 (Qualcomm)

    - QC: Propose to discuss extension of beam steering approach to more than 2 clusters under eMIMO WI.

  - Option 2: Use Equation 1 as beam steering model for Type II codebook performance requirements (CTC, Huawei, Ericsson)

    - Huawei: 1) The beam steering model should be suitable for more number of beams as introduced in Rel-15. 2) Extended beam steering model can be reused in Rel-16 NR eMIMO and easy for comparison in the future.

  - Option 3: Use option 1 if L = 2, and use option 2 if L > 2 (CTC)

- Recommended WF

  - Can we select option 2 by following majority’s view?

Sub-topic 3-4: MU-MIMO Type II PMI test parameters

Issue 3-4-1: Test metric for MU-MIMO Type II PMI

- Agreement in RAN4 #94e-bis (R4-2005549, WF)

  - Test metric

    - Option 1: TP ratio between following PMI and random PMI

    - Option 2: TP ratio between following Type II codebook and following SP Type I codebook

    - Other options are not precluded

- Proposals

  - Option 1: TP ratio between following PMI and random PMI

  - Option 2: TP ratio between following Type II codebook and following SP Type I codebook

- Recommended WF

  - TBA

Sub-topic 3-5: Plan for CRs

Issue 3-5: Plan for CRs

- Agreed CR work split for PMI reporting requirements

|  |  | CR Responsibility |
| --- | --- | --- |
| Tx ports larger than 8 and up to 32 | Applicability | Huawei |
|  | Requirements for type I single-panel codebook | Ericsson |
|  | Requirements for type II codebook | Samsung |
|  | FRC for type I single-panel codebook | Ericsson |
|  | FRC for type II codebook | Samsung |

  - In this meeting, Ericsson provided draft CR for type I tests, type I FRC and MIMO Correlation Matrices for 2D antenna arrays.

- Recommended WF

  - Endorse the draft CRs for type I tests, type I FRC and MIMO Correlation Matrices for 2D antenna arrays in this meeting

    - Encourage companies to provide comments for these draft CRs in section 3.3.2.

  - Endorse the draft CRs for type II and applicability in RAN4 #96e (Aug) meeting

  - Agree all the CRs for PMI reporting requirements together (including CRs for applicability, type I and type II) in RAN4 #97e (Oct-Nov) meeting

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| China Telecom | Sub-topic 3-1: Type I PMI test Issue 3-1-1: Whether to introduce subband PMI test for type I single-panel codebook Agree with the recommended WF for the sake for progress. Either solution A or B is ok for us. Sub-topic 3-2: Type II PMI test setup Issue 3-2-1: Test setup for type II Agree with the recommended WF. Sub-topic 3-3: SU-MIMO Type II PMI test parameters Issue 3-3-1: Type II codebook construction Agree with the recommended WF. Issue 3-3-6: Beam steering model for Type II Codebook Agree with the recommended WF. Sub-topic 3-5: Plan for CRs Issue 3-5: Plan for CRs Agree with the recommended WF. |
| Qualcomm | Sub-topic 3-1: Type I PMI test Issue 3-1-1: Whether to introduce subband PMI test for type I single-panel codebook At this point, our preference is to define subband PMI requirements for both the cases. For Type 1, it is for coverage perspective. For Type 2, structure of codebook itself is more beneficial for subband reporting compared to WB. We need more time to check if any of Solution A or Solution B is ok with us. Issue 3-1-2: Gamma (gain) values for Type I PMI test For 32Tx, based on Ericsson’s results, this proposal won’t work. So, we prefer to wait until next meeting to agree to this so that companies get enough time to double check their simulation results. Sub-topic 3-2: Type II PMI test setup Issue 3-2-1: Test setup for type II Ok with Option 1. Sub-topic 3-3: SU-MIMO Type II PMI test parameters Issue 3-3-1: Type II codebook construction Ok with recommended WF. Issue 3-3-2: Npsk (phaseAlphabetSize) for type II codebook construction Samsung’s simulations were run with WB reporting while our simulations were with subband reporting. So, that may be the reason that Samsung doesn’t see much gains with N_PSK = 8. We don’t understand Huawei’s comment of N_PSK = 4 being more typical. In our understanding, N_PSK = 8 will be more robust to quantization errors. Except Samsung and Qualcomm, no other company has provided any simulation results. Also, it may be possible that subband and WB reporting may show different gains. So, may be , we should wait until we decide on WB vs SB reporting and then, based on simulation results, we can make a decision. Therefore, at this point, we don’t prefer to make agreements based on majority. Our preference is option 2. Issue 3-3-3: subbandAmplitude for type II codebook construction In this case, we see slight gain with Option 2. So, we don’t have any strong preference on this one, but we slightly prefer Option 2. Issue 3-3-4: PMI-FormatIndicator for type II codebook Based on our simulations, we don’t see large throughput ratios with subband PMI reporting for 16Tx ports as shown in our paper. Only Samsung has provided simulation results apart from us. So, we encourage more companies to provide simulation results to make a decision on this. Also, in our opinion, Type II reporting is more useful for subband PMI reporting since it inherently has parameters related to subband reporting, even if reporting is WB. Therefore, we prefer Option 2. Issue 3-3-5: MIMO correlation for type II codebook Ok with recommended WF. Issue 3-3-6: Beam steering model for Type II Codebook We have already agreed to use L = 2 for Type II codebook. Therefore, we suggest to agree to Option 1 and discuss the extension under eMIMO WI. Our concern is two-fold with extending the model: - We don’t need to have same number of clusters in beam steering model as L parameter in codebook because in practice, it is very unlikely that UE will receive the equal power signals from more than 2 independent directions. - In Rel-15, TE vendors had mentioned a limit of 12 taps in channel realization. In case of beam steering, if we use 4 clusters, TE will have to generate 4 independent channel realizations of 12 taps each. So, they can either realize only 3 taps per channel that will be very poor implementation of channel model or they will have to surpass their limitations and that would mean more expensive test box. Therefore, we suggest to limit beam steering model to 2 clusters even if we use L > 2 as codebook parameter. |
| Huawei, HiSilicon | Sub-topic 3-1: Type I PMI test Issue 3-1-1: Whether to introduce subband PMI test for type I single-panel codebook As mentioned before and also evaluated by companies, no gain has been observed on subband over wideband for Type I codebook. And based on the previous agreements, it is reasonable to not define any requirements for subband in Type I codebook tests. If we follow the logic raised by some companies to ensure coverage, it means that RAN4 should define performance requirements for all core requirements, but actually RAN4 never define performance requirements for all features, it is the meaning of RAN4 group function to figure out useful and practical cases for UE verification. Issue 3-1-2: Gamma (gain) values for Type I PMI test We plan to update some of our simulation results in the summary sheet. Sub-topic 3-2: Type II PMI test setup Issue 3-2-1: Test setup for type II We think it is complicated for testing in MU-MIMO setup. And much more uncertainty will be introduced like UE grouping algorithms, which may depend on the specific UE implementation. Given that complexity of defining requirements under the setup for MU-MIMO, we prefer option 1. Sub-topic 3-3: SU-MIMO Type II PMI test parameters Issue 3-3-1: Type II codebook construction We agree with the recommended WF. Issue 3-3-2: Npsk (phaseAlphabetSize) for type II codebook construction Since there are only two companies that have provided the simulation results for this parameter, we prefer to further evaluate it and decide after we make a decision on the PMI-FormatIndicator. Issue 3-3-3: subbandAmplitude for type II codebook construction Big performance ratio was observed. But the performance of following PMI is not much improved by configuring ‘true’. Thus, we prefer option 1. Issue 3-3-4: PMI-FormatIndicator for type II codebook We prefer option 1. As per UE capability parameters codebookParameters clarified in TS 38.306: Parameters for type II codebook (type2) supported by the UE, which are optional: the “amplitudeScalingType indicates the amplitude scaling type supported by the UE (wideband or both wideband and sub-band);”, it means that wideband should be supported anyway if UE supports Type II codebook. Also as some companies evaluated, no gain for subband. Issue 3-3-5: MIMO correlation for type II codebook We prefer option 1. Issue 3-3-6: Beam steering model for Type II Codebook Extending the beam steering model is beneficial and it can be more flexible after the modification. We still can configure L = 2 as we agreed before for Type II codebook testing. We don’t see the harm of introducing it. Thus, we support option 2. |
| Ericsson | Sub-topic 3-1: Type I PMI test Issue 3-1-1: Whether to introduce subband PMI test for type I single-panel codebook We do not think that we should mix up SP Type I requirements with Type II requirements. They serve different purposes and thus should not be discussed under the same issue. Type II is designed for MU-MIMO which should not be compared with the SU-MIMO setup from SP Type I. Regarding SP Type I requirements, we are ok with both wideband and subband PMI reporting. Issue 3-1-2: Gamma (gain) values for Type I PMI test We have uploaded the alignment sheet in the drafts folder and encourage companies to fill in their values for gain into the summary sheet which was added to get a better overview over all SP Type I test cases. Sub-topic 3-2: Type II PMI test setup Issue 3-2-1: Test setup for type II We prefer Option 2 for a number of reasons: - Type II codebook was designed intended for fundamental MU-MIMO support (see excerpt below from WID RP-182067): - 3 Justification The Rel-15 NR includes a number of MIMO features that facilitate utilization of a large number of antenna elements at base station for both sub-6GHz and over-6GHz frequency bands. Some of these features are primarily based on Rel-14 LTE while others are introduced due to several newly identified deployment scenarios such as multi-panel arrays, hybrid analog-digital for high frequency bands. In particular, the following MIMO features are included: limited support for multi-TRP/panel operation, flexible CSI acquisition and beam management, Type I (low-resolution) and II (high-resolution) codebooks supporting up to 32 ports, and flexible RS for MIMO transmission (especially CSI-RS, DMRS, and SRS). Equipped with such features, NR MIMO can differentiate itself from LTE MIMO at least in the following aspects. First, Type II codebook can offer substantial (at least 30%) gain in average user throughput over the best of Rel-14 LTE. Second, flexible CSI acquisition and RS design permit scalability for future enhancements. Third, NR MIMO accommodates operation in high frequency bands (>6GHz) via beam management. Overall, the Rel-15 MIMO features offer ample foundation for further potential enhancements which can be unlocked in Rel-16 NR. Such enhancements include the following. First, although Type II CSI specified in Rel-15 offers large gain over advanced CSI of Rel-14 LTE, there is still some significant, yet attainable, performance gap from near-ideal CSI especially for multi-user (MU)-MIMO. Second, although Rel-15 NR MIMO provisionally accommodates multi-TRP/panel operation, the supported features are limited to standard-transparent transmission operations and small number of TRPs/panels. Third, although specification support for multi-beam operation has been largely specified in Rel-15 (targeting over-6GHz frequency band operation), some aspects such as beam failure recovery and enabling schemes for DL/UL beam selection are fairly basic and can potentially be improved for increased robustness, lower overhead, and/or lower latency. Fourth, there is a need for enhancement to allow full power transmission in case of uplink transmission with multiple power amplifiers. - 4 Objective - 4.1 Objective of SI or Core part WI or Testing part WI The work item aims to specify the enhancements identified for NR MIMO. The detailed objectives are as follows. - Extend specification support in the following areas [RAN1] - Enhancements on MU-MIMO support: - Specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead - Perform study and, if needed, specify extension of Type II CSI feedback to rank >2. - Type II is clearly intending to target much improved MU-MIMO operation, not SU-MIMO. For SU-MIMO network operation, Type II would not be needed and Type I would be sufficient. A test must hence ensure that Type II provides clear benefits over Type I for the intended MU-MIMO operation, not for an unintended operation like SU-MIMO. It is crucial that Type II reports can be used for efficient fast fading based nullforming towards victim UEs. - Type II sees large performance benefits over Type I in RAN1 evaluations for MU-MIMO. We have seen negative to marginally positive performance gains for SU-MIMO based simulations comparing Type II with Type I. This has also been observed in simulations from other companies for SU-MIMO throughput comparisons. - In our simulation papers for Type II codebook we see very large overall MU-MIMO gains for 4 User average throughput over Type I. - If RAN4 agree to do a proper type-II PMI reporting test that ensures good fast fading based nullforming capability from gNB side, RAN4 study further how to derive precoder based on the type-II PMI feedback from UE under test. Note that such a test can be made simple and only involve a single UE under test by faking transmissions to a ghost UE. Sub-topic 3-3: SU-MIMO Type II PMI test parameters Issue 3-3-1: Type II codebook construction We prefer Option 2: 32Tx port codebook construction from a feature testing perspective. The argument that the throughput ratio for 32Tx ports is too large stem from the fact that with increasing the Tx ports and the Rx ports means that the number of random channels (random feedback) increase manifold. Arguing that the throughput ratio is too large to be tested solely means that the test metric is poorly designed from a performance perspective. If we introduce features which we cannot design performance metrics for, then that sets a bad precedence for performance requirements. Issue 3-3-2: Npsk (phaseAlphabetSize) for type II codebook construction We prefer Option 2. When SU-MIMO configuration is considered, the performance difference will be negligible. but for MU-MIMO, which is the use case that should be tested here, using option 2 is important. Issue 3-3-3: subbandAmplitude for type II codebook construction We prefer Option 2. When SU-MIMO configuration is considered, the performance difference will be negligible. but for MU-MIMO, which is the use case that should be tested here, using option 2 is important. Issue 3-3-4: PMI-FormatIndicator for type II codebook We prefer Subband PMI reporting Option 2. Not having subband reporting means Type I cannot do fast fading based nullfomirng to victim UEs, which is the whole point of Type II. Issue 3-3-5: MIMO correlation for type II codebook We prefer option two, medium correlation. In high correlation, the difference between Type I and Type II is smaller, it is better to target the rich scattering channel which is more suitable for MU-MIMO. Sub-topic 3-4: MU-MIMO Type II PMI test parameters Issue 3-4-1: Test metric for MU-MIMO Type II PMI We prefer to measure the TP ratio between following Type II codebook with Type I codebook to fully see the performance benefits of Type II codebook for MU-MIMO scenario. Random precoding as a reference does make much sense. The more faders used; more incorrect precoders will be available to choose from, thus creating even worse performance. Sub-topic 3-5: Plan for CRs Issue 3-5: Plan for CRs We Agree with the sentiment to try for draft CRs for Type II PMI for RAN4#96, but given that we are far from aligned for simulation assumptions it will be difficult to endorse CRs for August with agreed Type II parameters. |
| Orange | Sub-topic 3-2: Type II PMI test setup Issue 3-2-1: Test setup for type II We support option 2 proposed by Ericsson since Type I yields similar performance to type II for SU-MIMO throughput. Therefore, SU-MIMO throughput performance should not be the tested metric. It is very important to ensure the right performance for MU-MIMO with codebook type II, i.e., to ensure that the UE feedback is the closest to the eigenvectors of the MIMO channel. |
| Verizon | Sub-topic 3-2: Type II PMI test setup Issue 3-2-1: Test setup for type II - We agree with Ericsson’s proposal (i.e., Option 2: MU-MIMO based test setup) above. Type-II codebook feedback is critical for FDD MU-MIMO operation and we would like to have a test to ensure proper implementation because based on our experience with LTE, MU-MIMO gain can only be achieved with the details properly taken care of. We also found that the performance is highly dependent on the presence of co-served UE (good scheduling algorithm with true MU feedback than not only magnifies the signal but also suppresses that interference). In order for gNB to have some confidence when it wants to group the UE in a certain spatial-related way, it should have some assurance on how UE will behave in this situation. We understand it will be more complicated to specify the test for a MU test set up, but we think it is well worth the effort because otherwise, a UE would have no incentive to look beyond the signal to itself. |
| Apple | Sub-topic 3-1: Type I PMI test Issue 3-1-1: Whether to introduce subband PMI test for type I single-panel codebook We support option 1 and to introduce requirements with subband PMI. We think it is necessary for coverage and don’t believe that it is a non-essential feature. It is a mandatory feature in Rel-15 and since we didn’t have any requirements defined for subband PMI reporting in Rel-15 it should be introduced at least with 16TX. Sub-topic 3-2: Type II PMI test setup Issue 3-2-1: Test setup for type II We support option 1. We understand that Type II codebook has been introduced for MU-MIMO scenario, but we don’t think UE performance will be affected by the set-up. We are defining requirements for PMI reporting with Type II codebook, the PMI computation should be the same for both MU-MIMO and SU-MIMO setup. Sub-topic 3-3: SU-MIMO Type II PMI test parameters Issue 3-3-1: Type II codebook construction We support the recommended WF. Issue 3-3-2: Npsk (phaseAlphabetSize) for type II codebook construction We support option 2 – NPSK = 8 Issue 3-3-3: subbandAmplitude for type II codebook construction We support option 1 – subbandAmplitude set to False Issue 3-3-4: PMI-FormatIndicator for type II codebook We would prefer not to combine it with Type I 16TX subband discussion. We should choose a mode that is practical and used in deployment. Issue 3-3-5: MIMO correlation for type II codebook We support the recommended WF. Issue 3-3-6: Beam steering model for Type II Codebook We support the recommended WF. Sub-topic 3-4: MU-MIMO Type II PMI test parameters Issue 3-4-1: Test metric for MU-MIMO Type II PMI Sub-topic 3-5: Plan for CRs Issue 3-5: Plan for CRs |
| Samsung | Sub-topic 3-1: Type I PMI test Issue 3-1-1: Whether to introduce sub-band PMI test for type I single-panel codebook In general, we should not mix up Type I and Type II test cases as separate UE features. Companies are repeating comments from previous agreements. Introducing sub-band PMI test cases is not conflicting with previous agreements. In previous meeting, we agreed to keep it open for further evaluation. Regarding performance gain with sub-band PMI over wideband PMI, according to all the companies’ results, the performance are difference (how much gain it is, that’s argued-able). We are not intended to introduce test cases with test metric: following sub-band PMI/following wideband PMI. If it is the intention, we are open to discuss how to design the test case to differentiate UE behaviour with sub-band PMI and wideband PMI. However, current test metric is between following PMI and random PMI, we believe no need to have larger performance gap to distinguish UE behaviour. The benefit of using sub-band PMI will have more test coverage without additional test cases. The key point from RAN4 UE performance requirements is always to introduce limited test cases with enough test coverage. If there is technical UE implementation issue for the proposed sub-band PMI test case, we are open to discuss that, till now we didn’t see any issues for that. To be noted, per RAN1 Rel-15 UE feature list, number of supported ports belong to UE capability, meanwhile no distinction for sub-band or wideband; which means if UE declare support 16 tx port Type I single panel, UE always need to support both wideband and sub-band reporting. In summary, for sake of progress we can accept to either not introduce 16 Tx ports test cases or 16Tx sub-band PMI test case. 16 Tx port wideband PMI test cases probably redundant as we already agreed to introduce 32 Tx port wideband PMI test cases. Sub-topic 3-2: Type II PMI test setup Issue 3-2-1: Test setup for type II We are glad to see more companies show more interest on this feature. First, I would like to clarify no doubt we agree Type II codebook can be used for MU-MIMO scheduling in RAN1 design major usage scenario and purpose of introducing such codebook. Secondly, from RAN4 perspective the purpose of introducing UE PMI test cases was to verify UE receiver properly processing and report PMI accurately according RAN1 codebook structure. No matter what test set-up we agreed in the end, as well as we serve above test purpose well, we can guarantee NW can schedule UE with Type II codebook quite well with MU-MIMO scenarios and or SU-MIMO scenarios. Thirdly, following normal RAN4 performance requirements work procedure, RAN4 target to introduce test cases for certain physical layer features i.e. Type II codebook, the deployment and usage scenarios is upper to NW scheduling. No matter what test set-up we agreed in the end, there is no restriction on NW scheduling, both MU-MIMO and SU-MIMO scheduling allowed. Then go to the detailed options, I have to say probably current wording for the candidate options of test set-up (SU-MIMO set-up and MU-MIMO set-up) quite misleading and scary, probably that’s the source of draw over-attentions from companies . I would like to suggest to change the wording as: - Option 1: One tested UE - Option 2: One tested UE + a co-schedule UE (generated by TE) Based on our evaluation results, option 1 already serves test purpose quite well. The option 1 test set-up is more straightforward and has been widely used and verified the feasibility in past release quite well. For option 2, till now it’s still not so clear what the detailed test set-up and no evaluation results submitted till now. We encourage proponent companies to provide more detailed information for the test set-up. The test feasibility of such test set-up also FFS. We saw a late submission contribution (R4-2007935) provided evaluated results for MU-MIMO Set-up, but the detailed information for evaluation set-up still missing in the paper. Not sure if it is LLS simulation or SLS level simulation? What’s simulation parameters i.e. MIMO correlation, number of beams, codebook construction. Based on following comments from E/// - “In our simulation papers for Type II codebook we see very large overall MU-MIMO gains for 4 User average throughput over Type I.” Seems like this evaluation is RAN1 style SLS evaluation, then it’s questionable how to match and reflect to RAN4 performance test cases. Keep in mind, in RAN4 we target to introduce performance test considering test feasibility and driven by LLS simulation which workable to align with companies to introduce test requirements based on aligned simulation assumption. From UE processing aspect, our view there is no UE processing difference among option1 and option2 test set-up. Probably we need to clarify this point whether we assume different UE implementation under option 1 and option 2 set-up ? Based on reviews the papers, the major concern from companies for option 1 was no much performance difference to show advantage of Type II over than Type I. As showed in our evaluation results in R4-2002978 (figure 2) show performance gain Rel-16 Type II > Rel-15 Type II > Rel-15 Type I under option 1 test set-up with selected test parameters i.e. XP Medium correlation and Rank 1 transmission. Figure 2: PMI performance with Type I, Type II and enhanced Type II codebook under XP Medium correlation model (16x2 Rank1 case) We are open to discuss the detailed test parameters to show the gain of Type II codebook and we are also open to discuss the test metric i.e. Rel-15Type II /Rel-15 Type I. -> To summary and be clear for our position: - We are open to discuss the detailed test set-up including test metric for option 1 test set-up - We are also open to further discuss the option 2 test set-up for detailed test configurations, test ability and feasibility with introducing RAN4 test cases by evaluation results. Our only concern this will be time and RAN4 effort consuming, We should make a decision on the timeline for this open issue for rel-16 i.e. deadline for make decision for this issue. Sub-topic 3-3: SU-MIMO Type II PMI test parameters Issue 3-3-1: Type II codebook construction We agree with the recommended WF. Issue 3-3-2: Npsk (phaseAlphabetSize) for type II codebook construction From our evaluation results, no much difference among two, we prefer option 1, but we are open to further discuss this with more evaluation results. Issue 3-3-3: subbandAmplitude for type II codebook construction We didn’t see much performance difference, we prefer option 1. Issue 3-3-4: PMI-FormatIndicator for type II codebook We prefer option 1. Two major points: - As Huawei mentioned, for Rel-15 Type II, wideband or wideband/sub-band belongs to UE capability; for test applicable to cover more UEs, we prefer wideband. - Through our simulation results under sub-band PMI , random PMI performance quite worse, not feasible to introduce feasible performance requirements. Issue 3-3-5: MIMO correlation for type II codebook We prefer option 2 XP medium. As show in our previous result from R4-2002978 (figure 2), Type II show performance difference over than Type I codebook. Better we can use XP Medium. Issue 3-3-6: Beam steering model for Type II Codebook Our preference is option 2. |
| Vodafone | Sub-topic 3-2: Type II PMI test setup Issue 3-2-1: Test setup for type II - We support option 2 proposed by Ericsson and echo the concerns described by Verizon and Orange. |
| Intel | Issue 3-1-1: Whether to introduce subband PMI test for type I single-panel codebook Our preference is Option 1, because sub-band PMI reporting is essential feature for Type I codebooks and is rather important from test coverage point of view. We suggest not to mix Type I and Type II discussions. |
| T-Mobile USA | Sub-topic 3-1: Type I PMI test Issue 3-1-1: Whether to introduce subband PMI test for type I single-panel codebook We agree with Ericsson that we should not mix up SP Type I requirements with Type II requirements. They serve different purposes and thus should not be discussed under the same issue. Type II is designed for MU-MIMO which should not be compared with the SU-MIMO setup from SP Type I. Regarding SP Type I requirements, we think that both wideband and subband PMI reporting are important, so we would prefer Solution C: - Solution C: Define subband PMI requirement for type I 16Tx - Use TDLC300-5 channel for type I 16Tx subband Define wideband PMI requirement for type I 16Tx - Use TDLA30-5 channel for type I 16Tx wideband Sub-topic 3-2: Type II PMI test setup Issue 3-2-1: Test setup for type II We prefer Option 2 for the same reason as Ericsson mentions above. Type II was developed primarily for MU-MIMO, and the performance difference using Type II can be significant compared to SU-MIMO. If only Type I is tested, it would enable a shortcut in UE implementation and only feed back CSI Type II that is “good enough” for SU-MIMO. The rich channel CSI is not needed if only SU is tested. Sub-topic 3-3: SU-MIMO Type II PMI test parameters Issue 3-3-1: Type II codebook construction We prefer Option 2: 32Tx port codebook construction from a feature testing perspective. Issue 3-3-2: Npsk (phaseAlphabetSize) for type II codebook construction We prefer Option 2. We agree with Ericsson that when SU-MIMO configuration is considered, the performance difference will be negligible. but for MU-MIMO, which is the use case that should be tested here, using option 2 is important. Issue 3-3-3: subbandAmplitude for type II codebook construction We prefer Option 2. We agree with Ericsson that when SU-MIMO configuration is considered, the performance difference will be negligible. but for MU-MIMO, which is the use case that should be tested here, using option 2 is important. Issue 3-3-4: PMI-FormatIndicator for type II codebook We prefer Subband PMI reporting Option 2. Not having subband reporting means Type I cannot do fast fading based nullfomirng to victim UEs, which is the whole point of Type II. Issue 3-3-5: MIMO correlation for type II codebook We prefer option two, medium correlation. In high correlation, the difference between Type I and Type II is smaller, it is better to target the rich scattering channel which is more suitable for MU-MIMO. Sub-topic 3-4: MU-MIMO Type II PMI test parameters Issue 3-4-1: Test metric for MU-MIMO Type II PMI We prefer to measure the TP ratio between following Type II codebook with Type I codebook to fully see the performance benefits of Type II codebook for MU-MIMO scenario. Sub-topic 3-5: Plan for CRs Issue 3-5: Plan for CRs We would like to have CRs for Type II PMI for RAN4#96. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007924, Correlation matrices for 2D antenna arrays | China Telecom: For the 16(4,2) x 2 MIMO correlation matrices given in Table 1 B.2.3.2.2-4, different correlation matrix is obtained based on our calculation. We wonder if Ericsson could provide the intermediate result, i.e., R_gnb to help us check if our understanding to the correlation matrix calculation is correct. |
|  | Ericsson: to China Telecom So for the first dimension (N1) on gNB side the matrix is: 1.0000 0.9884 0.9543 0.9000 0.9884 1.0000 0.9884 0.9543 0.9543 0.9884 1.0000 0.9884 0.9000 0.9543 0.9884 1.0000, For the second dimension (N2) on gNB side: 1.0000 0.9000 0.9000 1.0000 Note that the R_spatial need to be normalized with a=0.00012 for this case. i.e. (R_high + aI)/(1’a) Update: Correction on (N1,N2,P) values to align with Rel-15 corrections made in Intel CR R4-2006541 |
| R4-2007925, PMI FRC | China Telecom: we have uploaded an revision with our comments in the draft inbox: Revised R4-2007925_PMI FRC-CTC.docx |
|  | Qualcomm: Based on our calculations, we agree with China Telecom. |
|  | Ericsson: Based on the transport block calculation procedures from 38.214 Clause 5.1.3.2 we derived the new numbers. Could companies please check if the new numbers are correct. The payload size in the simulation assumptions paper (R4-2005550) were not derived using the methodology from 38.214 hence the difference in numbers. Update: we’ve revised our draftCR and changed to the correct values according to the Simulation assumptions, and comments from China Telecom, Qualcomm, and Intel. |
|  | China Telecom: Thanks Ericsson for the further check. But based on our calculation following 38.214, we still think the numbers in the simulation assumptions paper (R4-2005550) are correct. |
|  | Intel: Based on our calculation, TBS values from R4-2005550 are correct. |
| R4-2007926, type I PMI tests | Company A |
|  | Company B |

Note: To save time on typing the comments one by one, companies can also directly revise the draft CR and upload the revision in the draft inbox.

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Topic#3: PMI | Sub-topic 3-1: Type I PMI test - Issue 3-1-1: Whether to introduce subband PMI test for type I single-panel codebook - Option 1: Introduce subband PMI requirements for 16 Tx ports (QC, Ericsson, Apple, Samsung, Intel) - Option 2: Not introduce subband PMI requirements for 16Tx ports and covers 16Tx port requirements with wideband PMI (Huawei, Ericsson) - Option 3: Note introduce PMI requirements for 16Tx ports (Samsung) - Option 4: Introduce both subband and wideband PMI requirements for 16Tx ports (TMUS) Recommendations for 2nd round: - Since it has been agreed to make decision in RAN4 #95-e, encourage companies to be more flexible and try to make compromise. - Issue 3-1-2: Gamma (gain) values for Type I PMI test - For 32 Tx wideband: - Option 1: 5.0 for 2Rx, 8.0 for 4Rx (CTC) - Make decision in the next meeting based on more companies’ simulation results (QC) - For 16 Tx wideband/subband: FFS Tentative agreement: - Make decision in the next meeting Sub-topic 3-2: Type II PMI test setup - Issue 3-2-1: Test setup for type II - Option 1: Only use SU-MIMO test setup, i.e., one tested UE (CTC, Qualcomm, Huawei, Apple) - Option 2: MU-MIMO based test setup, i.e., one tested UE + one co-scheduled UE (generated by TE) (Ericsson, Verizon, Orange, VDF, TMUS) Recommendations for 2nd round: - Further discuss the two options. Sub-topic 3-3: SU-MIMO Type II PMI test parameters (applicable if option 1 in issue 3-2-1 is agreed) - Issue 3-3-1: Type II codebook construction - Option 1: 16Tx ports (N1, N2) = (4,2), (O1, O2) = (4,4) (CTC, Qualcomm, Huawei, Apple, Samsung) - Option 2: 32Tx ports (N1, N2) = (4,4), (O1, O2) = (4,4) (Ericsson, TMUS) Recommendations for 2nd round: - Check with the proponents of option 2 that is the option 2 proposed for SU-MIMO test setup or MU-MIMO test setup - Issue 3-3-2: Npsk (phaseAlphabetSize) for type II codebook construction - Option 1: 4 (Samsung) - Samsung: We are open to further discuss this with more evaluation results. - Option 2: 8 (Qualcomm, Apple) - QC: wait until we decide on WB vs SB reporting, and make decision based on simulation results - Option 3: TBD based on more companies’ simulation results in the next meeting (Huawei) Tentative agreement: - To be decided based on more companies’ simulation results in the next meeting. - Issue 3-3-3: subbandAmplitude for type II codebook construction - Option 1: False (Samsung, Huawei, Apple) - Option 2: True (Qualcomm) - QC: slightly prefer Option 2, since we see slight gain with Option 2 Recommendations for 2nd round: - Check if option 1 is agreeable. - Issue 3-3-4: PMI-FormatIndicator for type II codebook - Option 1: Wideband (Samsung, Huawei) - Huawei: Wideband should be supported anyway if UE supports Type II codebook as per UE capability parameters codebookParameters clarified in TS 38.306. - Samsung: Through our simulation results under sub-band PMI, random PMI performance quite worse, not feasible to introduce feasible performance requirements. - Option 2: Subband (Qualcomm) - QC: Encourage more simulation results Tentative agreement: - To be decided based on more companies’ simulation results in the next meeting. - Issue 3-3-5: MIMO correlation for type II codebook - Option 1: XP High (Qualcomm, Huawei, Apple) - Option 2: XP Medium (Samsung) Tentative agreement: - To be decided based on more companies’ simulation results in the next meeting. - Issue 3-3-6: Beam steering model for Type II Codebook - Option 1: Reusing beam steering approach with dual-cluster beams as specified in B.2.3B.4A of TS 36.101 (Qualcomm) - Qualcomm: 1) It is very unlikely that UE will receive the equal power signals from more than 2 independent directions. 2) If we use 4 clusters, TE will have to generate 4 independent channel realizations of 12 taps each. So, they can either realize only 3 taps per channel that will be very poor implementation of channel model or they will have to surpass their limitations and that would mean more expensive test box. - Option 2: Use Equation 1 as beam steering model for Type II codebook performance requirements (CTC, Huawei, Ericsson, Apple, Samsung) Recommendations for 2nd round: - Further discuss and try to make decision in this meeting Sub-topic 3-4: MU-MIMO Type II PMI test parameters (applicable if option 2 in issue 3-2-1 is agreed) - Issue 3-4-1: Test metric for MU-MIMO Type II PMI - Option 1: TP ratio between following PMI and random PMI - Option 2: TP ratio between following Type II codebook and following SP Type I codebook (Ericsson, T-Mobile) - Other options are not precluded - Issue 3-4-2: Type II codebook construction - Option 1: 32Tx ports (N1, N2) = (4,4), (O1, O2) = (4,4) (Ericsson, TMUS) - Other options are not precluded - Issue 3-4-3: Npsk (phaseAlphabetSize) for type II codebook construction - Option 1: 8 (Ericsson, TMUS) - Other options are not precluded - Issue 3-4-4: subbandAmplitude for type II codebook construction - Option 1: True (Ericsson, TMUS) - Other options are not precluded - Issue 3-4-5: PMI-FormatIndicator for type II codebook - Option 1: Subband (Ericsson, TMUS) - Other options are not precluded - Issue 3-4-6: MIMO correlation for type II codebook - Option 1: XP Medium (Ericsson, TMUS) - Ericsson, TMUS: In high correlation, the difference between Type I and Type II is smaller, it is better to target the rich scattering channel which is more suitable for MU-MIMO. - Other options are not precluded Sub-topic 3-5: Plan for CRs - Issue 3-5: Plan for CRs - Option 1: - Endorse the draft CRs for type I tests, type I FRC and MIMO Correlation Matrices for 2D antenna arrays in this meeting - Endorse the draft CRs for type II and applicability in RAN4 #96e (Aug) meeting - Agree all the CRs for PMI reporting requirements together (including CRs for applicability, type I and type II) in RAN4 #97e (Oct-Nov) meeting Tentative agreement: - Further discuss in the next meeting |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Way forward on PMI reporting requirements for Tx ports larger than 8 and up to 32 | Ericsson, Samsung |
| #2 | Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports | Ericsson |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2007924, Correlation matrices for 2D antenna arrays | to be revised |
| R4-2007925, PMI FRC | to be revised |
| R4-2007926, type I PMI tests | to be revised |

Discussion on 2nd round

Way forward

R4-2008846	Way forward on PMI reporting requirements for Tx ports larger than 8 and up to 32

Type: other		For: Approval
					Source: Ericsson, Samsung

Abstract:

Discussion:

Agreement in GTW Session on May.29th

- Issue 3-2-1: Test setup for type II PMI reporting requirements

  - Option 1: Only use SU-MIMO test setup, i.e., one tested UE (CTC, Qualcomm, Huawei, Apple)

  - Option 2: MU-MIMO based test setup,  i.e., one tested UE + one co-scheduled UE (generated by TE) (Ericsson, Verizon, Orange, VDF, TMUS)

It’s RAN4 common understanding under the baseline UE receiver assumption, the PMI calculation processing will not been changed with and without co-schedule UE for Rel-16 test cases (for both Rel-15/Rel-16 Type II codebook test cases).

It’s RAN4 common understanding when defining requirements; the baseline receiver assumption is UE without interference cancellation capability with/without co-schedule UE for Rel-16 test cases.

Keep these two options open; further discuss detailed test set-up and simulation assumption for option 1 and option 2 separately in 2nd round to facilitate evaluation work in future RAN4 meeting; and make decision among option 1 and option 2 in Q3 2020.

- TE vendors are encouraged to provide feedback for the test feasibility of option 2 set-up.

- Proponents for each option need to provide technical analysis for how the test set-up can guarantee UE PMI reporting requirements with type II codebook for its intended purpose.

- Issue 3-1-1: Whether to introduce subband PMI test for type I single-panel codebook

  - Option 1: Introduce subband PMI requirements for 16 Tx ports (QC, Ericsson, Apple, Samsung, Intel)

  - Option 2: Not introduce subband PMI requirements for 16Tx ports and covers 16Tx port requirements with wideband PMI (Huawei, Ericsson)

  - Option 3: Note introduce PMI requirements for 16Tx ports (Samsung)

  - Option 4: Introduce both subband and wideband PMI requirements for 16Tx ports (TMUS)

Option1:

- Introduce sub-band PMI test cases with current simulation assumption for 16 Tx port Type I PMI test case.

Email discussion in the 2nd round:

Moderator’s note: The WF, simulation assumptions and simulation result summary are discussed in sub-thread [95e][323] NR_perf_enh_Demod_UE - draft WF R4-2008846 (led by Ericsson, Samsung).

[CTC]:

1)  According to the GTW agreement on last Friday, we would suggest to add one more sub-bullet in slide #7:

- Test setup:

  - Option 1: Only use SU-MIMO test setup, i.e., one tested UE

  - Option 2: MU-MIMO based test setup,  i.e., one tested UE + one co-scheduled UE (generated by TE)

  - Keep these two options open and make decision among option 1 and option 2 in Q3 2020.

  - Proponents for each option need to provide technical analysis for how the test set-up can guarantee UE PMI reporting requirements with type II codebook for its intended purpose.

2) For slide #12, we have not discussed the detailed test setup in the 1st round, so suggest to mention this slide is for information.

[Ericsson]: Furthermore I’ve added more details on the MU-MIMO based test setup (slide 12). Please note that as per recommendation during the GTW session we are encouraged to discuss detailed test setup for MU-MIMO based testing in the 2nd round, thus we think “For information” part can still be discussed.

[CTC]: Agree that “For information” part can still be discussed in the 2nd round.

The problem with the slide #12 in the first version is that: there is only one option listed, seems that it is the agreement we have reached.

For the updated slide #12, since similar discussion is taking place in eMIMO demod thread, to avoid duplicated discussion, may be we can just refer to the slide in eMIMO demod WF?

[Huawei]: Related to the test setup for Type II codebook, same comments can be found under the discussion of eMIMO. To be in short, we think further investigations are needed since several questions have been raised and lack of common understanding on several issues and assumptions. As the test setup for Type II codebook can be applied on both Re-15 and Rel-16, I think we can hold one discussion to reduce confusing and duplicate comments.

[QC]: We prefer to keep Option 1 for test metric on slide 10 as we don’t want to combine the performance of two codebooks.

[Apple]: I’ m repeating the same comments as in eMIMO thread for MU-MIMO set-up. It is not very clear on how we need to simulate the MU-MIMO setup in order to evaluate performance between SU-MIMO and MU_MIMO. How is the ZF implemented at TX side? Is the 2nd UE also reporting PMI? Which UE’s PMI is the gNB following? We would need more details to simulate further and come back with evaluation results for next meeting. On Slide #10, could we please keep option 1 for test metric.

[Ericsson]: Regarding ZF algorithm:

Our goal is to create a test that is maximally sensitive to impairments in the nullforming potential towards victim UEs. We propose to use the well-known regularized ZF formula

W = H’*inv(H*H’ + lambda*eye)

where H = [H_A; H_B] and H_A is a channel estimate for UE A (DUT) and H_B a channel estimate for UE B. For DUT we use the reported overall precoder W_A for each subband to estimate the channel as H_A = W_A’ while for the co-scheduled UE B the simplest, without jeopardizing the sensitivity of the test, appears to be to use the generated channel for UE B as H_B. An alternative could be to also for UE B compute a Type II precoder W_B and use H_B=W_B’ but we don’t see why this would result in a more sensitive test hence we prefer the first approach of using the generated channel directly instead.

The regularization factor and the UE spatial separation are design parameters in the test for maximizing its sensitivity.

[Apple]: The clarification and additional information in WF is helpful for further evaluation. For clarification Hb is the ideal precoder generated from the channel and not the channel itself?

[Ericsson]: As previously mentioned this will need to be evaluated, hence the FFS in the WF.

[Ericsson]: A small correction on the ZF-algorithm for MU-MIMO. It should be the Hermitian transpose for the relation between the channel H and the precoder W.

This is for information and we don’t see the need to update the WF since it’s been already agreed.

- Issue 3-3-6: Beam steering model for Type II Codebook

  - Option 1: Reusing beam steering approach with dual-cluster beams as specified in B.2.3B.4A of TS 36.101 (Qualcomm)

    - Qualcomm: 1) It is very unlikely that UE will receive the equal power signals from more than 2 independent directions. 2) If we use 4 clusters, TE will have to generate 4 independent channel realizations of 12 taps each. So, they can either realize only 3 taps per channel that will be very poor implementation of channel model or they will have to surpass their limitations and that would mean more expensive test box.

  - Option 2: Use Equation 1 as beam steering model for Type II codebook performance requirements (CTC, Huawei, Ericsson, Apple, Samsung)

[R&S]: Sorry for missing this discussion in the first round, but the same discussion is happening in the eMIMO WI and we overlooked this one. We currently cannot agree to Option 2 and share the same view as QC. Also it seems we now have contradicting possible agreements in eMIMO and this thread. Our preference would be option 1. But we can compromise to the WF from the eMIMO discussion thread.

Recommendation:		Agreeable

R4-2008847	Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion:

Recommendation:		Agreeable

Draft CRs

R4-2008843	Addition of High spatial correlation matrices for 2D antenna arrays

Type: draftCR		For: Endorsement
					38.101-4 v16.0.0
					Source: Ericsson

Abstract:

In this contribution we provide the equation for 2D antenna array correlation matrices for high correlation

Discussion:

Moderator’s note: The 3 draft CRs for PMI reporting requirements are discussed in sub-thread [95e][323] NR_perf_enh_Demod_UE - draft CR R4-2008843 (led by Ericsson).

[Ericsson] we’ve double checked the values, and would like to see if more companies could check.

[CTC]: Thanks Ericsson for providing the intermediate results (R_gnb_1 & R_gnb_2) in the 1st round. We have the same R_gnb_1 and R_gnb_2 as Ericsson.

Then for R_gnb, based on our calculation, the first row is:

1.0000  0.9000  0.9884  0.8895  0.9543  0.8588   0.9000  0.8100

Could Ericson help check these values?

PS: As commented in the 1st round, for the 16(4,2) x 2 MIMO correlation matrices given in Table 1 B.2.3.2.2-4, different correlation matrix is obtained based on our calculation.

[Ericsson]: Update to China Telecom: Thanks for checking. The matrix was transposed from what it should’ve been thanks for checking.

[Apple]:

1. In paragraph before Table B.2.3.2.2-2: to ensure the correlation matrix is positive semi-definite

2. We checked the correlation matrix for 16x2 and its positive semidefinite without the adjustment. So, do we need an a=0.00012 for this case? Also, how are the values for different number of antennas derived?

3. In paragraph before Table B.2.3.2.2-2: a respectively at the end of the sentence would make it complete, but again should we check if the adjustments are needed?

4. Is there a plan to add matrices for all cases - 16x4, 32x2, 32x4 with high correlation?

[Ericsson]:

1: ok changed.

2: The values are taken from 36.101 Annex which also have 2D correlation matrices for 16 and 32 ports.

3: Added.

4: In 36.101 the largest matrix defined is 16x2. The 32x4 case will have a matrix of the size 128x128, since we already provide the method for deriving the matrices we think we can reuse what LTE did.

Hopefully you’re ok with these answers, otherwise I can come back with this CR in August.

[Apple]: Thanks for the clarifications. We are okay with the CR and needn’t postpone it.

Recommendation:		Endorsed

R4-2008844	Addition of Rel-16 PMI FRC

Type: draftCR		For: Endorsement
					38.101-4 v16.0.0
					Source: Ericsson

Abstract:

CR with addition of FRC for new PMI testing for 16, and 32 Tx ports

Discussion:

[Ericsson]: Updated FRC values based on comments from QC, Intel, CMCC.

[CTC] ok with this version

Recommendation:		Endorsed

R4-2008845	Addition of Rel-16 SP type I PMI tests

Type: draftCR		For: Endorsement
					38.101-4 v16.0.0
					Source: Ericsson

Abstract:

CR with addition of PMI tests for SP Type I 16, and 32Tx ports

Discussion:

[Ericsson]: Captured subband PMI reporting for 16Tx ports according to GTW Friday session.

[CTC] ok with this version

Recommendation:		Endorsed

Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2008846, WF | Agreeable |
| R4-2008847, Simulation assumptions | Agreeable |
| R4-2008843, Correlation matrices for 2D antenna arrays | Endorsed |
| R4-2008844, PMI FRC | Endorsed |
| R4-2008845, type I PMI tests | Endorsed |

Topic #4: TDD LTE-NR coexistence

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006532 | Intel Corporation | CR to TS 38.101-4: CR on TDD LTE-NR coexistence requirements finalization |

Open issues summary

No open issue.

Companies views’ collection for 1st round

Open issues

No open issue.

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2006532, Intel | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2006532, Intel | agreeable |

Topic #5: UE	power imbalance requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006039 | China Telecom | Propose 1: One candidate solution for option 3 of channel bandwidth combination: - Decide the MCS based on the simulation results for 100MHz CBW - Apply the same MCS for the requirements for different CBW combinations - Select the CBW combination with the largest aggregated channel bandwidth for testing Propose 2: For MIMO configuration, option 2 with 1x2 and 1x4 is preferred in order to simplify the test setup. Propose 3: Option 1 with WB PRB bundling is slightly preferred, if it is agreed to use 1 Tx. Observation 1: Based on our simulation results, 100% relative throughput can be achieved for 1T2R with MCS 27 and 1T4R with MCS 28. Propose 4: Use MCS 27 for 2Rx and MCS 28 for 4Rx. |
| R4-2006533 | Intel Corporation | Proposal 1: Define generic methodology for selection of CBW combination among all CBW combinations in supported CA configurations for NR CA power imbalance requirements. Consider selection of CBW combination with the largest aggregated channel bandwidth as one of candidate option. Proposal 2: Use the following configuration for NR CA requirements with power imbalance: - Full bandwidth allocation - PRB bundling size: WB - MIMO configuration: 1x2 and 1x4 Proposal 3: Define intra band contiguous EN-DC requirements for FDD and TDD duplex modes. Proposal 4: Define intra band contiguous EN-DC requirements for the following NR SCS configuration: - 15 kHz for FDD; - 15 and 30 kHz for TDD. Proposal 5: Use the following testing rule for intra band contiguous EN-DC requirements: - Test #1: LTE FDD + NR FDD 15 kHz - Test #2: LTE TDD + NR TDD 15 kHz, in case UE supports it, otherwise LTE TDD + NR TDD 30 kHz Proposal 6: Define intra band contiguous EN-DC requirements for NR TDD patterns fully aligned with LTE TDD UL/DL pattern 2 (i.e. DSU+DD for 15 kHz and 3DSU+4D for 30 kHz) Proposal 7: Consider intra-band non-contiguous EN-DC requirements only for UE with intraBandENDC-Support = non-contiguous. Further discussion the details of test design to ensure that RX image rejection can be verified. |
| R4-2006809 | CMCC | Proposal 1: It is proposed that: - Define requirements for 5+5 MHz bandwidth for FDD+FDD CA, 10+10 MHz bandwidth for TDD+TDD CA, with the following test applicability - The test is done for any one of the supported bandwidth combination, by using performance requirement for 5+5 MHz FDD+FDD CA or 10+10 MHz TDD+TDD CA. - The tested PRBs shall be placed in the highest part for the CC with lower carrier frequency, and placed in the lowest part for the CC with higher carrier frequency. - Select the CA combination with largest bandwidth, and select the CA configuration with the same BWs in each carrier for power imbalance test - If there is no supported CA configuration with the same BWs, additional power imbalance test can be considered if necessary. Proposal 2: Define FDD 15KHz and TDD 30KHz for intra-band contiguous EN-DC. |
| R4-2007140 | NTT DOCOMO, INC. | Observation 1 Regarding intra-band EN-DC, three type of UEs can be considered. 1. Support only intra-band contiguous EN-DC, i,e., if UE does not indicate “intraBandENDC-Support” 2. Support only support intra-band non-contiguous EN-DC, i.e., if UE indicates “non-contiguous” in “intraBandENDC-Support 3. Support both intra-band contiguous and non-contiguous EN-DC, i.e., if UE indicates “both” in “intraBandENDC-Support” Observation 2 Some inter-band EN-DC band combinations like DC_B42-n78 are treated as “intra-band” EN-DC, and non-contiguous EN-DC is supported as mandatory. Observation 3 Regarding special inter-band EN-DC like DC_B42-n78, two type of UEs can be considered. 1. Support Oonly intra-band non-contiguous EN-DC, i,e., if UE does not indicate “interBandContiguousMRDC” 2. Support both intra-band contiguous and non-contiguous EN-DC, i,e., if UE indicates "“interBandContiguousMRDC” Observation 4 Both contiguous EN-DC and non-contiguous EN-DC are possible scenario and some UEs only support non-contiguous EN-DC. Observation 5 Only co-located scenario is assumed for both intra-band contiguous and non-contiguous EN-DC cases in RAN4 requirements. It derives that single RF chain is assumed to receive CCs. Proposal 1: 15 kHz SCS for FDD and 30kHz SCS for TDD are applied to power imbalance test requirements Proposal 2: Introduce both contiguous and non-contiguous EN-DC requirements for power imbalance tests Proposal 3: Introduce test applicability rules according to UE capability as follows: - UE supports only intra-band contiguous EN-DC, i,e., if UE does not indicate “intraBandENDC-Support”, - power imbalance requirement for intra-band contiguous EN-DC is applied - UE supports only intra-band non-contiguous EN-DC, i.e., if UE indicates “non-contiguous” in “intraBandENDC-Support” or UE does not indicate “interBandContiguousMRDC”, - power imbalance requirement for intra-band non-contiguous EN-DC is applied - UE supports both intra-band contiguous and non-contiguous EN-DC, i.e., if UE indicates “both” in “intraBandENDC-Support” or UE indicates “interBandContiguousMRDC”, - apply either power imbalance requirement for FR1 intra-band contiguous EN-DC or FR1 intra-band non-contiguous EN-DC |
| R4-2007224 | Huawei, HiSilicon | Proposal 1: Updated option2: Define requirements for 5+5 MHz bandwidth for FDD+FDD CA, 10+10 MHz bandwidth for TDD+TDD CA, and the test applicability should be changed as follows: - The test is done for any one of the supported bandwidth combination, by using performance requirement for 5+5 MHz FDD+FDD CA or 10+10 MHz TDD+TDD CA. - The tested PRBs shall be placed in the highest lowest part for the CC with lower carrier frequency, and placed in the lowest highest part for the CC with higher carrier frequency. Proposal 2: If choose one generic method: Select the largest channel bandwidth combination among all CBW combinations in supported CA configurations, all RBs of CC with larger bandwidth are allocated, the tested RBs are placed on the position of initial BWP of CC with smaller bandwidth and only define the performance of CC with smaller bandwidth. Observation 1: When the SNR is 19dB for 1T4R, the normalized throughput will still be much higher than 85% even if the MCS reaches the maximum (256QAM, 948/1024). For other MIMO configurations, it is impossible to find a suitable MCS to make the normalized TP close to 85% (either 0 or 100%). Proposal 3: Increase the power difference (larger than 6dB) between two CCs to make the SNR lower than 19dB and only define the performance requirement for Scell. To make normalized TP close to 85%, more simulation are needed to find the suitable SNR under the condition of a pre-selected MCS. Proposal 4: Use 1x2 and 1x4 MIMO configuration. Proposal 5: Use PRB bundling size 2 RPBs. Proposal 6: Reuse the simulation assumptions from NR CA requirements to define EN-DC requirements with power imbalance for the following parameters PDSCH configuration, PDCCH allocation, antenna configuration and propagation conditions, both FDD and TDD need to be tested, use 15 kHz for FDD, 30 kHz for TDD. |
| R4-2007882 | Qualcomm Incorporated | Proposal 1: Do not define power imbalance requirements for non-contiguous intra-band EN-DC. Proposal 2: Use MIMO configuration of 1x2 and 1x4 for defining power imbalance test cases. Proposal 3: Use full RB PDSCH allocation for defining FR1 intra-band contiguous CA power imbalance tests with both carriers having same bandwidth. |

Open issues summary

Sub-topic 5-1: Requirements for FR1 intra-band contiguous CA

Issue 5-1-1: Channel bandwidth combination for defining performance requirements

- Agreement in RAN4 #94e-bis (R4-2005547, WF)

  - Option 2: Define requirements for 5+5 MHz bandwidth for FDD+FDD CA, 10+10 MHz bandwidth for TDD+TDD CA, with the following test applicability

    - The test is done for any one of the supported bandwidth combination, by using performance requirement for 5+5 MHz FDD+FDD CA or 10+10 MHz TDD+TDD CA.

    - The tested PRBs shall be placed in the highest part for the CC with lower carrier frequency, and placed in the lowest part for the CC with higher carrier frequency.

  - Option 3: Define generic methodology for selection of CBW combination among all CBW combinations in supported CA configurations

    - Methodology of Option 3 is to be clarified.

- Proposals

  - Option 2: Define requirements for 5+5 MHz bandwidth for FDD+FDD CA, 10+10 MHz bandwidth for TDD+TDD CA, with the following test applicability (HW, CMCC)

    - Test applicability for option 2:

      - The test is done for any one of the supported bandwidth combination, by using performance requirement for 5+5 MHz FDD+FDD CA or 10+10 MHz TDD+TDD CA.

      - The tested PRBs shall be placed in the highest part for the CC with lower carrier frequency, and placed in the lowest part for the CC with higher carrier frequency.

    - Potential issue raised for option 2 (Intel):

      - Option 2 is mainly limited to verification of scenarios with same channel bandwidth.

      - For scenarios with different channel bandwidth on different CCs, UE may have different implementation of LO allocation, which will lead to different image level and, as result, verification for partial PRB allocation will be different in comparison to full PRB allocation.

    - Improved solutions for option 2 to resolve the above issue:

      - Option 2a  (HW, changes are in red):

        - The test is done for any one of the supported bandwidth combination, by using performance requirement for 5+5 MHz FDD+FDD CA or 10+10 MHz TDD+TDD CA.

        - The tested PRBs shall be placed in the highest lowest part for the CC with lower carrier frequency, and placed in the lowest highest part for the CC with higher carrier frequency.

      - Option 2b  (CMCC, changes are in red):

        - The test is done for any one of the supported bandwidth combination, by using performance requirement for 5+5 MHz FDD+FDD CA or 10+10 MHz TDD+TDD CA.

        - The tested PRBs shall be placed in the highest part for the CC with lower carrier frequency, and placed in the lowest part for the CC with higher carrier frequency.

        - Select the CA combination with largest bandwidth, and select the CA configuration with the same BWs in each carrier for power imbalance test

        - If there is no supported CA configuration with the same BWs, additional power imbalance test can be considered if necessary.

        - Note that from 38.101-1, we can observe that most of the CA combinations have the configuration with same BWs, except CA_n71B and CA_n78B.

  - Option 3: Define generic methodology for selection of CBW combination among all CBW combinations in supported CA configurations (Intel, QC, HW)

    - Is it feasible to define bandwidth agnostic requirements for option 3?

      - Yes (CTC)

        - For FR1 normal PDSCH CA simulation results in R4-2004554 and R4-2004555, based on the average impairment results from 5 companies:

          - For 2Rx FDD 15 kHz, the performance gap for different CBW (5-50MHz) is up to 0.8dB.

          - For 4Rx FDD 15 kHz, the performance gap for different CBW (5-50MHz) is up to 0.4dB.

          - For 2Rx TDD 30 kHz, the performance gap for different CBW (5-100MHz) is up to 1.2dB.

          - For 4Rx TDD 30 kHz, the performance gap for different CBW (5-100MHz) is up to 0.8dB

          - Rank 2 and fading channel are assumed in normal PDSCH CA test

        - For power imbalance under rank 1 and AWGN channel, our simulation results in terms of relative TP for the minimal and maximal channel bandwidths for each SCS are similar.

- Recommended WF

  - In the 1st round, invite companies to provide feedback on:

    - For option 2, are the improved solutions feasible?

    - For option 3, is it feasible to define bandwidth agnostic requirements?

Issue 5-1-2: Channel bandwidth combination for testing

- Proposals

  - Option 1: First priority is to ensure both carriers have the same bandwidth. (QC, CMCC)

    - CMCC: from 38.101-1, we can observe that most of the CA combinations have the configuration with same BWs, except CA_n71B and CA_n78B.

  - Option 2: Select the CBW combination with the largest aggregated channel bandwidth (Intel, CTC, HW)

    - HW: all RBs of CC with larger bandwidth are allocated, the tested RBs are placed on the position of initial BWP of CC with smaller bandwidth and only define the performance of CC with smaller bandwidth.

- Recommended WF

  - Step 1: First select the CBW combinations with the same BWs in each carrier

    - If there is no such CBW combination, select the CBW combinations with smallest CBW difference between the two carriers, and the carrier with [larger or smaller] CBW will be used for test.

  - Step 2: Among the CBW combinations selected from step 1, select the CA combination with largest aggregated CBW

Issue 5-1-3: PDSCH RB allocation

- Agreement in RAN4 #94e-bis (R4-2005547, WF)

  - To be decided after the channel bandwidth combination is agreed

- Proposals

  - Option 1: Full allocation (Intel)

- Recommended WF

  - TBA after the channel bandwidth combination is agreed

Issue 5-1-4: MIMO configuration

- Agreement in RAN4 #94e-bis (R4-2005547, WF)

  - Option 1: 2x2 and 2x4

  - Option 2: 1x2 and 1x4

  - Option 3: Simulation is needed

    - Further evaluate the throughput at 19dB SNR point

- Proposals

  - Option 1: 2x2 and 2x4

  - Option 2: 1x2 and 1x4 (CTC, Intel, HW, QC)

    - HW, QC: avoid the situation of 2TX under the condition of static channel, because this will make the signal energy on some antennas at the receiving side to zero.

  - Option 3: Simulation is needed

- Recommended WF

  - Agree with Option 2

Issue 5-1-5: MCS

- Agreement in RAN4 #94e-bis (R4-2005547, WF)

  - Simulation is needed

    - Option A: use different MCSs for 2Rx and 4Rx.

    - Option B: use same MCS for 2 Rx and 4 Rx

- Proposals

  - Option 1: Use MCS 27 for 2Rx and MCS 28 for 4Rx  (CTC)

    - CTC: Based on our simulation results, at 16dB SNR (considering 2 dB impairment margin and 0.8 dB extra margin for 64QAM), 100% relative throughput can be achieved for 1T2R with MCS 27 and 1T4R with MCS 28.

  - Option 2: Increase the power difference (larger than 6dB) between two CCs to make the SNR lower than 19dB (HW)

    - HW: When the SNR is 19dB for 1T4R, the normalized throughput will still be much higher than 85% even if the MCS reaches the maximum (256QAM, 948/1024). For other MIMO configurations, it is impossible to find a suitable MCS to make the normalized TP close to 85% (either 0 or 100%).

- Recommended WF

  - Decide in the next meting based on more simulation results:

    - Encourage more simulations for 1T2R and 1T4R at [16] dB

    - Discuss whether to use 64QAM or 256QAM

Issue 5-1-6: PRB bundling size

- Agreement in RAN4 #94e-bis (R4-2005547, WF)

  - Option 1: WB

  - Option 2: 2 PRBs

- Proposals

  - Option 1: WB (CTC, Intel)

    - CTC: WB PRB bundling is slightly preferred, if it is agreed to use 1 Tx.

  - Option 2: 2 PRBs (HW)

- Recommended WF

  - Agree with Option 1, since it has been recommended to use 1Tx

Sub-topic 5-2: Requirements for intra-band contiguous EN-DC

- Description in the WID (RP-200472)

  - FR1 intra-band EN-DC PDSCH demodulation performance requirement with power imbalance

    - Intra-band contiguous EN-DC with 6dB power imbalance is assumed.

    - Only the NR cell is configured as the weaker power cell and to be tested.

Issue 5-2-1: Duplex mode

- Agreement in RAN4 #94e-bis (R4-2005547, WF)

  - Option 1: FDD and TDD

  - Option 2: FDD

- Proposals

  - Option 1: FDD and TDD (Intel, DCM, HW, CMCC)

- Recommended WF

  - Agree with Option 1

Issue 5-2-2: SCS

- Agreement in RAN4 #94e-bis (R4-2005547, WF)

  - Option 1: 15kHz for both FDD and TDD

  - Option 2: 15kHz for FDD, 30kHz for TDD

  - Option 3: 15kHz for FDD

- Proposals

  - Option 1: 15kHz for both FDD and TDD

  - Option 2: 15kHz for FDD, 30kHz for TDD (DCM, HW, CMCC)

  - Option 3: 15kHz for FDD

  - Option 4 (Intel)

    - Test #1: LTE FDD + NR FDD 15 kHz

    - Test #2: LTE TDD + NR TDD 15 kHz, in case UE supports it, otherwise LTE TDD + NR TDD 30 kHz

- Recommended WF

  - Considering no operator has shown interest in TDD 15 kHz case, can we agree to use option 2?

Issue 5-2-3: TDD pattern

- Agreement in RAN4 #94e-bis (R4-2005547, WF)

  - TDD pattern for 30kHz SCS

    - 7D1S2U

  - TDD pattern for 15kHz SCS

    - Option 1: DSU+DD

    - Other options are not precluded.

- Proposals

  - TDD pattern for 30kHz SCS

    - Option 1: 7D1S2U (agreement in the last meeting)

    - Option 2: 3DSU+4D (Intel)

  - TDD pattern for 15kHz SCS

    - Option 1: DSU+DD (Intel)

    - Other options are not precluded.

- Recommended WF

  - TDD pattern for 30 kHz SCS

    - Can we keep the agreement in the WF in the last meeting, i.e., option 1?

Sub-topic 5-3: Requirements for intra-band non-contiguous EN-DC

Issue 5-3-1: Whether to define power imbalance requirement for FR1 intra-band non-contiguous EN-DC

- Description in the WID (RP-200472)

  - FR1 intra-band EN-DC PDSCH demodulation performance requirement with power imbalance

    - Further study whether to introduce intra-band non-contiguous EN-DC requirements and applicable power imbalance level

- Agreement in GTW On-line session in RAN4 #94e-bis

  - FFS the requirements needed or not and further check the feasibility of single RF receiver chain to support intra-band Non-contiguous EN-DC (Co-location scenario).

- Proposals

  - Option 1: Yes (DCM)

    - Both contiguous EN-DC and non-contiguous EN-DC are possible scenario and some UEs only support non-contiguous EN-DC.

    - Only co-located scenario is assumed for both intra-band contiguous and non-contiguous EN-DC cases in RAN4 requirements. It derives that single RF chain is assumed to receive CCs.

    - Introduce test applicability rules according to UE capability as follows:

      - UE supports only intra-band contiguous EN-DC, i,e., if UE does not indicate “intraBandENDC-Support”,

        - power imbalance requirement for intra-band contiguous EN-DC is applied

      - UE supports only intra-band non-contiguous EN-DC, i.e., if UE indicates “non-contiguous” in “intraBandENDC-Support” or UE does not indicate “interBandContiguousMRDC”,

        - power imbalance requirement for intra-band non-contiguous EN-DC is applied

      - UE supports both intra-band contiguous and non-contiguous EN-DC, i.e., if UE indicates “both” in “intraBandENDC-Support” or UE indicates “interBandContiguousMRDC”,

        - apply either power imbalance requirement for FR1 intra-band contiguous EN-DC or FR1 intra-band non-contiguous EN-DC

  - Option 2: No (QC)

    - QC: Similar to CA requirements for power imbalance, only consider intra-band contiguous case

  - Option 3: More discussion is needed (Intel, HW)

    - Intel: Before agreement on definition of such requirements, further discuss the details of test design to ensure that RX image rejection can be verified.

      - Even if UE uses single RF chain then, depending on channel spacing and LO allocation, scenarios without image issue can be observed.

      - Consider intra-band non-contiguous EN-DC requirements only for UE with intraBandENDC-Support = non-contiguous.

    - HW: more discussion is needed whether it is reasonable to assume an RF chain receiver to receive two non-continuous carriers in co-located scenario.

- Recommended WF

  - In the 1st round, encourage more discussion on the two aspects:

    - Whether it is feasible to assume single RF chain to receive two non-continuous carriers in co-located scenario?

    - If UE uses single RF chain, depending on channel spacing and LO allocation, whether or not the image issue can be observed?

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| China Telecom | Sub-topic 5-1: Requirements for FR1 intra-band contiguous CA Issue 5-1-1: Channel bandwidth combination for defining performance requirements - For option 2, are the improved solutions feasible? For option 2a, it seems the test purpose cannot be fulfilled since the tested PRBs and the interfering PRBs are far away from each other. For option 2b, it could be feasible, and one issue is the additional efforts to define additional test if there is no supported CA configuration with the same BWs. - For option 3, is it feasible to define bandwidth agnostic requirements? Could be feasible in our view. Issue 5-1-2: Channel bandwidth combination for testing Given the discussion in issue 5-1-1, generally ok with the recommended WF, and more details need to be clarified further after this principle is agreed. Issue 5-1-4: MIMO configuration Ok with the recommended WF. Issue 5-1-5: MCS Ok with the recommended WF. Issue 5-1-6: PRB bundling size Ok with the recommended WF. |
| CMCC | Sub-topic 5-1: Requirements for FR1 intra-band contiguous CA Issue 5-1-1: Channel bandwidth combination for defining performance requirements For option 2, we think both 2a and 2b is feasible. For option 3, we need to further check the simulation results. Issue 5-1-2: Channel bandwidth combination for testingOK with the recommended WF. Issue 5-1-4: MIMO configuration OK with the recommended WF Issue 5-1-5: MCS OK with the recommended WF. For option2, the power difference 6dB is decided and used from LTE, we prefer to keep the 6dB assumption. But agree that more simulation is needed in order to make the decision. Issue 5-1-6: PRB bundling size OK with the recommended WF. Sub-topic 5-2: Requirements for intra-band contiguous EN-DC Issue 5-2-1: Duplex mode Support the recommended WF Issue 5-2-2: SCS Support option 2 Issue 5-2-3: TDD pattern Support option 1 (7D1S2U) for TDD 30KHz |
| Qualcomm | Sub-topic 5-1: Requirements for FR1 intra-band contiguous CA Issue 5-1-1: Channel bandwidth combination for defining performance requirements Our preference is to choose CBW combinations with same bandwidth on both carriers and full RB allocation. Based on our preliminary simulations for 15kHz SCS, we see a gap of up to 1dB. So, it may be possible to define bandwidth agnostic test. Issue 5-1-2: Channel bandwidth combination for testing Ok with recommended WF. Issue 5-1-3: PDSCH RB allocation Ok with recommended WF. Issue 5-1-4: MIMO configuration Ok with recommended WF. Issue 5-1-5: MCS Prefer to use 64QAM since 256QAM is not mandatory. We don’t agree with Option 2 since 6dB power imbalance was already agreed based on practical limitations. Ok to simulate at 16dB. Issue 5-1-6: PRB bundling size We slightly prefer Option 2. Sub-topic 5-2: Requirements for intra-band contiguous EN-DC Issue 5-2-1: Duplex mode Ok with recommended WF. Issue 5-2-2: SCS Ok with Option 2. Issue 5-2-3: TDD pattern Ok with recommended WF. Sub-topic 5-3: Requirements for intra-band non-contiguous EN-DC Issue 5-3-1: Whether to define power imbalance requirement for FR1 intra-band non-contiguous EN-DC It is questionable how image issue can be observed with non-contiguous EN-DC. But, we are open to discussion. |
| SoftBank | Sub-topic 5-3: Requirements for intra-band non-contiguous EN-DC Issue 5-3-1: Whether to define power imbalance requirement for FR1 intra-band non-contiguous EN-DC Support Option 1. The requirement should be defined if there is the possibility of using single RF receiver chain for intra-band non-contiguous EN-DC. My understanding is that the current spec does not prohibit the sigle RF receiver chain for intra-band non-contiguous EN-DC. |
| Huawei | Sub-topic 5-1: Requirements for FR1 intra-band contiguous CA Issue 5-1-1: Channel bandwidth combination for defining performance requirements Option 2a: From our understanding, the typical LO position is in the middle of the aggregated bandwidth, Option2b will lead the situation that maintenance is needed if more CA configurations with different BWs are introduced in future releases. We think that it is feasible to define bandwidth agnostic requirements. Issue 5-1-2: Channel bandwidth combination for testing OK with recommend WF, I think if there is no CBW combinations with the same BWs in each carrier, only the carrier with smaller CBW should be used for test since the carrier with smaller CBW will be affected by image interference from the carrier with larger CBW anyway. Issue 5-1-3: PDSCH RB allocation Ok with full bandwidth combination with the recommended WF of Issue 5-1-2. Issue 5-1-4: MIMO configuration Ok with recommended WF. Issue 5-1-5: MCS We think the margin of 3dB is too large and even at 16dB we still can’t find the MCS that satisfies 85%~90% of the maximum throughput based on our simulation results. Further evaluation from companies is needed based on more simulation results. Issue 5-1-6: PRB bundling size OK with recommend WF. Sub-topic 5-2: Requirements for intra-band contiguous EN-DC Issue 5-2-1: Duplex mode Ok with recommended WF. Issue 5-2-2: SCS Ok with Option 2. Issue 5-2-3: TDD pattern Ok with recommended WF for 30kHz SCS with the recommended WF of Issue 5-2-2 to only consider 30kHz SCS for TDD. Sub-topic 5-3: Requirements for intra-band non-contiguous EN-DC Issue 5-3-1: Whether to define power imbalance requirement for FR1 intra-band non-contiguous EN-DC Ok with recommended WF. |
| Ericsson | Sub-topic 5-2: Requirements for intra-band contiguous EN-DC Issue 5-2-1: Duplex mode Support the recommended WF. Issue 5-2-2: SCS Support the recommended WF (option 2). Issue 5-2-3: TDD pattern Support the recommended WF (option 1 for TDD SCS=30kHz) |
| Intel | Issue 5-1-1: - Option 2a: Based on our understanding, similar issue as for original option (i.e. no image in the weak CC) can be observed depending on UE implementation. - Option 2b: This option any way requires some testing procedure for scenarios with different CBW on different CCs. We think that it will be not transparent if different test procedures and configurations will be used for different scenarios. - Option 3: Usually it is feasible to define CBW agnostic requirements, especially for scenarios with static condition, which is used for this test. Issue 5-1-2: Recommended WF can be considered as baseline. It can be confirmed in the next RAN4 meeting if no technical concerns will be observed. Issue 5-1-5: Recommended WF is fine for us. Issue 5-2-2: Prefer Option 4. Similar to NR CA with different SCS on different CCs, support of LTE + NR 30 kHz is up to UE capability. Therefore, we suggest to define requirements for both scenarios and test only one scenario based on UE capability. Issue 5-2-3: For 30 kHz, both options are same TDD patterns. Option 2 allows to simplify test configuration and avoid shifting of LTE transmission by 2 subframes with respect to the start of NR transmission. If we are only one company who prefer such test simplification then we can compromise to Option 1. Issue 5-3-1: We think that single RF chain is one of possible implementation for non-contiguous case and depends on UE implementation. Same time, we think that more discussion on test conditions is needed before we agree on definition of such requirements. |
| docomo | Sub-topic 5-3: Requirements for intra-band non-contiguous EN-DC Issue 5-3-1: Whether to define power imbalance requirement for FR1 intra-band non-contiguous EN-DC Only co-located scenario is assumed for both intra-band contiguous and non-contiguous EN-DC cases in RAN4 requirements. It is feasible to assume single RF chain to receive two non-contiguous carriers in co-located scenario. In our understanding, Rx image issue can be observed with non-contiguous EN-DC. If the other companies say that the Rx image issue with non-contiguous EN-DC will not be observed in any case, we prefer to make agreement on this understanding. Otherwise, power imbalance requirement for FR1 intra-band non-contiguous EN-DC should be introduced. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Topic #5: Power imbalance | Sub-topic 5-1: Requirements for FR1 intra-band contiguous CA - Issue 5-1-1: Channel bandwidth combination for defining performance requirements - Candidate options: - Option 2: Define requirements for 5+5 MHz bandwidth for FDD+FDD CA, 10+10 MHz bandwidth for TDD+TDD CA, with the following test applicability - Option 2a - The test is done for any one of the supported bandwidth combination, by using performance requirement for 5+5 MHz FDD+FDD CA or 10+10 MHz TDD+TDD CA. - The tested PRBs shall be placed in the highest lowest part for the CC with lower carrier frequency, and placed in the lowest highest part for the CC with higher carrier frequency. - Option 2b - The test is done for any one of the supported bandwidth combination, by using performance requirement for 5+5 MHz FDD+FDD CA or 10+10 MHz TDD+TDD CA. - The tested PRBs shall be placed in the highest part for the CC with lower carrier frequency, and placed in the lowest part for the CC with higher carrier frequency. - Select the CA combination with largest bandwidth, and select the CA configuration with the same BWs in each carrier for power imbalance test - If there is no supported CA configuration with the same BWs, additional power imbalance test can be considered if necessary. - Note that from 38.101-1, we can observe that most of the CA combinations have the configuration with same BWs, except CA_n71B and CA_n78B. - Option 3: Define generic methodology for selection of CBW combination among all CBW combinations in supported CA configurations - Feedback on the above options in the 1st round: - Is option 2a feasible? - Feasible (CMCC, HW) - HW: From our understanding, the typical LO position is in the middle of the aggregated bandwidth - Issues with option 2a - CTC: it seems the test purpose cannot be fulfilled since the tested PRBs and the interfering PRBs are far away from each other. - Intel: Based on our understanding, similar issue as for original option (i.e. no image in the weak CC) can be observed depending on UE implementation. - Is option 2b feasible? - Feasible (CTC, CMCC) - Issues with option 2b - CTC: one issue is the additional efforts to define additional test if there is no supported CA configuration with the same BWs. - HW: Option 2b will lead the situation that maintenance is needed if more CA configurations with different BWs are introduced in future releases. - Intel: This option any way requires some testing procedure for scenarios with different CBW on different CCs. - Is it feasible to define bandwidth agnostic requirements for option 3? - Feasible (CTC, HW, Intel, [QC]) - QC: Based on our preliminary simulations for 15 kHz SCS, we see a gap of up to 1dB. So, it may be possible to define bandwidth agnostic test. - Need further check (CMCC) Recommendations for 2nd round:: - Check in the 2nd round if the following proposal is agreeable? - Companies to further check if it is feasible to define bandwidth agnostic requirements for option 3 till the next meeting. - If yes, use option 3. - Issue 5-1-2: Channel bandwidth combination for testing Tentative agreement: - As baseline, use the following approach (CTC, CMCC, QC, HW, Intel) - Step 1: First select the CBW combinations with the same BWs in each carrier - If there is no such CBW combination, select the CBW combinations with smallest CBW difference between the two carriers, and the carrier with [larger or smaller] CBW will be used for test. - Step 2: Among the CBW combinations selected from step 1, select the CA combination with largest aggregated CBW - Further discuss in the next meeting that: - In step 1, if there is no CBW combinations with the same BWs in each carrier, whether the carrier with larger or smaller CBW will be used for test? - Issue 5-1-3: PDSCH RB allocation Tentative agreement: - Keep open in this meeting, and TBD after the channel bandwidth combination is agreed - Issue 5-1-4: MIMO configuration Tentative agreement: - 1x2 and 1x4 (CTC, Intel, HW, QC, CMCC) - Issue 5-1-5: MCS Tentative agreement: - Decide in the next meting based on more simulation results for 1T2R and 1T4R - Discuss whether to use 64QAM or 256QAM - Assumptions related to the target SNR point for simulation - Power difference between two CCs - Option 1: 6dB (CMCC, QC, CTC) - Note that it is stated in the WID that 6dB power imbalance is assumed. - Option 2: larger than 6dB - Impairment margin + extra margin - Option 1: 3dB (CTC, QC) - Option 2: lower than 3dB (HW) - Issue 5-1-6: PRB bundling size - Option 1: WB (CTC, CMCC, HW) - Option 2: 2 PRBs (slightly preferred QC) Recommendations for 2nd round: Since option 2 is slightly preferred by QC, check if option 1 is agreeable. Sub-topic 5-2: Requirements for intra-band contiguous EN-DC - Issue 5-2-1: Duplex mode Tentative agreement: - FDD and TDD (Intel, DCM, HW, CMCC, QC, E///) - Issue 5-2-2: SCS - Option 1: 15kHz for both FDD and TDD - Option 2: 15kHz for FDD, 30kHz for TDD (DCM, HW, CMCC, QC, E///) - Option 3: 15kHz for FDD - Option 4 (Intel) - Test #1: LTE FDD + NR FDD 15 kHz - Test #2: LTE TDD + NR TDD 15 kHz, in case UE supports it, otherwise LTE TDD + NR TDD 30 kHz Recommendations for 2nd round: Further discuss option 2 and option 4, and try to select one in this meeting - Issue 5-2-3: TDD pattern - TDD pattern for 30kHz SCS - Option 1: 7D1S2U (agreement in the last meeting, QC, CMCC, HW, E///, Intel) - Option 2: 3DSU+4D (Intel) - Intel: If we are only one company who prefer such test simplification then we can compromise to Option 1. - TDD pattern for 15kHz SCS - Option 1: DSU+DD (Intel) - Other options are not precluded. Tentative agreement: - TDD pattern for 30kHz SCS: 7D1S2U Recommendations for 2nd round: - Discuss the TDD pattern for 15kHz SCS if needed Sub-topic 5-3: Requirements for intra-band non-contiguous EN-DC - Issue 5-3-1: Whether to define power imbalance requirement for FR1 intra-band non-contiguous EN-DC - Views on whether to define the requirements - Option 1: Yes (DCM, SoftBank) - Option 2: No (QC) - Option 3: More discussion is needed (Intel, HW) - Feedback on the two questions in the 1st round: - Whether it is feasible to assume single RF chain to receive two non-continuous carriers in co-located scenario? - Yes: SoftBank, Intel, DCM - SoftBank: the current spec does not prohibit the sigle RF receiver chain for intra-band non-contiguous EN-DC. - If UE uses single RF chain, depending on channel spacing and LO allocation, whether or not the image issue can be observed? - QC: It is questionable how image issue can be observed with non-contiguous EN-DC. But, we are open to discussion. - DCM: In our understanding, Rx image issue can be observed with non-contiguous EN-DC. If the other companies say that the Rx image issue with non-contiguous EN-DC will not be observed in any case, we prefer to make agreement on this understanding. Otherwise, power imbalance requirement for FR1 intra-band non-contiguous EN-DC should be introduced. Tentative agreement: - The possibility of using single RF chain to receive two non-continuous carriers in co-located scenario cannot be precluded. Recommendations for 2nd round: - For UE using single RF chain, further discuss the following proposal from DCM: - If the other companies say that the Rx image issue with non-contiguous EN-DC will not be observed in any case, we prefer to make agreement on this understanding. - Otherwise, power imbalance requirement for FR1 intra-band non-contiguous EN-DC should be introduced. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Way forward on UE power imbalance requirements for FR1 CA and EN-DC | NTT DOCOMO |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round

R4-2008848	Way forward on UE power imbalance requirements for FR1 CA and EN-DC

Type: other		For: Approval
					Source: NTT DoCoMo

Abstract:

Discussion:

Agreement in GTW Session on May.29th

RAN4 agree to introduce test cases for power imbalance requirement for FR1 intra-band non-contiguous EN-DC with test applicable rules

Email discussion in the 2nd round:

Moderator’s note: The WF is discussed in sub-thread [95e][323] NR_perf_enh_Demod_UE - draft WF R4-2008848 (led by NTT DoCoMo).

- Issue 5-1-1: Channel bandwidth combination for defining performance requirements

  - Candidate options:

    - Option 2: Define requirements for 5+5 MHz bandwidth for FDD+FDD CA, 10+10 MHz bandwidth for TDD+TDD CA, with the following test applicability

      - Option 2a

        - The test is done for any one of the supported bandwidth combination, by using performance requirement for 5+5 MHz FDD+FDD CA or 10+10 MHz TDD+TDD CA.

        - The tested PRBs shall be placed in the highest lowest part for the CC with lower carrier frequency, and placed in the lowest highest part for the CC with higher carrier frequency.

      - Option 2b

        - The test is done for any one of the supported bandwidth combination, by using performance requirement for 5+5 MHz FDD+FDD CA or 10+10 MHz TDD+TDD CA.

        - The tested PRBs shall be placed in the highest part for the CC with lower carrier frequency, and placed in the lowest part for the CC with higher carrier frequency.

        - Select the CA combination with largest bandwidth, and select the CA configuration with the same BWs in each carrier for power imbalance test

        - If there is no supported CA configuration with the same BWs, additional power imbalance test can be considered if necessary.

        - Note that from 38.101-1, we can observe that most of the CA combinations have the configuration with same BWs, except CA_n71B and CA_n78B.

    - Option 3: Define generic methodology for selection of CBW combination among all CBW combinations in supported CA configurations

[DCM] Could you check if the following sentence in draft WF is agreeable? RAN4 uses option 3 if it is feasible to define bandwidth agnostic requirements for option 3.

[CTC] OK with this proposal.

[Intel] Support this proposal

[CMCC] OK with the proposal proposed by DCM.

[Huawei] Ok with the proposal.

- Issue 5-1-5: MCS

  - Assumptions related to the target SNR point for simulation

    - Power difference between two CCs

      - Option 1: 6dB (CMCC, QC, CTC)

        - Note that it is stated in the WID that 6dB power imbalance is assumed.

      - Option 2: larger than 6dB

[QC]: On slide 4, we would like to remove option 2 in below because that is contradictory to the WID agreed in the plenary.

  - Power difference between two CCs

    - Option 1: 6dB (CMCC, QC, CTC)

    - Option 2: larger than 6dB

- Issue 5-1-6: PRB bundling size

  - Option 1: WB (CTC, CMCC, HW)

  - Option 2: 2 PRBs (slightly preferred QC)

[DCM]: Can we compromise with Option 1 ?

[Intel] We are fine with Option 1.

[CMCC] Support option 1

[Huawei] Fine with Option 1.

- Issue 5-2-2: SCS

  - Option 1: 15kHz for both FDD and TDD

  - Option 2: 15kHz for FDD, 30kHz for TDD (DCM, HW, CMCC, QC, E///)

  - Option 3: 15kHz for FDD

  - Option 4 (Intel)

    - Test #1: LTE FDD + NR FDD 15 kHz

    - Test #2: LTE TDD + NR TDD 15 kHz, in case UE supports it, otherwise LTE TDD + NR TDD 30 kHz

[DCM]: 15kHz for FDD is common for Option 2 and Option 4. It can be agreed. In terms of TDD, we should focus on whether 15kHz should be considered or not.

[Intel]: Option 4 allows to ensure that UE with different UE capabilities will be covered. Without introduction of TDD requirements for 15 kHz, some UE will not be tested for TDD case.

[CMCC] For TDD, we support LTE TDD+NR TDD 30KHz. Another option is that we can update the option 4 as:

Test #2: LTE TDD + NR TDD 15 30kHz, in case UE supports it, otherwise LTE TDD + NR TDD 30 15kHz

[Intel]: CMCC proposal is also fine for us

[Huawei]: Support option2. Most UE vendors and operators support option 2 and think that 30kHz TDD should be supported. We do not think that there is such UE that only support NR TDD 15kHz and not support TDD 30kHz SCS. Test LTE TDD + NR TDD 15 kHz proposed in Option 4 is not necessary.

- Sub-topic 5-3: Requirements for intra-band non-contiguous EN-DC

Issue test applicability rules

  - UE supports only intra-band contiguous EN-DC, i,e., if UE does not indicate “intraBandENDC-Support”,

      - power imbalance requirement for intra-band contiguous EN-DC is applied

  - UE supports only intra-band non-contiguous EN-DC, i.e., if UE indicates “non-contiguous” in “intraBandENDC-Support” or UE does not indicate “interBandContiguousMRDC”,

      - power imbalance requirement for intra-band non-contiguous EN-DC is applied

  - UE supports both intra-band contiguous and non-contiguous EN-DC, i.e., if UE indicates “both” in “intraBandENDC-Support” or UE indicates “interBandContiguousMRDC”,

      - apply either power imbalance requirement for FR1 intra-band contiguous EN-DC or FR1 intra-band non-contiguous EN-DC

[DCM]: We prefer to discuss test applicability in the 2nd round. Particularly, for the case of UE supports both intra-band contiguous and non-contiguous EN-DC.  From our perspective, we are fine with one test case. But we would like to hear your opinions.

[Intel]: As we commented in GTW, we can agree to define requirements for intra-band non-contiguous EN-DC only in case this scenario will be tested for UE indicates “non-contiguous” in “intraBandENDC-Support”. 
Therefore, we prefer to test only intra-band contiguous EN-DC for UE indicates “both” in “intraBandENDC-Support”, because this scenario is the worst case from image point of view.

Could you please clarify why indications of “interBandContiguousMRDC” should be included in the applicability rule? Based on our understanding, the scope of test is intra-band EN-DC scenario with 1 LTE CC and 1 NR CC.

[DCM] To Intel: RAN4 agreed that some inter-band EN-DC combinations like B42-n77 are treated as "intra-band EN-DC".
It means that all of intra-band EN-DC requirements are applied to such special band combinations, which are defined in 38.101-3. 
In addition, RAN4 also agreed that non-contiguous EN-DC is supported as mandatory in case of such special band combinations, then new UE capability "interBandContiguousMRDC" is introduced for them to indicate whether UE also supports contiguous EN-DC or not. Based on above, such band combinations should be considered for power imbalance tests.

[Intel]: Thank you very much for comments. Based on WID, the scope of requirements is only limited to intra-band EN-DC scenarios:

- FR1 intra-band EN-DC PDSCH demodulation performance requirement with power imbalance - Intra-band contiguous EN-DC with 6dB power imbalance is assumed. - Further study whether to introduce intra-band non-contiguous EN-DC requirements and applicable power imbalance level - Only the NR cell is configured as the weaker power cell and to be tested.

Therefore, our understanding is that inter-band EN-DC scenarios are out of scope, even if such scenario is treated as “intra-band EN-DC”. Same time, we would like to check the views from other companies.

[Huawei]: For the case of UE supports both intra-band contiguous and non-contiguous EN-DC, we prefer to test only intra-band Contiguous EN-DC that is more challenge for UE.

As per the previous agreement, RAN4 agreed to reuse the simulation assumptions for CA, but considering the maximum bandwidth for LTE is 20MHz, the paired channel bandwidth for NR for test is only not larger than 20MHz, we need to consider how to select the bandwidth for NR carrier.

[SoftBank]: To Intel: I would like to further explain the special band combinations treated as "intra-band EN-DC" in 38.101-3. In those band combinations, the frequency is fully overlapped between LTE and NR bands. One example is the combination of band 42 (3400 - 3600MHz) and n77 (3300 - 4200MHz). That is why the requirements for intra-band EN-DC are applied to them. Considering this point, I think that DCM's thought of including those band combinations is reasonable.

[Intel]: Could you please add Option 2 for test applicability for non-contiguous EN-DC?

- Test applicability rules - Option 1 - UE supports only intra-band contiguous EN-DC, i,e., if UE does not indicate “intraBandENDC-Support”, - power imbalance requirement for intra-band contiguous EN-DC is applied - UE supports only intra-band non-contiguous EN-DC, i.e., if UE indicates “non-contiguous” in “intraBandENDC-Support” or UE does not indicate “interBandContiguousMRDC”, - power imbalance requirement for intra-band non-contiguous EN-DC is applied - UE supports both intra-band contiguous and non-contiguous EN-DC, i.e., if UE indicates “both” in “intraBandENDC-Support” or UE indicates “interBandContiguousMRDC”, - apply power imbalance requirement for FR1 intra-band contiguous EN-DC - Option 2 - UE supports only intra-band contiguous EN-DC, i,e., if UE does not indicate “intraBandENDC-Support”, - power imbalance requirement for intra-band contiguous EN-DC is applied - UE supports only intra-band non-contiguous EN-DC, i.e., if UE indicates “non-contiguous” in “intraBandENDC-Support” - power imbalance requirement for intra-band non-contiguous EN-DC is applied - UE supports both intra-band contiguous and non-contiguous EN-DC, i.e., if UE indicates “both” in “intraBandENDC-Support” - apply power imbalance requirement for FR1 intra-band contiguous EN-DC - Other options are not precluded.

Recommendation:		Agreeable

Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2008848, WF | agreeable |

Topic #6: NR CA CQI reporting requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006040 | China Telecom | - Proposal 1: For duplex mode and SCS, reuse the combinations and applicability rules from PDSCH normal CA. Proposal 2: Use 2D1S1U with S=11:3:0 for 120kHz SCS. Proposal 3: Use 8 slots for CSI reporting periodicity. Proposal 4: Following the methodology used in LTE, the difference between the wideband CQI indices of Pcell and the first Scell as well as the difference between the wideband CQI indices of the first Scell and the other Scell(s) (if any) shall be not smaller than 2, for more than 90% of the time. - Proposal 5: SNRPcell = 10dB and SNRScell = 4dB for 2DL CA CQI test. Proposal 6: SNRPcell = 12dB, SNRScell1 = 6dB, SNRScell2, 3,… = 0dB for 3 or more DL CA CQI test. Proposal 7: Delta CQI threshold = 2 for 2 or more DL CA CQI test. Proposal 8: Use 1T2R and 1T4R for CA CQI test. Proposal 9: For 4Rx band, reduce the signal power density by 3dB compared to that for 2Rx. |
| R4-2007142 | NTT DOCOMO, INC. | Proposal 1: Adopt Option 1 for duplex mode and SCS, i.e. reuse the combinations and applicability rules from PDSCH CA Proposal 2: Adopt Option 1 for 120 kHz SCS, i.e. assume TDD pattern as 3D1S1U (S=10:2:2) for CA CQI requirements |
| R4-2007225 | Huawei, HiSilicon | Proposal 1: For Duplex mode and SCS, use option2 - FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS - FR2: TDD + TDD with 120 kHz SCS Proposal 2: For 120kHz SCS, use 3D1S1U, corresponding CSI reporting period is 10 slots. Proposal 3: Use 1T2R and 1T4R and reduce the signal power density by 3dB compared to that for 2Rx. Proposal 4: To find suitable SNR configuration and CQI threshold, more evaluations are needed based on common simulation assumption. Proposal 5: More evaluation are needed to verify if the CA CQI performance requirements can be defined in a bandwidth agnostic way based on common simulation assumption. If it has been verified, only the largest bandwidth combination under all CA configurations supported by the UE should be tested. |
| R4-2008113 | Qualcomm Incorporated | Proposal 1: Define the CA CQI requirements for following cases: - FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS - FR2: TDD + TDD with 120 kHz SCS Proposal 2: For defining FR2 CA CQI requirements, use DDSU (S = 11D+3G) TDD Pattern and CSI reporting periodicity of 8 slots. Proposal 3: Reuse the test framework for CA CQI reporting tests in LTE for defining NR CA CQI reporting tests. Proposal 4: Define FR1 CA CQI reporting requirements with the following configuration: - SNR configuration for 2DL CA CQI test: SNRPcell = 10dB and SNRScell = 4dB. - SNR configuration for 3 or more DL CA CQI test: SNRPcell = 12dB, SNRScell1 = 6dB, SNRScell2, 3,… = 0dB. - Delta CQI threshold for CA CQI test = 2 for 2 or more DL CA. |

Open issues summary

Sub-topic 6-1: Duplex mode and SCS combinations

Issue 6-1: Duplex mode and SCS combinations

- Agreement in RAN4 #94e-bis

  - Agreed duplex mode and SCS for CA normal PDSCH (R4-2005546, WF on CA normal)

    - Test #1: FDD 15 kHz + FDD 15 kHz

    - Test #2: FDD 15 kHz + TDD 30 kHz, in case UE supports different SCS on different carriers for FDD-TDD CA, otherwise FDD 15 kHz + TDD 15 kHz

    - Test #3: TDD 30 kHz + TDD 30 kHz, in case UE supports it, otherwise TDD 15 kHz + TDD 30 kHz

  - Options for duplex mode and SCS for CA CQI (R4-2005548, WF on CA CQI)

    - Option 1: Reuse the combinations and applicability rules from PDSCH CA

    - Option 2:

      - FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS

      - FR2: TDD + TDD with 120 kHz SCS

- Proposals

  - Option 1: Reuse the combinations and applicability rules from PDSCH CA (CTC, DCM)

    - CTC: Some typical CA scenarios, such as FDD 15 kHz + TDD 30 kHz, are missing in option 2.

  - Option 2: (Huawei, Qualcomm)

    - FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS

    - FR2: TDD + TDD with 120 kHz SCS

      - Huawei: FDD + TDD and TDD with 15 kHz scenarios have been verified in the Rel-15 PDSCH CA CSI reporting test.

- Moderator’s observation

  - The duplex mode and SCS combinations for performance requirements and testing can be discussed separately.

  - For performance requirements:

    - The difference between the two options is: FDD 15 kHz + TDD 30 kHz, FDD 15 kHz + TDD 15 kHz and TDD 15 kHz + TDD 30 kHz are not covered in option 2.

    - If one UE only supports FDD 15 kHz + TDD 30 kHz CA (or only supports FDD 15 kHz + TDD 15 kHz, or TDD 15 kHz + TDD 30 kHz), there will be no requirements for the UE.

  - For applicability rule:

    - The number of tests can be further discussed, taking into account the main purpose of CA CQI test is to verify independent CQI calculation in each CC.

- Recommended WF

  - For the performance requirements, reuse the combinations from PDSCH normal CA.

  - For the applicability rule, discuss whether or not the number of tests can be reduced compared to PDSCH normal CA.

Sub-topic 6-2: Channel bandwidth and test applicability rule

Issue 6-2: Channel bandwidth and test applicability rule

- Agreement in RAN4 #94e-bis (R4-2005548, WF)

  - Define performance requirements for all channel bandwidths listed in TS 38.101-1 and TS 38.101-2 for FR1 and FR2.

  - Further discuss the test applicability rule

- Proposals

  - Option 1: If it is verified that CA CQI performance requirements can be defined in a bandwidth agnostic way based on common simulation assumption, only test the largest bandwidth combination under all CA configurations supported by the UE (Huawei)

- Recommended WF

  - TBA based on tshe feedback from more companies.

Sub-topic 6-3: TDD UL-DL patterns and CSI reporting periodicity

Issue 6-3-1: TDD UL-DL pattern for 120 kHz SCS

- Agreement in RAN4 #94e-bis (R4-2005548, WF)

  - TDD pattern

    - For 120kHz SCS

      - Option 1: 3D1S1U with S=10:2:2

      - Option 2: 2D1S1U with S=11:3:0

- Proposals

  - Option 1: 3D1S1U with S=10:2:2 (DCM, Huawei)

    - DCM: align with CA PDSCH normal demodulation test

    - Huawei: For 120 kHz SCS, option 1 is the most typical scenario.

  - Option 2: 2D1S1U with S=11:3:0 (CTC, Qualcomm)

    - CTC, QC: Align with Rel-15 FR2 single carrier CQI test.

- Recommended WF

  - With AWGN channel assumed, the CQI reporting performance will not be impacted by the TDD pattern, so it is suggested to make discussion based on majority’s view after the 1st round email discussion.

Issue 6-3-2: CSI reporting periodicity for 120 kHz SCS

- Agreement in RAN4 #94e-bis (R4-2005548, WF)

  - CSI reporting periodicity

    - For 120kHz SCS

      - Option 1: 10 slots

      - Option 2: 8 slots

- Proposals

  - Option 1: 10 slots (Huawei)

  - Option 2: 8 slots (CTC, Qualcomm)

- Recommended WF

  - Depend on the agreement on TDD pattern in issue 6-3-1

    - If TDD pattern of 3D1S1U is agreed, use option 1

    - If TDD pattern of 2D1S1U is agreed, use option 2

Sub-topic 6-4: Antenna configuration

Issue 6-4-1: Antenna configuration

- Agreement in RAN4 #94e-bis (R4-2005548, WF)

  - Option 1: 1T2R and 1T4R (CTC, Ericsson, Intel, Huawei)

  - Other options are not precluded

- Proposals

  - Option 1: 1T2R and 1T4R (CTC, Huawei)

- Recommended WF

  - Can we agree with option 1?

Issue 6-4-2: Signal power density for 2Rx and 4Rx bands

- Agreement in RAN4 #94e-bis (R4-2005548, WF)

  - Option 1: For 4Rx band, reduce the signal power density by 3dB compared to that for 2Rx

  - Other options are not precluded

- Proposals

  - Option 1: For 4Rx band, reduce the signal power density by 3dB compared to that for 2Rx (CTC, Huawei)

- Recommended WF

  - Can we agree with option 1?

Sub-topic 6-5: Test metric

Issue 6-5-1: General principle for test metric

- Agreement in RAN4 #94e-bis (R4-2005548, WF)

  - Following the methodology used in LTE, measure the difference between the wideband CQI indices of Pcell and the first Scell as well as the difference between the wideband CQI indices of the first Scell and the other Scell(s) (if any).

  - Other options are not precluded

- Proposals

  - Option 1: Following the methodology used in LTE, the difference between the wideband CQI indices of Pcell and the first Scell as well as the difference between the wideband CQI indices of the first Scell and the other Scell(s) (if any) shall be not smaller than 2, for more than 90% of the time. (CTC, Qualcomm)

    - To make it more clear, additional clarification is added for the option 1, which is marked in red.

- Recommended WF

  - TBA based on the feedback from more companies.

Issue 6-5-2: SNR configuration for 2DL CA CQI test

- Agreement in RAN4 #94e-bis (R4-2005548, WF)

  - Option 1: SNRPcell = 10dB and SNRScell = 4dB

  - Other options are not precluded

- Proposals

  - For FR1

    - Option 1: SNRPcell = 10dB and SNRScell = 4dB (CTC, QC)

      - CTC, QC: have verified this proposal by simulation

    - Option 2: Need more time for simulation (HW)

  - For FR2

    - Option 1: SNRPcell = 10dB and SNRScell = 4dB (CTC)

    - Option 2: Need more time for simulation (QC, HW)

      - QC: may be able to confirm during the meeting

- Recommended WF

  - Considering the short time for tdoc preparation, it is suggested to keep the agreement in the last meeting, and make decision in the next meeting.

Issue 6-5-3: SNR configuration for 3DL CA CQI test

- Agreement in RAN4 #94e-bis (R4-2005548, WF)

  - Option 1: SNRPcell = 12dB, SNRScell1 = 6dB, SNRScell2, 3,… = 0dB

  - Other options are not precluded

- Proposals

  - For FR1

    - Option 1: SNRPcell = 12dB, SNRScell1 = 6dB, SNRScell2, 3,… = 0dB (CTC, Qualcomm)

    - Option 2: Need more time for simulation (HW)

  - For FR2

    - Option 1: SNRPcell = 12dB, SNRScell1 = 6dB, SNRScell2, 3,… = 0dB (CTC)

    - Option 2: Need more time for simulation (QC, HW)

- Recommended WF

  - Similar situation as issue 6-5-2 for 2DL CA, it is suggested to keep the agreement in the last meeting, and make decision in the next meeting.

Issue 6-5-4: Delta CQI threshold

- Agreement in RAN4 #94e-bis (R4-2005548, WF)

  - Option 1: thr = 2 for 2 or more DL CA

  - Other options are not precluded

- Proposals

  - For FR1

    - Option 1: thr = 2 for 2 or more DL CA (CTC, Qualcomm)

    - Option 2: Need more time for simulation (HW)

  - For FR2

    - Option 1: thr = 2 for 2 or more DL CA (CTC)

    - Option 2: Need more time for simulation (QC, HW)

- Recommended WF

  - Similar situation as issue 6-5-2 and 6-5-3 for SNR configuration, it is suggested to keep the agreement in the last meeting, and make decision in the next meeting.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| China Telecom | Issue 6-1: Duplex mode and SCS combinations Ok with the recommended WF, and can decide the applicability rule in the next meeting. Issue 6-2: Channel bandwidth and test applicability rule For the test applicability, OK to follow the LTE approach, i.e., for each agreed duplex mode and SCS combination for testing: - CA capability where the tests apply: Test any of one of the supported CA capabilities with largest aggregated CA bandwidth combination - CA configuration from the selected CA capability where the tests apply: Test any one of the supported CA configurations with largest aggregated CA bandwidth combination Issue 6-3-1: TDD UL-DL pattern for 120kHz SCS Either option 1 or option 2 is ok. Issue 6-3-2: CSI reporting periodicity for 120kHz SCS Ok with the recommended WF Issue 6-4-1: Antenna configuration Ok with the recommended WF Issue 6-4-2: Signal power density for 2Rx and 4Rx bands Ok with the recommended WF Issue 6-5-1: General principle for test metric Support option 1. Issue 6-5-2: SNR configuration for 2DL CA CQI test Ok with the recommended WF Issue 6-5-3: SNR configuration for 3DL CA CQI test Ok with the recommended WF Issue 6-5-4: Delta CQI threshold Ok with the recommended WF Others: |
| CMCC | Issue 6-1: Duplex mode and SCS combinations OK with the recommended WF. Issue 6-4-1: Antenna configuration OK with the recommended WF Issue 6-4-2: Signal power density for 2Rx and 4Rx bands OK with the recommended WF Issue 6-5-1: General principle for test metric OK with the improved option 1. |
| Qualcomm | Issue 6-1: Duplex mode and SCS combinations Our main concern is number of test cases. Since we have already tested all the combinations for PDSCH CA requirements, we don’t see why we need to test all the combinations again for CA CQI reporting. Same principle was used for normal demod requirements in Rel-15 where we defined requirements for different BW/SCS and TDD patterns for normal demod but only tested one of them in CSI reporting. Issue 6-2: Channel bandwidth and test applicability rule Based on preliminary simulation results, it seems ok. But, we would need more time to run all the simulations. So, we would like to keep it open until next meeting. Issue 6-3-1: TDD UL-DL pattern for 120kHz SCS Ok with recommended WF. Issue 6-3-2: CSI reporting periodicity for 120kHz SCS Ok with recommended WF. Issue 6-4-1: Antenna configuration Ok with recommended WF. Issue 6-4-2: Signal power density for 2Rx and 4Rx bands We have a question for clarification. Should this be for 4Rx requirements instead of 4Rx band because UE can declare to support 4Rx even for 2Rx bands? Issue 6-5-1: General principle for test metric As delta CQI threshold is still an open issue, we prefer to change the language to “shall not be smaller than a threshold, for more than 90% of time”. Issue 6-5-2: SNR configuration for 2DL CA CQI test Ok with recommended WF. Issue 6-5-3: SNR configuration for 3DL CA CQI test Ok with recommended WF. Issue 6-5-4: Delta CQI threshold Ok with recommended WF. |
| Huawei | Issue 6-1: Duplex mode and SCS combinations Still prefer Option 2. Supporting of different CA combinations are covered in PDSCH CA performance requirements and no need to repeat to verify them in CA CQI test. FDD CA and TDD CA with same numerology is mandatory for UE to support, it is enough to check the CA CQI performance by using the CA with same numerology. Issue 6-2: Channel bandwidth and test applicability rule Channel bandwidth: Based on our observations, it is feasible to define the CA CQI performance requirements in bandwidth agnostic way. Test applicability rule: we prefer to reuse the test applicability rules of LTE CA CQI reporting as stated by CTC. Issue 6-3-1: TDD UL-DL pattern for 120kHz SCS We support option 1 of DDDSU since it is the most typical pattern. Also the UL PUSCH performance requirements only consider this TDD pattern for 120kHz SCS. Issue 6-3-2: CSI reporting periodicity for 120kHz SCS OK with recommended WF Issue 6-4-1: Antenna configuration OK with recommended WF Issue 6-4-2: Signal power density for 2Rx and 4Rx bands OK with recommended WF Issue 6-5-1: General principle for test metric OK with option1, but the CQI threshold between two CCs should be further discussed based on simulation. Issue 6-5-2: SNR configuration for 2DL CA CQI test OK with recommended WF Issue 6-5-3: SNR configuration for 3DL CA CQI test OK with recommended WF Issue 6-5-4: Delta CQI threshold OK with recommended WF |
| Ericsson | Issue 6-3-1: TDD UL-DL pattern for 120kHz SCS We prefer Option 2: The benefit of option 2 (FR2.120-2, DDSU) is to reuse the single carrier CQI definition test setup such as CSI-RS configuration or reporting delay. It will save our time. Issue 6-3-2: CSI reporting periodicity for 120kHz SCS Option 2 (based on DDSU). Issue 6-4-1: Antenna configuration Support the recommended WF (option 1) Issue 6-4-2: Signal power density for 2Rx and 4Rx bands Support the recommended WF (option 1) Issue 6-5-1: General principle for test metric We are fine with the option 1 with texts in read. Issue 6-5-2: SNR configuration for 2DL CA CQI test Support the recommended WF (keep the options open). We guess option 1 comes from LTE, but the test points of NR CQI definition test are different from LTE. We need more time to check. Issue 6-5-3: SNR configuration for 3DL CA CQI test Same comment as 6-5-2. Keep options open. Issue 6-5-4: Delta CQI threshold Same comment as 6-5-2. Keep options open. Others: |
| Intel | Issue 6-1: We support Option 2. CQI requirements can be defined for more typical scenarios and testing of all scenarios from Normal CA requirements is rather redundant. Issue 6-4-2: Option 1 can be considered as baseline. It can be confirmed in the next RAN4 meeting if no technical concern will be observed. Issue 6-5-1: Principle is fine for us. Exact CQI delta value can be decided later based on results from companies. |
| docomo | Issue 6-1: Duplex mode and SCS combinations OK with the recommended WF Issue 6-3-1: TDD UL-DL pattern for 120kHz SCS Both Option 1 and Option 2 have their own benefits. We still prefer option 1. Issue 6-3-2: CSI reporting periodicity for 120kHz SCS OK with the recommended WF |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Topic #6: CA CQI | - Issue 6-1: Duplex mode and SCS combinations - For the performance requirements: - Option 1: Reuse the combinations from PDSCH normal CA (CTC, DCM, CMCC) - Option 2: (Huawei, Intel) - FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS - FR2: TDD + TDD with 120 kHz SCS - Applicability rule if option 1 for the performance requirements is agreed: - Further discuss whether or not the number of tests can be reduced compared to PDSCH normal CA. (CTC, CMCC, DCM) Recommendations for 2nd round: - Further discuss and come up with comprised proposal. - Issue 6-2: Channel bandwidth and test applicability rule - Is it feasible to define CA CQI performance requirements in a bandwidth agnostic way? - Yes (CTC, HW) - Yes based on preliminary simulation results, but would need more time to run all the simulations (QC) - For the applicability rule: - Option 1 (CTC, HW) - For each agreed duplex mode and SCS combination for testing: - CA capability where the tests apply: Test any of one of the supported CA capabilities with largest aggregated CA bandwidth combination - CA configuration from the selected CA capability where the tests apply: Test any one of the supported CA configurations with largest aggregated CA bandwidth combination Tentative agreement: - Decide in the next meeting if it is feasible to define CA CQI performance requirements in a bandwidth agnostic way. Recommendations for 2nd round: - For the applicability rule, check if option 1 is agreeable - Issue 6-3-1: TDD UL-DL pattern for 120kHz SCS - Option 1: 3D1S1U with S=10:2:2 (DCM, Huawei, CTC) - Option 2: 2D1S1U with S=11:3:0 (CTC, Qualcomm, Ericsson) - Ericsson: Reusing the single carrier CQI definition test setup such as CSI-RS configuration or reporting delay will save our time. - CTC, QC: Ok to follow the majority’s view, since under AWGN channel assumed, the CQI reporting performance will not be impacted by the TDD pattern Recommendations for 2nd round: - Further discuss and try to make decision - Issue 6-3-2: CSI reporting periodicity for 120kHz SCS Tentative agreement: - Depend on the agreement on TDD pattern in issue 6-3-1 (CTC, QC, Huawei, Ericsson, DCM) - If TDD pattern of 3D1S1U is agreed, use option 1 - If TDD pattern of 2D1S1U is agreed, use option 2 - Issue 6-4-1: Antenna configuration Tentative agreement: - 1T2R and 1T4R (CTC, Huawei, CMCC, QC, Ericsson) - Issue 6-4-2: Signal power density for 2Rx and 4Rx bands requirements - Option 1: For 4Rx bands requirements, reduce the signal power density by 3dB compared to that for 2Rx (CTC, Huawei, CMCC, Ericsson, Intel) - Intel: Option 1 can be considered as baseline. It can be confirmed in the next RAN4 meeting if no technical concern will be observed. Tentative agreement: - Use Option 1 as baseline, and it can be confirmed in the next RAN4 meeting if no technical concern will be observed. - Issue 6-5-1: General principle for test metric Tentative agreement: - Following the methodology used in LTE, the difference between the wideband CQI indices of Pcell and the first Scell as well as the difference between the wideband CQI indices of the first Scell and the other Scell(s) (if any) shall be not smaller than a threshold, for more than 90% of the time. (CTC, Qualcomm, CMCC, Huawei, Ericsson, Intel) - Issue 6-5-2: SNR configuration for 2DL CA CQI test Tentative agreement: - Option 1: SNRPcell = 10dB and SNRScell = 4dB - Other options are not precluded - Make decision in the next meeting - Issue 6-5-3: SNR configuration for 3DL CA CQI test Tentative agreement: - Option 1: SNRPcell = 12dB, SNRScell1 = 6dB, SNRScell2, 3,… = 0dB - Other options are not precluded - Make decision in the next meeting - Issue 6-5-4: Delta CQI threshold Tentative agreement: - Option 1: thr = 2 for 2 or more DL CA - Other options are not precluded - Make decision in the next meeting |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Way forward on CA CQI reporting requirements | China Telecom |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round

R4-2008849	Way forward on CA CQI reporting requirements

Type: other		For: Approval
					Source: China Telecom

Abstract:

Discussion:

Moderator’s note: The WF is discussed in sub-thread [95e][323] NR_perf_enh_Demod_UE - draft WF R4-2008849 (led by China Telecom).

- Issue 6-1: Duplex mode and SCS combinations

  - For the performance requirements:

    - Option 1: Reuse the combinations from PDSCH normal CA (CTC, DCM, CMCC)

    - Option 2: (Huawei, Intel)

  - FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS

  - FR2: TDD + TDD with 120 kHz SCS

  - Applicability rule if option 1 for the performance requirements is agreed:

    - Further discuss whether or not the number of tests can be reduced compared to PDSCH normal CA. (CTC, CMCC, DCM)

[CTC]: For the performance requirements, we still prefer option 1. If typical CA scenarios such as FDD 15 kHz + TDD 30 kHz, are not covered, for UEs only support FDD 15 kHz + TDD 30 kHz CA (or only support FDD 15 kHz + TDD 15 kHz, or TDD 15 kHz + TDD 30 kHz), there will be no requirements.

For the test applicability, note that the following 3 test cases have been defined for CA normal PDSCH test (R4-2005546):

- Test #1: FDD 15 kHz + FDD 15 kHz

- Test #2: FDD 15 kHz + TDD 30 kHz, in case UE supports different SCS on different carriers for FDD-TDD CA, otherwise FDD 15 kHz + TDD 15 kHz

- Test #3: TDD 30 kHz + TDD 30 kHz, in case UE supports it, otherwise TDD 15 kHz + TDD 30 kHz

As a compromise, we can accept only test 2 of the 3 cases above for CA CQI test applicability, and FFS on the detailed applicability rule.

So, for the sake for progress, we would propose a compromised WF is as follow, we would like to check if it is acceptable for all companies:

  - For the performance requirements: use option 1

  - For applicability rule: reduce the number of tests from 3 to 2, FFS the detailed applicability rule.

[Apple] We support option 2 to avoid many test cases.

[DCM]: We still prefer option 1 because option 2 cannot cover the FDD 15 kHz + TDD 30 kHz case. Also, we can compromise CTC’s proposal as far as FDD 15 kHz + TDD 30 kHz case is covered.

[Intel]: Based on our understanding, the purpose of test is to guarantee proper CQI reporting for CA scenarios and there is no impact on CQI reporting on whether it is FDD-FDD, TDD-TDD or FDD-TDD CA scenario. Demodulation and general processing for FDD-TDD scenarios will be verified by Normal CA requirements. Therefore, we suggest to focus on FDD-FDD and TDD-TDD scenarios for CQI requirements definition.

[QC] We prefer Option 2 to reduce the number of test cases since we already covered all combinations in PDSCH CA requirements.

[Huawei]: We prefer Option 2 considering TDD-FDD CA has been verified in normal CA performance requirements.

- Issue 6-2: Channel bandwidth and test applicability rule

  - Decide in the next meeting on whether it is feasible to define CA CQI performance requirements in a bandwidth agnostic way.

  - For the applicability rule:

    - Option 1 (CTC, HW)

  - For each agreed duplex mode and SCS combination for testing:

- CA capability where the tests apply: Test any of one of the supported CA capabilities with largest aggregated CA bandwidth combination

- CA configuration from the selected CA capability where the tests apply: Test any one of the supported CA configurations with largest aggregated CA bandwidth combination

[CTC]: For the applicability rule, option 1 is aligned with the applicability for CA CQI test in LTE. We would like to check if option 1 is agreeable for all companies.

[Intel]: More discussion on this test applicability rule is needed, because, probably, we need to take into account all existing NR UE capabilities (similar to SDR and Normal CA requirements).

[Huawei]: Our understanding for this test applicability rule is based on the agreement that it is feasible to define CA CQI performance requirements in a bandwidth agnostic way.

- Issue 6-3-1: TDD UL-DL pattern for 120kHz SCS

  - Option 1: 3D1S1U with S=10:2:2 (DCM, Huawei, CTC)

  - Option 2: 2D1S1U with S=11:3:0 (CTC, Qualcomm, Ericsson)

[CTC]: Our understanding of companies’ views in the 1st round is:

- CTC, QC: Ok to follow the majority’s view, since under AWGN channel assumed, the CQI reporting performance will not be impacted by the TDD pattern

- DCM, HW: prefer option 1 since it is the most typical pattern for FR2.

- E///: prefer option 2, since reusing the single carrier CQI definition test setup such as CSI-RS configuration or reporting delay will save our time.

Since anyway, the TDD pattern will not impact the CQI reporting performance, and either pattern have been tested in some of the existing tests, so we would like to ask DCM, Huawei and E/// the possibility of making compromise and moving forward in this meeting?

[Ericsson] In our knowledge, both DDDSU and DDSU are used (or will be used) in the commercial network. So we can say both options are most typical patterns. This is also the reason we want to use the single carrier setup. So we prefer to option 2.

[Apple] We support option 2, to re-use the same TDD pattern as single CC CQI reporting tests.

[DCM] For the sake of progress, both Option1 and Option 2 are fine.

[Intel] We are fine with both options. Same time, we have a slight preference for Option 2, because it is used for single carrier requirements.

[Huawei]: We still prefer Option 1, considering Option 2 is already used for single carrier, for test coverage, Option 1 can be used for CA CQI test.

Recommendation:		Agreeable

Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2008849 | agreeable |