3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2009611

Electronic Meeting, 17 – 28 Aug, 2020

Agenda item:	7.8.1.1.1

Source:	Apple Inc.

Title:	On UE Demodulation and CSI Reporting Requirements with Ultra-low BLER

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#95e URLLC requirements for UE demodulation and CSI reporting with ultra-low BLER were discussed and way forward [1] was approved.

The agreements and open issues for UE demodulation and CSI reporting requirements with ultra-low BLER are:

UE Demodulation - MCS 14 - Antenna configuration: 1x2, 1x4 - UE applicability agreement: Requirements are applicable for UEs that support both MCS Table 3 (dl-64QAM-MCS-TableAlt) and CQI Table 3 (cqi-TableAlt). - X is confirmed as 0.5dB for the UE and 1dB for the BS - Number of UE test cases for the FMCS requirement: 4 (FDD/2RX, FDD/4RX, TDD/2RX, TDD/4RX) - For UE, both TDD and FDD are tested with maximum supported RX CQI Reporting with ultra-low BLER - Option 1: No CQI requirement - Option 2: CQI requirement with median CQI and BLER with higher/lower CQI followed, together with test applicability rule on CQI/FMCS Test applicability rule for FMCS and CQI - Option 1: Introduce applicability rule - Option 2: No rule; both tested Requirements in FR2 - Option 1: FR2 requirements - Option 2: No FR2 requirements - Option 3: Defer decision to next meeting

In this contribution we address the open issues related to UE demodulation and CQI reporting requirements with ultra-low BLER.

# 2. Discussion

## UE Demodulation Requirements

Based on the simulation assumptions in [1] for UE demodulation requirements for ultra-low BLER, we provide simulation results below.

Table 1: Simulation Results for Ultra-low BLER

| SNR @1e-5 BLER | 1x2 | 1x4 |
| --- | --- | --- |
| FDD | -0.15 | -2.95 |
| TDD | 0 | -2.7 |

Requirements in FR2

There has been discussion for the past meetings to introduce requirements for URLLC in FR2. In FR2 we don’t see the necessity to introduce requirements with ultra-low BLER as there might not be use cases for it. Hence, we propose not to introduce UE demodulation requirements in FR2 for ultra-low BLER.

Proposal #1: Do no define requirements in FR2 for ultra-low BLER for URLLC

## Requirements for CQI Reporting

In the past RAN4 meeting CQI reporting requirements with Table 3 have been discussed. Based on the test feasibility study for introducing requirements with ultra-low BLER, it was determined that it is feasible to introduce requirements at slightly higher SNR than operating SNR in order to finish the test with early pass-fail criteria within a reasonable time. For CQI reporting in AWGN, the test will have to be run to test around the 10-5 BLER requirement, and there might not be an early pass-fail possibility in this case. Hence, we propose not to define CQI requirements with ultra-low BLER.

Proposal 2: Do not define CQI reporting test in AWGN channel with ultra-low BLER target.

# 3. Conclusion

In this paper, we provide our views on the remaining open issues related to UE demodulation and CQI reporting requirements with ultra-low BLER. Our proposals are captured below:

Proposal #1: Do no define requirements in FR2 for ultra-low BLER for URLLC

Proposal 2: Do not define CQI reporting test in AWGN channel with ultra-low BLER target.

# Reference

- R4-2008816, “Way Forward on PMI Reporting Test Cases with Rel-16 Type II Codebook”, Qualcomm, Ericsson.