3GPP TSG-RAN WG4 Meeting #96-e		R4-2009729

E-meeting, 17 – 28 August, 2020

Agenda item:	7.10.3.2

Source:	Intel Corporation

Title:	Discussion on SDR requirements for FR2 DL 256QAM

Document for:	Discussion

# Introduction

In the previous meeting WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM was agreed [1].

In this paper we provide our view on UE SDR requirements for FR2 256QAM.

# Discussion

Taking into account that FR2 requirements are limited by testable SNR, we suggest to identify scenarios which can be executed under existing testing methodology.

TR 38.810 [2] provides information on testable SNR for demodulation requirements. Same time, testable SNR for Normal and SDR requirements is different because generation of noise is not needed for SDR and more power can be used for generation of useful signal. Recently, in our paper [3], we analysed the testable SNR for SDR requirements. In Table 1 we provide estimations on testable SNR and MCS (using impairments results from Table 3) for different aggregation factors and different bands for DFF method.

Table 1. Testable SNR and MCS for SDR requirements

| Band |  | Aggregated channel bandwidth, [MHz] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 | 500 | 600 | 700 | 800 | 1000 |
| n257, 258, 261 | TE SNR, [dB] | 32.1 | 29.3 | 26.3 | 23.2 | 22.3 | 21.5 | 20.8 | 20.2 | 18.8 |
|  | Feasible MCS (Rank 1) | MCS27 | MCS27 | MCS26 | MCS23 | MCS22 | MCS22 | MCS21 | MCS21 | N/A |
|  | Feasible MCS (Rank 2) | MCS27 | MCS26 | MCS23 | MCS21 | N/A | N/A | N/A | N/A | N/A |
| n260 | TE SNR, [dB] | 29.5 | 26.7 | 23.7 | 20.6 | 19.7 | 18.9 | 18.2 | 17.6 | 16.2 |
|  | Feasible MCS (Rank 1) | MCS27 | MCS26 | MCS22 | MCS21 | MCS20 | N/A | N/A | N/A | N/A |
|  | Feasible MCS (Rank 2) | MCS26 | MCS23 | MCS21 | N/A | N/A | N/A | N/A | N/A | N/A |

Table 2. PDSCH ideal results

| Rank configuration | MCS 20 | MCS 21 | MCS 22 | MCS 23 | MCS 24 | MCS 25 | MCS 26 | MCS 27 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank 1 | 16.5 | 17.1 | 18.5 | 19.7 | 20.4 | 21.6 | 22.7 | 24.2 |
| Rank 2 | 19.5 | 20.0 | 21.4 | 22.7 | 23.3 | 24.4 | 25.5 | 27.2 |

Table 3. PDSCH impairments results

| Rank configuration | MCS 20 | MCS 21 | MCS 22 | MCS 23 | MCS 24 | MCS 25 | MCS 26 | MCS 27 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank 1 | 19.5 | 20.1 | 21.5 | 22.7 | 23.4 | 24.6 | 25.7 | 27.2 |
| Rank 2 | 22.5 | 23.0 | 24.4 | 25.7 | 26.3 | 27.4 | 28.5 | 30.2 |
| Note: Based on simulation results from [4], there is about 3 dB difference between average ideal and average impairment results for 64QAM modulation. We expect that SNR difference for 256QAM will be not lower and assume 3 dB difference for these results. |  |  |  |  |  |  |  |  |

From results in Table 1, we can observe that 256QAM MCS and Rank 2 can be tested for channel bandwidth up to 400 MHz for bands n257, n258, n261 and up to 200 MHz for band n260. Also, we can observe that rather high MCSs (i.e. MCS 24-27) can be tested mainly for 50 and 100 MHz aggregated channel bandwidth, which is not the case for SDR requirements. Therefore, we suggest not to define FR2 SDR requirements for 256QAM.

Proposal 1:	Do not define SDR requirements for FR2 256QAM.

# Conclusion

In this paper we provided view on FR2 256QAM SDR requirements definition and made the following proposals:

Proposal 1:	Do not define SDR requirements for FR2 256QAM.

# References

- R4-2008817 “WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM”, China Telecom, RAN4 #95-e, May 2020.

- TR 38.810 “Study on test methods”

- R4-1902881 “NR SDR performance requirements”, Intel, RAN4 #90bis, April 2019.

- R4-1905736 “Summary of simulation results of NR FR2 SDR requirements”, Intel, RAN4 #91, May 2019.