3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2009752

# Electronic Meeting, 17–28 Aug., 2020

Agenda item:       7.13.1.4

Source:                Intel Corporation

Title:                    Discussion on requirements for spatial relation info switch

Document for:    Discussion

# Introduction

In last meeting, there are some agreements about uplink spatial relation info switch:

- Define requirement for BC bit-0 UE. - No timing tracking is needed when PUSCH/PUCCH and SRS associated with different DL-RSs in one slot.

There are still some open issues:

- Whether UE should meet initial transmit timing accuracy requirements after UL spatial relation switch - Whether to consider timing tracking when associated DL-RS - UE behavior When the UL signal has spatial relation to an unknown DL RS - How to deduce the delay requirement for UE which only supports BC Bit-0 when spatial relation is associated with DL RS - Delay requirement for MAC CE based spatial relation info switching associated with DL-RS for PUCCH - Delay requirement for RRC based spatial relation info switching associated with DL-RS for P-SRS

In this contribution we continue to provide our views on requirements for uplink spatial relation info switch.

# Discussion

## UL signal has spatial relation to an unknown TCI-state

There are still open issues for UL signal which has spatial relation to an unknow TCI-state:

- When the UL signal has spatial relation to an unknown DL RS, - Option 1: UE transmits using previous TX beam - Option 2: Drop UL transmission until TCI state is known - Option 3: Up to UE implementation and no need to be specified.

For option 1, the Tx beam quality can’t be guaranteed, and network side may not receive UL signal correctly. For option 2, additional time is needed for acquiring new TCI state and the delayed time is not expectable. Since the scenario is a corner case in real network deployment, it’s better not to define the requirement and leave it to UE implementation.

Proposal 1: When UL transmission is configured with spatial relation info associated with DL RS and the TCI state of the DL RS is unknown, don’t define requirement.

## Timing tracking for UL when associated with DL-RS

In last meeting, whether DL timing tracking is needed is still under discussion:

- Whether UE should meet initial transmit timing accuracy requirements after UL spatial relation switch? - Option 1: No - Option 2: Yes - Whether to consider timing tracking when associated DL-RS? - Sub1. Whether to consider timing tracking when associated DL-RS is known but QCLed with a different qcl-Type1 RS? - Option 1: No - Option 2: Yes - Option 3: No requirement will be defined. - Option 4: - No for SRS spatial relation changes - Yes for PUCCH spatial relation changes - Sub2. Whether to consider timing tracking when associated DL-RS is an unknown DL RS? - Option 1: No - Option 2: Yes - Option 3: No requirement will be defined

In 38.133, the condition where timing accuracy will be verified is as follows：

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies: - when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission.

If the UL transmission with UL spatial info switch happened to be first transmission which satisfy the condition defined in the above table, then UL timing accuracy requirement shall be met. If the UL transmission with UL spatial info switch didn’t satisfy the condition defined above, no UL timing accuracy requirement needs to be met. Similar cases lie in handover, Scell activation where UL transmission will take place in new situation, while no extra timing accuracy test case are defined. Therefore, there is no need to design new timing accuracy test case specially for UL spatial relation info switch.

Observation 1: If the UL transmission with UL spatial info switch satisfies the transmission condition defined in timing accuracy requirement, UL timing accuracy requirement need to be satisfied.

Proposal 2:  There is no need for UE to define new initial transmit timing accuracy test case to verify timing error after UL spatial relation switch.

When the DL-RS is in tracking list, UE has already known the timing information and there is no need to spend extra tracking time. If the DL-RS isn’t in tracking list or is not known, timing of the DL-RS will have no impact on the serving cell timing. Since UE uses serving cell timing to transmit on the UL, The UL timing will not change according to the timing of DL-RS as well. If DL-RS is not in the active list, Rx beam sweeping will be needed. The impact of no DL timing tracking may lead to the inaccuracy of Rx beam sweeping. For simplicity, we prefer that don’t define requirement when new QCL-ed DL-RS is not in tracking list or unknown.

Proposal 3: When QCLed DL-RS is in tracking list, don’t need to consider DL tracking time. When QCLed DL-RS is not in tracking list, don’t define delay requirement.

According to the above proposal, delay requirement for MAC CE based spatial relation info switching associated with DL-RS for PUCCH could be defined as

If the spatial relation associated DL-RS is in the active TCI state list:

Tdelay = THARQ +3ms/NR slot length

If the spatial relation associated downlink RS is not in the active TCI state list or unknown, no requirement is defined.

Proposal 4: Delay requirement for MAC CE based spatial relation info switching associated with DL-RS for PUCCH could be defined as:

If the spatial relation associated DL-RS is in the active TCI state list: Tdelay = THARQ +3ms/NR slot length

If the spatial relation associated DL-RS is not in the active TCI state list or unknown, no requirement is defined.

For RRC based spatial relation info switching associated with DL-RS for P-SRS, the delay requirement is similar.

Proposal 5: Delay requirement for RRC spatial relation info switching associated with DL-RS for P-SRS could be defined as:

If the spatial relation associated DL-RS is in the active TCI state list: Tdelay = TRRCprocessing

If the spatial relation associated DL-RS is not in the active TCI state list or unknown, no requirement is defined.

## Delay requirement for BC Bit-0

In last meeting, the issue about the requirement for BC Bit-0 is raised:

- How to deduce the delay requirement for UE which only supports BC Bit-0 when spatial relation is associated with DL RS? - Option 1: The same delay requirement as BC Bit-1 UE - Option 2: Always plus the SRS beam sweeping duration compared to BC Bit-1 UE

In 38.101-2, the detail description about bit-0 and bit-1 UE are as follows:

- If [bit-1], the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.3-1 and spherical coverage requirement according to Table 6.2.1.3-3 with its autonomously chosen UL beams and without uplink beam sweeping. Such a UE is considered to have met the beam correspondence tolerance requirement. - If [bit-0], the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.3-1 and spherical coverage requirement according to Table 6.2.1.3-3 with uplink beam sweeping. Such a UE shall meet the beam correspondence tolerance requirement defined in Clause 6.6.4.2 and shall support uplink beam management, as defined in TS 38.306 [14].

Bit-1 UE can fulfil the beam correspondence requirement without the uplink beam sweeping and the delay requirement can be applied for Bit-1 UE. Bit-0 UE can meet the requirement only if it finishes the uplink beam sweeping. Bit-0 UE can still transmit the UL signal applying beam QCL-ed with DL RS with some performance loss. With the UL beam sweeping, 3dB improvement cam be expected. However, without UL beam sweeping, it can still meet the minimum requirement. There is no need to define extra delay for BC bit-0 UE which doesn’t support beamCorrespondenceWithoutUL-BeamSweeping.

Proposal 6: For Bit-0 UE, the same delay requirement as Bit-1 UE can be defined.

# Conclusion

In this paper we provide our views on introducing requirements for UL spatial relation info switching. Our observations and proposals are captured below:

Proposal 1: When UL transmission is configured with spatial relation info associated with DL RS and the TCI state of the DL RS is unknown, don’t define requirement.

Observation 1: If the UL transmission with UL spatial info switch satisfies the transmission condition defined in timing accuracy requirement, UL timing accuracy requirement need to be satisfied.

Proposal 2:  There is no need for UE to define new initial transmit timing accuracy test case to verify timing error after UL spatial relation switch.

Proposal 3: When QCLed DL-RS is in tracking list, don’t need to consider DL tracking time. When QCLed DL-RS is not in tracking list, don’t define delay requirement.

Proposal 4: Delay requirement for MAC CE based spatial relation info switching associated with DL-RS for PUCCH could be defined as:

If the spatial relation associated DL-RS is in the active TCI state list: Tdelay = THARQ +3ms/NR slot length

If the spatial relation associated DL-RS is not in the active TCI state list or unknown, no requirement is defined.

Proposal 5: Delay requirement for RRC spatial relation info switching associated with DL-RS for P-SRS could be defined as:

If the spatial relation associated DL-RS is in the active TCI state list: Tdelay = TRRCprocessing

If the spatial relation associated DL-RS is not in the active TCI state list or unknown, no requirement is defined.

Proposal 6: For Bit-0 UE, the same delay requirement as Bit-1 UE can be defined.

# Reference

1. R4-2008679, “WF on spatial relation switch”, Mediatek

# Appendix:

We provide our view regarding test case design for UL spatial info switch just for information.

According to the requirement agreement for UL spatial info switch:

#; Scenario; PHY channel; Triggering method; associated source; Whether to introduce requirement. 1; PUCCH-RRC-DL; PUCCH; RRC; DL RS; No. 2; PUCCH-RRC-UL; PUCCH; RRC; UL SRS; No. 3; PUCCH-MAC-DL; PUCCH; MAC; DL RS; Yes. 4; PUCCH-MAC-UL; PUCCH; MAC; UL SRS; No. 5; PUSCH-DCI; PUSCH; DCI; -; No. 6; pSRS-RRC-DL; P-SRS; RRC; DL RS; Yes. 7; pSRS-RRC-UL; P-SRS; RRC; UL SRS; No. 8; spSRS-MAC-DL; SP-SRS; MAC; DL RS; Yes. 9; spSRS-MAC-UL; SP-SRS; MAC; UL SRS; No. 10; aSRS-DCI-DL; A-SRS; DCI; DL RS; Yes. 11; aSRS-DCI-UL; A-SRS; DCI; UL SRS; No.

The scenarios where test cases may be needed in RAN4 are:

#; Scenario; PHY channel; Triggering method; associated source; Whether to introduce requirement. 1; PUCCH-MAC-DL; PUCCH; MAC; DL RS; Yes. 2; spSRS-MAC-DL; SP-SRS; MAC; DL RS; Yes. 3; pSRS-RRC-DL; P-SRS; RRC; DL RS; Yes. 4; aSRS-DCI-DL; A-SRS; DCI; DL RS; Yes.