3GPP TSG-RAN WG4 Meeting #96-e	R4-2009883

# Online Meeting, 17 – 28 August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 943 | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE Rx-Tx time difference measurement requirements for NR positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel- |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Per work split agreement in RAN4#94-e-Bis, the measurement requirements for UE Rx-Tx time difference in NR positioning is introduced with this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introductin section describing the overview and when this clause becomes applicable - Measurement requirements describing the measurement period formulation and agreements on handling of MG configuration, HO, TA change - Applicability of requirements (including scenarios when requirements are not applicable) - Measurement delay requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR positioning core requirements will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of change >

### 9.9.4	UE Rx-Tx time difference measurements

### 9.9.4.1 Introduction

The requirements in this clause shall apply, provided the UE has received nr-Multi-RTT-RequestLocationInformation message from LMF via LPP [31] requesting the UE to measure one or more UE Rx-Tx time difference measurements defined in TS 38.215 [4].

The requirements in this clause shall apply when UE is RRC_CONNECTED state assuming non-DRX; regardless of whether and which DRX configuration is configured to the UE.

The UE is not required to perform additional SSB measurements for the SSBs configured as QCL sources of DL PRS resources configured by DL-PRS-QCL-Info.

### 9.9.4.2 Measurement Requirements

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [31], UE shall be able to measure one or more UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in positioning frequency layer i according to its signaled capabilities within the measurement period TUERxTx,i as given below (also see Figure 9.9.4.2-1):

TUERxTx,i=NRxBeam.CSSF.Nsample.maxLPRS,iN,NPRS,islotN'.TMGRPe.MGRPe+T

where

- NRxBeam is the UE Rx beam sweeping factor with NRxBeam = 1 in FR1 and NRxBeam = [8] in FR2

- CSSF is the carrier-specific scaling factor CSSFwithin_gap,i as defined in clause 9.1.5.2

- Nsample is the number of PRS occasions needed to meet the accuracy requirements as defined in clause 10.1.25 where PRS occasion of a DL PRS resource is defined as the time duration spanned by its repetitions configured by DL-PRS-NumSymbols and DL-PRS-ResourceRepetitionFactor within one PRS period (TPRS) and Nsample = TBD

- LPRS,i is the time duration spanned by PRS occasions of all DL PRS resources of positioning frequency layer i within one PRS period (TPRS). For the purpose of DL PRS processing capability, LPRS,i is calculated by either Type I duration calculation with UE symbol level buffering capability or Type II duration calculation with UE slot level buffering capability as defined in clause 5.1.6.5 of TS 38.214 [26]

- NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot

- {N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth supported by UE as specified in clause 4.2.7.2 of TS 38.306 [14]

- N’ is UE capability for number of DL PRS resources that it can process in a slot as specified in clause 4.2.7.2 of TS 38.306 [14]

- MGRPe is the effective measurement gap repetition factor used in determining the measurement period and is defined as max(MGRP, MGRPmin) where MGRP is the measurement gap repetition factor configured to UE by RRC signaling and MGRPmin is the minimum MGRP required to align MGL with DL PRS resources as defined in Table 9.9.4.2-1

Table 9.9.4.2-1: Minimum MGRP (MGRPmin)

| max DL-PRS-Periodicity in positioning frequency layer i (TPRS) (ms) | MGRPmin (ms) |
| --- | --- |
| 10, 20, ≤ 5 | 20 |
| 8, 40 | 40 |
| 16, 80 | 80 |
| 32, 160 | 160 |
| 64 | 320 |
| ≥ 320 | TPRS |

Figure 9.9.4.2-1: Illustration of UE Rx-Tx time difference measurement period

Note: Figure 9.9.4.2-1 is for illustrative purpose only and is not intended to limit DL PRS configurations.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities, the maximum PRS periodicity among DL PRS resource sets is used to derive the measurement period of that positioning frequency layer. If more than one positioning frequency layer is configured to UE for measurement, total measurement period is defined as

TUERxTx,Total=i=1LTUERxTx,i

where L is total number of positioning frequency layers.

The time TUERxTx,i starts from the first MG instance aligned with DL PRS resources of positioning frequency layer i closest in time after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [31] are delivered to the physical layer of UE.

If during the measurement period of one or more positioning frequency layers, measurement gap is reconfigured, the measurement period will be longer to account for RRC reconfiguration of the measurement gap.

The UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete.

The UE Rx-Tx time difference measurement accuracy for all measured DL PRS resources shall be fulfilled according to the accuracy requirements specified in clause 10.1.25.

### 9.9.4.3 Requirements Applicability

The requirements in clause 9.9.4.2 apply provided that DL PRS resources being measured meet the side conditions in clause 10.1.25. UE assumes that all DL PRS resources to be measured are available for transmission from the corresponding TRPs during the measurement period. UE assumes that all UL SRS resources for transmission are not delayed or dropped due to no UL resources during the measurement period.

The requirements in clause 9.9.4.2 do not apply if UE is not configured with a measurement gap pattern that aligns in time with DL PRS resources.

The requirements in clause 9.9.4.2 do not apply if UE receives a TA change command during the UE Rx-Tx time difference measurement period.

If the time duration of a PRS occasion of a DL PRS resource, including its corresponding DL-PRS-expectedRSTD-uncertainty, exceeds the UE capability N as specified in clause 4.2.7.2 of TS 38.306 [14], the measurement requirements do not apply to that DL PRS resource.

### 9.9.4.4 Measurement Reporting Delay

This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

# < End of change >