3GPP TSG-RAN WG4 Meeting #96-e	R4-2009899

# Electronic Meeting, 17th – 28th Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0951 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 for inter-frequency measurement requirement without gap (Section 9.1.5, 9.3.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh-Core |  |  |  |  |  | Date: |  |  | 2020-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UE only has to conduct the inter-frequency measurements without gap if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is also configured by Network |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add “if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network” in carrier-specific scaling factor CSSFoutside_gap,i and CSSFwithin_gap,i - Add capabiliyt interFrequencyMeas-NoGap-r16 and flag interFrequencyConfig-NoGap-r16 in the introduction of inter-frequency SSB based measurements without measurement gaps |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-frequency measurements without gap is an optional feature and UE only has to perform it when network configure flag; however, current spec looks like it is an mandatory feature and UE always has to perform it. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5, 9.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------- Beginning of Change 1------------------------------------------------------------

### 9.1.5	Carrier-specific scaling factor

This clause specifies the derivation of carrier-specific scaling factor (CSSF) values, which scales the measurement delay requirements given in clause 9.2, 9.3 and 9.4 when UE is configured to monitor multiple measurement objects. The CSSF values are categorized into CSSFoutside_gap,i and CSSFwithin_gap,i, for the measurements conducted outside measurement gaps and within measurement gaps, respectively.

#### 9.1.5.1	Monitoring of multiple layers outside gaps

The carrier-specific scaling factor CSSFoutside_gap,i for measurement object i derived in this chapter is applied to following measurement types:

-	Intra-frequency measurement with no measurement gap in clause 9.2.5, when none of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	Intra-frequency measurement with no measurement gap in clause 9.2.5, when part of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	Inter-frequency measurement with no measurement gap in clause 9.3.5, when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network.

-	Inter-frequency measurement with no measurement gap in clause 9.3.5, when part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network.

UE is expected to conduct the measurement of this measurement object i only outside the measurement gaps.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, CSSFoutside_gap,i and requirements derived from CSSFoutside_gap,i are not specified.

The UE cell identification and measurement periods derived based on CSSFoutside_gap,i in clauses 9.2.5.1, 9.2.5.2 may be extended for measurement objects of which the cell identification and measurement periods are overlapped with Tmeasure_SFTD1 specified in clause 9.3.8 when no measurement gaps are provided.

The requirements in this clause apply provided that

- 	There are only SCCs in FR2, or

- 	The SMTC on all CCs in FR2 have the same offset, and one of following conditions is met

-	If smtc2 is configured on any FR2 CC,

-	All CCs have the same configuration for smtc1, and

-	All CCs configured with smtc2 have the same configuration for smtc2

-	If smtc2 is not configured on any FR2 CC,

-	The total number of different SMTC periodicities on all serving CCs does not exceed 4

Note:	Longer delays for cell identification and measurement periods derived based on CSSFoutside_gap,i in clauses 9.2.5.1, 9.2.5.2, can be expected, if the UE is configured with more than 4 different SMTC periodicities on FR2 serving carriers. The longer delay applies for the FR2 intra-frequency measurement objects with the longest SMTC periodicity/periodicities.

----------------------------------------------------------- End of Change 1----------------------------------------------------------------

------------------------------------------------------- Beginning of Change 2------------------------------------------------------------

#### 9.1.5.2	Monitoring of multiple layers within gaps

The carrier-specific scaling factor CSSFwithin_gap,i for a measurement object i derived in this chapter is applied to following measurement types:

-	Intra-frequency measurement object with no measurement gap in clause 9.2.5, when all of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	Intra-frequency measurement object with measurement gap in clause 9.2.6.

-	Inter-frequency measurement with no measurement gap in clause 9.3, when all of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network.

-	Inter-frequency measurement object with measurement gap in clause 9.3-	E-UTRA Inter-RAT measurement object in clauses 9.4.2 and 9.4.3.

-	E-UTRA Inter-RAT RSTD and E-CID measurements in clauses 9.4.4 and 9.4.5.

-	NR Inter-RAT measurement object configured by the E-UTRAN PCell (TS 36.133 [15] clause 8.17.4).

-	E-UTRAN Inter-frequency measurement object configured by the E-UTRAN PCell (TS 36.133 [15] clause 8.17.3) and by the E-UTRAN PSCell (TS 36.133 [15] clause 8.19.3).

-	E-UTRAN Inter-frequency RSTD measurement configured by the E-UTRAN PCell (TS 36.133 [15] clause 8.17.15).

-	UTRA Inter-RAT measurement object configured by the E-UTRAN PCell (TS 36.133 [15] clauses 8.17.5 to 8.17.12).

-	GSM Inter-RAT measurements configured by the E-UTRAN PCell (TS 36.133 [15] clauses 8.17.13 and 8.17.14).

UE is expected to conduct the measurement of this measurement object i only within the measurement gaps.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, CSSFwithin_gap,i and requirements derived from CSSFoutside_gap,i are not specified.

----------------------------------------------------------- End of Change 2----------------------------------------------------------------

------------------------------------------------------- Beginning of Change 3------------------------------------------------------------

### 9.3.1	Introduction

A measurement is defined as an SSB based inter-frequency measurement provided it is not defined as an intra-frequency measurement according to clause 9.2.

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified inter-frequency cells if carrier frequency information is provided by PCell or PSCell, even if no explicit neighbour list with physical layer cell identities is provided.

The UE can perform inter-frequency SSB based measurements without measurement gaps if

-	UE supports interFrequencyMeas-NoGap-r16, and

-    inter-frequency measurement without MG flag interFrequencyConfig-NoGap-r16 is indicated, and

-	the SSB is completely contained in the active BWP of the UE.

For inter-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in section 9.3.5.3.

SSB based measurements are configured along with a measurement timing configuration (SMTC) per carrier, which provides periodicity, duration and offset information on a window of up to 5ms where the measurements on the configured inter-frequency carrier are to be performed. For inter-frequency connected mode measurements, one measurement window periodicity may be configured per inter-frequency measurement object.

When measurement gaps are needed, the UE is not expected to detect SSB on an inter-frequency measurement object which start earlier than the gap starting time + switching time, nor detect SSB which ends later than the gap end – switching time. When the inter-frequency cells are in FR2 and the per-FR gap is configured to the UE in EN-DC, SA NR, NE-DC and NR-DC, or the serving cells are in FR2, the inter-frequency cells are in FR2 and the per-UE gap is configured to the UE in SA NR and NR-DC, the switching time is 0.25ms. Otherwise the switching time is 0.5ms.

The requirements in this section shall also apply, when the UE is configured to perform SRS carrier based switching and using measurement gaps.

------------------------------------------------------------ End of Change ----------------------------------------------------------------