3GPP RAN4#96-e	R4-2009990

Online Meeting, August 17th – August 28th, 2020

Title: 	Discussion regarding RLM requirements of IAB-MTs

Source: 	Qualcomm Incorporated

Agenda item:	7.4.3.1

Document for:	Discussion

# Introduction

During the last meeting [1], RAN4 made the following agreement regarding radio link monitoring evaluation period in IAB networks:

Agreement ## WF on relaxation factor K1 and K2 in FR1 and FR2 SSB based and CSI-RS based RLM evaluation period for IAB-MTs - K1 = 5 - Down-select K2 from following options: - K2 = 2 - Supported by: Samsung, Qualcomm - K2 = 5 - Supported by: ZTE, Huawei

We discuss this issue in this contribution.

# Discussion

## Relaxation Factor for RLM Evaluation Period

RAN4 has already agreed to a relaxation factor for RLM-RS evaluation period. Increasing evaluation period of RLM-LS will severely degrade latency in a backhaul network. In a backhaul network, if one backhaul node’s link quality becomes very poor, it will take a lot of time to detect the failure. All UEs that are in the downstream of the backhaul node suffer.

Observation 1: In an IAB network, if one backhaul node’s link quality becomes very poor, all UEs that are in the downstream of the backhaul node suffer.

Besides, assuming sharing factor to be 1 and 20 ms SSB periodicity, the FR2 RLM evaluation period in no-DRX mode becomes equal to 800 ms. Extending this RLM evaluation period by a factor of 5 will turn the RLM evaluation period to be 4 second. This will impact the communication of downstream UEs during radio link failure.

Observation 2: Assuming sharing factor to be 1 and SSB periodicity to be 20 ms, a relaxation factor of 5 will make FR2 RLM evaluation period for IAB-MTs to be 4 second.

- This will impact the communication of downstream UEs during radio link failure.

Hence, for FR2, at least, the relaxation factor of RLM evaluation period of IAB-MTs should not be greater than 2.

Proposal 1: For FR2, the relaxation factor for both SSB and CSI-RS based RLM evaluation period of IAB-MTs should be 2.

# Conclusion

Observation 1: In an IAB network, if one backhaul node’s link quality becomes very poor, all UEs that are in the downstream of the backhaul node suffer.

Observation 2: Assuming sharing factor to be 1 and SSB periodicity to be 20 ms, a relaxation factor of 5 will make FR2 RLM evaluation period for IAB-MTs to be 4 second.

- This will impact the communication of downstream UEs during radio link failure.

Proposal 1: For FR2, the relaxation factor for both SSB and CSI-RS based RLM evaluation period of IAB-MTs should be 2.

# References

- R4-2008595, “WF for [95e][211] NR_IAB_RRM”, 3GPP TSG RAN WG4 #95e.