3GPP TSG-RAN WG4 Meeting # 96-e	R4-2010012

# Electronic Meeting, 17 - 28 August, 2020

Source:	LG Electronics

Title:	Discussion on test cases for NR V2X performance

Agenda item:	7.3.7.3

Document for:	Discussion

Introduction

In this contribution, we provide our views on the demodulation test cases to define performance requirements.

Discussion

Work scope

Based on work plan [1], RAN4 needs to discuss work scope which V2X performance tests should be defined. Three performance parts could be categorized as follows:

- Basic channel demodulation performance tests

- Functional demodulation performance tests

- CSI performance tests

Detail test cases for each category could be listed as following Table.

Table 1 NR V2X performance test case list

| Category of Performance tests | Detail test cases |
| --- | --- |
| Basic demodulation performance | - PSSCH demodulation - PSCCH demodulation - PSBCH demodulation - PSFCH demodulation - 2nd stage SCI demodulation |
| Functional demodulation performance | - Power imbalance performance with two links - Soft buffer test - PSCCH/PSSCH decoding capability test |
| CSI performance | - CQI reporting - RI reporting |

Basic demodulation performance

- PSSCH demodulation performance

  - NR V2X supports 64QAM modulation as mandatory and 256QAM modulation as optional. Therefore, PSSCH modulation performance requirements with 64QAM should be considered.

  - In LTE V2X demodulation performance, 500km/h relative velocity has been considered based on LTE V2X WID, so the performance requirements has been defined using 2700Hz Doppler frequency. For NR V2X, RAN4 needs to discuss whether or not to verify demodulation performance to support high speed as 500km/h relative velocity.

  - The synchronization sources for NR V2X could be GNSS, gNB/eNB, and SyncRef UE. To verify demodulation performance according to synchronization sources, GNSS and gNB synchronization based performance requirements should be defined.

- PSCCH demodulation performance

  - The number of PSCCH symbols can be configured per resource pool among 2 and 3 symbols, and the number of PSCCH PRB can be configured among {10, 12, 15, 20, 25} PRBs. To verify PSCCH performance, one PSCCH configuration for the number of symbol and PRB is enough, e.g., 3 symbols and 10 PRBs for 10MHz bandwidth.

- PSBCH demodulation performance

  - PSBCH demodulation performance should be defined considering S-SSB slot with normal CP.

- PSFCH demodulation performance

  - There are two SL-HARQ-ACK feedback options; NACK only feedback and ACK/NACK feedback. RAN4 needs to discuss whether the performance requirements for both feedback options are defined or not.

- 2nd stage SCI demodulation performance

  - Two-stage SCI is supported for NR V2X. Unlike 1st stage SCI in PSCCH, 2nd stage SCI is decoded by using PSSCH DMRS, and polar coding used for PDCCH is applied, so new performance requirement for 2nd stage SCI should be defined.

Functional demodulation performance

- Power imbalance performance with two links

  - To verify demodulation performance when receiving PSSCH from two sidelink UEs with power imbalance in one subframe like LTE V2X, power imbalance performance requirement should be taken into account even in NR V2X.

- Soft buffer test

  - RAN1 does not yet define the maximum number of bits supported by NR V2X. RAN4 can consider two approaches;

    - Alt1. RAN4 can define the performance requirement for this after RAN1 defines the maximum number of bits.

    - Alt2. RAN4 can define the performance requirement using 64QAM with certain coding rate in 40MHz channel bandwidth.

- PSCCH/PSSCH decoding capability test

  - By RAN1 agreements, UE can receive X PSCCH in a slot, and UE can attempt to decode Y= NRB non-overlapping RBs per slot. NRB is the number of RBs defined per channel bandwidth, and X is {floor(NRB/10), 2*floor(NRB/10)}RBs. Depending on capability, UE can support floor(NRB/10) and/or 2*floor(NRB/10) for PSCCH reception in a slot. Therefore, the performance requirements for two maximum number of sidelink processes for X values should be defined.

CSI performance

In NR V2X, CSI reporting for CSI-RS is introduced. V2X UE should calculate CSI parameters such as CQI and RI. To verify the CSI performance, aperiodic wideband CQI and RI performance requirements should be defined.

- Proposal 1: Introduce performance requirements listed in Table 1 for NR V2X.

Work split for performance tests

Based on above three categories of V2X performance, we propose work split of V2X performance part for simulation results collection and draft CR.

| Category of performance part | Detail test cases | Volunteer company | note |
| --- | --- | --- | --- |
| Basic demodulation performance | - PSSCH demodulation - PSCCH demodulation - PSBCH demodulation - PSFCH demodulation - 2nd stage SCI demodulation |  | Volunteer company responsibility : - Collect simulation results - Submit draft CR |
| Functional demodulation performance | - Power imbalance performance with two links - Soft buffer test - PSCCH/PSSCH decoding capability test |  |  |
| CSI reporting performance | - CQI reporting - RI reporting |  |  |

- Proposal 2: Companies are encourage to apply volunteer for work split.

Initial simulation assumption for basic performance tests

In this section, we provide initial simulation assumption for basic performance tests in Table1. RAN4 can discuss simulation assumptions for NR V2X demodulation and CSI performance test with Table 2~6 as starting point.

- Proposal 3: discuss and finalize initial simulation assumptions with test cases in Table 1 to provide initial simulation results for next RAN4 meeting

Table 2 Simulation assumption for PSSCH demodulation performance

| Parameters | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test |  | SCH_Test1 | SCH_Test2 | SCH_Test3 | SCH_Test4 |
| Synchronization source |  | GNSS | GNSS | GNSS | gNB |
| Propagation condition |  | TDLC300-1400 | TDLC300-1400 | TDLC300-180 | TDLC300-180 |
| Channel bandwidth | MHz | 10 | 20 | 40 | 10 |
| Allocated resource blocks |  | 24 | 50 | 105 | 50 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 15 |
| CP-OFDM symbols per slot (Note 1) |  | 12 | 12 | 12 | 12 |
| The number of DMRS symbols |  | 4 | 4 | 3 | 2 |
| Modulation order |  | QPSK | 16QAM | 64QAM | QPSK |
| MCS index |  | 4 | 11 | 17 | 4 |
| The number of REs for 2nd stage SCI |  | 240 | 198 | 174 | 240 |
| Transport Block Size | Bits | 1416 | 8064 | 32776 | 3624 |
| Transport block CRC | bits | 24 | 24 | 24 | 24 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 |
| Binary Channel Bits (Note 2) | Bits | 4680 | 22008 | 76536 | 12000 |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission without first symbol for AGC, and there is no PSFCH resource. Note 2: 10 RBs and 3 symbols are allocated for PSCCH, and 2nd stage SCI is allocated |  |  |  |  |  |

Table 3 Simulation assumption for PSCCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | CCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | TDLC300-1400 |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 10 |
| Subcarrier spacing | kHz | 30 |
| The number of PSCCH symbols |  | 3 |
| Modulation order |  | QPSK |
| Transport Block Size (without CRC) | Bits | 40 |
| Transport block CRC | bits | 24 |
| Binary Channel Bits | Bits | 540 |

Table 4 Simulation assumption for PSBCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | BCH_Test1 |
| Synchronization source |  | S-SSB |
| Propagation condition |  | TDLC300-1400 |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 11 |
| Subcarrier spacing | kHz | 30 |
| The number of symbols (Note 1) |  | 8 |
| Modulation order |  | QPSK |
| Transport Block Size (without CRC) | Bits | 32 |
| Transport block CRC | bits | 24 |
| Binary Channel Bits | Bits | 1782 |
| Note 1: The first symbol is excluded. |  |  |

Table 5 Simulation assumption for PSFCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | FCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | TDLC300-180 |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 1 |
| Subcarrier spacing | kHz | 30 |
| The number of PSFCH symbols |  | 2 |
| PSFCH period | Slot | 1 |
| Cyclic shift pair index |  | 6 |
| Minimum time gap for PSFCH | Slot | 2 |
| Note 1: ACK/NACK transmission for single UE is considered |  |  |

Table 6 Simulation assumption for 2nd stage SCI demodulation performance

| Parameters |  | Unit | Value |
| --- | --- | --- | --- |
| Test |  |  | SCH_Test1 |
| Synchronization source |  |  | GNSS |
| Propagation condition |  |  | TDLC300-180 |
| Channel bandwidth |  | MHz | 10 |
| PSSCH | Allocated resource blocks |  | 24 |
|  | Subcarrier spacing | kHz | 30 |
|  | CP-OFDM symbols per slot (Note 1) |  | 12 |
|  | The number of DMRS symbols |  | 4 |
|  | Modulation order |  | QPSK |
|  | MCS index |  | 4 |
| 2nd stage SCI | The number of REs for 2nd stage SCI |  | 240 |
|  | Modulation order |  | QPSK |
|  | Transport Block Size (without CRC) | Bits | 48 |
|  | Transport block CRC | bits | 24 |
|  | Binary Channel Bits | Bits | 480 |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission without first symbol for AGC, and there is no PSFCH resource. |  |  |  |

Conclusion

In this contribution, we provide our views on the demodulation test cases to define performance requirements and initial simulation assumptions for some test cases. We propose

- Proposal 1: Introduce performance requirements listed in Table 1 for NR V2X.

Table 1 NR V2X performance test case list

| Category of Performance tests | Detail test cases |
| --- | --- |
| Basic demodulation performance | - PSSCH demodulation - PSCCH demodulation - PSBCH demodulation - PSFCH demodulation - 2nd stage SCI demodulation |
| Functional demodulation performance | - Power imbalance performance with two links - Soft buffer test - PSCCH/PSSCH decoding capability test |
| CSI performance | - CQI reporting - RI reporting |

- Proposal 2: Companies are encourage to apply volunteer for work split.

| Category of performance part | Detail test cases | Volunteer company | note |
| --- | --- | --- | --- |
| Basic demodulation performance | - PSSCH demodulation - PSCCH demodulation - PSBCH demodulation - PSFCH demodulation - 2nd stage SCI demodulation |  | Volunteer company responsibility : - Collect simulation results - Submit draft CR |
| Functional demodulation performance | - Power imbalance performance with two links - Soft buffer test - PSCCH/PSSCH decoding capability test |  |  |
| CSI reporting performance | - CQI reporting - RI reporting |  |  |

- Proposal 3: discuss and finalize initial simulation assumptions with test cases in Table 1 to provide initial simulation results for next RAN4 meeting

Reference

[1] RP-191723, “Revised WID on 5G V2X with NR sidelink,” LG Electronics.