3GPP TSG-RAN WG4 Meeting # 96-e	R4-2010013

# Electronic Meeting, 17 - 28 August, 2020

Source:	LG Electronics

Title:	REFSENS requirements and LLS results of REFSENS for NR V2X UE

Agenda item:	7.3.3.2

Document for:	Approval

Introduction

In last meeting, RAN4 had agreements for reference sensitivity for NR V2X under following condition [1]:

- Additional ΔRREFSENS shall be added to the reference sensitivity when any target SNR/RB number/diversity gain/Noise figure values are changed from those used to drive the REFSENS in the above table.

In this contribution, we provide simulation results for target SNR and our views on other parameters for V2X REFSENS.

Discussion

Parameters to derive REFSENS for NR V2X

In TS38.101-1, REFSENS for NR V2X has been defined in last RAN4 meeting under following assumptions:

- NF: Noise figure. 13 dB for NR V2X (n47) and 9dB NF for the licensed bands (n38)

- IM: 2.5 dB is assumed

- Target SNR: -1 dB

- Diversity gain: 3dB

RAN4 agreed that REFSENS value should be updated when any target SNR/RB number/diversity gain/Noise figure values are changed from those used to drive the REFSENS.

RB number (LCRB)

In RAN1#101-e, RAN1 has agreement for the number of PRB configured for a resource pool.

Agreements made in RAN1#101-E: - Regarding the number of PRBs configured for a resource pool, all PRBs: UE is not expect to use the remaining PRBs (i.e., not large enough for a full subchannel) in Rel-16 - Introduce a single subchannel size {12} PRBs.

According to RAN1 agreements, a new subchannel size {12} PRBs was introduced. So, LCRB to calculate REFSENS should be updated.

Table 1 LCRB for REFSENS

| NR operating Band / SCS/ Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | SCS (kHz) | 10 MHz (dBm) | 20 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| n47 | 15 | 50 | 105 | 160 | 216 | TDD |
|  | 30 | 24 | 50 | 75 | 105 |  |
|  | 60 | 10 | 24 | 36 | 50 |  |

- Proposal 1: LCRB for REFSENS should be updated using Table 1.

Target SNR

RAN4 had consensus that the target SNR for NR V2X reuse that for NR Uu to derive REFSENS value without simulation works. To confirm current target SNR level for REFSENS, simulation results Fig1~3 are provided based on simulation assumptions in Table 2. The SNR at 5% BLER for three cases is around -1.6 ~ -1.5dB. Therefore, -1 dB target SNR for REFSENS could be used.

- Proposal 2: -1dB target SNR for REFSENS should be confirmed.

Table 2 Simulation assumptions for REFSENS

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 |
| Allocated resource blocks |  | 24 | 50 | 105 |
| Subcarriers per resource block |  | 12 | 12 | 12 |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |
| MCS Index (Note1) |  | 4 | 4 | 4 |
| Number of DMRS per slot |  | 2 | 2 | 2 |
| Channel condition |  | AWGN | AWGN | AWGN |
| CP-OFDM symbols per slot (Note3) |  | 12 | 12 | 12 |
| Transport Block Size | bits | 1544 | 3624 | 7936 |
| Transport block CRC | Bits | 24 | 24 | 24 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |
| Payload size |  | 1544 | 3624 | 7936 |
| Binary Channel Bits per subframe | Bits | 5136 | 12000 | 26520 |
| Total modulated symbols per slot (Note2) |  | 2568 | 6000 | 13260 |
| Note 1: MCS Index is based on MCS table 5.1.3.1-1 defined in 38.214 Note 2: The following resources are excluded for PSSCH transmission: 10RBs and 3 symbols allocated to PSCCH; 240 REs allocated to second stage SCI; 2 symbols allocated to PSSCH DMRS. Note 3: PSSCH allocation consists of 12 OFDM symbols, i.e., there is no PSFCH resource |  |  |  |  |

Figure 1 BLER performance for 10MHz CBW

Figure 2 BLER performance for 20MHz CBW

Figure 3 BLER performance for 40MHz CBW

Diversity gain

Similar to target SNR, 3dB diversity gain for NR V2X reused that for NR Uu to derive REFSENS for NR V2X. For the background of 3dB diversity gain for NR Uu, RAN4 considered that some issues for 3dB diversity gain might be embedded in IM taking into account the frequency range, note that for test the 3dB apply as it is a conducted test. So, we prefer to keep the same approach for NR V2X diversity gain even though around 2.6~2.7dB diversity gain is observed based on simulation results in Fig1~3.

- Proposal 3: Use 3dB diversity gain for REFSENS similar to NR Uu approach for diversity gain

Update REFSENS for NR V2X

Based on Proposal 1~3, REFSENS level for NR V2X for n38 and n47 should be updated as shown in Table 3.

Table 3 Proposed REFSENS for NR V2X (PC5)

| NR Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V2X Band | SCS kHz | 10MHz (dBm) | 20MHz (dBm) | 30MHz (dBm) | 40MHz (dBm) | Duplex Mode |
| n38 | 15 | -97.0 | -93.7 | -91.9 | -90.6 | TDD |
|  | 30 | -97.1 | -93.9 | -92.2 | -90.7 |  |
|  | 60 | -97.9 | -94.1 | -92.4 | -90.9 |  |
| n47 | 15 | -93.0 | -89.7 | -87.9 | -86.6 | HD |
|  | 30 | -93.1 | -89.9 | -88.2 | -86.7 |  |
|  | 60 | -93.9 | -90.1 | -88.4 | -86.9 |  |

The REFSENS level for V2X con-current operation which are V2X_n71-n47 and EN-V2X_20_n38 can be reused by that for license band, so the REFSENS levels for V2X_n71-n47 and EN-V2X_20_n38 could be defined as shown in Table 4 and Table 5, respectively.

Table 4 Proposed RESENS for NR V2X con-current (V2X_n71-n47)

| Inter-band V2X reception |  | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | NR band | NR Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| n47 | n71 | n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 |  |  | FDD |
|  |  |  | 30 |  | -94.3 | -91.9 | -87.4 |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |
|  |  | n47 | 15 |  | -93.0 |  | -89.7 | -87.9 | -86.6 | HD |
|  |  |  | 30 |  | -93.1 |  | -89.9 | -88.2 | -86.7 |  |
|  |  |  | 60 |  | -93.9 |  | -90.1 | -88.4 | -86.9 |  |

Table 5 Proposed RESENS for NR V2X con-current (EN-V2X_20-n38)

| Inter-band EN-V2X reception |  |  | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | E-UTRA or V2X band | Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| n38 | Band 20 | Band 20 | 15 | -97 | -94 | -91.2 | -90 |  |  | FDD |
|  |  | n38 | 15 |  | -97.0 |  | -93.7 | -91.9 | -90.6 | HD |
|  |  |  | 30 |  | -97.1 |  | -93.9 | -92.2 | -90.7 |  |
|  |  |  | 60 |  | -97.9 |  | -94.1 | -92.4 | -90.9 |  |

- Proposal 4: Update the REFSENS levels for NR V2X single and con-current bands as shown in Table 3~5.

Conclusion

In this contribution, we provide simulation results for target SNR and our views on other parameters for V2X REFSENS. Based on our views, we propose and update REFSENS levels

- Proposal 1: LCRB for REFSENS should be updated using Table 1.

- Proposal 2: -1dB target SNR for REFSENS should be confirmed.

- Proposal 3: Use 3dB diversity gain for REFSENS similar to NR Uu approach for diversity gain

- Proposal 4: Update the REFSENS levels for NR V2X single and con-current bands as shown in Table 3~5.

Reference

[1] R4-2009168, “WF on remaining issues for TS38.101-1 and TS38.101-3 for NR V2X,” LG Electronics, Huawei, HiSilicon, CATT

- Appendix

------------------- Start of Text Proposal for TR38.886 --------------------

------------------      Start of Change 1    --------------------

# 9	Receiver characteristics

## 9.1	NR V2X UE Rx requirements in FR1

When a NR V2X UE is TDM operating between NR SL and LTE SL at n47, the NR V2X UE need to satisfy the individual V2X Rx requirements of each RAT.

### 9.1.1	Reference sensitivity power level

The reference sensitivity power level REFSENS is the minimum mean power applied to the UE antenna connector at which the throughput shall meet or exceed 95% of the maximum throughput of the reference measurement channels.

The V2X UE REFSENS is defined by the following equation:

REFSENSV2X=kTB + SNRV2X +10log10(LCRB*SCS*12/RX_BW) +( NFV2X+ IM) – Diversity gain

Where

-	kTB: Thermal noise level is [-174dBm(kT) + 10*log10(RX BW)]dBm.

-	NF: Noise figure. 13 dB is used for LAA and can be reused for NR V2X requirements. 9~10dB NF will be considered for the licensed bands.

-	IM: 2.5 dB is assumed.

-	Target SNR: -1 dB

-	Diversity gain: 3dB

The REFSENS requirements for NR V2X Bands (PC5) are in Table 9.1.1-1.

Table 9.1.1-1: Reference sensitivity for V2X Band (PC5)

| NR Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V2X Band | SCS kHz | 10MHz (dBm) | 20MHz (dBm) | 30MHz (dBm) | 40MHz (dBm) | Duplex Mode |
| n38 | 15 | -97.0 | -93.7 | -91.9 | -90.6 | HD |
|  | 30 | -97.1 | -93.9 | -92.2 | -90.7 |  |
|  | 60 | -97.9 | -94.1 | -92.4 | -90.9 |  |
| n47 | 15 | -93.0 | -89.7 | -87.9 | -86.6 | HD |
|  | 30 | -93.1 | -89.9 | -88.2 | -86.7 |  |
|  | 60 | -93.9 | -90.1 | -88.4 | -86.9 |  |

Table 9.1.1-2: Sidelink TX configuration for reference sensitivity of V2X Bands (PC5)

| NR operating Band / SCS/ Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V2X Band | SCS (kHz) | 10 MHz (dBm) | 20 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| n38 | 15 | 50 | 105 | 160 | 216 | HD |
|  | 30 | 24 | 50 | 75 | 105 |  |
|  | 60 | 10 | 24 | 36 | 50 |  |
| n47 | 15 | 50 | 105 | 160 | 216 | HD |
|  | 30 | 24 | 50 | 75 | 105 |  |
|  | 60 | 10 | 24 | 36 | 50 |  |

------------------      End of Change 1    --------------------

------------------      Start of Change 2    --------------------

# 10	Tx/Rx characteristics of concurrent operation

## 10.1	NR V2X SL operation in 5.9GHz or other potential ITS spectrum and LTE/NR Uu operation in licensed band for FR1

------------------      Omit unchanged parts     --------------------

    - Rx requirements for inter-band con-current NR V2X operation

      - REFSENS

For the NR V2X UE RF receiver requirements, RAN4 can refer the 2DL inter-band CA to define general UE RF Rx requirements for inter-band con-current NR V2X UE.

The legacy REFSENS requirement will be applied on CC of NR licensed band and NR V2X REFSENS requirements will be applied on CC of NR Band n47 if there was no self-interference problems in own receiver frequency band by own uplink and sidelink transmission.

Table 10.1.2.1-1 and Table 10.1.2.1-2 propose the uplink test configurations for inter-band con-current NR V2X REFSENS requirements. For the uplink configuration, RAN4 consider 10MHz Channel bandwidth.

Table 10.1.2.1-1: Uplink configuration for reference sensitivity of V2X UE (PC5)

| Inter-band NR V2X con-current band configuration |  | NR UL band / SCS/ Channel BW / NRB / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X band (PC5) | NR V2X band (Uu) | NR V2X UL band (Uu) | SCS (kHz) | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| n47 | n71 | n71 | 15 | 10 | 50 | FDD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 10 |  |

Table 10.1.2.1-2: SL Tx configuration for reference sensitivity of V2X UE (Uu)

| Inter-band NR V2X con-current band configuration |  | NR UL band / SCS/ Channel BW / NRB / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X band (PC5) | NR V2X band (Uu) | NR V2X band (PC5) | SCS (kHz) | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| n47 | n71 | n47 | 15 | 10 | 50 | HD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 10 |  |

Table 10.1.2.1-3 is proposed UE REFSENS requirements with inter-band con-current NR V2X UE reception.

Table 10.1.2.1-3: Reference sensitivity for V2X QPSK PREFSENS

| Inter-band V2X reception |  |  | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | NR band | NR Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| n47 | n71 | n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 |  |  |  | FDD |
|  |  |  | 30 |  | -94.3 | -91.9 | -87.4 |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |
|  |  | n47 | 15 |  | -93.0 |  | -89.7 |  | -87.9 | -86.6 | HD |
|  |  |  | 30 |  | -93.1 |  | -89.9 |  | -88.2 | -86.7 |  |
|  |  |  | 60 |  | -93.9 |  | -90.1 |  | -88.4 | -86.9 |  |

Table 10.1.2.1-4 is proposed additional Rx insertion loss according to harmonic trap filter to reduce the harmonic problem based on specific self desense analysis according to specific NR V2X inter-band con-current operation.

Table 10.1.2.1-4: ΔRIB,c (two bands)

| V2X inter-band con-current band Combination | NR V2X Band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_n71-n47 | n71 | 0.0 |

------------------      End of Change 2    --------------------

------------------      Start of Change 3    --------------------

## 10.2	NR V2X SL operation and LTE/NR Uu operation both in licensed band for FR1

------------------      Omit unchanged parts     --------------------

    - Rx requirements for inter-band con-current NR V2X operation

      - REFSENS

For the V2X UE RF receiver requirements, RAN4 can refer the 2DL inter-band CA to define general UE RF Rx requirements for inter-band con-current V2X UE.

The legacy REFSENS requirement will be applied on each CC of NR licensed bands if there was no self-interference problems in own receiver frequency band by own uplink and sidelink transmission.

Table 10.2.2.1-1 and Table 10.2.2.1-2 propose the uplink test configurations for inter-band con-current V2X REFSENS requirements. For the uplink configuration, RAN4 consider 10MHz Channel bandwidth.

Table 10.2.2.1-1: Uplink configuration for reference sensitivity of V2X UE (PC5)

| Inter-band V2X con-current band configuration |  | LTE or NR UL band / Channel BW / NRB / Duplex mode |  |  |  |
| --- | --- | --- | --- | --- | --- |
| V2X band (PC5) | LTE or NR band (Uu) | LTE or NR UL band | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| n38 | B20 | B20 | 10 | 50 | FDD |

Table 10.2.2.1-2: SL Tx configuration for reference sensitivity of V2X UE (Uu)

| Inter-band V2X con-current band configuration |  | LTE or NR UL band / Channel BW / NRB / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V2X band (PC5) | LTE or NR band (Uu) | NR V2X band (PC5) | Channel Bandwidth (MHz) | SCS (kHz) | NRB | Duplex Mode |
| n38 | B20 | n38 | 10 | 15 | 50 | HD |
|  |  |  |  | 30 | 24 |  |
|  |  |  |  | 60 | 10 |  |

Table 10.2.2.1-3 is proposed the REFSENS requirements with inter-band con-current V2X UE reception.

Table 10.2.2.1-3: Reference sensitivity for V2X QPSK PREFSENS

| Inter-band V2X reception |  | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X Band | LTE or NR V2X band (Uu) | LTE or NR Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| n38 | B20 | B20 | 15 | -97 | -94 | -91.2 | -90 |  |  | FDD |
|  |  | n38 | 15 |  | -97.0 |  | -93.7 | -91.9 | -90.6 | HD |
|  |  |  | 30 |  | -97.1 |  | -93.9 | -92.2 | -90.7 |  |
|  |  |  | 60 |  | -97.9 |  | -94.1 | -92.4 | -90.9 |  |

------------------      End of Change 3    --------------------

------------------ End of Text Proposal for TR38.886 -------------------