3GPP TSG-RAN WG4 Meeting # 95-e 		R4-2010050

Electronic Meeting, August 17-28, 2020

Agenda item:	8

Source: 	Apple.

Title: 	On RAN4 UE features list for Rel-16

Document for:	Discussion

- Introduction

The latest version of RAN4 UE feature list was agreed in R4-2009174. Still quite a few feature groups are subject to further discussion and decision in RAN4#96e. In this contribution, additional comments and proposals on the following feature groups are provided.

- Multi-RAT DC/CA enhancement

- CSI-RS based L3 measurement

- NR HST

- FR1 RF

- FR2 RF

- Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements

# Proposal 1: remove feature group 6-3 since it is identical to 6-2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6. LTE_NR_DC_CA_enh | [6-1] | [Configured output power for NR-DC] | NR-DC within FR1 (FR1-FR1) - Synchronous/Non-synchronous - Semi-static power sharing/Dynamic power sharing - Intra band NR-DC - Inter band NR-DC NR-DC between FR1 and FR2 - Non-synchronous - Independent power control | N/A | Yes | N/A | Impact on gNB Scheduling, UE is not compliant to necessary UE requirements | [Per BC] | No | Yes | Yes |  | [Mandatory for a UE that supports NR-DC within FR1 (FR1-FR1) and/or NR-DC between FR1 and FR2 (non-synchronous)] |
|  | [6-2] | Only supporting single switched UL | Only supporting single switched UL for specific inter-band ENDC combination | N/A | Yes | N/A | UE can’t support the specific inter-band ENDC combination | Per BC | No | FR1 only | N/A | This capability is restricted to specific band combinations. If this capability isn’t supported for the specific inter-band ENDC combination, the UE can support simultaneous UL transmissions when the inter-band ENDC combination is supported. UE can’t simultaneously report this capability and IE “singleUL-Transmission” for the specific inter-band ENDC combination. IE “tdm-Pattern” will be reused to indicate whether the UE supports the tdm-PatternConfig for this capability associated functionality. The field doesn’t apply to any other fallback band combinations. For super set combination, the UL configuration can refer to sub-clause 5.5B.4.1 from 38.101-3. | Optional with capability signalling |
|  | [6-3] | Only supporting single switched UL | Only supporting single switched UL for specific inter-band ENDC combination | N/A | Yes | N/A | UE can’t support the specific inter-band ENDC combination | Per BC | No | FR1 only | N/A | This capability is restricted to specific band combinations. If this capability isn’t supported for the specific inter-band ENDC combination, the UE can support simultaneous UL transmissions when the inter-band ENDC combination is supported. UE can’t simultaneously report this capability and IE “singleUL-Transmission” for the specific inter-band ENDC combination. IE “tdm-Pattern” will be reused to indicate whether the UE supports the tdm-PatternConfig for this capability associated functionality. The field doesn’t apply to any other fallback band combinations. For super set combination, the UL configuration can refer to sub-clause 5.5B.4.1 from 38.101-3. | Optional with capability signalling |
|  | [6-4] | Support of beam level Early Measurement Reporting | Supporting of beam level measurement and reporting for Early Measurement Reporting at connection setup. | TBD | Yes | N/A | Network cannot configure beam-level reporting to UE for EMR | Per UE | No | No | N/A |  | [Optional with capability signalling] |

- NR support for high speed train scenario

# Proposal 2: All NR HST features should be optional as defined in LTE HST

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR HST | 10-1 | RRM enhanced requirements specified within NR and NR-E-UTRAN inter-RAT measurement for NR HST | The enhanced RRM requirements specified within NR and NR-E-UTRAN inter-RAT measurement to support high speed up to 500 km/h, as specified in TS 38.133 |  | No |  | The performance of RRM in NR HST scenario cannot be guaranteed | Per UE | NO | FR1 only |  |  | Optional [mandatory with capability signalling] |
|  | 10-2 | Demodulation enhancement for HST-SFN joint transmission scheme | The enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h, as specified in TS 38.101-4 |  | No |  | The demodulation performance of HST-SFN joint transmission cannot be guaranteed | Per UE | NO | FR1 only |  |  | Optional [mandatory with capability signalling] |
|  | 10-3 | RRM enhancement for E-UTRAN -NR inter-RAT measurement for NR HST | The enhanced RRM requirements specified for E-UTRAN-NR inter-RAT measurement to support high speed up to 500 km/h, as specified in TS 36.133 |  | No |  | The performance of RRM in NR HST scenario cannot be guaranteed | Per UE | NO | FR1 only |  |  | Optional [mandatory with capability signalling] |

- NR RRM requirements for CSI-RS based L3 measurement

# Proposal 3: It is proposed to remove feature group 12-1, replaced by the following two feature groups

- Simultaneous reception of intra-frequency CSI-RS and SSB within the active BWP bandwidth and with the same numerology

- Simultaneous reception of intra-frequency CSI-RS and SSB within the active BWP bandwidth and with mixed numerologies

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12. NR RRM requirements for CSI-RS based L3 measurement | [12-1] | [Simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell] | UE support FDM-ed mix-numerology on simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell | TBA | YES |  | The performance of CSI-RS L3 measurement cannot be guaranteed | Per UE | No | No |  |  | Optional with capability signalling |
|  | [12-2] | [CSI-RS measurement] | - Support CSI-RS measurement based on timing of each of the detected associated SSB | csi-RSRP-AndRSRQ-MeasWithSSB | Yes | n/a | UE can only use a single common timing to measure CSI-RS resources per frequency layer, and can meet the accuracy requirements only under the timing error conditions defined in 38.133. | Per UE | no | no | n/a |  | Optional with capability signalling |
|  | [12-3] | Simultaneous reception of intra-frequency CSI-RS and SSB within the active BWP bandwidth and with the same numerology | Parallel processing capability for intra-frequency SSB and CSI-RS based L3 measurement within active BWP bandwidth and with the same numerology. This also includes the SSB and CSI-RS of serving cell(s) |  | Yes | n/a | UE can only measurement SSB and CSI-RS within active BWP bandwidth and with the same numerology in sequential order | Per UE | no | no |  |  | Optional with capability signalling |
|  | [12-4] | Simultaneous reception of intra-frequency CSI-RS and SSB within the active BWP bandwidth and with mixed numerologies | Parallel processing capability for intra-frequency SSB and CSI-RS based L3 measurement within active BWP bandwidth and with mixed numerologies. This also includes the SSB and CSI-RS of serving cell(s) | [12-3] | Yes | n/a | UE can only measurement SSB and CSI-RS within active BWP bandwidth and with mixed numerologies in sequential order | Per UE | no | no |  |  | Optional with capability signalling |
|  | [12-5] | Simultaneous reception of CSI-RS, SSB and data channel within the active BWP bandwidth and with three mixed numerologies | Parallel processing capability for intra-frequency SSB, CSI-RS based L3 measurement and data within active BWP bandwidth and with 3 mixed numerologies. This also includes the SSB and CSI-RS of serving cell(s) |  | Yes | n/a | UE can only measurement SSB, CSI-RS and data channel within active BWP bandwidth and with mixed numerologies in sequential order | Per UE | no | no |  |  | Optional with capability signalling |

- NR-based access to unlicensed spectrum

The table below illustrates the NR-U part of the feature list from R4-2009174:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4. NR-based access to unlicensed spectrum | [4-1] | Transmission in intra-carrier guardband | Capability of transmission in the intra-band carrier guardband if the UE is scheduled a contiguous allocation that is wider than a 20MHz subband. | None | yes | no | UE cannot transmit in the guardband, it could only transmit in the subbands | per UE | No | No |  |  | Optional |
|  | [4-2] | Reception in intra-carrier guardband | Capability of reception in the intra-band carrier guardband if the UE is scheduled a contiguous allocation that is wider than a 20MHz subband. | None | yes | no | UE cannot receive in the guardband, it could only receive in the subbands | per UE | No | No |  |  | Optional |

The definition of the wide-band capability depends on the outcome of the related discussion in the NR-U work item.  Our input on this topic is summarized in R4-2009933.  The following proposals can be transferred to the feature list discussion for completeness:

Proposal 4: Wide-band transmission modes should have separate UE capabilities

Proposal 5: It can be discussed further whether we need to have strict differentiation between all three modes / sub-modes or whether transmission mode 1 can be construed as the baseline NR-U functionality

Proposal 6: Wide-band transmission modes should be differentiated between DL and UL

- NR RF requirement enhancements for frequency range 2 (FR2)

The table below illustrates the FR2 part of the feature list from R4-2009174:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8. NR RF Requirement Enhancements for FR2 | 8-1 | MPE | 1 P-MPR reporting |  | yes | N/A | UE does not support MPE P-MPR reporting | per UE | TDD only | FR2 only | N/A |  | optional with capability signaling |
|  | 8-2 | Beam correspondence | [1 SSB based BC] [2 CSI-RS based BC] further discussion if common feature or separate features and capability signaling |  | yes | N/A | UE does not support eBC requirements | per UE | TDD only | FR2 only | N/A |  | further discussion but in any case capability signalling needed |
|  | 8-3 | Non-contiguous intra-DL CA | 1 Fsd (Frequency separatation class for DL only spectrum) as indicated in the draft LS in R4-2006632 |  | yes | N/A | UE does not support DL spectrum | per BC | TDD only | FR2 only | N/A |  | optional with capability signaling |
|  | 8-4 | Non-contiguous intra-band UL CA | Rel-15 supported UL CA configuration signalling is sufficient. |  | yes | N/A | Rel-15 signaling | per BC (re-use Rel-15 signaling) | TDD only | FR2 only | N/A |  | optional with capability signalling (re-use Rel.15 signaling) |
|  | 8-5 | Inter-band DL CA | 1 FFS Introduce IBM (Independent Beam Management)/CBM (Common Beam Management) |  | yes | N/A | UE does not support inter-band CA | per BC | TDD only | FR2 only | N/A |  | optional with capability signaling |
|  | 8-6 | MPR Enhancement | 1 zero dB MPR region extension, Rel-15 modifiedMPR signalling can be used |  | yes | N/A | UE does not support zero dB MPR region extension |  | TDD only | FR2 only | N/A |  | optional in Rel-16, re-use Rel-15 signaling |
|  | 8-7 | [MPR Enhancement | UE Tx power boost feature when IBE is suspended |  | yes | N/A | UE does not support Tx power boost feature when IBE is suspended |  | TDD only | FR2 only | N/A |  | optional with capability signaling |

Proposal 7: Allocate 2 bits for P-MPR reporting

Proposal 8: Determine the P-MPR reporting values based on the outcome of the related discussion in the FR2 RF enhancement agenda

Proposal 9: Whether a capability for SSB basecd BC is introduced depends on the outcome of the related discussion in the FR2 RF enhancement agenda

Proposal 10: Whether a capability for CSI-RS basecd BC is introduced depends on the outcome of the related discussion in the FR2 RF enhancement agenda

Proposal 11: Set the component text associated with 8-3 (Non-contiguous intra-DL CA) to the following:

| 8. NR RF Requirement Enhancements for FR2 | 8-3 | Non-contiguous intra-DL CA | Support for frequency separation class for DL-only spectrum (Fsd): DL-only spectrum is available for configuration of only DL CCs and not UL CCs. The spectrum covered by the DL-only frequency separation extends on one-side of the bidirectional spectrum in a contiguous manner with no frequency gap between the two. The bidirectional spectrum is defined as the UL/DL common spectrum in which the UE supports the configuration of uplink or downlink CCs and is signalled by UL and DL frequency separation from Rel-15. The combined downlink spectrum (DL Fs + Fsd) cannot exceed 2400 MHz. The component value range is defined in TS38.101-2 |  | yes | N/A | UE does not support DL spectrum | per BC | TDD only | FR2 only | N/A |  | optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 12: Remove FG 8-4 (Non-contiguous intra-band UL CA), because Rel-15 signaling for UL CA configuration support is sufficient

Proposal 13: Remove FG 8-5, FFS Introduce IBM (Independent Beam Management)/CBM (Common Beam Management); based on the analysis provided in R4-2009963

Proposal 14: Remove FG 8-6 (zero dB MPR region extension), because Rel-15 signaling for modifiedMPR behavior can be used

Proposal 15: Whether FG 8-7 is finalized in the feature list depends on the outcome of the related discussion in the FR2 RF enhancement agenda

- Conclusions

# In this contribution, it is proposed that

# Proposal 1: remove feature group 6-3 since it is identical to 6-2

# Proposal 2: All NR HST feature groups should be optional as defined in LTE HST

# Proposal 3: It is proposed to remove feature group 12-1 in CSI-RS based L3 measurement, replaced by the following two feature groups

- Simultaneous reception of intra-frequency CSI-RS and SSB within the active BWP bandwidth and with the same numerology

- Simultaneous reception of intra-frequency CSI-RS and SSB within the active BWP bandwidth and with mixed numerologies

Proposal 4: Wide-band transmission modes should have separate UE capabilities

Proposal 5: It can be discussed further whether we need to have strict differentiation between all three modes / sub-modes or whether transmission mode 1 can be construed as the baseline NR-U functionality

Proposal 6: Wide-band transmission modes should be differentiated between DL and UL

Proposal 7: Allocate 2 bits for P-MPR reporting

Proposal 8: Determine the P-MPR reporting values based on the outcome of the related discussion in the FR2 RF enhancement agenda

Proposal 9: Whether a capability for SSB basecd BC is introduced depends on the outcome of the related discussion in the FR2 RF enhancement agenda

Proposal 10: Whether a capability for CSI-RS basecd BC is introduced depends on the outcome of the related discussion in the FR2 RF enhancement agenda

Proposal 11: Set the component text associated with 8-3 (Non-contiguous intra-DL CA) to the following:

| 8. NR RF Requirement Enhancements for FR2 | 8-3 | Non-contiguous intra-DL CA | Support for frequency separation class for DL-only spectrum (Fsd): DL-only spectrum is available for configuration of only DL CCs and not UL CCs. The spectrum covered by the DL-only frequency separation extends on one-side of the bidirectional spectrum in a contiguous manner with no frequency gap between the two. The bidirectional spectrum is defined as the UL/DL common spectrum in which the UE supports the configuration of uplink or downlink CCs and is signalled by UL and DL frequency separation from Rel-15. The combined downlink spectrum (DL Fs + Fsd) cannot exceed 2400 MHz. The component value range is defined in TS38.101-2 |  | yes | N/A | UE does not support DL spectrum | per BC | TDD only | FR2 only | N/A |  | optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 12: Remove FG 8-4 (Non-contiguous intra-band UL CA), because Rel-15 signaling for UL CA configuration support is sufficient

Proposal 13: Remove FG 8-5, FFS Introduce IBM (Independent Beam Management)/CBM (Common Beam Management); based on the analysis provided in R4-2009963

Proposal 14: Remove FG 8-6 (zero dB MPR region extension), because Rel-15 signaling for modifiedMPR behavior can be used

Proposal 15: Whether FG 8-7 is finalized in the feature list depends on the outcome of the related discussion in the FR2 RF enhancement agenda