3GPP TSG-RAN4 Meeting #95e	R4-2010053

# Online, August 17-28, 2020

Agenda item:	7.14.1.3

Source:	Apple

Title:   On UE measurement capability of CSI-RS based L3 measurement

Document for:	Approval

# Introduction

In RAN4 #95e-bis meeting, the following UE measurement capability related issues are still to be decided

- FFS: New UE capability on the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell

- Whether or not to introduce new UE capability for minimum separation between two slots with CSI-RS resources

- Whether CSI-RS and SSB for mobility configured in the same MO are counted as [2] layers

- Whether multiple MOs can be counted as one frequency layer

- Number of CSI-RS layers/cells/beams needs to be confirmed

- Scheduling restriction: FFS: UE is not expected to transmit or receive on [1/2] data OFDM symbols impacted by CSI-RS resource symbols to be measured

- CSSF: FFS: CSSF for CSI-RS based measurement within measurement gap and outside of measurement gap.

In this contribution, our proposals on these issues are provided.

# New UE capability on the simultaneous reception of CSI-RS of neighbor cell and SSB of serving cell

Simultaneous reception of CSI-RS and SSB is an important issue to address. However, it is not very clear why only CSI-RS of “neighbor cell” and SSB of “serving cell” are specifically considered. In our opinion, both intra-frequency CSI-RS and the CSI-RS of the serving cell should be considered. Meanwhile, both intra-frequency SSB and the SSB of serving cell should be considered too.

Providing both the CSI-RS resource and SSB are confined with active BWP bandwidth, two UE capabilities should be investigated and specified

- Simultaneous reception of CSI-RS and SSB with the same numerology.

  - This UE capability is to indicate if UE can process both CSI-RS and SSB in parallel

- Simultaneous reception of CSI-RS and SSB with a mixed numerologies

  - It is noted that this UE capability is to indicate if UE can process both CSI-RS and SSB in parallel and, meanwhile, handle the mixed numerologies.

Proposal 1: it is proposed to remove feature group 12-1 and introduce the following UE capacities

- Feature group 12-3: Simultaneous reception of intra-frequency CSI-RS and SSB within the active BWP bandwidth and with the same numerology

- Feature group 12-4: Simultaneous reception of intra-frequency CSI-RS and SSB within the active BWP bandwidth and with mixed numerologies

When a UE supports 12-4, 12-3 should be supported too.

When UE indicates to support the feature group 12-4 and at the same time to support IE simultaneousRxSSBCSI-RS-DiffNumerology, UE may face the scenario to deal with up to three different numerologies, e.g. 15kHz for SSB of serving cell, 30kHz for CSI-RS of neighbour cell and 60kHz for PDSCH/PDCCH. In this case, one more UE capability should be introduced

Proposal 2: Introduce the following UE capability to support up to 3 mixed numerologies

- Feature group 12-5: Simultaneous reception of CSI-RS, SSB and data channel within the active BWP bandwidth and with three mixed numerologies

When a UE supports 12-5, both 12-3, 12-4 and IE simultaneousRxSSBCSI-RS-DiffNumerology should be supported.

When UE supports 12-4 and IE simultaneousRxSSBCSI-RS-DiffNumerology but not 12-5, scheduling restriction is expected when UE measures CSI-RS and SSB.

Alternatively, the scenario with three mixed numerologies can be precluded also. In this case, it is proposed

Proposal 2a: UE behaviour is not specified if 3 mixed numerologies are configured simultaneously.

# Introduce new UE capability for minimum separation between two slots with CSI-RS resources

Considering time domain restriction for CSI-RS resources configuration has been agreed, the motivation to further introduce separation between two slots with CSI-RS resources becomes unclear. It is proposed

Proposal 3: With time restriction introduced for CSI-RS resource configurations, it is not suggested introduce UE capability with minimum separation between two CSI-RS slots

# Whether CSI-RS and SSB for mobility configured in the same MO are counted as 2 layers

Since CSI-RS and SSB processing are completely different but may share the same resources, e.g. searcher and memory, it is essential to consider CSI-RS and SSB as different frequency layer even in the same MO.

Proposal 4: CSI-RS and SSB in the same MO should be considered as 2 layers.

# Whether multiple MOs can be counted as one frequency layer

In general, in order to count multiple MO as one frequency layer, the following two condition should be met

- different MO share the same center frequency and the same SCS

- the total number of CSI-RS resources associated with the same PCI should be no more than maxNrofCSI-RS-ResourcesRRM

Proposal 5: If the following two conditions can be met, CSI-RS resources configured in multiple MO can be counted as one frequency layer

- different MO share the same center frequency and the same SCS for CSI-RS resources

- the total number of CSI-RS resources associated with the same PCI should be no more than maxNrofCSI-RS-ResourcesRRM

# CSSF for CSI-RS based measurement outside of measurement gap

To specify CSSF for CSI-RS based measurement outside of measurement gap, it is necessary to firstly clarify that CSI-RS and SSB should be considered as separated frequency layers even they are configured in the same MO.

Proposal 6: For CSSFwoutside_gap,i, CSI-RS and SSB should be considered as different frequency layers, including the scenarios where they are configured in the same MO, and/or have the same center frequency and the same SCS.

Secondly, it should be also clarified how the searcher limitation is subjected to both CSI-RS and SSB measurement.

Proposal 7: In CSSF definition, it is assumed that CSI-RS and SSB measurement share the same searcher.

Thirdly, it should also clarify CSI-RS time domain restriction and SMTC, which are corresponding to the same PCI, should be aligned

Proposal 8: RAN4 only specifies the requirements based on the assumption that CSI-RS time domain restriction and SMTC, which are corresponding to the same PCI, should be aligned and confined with 5ms window.

As an example, in the spirit of proposal 7 and 8, CSSFoutside_gap,i scaling factor for SA mode can be revised as

| Scenario | CSSFoutside_gap, i for FR1 PCC | CSSFoutside_gap, i for FR1 SCC | CSSFoutside_ga p,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1: if only SSB or CSI-RS L3 configured for PCC 2 if both SSB and CSI-RS L3 configured for PCC | Number of configured FR1 SCell(s): if only SSB or CSI-RS L3 configured for the SCC 2xNumber of configured FR1 SCell(s): if both SSB and CSI-RS L3 configured for the SCC | N/A | N/A | N/A |
| FR2 only intra band CA | N/A | N/A | 1: if only SSB or CSI-RS L3 configured for PCC 2 if both SSB and CSI-RS L3 configured for PCC | N/A | Number of configured FR2 SCell(s) if only SSB or CSI-RS L3 configured for the SCC 2xNumber of configured FR1 SCell(s): if both SSB and CSI-RS L3 configured for the SCC |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1: if only SSB or CSI-RS L3 configured for PCC 2 if both SSB and CSI-RS L3 configured for PCC | 2×(Number of configured SCell(s)-1): if only SSB or CSI-RS L3 configured for the SCC 4×(Number of configured SCell(s)-1): if both SSB and CSI-RS L3 configured for the SCC | N/A | 2: if only SSB or CSI-RS L3 configured for the SCC 4: if both SSB and CSI-RS L3 configured for SCC | 2×(Number of configured SCell(s)-1): if only SSB or CSI-RS L3 configured for the SCC 4×(Number of configured SCell(s)-1): if both SSB and CSI-RS L3 configured for the SCC |

# CSSF for CSI-RS based measurement within measurement gap

For CSSFwithin_gap,i, since all frequency layers are measured in sequential order, we need to consider CSI-RS and SSB as different frequency layer for all scenarios.

Proposal 9: For CSSFwithin_gap,i, CSI-RS and SSB should be considered as different frequency layers, including the scenarios where they are configured in the same MO, and/or have the same center frequency and the same SCS.

# Conclusion

In this contribution, several remaining issues around UE measurement capabilities are discussed. It is proposed.

Proposal 1: it is proposed to remove feature group 12-1 and introduce the following UE capacities

- Feature group 12-3: Simultaneous reception of intra-frequency CSI-RS and SSB within the active BWP bandwidth and with the same numerology

- Feature group 12-4: Simultaneous reception of intra-frequency CSI-RS and SSB within the active BWP bandwidth and with mixed numerologies

Proposal 2: Introduce the following UE capability to support up to 3 mixed numerologies

- Feature group 12-5: Simultaneous reception of CSI-RS, SSB and data channel within the active BWP bandwidth and with three mixed numerologies

Proposal 2a: UE behaviour is not specified if 3 mixed numerologies are configured simultaneously.

Proposal 3: With time restriction introduced for CSI-RS resource configurations, it is not suggested introduce UE capability with minimum separation between two CSI-RS slots

Proposal 4: CSI-RS and SSB in the same MO should be considered as 2 layers.

Proposal 5: If the following two conditions can be met, CSI-RS resources configured in multiple MO can be counted as one frequency layer

- different MO share the same center frequency and the same SCS for CSI-RS resources

- the total number of CSI-RS resources associated with the same PCI should be no more than maxNrofCSI-RS-ResourcesRRM

Proposal 6: For CSSFwoutside_gap,i, CSI-RS and SSB should be considered as different frequency layers, including the scenarios where they are configured in the same MO, and/or have the same center frequency and the same SCS.

Proposal 7: In CSSF definition, it is assumed that CSI-RS and SSB measurement share the same searcher.

Proposal 8: RAN4 only specifies the requirements based on the assumption that CSI-RS time domain restriction and SMTC, which are corresponding to the same PCI, should be aligned and confined with 5ms window.

Proposal 9: For CSSFwithin_gap,i, CSI-RS and SSB should be considered as different frequency layers, including the scenarios where they are configured in the same MO, and/or have the same center frequency and the same SCS.