3GPP TSG-RAN WG4 Meeting #96-e	R4-2010120

Electronic Meeting, 17-28 Aug., 2020

Agenda Item:	7.13.1.6

Source: 		Qualcomm Incorporated

Title: 			Multiple SCell activation in NR

Document for:	Discussion

# 1.	Introduction

A Core requirement development for multiple SCell activation was completed in RAN4#95e and the agreed requirements have been implemented in the most recent RAN4 sepc (TS 38.133 v16.4.0). However, since one of side conditions for intra-band contiguous CA scenario cannot be known to UE, the UE may end up having to establish autonomous assumption on network beam/antenna operation over multiple CCs, which may contradict RAN1 spec. In order to avoid imposing non-spec compliant restriction on network beam operation, we propose to send an LS to RAN1 to see if RAN4 anticipates any potential issues.

# 2. 	Discussion

Though RAN4 defined a set of conditions to specify when the requirements are applied or not based on likely deployment scenarios, we have a concern on one particular condition that cannot be known to UE. A part of requirements implemented in the most recent RRM spec is copied in Table.1, and specific conditions/statements that we want to bring your attention to are highlighted in yellow. In principle, when there is a condition that UE cannot tell if it is fulfilled, UE may end up having to go with the most conservative option which is always assuming “SSB DL Tx beam is same as the corresponding SSB DL Tx beam at the same SSB position of FR1 known cell or FR1 active serving cell” in the highlighted case. This effectively means UE is allowed to autonomously establish an assumption that configured cells in intra-band contiguous CA will exploit a common beam across CCs, which seems to be beyond what RAN1 spec says. With this background, we propose to ask RAN1 if the condition made by RAN4 conflicts with RAN1 spec and if there are any adverse impacts that RAN1 can anticipate.

Proposal 1: RAN4 to send an LS to RAN1 to ask

- if UE is allowed to establish an assumption that configured cells in intra-band contiguous CA exploit a common Tx beam across CCs based on RAN4 side condition in section 8.3.7

- if the condition made by RAN4 conflicts with RAN1 spec

- if there are any adverse impacts that RAN1 can anticipate

Table.1 SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells

### 8.3.7 SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells … If the SCell is known and belongs to FR1 and the SCell measurement cycle is larger than 160ms, Tactivation_time_multiple_scells is: - TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells + Trs + 5ms, if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 unknown Scell - otherwise, TFirstSSB_MAX_multiple_scells + Trs + 5ms - (TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells)+Trs*N1 +Trs +5ms … N1 is the number counting for parallel FR1 unknown to-be-activated SCell(s) only except the ones which fulfilled the following conditions: - contiguous to an active serving cell, or to a known SCell being activated by the same MAC PDU, and - A single SSB is used in the unknown SCell; or multiple SSBs are used in the unknown SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and - its ssb-PositionInBurst is same as the one of FR1 known cell or FR1 active serving cell, and - its SSB DL Tx beam is same as the corresponding SSB DL Tx beam at the same SSB position of FR1 known cell or FR1 active serving cell, and - its SMTC offset is same as the one of FR1 known cell or FR1 active serving cell However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell: - A single SSB is used in the unknown SCell; or multiple SSBs are used in the unknown SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and - its ssb-PositionInBurst is same as the one of FR1 known cell or FR1 active serving cell, and - its SSB DL Tx beam is different from the corresponding SSB DL Tx beam at the same SSB position of FR1 known cell or FR1 active serving cell, and - its SMTC offset is same as the one of FR1 known cell or FR1 active serving cell ...

# 3.	Conclusion

Since one of side conditions for multiple SCell activation requirement for intra-band contiguous CA that cannot be known to UE may contradict RAN1 spec., we proposed:

Proposal 1: RAN4 to send an LS to RAN1 to ask

- if UE is allowed to establish an assumption that configured cells in intra-band contiguous CA exploit a common Tx beam across CCs based on RAN4 side condition in section 8.3.7

- if the condition made by RAN4 conflicts with RAN1 spec

- if there are any adverse impacts that RAN1 can anticipate