3GPP TSG-RAN WG4 Meeting #96e                                            	R4-2010142

Electrical Meeting, Aug.17th – 28th

Source:	Samsung

Title:	Views and simulation results for PMI test cases

Agenda item:			7.16.1.2

Document for:	Approval

- Introduction

In this contribution, we provided views on below open issues of PMI test cases:

- Performance requirements for Type I codebook with 16 ports and 32 ports

- Test set-up/parameters for R15 Type II codebook

- Discussion

  - Performance requirements for Type I codebook with 16 ports and 32 ports

The alignment simulation results for 16 ports and 32 ports Type I codebook cases were provided in Annex.

The TP ratio and SNR at 90% TP summarized in below table：

| SNR /TP ratio Test cases | FDD 2Rx | FDD 4Rx | TDD 2Rx | TDD 4Rx |
| --- | --- | --- | --- | --- |
| 16 ports | 12.3 dB/4.6 | 9.2 dB/5.2 | 13.9 dB/4.2 | 10.5 dB/5.0 |
| 32 ports | 9.7 dB/9.2 | 5.1 dB/11.35 | 9.1 dB/9.3 | 5.4 dB/14 |

Based on our evaluation results, below performance requirements proposed:

Proposal 1: Introduce requirement gamma as 4.5 for 32 ports wideband PMI test cases with Type I codebook for both FDD mode /TDD mode and 2Rx/4Rx cases;

Proposal 2: Introduce requirement gamma as 2.0 for 16 ports sub-band PMI test cases with Type I codebook for both FDD mode /TDD mode and 2Rx/4Rx cases;

  - Test case design for PMI requirements with R15 Type II codebook

In last RAN4 meeting, two candidate test-up “SU-MIMO vs MU-MIMO” were discussed. Both of them above test set-up have pros and cons as summarized below:

- Option 1: SU-MIMO Set-up

  - Pros: Widely used in existing LTE and NR PMI test cases, the feasibility already been verified; proceed the work quickly since RAN4 has lot of experience

  - Cons: not close to the real network scenario of scheduling  Type II codebook, not reflect the advantage of   Type II codebook over than Type I codebook under Multi-user co-scheduling scenario

- Option 2: MU-MIMO Set-up

  - Pros: Close to type II codebook usage scenario

  - Cons: More complexity, the detailed test set-up still unclear and the feasibility from performance requirements and test aspect need to be further confirmed; bring uncertainty of completing Type II codebook test cases in Rel-16 timeframe

The purpose of this test case was to verify UE reporting accuracy of Type II codebook, and either MU-MIMO or SU-MIMO set-up can serve such test purpose. The detailed schedule of Type II is up to network scheduling. There is no restriction of Type II codebook usage scenario no matter SU-MIMO or MU-MIMO set-up agreed in RAN4.

Based on above analysis, we proposed below candidate approach to conclude the work in Rel-16:

Proposal 3: Introducing Type II codebook PMI requirements with MU-MIMO Set-up only if RAN4 can reach consensus on test feasibility and detailed test set-up for MU-MIMO set-up in RAN4#96e; otherwise, introducing Type II codebook PMI requirements in Rel-16 with SU-MIMO Set-up and further evaluate and introducing Type II codebook PMI requirements with MU-MIMO set-up in future release.

2.2.1 Common parameters for set-up

There are some detailed test parameters which should be common irrespective SU-MIMO or MU-MIMO set-up still undecided with candidate options.

Issue 1: Number of Tx ports

- Number of CSI-RS ports: - Option 1: 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4) as baseline - Option 2 : 32 ports with (N1,N2) = (4,4) and (O1,O2)=(4,4)

Regarding number of Tx ports, 4 ~ 32 ports can be supported for Type II codebook. NR Rel-15 Type II codebook and Rel-16 advanced Type II codebook are the extension of LTE Rel-14 advanced codebook which they share similar codebook structure. It’s better we can align the number of Tx ports with PMI test case of LTE eFD-MIMO advanced codebook to provide comparable performance. In LTE Rel-14 eFD-MIMO WI, 16 tx ports was used for PMI test case with advanced codebook.

Figure 1:  RAN4 roadmap of MIMO PMI test cases

The test complexity especially the number of required individual MIMO channel faders also needs to be considered when we decide number of Tx ports：

In table 1 below, we summarized number of independent MIMO faders required for different PMI test cases for LTE and on-going Rel-16 eMIMO WI and performance enhancement WI. Compared to LTE, number of faders increased by 2/4 times pending on the decision of the number of Tx ports we selected.

Table 1: Summary of number of MIMO faders under PMI test cases

| Item | Rel-13 FD-MIMO | Rel-14 eFD-MIMO | Rel-15 NR | Rel-16 performance enhancement WI | Rel-16 eMIMO WI |
| --- | --- | --- | --- | --- | --- |
| Test cases | 32x2 Class A PMI test cases | 16x2 Advanced codebook PMI test case with 2 beams | 8x4 Type I PMI test case | 32x4 Type 1 PMI test case 16x4/32x4 Type II codebook with L=2 beams | 16x4/32x4 Type II codebook with 2 beams modeling |
| Required maximum MIMO faders | 64 faders | 64 faders | 32 faders | 128 faders for Type I 128/256 faders for Type II | 128/256 faders |

With above analysis, we proposed to

Proposal 4-Number of ports: introduce Rel-15 Type II codebook PMI test cases with 16 Tx ports considering test complexity and test coverage.

Issue 2: PMI feedback parameters

- L (numberOfBeams) - 2 - Npsk (phaseAlphabetSize) - Option 1: 4 - Option 2: 8 - SubbandAmplitude - Option 1: False - Option 2: True - PMI-FormatIndicator - Option 1: Wideband - Option 2: Subband

With narrow PMI sub-band size and larger value of Npsk, we can maximize number of candidate codebooks and number of sub-band for PMI reporting. From UE processing respective, this requires maximum UE calculation complexity and acts like a pressure test.

We proposed:

Proposal 5: Introduce Type II codebook test case with Npsk = 8, SubbandAmplitude as”TRUE” and PMI-FormatIndicator as “Sub-band”.

Issue 3: Beam-steering model

- Beam-Steering Model - Beam steering model - Option 1: Reusing beam steering approach with dual-cluster beams as specified in B.2.3B.4A of TS 36.101 - Option 2: Use Equation 1 as beam steering model for Type II codebook performance requirements.

For beam-steering model, we propose to introduce a generic beam steering model into specification which the number of beams configurable.  With this generic beam steering approach, it can be applied for PMI test cases with different codebook types i.e. single beam direction (Type I codebook), dual beam directions (Rel-15 Type II  codebook)  and multi-beam directions (Rel-16 Type II codebook) in a future proof manner.

The multi-beam steering approach as specified in B.2.3B.4A of TS 36.101 can be used as starting point to further extend applied for L beams:

- beam index

- ， relative power of the l beam compared to first beam

- , total power scaling factor

For simplicity, the power of beams can be fixed as equivalent to first beams then beam steering model can as follow

Proposal 6-Beam steering: Introduce a generic beam steering model into specification in a future proof manner which the number of beams configurable.

2.2.2 Other parameters for SU-MIMO Test set-up

Regarding channel model and MIMO correlation, below options agreed in last RAN4 meeting:

- Channel Model - TDLA30-5 as baseline - Other options not precluded - MIMO Correlation - Option 1: XP High - Option 2: XP Medium - Down-select to one option based on simulation results in the next meeting - MCS and Rank - MCS 20 (64QAM Table), Rank 2 as baseline - Other options not precluded

Based on the evaluation results in Annex, there are more performance difference among Rel-16 Type II codebook, Rel-15 Type II and Rel-15 Type I codebook under XP medium correlation compared to XP high correlation.

Proposal 7-Propagation condition: Introduce test case with MIMO correlation -XP Medium and TDLA30-5

Proposal 8-MCS&Rank: It’s feasible to use MCS20 (64QAM), Rank2 for introducing test cases.

2.2.3 Other test parameters for MU-MIMO test set-up

To be updated

3. Conclusion

In this contribution, we provided views for PMI test cases.

Performance requirements for Type I codebook

Proposal 1: Introduce requirement gamma as 4.5 for 32 ports wideband PMI test cases with Type I codebook for both FDD mode /TDD mode and 2Rx/4Rx cases;

Proposal 2: Introduce requirement gamma as 2.0 for 16 ports sub-band PMI test cases with Type I codebook for both FDD mode /TDD mode and 2Rx/4Rx cases;

Test case design for PMI requirements with R15 Type II codebook

SU-MIMO set-up Vs MU-MIMO set-up:

Proposal 3-SU-MIMO vs MU-MIMO: Introducing Type II codebook PMI requirements with MU-MIMO Set-up only if RAN4 can reach consensus on test feasibility and detailed test set-up for MU-MIMO set-up in RAN4#96e; otherwise, introducing Type II codebook PMI requirements in Rel-16 with SU-MIMO Set-up and further evaluate and introducing Type II codebook PMI requirements with MU-MIMO set-up in future release.

Common parameters:

Proposal 4-Number of ports: introduce Rel-15 Type II codebook PMI test cases with 16 Tx ports considering test complexity and test coverage.

Proposal 5-codebook parameter: Introduce Type II codebook test case with Npsk = 8, SubbandAmplitude as”TRUE” and PMI-FormatIndicator as “Sub-band”.

Proposal 6-Beam steering: Introduce a generic beam steering model into specification in a future proof manner which the number of beams configurable.

Other parameters for SU-MIMO set-up:

Proposal 7-Propagation condition: Introduce test case with MIMO correlation -XP Medium and TDLA30-5

Proposal 8-MCS&Rank: It’s feasible to use MCS20 (64QAM), Rank2 for introducing test cases.

Reference

[1] R4-2008846	Way forward on PMI reporting requirements for Tx ports larger than 8 and up to 32, Ericsson, Samsung

[2] R4-2008847	Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports, Ericsson

Annex

Type I results

16Tx Sub-band PMI test

Figure A-1-1: 16Tx FDD results

Figure A-1-2: 16Tx TDD results

32Tx Wide-band PMI test

Figure A-1-3: 32Tx FDD results

Figure A-1-4: 32Tx TDD results

Type II results

Based on the agreed simulation assumption in [2], simulation results with SU-MIMO set-up and MU-MIMO set-up were provided below.

For comparison, we evaluated performance with 3 types of codebook: Rel-16 Type II codebook, Rel-15 Type II codebook, Rel-15 Type I codebook.

SU-MIMO results

FR1 FDD mode

Figure A-2-1: FDD 16x2 XP High Throughput vs SNR

Figure A-2-2: FDD 16x2 XP Medium Throughput vs SNR

Figure A-2-3: TDD 16x2 XP High Throughput vs SNR

Figure A-2-4: TDD 16x 2 XP Medium Throughputs vs. SNR

MU-MIMO results

To be updated