3GPP TSG-RAN4#96e	R4-2010235

17th Aug – 28th Aug 2020

Source: 	Nokia

Title: 					TP for TR 37.717.21-11: DC_12A-48A_n5A

Agenda Item: 			10.4.2

Document for:	Approval

# 1	Introduction

This is a text proposal to introduce DC_12A-48A_n5A n71A into TR 37.717.21-11 Dual Connectivity of 2 bands LTE inter-band CA and 1 NR band.

# 2 	Text proposal

************************************Start of TP********************************************

## 5.x	DC_12-48_n5

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_12A-48A_n5A | DC_12A_n5A DC_48A_n5A |

### 5.x.2	Co-existence studies

When uplink is DC_12A_n5A or DC_48A_n5A there is no IMD interfering 3rd band downlink.

### 5.x.3	∆TIB and ∆RIB values

It is proposed to re-use relaxation values from E-UTRA CA_5-12-48.

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-48_n5 | 12 | 0.4 |
|  | 48 | 0.3 |
|  | n5 | 0.8 |

Table 5.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12-48_n5 | 12 | 0.3 |
|  | 48 | 0 |
|  | n5 | 0.5 |

### 5.x.4	Reference sensitivity exceptions

There is no need for additional REFSENS requirement.

************************************End of TP********************************************