3GPP TSG-RAN WG4 Meeting #96-e                                                              R4-2010256

Online, 17th Aug-28th Aug, 2020

Source:	Samsung, KDDI

Title:	TP for TR 38.717-04-02: CA_n3-n28-n78-n257 and DC_ n3-n28-n78-n257

Agenda item:			10.14.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-04-02 to include CA_n3-n28-n78-n257 and DC_n3-n28-n78-n257according to the request in [1].

# 2. Reference

- RP-200869, New WID on NR inter-band Carrier Aggregation and Dual connectivity for DL 4 bands and 2UL bands.

# 3. Text Proposal

<Start of Text Proposal>

## 5.2.x	CA_n3-n28-n78-n257

#### 5.2.x.1	Operating bands for CA

Table 5.2.x.1-1: Inter-band CA operating bands of CA_n3-n28-n78-n257

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n3-n28-n78-n257 | n3, n28, n78, n257 |

### 5.2.x.2	Channel bandwidths per operating bands for CA

Table 5.2.x.2-1: Supported channel bandwidths per CA configuration for 4DL/2UL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n28A-n78A-n257A | CA_n3A-n257A CA_n28A-n257A CA_n78A-n257A | n3 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes |  |
| CA_n3A-n28A-n78A-n257G | CA_n3A-n257A CA_n28A-n257A CA_n78A-n257A CA_n3A-n257G CA_n28A-n257G CA_n78A-n257G | n3 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 |  | See CA_n257G BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n78A-n257H | CA_n3A-n257A CA_n28A-n257A CA_n78A-n257A CA_n3A-n257G CA_n28A-n257G CA_n78A-n257G CA_n3A-n257H CA_n28A-n257H CA_n78A-n257H | n3 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 |  | See CA_n257H BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n78A-n257I | CA_n3A-n257A CA_n28A-n257A CA_n78A-n257A CA_n3A-n257G CA_n28A-n257G CA_n78A-n257G CA_n3A-n257H CA_n28A-n257H CA_n78A-n257H CA_n3A-n257I CA_n28A-n257I CA_n78A-n257I | n3 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 |  | See CA_n257I BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.2.x.3		UE co-existence studies

### The coexistence studies have been captured into the constituent fallback modes in TR 38.716-03-02, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 5.2.x.4	∆TIB,c and ∆RIB,c values

### The TIB,c and RIB,c  could reuse the values for CA_n3-n28-n78-n257 with 1UL that have been captured  into TR38.716-04-01.

### 5.2.x.5	REFSENS requirements

There is no need to specify additional MSD requirement for this UL CA configuration.

<End of Text Proposal>

<Start of Text Proposal>

## 6.2.x	DC_n3-n28-n78-n257

### 6.2.x.1	Operating bands for DC

Table 6.2.x.1-1: Inter-band DC operating bands of DC_n3-n28-n78-n257

| NR DC Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| DC_n3-n28-n78-n257 | n3, n28, n78, n257 |

### 6.2.x.2 Configurations for DC_n3-n28-n78-n257

### Table 6.2.x.2-1: Inter-band DC configuration of DC_n3-n28-n78-n257

| Downlink NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n3A-n28A-n78A-n257A | DC_n3A-n257A DC_n28A-n257A DC_n78A-n257A |
| DC_n3A-n28A-n78A-n257G | DC_n3A-n257A DC_n28A-n257A DC_n78A-n257A DC_n3A-n257G DC_n28A-n257G DC_n78A-n257G |
| DC_n3A-n28A-n78A-n257H | DC_n3A-n257A DC_n28A-n257A DC_n78A-n257A DC_n3A-n257G DC_n28A-n257G DC_n78A-n257G DC_n3A-n257H DC_n28A-n257H DC_n78A-n257H |
| DC_n3A-n28A-n78A-n257I | DC_n3A-n257A DC_n28A-n257A DC_n78A-n257A DC_n3A-n257G DC_n28A-n257G DC_n78A-n257G DC_n3A-n257H DC_n28A-n257H DC_n78A-n257H DC_n3A-n257I DC_n28A-n257I DC_n78A-n257I |
| NOTE 1: NR configuration for FR1 and FR2 are defined in TS 38.101-1 [2] and TS 38.101-2 [3] respectively. |  |

<End of Text Proposal>