3GPP TSG-RAN WG4 Meeting #96-e                                                              R4-2010252

Online, 17th Aug-28th Aug, 2020

Source:	Samsung, KDDI

Title:	TP for TR 38.717-03-01: CA_n3A-n28A-n41A

Agenda item:			10.9.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-01 to include CA_n3A-n28A-n41A according to the request in [1].

# 2. Reference

- RP-200922, New WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 1 bands UL.

# 3. Text Proposal

<Start of Text Proposal>

### 6.x	CA_n3-n28-n41

#### 6.x.1	Operating bands for CA

Table 6.x.1-1: CA band combination of band n3+n28+n41

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

#### 6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported bandwidths per CA band combination of band n3+n28+n41

| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n28A-n41A | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

#### 6.x.3	UE co-existence studies

Co-existence studies of CA_n3-n28-n41 with 1UL have been covered in the constituent fall-back modes.

#### 6.x.4	∆TIB and ∆RIB values

For three DLs of Band n3, n28 and n41, the same TIB,c and RIB,c values specified for LTE CA_3-28-41 are used as below.

Table 6.x.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n41 | n3 | 0.5 |
|  | n28 | 0.3 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 6.x.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n41 | n3 | 0 |
|  | n28 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

#### 6.x.5	REFSENS requirements

There are no additional MSD requirements for this band combination

<End of Text Proposal>