3GPP TSG-RAN WG4 Meeting #96-e             	R4-2010276

Electronic meeting, 17th Aug– 28th Aug, 2020

Agenda item:	6.2.3.2

Source:	Samsung

Title:	Initial simulation results for Rel-16 NB-IoT

Document for:	Discussion

# 1	Introduction

During the last RAN#95-e meeting, it was agreed to define the requirement for Rel-16 NB-IoT with multi-TB scheduling. Meanwhile, the details simulation assumption was agreed to facilitate the results alignment with captured in the WF [1]. In this contribution, we present the initial simulation results for alignment purpose.

# 2	Simulation Results

2.1	Simulation assumption

The simulation assumption for multi-TB scheduling is specified as indicted in the table 1,

Table 1: simulation assumption for NPUSCH performance requirements with multi-TB scheduling

| Parameters | Value |
| --- | --- |
| Number of tones | 12 |
| SCS | 15KHz |
| Antenna configuration | 1T2R |
| Channel model | ETU 1Hz |
| Frequency error | 0Hz |
| Timing error | 0us |
| Performance target | SNR@70% of maximum throughput |
| N_RU | 5 |
| Repetition number | 64 |
| HARQ process | 2 |
| Transmission mode | Interleaved |
| TB size | 424 |

.

2.2	Simulation Results

In this subsection, based on the agreed assumption, the initial simulation results are provided for alignment purpose.

Table 1: Ideal simulation results for NPUSCH with multi-TB scheduling

| TBS | N_RU | N_Rep | Antenna configuration | SNR @70% of maximum throughput (dB) |  | Gain of interleaving (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Interleaving | Continuous |  |
| 424 | 5 | 64 | 1T2R | -16.42 | -15.78 | 0.64 |

Table 2: Impairment simulation results for NPUSCH with multi-TB scheduling

| TBS | N_RU | N_Rep | Antenna configuration | SNR @70% of maximum throughput (dB) |  | Gain of interleaving (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Interleaving | Continuous |  |
| 424 | 5 | 64 | 1T2R | -14.42 | -13.78 | 0.64 |

Observation 1: About 0.64 dB gain can be achieved with multi-TB interleaving scheduling compared with continuous scheduling for NPUSCH

# 3	Conclusion

In this contribution, the simulation results for Rel-16 NB-IoT with multi-TB scheduling was presented.

Observation 1: About 0.64 dB gain can be achieved with multi-TB interleaving scheduling compared with continuous scheduling for NPUSCH

# References

R4-2008759, WF on LTE UE and BS perf requirements for additional enhancements for NB-IOT, Huawei