3GPP TSG-RAN WG4 Meeting #96-e              	R4-2010283

Electronic meeting, 17th Aug – 28th Aug, 2020

Agenda item:	7.18.3

Source:	Samsung

Title:	Discussion and simulation results for BS 2-step RACH requirement

Document for:	Discussion

# 1	Introduction

In the last RAN4#96-e meeting, the general issues with demodulation requirement for NR 2-step RACH was discussed and the related agreement was captured in the WF [1]. Meanwhile, the initial assumption was proposed for timing offset feasibility checking.

In this contribution, the view on remaining issue for NR 2-step RACH is provided.  Also, the initial simulation results are provided.

# 2	Discussion

DMRS configuration

Regarding the DMRS configuration, the following options are considered

- Option 1: 1+1+1 - Option 2: 1+1

The motivation is to verify the impact of timing offset on PUSCH MsgA performance.  Generally, the timing offset estimation and compensated are performed per symbol by symbol. Different with high Doppler scenario, network will configure more DMRS symbols for Doppler or frequency offset tracking, where the tracking capability depends on the distance between DMRS symbols. Since the current scenario is not targeting with high speed scenario, therefore, either configure 2 DMRS or 3 DMRS configuration can fulfil the requirement.  As indicated in our initial results, the similar performance can be achieved with two different DMRS configurations.

In current Rel-16 NR HST WI, the related requirement for UL timing adjustment was specified, where the 3 DMRS configuration is configured.  Since there is no new PHY design for NR 2 step RACH, we prefer to align with DMRS configuration for UL timing adjustment requirement.

Meanwhile, based on the agreement about the DMRS configuration. In case there is no RRC parameters configured, the UE will apply the value with “pos2”.

Proposal 1: 3 DMRS can be configured for MsgA PUSCH requirement

PUSCH mapping type

Regarding the PUSCH mapping type, the following options are considered

- Option 1: Type A for FR1, Type B for FR2 - Option 2: Both type A and type B for both FR1 and FR2 - Option 3: Both type A and type B for FR1, type B for FR2 - Option 4: only for mapping type declared to be supported in D.100, if both mapping type A and type B are declared to be supported, the tests shall be done for either type A and type B

In Rel-15 eMBB, the related with PUSCH requirement was specified for both type A and type B in FR1, and Type B for FR2.

Table 8.2.1.2-1: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | -2.3 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | 10.1 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | 12.3 |

Table 8.2.1.2-8: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | -2.3 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | 10.2 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | 12.5 |

As indicated, the performance with configured mapping type A and type B is very similar. In that sense, it is not necessary to replicate the test cases defined in NR Rel-15.

To reduce the test effort, we think it should be enough to define the requirement with mapping type A in FR1 from the test coverage and performance perspective.

Considering the test applicability of different configuration with mapping type has already been defined in NR Rel-15, thus, we are also fine to define requirement with both mapping type A and B in FR1.

Proposal 2: Define the PUSCH mapping type A in FR1 and mapping type B in FR2 for NR 2-step RACH MsgA PUSCH requirement.

Different level TO cycling

During the last meeting, it is agreed to introduce TO cycling (X, ∆t, Y) in defining core requirement. The X, Y should be related with CP length. Meanwhile, two different TO level sets are considered for performance evaluation as

- Medium level TO cycling Range/Cycles; 15KHz; 30KHz; 60KHz; 120KHz. Medium level; X; ∆t; Y; X; ∆t; Y; X; ∆t; Y; X; ∆t; Y. 0.8; 0.2; 2; 0.4; 0.1; 1; 0.2; 0.05; 0.5; 0.1; 0.025; 0.25. - High level TO cycling Range/Cycles; 15KHz; 30KHz; 60KHz; 120KHz. Medium level; X; ∆t; Y; X; ∆t; Y; X; ∆t; Y; X; ∆t; Y. 0; 0.1; 3.8; 0.; 0.1; 3.8; 0.; 0.1; 0.6; 0; 0.1; 0.6.

Generally, the value Y should be related with cell size and CP length.

In RAN1, both SLS and LLS simulation are introduced for the determination of payload size and mapping designs, etc. For SLS simulation assumptions, the targeting cell size is very small with round 200m, up to 25km. With agreed link-level simulation, format A1 is applied, where the use case is targeting for small cell.

| Format |  |  |  |  | Coverage(m) (15KHz) | Use case |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | 139 |  |  |  | 938 | Small cell |
| A2 | 139 |  |  |  | 2109 | Normal cell |
| A3 | 139 |  |  |  | 3516 | Normal cell |
| B1 | 139 |  |  |  | 469 | Small cell |
| B2 | 139 |  |  |  | 1055 | Normal cell |
| B3 | 139 |  |  |  | 1758 | Normal cell |
| B4 | 139 |  |  |  | 3867 | Normal cell |
| C0 | 139 |  |  |  | 5300 | Normal cell |
| C2 | 139 |  |  |  | 6000 | Normal cell |

Generally, the timing offset should be set large than CP, which will result in the serious ISI. Since there is no limitation for preamble format, the timing offset should be applied with all the preamble format. The CP length, Ncs, maximal delay of fading channel, and derive the maximum timing offset can be calculated for all the preamble and SCS as indicated as follows

| Format | SCS | CP | Ncs | (a) min {CP, Ncs} | (b) Maximum delay of fading channel (Note) | (a-b) Maximum configured timing offset |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 15 kHz | 288 | 339 | 288 | 80 | 208 | 6.77 us |
|  | 30 kHz | 144 | 339 | 144 | 80 | 64 | 2.08 us |
|  | 60 kHz | 72 | 254 | 72 | 9 | 63 | 2.05 us |
|  | 120 kHz | 36 | 127 | 36 | 9 | 27 | 0.88 us |
| A2 | 15 kHz | 576 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 30 kHz | 288 | 339 | 288 | 80 | 208 | 6.77 us |
|  | 60 kHz | 144 | 254 | 144 | 9 | 135 | 4.39 us |
|  | 120 kHz | 72 | 127 | 72 | 9 | 63 | 2.05 us |
| A3 | 15 kHz | 864 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 30 kHz | 432 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 60 kHz | 216 | 254 | 216 | 9 | 207 | 6.74 us |
|  | 120 kHz | 108 | 127 | 108 | 9 | 99 | 3.22 us |
| B4 | 15 kHz | 936 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 30 kHz | 468 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 60 kHz | 234 | 254 | 234 | 9 | 225 | 7.32 us |
|  | 120 kHz | 117 | 127 | 117 | 9 | 108 | 3.52 us |
| C0 | 15 kHz | 1240 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 30 kHz | 620 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 60 kHz | 310 | 254 | 254 | 9 | 245 | 7.98 us |
|  | 120 kHz | 155 | 127 | 127 | 9 | 118 | 3.84 us |
| C2 | 15 kHz | 2048 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 30 kHz | 1024 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 60 kHz | 512 | 254 | 254 | 9 | 245 | 7.98 us |
|  | 120 kHz | 256 | 127 | 127 | 9 | 118 | 3.84 us |
| Note: TDLC300 for FR1, TDLA30 for FR2 |  |  |  |  |  |  |  |

In order to simply the test, a unified timing offset should be considered for all the preamble formats. And it is seen from Table 1 that format A1 with 120 kHz SCS has the smallest maximum timing offset (due to the short CP length), which is 0.88us. During the Rel-15 PRACH test, the maximum timing offset is 0.8us. In that sense, it is not practical to define the requirement with high level TO cycling.

Proposal 3: 0.8us can be considered as the maximum timing offset for MsgA PUSCH

TBS and MCS configuration

As for initial simulation, MCS1 and 2PRB is selected with. In order to fulfill the targeting TBS with 56-72bits, MCS 0 can be selected for requirement, where the related TBS is 64 for 2 DMRS configuration, and 56 for 3 DMRS configuration.

Proposal 4: MCS 0 can be selected for requirement with NR step RACH, to fulfill the targeting TBS with 56-72bits

Test metric

Regarding the test metric, the following options are considered

- BLER of MsgA when preamble are correctly detected - Option 1: 0.01 - Option 2: 0.1

Since there is no retransmission, we prefer to use the test metric BLER as 0.1.

Proposal 5: Using BLER 0.1 as the test metric for requirement of MsgA PUSCH

# 3	Initial simulation results

In this section, the initial simulation are provided to investigate the impact of timing offset on the MsgA PUSCH.

Simulation assumption for MsgA PUSCH with timing offset

| Parameter | Values for FR1 |  | Values for FR2 |  |
| --- | --- | --- | --- | --- |
| Preamble Format | Free Choice |  |  |  |
| Waveform | CP-OFDM |  |  |  |
| Power offset between preamble and MsgA | Free choice |  |  |  |
| Subcarrier Spacing for PUSCH | 15KHz, 30KHz |  | 60KHz, 120KHz |  |
| PUSCH mapping type | Option 1: Type A and Type B Option 2: Type A |  | Option 1: Type A and Type B Option 2: Type A |  |
| MCS level | 1 |  | 3 |  |
| Number of Symbols | 14 |  | 10 |  |
| Number of PRBs | 2 |  | 2 |  |
| TBS | 56-72bits |  |  |  |
| DMRS | Option 1: 1+1+1 Option 2: 1+1 |  | Option 1: 1+1+1 Option 2: 1+1 |  |
| Antenna configuration | 1T2R |  | 1T2R |  |
| Propagation channel | TDLC300-100 |  | TDLA30-300 |  |
| TO values | Medium Level | SCS 15KHz: 0.8:0.2:2 | Medium Level | SCS 60KHz: 0.2:0.05:0.5 |
|  |  | SCS 30KHz: 0.4:0.1:1 |  | SCS 120KHz: 0.1:0.025:0.25 |
|  | High Level | SCS 15KHz: 0:0.1:3.8 | High level | SCS 60KHz: 0:0.1:0.6 |
|  |  | SCS 30KHz: 0:0.1:3.8 |  | SCS 120KHz: 0:0.1:0.6 |
| Test metric (BLER of MsgA when preambles are correctly detected) | Option 1: 0.01 Option 2: 0.1 |  | Option 1: 0.01 Option 2:0.1 |  |
| Re-transmission | No retransmission considered |  |  |  |

Figure 1: BLER requirement with different timing offset under 2 DMRS configuration, 15KHz SCS

Figure 2: BLER requirement with different timing offset under 2 DMRS configuration, 30KHz SCS

Figure 3: BLER requirement with different timing offset under 3 DMRS configuration, 15KHz SCS

Figure 4: BLER requirement with different timing offset under 3 DMRS configuration, 30KHz SCS

As indicated in figures

Similar BLER performance can be achieved with 2 DMRS and 3 DMRS configuration.

With small value of TO, such as smaller than 0.5 us, the impact on BLER performance is very minor without TO compensation. The existed requirement can be fulfilled.

With large value of TO, such as 2.0us, 3.8us, the performance has large degradation. Under this condition, TO compensation should be considered.

With the TO larger than CP, the BLER performance is still worse even with TO compensation operation, where the PUSCH detection error comes from the ISI.

Observation 1: Similar BLER performance can be achieved with 2 DMRS and 3 DMRS configuration

Observation 2: With small value of TO, the impact on BLER performance of MsgA is minor without TO compensation.

Observation 3: With TO larger than CP, even with TO compensation operation, large performance degradation is still existed.

Proposal 5:  Do not define the MsgA PUSCH requirement with timing offset lager than CP.

# 4 Conclusion

In this contribution, the view on the remaining issue of requirement is provided. Meanwhile, the initial simulation results are provided to investigate the timing offset impact on the MsgA PUSCH requirement.

Proposal 1: 3 DMRS can be configured for MsgA PUSCH requirement

Proposal 2: Define the PUSCH mapping type A in FR1 and mapping type B in FR2 for NR 2-step RACH MsgA PUSCH requirement.

Proposal 3: 0.8us can be considered as the maximum timing offset for MsgA PUSCH

Proposal 4: MCS 0 can be selected for requirement with NR step RACH, to fulfill the targeting TBS with 56-72bits

Proposal 5: Using BLER 0.1 as the test metric for requirement of MsgA PUSCH

Observation 1: Similar BLER performance can be achieved with 2 DMRS and 3 DMRS configuration

Observation 2: With small value of TO, the impact on BLER performance of MsgA is minor without TO compensation

Observation 3: With TO larger than CP, even with TO compensation operation, large performance degradation is still existed

Proposal 6:  Do not define the requirement with timing offset lager than CP

# References

- [1] R4-2008864 WF on BS demodulation requirements for 2-step RACH, ZTE