3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2010297

# Electronic Meeting, 17th– 28th August, 2020

Source:	Verizon, Qualcomm, Nokia, Apple

Title:	The P-MPR based solution for CA_n2-n77, CA_n5-n77, CA_n66-n77 for TP for 38.717-02-01

Agenda item:	10.2.2

Document for:	Approval

# Introduction

In RAN#88-e meeting, the “New WID on NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2)” [1]  was approved. This contribution is a text proposal for TP for TR 38.717-02-01 (V0.0.1) to introduce PC2 power class for following combos in supporting the UE-based (P-MPR) scheme discussed in [3].

- CA_n2-n77 and DC_n2-n77

- CA_n5-n77 and DC_n5-n77

- CA_n66_n77 and DC_n66-n77

# Reference

[1] RP-200672: 	New WID on NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), ZTE

[2] RP-200671: 	TR 38.716-02-00 v1.2.0 Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), ZTE

[3] R4-2010295:    Rel-16 Power Class 2 for EN-DC FDD+TDD UE, Verizon, Qualcomm, Nokia, Apple

# Text Proposal

## <Start of Text Proposal>

#### 6.x.2.1		Maximum output power for inter-band CA

For this inter-band carrier aggregation with uplink assigned to two NR bands, the maximum output powers are specified in Table 6.x.1.1-2

Table 6.x.1.1-2 UE Power Class for uplink inter-band CA (two bands)

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n77A |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n2A-n77(2A) |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |

## <Start of Text Proposal>

## <Start of Text Proposal>

## 6.x.2.1	Maximum output power for inter-band CA

For this inter-band carrier aggregation with uplink assigned to two NR bands, the maximum output powers are specified in Table 6.x.1.1-2

Table 6.x.1.1-2 UE Power Class for uplink inter-band CA (two bands)

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n77A |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n5A-n77(2A) |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |

## <Start of Text Proposal>

## <Start of Text Proposal>

#### 6.x.2.1		Maximum output power for inter-band CA

For this inter-band carrier aggregation with uplink assigned to two NR bands, the maximum output powers are specified in Table 6.x.1.1-2

Table 6.x.1.1-2 UE Power Class for uplink inter-band CA (two bands)

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n66A-n77A |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n66A-n77(2A) |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |

## <Start of Text Proposal>