3GPP TSG-RAN4 Meeting #96-e	R4-2010390

# Electronic Meeting, Aug. 17-28, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.133 CR on introduction of CSI-RS based intra-frequency measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Core |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The definition of CSI-RS based intra-frequency measurement and the scope of the Rel16 requirements were agreed in RAN4 #95e meeting. They needs to be introduced to TS 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR on CSI-RS based L3 measurement framework and introduction R4-2009004 was technically endorsed in RAN4#95e meeting. The following are added on basis of this CR: - the scope of Rel16 requirements. - the definition of CSI-RS based intra-frequency maesurement. - the applicable scenario for intra-frequency measurement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RS based measurement requirements are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 9.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 9.x	CSI-RS based L3 measurements

### 9.x.1	Introduction

This clause contains general requirements on the UE regarding CSI-RS based measurement reporting in RRC_CONNECTED state. The requirements are split in intra-frequency and inter-frequency measurements requirements. Single FFT is assumed for multiple cell measurements per frequency layer for both intra- and inter-frequency measurements.

The requirements in this clause apply, provided:

- all CSI-RS resources in the same MO have the same bandwidth, and

-  the bandwidth of CSI-RS resources in the target MO is the same as that of the CSI-RS resources configured for the serving cell, and

- associatied SSB is configured and detected.

- associated SSB is QCLed with the corresponding CSI-RS resources in FR2

### 9.x.2	CSI-RS based Intra-frequency measurements

#### 9.x.2.1	Introduction

A measurement is defined as a CSI-RS based intra-frequency measurement provided

- the subcarrier spacing of CSI-RS resources on the neighbour cell configured for measurement is the same as the subcarrier spacing of CSI-RS resource on the serving cell indicated for measurement, and

- the CP type of CSI-RS resources on neighbour cell configured for measurement is the same as the CP type of CSI-RS resources on the serving cell indicated for measurement, which is applied for SCS = 60kHz, and

- the centre frequency of CSI-RS resources on the neighbour cell configured for measurement is the same as centre frequency of CSI-RS resource on the serving cell indicated for measurement

When the measurement on one intra-frequency measurement object or inter-frequency measurement object transitions from measurements performed outside gaps to measurements performed within gaps or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply.

The UE shall be able to identify new intra-frequency cells based on the associatedSSB and cellId in the CSI-RS-ResourceCofigMobility and perform CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements of identified intra-frequency cells if carrier frequency information is provided by PCell or the PSCell, even if no explicit neighbour list with physical layer cell identities is provided.

The intra-frequency CSI-RS based measurement requirements defined in clause 9.x.2 are applicable if the bandwidth of the CSI-RS resources on the neighbor cell is completely within the active BWP of the UE. The UE can perform intra-frequency CSI-RS based measurements without measurement gaps. For intra-frequency CSI-RS based measurements, UE may cause scheduling restriction as specified in clause 9.x.5.3.

#### 9.x.2.2	Requirements applicability

The requirements in clause 9.x apply, provided:

-	The cell being identified or measured is detectable, and

-	The CSI-RS resources configured for CSI-RS based measurements are measurable.

An intra-frequency cell shall be considered detectable when for each associatedSSB:

-	SS-RSRP related side conditions given in clauses 10.1.2 and 10.1.3 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1.7 and 10.1.8 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1.12 and 10.1.13 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

#### 9.x.2.3	Number of cells and number of CSI-RS resources

#### 9.x.2.4	Measurement Reporting Requirements

#### 9.x.2.5	Intra-frequency measurements without measurement gaps

<End of Change 1>

<Start of Change 2>

### 9.x.3	Inter-frequency measurements

#### 9.x.3.1	Introduction

#### 9.x.3.2	Requirements applicability

#### 9.x.3.3	Number of cells and number of CSI-RS resources

#### 9.x.3.4	Measurement Reporting Requirements

#### 9.x.3.5	Inter-frequency measurements with measurement gaps

<End of Change 2>