3GPP TSG-RAN WG4 Meeting #96-e				  											R4-2010429 Online, 17th Aug. 2020 – 28th Aug. 2020

Source: 	Nokia

Title: 		TP for 37.717-11-21 to introduce DC_7A_n40A-n78A

Agenda Item: 			10.7.2 [DC_R17_xBLTE_2BNR_yDL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.717-11-21 to introduce DC_7A_n40A-n78A.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 6.x	DC_7_n40-n78

### 6.x.1	Operating bands for DC

Table 6.x.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n40-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.x.2	Configuration for DC

Table 6.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A_n40A-n78A | DC_7A_n40A DC_7A_n78A | 7A | CA_n40A-n78A |

### 6.x.3	Co-existence studies

Co-existence analysis for DC_7_n40 UL shows no impact to NR Band n78 DL.

Co-existence analysis for DC_7_n78 UL shows no impact to NR Band n40 DL.

### 6.x.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n40-n78 | 7 | 0.5 |
|  | n40 | 0.6 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n40-n78 | 7 | 0 |
|  | n40 | 0.5 |
|  | n78 | 0.5 |

### 6.x.5	MSD

No further MSD is needed.

*********************** End of the TP ***************************************************

# 3	References