3GPP TSG-RAN WG4 Meeting #96-e	R4-2010447

# Electronic meeting, August 17th-28th, 2020

Source:		Ericsson

Title:			47GHz band - Regulatory overview – Band plan - System parameters

Agenda item:		10.24.4

Document for:	Approval

# Introduction

A new WI [1] has been approved in last RAN#88 meeting to introduce the 47 GHz band. The goal with this new WI is to specify a new NR FR2 band covering the 47.2-48.2 GHz frequency range.

This contribution is giving an overview of the Regulatory requirements and propose system parameters for this new band

# Discussion

## Regulatory overview

### ITU-R

In last WRC-19, the 47.2-48.2 GHz frequency range was identified for IMT usage and following note was added to the ITU-R Radio Regulations ([2]):

### FCC

In November 2017, FCC released a 2nd Report & Order for the 24GHz bands with a 2nd FNPRM (Further Notice of Proposed Rulemaking) proposing the 47.2-48.2 GHz band for fixed and mobile operations.

It was finally decided to auction 10 blocks of 100 MHz in each PEA (Partial Economic Area) without the need for any Spectrum Access System.

In December 2019, FCC started the auction the 47GHz band (Auction 103) which was concluded in March 2020.

## Band plan - System parameters

### Band plan

As mentioned in the WI, the new NR FR2 band will be a TDD band and we propose to name it n262, using the next available band number:

Table 5.2-2: NR operating bands in FR2

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode |
| --- | --- | --- |
| n257 | 26500 MHz – 29500 MHz | TDD |
| n258 | 24250 MHz – 27500 MHz | TDD |
| n259 | 39500 MHz – 43500 MHz | TDD |
| n260 | 37000 MHz – 40000 MHz | TDD |
| n261 | 27500 MHz – 28350 MHz | TDD |
| n262 | 47200 MHz – 48200 MHz | TDD |

And the supported sub-carrier spacing should be:

Table 5.3.5-2: BS channel bandwidths and SCS per operating band in FR2

| NR band / SCS / BS channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n258 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n259 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n260 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n261 | 60 | Yes | Yes | Yes |  |
|  | 120 |  | Yes |  | Yes |  | Yes |  | Yes |  |
| n262 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |

### System parameters

The NR-AFRCN  for this new band should be defined as here after:

Table 5.4.2.3-2: Applicable NR-ARFCN per operating band in FR2

| NR operating band | ΔFRaster (kHz) | Uplink and Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n262 | 60 | 2399166 – <1> – 2415832 |
|  | 120 | 2399167 – <2> – 2415831 |

And the SS raster entries as per following table:

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2)

| NR operating band | SS Block SCS | SS Block pattern (note) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n262 | 120 kHz | Case D | 23586 – <1> – 23641 |
|  | 240 kHz | Case E | 23588 – <2> – 23640 |

# Conclusion

In this contribution, after giving a Regulatory overview, we made following proposal

Proposal: The 47GHz band will be n262 as defined in tables 5.2-2 and 5.3.5-2, with system parameters specified in tables 5.4.2.3-2 and 5.4.3.3-2.

# References

- RP-201232, New WID on introduction of NR 47 GHz band, T-Mobile USA

- World Radiocommunication Conference 2019 (WRC-19), Final Acts