3GPP TSG-RAN WG4 Meeting # 96-e	R4-2010484

# Electronic Meeting, 17-28 Aug., 2020

Agenda item:	11.1.1

Source: 	Huawei

Title: 	TP to TR 38.921: UE IMT technology related parameters

Document for:	Approval

- Introduction

The WF on on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz was approved in [1].

This paper provides a TP for TR 38.921 to capture the above agreements and our proposal on remaining requirements.

References

[1] R4-2009151, WF on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz, CATT

[2] R4-2010370, TR 38.921 V 0.1.0

TP to TR 38.921

< START OF CHANGE>

# 7	UE parameters

## 7.1	Transmitter characteristics

### 7.1.1	Power dynamic range

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value. For existing FR1 bands, the minimum output power is -33 dBm for 100 MHz channel bandwidth. The minimum output power can be reused for both frequency ranges, 6.425-7.125GHz and 10.0-10.5GHz, i.e. power dynamic range is 56 dB for 100 MHz channel bandwidth.

### 7.1.2	Spectral mask

[Co-existence simulations are carried out to derive the limits]

### 7.1.3	ACLR

[Co-existence simulations are carried out to derive the limits]

### 7.1.4	Spurious emissions

The general spurious emissions defined in TS 38.101-1 clause 6.5.3.1 can apply to both frequency ranges, 6.425-7.125GHz and 10.0-10.5GHz.

- 30MHz ≤ f ≤ 1 GHz: -36dBm/100kHz

- 1 GHz ≤ f ≤ 26 GHz: -30dBm/1MHz

### 7.1.5	Maximum output power

TR 38.820 indicates that 23dBm is feasible at 10-10.5GHz hence this is a good assumption to make as a worst case for any co-existence analysis (although actual power may be lower due to implementation at the higher frequencies). Hence the UE maximum output power

### 7.1.6	Average output power

It was agreed the average output power won’t be mentioned in the reply LS.

## 7.2	Receiver characteristics

### 7.2.1	Noise figure

The assumption from TR 38.820 is that the typical NF for NR UE at 10 GHz is 9 dB which is the same as the UE operating below 6 GHz. Hence 9 dB NF should be adopted in the co-existence study.

### 7.2.2.	Sensitivity

For mobile station, the following equation can be used to calculate the sensitivity level

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

Where SNR = -1 dB and IM =2.5 dB, and If 2Rx is considered, the diversity gain is 3dB.

The sensitivity would not be used directly in the sharing studies hence it was agreed the sensitivity level won’t be mentioned in the reply LS.

### 7.2.3	Blocking response

[Co-existence simulations are carried out to derive the limits]

### 7.2.4	ACS

[Co-existence simulations are carried out to derive the limits]

< END OF CHANGE>