3GPP TSG-RAN WG4 Meeting # 96-e 															R4-2010487

Electronic Meeting, 17 August – 28 August, 2020

Source: 	Huawei, HiSilicon

Title: 	Uplink co-existence simulation results on 6.425-7.125GHz and 10.0-10.5GHz for urban macro and indoor hotspot

Agenda Item:	11.1.3.3

Document for:	Approval

Introduction

The SI on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz was approved at TSG RAN#87-e [1]. Transmitter and receiver characteristics for the NR UE and BS need to be studied and evaluated. As for ACLR and ACS, RAN4 need to perform the adjacent channel co-existence simulations on both 6.425-7.125GHz and 10.0-10.5GHz.

In last meeting, the simulation assumptions are approved in WF [2] and TP [3]. In this paper, we’d like to provide the adjacent channel uplink co-existence simulation results on both 6.425-7.125GHz and 10.0-10.5GHz for urban macro and indoor hotspot.

Discussion

Simulation results on 6.425-7.125GHz

Based on the current specifications, we set 46dB ACS for BS and 30dB ACLR for UE as baseline. The corresponding UL ACIR is about 29.9dB. When we simulate the scenario, different delta-ACIR X values were considered in order to find the reasonable requirements. The CDF figures of SINR and throughput for Macro scenario are shown below.

Figure 1 The CDF figures of SINR and throughput for Macro scenario

The urban macro simulation results of the uplink throughput loss of the victim BS with different delta-ACIR values are shown at 7GHz centre frequency in table 1

Table 1 The urban macro simulation results of the uplink throughput loss at 7GHz centre frequency

| delta-ACIR X [dB] | 0 | -1 | -2 | -3 |
| --- | --- | --- | --- | --- |
| Average throughput (7GHz) baseline | 63.52744745 | 63.5476779 | 63.57265959 | 63.46620526 |
| Average throughput (7GHz) | 63.09001658 | 63.10162703 | 63.01823906 | 62.80567674 |
| Average throughput loss (7GHz) | 0.69% | 0.70% | 0.87% | 1.04% |
| 5%-tile throughput (7GHz) baseline | 15.7906253 | 16.5172905 | 15.82682768 | 14.87491459 |
| 5%-tile throughput (7GHz) | 15.00934646 | 15.57347018 | 14.75321777 | 13.4821861 |
| 5%-tile throughput loss (7GHz) | 4.95% | 5.71% | 6.78% | 9.36% |

The CDF figures of SINR and throughput for Indoor hotpot scenario are shown below.

Figure 2 The CDF figures of SINR and throughput for Indoor hotpot scenario

The indoor hotpot simulation results of the uplink throughput loss of the victim BS with different delta-ACIR values are shown at 7GHz centre frequency in table 2

Table 2 The indoor hotpot simulation results of the uplink throughput loss at 7GHz centre frequency

| delta-ACIR X [dB] | 0 | -2 | -4 | -6 |
| --- | --- | --- | --- | --- |
| Average throughput (7GHz) baseline | 64.43077251 | 64.50452647 | 64.39146457 | 64.37791136 |
| Average throughput (7GHz) | 64.37493592 | 64.42260475 | 64.21942166 | 64.06001737 |
| Average throughput loss (7GHz) | 0.09% | 0.13% | 0.27% | 0.49% |
| 5%-tile throughput (7GHz) baseline | 34.44734742 | 33.90311989 | 34.32613785 | 34.01385308 |
| 5%-tile throughput (7GHz) | 34.4441748 | 33.90130961 | 34.32506726 | 33.99160191 |
| 5%-tile throughput loss (7GHz) | 0.01% | 0.01% | 0.00% | 0.07% |

It can be found that the Urban Macro is the restricted scenario instead of the indoor hotpot. Based on the simulation results, urban macro scenario can be restricted to 5% throughput loss with uplink delta-ACIR of 0dB at 7GHz. The corresponding ACIR value is 29.9 dB which is equal to current FR1 requirements. If we keep 46dB ACS for BS at 7GHz, 30 dB ACLR for UE can be achieved to meet 29.9 dB ACIR.

Observation 1: When uplink ACIR is set to 29.9dB at 7GHz, the urban macro and indoor scenario can be restricted to 5% UL throughput loss.

Simulation results on 10.0-10.5GHz

The CDF figures of SINR and throughput for Macro scenario on 10.0-10.5GHz are shown below.

Figure 3 The CDF figures of SINR and throughput for Macro scenario at 10GHz

The urban macro simulation results of the uplink throughput loss of the victim BS with different delta-ACIR values are shown at 10GHz centre frequency in table 3.

Table 3 The urban macro simulation results of the uplink throughput loss at 10GHz centre frequency

| delta-ACIR X [dB] | 0 | -1 | -2 | -3 |
| --- | --- | --- | --- | --- |
| Average throughput (10GHz) baseline | 63.41765476 | 63.3310771 | 63.21587222 | 63.31738593 |
| Average throughput (10GHz) | 62.9506372 | 62.83531091 | 62.66383608 | 62.55193044 |
| Average throughput loss (10GHz) | 0.74% | 0.78% | 0.87% | 1.21% |
| 5%-tile throughput (10GHz) baseline | 11.04864891 | 12.16136267 | 11.45458039 | 12.82707659 |
| 5%-tile throughput (10GHz) | 10.80636676 | 11.83903805 | 11.06752199 | 12.24201683 |
| 5%-tile throughput loss (10GHz) | 2.19% | 2.65% | 3.38% | 4.56% |

The CDF figures of SINR and throughput for Indoor hotpot scenario are shown below.

Figure 4 The CDF figures of SINR and throughput for Indoor hotpot scenario at 10GHz

The indoor hotpot simulation results of the uplink throughput loss of the victim BS with different delta-ACIR values are shown at 10GHz centre frequency in table 4

Table 4 The indoor hotpot simulation results of the uplink throughput loss at 10GHz centre frequency

| delta-ACIR X [dB] | 0 | -2 | -4 | -6 |
| --- | --- | --- | --- | --- |
| Average throughput (10GHz) baseline | 64.35913437 | 64.59088413 | 64.64182807 | 64.62841218 |
| Average throughput (10GHz) | 64.3571779 | 64.31660513 | 64.25154349 | 64.10677273 |
| Average throughput loss (10GHz) | 0.00% | 0.42% | 0.60% | 0.81% |
| 5%-tile throughput (10GHz) baseline | 34.02740228 | 33.76926559 | 33.63832512 | 33.85384686 |
| 5%-tile throughput (10GHz) | 34.02625077 | 33.76920367 | 33.58995292 | 33.78209619 |
| 5%-tile throughput loss (10GHz) | 0.00% | 0.00% | 0.14% | 0.21% |

It can be found that the Urban Macro is the restricted scenario instead of the indoor hotpot at 10GHz as well. Based on the simulation results, urban macro scenario can be restricted to 5% throughput loss with uplink delta-ACIR of -3dB at 10GHz. The corresponding ACIR value is 26.9 dB which is less stringent than current FR1 requirements. If we keep 46dB ACS for BS at 10GHz, 27 dB ACLR for UE can be achieved to meet 26.9 dB ACIR.

Observation 2: When uplink ACIR is set to 26.9dB at 10GHz, the urban macro and indoor scenario can be restricted to 5% UL throughput loss.

Summary

Based on the simulation results and discussion, all the proposals are listed below:

Observation 1: When uplink ACIR is set to 29.9dB at 7GHz, the urban macro and indoor scenario can be restricted to 5% UL throughput loss.

Observation 2: When uplink ACIR is set to 26.9dB at 10GHz, the urban macro and indoor scenario can be restricted to 5% UL throughput loss.

- References

- RP-200513, “New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz”, Ericsson, Huawei, HiSilicon.

- R4-2008927, WF on Simulation Assumptions for the SI on 6.425-7.125GHz and 10.0-10.5GHz, Nokia, Ericsson, ZTE

- R4-2008928, TP to TR 38.9xx: System level simulation methodology and assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz, Nokia, Nokia Shanghai Bell