3GPP TSG-RAN WG4 Meeting #96-e								          R4-2010500

Online, 17th – 28th August 2020

Source: 	Huawei, HiSilicon

Title: 	Discussion on numerology and channel bandwidth for 52.6 GHz to 71 GHz

Agenda Item:	13.2.2

Document for:	Approval

Introduction

In RAN #86, study on supporting NR from 52.6GHz to 71GHz was agreed in [1] with the following objectives:

- Study of required changes to NR using existing DL/UL NR waveform to support operation between 52.6 GHz and 71 GHz

  - Study of applicable numerology including subcarrier spacing, channel BW (including maximum BW), and their impact to FR2 physical layer design to support system functionality considering practical RF impairments [RAN1, RAN4].

  - Identify potential critical problems to physical signal/channels, if any [RAN1].

In this contribution, we discuss numerology and channel bandwidth for 52.6 GHz to 71 GHz.

Discussion

In NR R15/R16, the supported numerologies are listed in the following tables [2]. The design for FR2 could be considered as the starting point, where 60 kHz/120 kHz SCS for data and 120 kHz/240 kHz SCS for SSB are supported. The maximum bandwidth for FR2 was 400MHz with 120 kHz SCS or 200MHz with 60 kHz SCS.

Table 5.1-1: Supported transmission numerologies.

|  |  | Cyclic prefix | Supported for data | Supported for synch |
| --- | --- | --- | --- | --- |
| 0 | 15 | Normal | Yes | Yes |
| 1 | 30 | Normal | Yes | Yes |
| 2 | 60 | Normal, Extended | Yes | No |
| 3 | 120 | Normal | Yes | Yes |
| 4 | 240 | Normal | No | Yes |

Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |

For frequencies above 52.6GHz, both licensed and unlicensed band are included. Although there are different regulatory requirements, the same/similar numerology between licensed and unlicensed band should be adopted as much as possible, which is also beneficial for the implementation.

Proposal 1: Maximum 400MHz carrier bandwidth with 120 kHz / 240 kHz SCS could be considered as the starting point for NR in frequency band between 52.6GHz and 71GHz.

Conclusions

In this contribution, we discuss numerology and channel bandwidth for 52.6 GHz to 71 GHz and the following observation and proposal was made:

Proposal 1: Maximum 400MHz carrier bandwidth with 120 kHz / 240 kHz SCS could be considered as the starting point for NR in frequency band between 52.6GHz and 71GHz.

References

RP-193259  New SID: Study on supporting NR from 52.6GHz to 71 GHz

38.300 V16.2.0 NR and NG-RAN Overall Description

38.104 V16.4.0  Base Station (BS) radio transmission and reception