3GPP TSG-RAN WG4 #96-e	R4-2010557

Online, 17th – 28th August, 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 37.717-21-11 DC_3A-40A_n78A

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

In this contribution, we propose a text proposal for TR 37.717-21-11 for EN-DC band combinations in Rel-17 to introduce DC_3A-40A_n78A.

# 2. Text Proposal

----- Unchanged sections omitted -----

## 5.x	DC_3-40_n78

### 5.x.1	Operating bands for DC

Table 5.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-40_n78 | CA_3-40 | n78 | DC_3_n78 |

### 5.x.2	Configurations for DC

Table 5.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-40A_n78A | DC_3A_n78A DC_40A_n78A | CA_3A-40A | n78 |

### 5.x.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n78 captured in 37.863-01-01, MSD shall be considered since

-	2nd 4th and 5th order IMD of the two bands may fall into Rx frequencies of band 3

- 	2nd harmonic of band 3 may fall into Rx frequencies of n78

- 	5th order IMD of the two bands may fall into Rx frequencies of band 40

And based on co-existence studies of Band 40 + Band n78 captured in 37.716-11-11, MSD shall be considered since

-	4th order IMD generated by dual uplink of the two bands may fall into own Rx of band 3.

### 5.x.4	∆TIB and ∆RIB values

Table 5.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-40-n78 | 3 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-40-n78 | 3 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.x.5	REFSENS requirements

As stated in 5.x.3, for MSD requirement caused by IMDs is specified below accordingly. To note that the MSD requirements are already defined for DC_3A_n40A-n78A in TS 38.101-3, among which the one for IMD4 on band n40 can be followed here for band 40 IMD4 impact.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-40A_n78A | 3 | 1775 | 5 | 25 | 1870 | 9.1 | IMD4 |
|  | 40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
|  | n78 | 3325 | 10 | 50 | 3325 | N/A | N/A |
| DC_3A-40A_n78A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 40 | 2360 | 5 | 25 | 2360 | 4.4 | IMD5 |
|  | n78 | 3760 | 10 | 50 | 3760 | N/A | N/A |

Other issues mentioned in 5.x.3 (i.e., 2nd harmonic from band 3 and MSD for band 3 caused by 2nd, 4th and 5th IMD of band 3 and n78) had already been specified in the corresponding specs. To note that due to the high IMD impact, single UL is allowed between band 3 and band n78.

----- Unchanged sections omitted -----

# Reference