3GPP TSG-RAN WG4 Meeting #94e	R4-2010608

Electronic Meeting, 17 - 28 August 2020

Source:	Ericsson

Title:	Simulation results for additional SCS/BW combinations for Rel-16 NR HST PUSCH demodulation performance at UE speed of 350 km/h and 500 km/h

Agenda item:	7.15.3.2.1

Document for:	Information

# Introduction

In this contribution, we presented our simulation results for additional SCS/BW combinations for NR HST PUSCH demodulation performance for single tap HST scenarios at HST speed = 350 km/h and 500 km/h.

# Simulation assumptions and simulation results

In this section, we presented our simulation assumptions based on Table 1 for HST scenarios. The corresponding ideal simulation results and results with impairments for HST scenarios are given in Table 2 to  Table 3.

| Parameters | Values |
| --- | --- |
|  | FR1 |
| Waveform | CP-OFDM |
| Number of Tx and Rx | Tunnel scenario (Dmin = 2 m, Ds = 300 m): 1Tx 1Rx and 1Tx 2Rx Open space scenario (Dmin = 150 m, Ds = 700 m): 1Tx 2Rx and 1Tx 8Rx |
| DMRS type | type 1 |
| Number of DMRS symbols | 1+1+1 DM-RS symbols with l0 = 2 |
| Number of symbols | 14 |
| Time domain resource allocation type | type A |
| Frequency domain resource | full RB allocation of the applicable BW |
| MCS index | 2, 16 |
| Carrier frequency (fc) | SCS = 15 kHz: fc = 2.1 GHz (for UE speed = 350 km/h) and fc = 1.8 GHz for (for UE speed = 500 km/h) SCS = 30 kHz: fc = 3.6 GHz |
| Propagation channel | HST tunnel channel model: Ds = 300 m and Dmin = 2 m HST open space channel model: Ds = 700 m and Dmin = 150 m |
| SCS and BW | 15kHz: 5 MHz 30kHz: 10 MHz |
| Speed (v) | 350 km/h and 500 km / h |
| Maximum Doppler shift (fd) | SCS = 15 kHz: fd = 1340 Hz (for UE speed = 350 km/h) and fd = 1740 Hz ((for UE speed = 500 km/h)) SCS = 30 kHz: fd = 23344 Hz (for UE speed = 350 km/h) and fd = 3334 Hz ((for UE speed = 500 km/h)) |
| Testing metric | SNR @70% of maximum throughput |

Table 1. Simulation assumptions for HST single tap scenarios at 350km/h and 500 km/h [1].

Table 2. Ideal simulation results for NR HST PUSCH demodulation performance at UE speed = 350 km/h.

| Scenario | SCS / BW | MCS | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Tunnel (1Tx1Rx) | 15 kHz / 5 MHz | 2 | -3.0 |
|  |  | 16 | 9.7 |
|  | 30 kHz / 10 MHz | 2 | -3.0 |
|  |  | 16 | 9.6 |
| Tunnel (1Tx2Rx) | 15 kHz / 5 MHz | 2 | -6.0 |
|  |  | 16 | 6.7 |
|  | 30 kHz / 10 MHz | 2 | -6.0 |
|  |  | 16 | 6.5 |
| Open space (1Tx2Rx) | 15 kHz / 5 MHz | 2 | -6.1 |
|  |  | 16 | 6.3 |
|  | 30 kHz / 10 MHz | 2 | -6.0 |
|  |  | 16 | 6.2 |
| Open space (1Tx8Rx) | 15 kHz / 5 MHz | 2 | -11.9 |
|  |  | 16 | 0.3 |
|  | 30 kHz / 10 MHz | 2 | -12.0 |
|  |  | 16 | 0.3 |

Table 3. Ideal simulation results for NR HST PUSCH demodulation performance at UE speed = 500 km/h.

| Scenario | SCS / BW | MCS | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Tunnel (1Tx1Rx) | 15 kHz / 5 MHz | 2 | -3.0 |
|  |  | 16 | 10.2 |
|  | 30 kHz / 10 MHz | 2 | -3.0 |
|  |  | 16 | 10.2 |
| Tunnel (1Tx2Rx) | 15 kHz / 5 MHz | 2 | -6.0 |
|  |  | 16 | 7.1 |
|  | 30 kHz / 10 MHz | 2 | -6.0 |
|  |  | 16 | 7.1 |
| Open space (1Tx2Rx) | 15 kHz / 5 MHz | 2 | -6.1 |
|  |  | 16 | 6.5 |
|  | 30 kHz / 10 MHz | 2 | -6.0 |
|  |  | 16 | 6.5 |
| Open space (1Tx8Rx) | 15 kHz / 5 MHz | 2 | -12.0 |
|  |  | 16 | 0.5 |
|  | 30 kHz / 10 MHz | 2 | -12.0 |
|  |  | 16 | 0.5 |

Table 3. Results with impairments for NR HST PUSCH demodulation performance at UE speed = 350 km/h.

| Scenario | SCS / BW | MCS | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Tunnel (1Tx1Rx) | 15 kHz / 5 MHz | 2 | -0.9 |
|  |  | 16 | 11.9 |
|  | 30 kHz / 10 MHz | 2 | -1.0 |
|  |  | 16 | 11.7 |
| Tunnel (1Tx2Rx) | 15 kHz / 5 MHz | 2 | -3.6 |
|  |  | 16 | 9.2 |
|  | 30 kHz / 10 MHz | 2 | -3.6 |
|  |  | 16 | 9.0 |
| Open space (1Tx2Rx) | 15 kHz / 5 MHz | 2 | -4.0 |
|  |  | 16 | 8.5 |
|  | 30 kHz / 10 MHz | 2 | -3.6 |
|  |  | 16 | 8.5 |
| Open space (1Tx8Rx) | 15 kHz / 5 MHz | 2 | -9.8 |
|  |  | 16 | 2.6 |
|  | 30 kHz / 10 MHz | 2 | -9.5 |
|  |  | 16 | 2.4 |

Table 5. Results with impairments for NR HST PUSCH demodulation performance at UE speed = 500 km/h.

| Scenario | SCS / BW | MCS | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Tunnel (1Tx1Rx) | 15 kHz / 5 MHz | 2 | -0.8 |
|  |  | 16 | 12.2 |
|  | 30 kHz / 10 MHz | 2 | -0.8 |
|  |  | 16 | 12.2 |
| Tunnel (1Tx2Rx) | 15 kHz / 5 MHz | 2 | -3.9 |
|  |  | 16 | 9.7 |
|  | 30 kHz / 10 MHz | 2 | -3.5 |
|  |  | 16 | 9.7 |
| Open space (1Tx2Rx) | 15 kHz / 5 MHz | 2 | -4.1 |
|  |  | 16 | 8.6 |
|  | 30 kHz / 10 MHz | 2 | -3.8 |
|  |  | 16 | 8.9 |
| Open space (1Tx8Rx) | 15 kHz / 5 MHz | 2 | -10.0 |
|  |  | 16 | 2.5 |
|  | 30 kHz / 10 MHz | 2 | -9.9 |
|  |  | 16 | 2.8 |

# Conclusion

In this contribution. we summarized our simulation results for {SCS. BW} combinations = {15kHz. 5MHz} and {30kHz. 10 MHz} for NR HST PUSCH demodulation performance at UE speed of 350 km/h and 500 km/h.

# References

R4-2008821. WF on Rel-16 NR HST BS demodulation requirements. Nokia. Nokia Shanghai Bell....